3GPP TSG RAN WG1 Meeting                                                                                             R1-151802

Belgrade, Serbia, 20th – 24th April 2015

Source: 	Ericsson

Title:	HARQ transmission on PUSCH for CA enhancement

Agenda Item:	7.2.2.2.2

Document for:	Discussion and Decision

# Introduction

A new WI was agreed at RAN #66 plenary meeting to support LTE carrier aggregation beyond 5 carriers [1]. In RAN1#80 meeting, it was observed that enhancements on UCI transmission on PUSCH need to be considered:

- Enhancements on UCI transmission on PUSCH

  - Details FFS including but not limited to

    - Supported payload size[s]

    - Channel coding and resource element mapping

In this contribution, we discuss the enhancements to support HARQ-ACK transmission on PUSCH for up to 32 DL component carriers.

# Discussion

Due to aggregation of a large number of DL carriers, the number of HARQ-ACK bits increases significantly and occupies many resources on PUSCH with current mechanism.

Several options have been discussed to enhance the piggyback of more HARQ-ACK bits on PUSCH. One option is to utilize more SC-FDMA symbols for HARQ-ACK transmission. However, this option results in excessive puncturing on PUSCH which affects the data transmissions on PUSCH. Another option is to use all SC-FDMA symbols in the PRB(s) for HARQ-ACK transmission. This option provides possibilities to piggyback a large number of HARQ-ACK bits. However, it results in inefficient resource utilization especially for HARQ-ACK with small to medium payload size. In addition, separating HARQ-ACK feedback and data multiplexing on PUSCH brings standards impacts as well as implementation impacts e.g. on eNB scheduling.

Herein, we discuss some enhancements to handle large number of HARQ-ACK bits on PUSCH for CA enhancements with minimum standard and implementation impacts.

Firstly, the piggyback of large number of HARQ-ACK bits on PUSCH can be largely handled by eNB scheduling. The increased payload size of HARQ-ACK is due to that there are a large number of configured DL carriers. For example, 32 DL carriers in FDD results in HARQ-ACK of 64 bits. In practice, a large number of carriers are configured and utilized mainly because there are large amount of DL traffic. Accordingly, by only considering the TCP-ACK for DL traffic, a lot of UL traffic can be expected. Therefore, eNB can schedule a number of PRBs for PUSCH data transmission and hence HARQ-ACK can be multiplexed on multiple PRBs.

Proposal:

- The piggyback of large number of HARQ-ACK bits on PUSCH can be largely handled by eNB scheduling.

Secondly, the HARQ-ACK encoding can be enhanced for large number of HARQ-ACK bits. In Rel-12, there are maximum 20 HARQ-ACK bits and dual RM code is applied. When HARQ-ACK payload size increases, tail biting convolutional code (TBCC) should be applied instead of RM code. It has been shown in [2] that there is an increased advantage with TBCC compared to RM code for increased payload size.

Proposal:

- Adopt TBCC for HARQ-ACK on PUSCH for CA enhancements.

Thirdly, the HARQ-ACK resource element mapping can be optimized. In current mechanism, the number of coded modulation symbols per layer Q' for HARQ-ACK or rank indicator is calculated as follow:

One observation on above equation is that with few HARQ-ACK bits, a large value of βoffsetPUSCH is needed in order to perform DTX detection i.e. to determine if HARQ-ACK bits are included in the transmission or not. When the number of HARQ-ACK bits increases, the problem of performing DTX detection gets much relaxed. The reason is that since the number of symbols Q' is proportional to the number of HARQ-ACK bits, the number of possible transmission hypotheses for the DTX case grows faster than the number of allowed codewords for the non-DTX case. Thus when above equation with a fixed value of βoffsetPUSCH is used to calculate the number of coded modulation symbols independently of the number of HARQ-ACK bits, there is inefficient use of PUSCH resources due to excessive puncturing.

In Figure 1, HARQ and PUSCH performance are shown for different number of HARQ-ACK bits using different beta offsets. In subfigure (a) and (b), βoffsetPUSCH=20 is used for HARQ-ACK of 2, 6, 12 and 18 bits. It can be seen that when using a large beta offset for large number of HARQ-ACK bits, the PUSCH BLER is very high. The reason is that with large beta offset the HARQ-ACK resource mapping is not optimized which results in excessive puncturing on PUSCH data. In subfigure (c) and (d), a smaller beta offset βoffsetPUSCH=6.25 is used for HARQ-ACK of 6, 12 and 18 bits. It can be observed that the less PUSCH resources are punctured and hence the PUSCH performance improves significantly.

Figure 1: HARQ-ACK and PUSCH performance with different beta offset (EPA5, 5 PRBs)

Therefore, by optimizing the HARQ-ACK resource element mapping, i.e., using a smaller beta offset for a large number of HARQ-ACK bits when calculating the number of coded modulation symbols for HARQ-ACK, the resource is better utilized with less puncturing on PUSCH and the link performance can be improved.

Proposal:

- The HARQ-ACK resource element mapping can be optimized

  - When calculating the number of coded modulation symbols for a large number of HARQ-ACK bits, a smaller beta offset can be used so that the resource is better utilized with improved link performance.

# Conclusion

In this contribution we discussed HARQ-ACK on PUSCH to support carrier aggregation with up to 32 carriers. The above discussion is summarized with the following proposals:

Proposal:

- The piggyback of large number of HARQ-ACK bits on PUSCH can be largely handled by eNB scheduling.

- Adopt TBCC for HARQ-ACK on PUSCH for CA enhancements.

- The HARQ-ACK resource element mapping can be optimized

  - When calculating the number of coded modulation symbols for a large number of HARQ-ACK bits, a smaller beta offset can be used so that the resource is better utilized with improved link performance

# References

RP-142286, “New WI proposal: LTE Carrier Aggregation Enhancement Beyond 5 Carriers On the number of antenna columns”.

R1-151800, “PUCCH format design for CA enhancement”, Ericsson