3GPP TSG RAN WG1 Meeting 80bis	R1-151849

Belgrade, Serbia, April 20 - 24, 2015

Agenda Item:	7.2.2.2.2

Source:	Huawei, HiSilicon

Title:	PUCCH resource allocation

Document for:	Discussion and decision

# Introduction

A set of observations were made in RAN1#80, including the following:

- For possible enhancements to UL control signaling to PUCCH formats and UCI on PUSCH,

- At least the following enhancements to PUCCH and PUSCH feedback formats could be considered in order to support the increase in UL control information based for the 36.300 CA deployment scenarios:

- The studies should take the effect on DL throughput and UL operation points into account

- One or more new PUCCH format for increasing PUCCH payload capacity including considerations on UL overhead

  - Details FFS including but not limited to

    - Supported payload size(s)

    - Channel coding

    - Detailed structure of the new format

The detailed structure of the new PUCCH format is not yet defined and we discuss this in [1] while evaluations of channel coding are contained in [2]. In this contribution, we consider aspects of PUCCH resource allocation. The key points we make, are that for large payloads and efficient resource utilization, the following features for PUCCH resource allocation should be considered further:

- Allocation of multiple frequency contiguous PRB pairs within a slot in order to accommodate large UCI payloads.

- Adaptation of the number of PRB pairs in order to accommodate varying UCI payloads.

- Adaptation of the locations of the PRB pairs in order to improve PUCCH resource utilization.

# PUCCH resource allocation

## Requirements on a new PUCCH format

# The maximum UCI payload has not yet been determined but it will likely be significantly larger than for PUCCH in Rel-12. For HARQ-ACK feedback, the range of 64-320 bits has been identified as relevant for selecting the maximum payload [3]. If a value is selected from the lower part of the range, more HARQ-ACK bundling will be needed, at least for TDD, and/or certain carrier aggregation configurations will not be supported. If a value is selected from the higher part of the range, less bundling will be needed but more PUCCH overhead is introduced. It was also suggested to reuse the potential new PUCCH format for periodic CSI feedback [3], in case transmission of multiple periodic CSI reports per reporting instance will be supported. Therefore, the following characteristics should be taken into account when allocating the resources. A new PUCCH format may:

- Have significantly larger maximum payload than existing PUCCH formats.

- Contain HARQ-ACK feedback.

- Contain periodic CSI (possibly from multiple serving cells).

- Contain HARQ-ACK feedback and periodic CSI (possibly from multiple serving cells).

Figure 1. Examples of PUCCH resource allocation using 3 PRBs per slot and non-contiguous allocation between slots (left) and contiguous allocation between slots (right).

## Impact on PUCCH resource allocation

Amount of PUCCH resources

One way to accommodate large UCI payloads is to allocate several PUCCH resources in a slot. In order to keep the low-PAPR SC-FDMA waveform, it is preferable to allocate multiple PRB pairs rather than multiple PUCCH resources (e.g., multiple sequences) within a PRB pair. Within a slot, the location of the PRB pairs should thus be contiguous in frequency to allow using DFT precoding for achieving low PAPR/CM. If a new PUCCH format could utilize multiple PRB pairs within a slot, it could be considered to allocate them either contiguously (Fig. 1 left) or non-contiguously (Fig. 1 right) between slots. The tradeoff between channel estimation performance and frequency diversity should be studied for these two options. Candidates for multiple-PRB PUCCH formats could include:

- Multiple-PRB PUCCH format 3

- PUSCH based transmission format

The former can be multiplexed with PUCCH format 3, while the latter will require a new set of PUCCH resources.

In [2], it was shown that sufficiently low code rates can be obtained by allocating multiple PRB pairs. Therefore, large UCI payloads are not problematic. However, it may not be necessary to always allocate a fixed (i.e., the maximum) number of PRB pairs for all kinds of different UCI payloads. Consequently, since the number of UCI feedback bits could vary among subframes, it could be considered to allow the eNodeB to adapt the number of allocated PRB pairs accordingly. It is also not desirable to specify many PUCCH formats of completely different structure, as that complicates the PUCCH resource utilization. Thus it will suffice to specify one or a few new PUCCH format, for which the number of allocated PRB pairs is variable.

Usage of PUCCH resources

In Rel-10, the HARQ-ACK feedback mechanisms for carrier aggregation were not optimized assuming a large number of UEs being simultaneously scheduled on multiple serving cells. This would apply also for Rel-13 [3] and further assuming that the number of UEs capable of aggregating more than 5 serving cells is supposedly going to be small, this suggests that the resources for the new PUCCH format may not be used very frequently, at least not in every subframe. A new PUCCH format utilizing multiple PRBs in a slot will require allocation of more resources than current PUCCH formats, albeit with less resource utilization. Therefore, it is crucial that the PUCCH resource allocation scheme can provide means for flexible allocation of the resources.

Allocation of PUCCH resources

For UCI consisting of HARQ-ACK feedback, PUCCH resources could be allocated using similar mechanism as for PUCCH format 3, i.e., bits in a downlink control information (DCI) format on the PDCCH/EPDCCH which are associated with higher layer configured resources (i.e., PRB pairs). On the other hand, if the UCI would consist of only CSI reports, there is no DCI and a set of higher layer configured resources may be needed. In that case, for better resource utilization, the eNodeB may prefer to let the higher layer configured PUCCH resources for CSI be located close to the carrier edges (similar to PUCCH format 2/2a/2b). Nevertheless, it would be advantageous if the resource allocation scheme can assure that such higher layer configured resources could be reused even when the UCI consists of only HARQ-ACK feedback and no CSI, or HARQ-ACK feedback and CSI.

If the new PUCCH format cannot be multiplexed on PRBs containing any of the existing PUCCH formats, it may be inefficient to allocate disjoint PUCCH resources for the new PUCCH format, since they may be used infrequently. It would be beneficial to be able to configure a set of PRB pairs which could overlap with the PUCCH resources for the existing PUCCH formats and let the eNodeB perform the allocation dynamically in this set, assuring that there is no simultaneous use of PUCCH resources for the different PUCCH formats in the overlapped PRB pairs. This improves the statistical multiplexing gain of the PUCCH resources.

Furthermore, in some cases there are performance gains of using 16-QAM instead of QPSK [2], which may impact the PUCCH resource allocation scheme, if 16-QAM on PUCCH will be supported.

Thus, for a multiple-PRB PUCCH format, the PUCCH resource allocation scheme should be able to provide:

- Any number of 1, 2, …, K contiguous PRBs in a slot, where K is FFS and K>1.

- The positions of the PRBs.

- The modulation level for the PRBs, if 16-QAM is supported.

# Conclusions

A multiple-PRB PUCCH format will be able to provide the increased UCI payloads. Considering the issues of varying payloads and efficient resource utilization, it would suffice to specify a new PUCCH format where the number of PRBs can be varied. For this, the following features for PUCCH resource allocation should be considered further:

Proposal: If a new multiple-PRB PUCCH format is defined, the PUCCH resource allocation scheme should be able to provide:

- Any number of 1, 2, …, K contiguous PRBs in a slot, where K is FFS and K>1.

- The positions of the PRBs.

- The modulation level for the PRBs, if 16-QAM is supported.

# References

Huawei, HiSilicon, “New PUCCH format design to support UCI transmission for up to 32 component carriers”, R1-151275, Belgrade, Serbia, April 20-24, 2015.

Huawei, HiSilicon, “UCI channel coding”, R1-151850, Belgrade, Serbia, April 20-24, 2015.

Huawei, HiSilicon, “On CA enhancements supporting up to 32 component carriers”, R1-151851, Belgrade, Serbia, April 20-24, 2015.