3GPP TSG-RAN WG1 Meeting #80bis	R1-152086

Belgrade, Serbia, 20th – 24th April 2015

Agenda Item:	6.3.2

Source: 	Ericsson

Title:  	Text proposal on offloading solutions for Network Assisted Interference Cancellation and Suppression for UMTS

Document for:	Discussion

# Introduction

This contribution provides the text proposal for the section on offloading enhancements in the Technical Report on Network-Assisted Interference Cancellation and Suppression for UMTS [1], reported in [2].

# Text Proposal

[------------------------------------------------------ TEXT START ---------------------------------------------------------]

### Information to perform offloading

In order to  motivate a cell change, the network needs, at the bare minimum, information on the load situation from the source and target cells as well as the link quality for the to-be-offloaded UE with respect to these source and target cells.

Load information is available in the network in a variety of forms and is not considered troublesome to obtain. For example, the RNC collects downlink transmit power C-PICH measurements, path-loss estimates information from cells in the active set and can use this information to estimate whether offloading will improve the load balance in the network.

Link quality information for HSDPA is typically given by Channel Quality Information (CQI). Downlink CQI is calculated in the UE based on CPICH measurement and then forwarded to the network via HS-DPCCH. To enable offloading, an estimate of the link quality, based on CQI measurements, from the serving cell as well as other cells in the active set are of interest (at least the second best cell).

The link quality information provided by CQI is very short term and varies on a TTI basis. In order to provide a stable offloading mechanism, that instantaneous measurement should be smoothed over a longer period. This will avoid creating a scenario where a UE gets offloaded back and forth as soon as the channel situation worsens.

### Definition of the second best cell

The cell that is designated as the target of the offloading procedure is called the second-best cell. In this definition, it is understood that the cell constitutes the second best choice for the UE from the network point of view.

The UE is aware of all the cells in its active set and is capable of performing link quality estimates on each of these cells. In order to define the second best cell, the UE shall measure CQI on each of the cells in the active cell and, after filtering of the measurements, reports the CQIs for the serving cell as well as the best available filtered CQI in the active set excluding the serving cell.  The cell with best CQI in the active set (i.e. excluding the serving cell) is defined as the second best cell.

Note that it is necessary for the UE to identify the cell corresponding to the transmitted filtered CQI. This is discussed in subclause 7.3.2.

### Rate of information exchange

In the current specification, CQI is delivered to the network on a periodic basis, controlled by the CQI feedback cycle. The link quality estimates used for offloading information can also be transmitted to the network periodically under a pre-established pattern, or on a need basis. In that case a mechanism for transmitting link quality estimates CQI for offloading calculation purposes should be established.

### Enabling offloading in the network with existing measurements

In order to minimize impact on HSDPCCH, another choice could be to base the offloading decision on existing measurements, for example the received CPICH power and path loss for each cell in the active cell list, plus the knowledge that the network has about the load situation on neighbor cells.  In that case, the network is in charge of deciding on the second best cell as well as deciding on the offloading. The choice on the second best cell and the offloading algorithm are left to proprietary solutions. Once the decision about offloading a UE has been taken by the network, most likely a cell reselection procedure would be required to be applied by following the legacy behavior.

The UE receives measurement requests from the network via RRC signalling. No additional changes to HSDPCCH are necessary.

### Channels for reporting link quality estimates

The UE produces the link quality information itself (i.e. the UE computes a filtered CQI measurement)  and reports link quality information in the form of filtered CQI for both the serving cell and the second best cell.  The report can be done using a pre-agreed period between reports, or instead follow a request from the network.

The serving cell CQI is transmitted on HS-DPCCH along with HARQ and PCI information. One possibility is to send the new filtered CQI information associated to the serving and second best cell in the TTI using a format similar to what HSDPCCH uses when configured with dual carrier HSPA (see 4.7.3 in [2]). The proposal is to adopt a subframe format with slot 0 containing HARQ for the serving cell, while slot 1 and 2 conveys the link quality information for the serving cell and the second best cell

The link quality information for both cells can be reported either using 1 slot for each cell (10 bits coding) or by creating a unique codeword for both CQI over the two slots (20 bits coding). As an example, the 20-bits codeword solution is considered. For implementation of the encoding, the encoding procedure explained in [2], subclause 4.7.3.2 could be reused.

In order to add second best cell identification, a 3-bit pattern corresponding to the cell index of the second best cell in the active set list is appended to the link quality information for both cells. Figure x1 shows an example of the mapping for the filtered CQIs for both, the serving cell and second best cell as well as the second best cell ID to a single 20 bits codeword.  Several mapping and coding solutions are possible.

Figure x1: construction of CQI codeword with second best cell identification

[------------------------------------------------------TEXT OMITTED-----------------------------------------------------]

# Conclusions

Upon reviewing the content of the Text Proposal, it is proposed to:

Proposal: Agree on the text proposal and capture the content presented in this document in the Technical Report on Network-Assisted Interference Cancellation and Suppression for UMTS [2].

# References

- RP-142250, " New Study Item proposal: Study on Network-Assisted Interference Cancellation and Suppression for UMTS ", Huawei, HiSilicon.

- 3GPP TR 25.766 v0.1.0, Technical Report on Network-Assisted Interference Cancellation and Suppression for UMTS.