3GPP TSG RAN WG1 Meeting #81                       					R1-152681

Fukuoka, Japan, 25 – 29 May 2015

Agenda Item:       6.2.1.4

Source:            NEC

Title: 				Uplink Reference Signal Enhancement for Rel-13 low complexity MTC

Document for:   	Discussion and Decision

- Introduction

In RAN1#80, the following agreements have been reached for the transmission of PUSCH for Rel-13 low complexity MTC UEs with a coverage enhancement mode[2]:

- For ‘physical channel(s) carrying UL data’ repetition (including different RVs) for Rel-13 low complexity MTC UEs with a coverage enhancement mode, the following techniques are supported

  - Multiple-SF channel estimation

  - Frequency hopping over system bandwidth across subframes

    - Network can enable or disable the hopping

      - FFS details of configuration

  - FFS on other techniques

According to the agreements, multiple-subframe channel estimation and frequency hopping over system bandwidth across subframe will be supported. Other techniques are FFS. In the summarized contribution of the simulation results from RAN1#80[3], the following observation has been reached.

For 18dB gain and SF ave of 1 and 4, 7 of 8 companies results shows that increasing DMRS by 2X is an effective method to reduce the number of PUSCH repeats necessary. For all other cases, increasing DM-RS had no effect.

The contributions [4-10] submitted to RAN1#80bis also support the observation.

In this contribution, we analyse and evaluate the possible uplink demodulation reference signal (RS) enhancement solutions.

- Reference signal enhancement for PUSCH coverage enhancement

It has been shown that the accuracy of channel estimation is important for the PUSCH coverage enhancement for MTC UE. Then the improvement to the reference signal becomes mandatory. The benefit of increasing the DMRS density in the uplink has been analysed, the performance gain is significant at lower SNR.

Increased reference symbol density

For PUSCH transmission, eNodeB performs PUSCH demodulation based on demodulation RS (DMRS). The principles for uplink DMRS are quite different compared to downlink CRS and DMRS. According to the specification [11], one OFDM symbol as shown in Fig. 1 is used exclusively for DMRS transmission in a slot while downlink RS and PDSCH REs are staggered in both frequency and time domain. This constraint limits the improvement solutions to uplink DMRS.

To improve the uplink channel estimation accuracy for MTC in the extremely low SNR region, it is better to increase the RS density in the time domain. One straightforward method is to increase the DMRS symbols shown in Fig.2. We define the new RS as MTCDMRS. They occupy the first symbol of either slot. The sequence could be the same as legacy uplink DMRS. So the uplink channel estimation could be carried out based on DMRS and MTCDMRS at the same time. In the example, the density of DMRS is increased to 4 REs/subcarrier.

Figure 1, Mapping of uplink DMRS

Figure 2, Mapping of uplink DMRS and MTCDMRS

- Evaluation results

In the performance evaluation, we studied the possible scheme to reduce the number of repetition when MTC UE needs coverage enhancement. The number of repetitions to achieve 10% BLER works as the evaluation output. Detailed simulation assumptions are shown in the appendix.

Table 1 shows the amount of repetitions for EPA channel when the number of PRBs is 1 and the MCS is 5. The resulting SINR for baseline case (no additional coverage enhancement techniques) is 2.5dB.

According to the simulation results, increased DMRS density can reduce the numbers of repetition especially the coverage enhancement gain is 18dB. The repetition times is decreased from 238 to 183, and increased DMRS density can save about 23% repetition times. Increased DMRS density can work together with cross-subframe channel estimation which can decrease the repetition times further.

Then following observation could be made according to the simulation results,

Observation 1: Increased DMRS density could reduce the number of repetition.

So we propose,

Proposal 1: Consider to increase the DMRS density for MTC PUSCH transmission.

Table 1.Repetition times with frequency tracking error =100 Hz, EPA channel

| RS Pattern | Cross-SF Ch. Est. | Required Gain(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 6 | 12 | 18 |
| Rel.8 DMRS | 1 | 5 | 31 | 238 |
|  | 4 | 4 | 20 | 113 |
|  | 8 | 4 | 19 | 88 |
| MTC DMRS | 1 | 4 | 28 | 183 |
|  | 4 | 4 | 19 | 105 |
|  | 8 | 4 | 18 | 86 |

- Conclusion

This contribution analyses the possible solutions to improve coverage for MTC PUSCH transmission and find,

Observation 1: Increased DMRS density could reduce the number of repetition.

Then we propose,

Proposal 1: Consider to increase the DMRS density for MTC PUSCH transmission.

References

[1] 3GPP TR 36.888 V12.0.0, “Study on provision of low-cost MTC UEs based on LTE (Release-12)”.

[2] RAN1Chairman’s notes (final version), RAN1#80.

[3] R1-150759 PUSCH simulation Summary for Rel-13 LC UEs, Sierra Wireless.

[4] R1-152054 Details on PUSCH for low complexity MTC, NTT DOCOMO, INC.

[5] R1-151665 DMRS density increase for MTC coverage enhancement, Panasonic.

[6] R1-151550 Uplink Reference Signal Enhancement for Rel-13 low complexity MTC, NEC.

[7] R1-151340 Evaluation of PUSCH coverage enhancement, CATT.

[8] R1-151309 PUSCH Coverage Enhancement Techniques, Nokia Networks.

[9] R1-152127 PUSCH coverage enhancement for MTC UE, INTERDIGITAL COMMUNICATIONS.

[10] R1-152189 PUSCH link performance for MTC, Ericsson.

[11] 3GPP TS 36.211, “Physical channels and modulation”, Release 11, 2012-12.

Appendix:  Simulation assumptions on PUSCH

| Parameter | Value |
| --- | --- |
| System bandwidth | 10MHz |
| Frame structure | FDD |
| Carrier frequency | 2.0 GHz for FDD/ 2.6GHz for TDD |
| Antenna configuration | 1x2, low correlation for FDD; 1x8, low correlation for TDD |
| Channel model | EPA |
| Doppler shift | 1Hz |
| TBS | 72 |
| Number of UL RBs | 1 |
| Transmission mode | TM1 |
| Frequency tracking error | 100Hz |
| Performance target | 10% iBLER |
| Channel estimation | Realistic channel estimation |
| Output | The amount of repetitions |