3GPP TSG RAN WG1 Meeting #81	R1-153128

Fukuoka, Japan, 25th - 29th May 2015

Source:	Ericsson

Title:	Coexistence Evaluation Results for DL+UL LAA with Cat 3 UL LBT

Agenda Item:	6.2.4.2

Document for:	Discussion and Decision

# Introduction

Substantial progress was made for the Study Item on Licensed-Assisted Access (LAA) in RAN1 WG #80bis [1] with additional simulation assumptions for DL+UL LAA.

In [2], we evaluated coexistence of Wi-Fi and DL+UL LAA with a regular Cat 3 UL LBT algorithm. We found DL+UL LAA with Cat 3 UL LBT can coexist in a fair manner with Wi-Fi by enabling the non-replaced Wi-Fi network to achieve better performance than when operating on the channel with another Wi-Fi network. However, we also identified several issues for further study:

The LTE/LAA UL transmission, when being self-scheduled from the eNB, requires two successful LBT procedures to commence transmission: the eNB needs to succeed at sending the UL grant and the scheduled UE needs to succeed in initiating UL transmissions. In the Wi-Fi network, UL transmissions are initiated by the mobile units autonomously. This results in the following behavior.

- The Wi-Fi network with both DL and UL traffic manages to achieve higher served traffic than the LAA network with both DL and UL traffic when the offered loads to both networks are identical.

- As the Wi-Fi network attempts to serve more and more UL traffic, the amount of the offered UL traffic that the LAA network is able to serve drops very fast.

It is concluded that further study is needed to enhance LAA UL transmission success rates while considering the performance of both LAA and coexisting Wi-Fi networks. Examples include faster LBT for UL transmissions and/or control signaling for UL grants.

The IEEE 802.11ax TG has recently approved the following motion on UL OFDMA operations [3]:

An UL MU PPDU (MU-MIMO or OFDMA) is sent as an immediate response (IFS TBD) to a Trigger frame (format TBD) sent by the AP.

That is, this new scheduled Wi-Fi UL mode will allow multiple transmissions to follow the grant with a short delay and without performing any LBT by any of the scheduled Wi-Fi stations. This is in addition to the reverse direction grant in the IEEE 802.11n protocol, which also allows the reverse direction transmission to follow the first transmission without LBT (within the TXOP of the first transmitter).

In light of these findings, we investigate in this contribution the coexistence performance of Wi-Fi and DL+UL LAA with a shorter Cat. 3 UL LBT algorithm. The UL grant transmission is based on self-scheduling with a grant delay of 4 ms.

# Description of DL and UL LBT algorithms

## LAA DL LBT algorithm

Category 3 LBT algorithm based on LBE LBT [5][6]

- A random backoff counter, N, is always drawn to start the LBT procedure.

- An initial CCA is always immediately followed by an extended CCA stage.

- A successful transmission always leads to a restart of the LBT procedure with a newly drawn random backoff counter, N.

- Default value of CCA slot duration is T1 = 20 μs.

- Q = 10

## LAA UL LBT algorithm

Category 3 LBT algorithm based on LBE LBT [5][6]

- Same as in the DL except Q = 4

# Wi-Fi – LAA coexistence evaluation results and discussion

The results presented in Figure 1 and also captured in Table 1 provide an overview on the coexistence of LAA with Wi-Fi when shorter Cat 3 UL LBT algorithm is used for LAA. Both networks carry both DL and UL traffic. Figure 1 shows that fair coexistence between Wi-Fi and LAA with both UL and DL traffic is ensured. However, Figure 1 also reveals that the performance of the shorter Cat 3 LAA UL in this coexistence scenario can be further improved. This is further clarified in Figure 2. From the ratio of served over offered traffic in the uplink depicted in Figure 2, it can be observed that the Wi-Fi network with both DL and UL traffic manages to achieve higher served traffic than the LAA network with both DL and UL traffic when the offered loads to both networks are identical. As the Wi-Fi network attempts to serve more and more UL traffic, the amount of the offered UL traffic that the LAA network is able to serve drops very fast.

Some of the major elements which make LAA a good neighbor to Wi-Fi in the unlicensed band where both technologies have to share the medium are discussed in the following. The LAA uplink traffic is scheduled by the LAA eNBs, which reduces the number of contending nodes at any given time. In the Wi-Fi network, all UEs with non-empty UL buffer contend for access to the medium. In contrast, in the LAA network, only the few LAA UEs that are scheduled at a given time attempt to access the medium. LTE, as compared to Wi-Fi, has more robust interference mitigation, error correction and retransmission schemes which result in greater efficiency in serving the traffic and reducing resource utilization thus providing more opportunities for other systems to access the medium. LAA uses the same sensing threshold irrespective of transmissions from LAA or Wi-Fi nodes as opposed to Wi-Fi which uses a higher sensing threshold for LAA as compared to Wi-Fi.

Observation:

- A DL+UL LAA network operating Category 3 DL LBT and shorter Category 3 UL LBT algorithms can coexist with a Wi-Fi network.

- DL user throughputs				(b) UL user throughputs

Figure 1:  Mean user throughputs of the indoor test scenario with FTP traffic in DL on the left and UL on the right. Each network has 4 eNBs/APs and 20 UEs. Operator B has 2 VoIP UEs. Both operator A and B networks have 50% DL and 50% UL traffic. For LAA, licensed band PCell is not used for DL traffic in this test.

Figure 2:  Ratio of served traffic in uplink over the offered traffic in uplink.

Table 1: Indoor deployment for Wi-Fi and LAA coexistence case with one shared unlicensed carrier and Mixed traffic. Both Operator A and Operator B network has DL and UL traffic with 50/50 split. Each network has 4 eNBs/APs and 20 UEs. The non-replaced Wi-Fi network is operator B. Operator B has 2 VoIP UEs.

| Reported parameters |  | Low load BO range for Wi-Fi in Step 1: 10%~25% |  |  |  | Medium load BO range for Wi-Fi in Step 1: 35%~50% |  |  |  | High load BO range for Wi-Fi in Step 1: above 55% |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Wi-Fi Opt.B in step 1 | Wi-Fi Opt.A in step 1 | Wi-Fi Opt. B in step 2 | LAA Opt.A in step 2 | Wi-Fi Opt.B in step 1 | Wi-Fi Opt.A in step 1 | Wi-Fi Opt. B in step 2 | LAA Opt.A in step 2 | Wi-Fi Opt.B in step 1 | Wi-Fi Opt.A in step 1 | Wi-Fi Opt. B in step 2 | LAA Opt.A in step 2 |
| DL: UPT CDF [Mbps] | 5% | 28.780 | 26.730 | 46.870 | 72.010 | 7.2600 | 8.6100 | 33.580 | 49.800 | 1.6200 | 1.9800 | 21.250 | 34.610 |
|  | 50% | 66.990 | 67.230 | 82.130 | 99.860 | 47.320 | 48.100 | 70.360 | 90.820 | 24.400 | 25.910 | 57.700 | 75.360 |
|  | 95% | 93.090 | 93.790 | 101.10 | 113.31 | 80.040 | 82.680 | 95.070 | 107.98 | 64.690 | 66.710 | 87.310 | 101.79 |
|  | Mean | 65.970 | 66.550 | 80.620 | 98.740 | 46.200 | 47.570 | 70.020 | 88.130 | 29.710 | 30.880 | 58.640 | 74.95 |
| DL: Delay CDF [s] | 5% | 0.0380 | 0.0420 | 0.0350 | 0.0310 | 0.0400 | 0.0590 | 0.0360 | 0.0320 | 0.0470 | 0.0480 | 0.0420 | 0.0360 |
|  | 50% | 0.2340 | 0.2000 | 0.0540 | 0.0400 | 0.3700 | 0.6550 | 0.0740 | 0.0480 | 1.2440 | 1.4730 | 0.2350 | 0.0970 |
|  | 95% | 3.2800 | 2.9240 | 0.1760 | 0.0850 | 11.493 | 13.264 | 0.8030 | 0.3040 | 17.307 | 15.763 | 1.9680 | 0.9230 |
|  | Mean | 0.8840 | 0.8720 | 0.0800 | 0.0500 | 2.5910 | 2.9840 | 0.2290 | 0.1100 | 4.4350 | 4.4920 | 0.6130 | 0.2890 |
| UL: UPT CDF [Mbps] | 5% | 24.010 | 25.450 | 39.830 | 49.170 | 6.4200 | 6.6500 | 26.700 | 27.990 | 1.5800 | 1.5700 | 16.810 | 14.740 |
|  | 50% | 64.330 | 62.840 | 76.990 | 71.340 | 43.330 | 42.840 | 64.650 | 53.500 | 23.230 | 23.700 | 52.120 | 36.200 |
|  | 95% | 90.720 | 88.990 | 98.980 | 84.610 | 77.150 | 77.920 | 91.980 | 67.330 | 60.650 | 62.870 | 85.800 | 51.400 |
|  | Mean | 63.470 | 62.400 | 76.370 | 71.120 | 44.160 | 44.070 | 64.920 | 52.670 | 28.110 | 28.750 | 54.430 | 36.090 |
| UL: Delay CDF [s] | 5% | 0.0380 | 0.0390 | 0.0350 | 0.0410 | 0.0440 | 0.0410 | 0.0380 | 0.0530 | 0.0480 | 0.0550 | 0.0430 | 0.0700 |
|  | 50% | 0.2580 | 0.2690 | 0.0600 | 0.0610 | 0.7100 | 0.6560 | 0.0890 | 0.0990 | 1.8020 | 1.3710 | 0.2370 | 0.2330 |
|  | 95% | 2.7330 | 2.9920 | 0.2150 | 0.1970 | 10.961 | 9.2730 | 1.0670 | 1.1060 | 14.655 | 14.784 | 2.4320 | 2.8940 |
|  | Mean | 0.7570 | 0.8300 | 0.0900 | 0.0880 | 2.7490 | 2.2940 | 0.2700 | 0.3020 | 4.2740 | 3.9020 | 0.7260 | 0.8440 |
| VoIP outage |  | 0.4100 | N/A | 0.2200 | N/A | 0.6300 | N/A | 0.2800 | N/A | 0.8300 | N/A | 0.4400 | N/A |
| VoIP outage(DL) |  | 0.3400 | N/A | 0.2100 | N/A | 0.5700 | N/A | 0.2400 | N/A | 0.7800 | N/A | 0.4100 | N/A |
| VoIP outage(UL) |  | 0.1600 | N/A | 0.0100 | N/A | 0.2800 | N/A | 0.0600 | N/A | 0.5200 | N/A | 0.1100 | N/A |
| 𝜌DL |  | 0.9500 | 0.9500 | 1.0000 | 1.0000 | 0.8100 | 0.8300 | 0.9900 | 1.0000 | 0.6700 | 0.6900 | 0.9700 | 0.9900 |
| 𝜌UL |  | 0.9600 | 0.9500 | 1.0000 | 1.0000 | 0.8500 | 0.8700 | 0.9900 | 0.9900 | 0.7500 | 0.7600 | 0.9600 | 0.9200 |
| BO |  | 0.2000 | 0.1900 | 0.0700 | 0.0600 | 0.4000 | 0.3900 | 0.1400 | 0.1100 | 0.6000 | 0.5900 | 0.2400 | 0.2000 |
| 𝜆 |  | 0.203897 |  |  |  | 0.246513 |  |  |  | 0.276395 |  |  |  |
| Company/tdoc: Ericsson / R1-153128 LBT category: Category 3 on DL and UL with self-scheduling Additional information: Sensing threshold used: -82 dBm Whether defer periods are used or not: yes CCA and ECCA slot length: 20 μs Whether or not intra and/or inter-RAT detection is assumed: only CCA-ED Any significant deviations from evaluations methodology and assumptions: DL Q=10, UL Q=4 |  |  |  |  |  |  |  |  |  |  |  |  |  |

# LAA – LAA coexistence evaluation results

The coexistence evaluation results of two LAA networks are summarized in Table 2. The two networks have synchronized subframe timing.

Observation:

- DL+UL LAA networks operating Category 3 DL LBT and shorter Category 3 UL LBT algorithms can coexist with each other.

Table 2: Indoor deployment for LAA and LAA coexistence case with one shared unlicensed carrier. Both operator networks have DL and UL traffic with 50/50 split. Each network has 4 eNBs/APs and 20 UEs. The two networks have synchronized subframe timing.

| Tdoc / Company | LAA LBT cat. | Reported parameters |  | Low load BO range for Wi-Fi in Step 1: 10%~25% |  | Medium load BO range for Wi-Fi in Step 1: 35%~50% |  | High load BO range for Wi-Fi in Step 1: above 55% |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | LAA Opt.1 | LAA Opt.2 | LAA Opt.1 | LAA Opt.2 | LAA Opt.1 | LAA Opt.2 |
| Ericsson / R1-153128 | Cat 3 DL, Cat 3 UL | DL: UPT CDF [Mbps] | 5% | 40.98 | 45.68 | 25.38 | 27.77 | 13.62 | 14.07 |
|  |  |  | 50% | 70.9 | 71.73 | 49.8 | 50.1 | 29.99 | 30.34 |
|  |  |  | 95% | 90.33 | 90.98 | 71.1 | 72.52 | 48.29 | 49.93 |
|  |  |  | Mean | 71.07 | 72.1 | 50.95 | 51.94 | 31.58 | 32.44 |
|  |  | DL: Delay CDF [s] | 5% | 0.041 | 0.042 | 0.057 | 0.056 | 0.086 | 0.083 |
|  |  |  | 50% | 0.065 | 0.065 | 0.106 | 0.106 | 0.168 | 0.168 |
|  |  |  | 95% | 0.106 | 0.103 | 0.2 | 0.195 | 0.337 | 0.331 |
|  |  |  | Mean | 0.07 | 0.07 | 0.12 | 0.119 | 0.195 | 0.193 |
|  |  | UL: UPT CDF [Mbps] | 5% | 22.79 | 22.05 | 9.51 | 9.15 | 0.83 | 0.78 |
|  |  |  | 50% | 39.62 | 38.49 | 21.06 | 20.44 | 5.47 | 5.36 |
|  |  |  | 95% | 51.57 | 51.23 | 31.54 | 31.85 | 12.48 | 12.97 |
|  |  |  | Mean | 39.74 | 39.03 | 21.55 | 21.37 | 6.25 | 6.38 |
|  |  | UL: Delay CDF [s] | 5% | 0.077 | 0.077 | 0.178 | 0.161 | 0.421 | 0.412 |
|  |  |  | 50% | 0.134 | 0.138 | 0.486 | 0.509 | 1.788 | 1.717 |
|  |  |  | 95% | 0.277 | 0.28 | 2.148 | 2.297 | 5.82 | 6.228 |
|  |  |  | Mean | 0.157 | 0.162 | 0.806 | 0.861 | 2.544 | 2.542 |
|  |  | 𝜌DL |  | 1 | 1 | 1 | 1 | 1 | 1 |
|  |  | 𝜌UL |  | 1 | 1 | 0.97 | 0.96 | 0.81 | 0.81 |
|  |  | BO |  | 0.2 | 0.2 | 0.4 | 0.4 | 0.6 | 0.6 |
|  |  | 𝜆 |  | 0.501374 |  | 0.605591 |  | 0.759579 |  |
|  | Additional comments: Sensing threshold used, -82 dBm Whether defer periods are used or not: Yes CCA and ECCA slot length: 20 μs Inter-operator synchronization for LAA-LAA coexistence: synchronized Whether or not intra and/or inter-RAT detection is assumed: only CCA-ED Any significant deviations from evaluations methodology and assumptions: DL Q=10, UL Q=4 |  |  |  |  |  |  |  |  |

# Conclusions

In this contribution, we report coexistence evaluation results when the shorter Cat 3 LBT is used for LAA uplink transmission. Both LAA and WiFi networks have UL and DL traffic and the non-replaced WiFi network has VoIP users as well. We provide an analysis of the UL LAA performance with self-scheduling. We observe the following.

Observations:

- A DL+UL LAA network operating Category 3 DL LBT and shorter Category 3 UL LBT algorithms can coexist with a Wi-Fi network both in terms of FTP and VoIP performance.

- DL+UL LAA networks operating Category 3 DL LBT and shorter Category 3 UL LBT algorithms can coexist with each other.

Based on the investigation, we propose the following.

Proposal:

- Shorter Category 3 UL LBT should be considered as an option to achieve fair coexistence with Wi-Fi network considering the performance of both LAA and Wi-Fi networks.

# References

Chairman’s notes, 3GPP RAN1 WG #80bis

R1-152109 Coexistence Evaluation Results for DL+UL LAA and Wi-Fi, Ericsson

IEEE TGax March 2015 Minutes 11-15/0309r0.

Section 9.25 “Reverse Direction Protocol” IEEE Std 802.11-2012, Part II: Wireless LAN MAC and PHY Specifications

R1-150584 Further details on LBT design in DL for LAA, Ericsson

R1-151996 Discussion on LBT Protocols, Ericsson

3GPP TR 36.899 Study on Licensed-Assisted Access to Unlicensed Spectrum, v.0.4.0

# Appendix

Annex A: Coexistence Evaluation Assumptions

The simulation assumptions are based on the agreed coexistence assumptions in [5]. However our preferences on the assumptions that remained optional or need clarifications when results are presented are provided below. In all the indoor coexistence evaluations, the transmit power of the base station in the unlicensed band is assumed to be 18 dBm. Moreover, FTP model 3 is used for generating FTP traffic.

Table 3: Additional Wi-Fi system evaluation assumptions

| Parameter |  | Value |
| --- | --- | --- |
| MCS |  | 802.11ac MCS table with 256 QAM |
| Antenna configuration |  | 2Tx2Rx, Cross-polarized Baseline: open loop 2x2 MIMO |
| Channel coding |  | LDPC |
| Frame aggregation |  | A-MPDU |
| MPDU size |  | 1500B MSDU + 14 B header |
| Max PPDU duration |  | Baseline:< 4 ms (Asynchronous to LTE timing) |
| MAC | Coordination | EDCA |
|  | SIFS, DIFS | SIFS, DIFS |
|  | Detection | Energy detection & preamble detection |
|  | RTS/CTS | No |
|  | Contention window | Per EDCA |
| CCA-CS |  | -82dBm and preamble decoding (Note preamble occupies the 20MHz system bandwidth with rate 1/2 coding and BPSK modulation) |
| CCA-ED |  | -62dBm |
| ACK Modeled (successful reception, resources utilized) |  | Yes |
| DL/UL Duplexing |  | For the DL-only LAA coexistence evaluations: - DL traffic only for the replaced Wi-Fi network - DL and UL for the non-replaced Wi-Fi network |
| Rate control |  | Minstrel algorithm |
| Channel selection |  | Based on the minimum interference level while ensuring that each unlicensed carrier is shared by two operators in each cluster |
| OFDM symbol length |  | 4 micro second |

Table 4: Additional LAA system evaluations assumptions

| Parameters | Value |
| --- | --- |
| PCI planning for each NW | Planned |
| Antenna configuration | 2Tx2Rx, Cross-polarized. |
| Transmission schemes | Open loop 2x2 MIMO based on TM10, QPSK/16QAM/64QAM/256QAM |
| Turbo code block interleaving depth | Per LTE specs (1-14 LTE OFDM symbols dependent on MCS and PRB allocation) |
| Scheduling | Proportional fair |
| Link adaptation | Realistic |
| CCA-ED | -82 dBm |
| Cyclic Prefix | Normal |