3GPP TSG RAN WG1 Meeting #81	R1-153522

Fukuoka, Japan, 25th – 29th May 2015

Agenda item:	6.2.5.2.1

Source: 	Samsung, Ericsson, LG Electronics

Title: 	TP on Codebook Subset Restriction and Reduced Feedback for

TR36.897

Document for:	Discussion and Decision

# Introduction

This contribution proposes a text proposal (TP) to capture potential enhancements on feedback reduction (including the use of open-loop diversity) and codebook subset restriction (CSR) which were discussed in a number of contributions for RAN1#80bis (e.g. [1-7]). The text proposal is intended for section 6.2.1.1 and 6.2.1.2 of TR36.897.

# TP for section 6.2.1.1 (beamformed CSI-RS) of TR36.897

-----------------------------------START --------------------------------------

…

Codebook subset restriction (CSR) configured in a UE-specific manner where CSI feedback payload depend on the size of remaining codebook subset after CSR can be considered. As the codebook size may be large, restricting the set of beam directions, i.e. PMIs, for which CSR can be applied while taking into account the DL performance can also be considered. In addition, reporting CQI and RI without PMI, in conjunction with an open-loop diversity scheme can also be considered.

-----------------------------------STOP --------------------------------------

# TP for section 6.2.1.2 (non-precoded CSI-RS) of TR36.897

-----------------------------------START --------------------------------------

…

Codebook subset restriction (CSR) configured in a UE-specific manner where CSI feedback payload depend on the size of remaining codebook subset after CSR can be considered. As the codebook size may be large, restricting the set of beam directions, i.e. PMIs, for which CSR can be applied while taking into account the DL performance can also be considered. In addition, reporting long-term PMI of at least one dimension (e.g. V or H) without CQI can be considered for providing long-term channel statistics to the serving eNB. Furthermore, reporting long-term PMI of at least one dimension (e.g. V or H), CQI, and RI, without short-term PMI, in conjunction with an open-loop diversity scheme can also be considered.

-----------------------------------STOP --------------------------------------

# References

- R1-151538, LG Electronics, “Discussion on OL-MIMO and channel reciprocity based schemes” 3GPP TSG RAN1#80bis

- R1-151634, Samsung, “Configuration and Control Signaling for Rel.13 FD-MIMO” 3GPP TSG RAN1#80bis

- R1-151636, Samsung, “Reduced Feedback Transmission Scheme for Rel.13 FD-MIMO” 3GPP TSG RAN1#80bis

- R1-151743, ZTE, “Discussion on Feedback Scheme for Reduced Dimension Channel” 3GPP TSG RAN1#80bis

- R1-151753, NVIDIA, “Performance of FD-MIMO codebook enhancements” 3GPP TSG RAN1#80bis

- R1-151933, Ericsson, “Codebook subset restriction for FD-MIMO” 3GPP TSG RAN1#80bis

- R1-152044, CATR, “Consideration on open loop transmission for FD-MIMO” 3GPP TSG RAN1#80bis