3GPP TSG RAN WG1 Meeting #81                                                                               R1-153528

Fukuoka, Japan, 24th – 29th  May 2015

Source: 	Ericsson

Title:	Summary of the Coexistence Evaluation Results for DL+UL LAA

Agenda Item:	6.2.4.2

Document for:	Discussion

# Introduction

The following tables capture the coexistence evaluation results for DL+UL LAA and DL+UL Wi-Fi.

Some clarifications are given below to facilitate understanding the tables’ entries:

- Step 1 and step 2 in Wi-Fi and LAA coexistence evaluations are described in section 8.1 in TR 36.889.

- LAA operator 1 and LAA operator 2 refer to the two operators in LAA and LAA coexistence case.

- UPT and delay CDF are intended for the FTP traffic.

- 𝜆 denotes the traffic arrival rate.

- VoIP outage percentage refers to the percentage of the VoIP users with 98%-ile latency greater than 50ms.

In the case of both DL and UL traffic, 98%ile latency is measured independently for DL and UL and three following metrics are reported:

  - VoIP outage(DL): The percentage of outage VoIP users for DL

    - If 98%ile latency of DL is greater than 50ms, the VoIP user is declared to be in outage for DL.

  - VoIP outage (UL): The percentage of outage VoIP users for UL

    - If 98%ile latency of UL is greater than 50ms, the VoIP user is declared to be in outage for UL.

  - VoIP outage: The percentage of outage VoIP users

    - If max(98%ile latency of DL, 98%ile latency of UL) is greater than 50ms, the VoIP user is declared to be in outage.

- 𝜌 denotes the ratio between the mean served cell throughput to the mean offered cell throughput for a given traffic arrival rate as the following:

ρ=Mean served cell throughputMean offered cell throughput

The following 𝜌 is reported independently for the UL and DL traffic as the following:

- ρDL=Mean DL served cell throughputMean DL offered cell throughput

- ρUL=Mean UL served cell throughputMean UL offered cell throughput

- BO denotes the mean buffer:

  - For operators with DL-only traffic, BO is defined as:

    - Buffer occupancy of the i-th small cell/UE (Wi-Fi & LAA) = sum of the period of time during which the i-th small cell/UE has data to transmit including retransmissions (i.e., its queue is not empty) / total simulation time

    - Average buffer occupancy of the i-th small cell/UE (Wi-Fi & LAA) = sum of the period of time during which the i-th small cell/UE has data to transmit including retransmissions (i.e., its queue is not empty) / total simulation time

  - For operators with UL+DL traffic, BO is defined as:

    - Buffer occupancy of the i-th small cell (Wi-Fi & LAA) = sum of the period of time during which at least one of the i-th small cell and UEs (belonging to the i-th small cell) has data to transmit including retransmissions (i.e., its queue is not empty) / total simulation time

    - Average buffer occupancy: buffer occupancy averaged over the all small cells of the same operator

- As agreed in RAN1#80 meeting, the evaluated channel access schemes are classified according to the following categories:

  - Category 1: No LBT

  - Category 2: LBT without random back-off

  - Category 3: LBT with random back-off with fixed size of contention window

  - Category 4: LBT with random back-off with variable size of contention window

- “Additional comments” is a place holder where companies can provide important information regarding optional assumptions that have been used or assumptions different from the baseline assumptions that are assumed as well as provide brief descriptions of the LBT schemes.

# DL+UL LAA and UL+DL Wi-Fi Coexistence Evaluation Results

In this section the following tables capture the results:

- Table 1: Indoor deployment for Wi-Fi and LAA coexistence case with one shared unlicensed carrier and FTP traffic

- Table 2: Indoor deployment for Wi-Fi and LAA coexistence case with one shared unlicensed carrier and Mixed traffic

- Table 3: Indoor deployment for LAA and LAA coexistence case with one shared unlicensed carrier and FTP traffic

- Table 4: Outdoor deployment for Wi-Fi and LAA coexistence case with one shared unlicensed carrier and FTP traffic

- Table 5: Additional coexistence evaluation results for Indoor deployment for Wi-Fi and LAA coexistence case with one shared unlicensed carrier and FTP traffic with 50 users/operator

- Table 6: Additional coexistence evaluation results for Outdoor deployment for Wi-Fi and LAA coexistence case with one shared unlicensed carrier and FTP traffic with 50 users/operator

Table 1: Indoor deployment for Wi-Fi and LAA coexistence case with one shared unlicensed carrier and FTP traffic

| Reported parameters |  | Low load BO range for Wi-Fi Opt.B in Step 1: 10%~25% |  |  |  | Medium load BO range for Wi-Fi Opt. B in Step 1: 35%~50% |  |  |  | High load BO range for Wi-Fi Opt.B in Step 1: above 55% |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Wi-Fi Opt.B in step 1 | Wi-Fi Opt.A in step 1 | Wi-Fi Opt. B in step 2 | LAA Opt.A in step 2 | Wi-Fi Opt.B in step 1 | Wi-Fi Opt.A in step 1 | Wi-Fi Opt. B in step 2 | LAA Opt.A in step 2 | Wi-Fi Opt.B in step 1 | Wi-Fi Opt.A in step 1 | Wi-Fi Opt. B in step 2 | LAA Opt.A in step 2 |
| DL: UPT CDF [Mbps] | 5% | 18.45 | 18.86 | 25.88 | 50.96 | 1.12 | 1.05 | 19.89 | 50.18 | 0.52 | 0.68 | 14.94 | 43.25 |
|  | 50% | 46.58 | 44.90 | 50.63 | 89.93 | 12.64 | 15.21 | 46.92 | 83.91 | 2.72 | 1.89 | 31.46 | 70.35 |
|  | 95% | 76.25 | 78.69 | 76.00 | 120.62 | 69.97 | 56.74 | 71.78 | 119.84 | 13.40 | 24.37 | 68.55 | 99.09 |
|  | Mean | 47.66 | 45.59 | 49.87 | 87.97 | 19.86 | 19.21 | 47.34 | 84.31 | 5.45 | 5.65 | 35.17 | 69.45 |
| DL: Delay CDF [s] | 5% | 0.05 | 0.05 | 0.05 | 0.03 | 0.06 | 0.07 | 0.06 | 0.03 | 0.14 | 0.10 | 0.06 | 0.04 |
|  | 50% | 0.09 | 0.09 | 0.08 | 0.05 | 0.20 | 0.23 | 0.09 | 0.05 | 0.93 | 0.91 | 0.13 | 0.06 |
|  | 95% | 0.23 | 0.26 | 0.16 | 0.08 | 2.38 | 2.36 | 0.20 | 0.08 | 5.79 | 4.76 | 0.27 | 0.09 |
|  | Mean | 0.10 | 0.12 | 0.09 | 0.05 | 0.46 | 0.51 | 0.10 | 0.05 | 60.55 | 1.48 | 0.14 | 0.06 |
| UL: UPT CDF [Mbps] | 5% | 8.05 | 12.03 | 15.46 | 3.60 | 0.71 | 0.57 | 16.74 | 3.27 | 0.50 | 0.33 | 10.60 | 1.73 |
|  | 50% | 28.74 | 23.27 | 30.20 | 9.44 | 13.12 | 8.87 | 29.34 | 8.41 | 1.61 | 1.70 | 21.05 | 4.06 |
|  | 95% | 50.77 | 48.86 | 46.47 | 15.36 | 40.94 | 39.98 | 44.65 | 16.30 | 8.35 | 6.83 | 41.80 | 9.46 |
|  | Mean | 29.10 | 26.29 | 31.53 | 9.35 | 14.30 | 13.63 | 29.14 | 8.64 | 2.75 | 2.20 | 23.35 | 4.65 |
| UL: Delay CDF [s] | 5% | 0.08 | 0.08 | 0.09 | 0.26 | 0.11 | 0.09 | 0.09 | 0.26 | 0.20 | 0.24 | 0.10 | 0.43 |
|  | 50% | 0.14 | 0.17 | 0.13 | 0.44 | 0.30 | 0.33 | 0.14 | 0.48 | 2.22 | 1.61 | 0.19 | 1.03 |
|  | 95% | 0.50 | 0.33 | 0.26 | 1.14 | 4.62 | 3.12 | 0.28 | 1.23 | 7.32 | 6.54 | 0.40 | 2.77 |
|  | Mean | 0.18 | 0.19 | 0.15 | 0.53 | 0.92 | 0.79 | 0.16 | 0.62 | 2.47 | 2.25 | 0.22 | 1.23 |
| 𝜌DL |  | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 0.90 | 0.97 | 1.00 | 1.00 |
| 𝜌UL |  | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 0.95 | 1.00 | 1.00 | 0.95 | 0.88 | 1.00 | 1.00 |
| BO |  | 0.11 | 0.12 | 0.10 | 0.23 | 0.35 | 0.35 | 0.12 | 0.28 | 0.61 | 0.62 | 0.18 | 0.47 |
| 𝜆 |  | 0.2 |  |  |  | 0.22 |  |  |  | 0.26 |  |  |  |
| Company/tdoc: Intel / R1-152644 LAA LBT category: Category 4 on DL (regular) and category 3 on UL (faster), traffic via unlicensed only, LAA ED threshold: -82 dBm for DL and -62 dBm for UL, initial CCA period = defer period during eCCA = 34 us, ECCA slot = 9 us, exponential back off for DL with CW [16, 1024], CW is fixed to 4 for UL, no intra/inter-RAT detection used, PDCCH overhead = 3 symbols, 2x2 for LAA DL with CSI feedback, 1x2 for LAA UL-MCS selection based on SRS, 2x2 MIMO (spatial multiplexing) for Wi-Fi DL-open loop w/o CSI feedback, 1x2 for Wi-Fi UL-open loop rate adaptation based on ACK, LAA TXOP: up to 10 ms for DL and up to 5ms for UL, Wi-Fi TXOP: 4 ms, long GI for Wi-Fi. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DL: UPT CDF [Mbps] | 5% | 7.800 | 7.826 | 39.654 | 63.417 | 3.632 | 3.646 | 22.528 | 35.991 | 0.946 | 0.950 | 5.694 | 9.126 |
|  | 50% | 55.475 | 55.218 | 62.564 | 96.710 | 35.993 | 35.732 | 40.666 | 63.947 | 16.700 | 16.454 | 18.816 | 31.089 |
|  | 95% | 71.843 | 70.949 | 78.824 | 113.51 | 57.425 | 56.546 | 65.122 | 96.136 | 43.210 | 42.372 | 51.540 | 78.902 |
|  | Mean | 51.778 | 51.592 | 61.286 | 94.302 | 35.098 | 34.870 | 41.919 | 65.621 | 18.751 | 18.500 | 22.661 | 36.994 |
| DL: Delay CDF [s] | 5% | 0.512 | 0.512 | 0.100 | 0.064 | 1.100 | 1.096 | 0.176 | 0.112 | 4.228 | 4.212 | 0.704 | 0.440 |
|  | 50% | 0.072 | 0.072 | 0.064 | 0.040 | 0.112 | 0.112 | 0.100 | 0.064 | 0.240 | 0.244 | 0.212 | 0.128 |
|  | 95% | 0.056 | 0.056 | 0.052 | 0.036 | 0.068 | 0.072 | 0.060 | 0.040 | 0.092 | 0.096 | 0.076 | 0.052 |
|  | Mean | 0.076 | 0.076 | 0.064 | 0.044 | 0.112 | 0.116 | 0.096 | 0.060 | 0.212 | 0.216 | 0.176 | 0.108 |
| UL: UPT CDF [Mbps] | 5% | 6.696 | 6.076 | 39.377 | 61.591 | 2.983 | 2.641 | 22.385 | 35.035 | 0.840 | 0.807 | 5.716 | 8.932 |
|  | 50% | 55.039 | 54.774 | 61.845 | 89.326 | 35.243 | 35.056 | 39.046 | 57.220 | 15.643 | 15.570 | 16.290 | 25.007 |
|  | 95% | 70.268 | 69.802 | 77.086 | 104.03 | 55.918 | 55.031 | 63.345 | 87.442 | 41.808 | 40.521 | 49.761 | 71.008 |
|  | Mean | 51.335 | 51.057 | 60.406 | 86.983 | 34.588 | 34.373 | 40.874 | 59.329 | 18.193 | 18.072 | 21.470 | 31.726 |
| UL: Delay CDF [s] | 5% | 0.596 | 0.660 | 0.100 | 0.064 | 1.340 | 1.516 | 0.180 | 0.116 | 4.760 | 4.956 | 0.700 | 0.448 |
|  | 50% | 0.072 | 0.072 | 0.064 | 0.044 | 0.112 | 0.116 | 0.104 | 0.068 | 0.256 | 0.256 | 0.244 | 0.160 |
|  | 95% | 0.056 | 0.056 | 0.052 | 0.040 | 0.072 | 0.072 | 0.064 | 0.044 | 0.096 | 0.100 | 0.080 | 0.056 |
|  | Mean | 0.076 | 0.080 | 0.068 | 0.044 | 0.116 | 0.116 | 0.096 | 0.068 | 0.220 | 0.220 | 0.188 | 0.128 |
| 𝜌DL |  | 0.99 | 0.99 | 0.99 | 1.00 | 0.93 | 0.93 | 0.93 | 0.96 | 0.89 | 0.89 | 0.88 | 0.93 |
| 𝜌UL |  | 1.00 | 1.00 | 1.00 | 1.00 | 0.98 | 0.98 | 0.97 | 0.97 | 0.97 | 0.97 | 0.95 | 0.95 |
| BODL |  | 0.20 | 0.19 | 0.11 | 0.07 | 0.40 | 0.39 | 0.34 | 0.23 | 0.60 | 0.58 | 0.57 | 0.40 |
| BOUL |  | 0.20 | 0.20 | 0.12 | 0.07 | 0.40 | 0.39 | 0.34 | 0.26 | 0.60 | 0.58 | 0.57 | 0.45 |
| 𝜆 |  | 1.48 Mbps |  |  |  | 1.71 Mbps |  |  |  | 1.93 Mbps |  |  |  |
| Company/tdoc : QCOM/R1-152782 LBT category 3 with q = 25 and 20us CCA slots and 9.5ms TXOP for both DL and UL LAA transmission Additional information: ED threshold: Wi-Fi: -62 dBm; LAA: -82 dBm. WiFi preamble detection threshold = 4dB SINR. RTS-CTS enabled on Wi-Fi |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DL: UPT CDF [Mbps] | 5% | 10.210 |  | 23.083 | 18.836 | 3.855 |  | 14.039 | 15.389 | 1.412 |  | 13.036 | 7.844 |
|  | 50% | 51.531 |  | 99.219 | 88.628 | 15.490 |  | 55.326 | 57.975 | 5.952 |  | 48.961 | 40.312 |
|  | 95% | 120.196 |  | 120.540 | 111.631 | 102.690 |  | 120.595 | 109.133 | 93.526 |  | 120.195 | 108.800 |
|  | Mean | 60.411 |  | 85.808 | 79.254 | 32.322 |  | 66.226 | 63.082 | 19.036 |  | 57.394 | 49.156 |
| DL: Delay CDF [s] | 5% | 0.035 |  | 0.035 | 0.037 | 0.041 |  | 0.035 | 0.038 | 0.043 |  | 0.035 | 0.038 |
|  | 50% | 0.081 |  | 0.042 | 0.048 | 0.301 |  | 0.071 | 0.074 | 0.698 |  | 0.082 | 0.099 |
|  | 95% | 0.437 |  | 0.157 | 0.142 | 1.145 |  | 0.299 | 0.238 | 2.821 |  | 0.321 | 0.408 |
|  | Mean | 0.128 |  | 0.066 | 0.068 | 0.414 |  | 0.102 | 0.096 | 1.074 |  | 0.130 | 0.160 |
| UL: UPT CDF [Mbps] | 5% | 7.290 |  | 17.763 | 8.405 | 4.063 |  | 15.929 | 4.971 | 3.202 |  | 14.170 | 2.739 |
|  | 50% | 38.897 |  | 39.202 | 19.757 | 22.465 |  | 34.203 | 11.090 | 14.011 |  | 30.834 | 7.422 |
|  | 95% | 59.336 |  | 59.366 | 26.035 | 57.976 |  | 59.212 | 25.702 | 50.407 |  | 54.822 | 21.433 |
|  | Mean | 38.972 |  | 43.461 | 18.612 | 26.902 |  | 35.552 | 13.175 | 19.348 |  | 32.577 | 9.600 |
| UL: Delay CDF [s] | 5% | 0.071 |  | 0.072 | 0.164 | 0.071 |  | 0.071 | 0.165 | 0.081 |  | 0.077 | 0.178 |
|  | 50% | 0.103 |  | 0.101 | 0.217 | 0.179 |  | 0.121 | 0.386 | 0.280 |  | 0.136 | 0.550 |
|  | 95% | 0.321 |  | 0.201 | 0.437 | 0.773 |  | 0.255 | 0.713 | 1.194 |  | 0.276 | 1.221 |
|  | Mean | 0.136 |  | 0.110 | 0.261 | 0.280 |  | 0.144 | 0.416 | 0.428 |  | 0.156 | 0.648 |
| 𝜌DL |  | 1.00 |  | 1.00 | 1.00 | 1.00 |  | 1.00 | 1.00 | 1.00 |  | 1.00 | 1.00 |
| 𝜌UL |  | 0.8817 |  | 0.895 | 0.947 | 0.8182 |  | 0.826 | 0.830 | 1.00 |  | 1.00 | 0.757 |
| BO(%) |  | 16.00 |  | 11.50 | 20.92 | 40.62 |  | 17.70 | 37.86 | 65.88 |  | 26.95 | 52.32 |
| 𝜆 |  | 0.15 |  |  |  | 0.225 |  |  |  | 0.25 |  |  |  |
| Company/tdoc : InterDigital / R1-152933 Additional comments: DL/UL split : 50/50 For LAA, only UEs scheduled for the UL will contend for the channel. LTE utilizes TDD UL/DL configuration #3 [D S U U U D D D D D]. FBE based CCA in the UL with a FFP of 10ms. CCA with 48 microsecond duration is performed prior to the start of SF #2. Only one CCA per UE per FFP. Any scheduled UE can transmit a single burst of up to 3 SFs subject to eNB scheduling. No dynamic reassignment of unused UL SFs for the DL. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DL: UPT CDF [Mbps] | 5% | 10.801 |  | 20.242 | 19.218 | 4.867 |  | 5.772 | 1.443 | 2.456 |  | 1.911 | 1.291 |
|  | 50% | 65.735 |  | 89.144 | 63.771 | 21.478 |  | 28.262 | 15.299 | 10.685 |  | 18.160 | 9.618 |
|  | 95% | 120.512 |  | 120.624 | 111.631 | 104.648 |  | 120.153 | 88.334 | 102.344 |  | 99.447 | 57.887 |
|  | Mean | 65.467 |  | 80.786 | 67.397 | 34.390 |  | 41.481 | 23.530 | 24.257 |  | 32.703 | 15.851 |
| DL: Delay CDF [s] | 5% | 0.035 |  | 0.035 | 0.038 | 0.039 |  | 0.035 | 0.047 | 0.098 |  | 0.040 | 0.075 |
|  | 50% | 0.065 |  | 0.048 | 0.062 | 0.354 |  | 0.152 | 0.276 | 0.319 |  | 0.229 | 0.427 |
|  | 95% | 0.363 |  | 0.185 | 0.210 | 1.459 |  | 0.678 | 2.622 | 0.871 |  | 1.739 | 2.576 |
|  | Mean | 0.129 |  | 0.074 | 0.083 | 0.559 |  | 0.261 | 0.669 | 0.420 |  | 0.385 | 0.861 |
| UL: UPT CDF [Mbps] | 5% | 7.717 |  | 21.058 | 7.313 | 3.642 |  | 2.878 | 1.155 | 3.952 |  | 1.488 | 1.225 |
|  | 50% | 38.700 |  | 35.046 | 19.065 | 22.629 |  | 15.112 | 5.128 | 12.894 |  | 11.528 | 3.035 |
|  | 95% | 59.165 |  | 56.450 | 25.976 | 48.630 |  | 49.156 | 12.200 | 30.821 |  | 23.982 | 13.635 |
|  | Mean | 37.178 |  | 38.883 | 18.092 | 25.281 |  | 19.663 | 7.086 | 15.891 |  | 13.255 | 4.375 |
| UL: Delay CDF [s] | 5% | 0.071 |  | 0.071 | 0.165 | 0.073 |  | 0.078 | 0.193 | 0.098 |  | 0.121 | 0.318 |
|  | 50% | 0.106 |  | 0.110 | 0.225 | 0.181 |  | 0.250 | 0.871 | 0.319 |  | 0.307 | 1.321 |
|  | 95% | 0.255 |  | 0.190 | 0.419 | 0.861 |  | 0.923 | 2.502 | 0.871 |  | 1.273 | 3.827 |
|  | Mean | 0.161 |  | 0.121 | 0.284 | 0.311 |  | 0.458 | 1.250 | 0.420 |  | 0.538 | 1.609 |
| 𝜌DL |  | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 |
| 𝜌UL |  | 1.00 | 1.00 | 0.966 | 0.8034 | 0.798 | 0.643 | 0.818 | 0.768 | 0.473 | 1.00 | 1.00 | 0.966 |
| BO(%) |  | 16.83 | 11.65 | 16.50 | 41.23 | 47.80 | 80.44 | 58.07 | 59.60 | 91.38 | 16.83 | 11.65 | 16.50 |
| 𝜆 |  | DL/UL - 0.24/0.06 |  |  |  | DL/UL - 0.36/0.09 |  |  |  | DL/UL- 0.40/0.10 |  |  |  |
| Company/tdoc : InterDigital / R1-152933 Additional comments: DL/UL split : 80/20 For LAA, only UEs scheduled for the UL will contend for the channel. LTE utilizes TDD UL/DL configuration #3 [D S U U U D D D D D]. FBE based CCA in the UL with a FFP of 10ms. CCA with 48 microsecond duration is performed prior to the start of SF #2. Only one CCA per UE per FFP. Any scheduled UE can transmit a single burst of up to 3 SFs subject to eNB scheduling. No dynamic reassignment of unused UL SFs for the DL. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DL: UPT CDF [Mbps] | 5% | 20.9909 | 24.2900 | 36.767 | 41.060 | 15.818 | 14.514 | 27.426 | 19.141 | 7.232 | 7.927 | 13.137 | 1.720 |
|  | 50% | 39.1056 | 36.7449 | 50.582 | 59.415 | 19.456 | 20.918 | 40.055 | 36.761 | 12.481 | 12.487 | 32.596 | 16.713 |
|  | 95% | 46.0805 | 54.4901 | 61.839 | 78.981 | 35.252 | 31.828 | 47.469 | 52.040 | 17.584 | 16.876 | 37.608 | 24.512 |
|  | Mean | 37.0662 | 37.4945 | 50.892 | 60.056 | 22.496 | 22.363 | 39.639 | 35.822 | 12.651 | 12.545 | 31.432 | 16.028 |
| DL: Delay CDF [s] | 5% | 0.0633 | 0.0641 | 0.063 | 0.045 | 0.065 | 0.065 | 0.064 | 0.046 | 0.104 | 0.107 | 0.065 | 0.050 |
|  | 50% | 0.1207 | 0.1079 | 0.071 | 0.054 | 0.204 | 0.201 | 0.096 | 0.144 | 0.436 | 0.398 | 0.137 | 0.501 |
|  | 95% | 0.2580 | 0.2541 | 0.142 | 0.348 | 0.482 | 0.705 | 0.266 | 2.436 | 1.348 | 1.208 | 0.426 | 3.411 |
|  | Mean | 0.1329 | 0.1359 | 0.087 | 0.106 | 0.246 | 0.275 | 0.125 | 0.470 | 0.545 | 0.528 | 0.173 | 0.948 |
| UL: UPT CDF [Mbps] | 5% | 24.9078 | 23.7316 | 35.1417 | 42.1053 | 20.187 | 15.812 | 21.356 | 13.451 | 6.643 | 5.806 | 16.610 | 2.060 |
|  | 50% | 34.7983 | 37.1737 | 48.933 | 65.179 | 25.225 | 23.292 | 39.936 | 47.882 | 14.568 | 13.127 | 33.194 | 8.692 |
|  | 95% | 52.9816 | 52.4336 | 57.458 | 80.161 | 35.856 | 35.516 | 58.057 | 65.822 | 20.977 | 22.418 | 41.409 | 20.606 |
|  | Mean | 37.8253 | 39.9669 | 46.553 | 65.719 | 27.461 | 23.967 | 40.908 | 46.963 | 14.089 | 13.941 | 32.193 | 10.379 |
| UL: Delay CDF [s] | 5% | 0.0634 | 0.0644 | 0.063 | 0.042 | 0.065 | 0.068 | 0.063 | 0.042 | 0.103 | 0.113 | 0.065 | 0.073 |
|  | 50% | 0.1135 | 0.1027 | 0.068 | 0.066 | 0.173 | 0.209 | 0.100 | 0.103 | 0.344 | 0.365 | 0.133 | 0.998 |
|  | 95% | 0.2613 | 0.2505 | 0.167 | 0.295 | 0.367 | 0.514 | 0.247 | 0.801 | 0.917 | 1.105 | 0.355 | 5.347 |
|  | Mean | 0.1337 | 0.1250 | 0.091 | 0.109 | 0.201 | 0.238 | 0.122 | 0.230 | 0.413 | 0.451 | 0.158 | 1.623 |
| 𝜌DL(%) |  | 100 | 100 | 100 | 100 | 98.788 | 99.398 | 100 | 100 | 98.214 | 98.907 | 100 | 94.500 |
| 𝜌UL(%) |  | 100 | 99.0196 | 98.810 | 100 | 98.658 | 100 | 100 | 99.333 | 100 | 98.537 | 99.005 | 84.000 |
| BO(%) |  | 14.572 | 12.418 | 9.385 | 11.289 | 32.867 | 31.212 | 17.441 | 34.434 | 56.369 | 60.433 | 29.135 | 84.340 |
| 𝜆 |  | 0.13 |  |  |  | 0.20 |  |  |  | 0.26 |  |  |  |
| Company/tdoc: ZTE/ R1-153392 LBT category: 3 Additional information: 256QAM enabled, no LDPC code, 24 dBm TX power for both LAA eNB and WiFi AP, 18 dBm TX power for LAA UE and WiFi STA. no licensed carrier for LAA, RTS/CTS enabled for WiFi LAA SCell is configured to use TDD UL/DL configuration [D D D D D U U U U U] 50% DL traffic and 50% UL traffic Sensing threshold: CCA-CS -82 dBm for WiFi; CCA-ED -62 dBm for both WiFi and LAA. No defer period. 34 us CCA slot for LBT category 3 Asynchronous between different LAA operators No inter- or intra-RAT detection assumed for LAA. WiFi use CCA-CS and CCA-ED for intra- and inter-RAT detection. The same Tdoc R1-153392 describing LBT schemes of category 3. |  |  |  |  |  |  |  |  |  |  |  |  |  |

| DL: UPT CDF [Mbps] | 5% | 22.57 | 21.88 | 53.15 | 53.75 | 8.12 | 7.37 | 39.1 | 36.39 | 4.11 | 3.74 | 29.1 | 25.03 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 50% | 63.75 | 64.23 | 87.69 | 88.54 | 41.62 | 43.29 | 80.0 | 78.08 | 23.37 | 24.07 | 69.76 | 66.74 |
|  | 95% | 95.01 | 97.15 | 104.9 | 107.4 | 82.72 | 83.17 | 101.83 | 101.26 | 62.95 | 62.9 | 96.03 | 95.18 |
|  | Mean | 64.13 | 64.85 | 86.8 | 87.72 | 45.48 | 46.6 | 79.14 | 77.58 | 29.65 | 30.18 | 69.34 | 66.47 |
| DL: Delay CDF [s] | 5% | 0.033 | 0.032 | 0.03 | 0.03 | 0.042 | 0.035 | 0.03 | 0.031 | 0.067 | 0.039 | 0.034 | 0.034 |
|  | 50% | 0.156 | 0.095 | 0.043 | 0.043 | 0.469 | 0.301 | 0.06 | 0.063 | 1.071 | 0.762 | 0.09 | 0.102 |
|  | 95% | 1.857 | 1.99 | 0.071 | 0.076 | 5.583 | 4.395 | 0.43 | 0.322 | 7.811 | 7.053 | 1.417 | 0.982 |
|  | Mean | 0.481 | 0.478 | 0.049 | 0.049 | 1.411 | 1.159 | 0.129 | 0.116 | 2.394 | 2.093 | 0.348 | 0.286 |
| UL: UPT CDF [Mbps] | 5% | 16.78 | 17.97 | 45.06 | 48.03 | 7.5 | 5.17 | 40.66 | 34.32 | 3.8 | 2.64 | 28.71 | 23.16 |
|  | 50% | 61.98 | 61.78 | 86.56 | 87.17 | 40.51 | 41.23 | 76.9 | 76.97 | 22.45 | 23.07 | 66.83 | 65.9 |
|  | 95% | 95.84 | 94.74 | 106.14 | 106.4 | 80.27 | 80.3 | 100.73 | 101.51 | 60.61 | 59.48 | 94.77 | 94.29 |
|  | Mean | 62.04 | 62.28 | 85.02 | 85.82 | 44.42 | 44.09 | 76.95 | 76.33 | 28.63 | 28.26 | 66.51 | 64.97 |
| UL: Delay CDF [s] | 5% | 0.032 | 0.033 | 0.031 | 0.03 | 0.042 | 0.04 | 0.031 | 0.031 | 0.067 | 0.06 | 0.033 | 0.035 |
|  | 50% | 0.145 | 0.134 | 0.044 | 0.046 | 0.636 | 0.506 | 0.055 | 0.065 | 1.408 | 1.202 | 0.077 | 0.103 |
|  | 95% | 2.469 | 2.068 | 0.087 | 0.084 | 4.447 | 5.071 | 0.444 | 0.437 | 7.438 | 7.693 | 1.402 | 1.352 |
|  | Mean | 0.645 | 0.545 | 0.052 | 0.054 | 1.416 | 1.442 | 0.135 | 0.136 | 2.498 | 2.467 | 0.351 | 0.337 |
| 𝜌DL |  | 0.95 | 0.94 | 1 | 1 | 0.82 | 0.81 | 0.99 | 1 | 0.66 | 0.65 | 0.97 | 0.98 |
| 𝜌UL |  | 0.96 | 0.95 | 1 | 1 | 0.85 | 0.85 | 0.99 | 0.99 | 0.72 | 0.72 | 0.97 | 0.94 |
| BO |  | 0.2 | 0.2 | 0.06 | 0.06 | 0.4 | 0.4 | 0.1 | 0.1 | 0.6 | 0.6 | 0.18 | 0.18 |
| 𝜆 |  | 0.246937 |  |  |  | 0.301094 |  |  |  | 0.350344 |  |  |  |
| Company/tdoc: Ericsson / R1-150584 / R1-152109 LBT category: Category 3 on DL and UL with self-scheduling Additional information: Without licensed carrier DL/UL split: 50/50 Sensing threshold used: -82 dBm Whether defer periods are used or not: yes CCA and ECCA slot length: 20 μs Whether or not intra and/or inter-RAT detection is assumed: only CCA-ED Any significant deviations from evaluations methodology and assumptions: |  |  |  |  |  |  |  |  |  |  |  |  |  |

| DL: UPT CDF [Mbps] | 5% | 0.00 | 1.74 | 10.5 | 0.00 | 0.05 | 0.04 | 1.40 | 4.56 | 0.07 | 0.03 | 0.31 | 3.72 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 50% | 36.7 | 38.3 | 51.6 | 83.3 | 13.5 | 15.2 | 29.1 | 54.9 | 2.42 | 3.44 | 23.6 | 49.4 |
|  | 95% | 88.6 | 99.8 | 108.9 | 151.0 | 76.0 | 77.6 | 92.1 | 147.3 | 58.6 | 62.8 | 86.3 | 140.4 |
|  | Mean | 39.9 | 43.8 | 54.6 | 84.4 | 21.4 | 24.2 | 35.2 | 62.0 | 14.8 | 15.3 | 29.9 | 55.7 |
| DL: Delay CDF [s] | 5% | 0.032 | 0.032 | 0.032 | 0.026 | 0.032 | 0.032 | 0.032 | 0.026 | 0.033 | 0.032 | 0.032 | 0.026 |
|  | 50% | 0.064 | 0.061 | 0.053 | 0.031 | 0.093 | 0.090 | 0.061 | 0.042 | 0.096 | 0.104 | 0.068 | 0.049 |
|  | 95% | 0.398 | 0.421 | 0.253 | 0.188 | 3.452 | 3.336 | 0.594 | 0.413 | 7.083 | 8.467 | 1.064 | 1.044 |
|  | Mean | 0.151 | 0.155 | 0.088 | 0.067 | 0.748 | 0.735 | 0.193 | 0.139 | 1.124 | 1.271 | 0.388 | 0.296 |
| UL: UPT CDF [Mbps] | 5% | 2.08 | 0.00 | 4.14 | 0.00 | 0.29 | 0.44 | 2.81 | 6.11 | 0.27 | 0.27 | 1.42 | 3.26 |
|  | 50% | 25.6 | 27.4 | 30.4 | 47.1 | 13.7 | 13.0 | 20.8 | 34.7 | 5.38 | 5.04 | 17.4 | 32.9 |
|  | 95% | 47.4 | 46.0 | 51.9 | 66.4 | 39.9 | 38.1 | 45.4 | 62.0 | 35.1 | 34.6 | 43.2 | 62.0 |
|  | Mean | 25.4 | 25.7 | 29.6 | 45.3 | 15.6 | 15.4 | 22.2 | 35.0 | 10.8 | 10.9 | 18.9 | 32.4 |
| UL: Delay CDF [s] | 5% | 0.064 | 0.064 | 0.064 | 0.059 | 0.064 | 0.064 | 0.064 | 0.059 | 0.064 | 0.064 | 0.064 | 0.059 |
|  | 50% | 0.109 | 0.105 | 0.086 | 0.064 | 0.134 | 0.123 | 0.097 | 0.084 | 0.155 | 0.146 | 0.107 | 0.093 |
|  | 95% | 0.372 | 0.335 | 0.234 | 0.217 | 1.478 | 1.417 | 0.402 | 0.491 | 2.444 | 2.624 | 0.627 | 0.690 |
|  | Mean | 0.165 | 0.141 | 0.110 | 0.099 | 0.444 | 0.423 | 0.154 | 0.170 | 0.660 | 0.668 | 0.236 | 0.242 |
| 𝜌DL |  | 0.99 | 0.99 | 0.99 | 1.00 | 0.82 | 0.86 | 0.95 | 0.99 | 0.69 | 0.72 | 0.93 | 0.99 |
| 𝜌UL |  | 0.99 | 0.99 | 0.98 | 1.00 | 0.92 | 0.93 | 0.96 | 1.00 | 0.90 | 0.89 | 0.96 | 0.99 |
| BO[%] |  | 15.1 | 14.9 | 11.2 | 7.1 | 43.2 | 43.0 | 22.5 | 13.2 | 56.5 | 56.5 | 28.5 | 17.7 |
| Discarding rate of UL grants[%] |  | N/A | N/A | N/A | 12.1 | N/A | N/A | N/A | 20.3 | N/A | N/A | N/A | 22.3 |
| 𝜆 [Files/s/UE] |  | 0.1 |  |  |  | 0.12 |  |  |  | 0.1275 |  |  |  |
| Company/tdoc: : Fujitsu / R1-152654 LBT category: category 4 for DL transmission, category 2 for UL transmission Additional information: no UE multiplexing in frequency domain for LAA UL Sensing threshold used: -62dBm Whether defer periods are used or not: yes CCA and ECCA slot length: 9 us for DL Whether or not intra and/or inter-RAT detection is assumed: only CCA-ED Any significant deviations from evaluations methodology and assumptions: FTP model 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DL: UPT CDF [Mbps] | 5% | 14.82 | 14.16 | 30.18 | 35.34 | 1.29 | 1.70 | 26.59 | 30.95 | 0.66 | 0.44 | 19.97 | 22.54 |
|  | 50% | 32.92 | 32.79 | 40.46 | 46.85 | 24.71 | 25.19 | 37.49 | 43.72 | 9.37 | 9.20 | 33.91 | 39.61 |
|  | 95% | 42.92 | 42.79 | 49.11 | 56.93 | 37.77 | 37.88 | 46.60 | 54.23 | 31.31 | 31.97 | 44.48 | 51.32 |
|  | Mean | 31.63 | 31.61 | 40.24 | 46.54 | 22.10 | 21.96 | 37.22 | 43.33 | 12.29 | 12.25 | 33.27 | 38.80 |
| DL: Delay CDF [s] | 5% | 0.124 | 0.123 | 0.099 | 0.085 | 0.152 | 0.149 | 0.107 | 0.092 | 0.216 | 0.199 | 0.117 | 0.099 |
|  | 50% | 0.208 | 0.207 | 0.142 | 0.120 | 0.374 | 0.374 | 0.163 | 0.138 | 2.835 | 2.915 | 0.201 | 0.164 |
|  | 95% | 1.598 | 2.286 | 0.250 | 0.313 | 24.117 | 21.793 | 0.355 | 0.430 | 39.335 | 39.185 | 0.676 | 0.869 |
|  | Mean | 0.708 | 0.921 | 0.155 | 0.158 | 4.361 | 4.080 | 0.201 | 0.193 | 9.442 | 9.733 | 0.330 | 0.321 |
| UL: UPT CDF [Mbps] | 5% | 16.12 | 16.32 | 31.35 | 25.40 | 2.33 | 2.57 | 27.79 | 21.43 | 1.24 | 1.24 | 20.99 | 14.92 |
|  | 50% | 34.85 | 34.80 | 41.32 | 36.72 | 26.91 | 27.06 | 38.47 | 33.49 | 10.91 | 10.66 | 34.92 | 29.80 |
|  | 95% | 44.82 | 44.49 | 50.16 | 45.88 | 40.11 | 40.65 | 47.69 | 43.51 | 33.47 | 33.77 | 45.33 | 40.66 |
|  | Mean | 33.56 | 33.44 | 41.13 | 36.19 | 23.67 | 23.96 | 38.21 | 32.94 | 13.71 | 13.86 | 34.36 | 29.20 |
| UL: Delay CDF [s] | 5% | 0.114 | 0.115 | 0.095 | 0.112 | 0.141 | 0.135 | 0.103 | 0.125 | 0.193 | 0.182 | 0.113 | 0.141 |
|  | 50% | 0.193 | 0.194 | 0.140 | 0.176 | 0.335 | 0.330 | 0.165 | 0.213 | 1.440 | 1.471 | 0.199 | 0.272 |
|  | 95% | 1.243 | 1.288 | 0.286 | 0.627 | 8.620 | 9.275 | 0.380 | 1.102 | 18.066 | 18.327 | 0.677 | 2.105 |
|  | Mean | 0.465 | 0.589 | 0.160 | 0.285 | 2.066 | 2.207 | 0.198 | 0.569 | 4.162 | 4.419 | 0.299 | 0.904 |
| 𝜌DL |  | 0.9927 | 0.9880 | 0.9994 | 0.9994 | 0.9350 | 0.9392 | 0.9995 | 0.9991 | 0.8547 | 0.8526 | 0.9982 | 0.9983 |
| 𝜌UL |  | 0.9968 | 0.9933 | 0.9996 | 0.9957 | 0.9777 | 0.9757 | 0.9994 | 0.9955 | 0.9542 | 0.9446 | 0.9988 | 0.9874 |
| BO[%] |  | 0.2166 | 0.2181 | 0.1425 | 0.1725 | 0.3971 | 0.3979 | 0.1757 | 0.2128 | 0.6132 | 0.6206 | 0.2287 | 0.2729 |
| 𝜆 |  | 0.22 |  |  |  | 0.24 |  |  |  | 0.26 |  |  |  |
| Company/tdoc : LG/R1-152731 - LBT scheme for DL: Cat. 3 with q = 10 and 24us CCA/ECCA slots and 4ms TXOP - LBT scheme for UL: Cat. 2 with 16us CCA slot and 1ms TXOP - Sensing threshold used: -62dBm for CCA-ED of LAA - Whether defer periods are used or not: Not used - Inter-operator synchronization for LAA-LAA coexistence: N/A - Whether or not intra and/or inter-RAT detection is assumed: N/A - Any significant deviations from evaluations methodology and assumptions: LAA without licensed carrier, no 256 QAM, LDPC, 1Tx2Rx - Tdoc numbers for the contributions describing their LBT schemes: R1-152731 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DL: UPT CDF [Mbps] | 5% | 14.82 | 14.16 | 25.96 | 33.16 | 1.29 | 1.70 | 18.97 | 24.60 | 0.66 | 0.44 | 9.09 | 14.08 |
|  | 50% | 32.92 | 32.79 | 36.55 | 45.47 | 24.71 | 25.19 | 32.66 | 41.65 | 9.37 | 9.20 | 27.73 | 36.80 |
|  | 95% | 42.92 | 42.79 | 45.72 | 56.00 | 37.77 | 37.88 | 42.90 | 53.60 | 31.31 | 31.97 | 39.55 | 50.14 |
|  | Mean | 31.63 | 31.61 | 36.21 | 45.18 | 22.10 | 21.96 | 31.98 | 40.86 | 12.29 | 12.25 | 26.73 | 35.36 |
| DL: Delay CDF [s] | 5% | 0.124 | 0.123 | 0.112 | 0.087 | 0.152 | 0.149 | 0.127 | 0.095 | 0.216 | 0.199 | 0.148 | 0.105 |
|  | 50% | 0.208 | 0.207 | 0.177 | 0.128 | 0.374 | 0.374 | 0.215 | 0.151 | 2.835 | 2.915 | 0.301 | 0.190 |
|  | 95% | 1.598 | 2.286 | 0.345 | 0.345 | 24.117 | 21.793 | 0.760 | 0.669 | 39.335 | 39.185 | 3.564 | 1.965 |
|  | Mean | 0.708 | 0.921 | 0.196 | 0.177 | 4.361 | 4.080 | 0.436 | 0.284 | 9.442 | 9.733 | 1.016 | 0.609 |
| UL: UPT CDF [Mbps] | 5% | 16.12 | 16.32 | 27.65 | 28.40 | 2.33 | 2.57 | 21.48 | 19.34 | 1.24 | 1.24 | 10.89 | 10.05 |
|  | 50% | 34.85 | 34.80 | 37.70 | 42.32 | 26.91 | 27.06 | 34.13 | 38.57 | 10.91 | 10.66 | 29.71 | 33.33 |
|  | 95% | 44.82 | 44.49 | 47.15 | 53.07 | 40.11 | 40.65 | 43.78 | 49.93 | 33.47 | 33.77 | 40.69 | 47.38 |
|  | Mean | 33.56 | 33.44 | 37.62 | 41.63 | 23.67 | 23.96 | 33.57 | 37.17 | 13.71 | 13.86 | 28.54 | 32.06 |
| UL: Delay CDF [s] | 5% | 0.114 | 0.115 | 0.106 | 0.093 | 0.141 | 0.135 | 0.121 | 0.103 | 0.193 | 0.182 | 0.138 | 0.113 |
|  | 50% | 0.193 | 0.194 | 0.170 | 0.144 | 0.335 | 0.330 | 0.205 | 0.172 | 1.440 | 1.471 | 0.268 | 0.228 |
|  | 95% | 1.243 | 1.288 | 0.352 | 0.734 | 8.620 | 9.275 | 0.666 | 1.611 | 18.066 | 18.327 | 1.659 | 6.009 |
|  | Mean | 0.465 | 0.589 | 0.193 | 0.292 | 2.066 | 2.207 | 0.325 | 0.692 | 4.162 | 4.419 | 0.604 | 1.576 |
| 𝜌DL |  | 0.9927 | 0.9880 | 0.9993 | 0.9993 | 0.9350 | 0.9392 | 0.9977 | 0.9985 | 0.8547 | 0.8526 | 0.9877 | 0.9937 |
| 𝜌UL |  | 0.9968 | 0.9933 | 0.9995 | 0.9961 | 0.9777 | 0.9757 | 0.9982 | 0.9910 | 0.9542 | 0.9446 | 0.9958 | 0.9788 |
| BO[%] |  | 0.2166 | 0.2181 | 0.1664 | 0.1725 | 0.3971 | 0.3979 | 0.2226 | 0.2218 | 0.6132 | 0.6206 | 0.3037 | 0.2915 |
| 𝜆 |  | 0.22 |  |  |  | 0.24 |  |  |  | 0.26 |  |  |  |
| Company/tdoc : LG/R1-152731 - LBT scheme for DL: Cat. 3 with q = 10 and 24us CCA/ECCA slots and 4ms TXOP - LBT scheme for UL: Cat. 3 with q = 4 and 16us CCA/ECCA slots and 1.6ms TXOP - Sensing threshold used: -62dBm for CCA-ED of LAA - Whether defer periods are used or not: Not used - Inter-operator synchronization for LAA-LAA coexistence: N/A - Whether or not intra and/or inter-RAT detection is assumed: N/A - Any significant deviations from evaluations methodology and assumptions: LAA without licensed carrier, no 256 QAM, LDPC, 1Tx2Rx - Tdoc numbers for the contributions describing their LBT schemes: R1-152731 |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 2: Indoor deployment for Wi-Fi and LAA coexistence case with one shared unlicensed carrier and Mixed traffic

| Reported parameters |  | Low load BO range for Wi-Fi in Step 1: 10%~25% |  |  |  | Medium load BO range for Wi-Fi in Step 1: 35%~50% |  |  |  | High load BO range for Wi-Fi in Step 1: above 55% |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Wi-Fi Opt.B in step 1 | Wi-Fi Opt.A in step 1 | Wi-Fi Opt. B in step 2 | LAA Opt.A in step 2 | Wi-Fi Opt.B in step 1 | Wi-Fi Opt.A in step 1 | Wi-Fi Opt. B in step 2 | LAA Opt.A in step 2 | Wi-Fi Opt.B in step 1 | Wi-Fi Opt.A in step 1 | Wi-Fi Opt. B in step 2 | LAA Opt.A in step 2 |
| DL: UPT CDF [Mbps] | 5% | 28.780 | 26.730 | 49.490 | 70.380 | 7.2600 | 8.6100 | 40.610 | 58.910 | 1.6200 | 1.9800 | 29.660 | 45.990 |
|  | 50% | 66.990 | 67.230 | 82.540 | 99.590 | 47.320 | 48.100 | 74.430 | 91.380 | 24.400 | 25.910 | 66.990 | 85.060 |
|  | 95% | 93.090 | 93.790 | 102.38 | 114.11 | 80.040 | 82.680 | 96.500 | 109.99 | 64.690 | 66.710 | 91.680 | 105.79 |
|  | Mean | 65.970 | 66.550 | 82.050 | 98.770 | 46.200 | 47.570 | 74.510 | 90.870 | 29.710 | 30.880 | 66.740 | 83.390 |
| DL: Delay CDF [s] | 5% | 0.0380 | 0.0420 | 0.0340 | 0.0300 | 0.0400 | 0.0590 | 0.0370 | 0.0320 | 0.0470 | 0.0480 | 0.0390 | 0.0330 |
|  | 50% | 0.2340 | 0.2000 | 0.0520 | 0.0410 | 0.3700 | 0.6550 | 0.0670 | 0.0500 | 1.2440 | 1.4730 | 0.0860 | 0.0620 |
|  | 95% | 3.2800 | 2.9240 | 0.1200 | 0.0740 | 11.493 | 13.264 | 0.2120 | 0.1400 | 17.307 | 15.763 | 1.1350 | 0.4950 |
|  | Mean | 0.8840 | 0.8720 | 0.0670 | 0.0470 | 2.5910 | 2.9840 | 0.0990 | 0.0700 | 4.4350 | 4.4920 | 0.3180 | 0.1670 |
| UL: UPT CDF [Mbps] | 5% | 24.010 | 25.450 | 41.460 | 61.220 | 6.4200 | 6.6500 | 34.200 | 43.000 | 1.5800 | 1.5700 | 25.230 | 26.400 |
|  | 50% | 64.330 | 62.840 | 78.190 | 87.200 | 43.330 | 42.840 | 69.250 | 70.190 | 23.230 | 23.700 | 61.650 | 53.920 |
|  | 95% | 90.720 | 88.990 | 100.71 | 101.14 | 77.150 | 77.920 | 93.630 | 86.570 | 60.650 | 62.870 | 89.590 | 69.800 |
|  | Mean | 63.470 | 62.400 | 77.670 | 86.570 | 44.160 | 44.070 | 69.290 | 69.910 | 28.110 | 28.750 | 62.260 | 53.270 |
| UL: Delay CDF [s] | 5% | 0.0380 | 0.0390 | 0.0340 | 0.0350 | 0.0440 | 0.0410 | 0.0370 | 0.0420 | 0.0480 | 0.0550 | 0.0390 | 0.0520 |
|  | 50% | 0.2580 | 0.2690 | 0.0580 | 0.0490 | 0.7100 | 0.6560 | 0.0850 | 0.0710 | 1.8020 | 1.3710 | 0.1080 | 0.1010 |
|  | 95% | 2.7330 | 2.9920 | 0.1680 | 0.1150 | 10.961 | 9.2730 | 0.3430 | 0.2660 | 14.655 | 14.784 | 1.0410 | 1.3730 |
|  | Mean | 0.7570 | 0.8300 | 0.0820 | 0.0620 | 2.7490 | 2.2940 | 0.1380 | 0.1090 | 4.2740 | 3.9020 | 0.3150 | 0.3490 |
| VoIP outage |  | 0.4100 | N/A | 0.2500 | N/A | 0.6300 | N/A | 0.3300 | N/A | 0.8300 | N/A | 0.3800 | N/A |
| VoIP outage(DL) |  | 0.3400 | N/A | 0.2300 | N/A | 0.5700 | N/A | 0.3300 | N/A | 0.7800 | N/A | 0.3600 | N/A |
| VoIP outage(UL) |  | 0.1600 | N/A | 0.0200 | N/A | 0.2800 | N/A | 0.0100 | N/A | 0.5200 | N/A | 0.0600 | N/A |
| 𝜌DL |  | 0.9500 | 0.9500 | 1.0000 | 1.0000 | 0.8100 | 0.8300 | 1.0000 | 1.0000 | 0.6700 | 0.6900 | 0.9900 | 0.9900 |
| 𝜌UL |  | 0.9600 | 0.9500 | 1.0000 | 1.0000 | 0.8500 | 0.8700 | 1.0000 | 1.0000 | 0.7500 | 0.7600 | 0.9900 | 0.9800 |
| BO |  | 0.2000 | 0.1900 | 0.0700 | 0.0500 | 0.4000 | 0.3900 | 0.1100 | 0.0800 | 0.6000 | 0.5900 | 0.1600 | 0.1300 |
| 𝜆 |  | 0.203897 |  |  |  | 0.246513 |  |  |  | 0.276395 |  |  |  |
| Company/tdoc: Ericsson / R1-153124 LBT category: Category 4 on DL and UL with self-scheduling Additional information: Without licensed carrier DL/UL split: 50/50 Sensing threshold used: -82 dBm Whether defer periods are used or not: yes CCA and ECCA slot length: 20 μs Whether or not intra and/or inter-RAT detection is assumed: only CCA-ED Any significant deviations from evaluations methodology and assumptions: DL CWmin=15, DL CWmax=1023, UL CWmin=7, UL CWmax=15, fast LBT (Cat 3 LBE with Q=1) for DL control channel to send UL grants when the eNB DL data buffer is empty |  |  |  |  |  |  |  |  |  |  |  |  |  |

| DL: UPT CDF [Mbps] | 5% | 28.780 | 26.730 | 44.920 | 66.260 | 7.2600 | 8.6100 | 30.580 | 48.210 | 1.6200 | 1.9800 | 17.930 | 27.530 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 50% | 66.990 | 67.230 | 79.050 | 96.880 | 47.320 | 48.100 | 66.760 | 84.390 | 24.400 | 25.910 | 57.080 | 71.930 |
|  | 95% | 93.090 | 93.790 | 100.59 | 113.37 | 80.040 | 82.680 | 94.100 | 107.48 | 64.690 | 66.710 | 87.530 | 100.95 |
|  | Mean | 65.970 | 66.550 | 79.310 | 96.150 | 46.200 | 47.570 | 67.370 | 84.700 | 29.710 | 30.880 | 56.780 | 70.430 |
| DL: Delay CDF [s] | 5% | 0.0380 | 0.0420 | 0.0350 | 0.0300 | 0.0400 | 0.0590 | 0.0380 | 0.0330 | 0.0470 | 0.0480 | 0.0430 | 0.0360 |
|  | 50% | 0.2340 | 0.2000 | 0.0590 | 0.0460 | 0.3700 | 0.6550 | 0.1010 | 0.0880 | 1.2440 | 1.4730 | 0.1840 | 0.1200 |
|  | 95% | 3.2800 | 2.9240 | 0.2180 | 0.1470 | 11.493 | 13.264 | 1.0580 | 0.7760 | 17.307 | 15.763 | 3.7410 | 2.3640 |
|  | Mean | 0.8840 | 0.8720 | 0.0990 | 0.0680 | 2.5910 | 2.9840 | 0.3420 | 0.2370 | 4.4350 | 4.4920 | 1.0040 | 0.5670 |
| UL: UPT CDF [Mbps] | 5% | 24.010 | 25.450 | 38.710 | 47.450 | 6.4200 | 6.6500 | 27.530 | 29.020 | 1.5800 | 1.5700 | 15.210 | 14.800 |
|  | 50% | 64.330 | 62.840 | 73.830 | 72.250 | 43.330 | 42.840 | 62.090 | 55.710 | 23.230 | 23.700 | 50.790 | 40.940 |
|  | 95% | 90.720 | 88.990 | 98.640 | 85.520 | 77.150 | 77.920 | 92.350 | 72.130 | 60.650 | 62.870 | 83.710 | 56.930 |
|  | Mean | 63.470 | 62.400 | 74.070 | 71.680 | 44.160 | 44.070 | 63.750 | 55.600 | 28.110 | 28.750 | 52.160 | 39.520 |
| UL: Delay CDF [s] | 5% | 0.0380 | 0.0390 | 0.0360 | 0.0410 | 0.0440 | 0.0410 | 0.0390 | 0.0480 | 0.0480 | 0.0550 | 0.0440 | 0.0640 |
|  | 50% | 0.2580 | 0.2690 | 0.0700 | 0.0630 | 0.7100 | 0.6560 | 0.1370 | 0.1180 | 1.8020 | 1.3710 | 0.2110 | 0.1710 |
|  | 95% | 2.7330 | 2.9920 | 0.2800 | 0.2380 | 10.961 | 9.2730 | 1.2210 | 1.3510 | 14.655 | 14.784 | 3.5080 | 4.6790 |
|  | Mean | 0.7570 | 0.8300 | 0.1130 | 0.1020 | 2.7490 | 2.2940 | 0.3790 | 0.3980 | 4.2740 | 3.9020 | 0.8650 | 1.1290 |
| VoIP outage |  | 0.4100 | N/A | 0.2000 | N/A | 0.6300 | N/A | 0.3400 | N/A | 0.8300 | N/A | 0.5000 | N/A |
| VoIP outage(DL) |  | 0.3400 | N/A | 0.2000 | N/A | 0.5700 | N/A | 0.3300 | N/A | 0.7800 | N/A | 0.4700 | N/A |
| VoIP outage(UL) |  | 0.1600 | N/A | 0 | N/A | 0.2800 | N/A | 0.0500 | N/A | 0.5200 | N/A | 0.1000 | N/A |
| 𝜌DL |  | 0.9500 | 0.9500 | 1.0000 | 1.0000 | 0.8100 | 0.8300 | 0.9900 | 1.0000 | 0.6700 | 0.6900 | 0.9500 | 0.9800 |
| 𝜌UL |  | 0.9600 | 0.9500 | 1.0000 | 1.0000 | 0.8500 | 0.8700 | 0.9900 | 0.9800 | 0.7500 | 0.7600 | 0.9600 | 0.9300 |
| BO |  | 0.2000 | 0.1900 | 0.0800 | 0.0600 | 0.4000 | 0.3900 | 0.1600 | 0.1300 | 0.6000 | 0.5900 | 0.2600 | 0.2200 |
| 𝜆 |  | 0.203897 |  |  |  | 0.246513 |  |  |  | 0.276395 |  |  |  |
| Company/tdoc: Ericsson / R1-153125 LBT category: Category 4 DL LBT and fast UL LBT with self-scheduling Additional information: Without licensed carrier DL/UL split: 50/50 Sensing threshold used: -82 dBm Whether defer periods are used or not: yes CCA and ECCA slot length: 20 μs Whether or not intra and/or inter-RAT detection is assumed: only CCA-ED Any significant deviations from evaluations methodology and assumptions: UL Q=1 |  |  |  |  |  |  |  |  |  |  |  |  |  |

| DL: UPT CDF [Mbps] | 5% | 28.78 | 26.73 | 41.42 | 64.56 | 7.26 | 8.61 | 22.19 | 36.63 | 1.62 | 1.98 | 7.33 | 13.61 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 50% | 66.99 | 67.23 | 75.15 | 95.04 | 47.32 | 48.1 | 55.54 | 75.62 | 24.4 | 25.91 | 30.55 | 44.99 |
|  | 95% | 93.09 | 93.79 | 97.41 | 112.5 | 80.04 | 82.68 | 86.46 | 104.34 | 64.69 | 66.71 | 63.31 | 82.51 |
|  | Mean | 65.97 | 66.55 | 75.35 | 94.34 | 46.2 | 47.57 | 57.21 | 75.94 | 29.71 | 30.88 | 33.99 | 47.76 |
| DL: Delay CDF [s] | 5% | 0.038 | 0.042 | 0.036 | 0.03 | 0.04 | 0.059 | 0.039 | 0.034 | 0.047 | 0.048 | 0.088 | 0.055 |
|  | 50% | 0.234 | 0.2 | 0.073 | 0.049 | 0.37 | 0.655 | 0.175 | 0.112 | 1.244 | 1.473 | 0.84 | 0.502 |
|  | 95% | 3.28 | 2.924 | 0.399 | 0.229 | 11.493 | 13.264 | 2.875 | 1.601 | 17.307 | 15.763 | 8.448 | 5.341 |
|  | Mean | 0.884 | 0.872 | 0.142 | 0.086 | 2.591 | 2.984 | 0.769 | 0.408 | 4.435 | 4.492 | 2.49 | 1.541 |
| UL: UPT CDF [Mbps] | 5% | 24.01 | 25.45 | 35.46 | 50.43 | 6.42 | 6.65 | 17.29 | 24.46 | 1.58 | 1.57 | 5.88 | 9.02 |
|  | 50% | 64.33 | 62.84 | 72.18 | 77.31 | 43.33 | 42.84 | 51.62 | 56.16 | 23.23 | 23.7 | 29.27 | 30.78 |
|  | 95% | 90.72 | 88.99 | 96.98 | 91.64 | 77.15 | 77.92 | 87.87 | 76.55 | 60.65 | 62.87 | 63.78 | 54.44 |
|  | Mean | 63.47 | 62.4 | 71.95 | 76.71 | 44.16 | 44.07 | 54.19 | 56.19 | 28.11 | 28.75 | 32.94 | 32.29 |
| UL: Delay CDF [s] | 5% | 0.038 | 0.039 | 0.037 | 0.039 | 0.044 | 0.041 | 0.043 | 0.045 | 0.048 | 0.055 | 0.093 | 0.072 |
|  | 50% | 0.258 | 0.269 | 0.077 | 0.058 | 0.71 | 0.656 | 0.183 | 0.117 | 1.802 | 1.371 | 0.744 | 0.585 |
|  | 95% | 2.733 | 2.992 | 0.397 | 0.23 | 10.961 | 9.273 | 2.601 | 1.409 | 14.655 | 14.784 | 7.197 | 7.465 |
|  | Mean | 0.757 | 0.83 | 0.147 | 0.101 | 2.749 | 2.294 | 0.699 | 0.457 | 4.274 | 3.902 | 2.133 | 2.033 |
| VoIP outage |  | 0.41 | N/A | 0.3 | N/A | 0.63 | N/A | 0.54 | N/A | 0.83 | N/A | 0.78 | N/A |
| VoIP outage(DL) |  | 0.34 | N/A | 0.3 | N/A | 0.57 | N/A | 0.48 | N/A | 0.78 | N/A | 0.72 | N/A |
| VoIP outage(UL) |  | 0.16 | N/A | 0.01 | N/A | 0.28 | N/A | 0.18 | N/A | 0.52 | N/A | 0.36 | N/A |
| 𝜌DL |  | 0.95 | 0.95 | 1 | 1 | 0.81 | 0.83 | 0.98 | 0.99 | 0.67 | 0.69 | 0.9 | 0.95 |
| 𝜌UL |  | 0.96 | 0.95 | 1 | 1 | 0.85 | 0.87 | 0.98 | 0.98 | 0.75 | 0.76 |  |  |
| BO |  | 0.2 | 0.19 | 0.09 | 0.06 | 0.4 | 0.39 | 0.21 | 0.15 | 0.6 | 0.59 | 0.45 | 0.36 |
| 𝜆 |  | 0.203897 |  |  |  | 0.246513 |  |  |  | 0.276395 |  |  |  |
| Company/tdoc: Ericsson / R1-153126 LBT category: Category 4 on DL and Cat 1 on UL with self-scheduling Additional information: Without licensed carrier DL/UL split: 50/50 Sensing threshold used: -82 dBm Whether defer periods are used or not: Yes for DL, No for UL CCA and ECCA slot length: 20 μs Whether or not intra and/or inter-RAT detection is assumed: only CCA-ED Any significant deviations from evaluations methodology and assumptions: |  |  |  |  |  |  |  |  |  |  |  |  |  |

| DL: UPT CDF [Mbps] | 5% | 28.78 | 26.73 | 54.11 | 76.09 | 7.26 | 8.61 | 46.61 | 63.23 | 1.62 | 1.98 | 39.93 | 55.77 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 50% | 66.99 | 67.23 | 86.88 | 102.38 | 47.32 | 48.1 | 80.66 | 96.21 | 24.4 | 25.91 | 75.73 | 90.09 |
|  | 95% | 93.09 | 93.79 | 105.52 | 116.08 | 80.04 | 82.68 | 101.23 | 111.85 | 64.69 | 66.71 | 99.36 | 108.27 |
|  | Mean | 65.97 | 66.55 | 86.45 | 101.92 | 46.2 | 47.57 | 80.22 | 94.72 | 29.71 | 30.88 | 75.82 | 88.79 |
| DL: Delay CDF [s] | 5% | 0.038 | 0.042 | 0.033 | 0.03 | 0.04 | 0.059 | 0.034 | 0.031 | 0.047 | 0.048 | 0.035 | 0.032 |
|  | 50% | 0.234 | 0.2 | 0.047 | 0.038 | 0.37 | 0.655 | 0.054 | 0.042 | 1.244 | 1.473 | 0.064 | 0.053 |
|  | 95% | 3.28 | 2.924 | 0.097 | 0.062 | 11.493 | 13.264 | 0.196 | 0.099 | 17.307 | 15.763 | 0.443 | 0.456 |
|  | Mean | 0.884 | 0.872 | 0.058 | 0.043 | 2.591 | 2.984 | 0.085 | 0.054 | 4.435 | 4.492 | 0.156 | 0.128 |
| UL: UPT CDF [Mbps] | 5% | 24.01 | 25.45 | 46.58 | 27.05 | 6.42 | 6.65 | 39.15 | 19.4 | 1.58 | 1.57 | 33.73 | 12.77 |
|  | 50% | 64.33 | 62.84 | 82.28 | 41.32 | 43.33 | 42.84 | 76.37 | 30.72 | 23.23 | 23.7 | 71.48 | 21.19 |
|  | 95% | 90.72 | 88.99 | 103.4 | 50.21 | 77.15 | 77.92 | 100 | 38.85 | 60.65 | 62.87 | 96.44 | 27.57 |
|  | Mean | 63.47 | 62.4 | 82.08 | 41.65 | 44.16 | 44.07 | 76.07 | 30.95 | 28.11 | 28.75 | 71.46 | 21.45 |
| UL: Delay CDF [s] | 5% | 0.038 | 0.039 | 0.034 | 0.072 | 0.044 | 0.041 | 0.034 | 0.097 | 0.048 | 0.055 | 0.036 | 0.139 |
|  | 50% | 0.258 | 0.269 | 0.052 | 0.101 | 0.71 | 0.656 | 0.061 | 0.142 | 1.802 | 1.371 | 0.075 | 0.223 |
|  | 95% | 2.733 | 2.992 | 0.137 | 0.217 | 10.961 | 9.273 | 0.297 | 0.778 | 14.655 | 14.784 | 0.598 | 1.282 |
|  | Mean | 0.757 | 0.83 | 0.07 | 0.123 | 2.749 | 2.294 | 0.12 | 0.266 | 4.274 | 3.902 | 0.198 | 0.48 |
| VoIP outage |  | 0.41 | N/A | 0.18 | N/A | 0.63 | N/A | 0.2 | N/A | 0.83 | N/A | 0.27 | N/A |
| VoIP outage(DL) |  | 0.34 | N/A | 0.18 | N/A | 0.57 | N/A | 0.2 | N/A | 0.78 | N/A | 0.26 | N/A |
| VoIP outage(UL) |  | 0.16 | N/A | 0 | N/A | 0.28 | N/A | 0.01 | N/A | 0.52 | N/A | 0.02 | N/A |
| 𝜌DL |  | 0.95 | 0.95 | 1 | 1 | 0.81 | 0.83 | 1 | 1 | 0.67 | 0.69 | 1 | 1 |
| 𝜌UL |  | 0.96 | 0.95 | 1 | 1 | 0.85 | 0.87 | 1 | 1 | 0.75 | 0.76 | 1 | 1 |
| BO |  | 0.2 | 0.19 | 0.06 | 0.07 | 0.4 | 0.39 | 0.09 | 0.11 | 0.6 | 0.59 | 0.12 | 0.17 |
| 𝜆 |  | 0.203897 |  |  |  | 0.246513 |  |  |  | 0.276395 |  |  |  |
| Company/tdoc: Ericsson / R1-153127 LBT category: Category 4 DL and Category 2 UL with self-scheduling Additional information: Without licensed carrier DL/UL split: 50/50 Sensing threshold used: -82 dBm Whether defer periods are used or not: yes CCA and ECCA slot length: 20 μs Whether or not intra and/or inter-RAT detection is assumed: only CCA-ED Any significant deviations from evaluations methodology and assumptions: |  |  |  |  |  |  |  |  |  |  |  |  |  |

| DL: UPT CDF [Mbps] | 5% | 28.780 | 26.730 | 46.870 | 72.010 | 7.2600 | 8.6100 | 33.580 | 49.800 | 1.6200 | 1.9800 | 21.250 | 34.610 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 50% | 66.990 | 67.230 | 82.130 | 99.860 | 47.320 | 48.100 | 70.360 | 90.820 | 24.400 | 25.910 | 57.700 | 75.360 |
|  | 95% | 93.090 | 93.790 | 101.10 | 113.31 | 80.040 | 82.680 | 95.070 | 107.98 | 64.690 | 66.710 | 87.310 | 101.79 |
|  | Mean | 65.970 | 66.550 | 80.620 | 98.740 | 46.200 | 47.570 | 70.020 | 88.130 | 29.710 | 30.880 | 58.640 | 74.95 |
| DL: Delay CDF [s] | 5% | 0.0380 | 0.0420 | 0.0350 | 0.0310 | 0.0400 | 0.0590 | 0.0360 | 0.0320 | 0.0470 | 0.0480 | 0.0420 | 0.0360 |
|  | 50% | 0.2340 | 0.2000 | 0.0540 | 0.0400 | 0.3700 | 0.6550 | 0.0740 | 0.0480 | 1.2440 | 1.4730 | 0.2350 | 0.0970 |
|  | 95% | 3.2800 | 2.9240 | 0.1760 | 0.0850 | 11.493 | 13.264 | 0.8030 | 0.3040 | 17.307 | 15.763 | 1.9680 | 0.9230 |
|  | Mean | 0.8840 | 0.8720 | 0.0800 | 0.0500 | 2.5910 | 2.9840 | 0.2290 | 0.1100 | 4.4350 | 4.4920 | 0.6130 | 0.2890 |
| UL: UPT CDF [Mbps] | 5% | 24.010 | 25.450 | 39.830 | 49.170 | 6.4200 | 6.6500 | 26.700 | 27.990 | 1.5800 | 1.5700 | 16.810 | 14.740 |
|  | 50% | 64.330 | 62.840 | 76.990 | 71.340 | 43.330 | 42.840 | 64.650 | 53.500 | 23.230 | 23.700 | 52.120 | 36.200 |
|  | 95% | 90.720 | 88.990 | 98.980 | 84.610 | 77.150 | 77.920 | 91.980 | 67.330 | 60.650 | 62.870 | 85.800 | 51.400 |
|  | Mean | 63.470 | 62.400 | 76.370 | 71.120 | 44.160 | 44.070 | 64.920 | 52.670 | 28.110 | 28.750 | 54.430 | 36.090 |
| UL: Delay CDF [s] | 5% | 0.0380 | 0.0390 | 0.0350 | 0.0410 | 0.0440 | 0.0410 | 0.0380 | 0.0530 | 0.0480 | 0.0550 | 0.0430 | 0.0700 |
|  | 50% | 0.2580 | 0.2690 | 0.0600 | 0.0610 | 0.7100 | 0.6560 | 0.0890 | 0.0990 | 1.8020 | 1.3710 | 0.2370 | 0.2330 |
|  | 95% | 2.7330 | 2.9920 | 0.2150 | 0.1970 | 10.961 | 9.2730 | 1.0670 | 1.1060 | 14.655 | 14.784 | 2.4320 | 2.8940 |
|  | Mean | 0.7570 | 0.8300 | 0.0900 | 0.0880 | 2.7490 | 2.2940 | 0.2700 | 0.3020 | 4.2740 | 3.9020 | 0.7260 | 0.8440 |
| VoIP outage |  | 0.4100 | N/A | 0.2200 | N/A | 0.6300 | N/A | 0.2800 | N/A | 0.8300 | N/A | 0.4400 | N/A |
| VoIP outage(DL) |  | 0.3400 | N/A | 0.2100 | N/A | 0.5700 | N/A | 0.2400 | N/A | 0.7800 | N/A | 0.4100 | N/A |
| VoIP outage(UL) |  | 0.1600 | N/A | 0.0100 | N/A | 0.2800 | N/A | 0.0600 | N/A | 0.5200 | N/A | 0.1100 | N/A |
| 𝜌DL |  | 0.9500 | 0.9500 | 1.0000 | 1.0000 | 0.8100 | 0.8300 | 0.9900 | 1.0000 | 0.6700 | 0.6900 | 0.9700 | 0.9900 |
| 𝜌UL |  | 0.9600 | 0.9500 | 1.0000 | 1.0000 | 0.8500 | 0.8700 | 0.9900 | 0.9900 | 0.7500 | 0.7600 | 0.9600 | 0.9200 |
| BO |  | 0.2000 | 0.1900 | 0.0700 | 0.0600 | 0.4000 | 0.3900 | 0.1400 | 0.1100 | 0.6000 | 0.5900 | 0.2400 | 0.2000 |
| 𝜆 |  | 0.203897 |  |  |  | 0.246513 |  |  |  | 0.276395 |  |  |  |
| Company/tdoc: Ericsson / R1-153128 LBT category: Category 3 on DL and UL with self-scheduling Additional information: Without licensed carrier DL/UL split: 50/50 Sensing threshold used: -82 dBm Whether defer periods are used or not: yes CCA and ECCA slot length: 20 μs Whether or not intra and/or inter-RAT detection is assumed: only CCA-ED Any significant deviations from evaluations methodology and assumptions: DL Q=10, UL Q=4 |  |  |  |  |  |  |  |  |  |  |  |  |  |

| DL: UPT CDF [Mbps] | 5% | 28.780 | 26.730 | 47.590 | 69.3700 | 7.2600 | 8.6100 | 36.650 | 56.140 | 1.6200 | 1.9800 | 25.190 | 37.9300 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 50% | 66.990 | 67.230 | 82.770 | 99.4800 | 47.320 | 48.100 | 74.640 | 92.020 | 24.400 | 25.910 | 65.280 | 81.4300 |
|  | 95% | 93.090 | 93.790 | 101.69 | 114.110 | 80.040 | 82.680 | 95.530 | 109.79 | 64.690 | 66.710 | 90.690 | 103.870 |
|  | Mean | 65.970 | 66.550 | 81.510 | 98.5200 | 46.200 | 47.570 | 73.990 | 91.030 | 29.710 | 30.880 | 63.710 | 78.840 |
| DL: Delay CDF [s] | 5% | 0.0380 | 0.0420 | 0.0340 | 0.0300 | 0.0400 | 0.0590 | 0.0370 | 0.0320 | 0.0470 | 0.0480 | 0.0400 | 0.0360 |
|  | 50% | 0.2340 | 0.2000 | 0.0520 | 0.0400 | 0.3700 | 0.6550 | 0.0630 | 0.0460 | 1.2440 | 1.4730 | 0.1080 | 0.0770 |
|  | 95% | 3.2800 | 2.9240 | 0.1500 | 0.0960 | 11.493 | 13.264 | 0.3150 | 0.3080 | 17.307 | 15.763 | 1.3780 | 0.6990 |
|  | Mean | 0.8840 | 0.8720 | 0.1020 | 0.0510 | 2.5910 | 2.9840 | 0.1170 | 0.0950 | 4.4350 | 4.4920 | 0.4420 | 0.2150 |
| UL: UPT CDF [Mbps] | 5% | 24.010 | 25.450 | 40.360 | 50.150 | 6.4200 | 6.6500 | 30.850 | 34.940 | 1.5800 | 1.5700 | 19.850 | 19.800 |
|  | 50% | 64.330 | 62.840 | 77.680 | 71.360 | 43.330 | 42.840 | 70.050 | 58.110 | 23.230 | 23.700 | 59.650 | 42.010 |
|  | 95% | 90.720 | 88.990 | 100.94 | 85.000 | 77.150 | 77.920 | 94.700 | 72.790 | 60.650 | 62.870 | 87.940 | 55.780 |
|  | Mean | 63.470 | 62.400 | 77.340 | 71.160 | 44.160 | 44.070 | 69.220 | 57.660 | 28.110 | 28.750 | 58.560 | 41.060 |
| UL: Delay CDF [s] | 5% | 0.0380 | 0.0390 | 0.0340 | 0.0410 | 0.0440 | 0.0410 | 0.0370 | 0.0490 | 0.0480 | 0.0550 | 0.0420 | 0.0660 |
|  | 50% | 0.2580 | 0.2690 | 0.0570 | 0.0590 | 0.7100 | 0.6560 | 0.0750 | 0.0830 | 1.8020 | 1.3710 | 0.1400 | 0.1860 |
|  | 95% | 2.7330 | 2.9920 | 0.2120 | 0.1580 | 10.961 | 9.2730 | 0.4090 | 0.3810 | 14.655 | 14.784 | 1.7730 | 2.7510 |
|  | Mean | 0.7570 | 0.8300 | 0.1140 | 0.0850 | 2.7490 | 2.2940 | 0.1950 | 0.2020 | 4.2740 | 3.9020 | 0.5350 | 0.7610 |
| VoIP outage |  | 0.4100 | N/A | 0.1800 | N/A | 0.6300 | N/A | 0.2000 | N/A | 0.8300 | N/A | 0.3100 | N/A |
| VoIP outage(DL) |  | 0.3400 | N/A | 0.1800 | N/A | 0.5700 | N/A | 0.2000 | N/A | 0.7800 | N/A | 0.3100 | N/A |
| VoIP outage(UL) |  | 0.1600 | N/A | 0 | N/A | 0.2800 | N/A | 0.0100 | N/A | 0.5200 | N/A | 0.0900 | N/A |
| 𝜌DL |  | 0.9500 | 0.9500 | 1.0000 | 1.0000 | 0.8100 | 0.8300 | 1.0000 | 1.0000 | 0.6700 | 0.6900 | 0.9900 | 0.9900 |
| 𝜌UL |  | 0.9600 | 0.9500 | 1.0000 | 1.0000 | 0.8500 | 0.8700 | 1.0000 | 0.9900 | 0.7500 | 0.7600 | 0.9800 | 0.9600 |
| BO |  | 0.2000 | 0.1900 | 0.0700 | 0.0600 | 0.4000 | 0.3900 | 0.1100 | 0.0900 | 0.6000 | 0.5900 | 0.1900 | 0.1700 |
| 𝜆 |  | 0.203897 |  |  |  | 0.246513 |  |  |  | 0.276395 |  |  |  |
| Company/tdoc: Ericsson / R1-153129 LBT category: Category 4 on DL and UL with self-scheduling Additional information: Sensing threshold used: -82 dBm Whether defer periods are used or not: yes CCA and ECCA slot length: 9 μs Whether or not intra and/or inter-RAT detection is assumed: only CCA-ED Any significant deviations from evaluations methodology and assumptions: DL CWmin=15, DL CWmax=1023, UL CWmin=7, UL CWmax=15 |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 3: Indoor deployment for LAA and LAA coexistence case with one shared unlicensed carrier and FTP traffic

| Tdoc / Company | LAA LBT cat. | Reported parameters |  | Low load BO range for Wi-Fi in Step 1: 10%~25% |  | Medium load BO range for Wi-Fi in Step 1: 35%~50% |  | High load BO range for Wi-Fi in Step 1: above 55% |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | LAA Opt.1 | LAA Opt.2 | LAA Opt.1 | LAA Opt.2 | LAA Opt.1 | LAA Opt.2 |
| Ericsson / R1-153125 | Cat 4 DL, Cat 3 UL | DL: UPT CDF [Mbps] | 5% | 42.5600 | 41.4700 | 24.1500 | 23.3300 | 7.4000 | 7.0200 |
|  |  |  | 50% | 68.1600 | 69.0100 | 45.5300 | 46.2100 | 24.2600 | 24.8100 |
|  |  |  | 95% | 88.8700 | 90.0500 | 68.3000 | 69.0300 | 48.2800 | 48.5900 |
|  |  |  | Mean | 68.9200 | 69.4400 | 47.4000 | 47.7600 | 27.1600 | 27.3900 |
|  |  | DL: Delay CDF [s] | 5% | 0.0420 | 0.0410 | 0.0670 | 0.0640 | 0.0930 | 0.0890 |
|  |  |  | 50% | 0.0770 | 0.0750 | 0.1680 | 0.1590 | 0.2660 | 0.2490 |
|  |  |  | 95% | 0.1450 | 0.1440 | 0.4250 | 0.4430 | 0.7340 | 0.7780 |
|  |  |  | Mean | 0.0880 | 0.0860 | 0.2090 | 0.2090 | 0.3420 | 0.3430 |
|  |  | UL: UPT CDF [Mbps] | 5% | 23.4600 | 22.4900 | 11.5900 | 10.8800 | 1.7900 | 1.3500 |
|  |  |  | 50% | 40.8600 | 40.8300 | 24.1000 | 23.9400 | 9.4000 | 9.1800 |
|  |  |  | 95% | 54.6300 | 54.4800 | 36.5200 | 36.4400 | 20.0300 | 20.2500 |
|  |  |  | Mean | 41.3800 | 41.2500 | 24.9300 | 24.8500 | 10.4500 | 10.5200 |
|  |  | UL: Delay CDF [s] | 5% | 0.0690 | 0.0700 | 0.1340 | 0.1320 | 0.2100 | 0.2090 |
|  |  |  | 50% | 0.1250 | 0.1230 | 0.3810 | 0.3750 | 0.7270 | 0.6990 |
|  |  |  | 95% | 0.2360 | 0.2300 | 1.5790 | 1.7690 | 3.2140 | 3.5740 |
|  |  |  | Mean | 0.1420 | 0.1410 | 0.6210 | 0.6510 | 1.2140 | 1.2640 |
|  |  | 𝜌DL |  | 1.0000 | 1.0000 | 1.0000 | 1.0000 | 0.9900 | 0.9900 |
|  |  | 𝜌UL |  | 1.0000 | 1.0000 | 0.9800 | 0.9700 | 0.9400 | 0.9300 |
|  |  | BO |  | 0.2000 | 0.2000 | 0.4000 | 0.3900 | 0.6000 | 0.5800 |
|  |  | 𝜆 |  | 0.483934 |  | 0.582384 |  | 0.674512 |  |
|  | Additional comments: Without licensed carrier DL/UL split: 50/50 Sensing threshold used, -82 dBm Whether defer periods are used or not: Yes CCA and ECCA slot length: 20 μs Inter-operator synchronization for LAA-LAA coexistence: synchronized Whether or not intra and/or inter-RAT detection is assumed: only CCA-ED Any significant deviations from evaluations methodology and assumptions: UL Q=1 |  |  |  |  |  |  |  |  |

| Ericsson / R1-153126 | Cat 4 DL, Cat 1 UL | DL: UPT CDF [Mbps] | 5% | 29.45 | 31.31 | 10.86 | 11.08 | 5.35 | 5.33 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50% | 62.13 | 63.58 | 34.57 | 34.95 | 19.67 | 19.58 |
|  |  |  | 95% | 85.6 | 87.45 | 55.74 | 58.79 | 38.22 | 39.38 |
|  |  |  | Mean | 62.8 | 64.19 | 35.68 | 36.58 | 21.59 | 21.93 |
|  |  | DL: Delay CDF [s] | 5% | 0.05 | 0.047 | 0.092 | 0.082 | 0.134 | 0.127 |
|  |  |  | 50% | 0.118 | 0.114 | 0.28 | 0.275 | 0.56 | 0.563 |
|  |  |  | 95% | 0.287 | 0.266 | 0.793 | 0.819 | 2.026 | 2.275 |
|  |  |  | Mean | 0.147 | 0.139 | 0.374 | 0.369 | 0.842 | 0.909 |
|  |  | UL: UPT CDF [Mbps] | 5% | 26.17 | 23.58 | 7.34 | 7.89 | 3.34 | 3.5 |
|  |  |  | 50% | 49.41 | 48.84 | 26.78 | 25.74 | 14.54 | 13.61 |
|  |  |  | 95% | 69.32 | 67.36 | 45.89 | 44.35 | 30.93 | 29.92 |
|  |  |  | Mean | 50.61 | 49.39 | 28.26 | 27.34 | 16.55 | 15.81 |
|  |  | UL: Delay CDF [s] | 5% | 0.056 | 0.057 | 0.102 | 0.096 | 0.16 | 0.158 |
|  |  |  | 50% | 0.13 | 0.133 | 0.32 | 0.365 | 0.799 | 1.014 |
|  |  |  | 95% | 0.355 | 0.374 | 1.402 | 1.466 | 5.418 | 5.353 |
|  |  |  | Mean | 0.168 | 0.179 | 0.534 | 0.579 | 1.713 | 1.87 |
|  |  | 𝜌DL |  | 1 | 1 | 0.99 | 0.99 | 0.97 | 0.97 |
|  |  | 𝜌UL |  | 1 | 1 | 0.99 | 0.98 | 0.91 | 0.89 |
|  |  | BO |  | 0.2 | 0.2 | 0.4 | 0.4 | 0.6 | 0.6 |
|  |  | 𝜆 |  | 0.413224 |  | 0.510972 |  | 0.599834 |  |
|  | Additional comments: Without licensed carrier DL/UL split: 50/50 Sensing threshold used, -82 dBm for DL only Whether defer periods are used or not: Yes for DL, No for UL CCA and ECCA slot length: 20 μs Inter-operator synchronization for LAA-LAA coexistence: synchronized Whether or not intra and/or inter-RAT detection is assumed: only CCA-ED Any significant deviations from evaluations methodology and assumptions |  |  |  |  |  |  |  |  |

| Ericsson / R1-153127 | Cat 4 DL, Cat 2 UL | DL: UPT CDF [Mbps] | 5% | 69.79 | 70.32 | 53.79 | 54.34 | 34.55 | 33.6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50% | 94.34 | 94.43 | 75.88 | 76.8 | 52.42 | 53.13 |
|  |  |  | 95% | 108.98 | 108.57 | 92.39 | 92.75 | 68.88 | 69.21 |
|  |  |  | Mean | 94.48 | 94.15 | 76.43 | 77.14 | 53.28 | 53.86 |
|  |  | DL: Delay CDF [s] | 5% | 0.032 | 0.032 | 0.039 | 0.039 | 0.076 | 0.077 |
|  |  |  | 50% | 0.042 | 0.041 | 0.055 | 0.054 | 0.22 | 0.207 |
|  |  |  | 95% | 0.053 | 0.055 | 0.075 | 0.075 | 0.506 | 0.507 |
|  |  |  | Mean | 0.043 | 0.043 | 0.057 | 0.057 | 0.264 | 0.253 |
|  |  | UL: UPT CDF [Mbps] | 5% | 16.15 | 15.94 | 6.4 | 6.39 | 0.9 | 0.88 |
|  |  |  | 50% | 25.16 | 25.05 | 11.52 | 11.52 | 3.18 | 3.26 |
|  |  |  | 95% | 32.44 | 32.84 | 16.71 | 16.45 | 17.6 | 13.62 |
|  |  |  | Mean | 25.53 | 25.44 | 11.93 | 11.86 | 5.19 | 5.19 |
|  |  | UL: Delay CDF [s] | 5% | 0.119 | 0.121 | 0.317 | 0.329 | 0.418 | 0.433 |
|  |  |  | 50% | 0.18 | 0.181 | 0.716 | 0.692 | 1.464 | 1.541 |
|  |  |  | 95% | 0.262 | 0.265 | 1.803 | 1.765 | 4.105 | 4.051 |
|  |  |  | Mean | 0.19 | 0.192 | 0.889 | 0.895 | 1.869 | 1.932 |
|  |  | 𝜌DL |  | 1 | 1 | 1 | 1 | 0.96 | 0.96 |
|  |  | 𝜌UL |  | 1 | 1 | 0.97 | 0.97 | 0.78 | 0.78 |
|  |  | BO |  | 0.2 | 0.2 | 0.4 | 0.4 | 0.6 | 0.6 |
|  |  | 𝜆 |  | 0.435144 |  | 0.577946 |  | 0.893606 |  |
|  | Additional comments: Without licensed carrier DL/UL split: 50/50 Sensing threshold used, -82 dBm Whether defer periods are used or not: Yes CCA and ECCA slot length: 20 μs Inter-operator synchronization for LAA-LAA coexistence: synchronized Whether or not intra and/or inter-RAT detection is assumed: only CCA-ED Any significant deviations from evaluations methodology and assumptions: |  |  |  |  |  |  |  |  |

| Ericsson / R1-153128 | Cat 3 DL, Cat 3 UL | DL: UPT CDF [Mbps] | 5% | 40.98 | 45.68 | 25.38 | 27.77 | 13.62 | 14.07 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50% | 70.9 | 71.73 | 49.8 | 50.1 | 29.99 | 30.34 |
|  |  |  | 95% | 90.33 | 90.98 | 71.1 | 72.52 | 48.29 | 49.93 |
|  |  |  | Mean | 71.07 | 72.1 | 50.95 | 51.94 | 31.58 | 32.44 |
|  |  | DL: Delay CDF [s] | 5% | 0.041 | 0.042 | 0.057 | 0.056 | 0.086 | 0.083 |
|  |  |  | 50% | 0.065 | 0.065 | 0.106 | 0.106 | 0.168 | 0.168 |
|  |  |  | 95% | 0.106 | 0.103 | 0.2 | 0.195 | 0.337 | 0.331 |
|  |  |  | Mean | 0.07 | 0.07 | 0.12 | 0.119 | 0.195 | 0.193 |
|  |  | UL: UPT CDF [Mbps] | 5% | 22.79 | 22.05 | 9.51 | 9.15 | 0.83 | 0.78 |
|  |  |  | 50% | 39.62 | 38.49 | 21.06 | 20.44 | 5.47 | 5.36 |
|  |  |  | 95% | 51.57 | 51.23 | 31.54 | 31.85 | 12.48 | 12.97 |
|  |  |  | Mean | 39.74 | 39.03 | 21.55 | 21.37 | 6.25 | 6.38 |
|  |  | UL: Delay CDF [s] | 5% | 0.077 | 0.077 | 0.178 | 0.161 | 0.421 | 0.412 |
|  |  |  | 50% | 0.134 | 0.138 | 0.486 | 0.509 | 1.788 | 1.717 |
|  |  |  | 95% | 0.277 | 0.28 | 2.148 | 2.297 | 5.82 | 6.228 |
|  |  |  | Mean | 0.157 | 0.162 | 0.806 | 0.861 | 2.544 | 2.542 |
|  |  | 𝜌DL |  | 1 | 1 | 1 | 1 | 1 | 1 |
|  |  | 𝜌UL |  | 1 | 1 | 0.97 | 0.96 | 0.81 | 0.81 |
|  |  | BO |  | 0.2 | 0.2 | 0.4 | 0.4 | 0.6 | 0.6 |
|  |  | 𝜆 |  | 0.501374 |  | 0.605591 |  | 0.759579 |  |
|  | Additional comments: Without licensed carrier DL/UL split: 50/50 Sensing threshold used, -82 dBm Whether defer periods are used or not: Yes CCA and ECCA slot length: 20 μs Inter-operator synchronization for LAA-LAA coexistence: synchronized Whether or not intra and/or inter-RAT detection is assumed: only CCA-ED Any significant deviations from evaluations methodology and assumptions: DL Q=10, UL Q=4 |  |  |  |  |  |  |  |  |

| Ericsson / R1-153129 | Cat 4 DL, Cat 4 UL | DL: UPT CDF [Mbps] | 5% | 46.54 | 47.01 | 27.56 | 27.04 | 16.95 | 17.09 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50% | 73.94 | 74.03 | 50.42 | 52.43 | 32.96 | 34.3 |
|  |  |  | 95% | 91.88 | 92.03 | 72.55 | 71.55 | 51.06 | 50.76 |
|  |  |  | Mean | 74.16 | 74.33 | 52.45 | 52.96 | 34.81 | 35.39 |
|  |  | DL: Delay CDF [s] | 5% | 0.04 | 0.041 | 0.051 | 0.054 | 0.08 | 0.08 |
|  |  |  | 50% | 0.061 | 0.061 | 0.094 | 0.092 | 0.149 | 0.145 |
|  |  |  | 95% | 0.097 | 0.098 | 0.162 | 0.154 | 0.27 | 0.257 |
|  |  |  | Mean | 0.066 | 0.066 | 0.104 | 0.102 | 0.166 | 0.164 |
|  |  | UL: UPT CDF [Mbps] | 5% | 22.77 | 23.28 | 6.97 | 7.05 | 1.61 | 1.65 |
|  |  |  | 50% | 39.13 | 38.7 | 17.46 | 16.95 | 5.76 | 5.58 |
|  |  |  | 95% | 51.12 | 50.58 | 26.69 | 27.55 | 11.82 | 12.13 |
|  |  |  | Mean | 39.66 | 39.11 | 17.98 | 17.73 | 6.5 | 6.42 |
|  |  | UL: Delay CDF [s] | 5% | 0.077 | 0.079 | 0.154 | 0.154 | 0.437 | 0.453 |
|  |  |  | 50% | 0.133 | 0.136 | 0.331 | 0.355 | 1.455 | 1.586 |
|  |  |  | 95% | 0.254 | 0.27 | 0.956 | 1.012 | 5.205 | 5.135 |
|  |  |  | Mean | 0.153 | 0.157 | 0.453 | 0.475 | 2.175 | 2.249 |
|  |  | 𝜌DL |  | 1 | 1 | 1 | 1 | 1 | 1 |
|  |  | 𝜌UL |  | 1 | 1 | 0.99 | 0.99 | 0.84 | 0.83 |
|  |  | BO |  | 0.2 | 0.2 | 0.4 | 0.4 | 0.6 | 0.6 |
|  |  | 𝜆 |  | 0.512869 |  | 0.662242 |  | 0.798057 |  |
|  | Additional comments: Without licensed carrier DL/UL split: 50/50 Sensing threshold used, -82 dBm Whether defer periods are used or not: Yes CCA and ECCA slot length: 9 μs Inter-operator synchronization for LAA-LAA coexistence: synchronized Whether or not intra and/or inter-RAT detection is assumed: only CCA-ED Any significant deviations from evaluations methodology and assumptions: DL CWmin=15, DL CWmax=1023, UL CWmin=7, UL CWmax=15 |  |  |  |  |  |  |  |  |
| Fujitsu, R1-152654 | Cat 4 DL, Cat 2 UL | DL: UPT CDF [Mbps] | 5% | 23.8 | 0.00 | 23.7 | 24.0 | 32.0 | 32.4 |
|  |  |  | 50% | 108.9 | 106.7 | 87.0 | 88.6 | 86.3 | 81.4 |
|  |  |  | 95% | 148.2 | 151.0 | 148.2 | 151.0 | 148.2 | 151.0 |
|  |  |  | Mean | 104.2 | 101.4 | 87.5 | 89.2 | 89.4 | 85.3 |
|  |  | DL: Delay CDF [s] | 5% | 0.027 | 0.026 | 0.027 | 0.026 | 0.027 | 0.026 |
|  |  |  | 50% | 0.027 | 0.027 | 0.029 | 0.028 | 0.031 | 0.030 |
|  |  |  | 95% | 0.113 | 0.108 | 0.162 | 0.172 | 0.159 | 0.169 |
|  |  |  | Mean | 0.043 | 0.043 | 0.057 | 0.058 | 0.056 | 0.057 |
|  |  | UL: UPT CDF [Mbps] | 5% | 24.7 | 26.8 | 19.8 | 17.0 | 23.6 | 27.2 |
|  |  |  | 50% | 53.7 | 56.3 | 47.7 | 50.3 | 48.1 | 51.4 |
|  |  |  | 95% | 66.7 | 67.0 | 64.9 | 65.3 | 64.9 | 66.1 |
|  |  |  | Mean | 50.9 | 53.0 | 46.4 | 47.9 | 46.6 | 49.0 |
|  |  | UL: Delay CDF [s] | 5% | 0.060 | 0.059 | 0.060 | 0.059 | 0.060 | 0.059 |
|  |  |  | 50% | 0.063 | 0.060 | 0.065 | 0.063 | 0.066 | 0.064 |
|  |  |  | 95% | 0.153 | 0.139 | 0.198 | 0.176 | 0.193 | 0.182 |
|  |  |  | Mean | 0.080 | 0.076 | 0.094 | 0.088 | 0.092 | 0.086 |
|  |  | 𝜌DL |  | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 |
|  |  | 𝜌UL |  | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 |
|  |  | BO |  | 5.7 | 5.8 | 8.3 | 8.0 | 8.7 | 8.5 |
|  |  | 𝜆 |  | 0.1 |  | 0.12 |  | 0.1275 |  |
|  | LBT category: category 4 for DL transmission, category 2 for UL transmission Additional information: no UE multiplexing in frequency domain for LAA UL Sensing threshold used: -62dBm Whether defer periods are used or not: yes CCA and ECCA slot length: 9 us for DL Whether or not intra and/or inter-RAT detection is assumed: only CCA-ED Any significant deviations from evaluations methodology and assumptions: FTP model 1 |  |  |  |  |  |  |  |  |

Table 4: Outdoor deployment for Wi-Fi and LAA coexistence case with one shared unlicensed carrier and FTP traffic

| Reported parameters |  | Low load BO range for Wi-Fi Opt.1 in Step 1: 10%~25% |  |  |  | Medium load BO range for Wi-Fi Opt. 1 in Step 1: 35%~50% |  |  |  | High load BO range for Wi-Fi Opt.1 in Step 1: above 55% |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Wi-Fi Opt.1 in step 1 | Wi-Fi Opt.2 in step 1 | Wi-Fi Opt. 1 in step 2 | LAA Opt.2 in step 2 | Wi-Fi Opt.1 in step 1 | Wi-Fi Opt.2 in step 1 | Wi-Fi Opt. 1 in step 2 | LAA Opt.2 in step 2 | Wi-Fi Opt.1 in step 1 | Wi-Fi Opt.2 in step 1 | Wi-Fi Opt. 1 in step 2 | LAA Opt.2 in step 2 |
| DL: UPT CDF [Mbps] | 5% | 12.16 | 12.363 | 23.819 | 42.285 | 2.174 | 2.537 | 12.738 | 22.345 | 0.065 | 0.231 | 4.416 | 8.108 |
|  | 50% | 42.03 | 42.665 | 58.437 | 88.236 | 24.714 | 25.539 | 46.884 | 69.367 | 13.379 | 13.905 | 30.424 | 41.793 |
|  | 95% | 68.51 | 67.721 | 81.588 | 113.79 | 56.633 | 57.272 | 75.394 | 102.49 | 43.810 | 45.262 | 63.591 | 82.247 |
|  | Mean | 41.64 | 41.945 | 56.466 | 84.733 | 26.630 | 27.167 | 46.101 | 66.962 | 16.526 | 17.031 | 32.073 | 42.996 |
| DL: Delay CDF [s] | 5% | 0.328 | 0.324 | 0.168 | 0.096 | 1.840 | 1.576 | 0.316 | 0.180 | 61.540 | 17.316 | 0.904 | 0.492 |
|  | 50% | 0.096 | 0.092 | 0.068 | 0.044 | 0.160 | 0.156 | 0.084 | 0.056 | 0.300 | 0.288 | 0.132 | 0.096 |
|  | 95% | 0.060 | 0.060 | 0.048 | 0.036 | 0.072 | 0.068 | 0.052 | 0.040 | 0.092 | 0.088 | 0.064 | 0.048 |
|  | Mean | 0.096 | 0.096 | 0.072 | 0.048 | 0.152 | 0.148 | 0.088 | 0.060 | 0.244 | 0.236 | 0.124 | 0.092 |
| UL: UPT CDF [Mbps] | 5% | 9.495 | 9.481 | 19.069 | 33.183 | 1.666 | 1.698 | 8.127 | 16.172 | 0.036 | 0.052 | 2.479 | 5.411 |
|  | 50% | 36.08 | 37.166 | 51.100 | 77.883 | 18.042 | 19.240 | 38.842 | 60.972 | 8.243 | 8.940 | 22.548 | 35.760 |
|  | 95% | 66.41 | 66.099 | 78.961 | 106.40 | 53.015 | 52.078 | 70.677 | 96.227 | 36.857 | 37.135 | 57.836 | 78.353 |
|  | Mean | 36.98 | 37.531 | 50.592 | 75.410 | 21.637 | 22.089 | 39.273 | 59.240 | 12.018 | 12.485 | 25.395 | 37.731 |
| UL: Delay CDF [s] | 5% | 0.421 | 0.422 | 0.210 | 0.121 | 2.401 | 2.356 | 0.492 | 0.247 | 111.11 | 76.923 | 1.614 | 0.739 |
|  | 50% | 0.111 | 0.108 | 0.078 | 0.051 | 0.222 | 0.208 | 0.103 | 0.066 | 0.485 | 0.447 | 0.177 | 0.112 |
|  | 95% | 0.060 | 0.061 | 0.051 | 0.038 | 0.075 | 0.077 | 0.057 | 0.042 | 0.109 | 0.108 | 0.069 | 0.051 |
|  | Mean | 0.108 | 0.107 | 0.079 | 0.053 | 0.185 | 0.181 | 0.102 | 0.068 | 0.333 | 0.320 | 0.158 | 0.106 |
| 𝜌DL |  | 1.00 | 0.99 | 1.00 | 1.00 | 0.97 | 0.98 | 1.00 | 0.99 | 0.92 | 0.92 | 0.97 | 0.98 |
| 𝜌UL |  | 0.99 | 1.00 | 1.00 | 1.00 | 0.96 | 0.97 | 0.99 | 1.00 | 0.89 | 0.90 | 0.96 | 0.98 |
| BODL |  | 0.20 | 0.20 | 0.10 | 0.06 | 0.40 | 0.39 | 0.18 | 0.11 | 0.60 | 0.59 | 0.37 | 0.27 |
| BOUL |  | 0.24 | 0.23 | 0.12 | 0.07 | 0.46 | 0.46 | 0.21 | 0.13 | 0.67 | 0.66 | 0.42 | 0.31 |
| 𝜆 |  | 1.04 Mbps |  |  |  | 1.17 Mbps |  |  |  | 1.31 Mbps |  |  |  |
| Company/tdoc: QCOM/R1-152782 LBT category 3 with q = 25 and 20us CCA slots and 9.5ms TXOP for both DL and UL LAA transmission Additional information: ED threshold: Wi-Fi: -62 dBm; LAA: -82 dBm. WiFi preamble detection threshold = 4dB SINR. RTS-CTS enabled on Wi-Fi |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 5: Additional coexistence evaluation results for Indoor deployment for Wi-Fi and LAA coexistence case with one shared unlicensed carrier and FTP traffic with 50 users/operator

| Reported parameters |  | Low load BO range for Wi-Fi Opt.1 in Step 1: 10%~25% |  |  |  | Medium load BO range for Wi-Fi Opt. 1 in Step 1: 35%~50% |  |  |  | High load BO range for Wi-Fi Opt.1 in Step 1: above 55% |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Wi-Fi Opt1 in step 1 | Wi-Fi Opt.2 in step 1 | Wi-Fi Opt. 1 in step 2 | LAA Opt.2 in step 2 | Wi-Fi Opt.1 in step 1 | Wi-Fi Opt.2 in step 1 | Wi-Fi Opt. 1 in step 2 | LAA Opt.2 in step 2 | Wi-Fi Opt.1 in step 1 | Wi-Fi Opt.2 in step 1 | Wi-Fi Opt. 1 in step 2 | LAA Opt.2 in step 2 |
| DL: UPT CDF [Mbps] | 5% | 21.306 | 22.073 | 35.700 | 69.143 | 3.064 | 3.535 | 21.644 | 40.309 | 1.766 | 2.036 | 12.414 | 23.115 |
|  | 50% | 51.082 | 51.109 | 61.143 | 100.647 | 32.822 | 33.273 | 45.228 | 76.354 | 19.145 | 19.310 | 26.703 | 46.583 |
|  | 95% | 72.616 | 72.616 | 81.298 | 121.291 | 55.835 | 55.541 | 65.973 | 100.47 | 40.395 | 40.132 | 45.980 | 70.805 |
|  | Mean | 49.985 | 49.874 | 60.125 | 98.526 | 32.401 | 32.424 | 44.648 | 74.011 | 20.643 | 20.576 | 27.825 | 46.554 |
| DL: Delay CDF [s] | 5% | 0.188 | 0.181 | 0.112 | 0.058 | 1.305 | 1.132 | 0.185 | 0.099 | 2.265 | 1.965 | 0.322 | 0.173 |
|  | 50% | 0.078 | 0.078 | 0.065 | 0.040 | 0.122 | 0.120 | 0.088 | 0.052 | 0.209 | 0.207 | 0.150 | 0.086 |
|  | 95% | 0.055 | 0.055 | 0.049 | 0.033 | 0.072 | 0.072 | 0.061 | 0.040 | 0.099 | 0.100 | 0.087 | 0.056 |
|  | Mean | 0.080 | 0.080 | 0.067 | 0.041 | 0.123 | 0.123 | 0.090 | 0.054 | 0.194 | 0.194 | 0.144 | 0.086 |
| UL: UPT CDF [Mbps] | 5% | 22.176 | 21.934 | 35.268 | 40.830 | 3.005 | 3.048 | 20.474 | 32.455 | 1.744 | 1.769 | 11.763 | 21.377 |
|  | 50% | 50.724 | 50.784 | 60.757 | 65.021 | 32.385 | 32.439 | 44.813 | 55.722 | 19.718 | 19.717 | 26.835 | 45.861 |
|  | 95% | 72.084 | 72.062 | 80.213 | 83.377 | 54.543 | 54.435 | 64.552 | 74.104 | 38.099 | 38.153 | 43.624 | 65.764 |
|  | Mean | 49.706 | 49.671 | 59.610 | 64.085 | 31.695 | 31.661 | 43.839 | 54.933 | 20.156 | 20.130 | 27.077 | 45.219 |
| UL: Delay CDF [s] | 5% | 0.180 | 0.182 | 0.113 | 0.098 | 1.331 | 1.312 | 0.195 | 0.123 | 2.294 | 2.261 | 0.340 | 0.187 |
|  | 50% | 0.079 | 0.079 | 0.066 | 0.062 | 0.124 | 0.123 | 0.089 | 0.072 | 0.203 | 0.203 | 0.149 | 0.087 |
|  | 95% | 0.055 | 0.056 | 0.050 | 0.048 | 0.073 | 0.073 | 0.062 | 0.054 | 0.105 | 0.105 | 0.092 | 0.061 |
|  | Mean | 0.080 | 0.081 | 0.067 | 0.062 | 0.126 | 0.126 | 0.091 | 0.073 | 0.198 | 0.199 | 0.148 | 0.088 |
| 𝜌DL |  | 0.99 | 0.99 | 1.00 | 1.00 | 0.94 | 0.95 | 0.96 | 0.98 | 0.84 | 0.83 | 0.86 | 0.92 |
| 𝜌UL |  | 1.00 | 1.00 | 1.00 | 1.00 | 0.99 | 0.99 | 0.99 | 1.00 | 0.98 | 0.96 | 0.98 | 1.00 |
| BODL |  | 0.20 | 0.20 | 0.12 | 0.06 | 0.40 | 0.40 | 0.27 | 0.17 | 0.60 | 0.60 | 0.52 | 0.40 |
| BOUL |  | 0.20 | 0.20 | 0.12 | 0.08 | 0.40 | 0.40 | 0.27 | 0.15 | 0.61 | 0.61 | 0.53 | 0.25 |
| 𝜆 |  | 0.49Mbps |  |  |  | 0.62Mbps |  |  |  | 0.70Mbps |  |  |  |
| Company/tdoc: QCOM/R1-153429 LBT category 3 with q = 25 and 20us CCA slots and 9.5ms TXOP for both DL and UL LAA transmission and 50 UEs per operator Additional information: ED threshold: Wi-Fi: -62 dBm; LAA: -82 dBm. WiFi preamble detection threshold = 4dB SINR. RTS-CTS enabled on Wi-Fi |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6: Additional coexistence evaluation results for Outdoor deployment for Wi-Fi and LAA coexistence case with one shared unlicensed carrier and FTP traffic with 50 users/operator

| Reported parameters |  | Low load BO range for Wi-Fi Opt.1 in Step 1: 10%~25% |  |  |  | Medium load BO range for Wi-Fi Opt. 1 in Step 1: 35%~50% |  |  |  | High load BO range for Wi-Fi Opt.1 in Step 1: above 55% |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Wi-Fi Opt.1 in step 1 | Wi-Fi Opt.2 in step 1 | Wi-Fi Opt. 1 in step 2 | LAA Opt.2 in step 2 | Wi-Fi Opt.1 in step 1 | Wi-Fi Opt.2 in step 1 | Wi-Fi Opt. 1 in step 2 | LAA Opt.2 in step 2 | Wi-Fi Opt.1 in step 1 | Wi-Fi Opt.2 in step 1 | Wi-Fi Opt. 1 in step 2 | LAA Opt.2 in step 2 |
| DL: UPT CDF [Mbps] | 5% | 11.957 | 11.489 | 23.668 | 49.289 | 2.164 | 2.063 | 17.256 | 33.965 | 1.191 | 1.137 | 9.597 | 19.318 |
|  | 50% | 39.388 | 40.319 | 57.107 | 96.915 | 23.861 | 25.005 | 49.200 | 80.353 | 15.129 | 15.949 | 36.271 | 56.160 |
|  | 95% | 68.026 | 68.034 | 83.258 | 124.595 | 55.204 | 54.153 | 77.110 | 112.18 | 41.775 | 41.341 | 68.117 | 95.509 |
|  | Mean | 39.760 | 40.117 | 55.785 | 93.470 | 25.703 | 26.054 | 48.516 | 77.798 | 17.449 | 17.802 | 37.314 | 56.799 |
| DL: Delay CDF [s] | 5% | 0.335 | 0.348 | 0.169 | 0.081 | 1.848 | 1.939 | 0.232 | 0.118 | 3.359 | 3.518 | 0.417 | 0.207 |
|  | 50% | 0.102 | 0.099 | 0.070 | 0.041 | 0.168 | 0.160 | 0.081 | 0.050 | 0.264 | 0.251 | 0.110 | 0.071 |
|  | 95% | 0.059 | 0.059 | 0.048 | 0.032 | 0.072 | 0.074 | 0.052 | 0.036 | 0.096 | 0.097 | 0.059 | 0.042 |
|  | Mean | 0.101 | 0.100 | 0.072 | 0.043 | 0.156 | 0.154 | 0.082 | 0.051 | 0.229 | 0.225 | 0.107 | 0.070 |
| UL: UPT CDF [Mbps] | 5% | 10.414 | 9.910 | 20.010 | 39.053 | 1.108 | 1.068 | 14.030 | 26.098 | 0.613 | 0.592 | 7.721 | 14.476 |
|  | 50% | 35.237 | 35.742 | 52.444 | 86.472 | 18.599 | 19.041 | 43.371 | 70.409 | 11.107 | 11.440 | 29.787 | 48.748 |
|  | 95% | 65.834 | 66.144 | 80.629 | 116.017 | 50.436 | 49.665 | 73.107 | 105.04 | 35.646 | 35.256 | 62.373 | 90.259 |
|  | Mean | 36.233 | 36.789 | 51.779 | 83.418 | 21.250 | 21.543 | 43.376 | 69.013 | 13.661 | 13.948 | 31.723 | 50.058 |
| UL: Delay CDF [s] | 5% | 0.384 | 0.404 | 0.200 | 0.102 | 3.610 | 3.745 | 0.285 | 0.153 | 6.525 | 6.757 | 0.518 | 0.276 |
|  | 50% | 0.114 | 0.112 | 0.076 | 0.046 | 0.215 | 0.210 | 0.092 | 0.057 | 0.360 | 0.350 | 0.134 | 0.082 |
|  | 95% | 0.061 | 0.060 | 0.050 | 0.034 | 0.079 | 0.081 | 0.055 | 0.038 | 0.112 | 0.113 | 0.064 | 0.044 |
|  | Mean | 0.110 | 0.109 | 0.077 | 0.048 | 0.188 | 0.186 | 0.092 | 0.058 | 0.293 | 0.287 | 0.126 | 0.080 |
| 𝜌DL |  | 0.99 | 0.99 | 1.00 | 1.00 | 0.98 | 0.98 | 1.00 | 0.99 | 0.92 | 0.92 | 0.98 | 0.99 |
| 𝜌UL |  | 0.99 | 0.99 | 1.00 | 1.00 | 0.98 | 0.97 | 1.00 | 1.00 | 0.91 | 0.92 | 0.98 | 0.98 |
| BODL |  | 0.20 | 0.21 | 0.08 | 0.04 | 0.40 | 0.41 | 0.14 | 0.08 | 0.60 | 0.60 | 0.30 | 0.19 |
| BOUL |  | 0.24 | 0.24 | 0.09 | 0.05 | 0.46 | 0.47 | 0.16 | 0.10 | 0.66 | 0.66 | 0.34 | 0.23 |
| 𝜆 |  | 0.34Mbps |  |  |  | 0.41Mbps |  |  |  | 0.45Mbps |  |  |  |
| Company/tdoc: QCOM/R1-153429 LBT category 3 with q = 25 and 20us CCA slots and 9.5ms TXOP for both DL and UL LAA transmission and 50 UEs per operator Additional information: ED threshold: Wi-Fi: -62 dBm; LAA: -82 dBm. WiFi preamble detection threshold = 4dB SINR. RTS-CTS enabled on Wi-Fi |  |  |  |  |  |  |  |  |  |  |  |  |  |