3GPP TSG RAN WG1 Meeting #81	R1-153407

# Fukuoka, Japan, 25th – 29th May 2015

Agenda item:	6.2.5.2.3

Source: 	Samsung

Title: 	Specification impacts and evaluation results with linear-combination codebook enhancements

Document for:	Discussion and Decision

# Introduction

This contribution presents updated results for performance benefits of FD-MIMO systems with subarray-partition architecture, along with the linear-combination codebook.

# Codebook Based Upon Linear Combination

It has been demonstrated in R1-152890 that KP codebook can achieve significant gain relative to the Category 2 baseline. However, as KP codebook is designed based upon an assumption that the wireless channel for a link has a single dominant path (per layer), the performance advantage of KP can be limited for the channel with multiple paths and rays. Based on performance evaluation of the past releases, KP PMI feedback is expected to offer competitive performance for SU-MIMO.  For MU-MIMO, however, the availability of accurate CSI at the eNB is critical to achieve high MU-MIMO gain with advanced MU-MIMO precoding schemes such as SLNR; it is of a valid question whether KP PMI pre-coder feedback is sufficient for MU-MIMO precoding. To close the gap towards the ideal-CSI bound, an approach based upon linear combination (LC) of basis vectors can be considered in order to capture the effects of multiple paths per precoding layer. For example, a rank-1 PMI pre-coder can be represented as: ,

where

- is a set of L basis vectors selected out of a master set such that each basis vector  can be decomposed as a KP: , where  and  are DFT vectors of sizes N×1 and MTXRU×1, respectively, and that respectively represent azimuth and elevation channel responses for a given pair of azimuth and elevation angles;

- is the set of cross-pol co-phase values for L basis vectors selected from a co-phase codebook Cco-ph;  and

- is the set of corresponding complex coefficients selected from a coefficient codebook Ccoeff.

It can be seen that with L = 1, this codebook reduces to the KP codebook. In addition, PMI quantization will be progressively refined as L increases at the expense of more feedback overhead.

In the following analysis, we consider N = 4 columns, and L = 4 basis vectors obtained by the KP of L = 4 horizontal DFT vectors with one vertical DFT vector. The master set of all candidate basis vectors is given by:

,

where  and , respectively are the 4-Tx DFT codebook with oversampling factor OH=8 and the MTXRU-Tx DFT codebook with oversampling factor OV=4. Here is the Rel. 10 8-Tx W1 codebook; this implies that we can use 4 bits to indicate 4 horizontal DFT vectors (index i1) in the basis calculation. So, we only need additional indication for the vertical DFT vector in the basis calculation, which is 3 bits for MTXRU=2, and is 4 bits for MTXRU=4.

To quantize co-phase and coefficient , a double-stage codebook is considered in which both co-phase and coefficient codebooks have a double-codebook structure, i.e.,   and  where  and  respectively are the WB and SB co-phase codebooks in order to capture the WB and SB variations in the co-phase of the basis vectors. and  are analogously defined. We use the following co-phase and coefficient codebooks in the analysis later:  and .

The rank-2 PMI pre-coder is derived from the rank-1 pre-coder as: 12p1p1p1p1p2p2-p2p2, where p1p2 is the rank 1 LC pre-coder.

The feedback overhead comparison in terms of number of WB and SB feedback bits is shown in Table 1.

Table 1: Feedback Overhead

| MTXRU | Scheme | Number of WB feedback bits | Number of SB feedback bits |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Co-phase (bits) | Coefficient (bits) | Total (bits) |
| 2 | KP | 4+2=6 | 2 | 2+2=4 | 6 |
|  | LC | 4+3+8+8=23 | 4 | 4 | 8 |
| 4 | KP | 4+3=7 | 2 | 2+2=4 | 6 |
|  | LC | 4+4+8+8=24 | 4 | 4 | 8 |

A summary of non-full-buffer MU-MIMO system-level performance evaluation results are presented in Figure 1 through Figure 4 for UMa-500m, UMa-200m, UMi-2GHz and UMi-3.5GHz in order to demonstrate the benefits of hybrid CSI-RS scheme in the FD-MIMO system with respect to the Category 2 baseline scheme. Detailed results can be found in Appendix I. The results are provided for 1D subarray-partition architecture with MTXRU = 2, 4 and (N, P) = (4, 2); In these simulations, the full-port un-precoded CSI-RS are provided for CSI estimation, and the corresponding CSI-RS overhead is taken into account in the final throughput calculation.

Cell association antenna pattern is approximated by one-TXRU pattern, and SLNR precoding and proportional fair scheduling (max 4 layers per time-frequency resource) have been used. The rest of the simulation assumption is according to [2].

Figure 1 Summary for UMa-500m

Figure 2 Summary for UMa-200m

Figure 3 Summary for UMi-2GHz

Figure 4 Summary for UMi-3.5GHz

Proposal 1: Capture the results in Table 2 and Table 3 in the relevant section (e.g., results for non-precoded CSI-RS related enhancement) of the TR.

# Conclusion

This contribution has considered performance benefits of LC PMI codebook design, and made the following observation and proposal:

Proposal 1: Capture the results in Table 2 and Table 3 in the relevant section (e.g., results for non-precoded CSI-RS related enhancement) of the TR.

# Appendix I

Table 2: Summary of Results for UMi-2GHz and 16 TXRU

| Scenario | Lambda | Target RU | Avg UPT | 50% UPT | 5% UPT | RU |
| --- | --- | --- | --- | --- | --- | --- |
| UMa-500m | 1.6 | 20% | 35.88 | 33.12 | 12.17 | 19% |
|  | 3.2 | 50% | 25.71 | 21.64 | 7.67 | 42% |
|  | 4.4 | 70% | 21.03 | 17.87 | 8.09 | 58% |
| UMa-200m | 1.7 | 20% | 37.33 | 36.56 | 15.28 | 16% |
|  | 3.4 | 50% | 27.10 | 23.43 | 10.03 | 40% |
|  | 4.5 | 70% | 22.16 | 18.95 | 9.98 | 51% |
| UMi-2GHz | 1.8 | 20% | 37.34 | 35.53 | 13.77 | 18% |
|  | 3.4 | 50% | 27.09 | 22.59 | 9.26 | 41% |
|  | 4.5 | 70% | 23.08 | 20.57 | 9.05 | 53% |
| UMi-3.5GHz | 1.7 | 20% | 38.93 | 38.81 | 13.84 | 18% |
|  | 3.2 | 50% | 26.92 | 22.97 | 6.83 | 42% |
|  | 3.9 | 70% | 20.86 | 17.26 | 4.84 | 58% |

Table 3: Summary of Results for UMi-2GHz and 32 TXRU

| Scenario | Lambda | Target RU | Avg UPT | 50% UPT | 5% UPT | RU |
| --- | --- | --- | --- | --- | --- | --- |
| UMa-500m | 1.6 | 20% | 35.06 | 31.96 | 12.26 | 18% |
|  | 3.2 | 50% | 25.43 | 20.26 | 7.93 | 41% |
|  | 4.4 | 70% | 18.83 | 15.48 | 7.57 | 57% |
| UMa-200m | 1.8 | 20% | 36.21 | 35.27 | 14.37 | 18% |
|  | 3.5 | 50% | 26.92 | 23.14 | 10.13 | 39% |
|  | 4.7 | 70% | 20.83 | 17.33 | 8.37 | 54% |
| UMi-2GHz | 1.7 | 20% | 37.66 | 36.16 | 14.95 | 19% |
|  | 3.6 | 50% | 26.56 | 21.98 | 8.05 | 41% |
|  | 4.7 | 70% | 21.24 | 19.24 | 7.72 | 55% |
| UMi-3.5GHz | 1.8 | 20% | 39.40 | 37.88 | 14.64 | 18% |
|  | 3.3 | 50% | 27.44 | 24.10 | 7.92 | 41% |
|  | 4.2 | 70% | 20.35 | 17.32 | 4.84 | 60% |

# References

R1-150378, Samsung, “FD-MIMO performance with ideal CSI with 8 to 64 TXRUs” 3GPP TSG RAN1#80

TR36.897, Study on elevation beamforming/full-dimension MIMO for LTE.