3GPP TSG RAN WG1 Meeting #82                                                                                         R1-153765

Beijing, China, August 24-28, 2015

Agenda Item:	7.2.1.7

Source:	Huawei, HiSilicon

Title:	On MTC SIB1 transmission

Document for:	Discussion and decision

# Introduction

The following agreements were achieved about MTC SIB transmission in the last meeting [1]:

Agreements:

- The number of resource blocks used for MTC SIB transmission is fixed to 6 PRBs.

- Scheduling information for MTC SIB1

  - TBS of MTC SIB1 is based on information in the MIB.

  - Frequency location of MTC SIB1 is derived from at least PCID.

  - Time location

    - Possible subframes are {0,4,5,9} for FDD and {0,5} for TDD. FFS subframes {1,6} for TDD.

    - FFS: Whether the subframes and frames are signaled in MIB and/or fixed/predefined in specification.

- Scheduling information for MTC SIBs other than MTC SIB1 are given in MTC SIB1.

- The number of repetitions for MTC SIBs other than MTC SIB1 is configurable by the network.

  - FFS: MTC SIB1.

  - FFS: How the network will signal the number of repetitions

Agreements:

- Working assumption: At least in case the network supports enhanced coverage, frequency hopping for MTC SIB-1 is always used at least system bandwidth >= 5Mhz

  - Working assumption: The frequency location of MTC SIB-1 is determined based on subframe index (and/or SFN), cell ID and system bandwidth.

These agreements imply DCI control-less for SIB transmission, but how to derive the scheduling information is not decided yet. This contribution focuses on MTC SIB1 and proceeds to analyze it in detail taking into account the discussion and the PBCH agreements achieved in the last meeting.

# Determining MTC SIB1 time location

The time location for MTC SIB1 transmission includes how many subframes are used and which radio frames and which subframes are taken, which are affected by the coverage enhancement (CE) requirement, TDD configuration, the subframes used for PBCH repetitions and the subframes configured for MBSFN.

## The number of repetitions

SIB transmissions address all UEs in the cell, so the number of repetitions needs to take into account the UEs in the worst case or the average requirement for CE which, however, may not be the same in different cells. It is desirable that eNBs could determine which CE level will be supported in the cell, and support MTC SIB1 transmission targeting different CE levels in different cells.

Proposal 1: The network can transmit MTC SIB1 with the number of repetitions targeting different CE levels in different cells.

For an MTC SIB1 with a specific TBS, the number of repetitions for MTC SIB1 transmission can be different for diverse CE requirements to optimize resource utilization. For MTC SIB1 transmissions with different TBSs, the number of repetitions for MTC SIB1 transmission should also be different. Even for the same TBS and CE requirement, the number of repetitions a UE is required to combine also depends on whether frequency hopping is applied or not and the frequency hopping gain.

As the UE would experience better channel condition than the worst case, the UE may perform blind trials to combine MTC SIB1 repetitions to save power consumption. If the UE does not know the number of repetitions for MTC SIB1, it does not know the CE level supported by MTC SIB1 in the cell, i.e. what the worst case is. As a consequence, it will perform excessive combination of the repetitions than the actual number of repetitions eNB transmits and UEs’ power consumption will be wasted. For example, eNB transmits MTC SIB1 with 50 repetitions to supports 5dB CE in a cell and a UE needs 10dB CE. The UE cannot successfully decode MTC SIB1 with 50 repetitions and then will have another try to decode MTC SIB1 with 100 repetitions which is not the actually MTC SIB1. So the number of repetitions for MTC SIB1 should be known by MTC UEs to eliminate power consumption waste on UE decoding.

eNB can configure a period for MTC SIB1, in which MTC SIB1 is repeated and does not change. MTC UEs can know the number of repetitions for MTC SIB1 from the configured period, together with the subframe and frame usage of MTC SIB1.

Proposal 2: A period in which MTC SIB1 does not change is indicated in MIB.

## The subframes and frames

According to the agreements, there are 4 possible subframes for MTC SIB1 transmission for FDD. For TDD, subframes #0 and #5 are always downlink so that they are possible subframes for MTC SIB1 transmission. Whether subframes #1 (special subframe) and #6 (downlink or special subframe) can be used for MTC SIB1 transmission are left FFS. Because the downlink symbols in the two subframes depends on TDD configuration which can only be known after decoding MTC SIB1 if it is included in MTC SIB1. Since subframes {0, 4, 5, 9} for FDD and {0, 1, 5, 6} for TDD cannot be configured as MBSFN subframes [2], possible subframes for MTC SIB1 can only be these subframes if the MBSFN configuration is contained in MTC SIB2 as in the legacy SIB2 for normal UEs.

For TDD, if the possible subframes for MTC SIB1 are restricted to {0, 5}, the cell access time will not be balanced between FDD and TDD when operating CE because the delay for the UEs to decode MTC SIB1 will be longer for TDD than FDD. Especially for 1.4MHz system bandwidth when PBCH repetition is not supported [1], subframes for MTC SIB1 repetition are {4, 5 in odd frames, 9} for FDD and {5 in odd frames} for TDD. The delay for MTC SIB1 decoding for TDD is 5 times longer than the delay for FDD. TDD system should not be jeopardized with such restriction. When there are enough downlink symbols in subframes 1 and 6, they can also be used for MTC SIB1 transmission to reduce the delay of decoding.

Proposal 3: Subframes {1, 6} should also be possible subframes for MTC SIB1 transmission for TDD.

When a UE receives MTC SIB1 on special subframes, it does not know the specific special subframe configuration yet, so it can not know the duration of DwPTS. There are some ways to use the special subframes for MTC SIB1.

One alternative is to use the special subframes with fixed DwPTS duration assumption at the UE. However, this alternative cannot be supported in some special subframe configurations. In current TDD system as shown in Table 1 [3], special subframe configurations 0 and 5 do not provide sufficient downlink symbols in special subframes for MTC SIB1 transmission in normal CP. Similarly, special subframe configurations 0 and 4 do not provide sufficient downlink symbols in special subframes for MTC SIB1 transmission in extended CP. Fixed DwPTS duration for MTC SIB1 transmission cannot be assumed in these special subframe configurations.

Another alternative is that MIB can indicate whether special subframes can be used for MTC SIB1. For the above special subframe configurations, and possible special subframe configuration 9 for normal CP and 7 for extended CP, MIB can indicate special subframes are not be used for MTC SIB1 transmission. For other special subframe configurations in which MIB can indicate to use special subframes for MTC SIB1, UE could assume a default special subframe configuration or fixed OFDM symbols used for MTC SIB1 transmission in special subframes, for example, 9 symbols DwPTS for normal CP and 8 symbol DwPTS for extended CP. Thus, all special subframe configurations can be used for MTC.

Due to the UE cannot decide the type of subframe 6 when receiving MTC SIB1, it could also be indicated by MIB whether subframe 6 is a special subframe or downlink subframe.

Table 1 Configuration of special subframe and duration of DwPTS

| Special subframe configuration | DwPTS |  |
| --- | --- | --- |
|  | Normal cyclic prefix in downlink | Extended cyclic prefix in downlink |
| 0 | (3 symbols) | (3 symbols) |
| 1 | (9 symbols) | (8 symbols) |
| 2 | (10 symbols) | (9 symbols) |
| 3 | (11 symbols) | (10 symbols) |
| 4 | (12 symbols) | (3 symbols) |
| 5 | (3 symbols) | (8 symbols) |
| 6 | (9 symbols) | (9 symbols) |
| 7 | (10 symbols) | (5 symbols) |
| 8 | (11 symbols) | - |
| 9 | (6 symbols) | - |

Proposal 4: Whether special subframes can be used for MTC SIB1 transmission and the type of subframe 6 are indicated by MIB.

PBCH takes two subframes per frame and it is agreed the other subframe for PBCH repetition is subframe 9 for FDD and 5 for TDD [1]. If MTC SIB1 utilizes frequency hopping, it can be frequency division multiplexed with PBCH in the PBCH subframes. Subframes other than {1, 6} for MTC SIB1 transmission can be fixed in specification to avoid UE blindly decoding and save MIB spare bits usage.

In conclusion, the subframes for MTC SIB1 transmission could be:

- For 1.4MHz system bandwidth: {4, 5 in odd frames, 9} for FDD, {5 in odd frames} and possible {1,6} for TDD.

- For other system bandwidth: {0,4,5,9} for FDD, {0,5} and possible {1,6} for TDD.

The previous evaluations of SIBs for MTC UE operating CE shows that discontinuous repetition needs less repetition number than continuous repetition [4]. The frames for MTC SIB1 transmission can be discontinuous which could be fixed in specification.

# Determining the frequency location

The frequency location for MTC SIB1 is not desirable to be fixed in the carrier center, as it cannot be frequency division multiplexed with PBCH and use PBCH subframes for repetition. Some companies verified that frequency hopping reduces the required number of repetitions to operate CE in their simulations for SIBs [4]. As the number of resource blocks used for MTC SIB transmission is fixed to 6 PRBs, frequency hopping cannot be applied in 1.4MHz system. In 3MHz system, the frequency diversity gain from frequency hopping is considered to be limited. Then at least in case the network supports enhanced coverage, frequency hopping for MTC SIB1 is always used at least system bandwidth >= 5Mhz.

To make it easier to schedule the transmissions of MTC SIB1, which occur always, around the other MTC traffic and non-MTC traffic, the frequency location of MTC SIB1 transmission can be indicated by MIB. This can be better than using fixed frequency hopping pattern which cannot take account of system bandwidth and even long-term variations in cell traffic. The frequency location of MTC SIB1 transmission can be determined according to the current fields of MIB (dl-Bandwidth, phich-Config, and systemFrameNumber) and have a relation to cell ID. This can provide some flexibility and avoids using additional spare MIB bits.

Proposal 5: Confirm the following working assumptions:

- At least in case the network supports enhanced coverage, frequency hopping for MTC SIB-1 is always used at least system bandwidth >= 5Mhz

  - The frequency location of MTC SIB-1 is determined based on subframe index (and/or SFN), cell ID and system bandwidth.

# Determining the TBS

RAN2 agreed to maintain the flexibility of MTC SIB1, which implies UE shall not assume the size of the SIBs is fixed. And it was agreed in last meeting TBS of MTC SIB1 is based on information in the MIB. Whether the TBS of MTC SIB1 is explicitly indicated by MIB or implicitly determined by UEs needs further discussion.

In current specification, SIB transmission for legacy UEs only uses QPSK. As there is coverage loss arising from cost saving techniques and coverage enhancement for MTC UEs, MTC SIBs should also utilize QPSK modulation. As the number of resource blocks used for MTC SIB transmission is fixed to 6 PRBs, the number of coded bits available for MTC SIB1 transmission within one subframe is known based on QPSK and 6PRBs. Due to MTC SIB1 should be repeatedly transmitted across multiple subframes, the total coded bits number available for MTC SIB1 transmission actually depends on the number of repetitions for MTC SIBs transmission.

To get 1% BLER detection performance of MTC SIB1, a certain effective code rate for the muti-subframe transmission (reflected by MCS) is required for MTC SIBs transmission which can be determined by the supported CE level for MIC SIB1 of a cell and the frequency hopping gain. The supported CE level is reflected by the range that the repetition number belongs and frequency hopping gain is linked to MIB (related to system bandwidth). When the UE has acquired MIB, it will get the knowledge of the effective code rate required as long as MIB provides additional repetition number information for MTC SIB1 discussed above. As the code rate equals to TBS divided by the total coded bits available for MTC SIB1 transmission, the TBS of MTC SIB1 can be derived implicitly from the information in MIB and does not need to be explicitly indicated to avoid the use of much spare bits for some redundant information.

Proposal 6: The TBS for MTC SIB1 is derived implicitly from the information in MIB, e.g. the period of MTC SIB1.

# Conclusions

This contribution analyzes the time location, frequency location, as well as the TBS for MTC SIB1 transmission, which leads to the following proposals:

Proposal 1: The network can transmit MTC SIB1 with the number of repetitions targeting different CE levels in different cells.

Proposal 2: A period in which MTC SIB1 does not change is indicated in MIB.

Proposal 3: Subframes {1, 6} should also be possible subframes for MTC SIB1 transmission for TDD.

Proposal 4: Whether special subframes can be used for MTC SIB1 transmission and the type of subframe 6 are indicated by MIB.

Proposal 5: Confirm the following working assumptions:

- At least in case the network supports enhanced coverage, frequency hopping for MTC SIB-1 is always used at least system bandwidth >= 5Mhz

  - The frequency location of MTC SIB-1 is determined based on subframe index (and/or SFN), cell ID and system bandwidth.

Proposal 6: The TBS for MTC SIB1 is derived implicitly from the information in MIB, e.g. the period of MTC SIB1.

# References

- 3GPP RAN1 #81 meeting, Chairman notes, Fukuoka, Japan, 25 – 29 May, 2015.

- TS 36.331, “(E-UTRA); Radio Resource Control (RRC); Protocol specifications (Release 12),” V12.5.0 (2015-03).

- TS 36.211, “(E-UTRA); Physical channels and modulation; Protocol specifications (Release 12),” V12.5.0 (2015-03).

- R1-150873, “LS on SIB performance for Rel-13 coverage enhanced UE for MTC”, RAN1 #80, Athens, Greece, 9th – 13th February 2015