3GPP TSG-RAN WG1 #82 	R1-153885

24th – 28th August 2015

Beijing, China

Agenda item:	7.2.5.3.1

Source: 	Qualcomm Incorporated

Title: 	Codebook design for 12TX and 16TX with 2D cross-polarized array

Document for:	Discussion/Decision

# Introduction

In RAN1#81, the study of performance and feasibility of elevation beamforming and full-dimensional (FD) MIMO [1] was completed.  It has been concluded that [2]

- Non-precoded, beamformed and hybrid CSI-RS based schemes demonstrate significant throughput gain in realistic non-full buffer traffic models over the best baseline using implementation based enhancements in many scenarios

  - The best choice between these schemes may depend on factors including the number of TXRUs

- Non-codebook based CSI reporting is beneficial for EB/FD-MIMO compared to the best baseline using implementation based enhancements.

- SRS enhancement is beneficial for EB/FD-MIMO compared to the best baseline using implementation based enhancements.

- From the performance perspective, DMRS enhancements are beneficial for EB/FD-MIMO.

As a follow-up, a new work item [3] has been approved in RAN 68 for Release 13.  The work item aims to specify the enhancements identified for utilizing both elevation and azimuth domains with 2D antenna array with cross-poles at eNBs.  The detailed objectives include

- Specify enhancements on CSI reporting in the following areas [RAN1]

  - For non-precoded CSI-RS, codebook for 2D antenna arrays for support of {8,12,16} CSI-RS ports and associated necessary channel state information.

    - If there is not significant gain shown for new codebook for 8 CSI-RS ports, the current codebook for 8 CSI-RS ports is retained.

  - Necessary channel state information for beamformed CSI-RS

  - Extension of Rel-12 CSI reporting mechanism for both periodic and aperiodic CSI reports

In this contribution, we discuss codebook design for 2D antenna arrays for support of {12, 16} CSI-RS ports.

# Codebook design for support of {12, 16} CSI-RS ports

## General framework

A CSI reporting scheme is associated with at least two components: codebook(s) for the purpose of PMI reporting of 2-D antenna arrays and its associated CSI reporting modes.  The dual-stage precoding structure introduced since Rel-10 has been proven to provide trade-off between the feedback overhead and the precoding resolution.  It can be extended to construct codebooks for 2D antenna array with more than 8 CSI-RS ports.  The precoding matrix in the codebook can be described as W=W1W2, where W is used as a downlink transmission hypothesis for CSI calculation at a UE.  As summarized in [2], a potential specification enhancement on CSI reporting consists of the following CSI parameters:

-	PMI(s) corresponding to W1 and/or W2. Here one or multiple PMIs, such as H-PMI (horizontal dimension) and V-PMI (vertical dimension), are reported for W1 and W2, respectively. If multiple PMIs are reported, different reporting rates and/or granularities for different PMIs may or may not be used, and each of these PMIs can be reported either periodically or aperiodically.

-	RI: a single RI or multiple RIs

-	CQI

Both feedback overhead and UE complexity should be considered when designing a new codebook.  It is unavoidable to increase the codebook size as the number of CSI-RS ports increases.  In general, increase number hypotheses of W2 should be justified by sufficient performance gain, as the PMI reporting related to W2 could be a short-term-wise and/or subband-wise.  A small increase in short-term/subband reporting may results in significant impact on the feedback overhead as well as the UE complexity for PMI selection.

In this contribution, we consider the KP type codebook design for (M, N, 2) antenna prot array by assuming k=k' and l=l'.  We assume 1D antenna port virtualization as described in [5]. The first matrix W1 can be written as

W1=XHk⊗XVlXHk⊗XVl

XHk defines kth group of horizontal beams which are associated with a subset of quantized horizontal directions.  Similarly, XVl is the lth groups of vertical beams which are associated with a subset of quantize vertical directions.  Each XHk contains NBH beams and each XVl contains NBV beams,

XHk=vkvk+ΔH⋯vk+NBH⋅ΔH,

XVl=ulul+ΔV⋯ul+NBV⋅ΔV,

where vk=1ej2πLHNk⋯ej2πLHNN-1kT is N-element DFT vector with oversampling factor (OVSF) of LH; ul=1ej2πLVMl⋯ej2πLVMM-1lTis M-element DFT vector with OVSF of LV; k and l are the indices of the first DFT beam in the group; ΔH and ΔV defines the interval between two adjacent beams.  Given (M, N), the design parameters {NBH,NBV,ΔH,ΔV} are fixed and depending on the number of transmission layers.  Two PMIs, i1V and i1H are used for indexing W1.  There are two options for the association between k,l and i1V,i1H given a rank hypothesis.

- Option-1	k is solely depending on i1V, while l is solely depending on i1H, or

- Option-2 	At least either k or l is depending on both i1V and i1H

Option-1 allows all possible pairing of k,l, while Option-2 can be employed to get a subset of Option-1.  In our design, Option-2 is used to tailor some overlapped horizontal/vertical directions.

The second matrix W2 performs distinct selection of beams per layer and quantized co-phasing between two polarization groups.  For rank-r, the second matrix W2 can be written as

W2=y1i2⋯yri2ϕ1i2y1i2⋯ϕri2yri2

where yki2 is selected from columns of the identity matrix INBHNBV; ϕki2 is the co-pahsing between two polarization groups. The second matrix W2 is index by the PMI i2.  In order to keep the subband feedback at current level, we further assume there are up to 16 hypotheses of the second matrix.

## Codebook Parameters

The design parameters of the scalable codebook are summarized in Table 1 for rank-1, 2, 3, and 4.

Table 1: Codebook Parameters

| Rank | LH | NBH | ΔH | LV | NBV | ΔV |
| --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | 8 | 4 | 1 | 4 | 1 or 2 | 1 |
| 3, 4 | 4 | 8 | 1 | 4 | 1 or 2 | 1 |

For the association between (k,l) and (i1V,i1H), we use l=i1V and k=NBHi1H+LHN/2i1V to tailors some overlapped beam grids.  Note that for each rank, NBV can be either 1 or 2.  UE may choose one value depending on the rank of the its channel in vertical.  In order to indicate the eNB with the value of  NBV, a UE needs to feedback a one bit indicator to the eNB.  The eNB may also configure the UE to use one out of the two possible values via higher-layer signaling.

For the second matrix, we can reuse the design of Rel-10 8-TX second matrix for  NBV=1.  For NBV=2, there are 8 beams in each beam group.  For rank-1, we can subsample 4 beams out of the 8 beams and then reuse the design of Rel-10 8-TX second matrix again, as shown in Figure 1.

Figure 1: Subsampling pattern for rank-1 when NBV=2

For rank-2, we use following W2 hypotheses ekekϕek-ϕek:k=1,2,…,8; ϕ=1,j.  For rank-3 and rank-4, we subsample 8 beams out of the 16 beams and the reuse the design of Rel-10 8-TX second matrix again, as shown in Figure 2.

Figure 2: Subsampling pattern for rank-3 and rank-4 when NBV=2

The feedback overhead can be derived by these parameters.  In Table 2, we summarized the overhead of the codebook for 16 CSI-RS ports aligned in a (2, 4, 2) array as an example.

Table 2: Feedback overhead for the (2, 4, 2) antenna port configuration

| Rank | i1V | i1H | i2 |
| --- | --- | --- | --- |
| 1, 2 | 0 – 7 (3-bit) | 0 – 7 (3-bit) | 0 – 15 (4-bit) |
| 3 | 0 – 7 (3-bit) | 0 – 3 (2-bit) | 0 – 15 (4-bit) |
| 4 | 0 – 7 (3-bit) | 0 – 3 (2-bit) | 0 – 7 (3-bit) |

# Simulation Results

To evaluation the performance benefit of the proposed codebook design, system level simulation is performed.  We compare the MU-MIMO performance of the proposed codebook and the KP based codebook.  In the KP based codebook, XVl contains only one vector.  Up to 4 UEs can be co-scheduled while each UE may be scheduled with 1 layer in MU operation and up to 2 layer in SU-MIMO.  As shown in Table 3, the proposed codebook provides about 22% gain over the Rel-10 8-TX configuration for cell center UEs.  A 6% gain over the KP coebook for cell center UEs can be observed with the same feedback overhead.

| Scheme | Cell-edge UE SE Gain | Average UE SE Gain |
| --- | --- | --- |
| (1, 4, 2) w/ Rel-10 codebook | 100% | 100% |
| (2, 4, 2) w/ KP codebook | 41.6% | 15.3% |
| (2, 4, 2) w/ proposed codebook | 43.1% | 21.9% |

# Conclusions

In summary, we discuss a scalable codebook design framework for 2D antenna array for support of 12 and 16 CSI-RS ports.  The simulation results show 6% gain for cell center UEs over the KP codebook under the same feedback overhead.  We propose consider introducing the proposed scalable codebook framework for 2D antenna array to support 12 and 16 CSI-RS ports.

# References

- RP-141831, Revised SID: Study on Elevation Beamforming/Full-Dimensional (FD) MIMO for LTE, Samsung and Nokia Networks.

- 3GPP TR 36.897, Study on Elevation Beamforming/Full-Dimensional (FD) Multiple Input Multiple Output (MIMO) for LTE.

- RP-151085, WID: Elevation Beamforming/Full-Dimensional (FD) MIMO, Samsung.

- 3GPP TS 36.213, Evolved Universal Terrestrial Radio Access (E-UTRA); Physical Layer Procedures (Release 10).

- R1-153883, Views on CSI Feedback Enhancements for Elevation Beamforming and FD-MIMO, Qualcomm.