3GPP TSG RAN WG1 Meeting #82 	                                  R1-154397

Beijing, China, 24th – 28th August 2015

Source: 	ETRI

Title: 	Codebook proposal for 2D cross-polarized ULA

Agenda Item:	7.2.5.3.1 2D codebook design

Document for:	Discussion/Decision

# Introduction

In the WID of EBF/FD-MIMO, specification enhancements on CSI reporting was included [1]:

  - For non-precoded CSI-RS, codebook for 2D antenna arrays for support of {8,12,16} CSI-RS ports and associated necessary channel state information.

    - If there is not significant gain shown for new codebook for 8 CSI-RS ports, the current codebook for 8 CSI-RS ports is retained.

It was agreed that codebook for the purpose of PMI reporting of 2D antenna arrays has dual-stage precoding structure [2]. Also the flexibility of antenna configuration was addressed [3].

In this contribution, we propose a flexible codebook design.

# Discussion

Precoding matrix of dual codebook can be represented as multiplication of wideband precoding W1(i1) and subband precoding W2(i2), where indices i1 and i2 denotes wideband/subband PMI respectively.

Wideband precoding W1(i1) implies 2D grid-of-beams.

W1i1=XH⊗XVXH⊗XV

Each row corresponds to a TXU, and the number of rows in submatrices XH and XV equal to NTXRU and MTXRU respectively. Column vectors of the submatrices can be taken from Nr×Nc DFT matrix Bw;Nr,Nc,Fc,Fw whose element is given as

br,c=1Nrexpj2πrcFc+wFwNr , 0≤r<Nr, 0≤c<Nc

Subband precoding W2(i2) performs beam selection and co-phasing. Column vectors are taken from the identity matrix IN.

IN=e0Ne1N⋯eN-1N

In principle, given RI, wideband precoding can be determined in component-wise manner, i.e. XH and XV. Beam selections of subband precoding should be performed on 2D grid-of-beam jointly.

For rank 1-2 wideband precoding, legacy dual stage codebook designs specify normalized beam spacing Fc,Fw=0.5,0.0625, (0.125,0.25) for Nr,Nc=2,4,(4,4) respectively. For array size Nr other than 2 and 4, spacing parameters in Table 1 can be considered.

XH=Bi1,H;NTXRU,4,Fc,Fw,0≤i1,H<16

XV=Bi1,V;MTXRU,4,Fc,Fw,0≤i1,V<16

i1=16i1,V+i1,H

Table 1 Beam spacing parameters for rank 1-2

| Nr,Nc | (2, 4) | (3, 4) | (4, 4) | (6, 4) | (8, 4) |
| --- | --- | --- | --- | --- | --- |
| Fc,Fw | (0.5, 0.0625) | (0.75, 0.09375) | (0.125, 0.25) | (0.1875, 0.375) | (0.25, 0.5) |

Subband precoding can be extended from the legacy codebook as follows, since transmission layers are multiplexed in polarization domain.

W2i2=12ek4⊗ek'4el4⊗el'4αek4⊗ek'4-αel4⊗el'4P

For rank 1,

P=e02,α=expj2πi2,P4,k=i2,H,k'=i2,V

0≤i2,P,i2,H,i2,V<4 and i2=16i2,V+4i2,H+i2,P

For rank 2, the beam selection pattern of legacy codebook can be reused as a baseline and it was rewritten in Table 2 for completeness.

P=12e02e12,α=expj2πi2,P4

0≤i2,P<2,0≤i2,H,i2,V<8 and i2=16i2,V+2i2,H+i2,P

Table 2 Beam selection pattern for rank 2

| i2,H or i2,V | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| k,l or k',l' | (0, 0) | (1, 1) | (2, 2) | (3, 3) | (0, 1) | (1, 2) | (0, 3) | (1, 3) |

2D codebook designs are more involved for higher rank cases than low rank cases. And prioritization may be needed considering tight time budget.

# Conclusions

In this contribution, a 2D codebook design has been proposed for rank 1-2.

# References

[1] RP-151085, “New WID Proposal: Elevation Beamforming/Full-Dimension (FD) MIMO for LTE”, Samsung.

[2] TR 36.897 v13.0.0, “Study on elevation beamforming/FD-MIMO for LTE”

[3] R1-153586, “Motivation for flexible framework for non-precoded CSI-RS”, Ericsson