3GPP TSG-RAN WG1 Meeting #82	R1-154464

Beijing, China, 24th – 28th August 2015

Agenda Item:	7.2.4.1

Source:	InterDigital Communications

Title:	Coexistence evaluation results for Wi-Fi and UL+DL LAA

Document for:	Discussion

# Introduction

In this contribution we present and discuss additional evaluation results for the Wi-Fi and UL+DL LAA coexistence case with mixed FTP and VoIP traffic. We use LBT Cat 4 for the LAA DL and LBT Cat 2 for the LAA UL.

# Evaluation methodology and assumptions

Our evaluation results are for the Indoor scenario using Y=1 (a single 5GHz channel). Both 50% DL / 50% UL traffic ratio and DL-heavy 80% DL / 20% UL traffic ratios are evaluated. We show mixed traffic results with FTP and VoIP.

The LAA eNB employs LBT Cat 4 for DL channel access. In the LAA UL, UEs which are dynamically scheduled by the eNB will employ LBT Cat 2 prior to the start of their first UL subframe in a reserved UL fixed frame period (FFP).

We show LAA UL performance and coexistence impacts onto Wi-Fi when using either short 24 us or long 48 us CCA durations with LAA UL channel access and the evaluated UL LBT Cat 2 scheme. In addition, we show performance for both UL CCA durations when operating in presence and absence of RTS/CTS.

DTX is used in the LAA DL and the UL to limit the maximum possible transmission duration for LAA transmitters. No carrier selection mechanism is employed by the LAA eNB.

Details of our evaluation assumptions, a description of the LAA channel access procedure used in our evaluation and the detailed parameterization for DL LBT Cat 4 and UL LBT Cat2 are shown in Appendix B.

# Coexistence evaluation results for Wi-Fi and UL+DL LAA

For the 2 coexistence evaluation cases, Wi-Fi/Wi-Fi and Wi-Fi/LAA, our UL+DL results for the Indoor Scenario are summarized in Appendix A.

Table A.1-A.8 in Appendix A contain our evaluation results for Wi-Fi/Wi-Fi and Wi-Fi/LAA. Separate tables are provided for short (24 us) and long (48 us) CCA durations with UL LBT Cat 2, for symmetric 50% DL / 50% UL and DL heavy 80% DL / 20% UL traffic ratios and when operating presence or absence of RTS/CTS.

Figures 1-4 summarize our findings from these evaluation results. Each figure shows UL LBT Cat 2 performance when using either 24 or 48 us CCA durations. Figures 1-2 show results in absence of RTS/CTS for 50%DL / 50% UL and 80% DL / 20% UL traffic ratios respectively. Figures 3-4 show the corresponding set of results in presence of RTS/CTS.

From these evaluation results, it can be observed that when considering FTP traffic, Wi-Fi does better with LAA as a neighbor as opposed to Wi-Fi as a neighbor across all load points. This holds true for both symmetric as well as asymmetric traffic cases and in absence or presence of RTS/CTS.

For Wi-Fi/LAA, the FTP throughput in DL and UL improves when compared to the Wi-Fi/Wi-Fi coexistence case, whereas the overall VoIP outage is no worse than in the Wi-Fi/Wi-Fi coexistence case. A slight improvement in DL and UL Wi-Fi FTP throughput can be observed for the case where the LAA UL LBT Cat 2 uses the longer 48 us CCA duration when compared to the shorter 24 us CCA duration.

Our evaluation results in Figure 1 and 2 without RTS/CTS show that the LAA UL LBT Cat 2 CCA duration can have a an impact onto Wi-Fi VoIP UE outage with both symmetric and asymmetric traffic.

For the example of 50% DL/50% UL traffic in Figure 1, at high load the short LAA UL LBT Cat 2 CCA duration of 24 us yields slightly worse VoIP outage ratios even though better than in the Wi-Fi/Wi-Fi coexistence case when compared to the case where the UL CCA duration is chosen to be 48 us. Similar behavior can be observed in Figure 2 for 80% DL/20% UL traffic, at medium and high load points, a CCA duration of 48 us would result in more favorable VoIP outage ratios than when using 24 us CCA. It can also be seen in Figure 4 for 80% DL/20% UL traffic with RTS/CTS, that a short LAA UL LBT Cat 2 CCA duration of 24 us would produce VoIP outage, whereas a CCA duration of 48 us or use of RTS/CTS would not produce any VoIP outage.

We note that these evaluation results were obtained under the deliberate assumption that no particular optimization is implemented on the LTE LAA side, i.e. we use a limited and only semi-statically configured TDD frame configuration #3 and there are no DL LBT Cat 4 freeze periods. In consequence, these results should be seen as pessimistic and serve as a lower bound onto achievable system performance for Wi-Fi when operating in presence of the LAA DL and UL.

Figure 1: Wi-Fi/LAA DL+UL; FTP and VoIP; 50%DL/50%UL traffic; no RTS/CTS

Figure 2: Wi-Fi/LAA DL+UL; FTP and VoIP; 80%DL/20%UL traffic; no RTS/CTS

Figure 3: Wi-Fi/LAA DL+UL; FTP and VoIP; 50%DL/50%UL traffic; with RTS/CTS

Figure 4: Wi-Fi/LAA DL+UL; FTP and VoIP; 80%DL/20%UL traffic; with RTS/CTS

# Conclusion

In this contribution we present and discuss additional evaluation results for the Wi-Fi and UL+DL LAA coexistence case with mixed FTP and VoIP traffic for 50% DL/50% UL and 80% DL/20% UL traffic ratios in the Y=1 Indoor Scenario.

We use LBT Cat 4 for the LAA DL and LBT Cat 2 with CCA periods of either 24 or 48 us duration for the LAA UL. In addition, we showed performance for both UL CCA durations when operating in presence and absence of RTS/CTS.

From these evaluation results, it can be observed that when considering FTP traffic, Wi-Fi does better with LAA as a neighbor when using DL LBT Cat 4 and UL LBT Cat 2 as opposed to Wi-Fi as a neighbor across all load points. This holds true for both symmetric as well as asymmetric traffic cases and in absence or presence of RTS/CTS. A slight improvement in DL and UL Wi-Fi FTP throughput can be observed for the case where the LAA UL LBT Cat 2 uses the longer 48 us CCA duration when compared to the shorter 24 us CCA duration.

We also observe that in absence of RTS/CTS, the LAA UL LBT Cat 2 CCA duration can have an impact onto Wi-Fi VoIP UE outage with both symmetric and asymmetric traffic.

For 50% DL/50% UL traffic at high load the short LAA UL LBT Cat 2 CCA duration of 24 us yields slightly worse VoIP outage ratios even though better than in the Wi-Fi/Wi-Fi coexistence case than when compared to the case of an UL CCA duration of 48 us. Similar behavior is observed for 80% DL/20% UL traffic at medium and high load points where a CCA duration of 48 us would result in more favorable VoIP outage ratios than when using 24 us CCA. We also show that for 80% DL/20% UL traffic with RTS/CTS, a short LAA UL LBT Cat 2 CCA duration of 24 us would produce VoIP outage, whereas a CCA duration of 48 us or use of RTS/CTS would not produce any VoIP outage.

We note that these evaluation results were obtained under the deliberate assumption that no particular optimization is implemented on the LTE LAA side, i.e. we use a limited and only semi-statically configured TDD frame configuration #3 and there are no DL LBT Cat 4 freeze periods. In consequence, these results should be seen as pessimistic and serve as a lower bound onto achievable system performance for Wi-Fi when operating in presence of the LAA DL and UL.

# References

R1-152933 Coexistence evaluation results for Wi-Fi and UL+DL LAA; InterDigital

R1-152932 LBT Cat 4 evaluation results for DL-only LAA with WiFi and VoIP; InterDigital

R1-152161 Evaluation results for DL-only LAA with WiFi and VoIP; InterDigital

R1-151458 Coexistence evaluation results for DL+UL LAA and WiFi; InterDigital

R1-151066 Coexistence evaluation results for LTE LAA DL-only and Wi-Fi; InterDigital

R1-151068 Design considerations on LTE LAA DL and UL operation; InterDigital

# Appendix A – Wi-Fi and LAA UL+DL evaluation results

Table A.1: Wi-Fi/LAA UL+DL; Indoor Deployment; Y=1; Mixed traffic with 50% DL/50% UL;
DL LAA LBT: Cat 4, UL LAA LBT Cat 2: 24 µs CCA; Wi-Fi RTS/CTS disabled

| Reported parameters |  | Low load BO range for Wi-Fi in Step 1: 10%~25% |  |  |  |  | Medium load BO range for Wi-Fi in Step 1: 35%~50% |  |  |  | High load BO range for Wi-Fi in Step 1: above 55% |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Wi-Fi in step 1 (Op. A) |  | Wi-Fi in step 1 (Op. B) | Wi-Fi in step 2 (Op. A) | LAA in step 2 (Op. B) | Wi-Fi in step 1 (Op. A) | Wi-Fi in step 1 (Op. B) | Wi-Fi in step 2 (Op. A) | LAA in step 2 (Op. B) | Wi-Fi in step 1 (Op. A) | Wi-Fi in step 1 (Op. B) |  | Wi-Fi in step 2 (Op. A) | LAA in step 2 (Op. B) |
| DL: UPT CDF [Mbps] | 5% | 23.376 |  | 22.575 | 26.552 | 18.809 | 5.367 | 7.880 | 12.122 | 5.872 | 1.287 | 4.076 |  | 8.291 | 3.229 |
|  | 50% | 81.023 |  | 80.921 | 92.397 | 72.710 | 34.382 | 31.382 | 53.544 | 32.293 | 13.350 | 14.385 |  | 31.071 | 16.2326 |
|  | 95% | 120.277 |  | 120.440 | 120.447 | 108.043 | 98.990 | 106.385 | 108.508 | 99.302 | 72.060 | 70.073 |  | 94.407 | 62.552 |
|  | Mean | 78.692 |  | 80.100 | 84.409 | 68.734 | 41.298 | 43.603 | 59.259 | 41.427 | 22.149 | 23.967 |  | 42.645 | 23.703 |
| DL: Delay CDF [s] | 5% | 0.035 |  | 0.035 | 0.035 | 0.039 | 0.046 | 0.040 | 0.040 | 0.041 | 0.071 | 0.059 |  | 0.044 | 0.084 |
|  | 50% | 0.055 |  | 0.053 | 0.047 | 0.061 | 0.168 | 0.160 | 0.095 | 0.154 | 0.4738 | 0.469 |  | 0.183 | 0.788 |
|  | 95% | 0.162 |  | 0.157 | 0.162 | 0.223 | 1.380 | 0.545 | 0.498 | 0.969 | 3.130 | 2.541 |  | 1.027 | 2.812 |
|  | Mean | 0.074 |  | 0.071 | 0.068 | 0.089 | 0.363 | 0.216 | 0.149 | 0.303 | 0.928 | 0.808 |  | 0.304 | 1.024 |
| UL: UPT CDF [Mbps] | 5% | 14.635 |  | 10.408 | 21.255 | 5.246 | 7.624 | 5.653 | 14.006 | 2.463 | 3.967 | 3.181 |  | 8.210 | 0.661 |
|  | 50% | 38.857 |  | 41.870 | 42.328 | 14.094 | 22.398 | 26.484 | 33.309 | 7.918 | 14.071 | 16.365 |  | 24.116 | 4.570 |
|  | 95% | 59.256 |  | 59.248 | 59.254 | 24.531 | 56.403 | 56.086 | 57.862 | 19.553 | 47.047 | 49.131 |  | 51.650 | 14.695 |
|  | Mean | 40.587 |  | 41.343 | 42.849 | 14.909 | 26.641 | 28.978 | 35.034 | 9.302 | 18.895 | 20.577 |  | 27.353 | 5.895 |
| UL: Delay CDF [s] | 5% | 0.071 |  | 0.071 | 0.071 | 0.165 | 0.073 | 0.076 | 0.072 | 0.170 | 0.089 | 0.086 |  | 0.078 | 0.186 |
|  | 50% | 0.104 |  | 0.098 | 0.098 | 0.290 | 0.186 | 0.168 | 0.126 | 0.697 | 0.308 | 0.281 |  | 0.178 | 1.455 |
|  | 95% | 0.2251 |  | 0.216 | 0.180 | 0.567 | 0.552 | 0.761 | 0.332 | 2.196 | 1.198 | 1.700 |  | 0.618 | 3.800 |
|  | Mean | 0.119 |  | 0.128 | 0.108 | 0.329 | 0.248 | 0.262 | 0.158 | 0.943 | 0.422 | 0.516 |  | 0.240 | 1.673 |
| VoIP outage (DL) |  | 0.0 |  | - | 0.0 | - | 31.25 | - | 0.0 | - | 87.5 | - |  | 37.5 | - |
| VoIP outage (UL) |  | 0.0 |  | - | 0.0 | - | 37.5 | - | 0.0 | - | 62.5 | - |  | 18.75 | - |
| VoIP outage |  | 0.0 |  | - | 0.0 | - | 43.75 | - | 0.0 | - | 87.5 | - |  | 43.75 | - |
| 𝜌DL |  | 0.999 |  | 0.999 | 1.00 | 1.00 | 0.990 | 0.990 | 0.999 | 0.994 | 0.957 | 0.957 |  | 0.983 | 0.952 |
| 𝜌UL |  | 0.995 |  | 0.987 | 1.00 | 0.982 | 0.989 | 0.989 | 0.997 | 0.952 | 0.981 | 0.983 |  | 0.986 | 0.869 |
| BO |  | 14.98 |  | 13.00 | 14.05 | 19.512 | 39.01 | 37.00 | 27.758 | 53.14 | 65.37 | 59.08 |  | 44.50 | 75.252 |
| 𝜆 |  |  | 0.15 |  |  |  |  | 0.225 |  |  |  |  | 0.275 |  |  |
| Additional comments: UEs scheduled for the LAA UL contend for the channel using LBT Cat 2 with a FFP of 10ms. LTE uses TDD UL/DL configuration #3 [DSUUUDDDDD]. UL CCA lasts 24 us and is performed prior to start of SF2. Only one CCA per FFP is done per UE. Scheduled UEs can transmit a single burst of up to 3 SFs. No dynamic reassignment of unused UL SFs is done. The DL uses DL LBT Cat 4 (UE NACK based with defer period) with details described in R1-152932. LAA uses a sensing threshold of -82 dBm. LAA slot size is the same as used for Wi-Fi: 8 us. RTS/CTS is disabled for WiFi. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2: Wi-Fi/LAA UL+DL; Indoor Deployment; Y=1; Mixed traffic with 50% DL/50% UL;
DL LAA LBT: Cat 4, UL LAA LBT Cat 2: 48 µs CCA; Wi-Fi RTS/CTS disabled

| Reported parameters |  | Low load BO range for Wi-Fi in Step 1: 10%~25% |  |  |  |  | Medium load BO range for Wi-Fi in Step 1: 35%~50% |  |  |  | High load BO range for Wi-Fi in Step 1: above 55% |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Wi-Fi in step 1 (Op. A) |  | Wi-Fi in step 1 (Op. B) | Wi-Fi in step 2 (Op. A) | LAA in step 2 (Op. B) | Wi-Fi in step 1 (Op. A) | Wi-Fi in step 1 (Op. B) | Wi-Fi in step 2 (Op. A) | LAA in step 2 (Op. B) | Wi-Fi in step 1 (Op. A) | Wi-Fi in step 1 (Op. B) |  | Wi-Fi in step 2 (Op. A) | LAA in step 2 (Op. B) |
| DL: UPT CDF [Mbps] | 5% | 23.376 |  | 22.575 | 27.794 | 19.865 | 5.367 | 7.880 | 10.435 | 5.979 | 1.287 | 4.076 |  | 9.051 | 3.958 |
|  | 50% | 81.023 |  | 80.921 | 91.826 | 76.733 | 34.382 | 31.382 | 53.376 | 31.464 | 13.350 | 14.385 |  | 31.466 | 19.495 |
|  | 95% | 120.277 |  | 120.440 | 120.388 | 107.356 | 98.990 | 106.385 | 115.498 | 93.521 | 72.060 | 70.073 |  | 96.862 | 64.135 |
|  | Mean | 78.692 |  | 80.100 | 84.919 | 71.453 | 41.298 | 43.603 | 59.062 | 40.207 | 22.149 | 23.967 |  | 44.661 | 25.383 |
| DL: Delay CDF [s] | 5% | 0.035 |  | 0.035 | 0.035 | 0.039 | 0.046 | 0.040 | 0.037 | 0.043 | 0.071 | 0.059 |  | 0.042 | 0.102 |
|  | 50% | 0.055 |  | 0.053 | 0.048 | 0.058 | 0.168 | 0.160 | 0.098 | 0.173 | 0.4738 | 0.469 |  | 0.187 | 0.789 |
|  | 95% | 0.162 |  | 0.157 | 0.130 | 0.184 | 1.380 | 0.545 | 0.717 | 0.991 | 3.130 | 2.541 |  | 0.954 | 2.685 |
|  | Mean | 0.074 |  | 0.071 | 0.065 | 0.082 | 0.363 | 0.216 | 0.178 | 0.323 | 0.928 | 0.808 |  | 0.290 | 1.039 |
| UL: UPT CDF [Mbps] | 5% | 14.635 |  | 10.408 | 21.602 | 5.125 | 7.624 | 5.653 | 12.700 | 2.554 | 3.967 | 3.181 |  | 9.498 | 1.259 |
|  | 50% | 38.857 |  | 41.870 | 43.026 | 14.330 | 22.398 | 26.484 | 32.467 | 7.923 | 14.071 | 16.365 |  | 24.490 | 4.991 |
|  | 95% | 59.256 |  | 59.248 | 59.276 | 24.515 | 56.403 | 56.086 | 57.530 | 19.312 | 47.047 | 49.131 |  | 51.836 | 14.970 |
|  | Mean | 40.587 |  | 41.343 | 43.425 | 14.932 | 26.641 | 28.978 | 34.533 | 9.060 | 18.895 | 20.577 |  | 27.654 | 6.121 |
| UL: Delay CDF [s] | 5% | 0.071 |  | 0.071 | 0.071 | 0.165 | 0.073 | 0.076 | 0.072 | 0.170 | 0.089 | 0.086 |  | 0.077 | 0.185 |
|  | 50% | 0.104 |  | 0.098 | 0.096 | 0.292 | 0.186 | 0.168 | 0.129 | 0.808 | 0.308 | 0.281 |  | 0.176 | 1.373 |
|  | 95% | 0.2251 |  | 0.216 | 0.171 | 0.580 | 0.552 | 0.761 | 0.364 | 2.374 | 1.198 | 1.700 |  | 0.621 | 3.638 |
|  | Mean | 0.119 |  | 0.128 | 0.106 | 0.331 | 0.248 | 0.262 | 0.173 | 1.038 | 0.422 | 0.516 |  | 0.235 | 1.669 |
| VoIP outage (DL) |  | 0.0 |  | - | - | - | 31.25 | - | 0.0 | - | 87.5 | - |  | 25.0 | - |
| VoIP outage (UL) |  | 0.0 |  | - | - | - | 37.5 | - | 0.0 | - | 62.5 | - |  | 18.75 | - |
| VoIP outage |  | 0.0 |  | - | - | - | 43.75 | - | 0.0 | - | 87.5 | - |  | 31.25 | - |
| 𝜌DL |  | 0.999 |  | 0.999 | 1.00 | 1.00 | 0.990 | 0.990 | 0.996 | 0.992 | 0.957 | 0.957 |  | 0.987 | 0.949 |
| 𝜌UL |  | 0.995 |  | 0.987 | 1.00 | 0.978 | 0.989 | 0.989 | 0.996 | 0.940 | 0.981 | 0.983 |  | 0.988 | 0.887 |
| BO |  | 14.98 |  | 13.00 | 13.82 | 19.08 | 39.01 | 37.00 | 29.16 | 53.51 | 65.37 | 59.08 |  | 43.72 | 73.748 |
| 𝜆 |  |  | 0.15 |  |  |  |  | 0.225 |  |  |  |  | 0.275 |  |  |
| Additional comments: UEs scheduled for the LAA UL contend for the channel using LBT Cat 2 with a FFP of 10ms. LTE uses TDD UL/DL configuration #3 [DSUUUDDDDD]. UL CCA lasts 48 us and is performed prior to start of SF2. Only one CCA per FFP is done per UE. Scheduled UEs can transmit a single burst of up to 3 SFs. No dynamic reassignment of unused UL SFs is done. The DL uses DL LBT Cat 4 (UE NACK based with defer period) with details described in R1-152932. LAA uses a sensing threshold of -82 dBm. LAA slot size is the same as used for Wi-Fi: 8 us. RTS/CTS is disabled for WiFi. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.3: Wi-Fi/LAA UL+DL; Indoor Deployment; Y=1; Mixed traffic with 80% DL/20% UL;
DL LAA LBT: Cat 4, UL LAA LBT Cat 2: 24 µs CCA; Wi-Fi RTS/CTS disabled

| Reported parameters |  | Low load BO range for Wi-Fi in Step 1: 10%~25% |  |  |  |  | Medium load BO range for Wi-Fi in Step 1: 35%~50% |  |  |  |  | High load BO range for Wi-Fi in Step 1: above 55% |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Wi-Fi in step 1 (Op. A) |  | Wi-Fi in step 1 (Op. B) | Wi-Fi in step 2 (Op. A) | LAA in step 2 (Op. B) | Wi-Fi in step 1 (Op. A) |  | Wi-Fi in step 1 (Op. B) | Wi-Fi in step 2 (Op. A) | LAA in step 2 (Op. B) | Wi-Fi in step 1 (Op. A) |  | Wi-Fi in step 1 (Op. B) | Wi-Fi in step 2 (Op. A) | LAA in step 2 (Op. B) |
| DL: UPT CDF [Mbps] | 5% | 20.287 |  | 23.150 | 18.373 | 15.407 | 11.704 |  | 15.528 | 12.087 | 6.109 | 4.640 |  | 5.098 | 5.551 | 1.050 |
|  | 50% | 84.201 |  | 82.900 | 83.292 | 57.618 | 47.001 |  | 48.454 | 44.257 | 30.534 | 25.322 |  | 25.778 | 25.480 | 9.459 |
|  | 95% | 120.468 |  | 120.405 | 119.455 | 105.998 | 108.208 |  | 107.466 | 100.020 | 71.775 | 87.261 |  | 96.131 | 94.267 | 70.875 |
|  | Mean | 77.942 |  | 78.221 | 77.341 | 59.689 | 54.394 |  | 55.537 | 50.869 | 34.189 | 36.083 |  | 36.124 | 36.026 | 18.923 |
| DL: Delay CDF [s] | 5% | 0.035 |  | 0.035 | 0.035 | 0.39 | 0.040 |  | 0.040 | 0.044 | 0.082 | 0.048 |  | 0.054 | 0.045 | 0.088 |
|  | 50% | 0.054 |  | 0.052 | 0.057 | 0.082 | 0.1334 |  | 0.127 | 0.147 | 0.450 | 0.358 |  | 0.253 | 0.231 | 0.893 |
|  | 95% | 0.191 |  | 0.175 | 0.243 | 0.365 | 0.744 |  | 0.665 | 0.674 | 1.845 | 2.497 |  | 1.814 | 1.066 | 3.775 |
|  | Mean | 0.077 |  | 0.072 | 0.090 | 0.129 | 0.2189 |  | 0.192 | 0.217 | 0.675 | 0.737 |  | 0.450 | 0.363 | 1.306 |
| UL: UPT CDF [Mbps] | 5% | 19.039 |  | 16.245 | 22.265 | 8.064 | 11.194 |  | 7.162 | 10.586 | 2.840 | 5.744 |  | 6.056 | 6.136 | 0.148 |
|  | 50% | 37.143 |  | 46.172 | 39.073 | 13.969 | 23.380 |  | 30.000 | 27.112 | 5.724 | 17.315 |  | 21.964 | 20.025 | 2.968 |
|  | 95% | 58.516 |  | 59.301 | 57.179 | 25.823 | 52.730 |  | 50.241 | 46.449 | 12.615 | 47.671 |  | 46.548 | 44.666 | 9.687 |
|  | Mean | 39.763 |  | 43.116 | 40.956 | 16.162 | 29.1171 |  | 30.379 | 29.034 | 7.053 | 22.013 |  | 24.081 | 23.352 | 3.846 |
| UL: Delay CDF [s] | 5% | 0.071 |  | 0.071 | 0.072 | 0.165 | 0.072 |  | 0.079 | 0.088 | 0.185 | 0.073 |  | 0.087 | 0.098 | 0.334 |
|  | 50% | 0.113 |  | 0.091 | 0.106 | 0.284 | 0.179 |  | 0.158 | 0.168 | 1.372 | 0.251 |  | 0.249 | 0.217 | 1.788 |
|  | 95% | 0.191 |  | 0.349 | 0.158 | 0.638 | 0.311 |  | 0.594 | 0.384 | 3.338 | 0.672 |  | 1.250 | 0.558 | 4.269 |
|  | Mean | 0.125 |  | 0.131 | 0.1187 | 0.332 | 0.197 |  | 0.238 | 0.204 | 1.665 | 0.309 |  | 0.433 | 0.266 | 2.243 |
| VoIP outage (DL) |  | 0.0 |  | - | 0.0 | - | 12.5 |  | - | 12.5 | - | 12.5 |  | - | 25.0 | - |
| VoIP outage (UL) |  | 0.0 |  | - | 0.0 | - | 6.25 |  | - | 12.5 | - | 37.5 |  | - | 25.0 | - |
| VoIP outage |  | 0.0 |  | - | 0.0 | - | 18.75 |  | - | 18.75 | - | 37.5 |  | - | 31.25 | - |
| 𝜌DL |  | 1.00 |  | 1.0 | 1.00 | 1.00 | 0.994 |  | 0.994 | 0.994 | 0.967 | 0.975 |  | 0.975 | 0.984 | 0.922 |
| 𝜌UL |  | 1.00 |  | 1.0 | 1.00 | 1.00 | 1.00 |  | 0.979 | 1.00 | 0.845 | 0.993 |  | 0.976 | 0.990 | 0.710 |
| BO |  | 14.30 |  | 12.25 | 16.51 | 18.34 | 37.00 |  | 35.73 | 35.9 | 50.93 | 58.08 |  | 55.00 | 51.88 | 69.20 |
| 𝜆 |  |  | DL/UL - 0.24/0.06 |  |  |  |  | DL/UL - 0.36/0.09 |  |  |  |  | DL/UL- 0.44/0.11 |  |  |  |
| Additional comments: UEs scheduled for the LAA UL contend for the channel using LBT Cat 2 with a FFP of 10ms. LTE uses TDD UL/DL configuration #3 [DSUUUDDDDD]. UL CCA lasts 24 us and is performed prior to start of SF2. Only one CCA per FFP is done per UE. Scheduled UEs can transmit a single burst of up to 3 SFs. No dynamic reassignment of unused UL SFs is done. The DL uses DL LBT Cat 4 (UE NACK based with defer period) with details described in R1-152932. LAA uses a sensing threshold of -82 dBm. LAA slot size is the same as used for Wi-Fi: 8 us. RTS/CTS is disabled for WiFi. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.4: Wi-Fi/LAA UL+DL; Indoor Deployment; Y=1; Mixed traffic with 80% DL/20% UL;
DL LAA LBT: Cat 4, UL LAA LBT Cat 2: 48 µs CCA; Wi-Fi RTS/CTS disabled

| Reported parameters |  | Low load BO range for Wi-Fi in Step 1: 10%~25% |  |  |  |  | Medium load BO range for Wi-Fi in Step 1: 35%~50% |  |  |  |  | High load BO range for Wi-Fi in Step 1: above 55% |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Wi-Fi in step 1 (Op. A) | Wi-Fi in step 1 (Op. B) |  | Wi-Fi in step 2 (Op. A) | LAA in step 2 (Op. B) | Wi-Fi in step 1 (Op. A) |  | Wi-Fi in step 1 (Op. B) | Wi-Fi in step 2 (Op. A) | LAA in step 2 (Op. B) | Wi-Fi in step 1 (Op. A) |  | Wi-Fi in step 1 (Op. B) | Wi-Fi in step 2 (Op. A) | LAA in step 2 (Op. B) |
| DL: UPT CDF [Mbps] | 5% | 20.287 | 23.150 |  | 18.523 | 15.595 | 11.704 |  | 15.528 | 12.791 | 7.682 | 4.640 |  | 5.098 | 5.745 | 1.289 |
|  | 50% | 84.201 | 82.900 |  | 84.969 | 58.915 | 47.001 |  | 48.454 | 48.744 | 33.823 | 25.322 |  | 25.778 | 25.574 | 9.550 |
|  | 95% | 120.468 | 120.405 |  | 120.388 | 105.448 | 108.208 |  | 107.466 | 100.299 | 76.838 | 87.261 |  | 96.131 | 92.761 | 68.332 |
|  | Mean | 77.942 | 78.221 |  | 77.760 | 60.189 | 54.394 |  | 55.537 | 54.809 | 37.639 | 36.083 |  | 36.124 | 36.587 | 19.134 |
| DL: Delay CDF [s] | 5% | 0.035 | 0.035 |  | 0.035 | 0.39 | 0.040 |  | 0.040 | 0.044 | 0.083 | 0.048 |  | 0.054 | 0.049 | 0.262 |
|  | 50% | 0.054 | 0.052 |  | 0.054 | 0.077 | 0.1334 |  | 0.127 | 0.117 | 0.474 | 0.358 |  | 0.253 | 0.207 | 0.984 |
|  | 95% | 0.191 | 0.175 |  | 0.242 | 0.379 | 0.744 |  | 0.665 | 0.484 | 1.991 | 2.497 |  | 1.814 | 1.241 | 3.415 |
|  | Mean | 0.077 | 0.072 |  | 0.089 | 0.133 | 0.2189 |  | 0.192 | 0.173 | 0.692 | 0.737 |  | 0.450 | 0.381 | 1.313 |
| UL: UPT CDF [Mbps] | 5% | 19.039 | 16.245 |  | 21.885 | 8.015 | 11.194 |  | 7.162 | 11.542 | 2.942 | 5.744 |  | 6.056 | 5.111 | 0.381 |
|  | 50% | 37.143 | 46.172 |  | 37.716 | 13.949 | 23.380 |  | 30.000 | 29.066 | 6.132 | 17.315 |  | 21.964 | 20.659 | 3.483 |
|  | 95% | 58.516 | 59.301 |  | 57.652 | 25.863 | 52.730 |  | 50.241 | 47.314 | 15.071 | 47.671 |  | 46.548 | 48.686 | 10.541 |
|  | Mean | 39.763 | 43.116 |  | 40.771 | 16.073 | 29.1171 |  | 30.379 | 30.356 | 7.792 | 22.013 |  | 24.081 | 24.347 | 4.318 |
| UL: Delay CDF [s] | 5% | 0.071 | 0.071 |  | 0.072 | 0.165 | 0.072 |  | 0.079 | 0.093 | 0.181 | 0.073 |  | 0.087 | 0.087 | 0.321 |
|  | 50% | 0.113 | 0.091 |  | 0.106 | 0.285 | 0.179 |  | 0.158 | 0.153 | 1.186 | 0.251 |  | 0.249 | 0.213 | 1.815 |
|  | 95% | 0.191 | 0.349 |  | 0.152 | 0.654 | 0.311 |  | 0.594 | 0.390 | 2.639 | 0.672 |  | 1.250 | 0.486 | 4.472 |
|  | Mean | 0.125 | 0.131 |  | 0.118 | 0.339 | 0.197 |  | 0.238 | 0.192 | 1.457 | 0.309 |  | 0.433 | 0.262 | 2.255 |
| VoIP outage (DL) |  | 0.0 | - |  | 0.0 | - | 12.5 |  | - | 6.25 | - | 12.5 |  | - | 12.5 | - |
| VoIP outage (UL) |  | 0.0 | - |  | 0.0 | - | 6.25 |  | - | 12.5 | - | 37.5 |  | - | 12.5 | - |
| VoIP outage |  | 0.0 | - |  | 0.0 | - | 18.75 |  | - | 12.5 | - | 37.5 |  | - | 12.5 | - |
| 𝜌DL |  | 1.00 | 1.0 |  | 1.00 | 1.00 | 0.994 |  | 0.994 | 1.00 | 0.974 | 0.975 |  | 0.975 | 0.981 | 0.912 |
| 𝜌UL |  | 0.99 | 1.0 |  | 1.00 | 1.00 | 1.00 |  | 0.979 | 1.00 | 0.872 | 0.993 |  | 0.976 | 0.988 | 0.687 |
| BO |  | 14.30 | 12.25 |  | 16.25 | 18.15 | 37.00 |  | 35.73 | 32.6 | 47.29 | 58.08 |  | 55.00 | 50.15 | 68.76 |
| 𝜆 |  |  |  | DL/UL - 0.24/0.06 |  |  |  | DL/UL - 0.36/0.09 |  |  |  |  | DL/UL- 0.44/0.11 |  |  |  |
| Additional comments: UEs scheduled for the LAA UL contend for the channel using LBT Cat 2 with a FFP of 10ms. LTE uses TDD UL/DL configuration #3 [DSUUUDDDDD]. UL CCA lasts 48 us and is performed prior to start of SF2. Only one CCA per FFP is done per UE. Scheduled UEs can transmit a single burst of up to 3 SFs. No dynamic reassignment of unused UL SFs is done. The DL uses DL LBT Cat 4 (UE NACK based with defer period) with details described in R1-152932. LAA uses a sensing threshold of -82 dBm. LAA slot size is the same as used for Wi-Fi: 8 us. RTS/CTS is disabled for WiFi. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.5: Wi-Fi/LAA UL+DL; Indoor Deployment; Y=1; Mixed traffic with 50% DL/50% UL;
DL LAA LBT: Cat 4, UL LAA LBT Cat 2: 24 µs CCA; Wi-Fi RTS/CTS enabled

| Reported parameters |  | Low load BO range for Wi-Fi in Step 1: 10%~25% |  |  |  |  | Medium load BO range for Wi-Fi in Step 1: 35%~50% |  |  |  | High load BO range for Wi-Fi in Step 1: above 55% |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Wi-Fi in step 1 (Op. A) |  | Wi-Fi in step 1 (Op. B) | Wi-Fi in step 2 (Op. A) | LAA in step 2 (Op. B) | Wi-Fi in step 1 (Op. A) | Wi-Fi in step 1 (Op. B) | Wi-Fi in step 2 (Op. A) | LAA in step 2 (Op. B) | Wi-Fi in step 1 (Op. A) | Wi-Fi in step 1 (Op. B) |  | Wi-Fi in step 2 (Op. A) | LAA in step 2 (Op. B) |
| DL: UPT CDF [Mbps] | 5% | 24.951 |  | 22.736 | 26.784 | 17.745 | 6.124 | 7.364 | 10.127 | 5.183 | 1.361 | 3.004 |  | 9.978 | 2.489 |
|  | 50% | 64.942 |  | 76.269 | 92.614 | 72.451 | 27.827 | 27.018 | 44.952 | 26.546 | 8.601 | 11.480 |  | 29.121 | 11.242 |
|  | 95% | 118.789 |  | 120.471 | 120.374 | 108.103 | 101.673 | 102.197 | 104.250 | 86.695 | 44.652 | 60.759 |  | 81.376 | 48.508 |
|  | Mean | 73.189 |  | 77.246 | 84.750 | 70.105 | 35.769 | 40.427 | 53.527 | 35.690 | 13.740 | 16.928 |  | 38.832 | 17.505 |
| DL: Delay CDF [s] | 5% | 0.035 |  | 0.035 | 0.035 | 0.039 | 0.044 | 0.040 | 0.042 | 0.053 | 0.157 | 0.153 |  | 0.064 | 0.121 |
|  | 50% | 0.067 |  | 0.057 | 0.046 | 0.060 | 0.176 | 0.153 | 0.112 | 0.220 | 0.800 | 0.596 |  | 0.185 | 1.170 |
|  | 95% | 0.152 |  | 0.163 | 0.135 | 0.210 | 0.989 | 0.486 | 0.456 | 1.109 | 3.196 | 2.071 |  | 0.823 | 3.224 |
|  | Mean | 0.076 |  | 0.072 | 0.065 | 0.085 | 0.318 | 0.202 | 0.156 | 0.371 | 1.190 | 0.895 |  | 0.280 | 1.332 |
| UL: UPT CDF [Mbps] | 5% | 15.408 |  | 7.061 | 22.400 | 4.758 | 5.500 | 4.881 | 11.146 | 2.154 | 2.873 | 2.333 |  | 7.895 | 0.534 |
|  | 50% | 36.449 |  | 34.943 | 41.768 | 13.448 | 14.964 | 15.446 | 28.687 | 6.987 | 6.953 | 7.131 |  | 19.653 | 3.790 |
|  | 95% | 58.521 |  | 58.611 | 59.262 | 23.571 | 44.331 | 56.729 | 53.496 | 16.531 | 21.731 | 21.798 |  | 40.533 | 12.921 |
|  | Mean | 38.193 |  | 38.261 | 42.714 | 14.132 | 19.983 | 22.346 | 30.693 | 8.215 | 9.111 | 9.245 |  | 21.891 | 5.139 |
| UL: Delay CDF [s] | 5% | 0.072 |  | 0.071 | 0.071 | 0.165 | 0.084 | 0.073 | 0.077 | 0.176 | 0.206 | 0.1416 |  | 0.113 | 0.193 |
|  | 50% | 0.110 |  | 0.106 | 0.098 | 0.300 | 0.263 | 0.261 | 0.145 | 0.948 | 0.637 | 0.643 |  | 0.232 | 1.526 |
|  | 95% | 0.236 |  | 0.229 | 0.172 | 0.584 | 0.652 | 0.838 | 0.353 | 2.679 | 1.442 | 1.668 |  | 0.677 | 3.922 |
|  | Mean | 0.127 |  | 0.142 | 0.107 | 0.340 | 0.316 | 0.322 | 0.178 | 1.177 | 0.737 | 0.771 |  | 0.288 | 1.728 |
| VoIP outage (DL) |  | 0.0 |  | - | 0.0 | - | 0.0 | - | 0.0 | - | 0.0 | - |  | 0.0 | - |
| VoIP outage (UL) |  | 0.0 |  | - | 0.0 | - | 0.0 | - | 0.0 | - | 6.25 | - |  | 0.0 | - |
| VoIP outage |  | 0.0 |  | - | 0.0 | - | 0.0 | - | 0.0 | - | 6.25 | - |  | 0.0 | - |
| 𝜌DL |  | 1.00 |  | 1.00 | 1.0 | 1.00 | 0.998 | 0.998 | 1.00 | 0.986 | 0.938 | 0.938 |  | 0.993 | 0.911 |
| 𝜌UL |  | 0.996 |  | 0.975 | 1.0 | 0.978 | 0.975 | 0.975 | 0.989 | 0.933 | 0.949 | 0.961 |  | 0.984 | 0.886 |
| BO |  | 15.87 |  | 13.80 | 13.98 | 19.73 | 43.32 | 40.66 | 30.14 | 57.35 | 75.75 | 73.38 |  | 47.86 | 78.30 |
| 𝜆 |  |  | 0.15 |  |  |  |  | 0.225 |  |  |  |  | 0.275 |  |  |
| Additional comments: UEs scheduled for the LAA UL contend for the channel using LBT Cat 2 with a FFP of 10ms. LTE uses TDD UL/DL configuration #3 [DSUUUDDDDD]. UL CCA lasts 24 us and is performed prior to start of SF2. Only one CCA per FFP is done per UE. Scheduled UEs can transmit a single burst of up to 3 SFs. No dynamic reassignment of unused UL SFs is done. The DL uses DL LBT Cat 4 (UE NACK based with defer period) with details described in R1-152932. LAA uses a sensing threshold of -82 dBm. LAA slot size is the same as used for Wi-Fi: 8 us. RTS/CTS is enabled for WiFi. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.6: Wi-Fi/LAA UL+DL; Indoor Deployment; Y=1; Mixed traffic with 50% DL/50% UL;
DL LAA LBT: Cat 4, UL LAA LBT Cat 2: 48 µs CCA; Wi-Fi RTS/CTS enabled

| Reported parameters |  | Low load BO range for Wi-Fi in Step 1: 10%~25% |  |  |  |  | Medium load BO range for Wi-Fi in Step 1: 35%~50% |  |  |  | High load BO range for Wi-Fi in Step 1: above 55% |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Wi-Fi in step 1 (Op. A) |  | Wi-Fi in step 1 (Op. B) | Wi-Fi in step 2 (Op. A) | LAA in step 2 (Op. B) | Wi-Fi in step 1 (Op. A) | Wi-Fi in step 1 (Op. B) | Wi-Fi in step 2 (Op. A) | LAA in step 2 (Op. B) | Wi-Fi in step 1 (Op. A) | Wi-Fi in step 1 (Op. B) |  | Wi-Fi in step 2 (Op. A) | LAA in step 2 (Op. B) |
| DL: UPT CDF [Mbps] | 5% | 24.951 |  | 22.736 | 28.583 | 18.480 | 6.124 | 7.364 | 11.025 | 5.095 | 1.361 | 3.004 |  | 8.836 | 2.378 |
|  | 50% | 64.942 |  | 76.269 | 93.488 | 71.112 | 27.827 | 27.018 | 50.569 | 25.628 | 8.601 | 11.480 |  | 29.856 | 13.239 |
|  | 95% | 118.789 |  | 120.471 | 120.391 | 109.716 | 101.673 | 102.197 | 110.795 | 93.464 | 44.652 | 60.759 |  | 80.903 | 52.256 |
|  | Mean | 73.189 |  | 77.246 | 85.891 | 69.210 | 35.769 | 40.427 | 57.880 | 38.204 | 13.740 | 16.928 |  | 39.584 | 18.832 |
| DL: Delay CDF [s] | 5% | 0.035 |  | 0.035 | 0.035 | 0.038 | 0.044 | 0.040 | 0.040 | 0.043 | 0.157 | 0.153 |  | 0.067 | 0.140 |
|  | 50% | 0.067 |  | 0.057 | 0.046 | 0.062 | 0.176 | 0.153 | 0.095 | 0.201 | 0.800 | 0.596 |  | 0.183 | 1.194 |
|  | 95% | 0.152 |  | 0.163 | 0.127 | 0.194 | 0.989 | 0.486 | 0.498 | 1.064 | 3.196 | 2.071 |  | 0.809 | 3.356 |
|  | Mean | 0.076 |  | 0.072 | 0.063 | 0.084 | 0.318 | 0.202 | 0.153 | 0.337 | 1.190 | 0.895 |  | 0.271 | 1.376 |
| UL: UPT CDF [Mbps] | 5% | 15.408 |  | 7.061 | 22.672 | 5.242 | 5.500 | 4.881 | 11.961 | 2.354 | 2.873 | 2.333 |  | 8.321 | 0.889 |
|  | 50% | 36.449 |  | 34.943 | 41.976 | 13.397 | 14.964 | 15.446 | 29.425 | 6.681 | 6.953 | 7.131 |  | 21.105 | 3.936 |
|  | 95% | 58.521 |  | 58.611 | 59.254 | 24.241 | 44.331 | 56.729 | 56.411 | 16.748 | 21.731 | 21.798 |  | 42.360 | 13.309 |
|  | Mean | 38.193 |  | 38.261 | 43.160 | 14.239 | 19.983 | 22.346 | 31.895 | 8.459 | 9.111 | 9.245 |  | 22.729 | 5.276 |
| UL: Delay CDF [s] | 5% | 0.072 |  | 0.071 | 0.071 | 0.165 | 0.084 | 0.073 | 0.073 | 0.175 | 0.206 | 0.1416 |  | 0.113 | 0.189 |
|  | 50% | 0.110 |  | 0.106 | 0.096 | 0.307 | 0.263 | 0.261 | 0.137 | 0.818 | 0.637 | 0.643 |  | 0.216 | 1.403 |
|  | 95% | 0.236 |  | 0.229 | 0.160 | 0.584 | 0.652 | 0.838 | 0.333 | 2.424 | 1.442 | 1.668 |  | 0.685 | 3.669 |
|  | Mean | 0.127 |  | 0.142 | 0.106 | 0.341 | 0.316 | 0.322 | 0.170 | 1.074 | 0.737 | 0.771 |  | 0.275 | 1.683 |
| VoIP outage (DL) |  | 0.0 |  | - | 0.0 | - | 0.0 | - | 0.0 | - | 0.0 | - |  | 0.0 | - |
| VoIP outage (UL) |  | 0.0 |  | - | 0.0 | - | 0.0 | - | 0.0 | - | 6.25 | - |  | 0.0 | - |
| VoIP outage |  | 0.0 |  | - | 0.0 | - | 0.0 | - | 0.0 | - | 6.25 | - |  | 0.0 | - |
| 𝜌DL |  | 1.00 |  | 1.00 | 1.00 | 1.00 | 0.998 | 0.998 | 0.999 | 0.988 | 0.938 | 0.938 |  | 0.994 | 0.918 |
| 𝜌UL |  | 0.996 |  | 0.975 | 1.00 | 0.980 | 0.975 | 0.975 | 0.994 | 0.939 | 0.949 | 0.961 |  | 0.984 | 0.889 |
| BO |  | 15.87 |  | 13.80 | 13.79 | 19.67 | 43.32 | 40.66 | 28.91 | 56.13 | 75.75 | 73.38 |  | 46.60 | 77.85 |
| 𝜆 |  |  | 0.15 |  |  |  |  | 0.225 |  |  |  |  | 0.275 |  |  |
| Additional comments: UEs scheduled for the LAA UL contend for the channel using LBT Cat 2 with a FFP of 10ms. LTE uses TDD UL/DL configuration #3 [DSUUUDDDDD]. UL CCA lasts 48 us and is performed prior to start of SF2. Only one CCA per FFP is done per UE. Scheduled UEs can transmit a single burst of up to 3 SFs. No dynamic reassignment of unused UL SFs is done. The DL uses DL LBT Cat 4 (UE NACK based with defer period) with details described in R1-152932. LAA uses a sensing threshold of -82 dBm. LAA slot size is the same as used for Wi-Fi: 8 us. RTS/CTS is enabled for WiFi. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.7: Wi-Fi/LAA UL+DL; Indoor Deployment; Y=1; Mixed traffic with 80% DL/20% UL;
DL LAA LBT: Cat 4, UL LAA LBT Cat 2: 24 µs CCA; Wi-Fi RTS/CTS enabled

| Reported parameters |  | Low load BO range for Wi-Fi in Step 1: 10%~25% |  |  |  |  | Medium load BO range for Wi-Fi in Step 1: 35%~50% |  |  |  |  | High load BO range for Wi-Fi in Step 1: above 55% |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Wi-Fi in step 1 (Op. A) |  | Wi-Fi in step 1 (Op. B) | Wi-Fi in step 2 (Op. A) | LAA in step 2 (Op. B) | Wi-Fi in step 1 (Op. A) |  | Wi-Fi in step 1 (Op. B) | Wi-Fi in step 2 (Op. A) | LAA in step 2 (Op. B) | Wi-Fi in step 1 (Op. A) |  | Wi-Fi in step 1 (Op. B) | Wi-Fi in step 2 (Op. A) | LAA in step 2 (Op. B) |
| DL: UPT CDF [Mbps] | 5% | 22.167 |  | 26.125 | 16.844 | 16.458 | 9.266 |  | 13.352 | 7.725 | 0.832 | 4.122 |  | 5.241 | 6.205 | 0.329 |
|  | 50% | 78.894 |  | 84.442 | 80.767 | 56.879 | 34.622 |  | 34.101 | 28.060 | 6.962 | 13.761 |  | 15.467 | 18.587 | 3.734 |
|  | 95% | 120.388 |  | 120.377 | 120.202 | 104.362 | 105.518 |  | 111.087 | 85.785 | 59.868 | 69.121 |  | 72.093 | 65.564 | 55.886 |
|  | Mean | 75.759 |  | 78.234 | 74.895 | 58.673 | 44.909 |  | 44.903 | 36.514 | 15.458 | 21.593 |  | 22.331 | 25.400 | 10.597 |
| DL: Delay CDF [s] | 5% | 0.035 |  | 0.035 | 0.035 | 0.039 | 0.040 |  | 0.037 | 0.058 | 0.125 | 0.127 |  | 0.102 | 0.070 | 0.202 |
|  | 50% | 0.055 |  | 0.052 | 0.060 | 0.084 | 0.144 |  | 0.131 | 0.193 | 1.114 | 0.537 |  | 0.338 | 0.292 | 1.911 |
|  | 95% | 0.170 |  | 0.149 | 0.295 | 0.362 | 0.622 |  | 0.324 | 1.002 | 3.794 | 2.098 |  | 1.294 | 1.126 | 4.975 |
|  | Mean | 0.077 |  | 0.069 | 0.100 | 0.131 | 0.212 |  | 0.1482 | 0.317 | 1.446 | 0.710 |  | 0.465 | 0.397 | 2.192 |
| UL: UPT CDF [Mbps] | 5% | 18.664 |  | 11.564 | 22.624 | 8.309 | 8.425 |  | 5.969 | 10.114 | 0.187 | 3.753 |  | 4.503 | 7.477 | 0.354 |
|  | 50% | 34.807 |  | 43.923 | 37.945 | 12.945 | 19.331 |  | 27.057 | 21.216 | 3.527 | 10.776 |  | 15.764 | 17.653 | 1.804 |
|  | 95% | 58.508 |  | 59.309 | 56.091 | 24.061 | 52.919 |  | 49.841 | 43.546 | 11.304 | 28.740 |  | 23.075 | 35.757 | 6.766 |
|  | Mean | 38.616 |  | 40.782 | 40.296 | 15.500 | 23.792 |  | 28.256 | 24.711 | 4.8131 | 13.160 |  | 14.987 | 20.604 | 2.672 |
| UL: Delay CDF [s] | 5% | 0.071 |  | 0.071 | 0.072 | 0.165 | 0.071 |  | 0.072 | 0.104 | 0.204 | 0.161 |  | 0.139 | 0.117 | 0.373 |
|  | 50% | 0.120 |  | 0.098 | 0.110 | 0.312 | 0.212 |  | 0.172 | 0.203 | 1.449 | 0.375 |  | 0.340 | 0.227 | 2.050 |
|  | 95% | 0.192 |  | 0.437 | 0.173 | 0.636 | 0.403 |  | 0.546 | 0.425 | 3.724 | 0.773 |  | 0.841 | 0.465 | 4.978 |
|  | Mean | 0.127 |  | 0.153 | 0.117 | 0.341 | 0.230 |  | 0.226 | 0.227 | 1.788 | 0.428 |  | 0.423 | 0.256 | 2.382 |
| VoIP outage (DL) |  | 0.0 |  | - | 0.0 | - | 0.0 |  | - | 6.25 | - | 0.0 |  | - | 6.25 | - |
| VoIP outage (UL) |  | 0.0 |  | - | 0.0 | - | 0.0 |  | - | 0.0 | - | 0.0 |  | - | 0.0 | - |
| VoIP outage |  | 0.0 |  | - | 0.0 | - | 0.0 |  | - | 6.25 | - | 0.0 |  | - | 6.25 | - |
| 𝜌DL |  | 1.00 |  | 1.0 | 1.0 | 1.0 | 0.994 |  | 0.994 | 0.985 | 0.896 | 0.973 |  | 0.973 | 0.980 | 0.849 |
| 𝜌UL |  | 1.00 |  | 1.0 | 1.0 | 1.0 | 0.996 |  | 0.971 | 0.999 | 0.770 | 0.963 |  | 0.970 | 0.988 | 0.739 |
| BO |  | 14.14 |  | 12.15 | 16.56 | 18.20 | 39.18 |  | 37.61 | 45.12 | 70.23 | 67.85 |  | 64.45 | 56.35 | 85.78 |
| 𝜆 |  |  | DL/UL - 0.24/0.06 |  |  |  |  | DL/UL - 0.44/0.11 |  |  |  |  | DL/UL- 0.52/0.13 |  |  |  |
| Additional comments: UEs scheduled for the LAA UL contend for the channel using LBT Cat 2 with a FFP of 10ms. LTE uses TDD UL/DL configuration #3 [DSUUUDDDDD]. UL CCA lasts 24 us and is performed prior to start of SF2. Only one CCA per FFP is done per UE. Scheduled UEs can transmit a single burst of up to 3 SFs. No dynamic reassignment of unused UL SFs is done. The DL uses DL LBT Cat 4 (UE NACK based with defer period) with details described in R1-152932. LAA uses a sensing threshold of -82 dBm. LAA slot size is the same as used for Wi-Fi: 8 us. RTS/CTS is enabled for WiFi. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.8: Wi-Fi/LAA UL+DL; Indoor Deployment; Y=1; Mixed traffic with 80% DL/20% UL;
DL LAA LBT: Cat 4, UL LAA LBT Cat 2: 48 µs CCA; Wi-Fi RTS/CTS enabled

| Reported parameters |  | Low load BO range for Wi-Fi in Step 1: 10%~25% |  |  |  |  | Medium load BO range for Wi-Fi in Step 1: 35%~50% |  |  |  |  | High load BO range for Wi-Fi in Step 1: above 55% |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Wi-Fi in step 1 (Op. A) |  | Wi-Fi in step 1 (Op. B) | Wi-Fi in step 2 (Op. A) | LAA in step 2 (Op. B) | Wi-Fi in step 1 (Op. A) |  | Wi-Fi in step 1 (Op. B) | Wi-Fi in step 2 (Op. A) | LAA in step 2 (Op. B) | Wi-Fi in step 1 (Op. A) |  | Wi-Fi in step 1 (Op. B) | Wi-Fi in step 2 (Op. A) | LAA in step 2 (Op. B) |
| DL: UPT CDF [Mbps] | 5% | 22.167 |  | 26.125 | 18.400 | 17.146 | 9.266 |  | 13.352 | 8.074 | 0.858 | 4.122 |  | 5.241 | 5.582 | 0.362 |
|  | 50% | 78.894 |  | 84.442 | 83.557 | 56.540 | 34.622 |  | 34.101 | 28.822 | 7.357 | 13.761 |  | 15.467 | 19.064 | 3.262 |
|  | 95% | 120.388 |  | 120.377 | 120.360 | 103.231 | 105.518 |  | 111.087 | 84.856 | 57.965 | 69.121 |  | 72.093 | 64.511 | 56.330 |
|  | Mean | 75.759 |  | 78.234 | 76.940 | 58.486 | 44.909 |  | 44.903 | 37.148 | 15.570 | 21.593 |  | 22.331 | 25.694 | 10.403 |
| DL: Delay CDF [s] | 5% | 0.035 |  | 0.035 | 0.035 | 0.039 | 0.040 |  | 0.037 | 0.058 | 0.110 | 0.127 |  | 0.102 | 0.070 | 0.147 |
|  | 50% | 0.055 |  | 0.052 | 0.055 | 0.082 | 0.144 |  | 0.131 | 0.186 | 1.122 | 0.537 |  | 0.338 | 0.262 | 1.953 |
|  | 95% | 0.170 |  | 0.149 | 0.204 | 0.257 | 0.622 |  | 0.324 | 0.945 | 3.715 | 2.098 |  | 1.294 | 0.965 | 5.192 |
|  | Mean | 0.077 |  | 0.069 | 0.082 | 0.113 | 0.212 |  | 0.1482 | 0.289 | 1.380 | 0.710 |  | 0.465 | 0.363 | 2.283 |
| UL: UPT CDF [Mbps] | 5% | 18.664 |  | 11.564 | 21.449 | 8.046 | 8.425 |  | 5.969 | 10.933 | 0.539 | 3.753 |  | 4.503 | 8.191 | 0.241 |
|  | 50% | 34.807 |  | 43.923 | 38.307 | 13.395 | 19.331 |  | 27.057 | 21.581 | 4.663 | 10.776 |  | 15.764 | 18.343 | 1.570 |
|  | 95% | 58.508 |  | 59.309 | 57.147 | 25.723 | 52.919 |  | 49.841 | 42.810 | 11.561 | 28.740 |  | 23.075 | 39.229 | 6.059 |
|  | Mean | 38.616 |  | 40.782 | 40.884 | 16.025 | 23.792 |  | 28.256 | 24.925 | 5.432 | 13.160 |  | 14.987 | 20.954 | 2.449 |
| UL: Delay CDF [s] | 5% | 0.071 |  | 0.071 | 0.071 | 0.165 | 0.071 |  | 0.072 | 0.103 | 0.200 | 0.161 |  | 0.139 | 0.117 | 0.406 |
|  | 50% | 0.120 |  | 0.098 | 0.108 | 0.285 | 0.212 |  | 0.172 | 0.198 | 1.380 | 0.375 |  | 0.340 | 0.220 | 2.161 |
|  | 95% | 0.192 |  | 0.437 | 0.167 | 0.601 | 0.403 |  | 0.546 | 0.393 | 3.716 | 0.773 |  | 0.841 | 0.467 | 5.049 |
|  | Mean | 0.127 |  | 0.153 | 0.117 | 0.319 | 0.230 |  | 0.226 | 0.217 | 1.795 | 0.428 |  | 0.423 | 0.249 | 2.508 |
| VoIP outage (DL) |  | 0.0 |  | - | 0.0 | - | 0.0 |  | - | 0.0 | - | 0.0 |  | - | 0.0 | - |
| VoIP outage (UL) |  | 0.0 |  | - | 0.0 | - | 0.0 |  | - | 0.0 | - | 0.0 |  | - | 0.0 | - |
| VoIP outage |  | 0.0 |  | - | 0.0 | - | 0.0 |  | - | 0.0 | - | 0.0 |  | - | 0.0 | - |
| 𝜌DL |  | 1.00 |  | 1.0 | 1.00 | 1.00 | 0.994 |  | 0.994 | 0.985 | 0.897 | 0.973 |  | 0.973 | 0.983 | 0.842 |
| 𝜌UL |  | 1.00 |  | 1.0 | 1.00 | 1.00 | 0.996 |  | 0.971 | 0.997 | 0.774 | 0.963 |  | 0.970 | 0.992 | 0.737 |
| BO |  | 14.14 |  | 12.15 | 15.81 | 17.70 | 39.18 |  | 37.61 | 44.24 | 70.10 | 67.85 |  | 64.45 | 55.7 | 86.08 |
| 𝜆 |  |  | DL/UL - 0.24/0.06 |  |  |  |  | DL/UL - 0.44/0.11 |  |  |  |  | DL/UL- 0.52/0.13 |  |  |  |
| Additional comments: UEs scheduled for the LAA UL contend for the channel using LBT Cat 2 with a FFP of 10ms. LTE uses TDD UL/DL configuration #3 [DSUUUDDDDD]. UL CCA lasts 48 us and is performed prior to start of SF2. Only one CCA per FFP is done per UE. Scheduled UEs can transmit a single burst of up to 3 SFs. No dynamic reassignment of unused UL SFs is done. The DL uses DL LBT Cat 4 (UE NACK based with defer period) with details described in R1-152932. LAA uses a sensing threshold of -82 dBm. LAA slot size is the same as used for Wi-Fi: 8 us. RTS/CTS is enabled for WiFi. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# Appendix B – Evaluation methodology and assumptions

For UL transmissions, all LAA UEs are subject to dynamic scheduling by the eNB. The LAA SCell is configured to use TDD UL/DL configuration #3 [DSUUUDDDDD]. Only LAA UEs which were scheduled for the UL by the eNB will contend for the channel. A nominal FFP of 10 ms is configured. CCA with either 24 us or 48 us duration is performed prior to the start of SF #2 by any UE that was scheduled in the UL. Only one CCA attempt per UE is executed per FFP. Any scheduled UE can transmit a single burst of up to 3 SFs total duration, subject however to per-subframe eNB scheduling.

LAA DL and UL operation for evaluation purposes is illustrated Figure B.1. It is one consequence of the LBT Cat 2 channel access scheme assumed for the LAA UL that CCA will be done more or less synchronously by all scheduled LAA UEs during the same 24 or 48 us silence interval. Not all LAA UEs may use all of the up to 3 available UL subframes for their transmissions. This is shown in the first radio frame of the example in Figure B.1. Under typical conditions and when operating in presence of Wi-Fi neighbor BSSs, most LAA UEs will declare CCA to be busy simultaneously in case there is ongoing Wi-Fi activity on the channel. This is shown in the second radio frame of the example in Figure 1.

Furthermore, no particular optimization is used to increase the spectral efficiency for LAA UL and DL transmissions. A maximum of 3 consecutive UL subframes is semi-statically configured and available for UL traffic per 10 ms fixed frame period. No short-term adaptation to adjust for time-varying DL and UL offered load is performed by the LAA eNB. In addition, eNB scheduling of DL TXOP takes priority over an upcoming UL transmission period. For example, if the eNB wins channel access to serve DL traffic in subframe #1, it will schedule subframes #2-#5 as DL in order to maximize the use of the DL TXOP under the condition that enough DL traffic can be served. In such a case, the eNB will simply not issue any UL grants for the candidate UL subframes #3-#5 and the next UL transmission may take place only one radio frame later.

DTX for the LAA DL is modeled as a maximum allowed time duration to schedule a number of consecutive DL subframes by the LAA eNB. We use maximum TXOP durations of 4 ms. The duration of the busy signal when transmitted by the eNB prior to regular DL scheduling does not impact the allowed the maximum number of schedulable DL subframes. Therefore, the LTE LAA DL will transmit up to 4 consecutive DL subframes once it obtains channel access using LBT Cat 4. When a DL busy signal is transmitted by the LAA eNB, it is accounted for as overhead until the beginning of the next regularly occurring SCell subframe boundary. DTX for the LAA UL is modeled implicitly in that no UE transmission can exceed a total of 3 consecutive UL subframes as by TDD UL/DL configuration #3.

Figure B.1: LTE LAA UL operation with LBT Cat 2

| Parameter |  | Value for additional Wi-Fi system evaluation assumptions |
| --- | --- | --- |
| MCS |  | 802.11ac MCS table without 256 QAM |
| Antenna configuration |  | 2Tx2Rx in DL, cross-polarized, open loop 1Tx2Rx in UL |
| Channel coding |  | BCC |
| Frame aggregation |  | A-MPDU |
| MPDU size |  | Fixed 1ms (A)-MPDU transmission duration |
| Max PPDU duration |  | 4 ms |
| MAC | Coordination | EDCA AC4 |
|  | SIFS, DIFS | SIFS, DIFS |
|  | Detection | CCA-CS |
|  | RTS/CTS | Results both with and without RTS/CTS |
|  | Contention window | Min: 15 slot, Max: 1023 slot |
| CCA-CS |  | -82dBm |
| CCA-ED |  | -62dBm |
| ACK Modeled (successful reception, resources utilized) |  | Yes |
| DL/UL Duplexing |  | DL only |
| Rate control |  | Minstrel algorithm with 100 ms reconfiguration period |
| Channel selection |  | Minimum neighbour |
| OFDM symbol length |  | 4 us |

| Parameter | Value for LAA Indoor Deployment |
| --- | --- |
| Layout for nodes | Two operators deploy 4 small cells each in the single-floor building. The small cells of each operator are equally spaced and centered along the shorter dimension of the building. The distance between two closest nodes from two operators is random. The set of small cells for both operators is centered along the longer dimension of the building. |
| System bandwidth per carrier | 20 MHz |
| Carrier frequency | 5.0 GHz |
| Number of carriers | 1 (to be shared between two operators) |
| Total BS TX power | 18 dBm |
| Total UE TX power | 18 dBm |
| Distance-dependent path loss | Small cell-to-Small cell, Small cell-to-UE: ITU InH [Table B.1.2.1-1 in TR36.814] UE-to-UE: 3GPP TR 36.843 (D2D). (3D distance between an eNB and a UE is applied. Working assumption is that 3D distance is also used for LOS probability.) |
| Penetration | 0dB |
| Shadowing | ITU InH [referring to Table A.2.1.1.5-1 in TR36.814] Working assumption is that 3D distance is used for shadowing correlation distance |
| Antenna pattern | 2D Omni-directional |
| Antenna Height: | 6m |
| UE antenna Height | 1.5m |
| Antenna gain + connector loss | 5dBi |
| Antenna gain of UE | 0 dBi |
| Fast fading channel between eNB and UE | ITU InH |
| Number of clusters/buildings per macro cell geographical area | N/A |
| Number of small cells per cluster | N/A |
| Number of small cells per Macro cell | N/A |
| Number of UEs | 20 UEs per unlicensed band carrier per operator (DL+UL, Y=1) |
| UE dropping per network | Uniform random drop within indoor coverage |
| Radius for small cell dropping in a cluster | N/A |
| Radius for UE dropping in a cluster | N/A |
| Minimum UE-to-UE distance (2D distance) | 3m |
| Traffic model | FTP3, 0.5 MB file size Independent traffic generation on the DL and UL for both Wi-Fi and LAA for FTP traffic model. Each UE has the same UL/DL traffic arrival rate ratio. Overall offered load is the same for both the coexisting networks. DL/UL traffic ratio for DL+UL coexistence evaluation: 50% DL traffic / 50% UL traffic 80% DL traffic / 20% UL traffic |
| UE receiver | MMSE-IRC as baseline |
| UE noise figure | 9dB |
| UE speed | 3km/h |
| Cell selection criteria | For LAA UEs, cell selection is based on RSRP in the unlicensed band. For WiFi STAs, cell selection is based on RSS (Received signal power strength) of WiFi APs. RSS threshold is -82 dBm. |
| UE Bandwidth | UE bandwidth for LAA and Wi-Fi: 20 MHz unlicensed |
| Network synchronization | For the same operator, the network is synchronized. Small cells of different operators are not synchronized. |
| Performance metrics | UPT, Delay, Buffer occupancy |