3GPP TSG RAN WG1 Meeting #82		R1-154689

Beijing, China, 24th - 28th August 2015

Source: 	Xinwei

Title:	Discussion on Non-precoded CSI-RS enhancement in FD-MIMO

Agenda Item:	7.2.5.2.1

Document for:	Discussion

Introduction

In 3GPP RAN #68 meeting, a new Work Item on Elevation Beamforming/Full-Dimension (FD) MIMO for LTE was agreed.

According to the WID [1], one of the key objectives is to identify specify enhancements on reference signal including Non-precoding CSI-RS, beamforming CSI-RS, SRS and DMRS.

- Specify enhancements on reference signal in the following areas [RAN1]

  - Non-precoded CSI-RS, extending the existing numbers {1,2,4,8} of CSI-RS antenna ports for support of 12 and 16 CSI-RS ports, using full-port mapping Beamformed CSI-RS

  - SRS capacity improvement

    - Support for SRS transmission with 4TX antennas as a second priority

  - Support of additional ports for DMRS targeting higher dimensional MU-MIMO

    - The maximum number of DMRS ports that a UE may be able to receive is kept as 8

In the contribution we present our views on Non-precoding CSI-RS with support to 12 and 16 ports using full port mapping beamformed CSI-RS.

CSI-RS Design Aspects

Different aspects of the CSI-RS design such as time and frequency density, nested structure, CSI-RS pattern were discussed in previous Rel-12 CSI-RS design. In the following we look at these similar details of CSI-RS design.

- CSI-RS port numbering

In Rel-10, CSI-RS can be transmitted on one, two, four or eight antenna ports using , ,  and , respectively. If Non-precoding CSI-RS is extended to support of 12 and 16 ports, CSI-RS ports numbering can be extended to  and  accordingly.

Proposal 1: CSI-RS can be extended transmitted on 12 or 16 antenna ports using and , respectively.

- CSI-RS density

CSI-RS RE density with 1 RE/PRB/port can provide acceptable link-level performance, while ensuring minimal Rel-8 UE performance degradation. One key point is to decide the density of 12 and 16 ports for CSI-RS pattern.

- Nested structure

CSI-RS pattern for 2Tx/4Tx/8Tx antenna configuration with a nested structure, which means the resource of the pattern with smaller number of CSI-RS ports is a subset of the resource of the pattern with larger number of CSI-RS ports. Nested structure can simplify the implementation.

It should be decided whether CSI-RS pattern with a nested structure for 12, 16 antenna ports.

- Number of CSI-RS configurations

In FDD, there are 20 CSI-RS configurations for 2Tx, 10 configurations for 4Tx, and 5 configurations for 8Tx. Whether there is a need to prepare more than one configuration for 12, 16 antenna ports.

Non-precoded CSI-RS Schemes

In this part, we discuss the two schemes according to CSI-RS locations and patterns with all the above aspects

Scheme1: Reusing Rel-10 CSI-RS resources.

Scheme2: New CSI-RS resources

  - Reusing Rel-10 CSI-RS resources

Option1: Intra Subframe CSI-RS

All ports of CSI-RS RE still in one subframe and CSI-RS RE density is keep to 1 RE/PRB/port, which means 12 ports CSI-RS need 12 RE and 16 ports CSI-RS need 16 RE. In order to provide more RE resources, one straight way forward is allocate more REs.

For example, 16 port CSI-RS can be mixed up with two  8 port CSI-RS configurations, and 12 port CSI-RS can be mixed up with one 8 port CSI-RS REs and one 4 port CSI-RS REs, as shown in the following figure, Red resources represent a 16 ports CSI-RS, green and purple resources are two choices of 12 ports CSI-RS for frame structure type 1.

Figure 1 Intra Subframe, 16 CSI-RS ports, 12 CSI-RS ports

Option2: Inter Subframe CSI-RS

As proposed by some companies in previous RAN1 meetings, it is possible to consider transmission of CSI-RS on multiple-subframes as opposed to over a single subframe in a given CSI-RS transmission period. In this approach CSI-RSs for different antenna ports of the same cell or CSI-RSs across different cells are transmitted in different sub-frames.

The following giving a example of CSI-RS REs across 2 subframes and CSI-RS RE density is keep to 1 RE/PRB/port. 16 ports CSI-RS can be aggregated existing 8 ports CSI-RS resources of two subframes, and 12 port CSI-RS also can be aggregated existing 8 ports CSI-RS resources in a subframes and 4 ports CSI-RS resources in a subframes.

Figure 2 Inter Subframe, 16 CSI-RS ports, 12 CSI-RS ports

CSI-RS ports transmitted in different sub-frames can provide more flexibility in pattern design, and reuse the legacy CSI-RS design.

However, CSI computation based on CSI-RS could be more complex than all CSI-RS ports in one subframe, for it is carried out across multiple subframes.

  - New CSI-RS resources.

Option3:New CSI-RS resources

In Rel-10, CSI-RS port multiplexing is based on OCC2 for each pair of CSI RS port. In order to support 12 port and 16 port CSI-RS, one method is to extend OCC, e.g. OCC=4 as DMRS, as shown in Table 1.

Table 1 OCC used in DMRS

| port number |  |
| --- | --- |
| 7 |  |
| 8 |  |
| 11 |  |
| 13 |  |

In this way, two resource elements is used for 4 CSI-RS ports. 16 CSI-RS ports using FDM4+CDM4 illustrates in the following, and CSI-RS RE density reduced to 0.5 RE/PRB/port .

Figure 3 FDM4+CDM4 for 16 CSI-RS ports

Table 2 summarizes the characteristics of the three alternatives.

Table 2: compare of CSI-RS Schemes

| Alternative | Option1 | Option2 | Option3 |
| --- | --- | --- | --- |
| 12 CSI-RS ports description | one subframe, FDM6+CDM2 | 2 subframes, one subframe is FDM4+CDM2, the other subframe is FDM2+CDM2 | one subframe, FDM3+CDM4 |
| 16 CSI-RS ports description | one subframe, FDM8+CDM2 | 2 subframes, FDM4+CDM2 | one subframe, FDM4+CDM4 |
| CSI-RS density | 1 RE/PRB/port | 1 RE/PRB/port | 0.5 RE/PRB/port |
| Nested structure | Yes | Yes | No |

Conclusions

In this contribution we provided our view on some of aspects in design of 12Tx/16Tx CSI-RS.

- CSI-RS port numbering

- CSI-RS density

- Nested structure

- Number of CSI-RS configurations

Based on the consideration and analyses, three alternatives are proposed:

| Alternative | Option1 | Option2 | Option3 |
| --- | --- | --- | --- |
| 12 CSI-RS ports description | one subframe, FDM6+CDM2 | 2 subframes, one subframe is FDM4+CDM2, the other subframe is FDM2+CDM2 | one subframe, FDM3+CDM4 |
| 16 CSI-RS ports description | one subframe, FDM8+CDM2 | 2 subframes, FDM4+CDM2 | one subframe, FDM4+CDM4 |
| CSI-RS density | 1 RE/PRB/port | 1 RE/PRB/port | 0.5 RE/PRB/port |
| Nested structure | Yes | Yes | No |

- References

- RP-151028, New WID Proposal: Elevation Beamforming/Full-Dimension (FD) MIMO for LTE