3GPP TSG RAN WG1 Meeting #82  		R1-154695

# Beijing, China, 24th – 28th August 2015

Agenda item:		6.2.2

Source:	Huawei, HiSilicon

Title:	TP on summary of the enhanced offloading solutions

Document for:		Discussion/Decision

# Introduction

This contribution provides the summary on the enhanced offloading solutions for the technical report. It is proposed to include the findings for the study item on UMTS NAICS [1].

# Text Proposal

[------------------------------------------------------------- TEXT START --------------------------------------------------------------]

#### 7.3.3 Summary on the enhanced offloading solutions

During the NAICS study, simulation results have shown that at the same location with large interference, the link level performance of a pre-decoding IC UE can be 309%~564% better than that of a Type 3 UE in HetNet scenario, and 38%~251% better than that of a Type 3 UE in HomoNet scenario. With the knowledge of the long term CQI/SINR reflecting link level performance, the network is able to offload UEs with acceptable link level performance to the candidate cell. Range extension can then be applied to UE with advanced receiver according to link level performance after offloading.

Due to the benefit of long term CQI/SINR reporting, several solutions have been proposed to report long term CQI/SINR:

a. SINR measurement report for the serving cell and the candidate cell on RRC signalling [4]

b. Long term CQI report for the serving cell and the second best cell on HS-DPCCH [5]

c. SF-DC CQI report on HS-DPCCH [6]

For solution a, the RRC signalling is reported when a pre-determined criterion is met or reported periodically to the RNC. The SINR is reported from the UE to the RNC directly. For example, when the SINR of the candidate serving cell is higher than a certain threshold set by the network, or when the SINR difference of the serving cell and the candidate serving cell is within a threshold set by the network. The benefit of solution a with pre-determined criterion is the reduced amount of interference in the uplink.

For solution b, long term CQI is firstly reported to the NodeB, then the NodeB reports the long term CQI to the RNC. The long term CQIs are reported with a certain reporting cycle. RNC can determine when to offload the UE with various methods based on the long term CQIs. It is up to the RNC to use any feasible algorithms, including the two criteria mentioned for solution a. The amount of uplink interference can be controlled by using an extended long term CQI reporting cycle. However, the Cell ID introduced in the HS-DPCCH would require additional complexity at the UE. If the second best cell is fixed and known at both the network and the UE, Cell ID can be indicated implicitly to the NodeB and the legacy HS-DPCCH slot format can be reused.

Solution c is very similar to solution b. However, legacy CQI is reported and long term CQI derivation is done at the NodeB. Similar to solution b, the NodeB sends the long term CQI to the RNC and it is up to the RNC to make offloading decisions based on the long term CQIs. Legacy HS-DPCCH slot format is reused. As the UE is always reporting 2 CQIs with a short cycle, uplink interference is to be considered.

In addition, as discussed in [3], further enhancements considering the benefits of solution b and c are also discussed. By reporting long term CQI for candidate serving cell with a long cycle while legacy CQI for serving cell with a short cycle, uplink interference can be reduced, without introducing a new slot format of HS-DPCCH.

In summary, all solutions are able to obtain long term CQI/SINR information at the RNC. It is desired to consider a solution with minimum amount of impact to implementation as well as reduced uplink interference to the network.

[-------------------------------------------------------------TEXT OMITTED------------------------------------------------------------]

# Conclusion

It is proposed to agree to and capture the text proposal presented in this document in the UMTS NAICS TR [2]

# References

- RP-142250, " New Study Item proposal: Study on Network-Assisted Interference Cancellation and Suppression for UMTS ", Huawei, HiSilicon

- 3GPP TR 25.766 v1.0.0, Technical Report on Network-Assisted Interference Cancellation and Suppression for UMTS

- R1-154694, Enhanced offloading based on SF-DC UE capability, Huawei, HiSilicon

- R1-151893, Solution on offloading enhancements for UMTS NAICS, Huawei, HiSilicon

- R1-152084, On enabling offloading of ICS UEs in a network with NAICS support, Ericsson

- R1-153100, Offloading solution for NAICS, Qualcomm Incorporated