3GPP TSG RAN WG1 Meeting #82	R1-153799

Beijing, China, August 24 – 28, 2015

Agenda Item:		7.2.7.2

Source:		Huawei, HiSilicon

Title:		System-level performance of MUST schemes

Document for:		Discussion/Decision

Introduction

At the RAN#81 meeting, the evaluation assumptions regarding to downlink multiuser superposition transmission were agreed [1, 2]. In this contribution, we present the simulation results for SOMA and NOMA [3] with different UE receiver assumptions according to evaluation assumption agreements of the MUST deployment scenario 1.

System-Level simulation assumptions

UE receiver assumptions

The assumptions for the inter-cell interference and inter spatial layer interference receiver for different MUST schemes and the baseline scheme are the same. Hard CW-IC receiver could be applied to both SOMA and NOMA scheme. For SOMA, since the multiple users are superposed with gray mapping, it is also possible to apply ML algorithm instead of CW-IC algorithm. In addition, SOMA performance with ML and hard CW-IC receivers are both presented.

Specific system settings for the simulation

More detailed simulation assumptions are provided in the appendix according to [2], and some of the parameters are highlighted in this section. In the following sections, the simulation results of MUST scenario 1 is presented for 2 TX and 2 RX antenna configurations for transmission mode 4. For the CSI feedback, the existing channel state information (CSI) feedback including RI, PMI, and CQI is used  where wideband PMI/RI and subband CQI are reported to the eNB by the UE. 3km/h UE velocity is assumed for the simulations. For traffic model, both full buffer and burst buffer are assumed; and for the burst buffer traffic, FTP traffic model I is applied and the file size is 100KB. The channel estimation error is also considered.

High level criteria for UE selection and scheduling

Preparation and assumptions

UE proportional fair weighting factor calculation is as wi=1/Ri', where Ri' is the historic average throughput for user i in the past T subframes up to the current TTI. For the scheduling of the current TTI, it is assumed that there are M users in the queue ready to be scheduled and there are S subbands available. For each user m, the wideband PMI is vm and the corresponding reported CQI on subband s is γm,s .

Superposition PF metric calculation

For user m and n pairing, and power allocation factor αj, j=1,2,….J for subband s, the metric Em,n,j,s is calculated as Em,n,j,s=wmRm,j,s+wnRn,j,s if the PMI of the two users are the same, where Rm,j,s and Rn,j,s are the payload could be carried in the subband s for user m and n after pairing with power allocation factor αj. And for each subband and for each user m, the single user scheduling metric is also calculated as Em,s=Rm,0,s/Rm', where Rm,0,s is the payload that can be carrier in the subband s, when only user m is scheduled in the subband.

Users candidates and power allocation selection

For each subband, all the PF values (Em,n,j,s and Rm,0,s) are compared and the highest one is selected as well as the corresponding user(s) candidate and the power allocation candidate. An example of the selection results for one TTI may be given as the following. (Example: user #2 could be far user in subband #1 and single user in subband #2.)

Fig.1 example of the user pairing status

User/power reselection

When the optimum pairing is done, user reselection and power reallocation will be performed according the LTE specification limitations, where the power and near-far status of one UE should be the same for all of its allocated resource blocks. The final scheduling results for the example in Fig.1 are shown in Fig.2.

Fig.2. the final scheduling results for the example above

System level simulation results

In this section, the performance of SOMA and NOMA with hard CW-IC receiver is provided. The performance of SOMA with ML receiver is also presented to further investigate the performance of the MUST schemes.

SOMA and NOMA with hard CW-IC receiver

The simulation results are in the following tables

TABLE I simulation results for SOMA/NOMA with hard CW-IC receiver of full buffer

| Cell average gain (%) | Cell edge gain (%) |
| --- | --- |
| 13.4 | 16.6 |

TABLE II simulation results for SOMA/NOMA with hard CW-IC receiver of burst buffer

| RU (%) | MUST throughput gain (%) |  |  |  | SCT/OCT (%) |
| --- | --- | --- | --- | --- | --- |
|  | Average UPT | 5% | 50% | 95% |  |
| 66.8 | 2.3 | 3.5 | 3.7 | 0.0 | 96.0 |
| 88.1 | 11.1 | 15.9 | 16.2 | 6.0 | 97.2 |

Observation 1:

SOMA and NOMA have the same system level performance when hard CW-IC receiver is applied for both of the schemes due to the same L2S modeling of hard CW-IC for SOMA and NOMA.

Observation 2:

When hard CW-IC is used as the UE receiver for the near UE, ~14% cell average and ~17% cell edge throughput gain are achieved for full buffer traffic.

Observation 3:

For burst buffer traffic model, the gain varies according to the traffic load. Maximally ~11% user average throughput gain and 16% “5% user throughput” gain can be achieved for subband scheduling. The gain increases note worthily as the resource utilization (RU) increases.

Observation 4:

The SCT to OCT ratio for the simulated cases are very close to 1, the reason for the ratio not equal to 1 is that some of the packets are not finished yet before the end of the simulation.

SOMA performance comparison between using ML and hard CW-IC receivers

The simulation results are in the following tables.

TABLE III simulation results for SOMA with ML receiver of full buffer

| Cell avg. gain (%) | Cell edge gain (%) | Cell avg. loss over Hard CW-IC (%) | Cell edge loss over Hard CW-IC (%) |
| --- | --- | --- | --- |
| 12.7 | 16.1 | 0.6 | 0.4 |

TABLE IV simulation results for SOMA with ML receiver of burst buffer

| RU (%) | SOMA throughput gain (%) |  |  |  | SCT/OCT (%) |
| --- | --- | --- | --- | --- | --- |
|  | Average UPT | 5% | 50% | 95% |  |
| 66.5 | 2.7 | 3.7 | 4.5 | 0 | 95.7 |
| 89.5 | 12.8 | 14 | 17.3 | 9.2 | 96.6 |

TABLE V The throughput gain difference between CW-IC and ML receiver for SOMA

| RU (%) | Throughput loss of ML over Hard CW-IC (%) |  |  |  | SCT/OCT (%) |
| --- | --- | --- | --- | --- | --- |
|  | Average UPT | 5% | 50% | 95% |  |
| 66.5 | -0.4 | -0.2 | -0.8 | 0 | 96 |
| 89.5 | -1.5 | 1.6 | -0.9 | -3.0 | 97.2 |

Observation 5:

With the ML receiver for the near UE for superposition coding, the performance is slightly degraded compared with hard CW-IC receiver for full buffer traffic mode. But for the burst buffer case, the scheduling behavior is changed and the loss is not reflected on the UPT peformance. But overall the ML and the hard CW-IC receivers lead to very similar performance for SOMA.

Conclusions

In this contribution, the system evaluation results on MUST schemes SOMA and NOMA are provided. The followings are observed:

Observation 1:

SOMA and NOMA have the same system level performance when hard CW-IC receiver is applied for both of the schemes due to the same L2S modeling of hard CW-IC for SOMA and NOMA.

Observation 2:

When hard CW-IC is used as the UE receiver for the near UE, ~14% cell average and ~17% cell edge throughput gain are achieved for full buffer traffic.

Observation 3:

For burst buffer traffic model, the gain varies according to the traffic load. Maximally ~11% user average throughput gain and 16% “5% user throughput” gain can be achieved for subband scheduling. The gain increases note worthily as the resource utilization (RU) increases.

Observation 4:

The SCT to OCT ratio for the simulated cases are very close to 1, the reason for the ratio not equal to 1 is that some of the packets are not finished yet before the end of the simulation.

Observation 5:

With the ML receiver for the near UE for superposition coding, the performance is slightly degraded compared with hard CW-IC receiver for full buffer traffic mode. But for the burst buffer case, the scheduling behavior is changed and the loss is not reflected on the UPT peformance. But overall the ML and the hard CW-IC receivers lead to very similar performance for SOMA.

- References

3GPP, Draft_Minutes_report_RAN1#81_v020 – final, Japan, May, 2015.

MediaTek, “Summary of System-level Evaluation Assumptions for MUST,” R1-153626, Japan, Apr. 2015.

Huawei, HiSilicon, “Candidate schemes for superposition transmission”, R1-152493, Fukuoka, Japan, May, 2015.

Huawei, HiSilicon, “Link to system modeling for superposition transmission”, R1-151873, Belgrade, Serbia, Apr., 2015.

- Appendix: detailed simulation parameters

| Parameters | Values |
| --- | --- |
| Scenario | MUST Scenario 1 |
| Layout | Hexagonal grid, 3 sectors per site, 19 macro sites |
| System bandwidth per carrier | 10 MHz |
| Carrier frequency | 2.0GHz |
| Total BS TX power | 46dBm |
| Distance-dependent path loss | ITU UMa |
| Penetration loss | For outdoor UEs:0dB For indoor UEs: 20dB+0.5din (din : independent uniform random value between [ 0, min(25,d) ] for each link) |
| Shadowing | ITU UMa |
| Fast fading channel between eNB and UE | ITU UMa |
| Antenna pattern | 3D |
| Antenna Height: | 25m |
| Antenna gain + connector loss | 17 dBi |
| UE antenna Height | 1.5m |
| Antenna gain of UE | 0 dBi |
| Antenna configuration | 2x2, cross-polarized, 0.5-wave length between antenna groups |
| Transmission mode | TM4 |
| Traffic model | Full buffer (10/40UE/Cell) and burst buffer (FTP mode I of 100KB file size) |
| UE dropping | 20% UEs are outdoor and 80% UEs are indoor. |
| UE receiver | In baseline, MMSE-IRC for inter-cell interference suppression MMSE for inter-spatial layer interference suppression for SU-MIMO In MUST, for all users, MMSE-IRC is assumed for inter-cell interference suppression For MUST near users, CW-IC is assumed for intra spatial layer interference cancellation for both SOMA and NOMA, ML is also applied for SOMA for comparison, and MMSE is assumed for inter spatial layer interference suppression for SU-MIMO For other users, MMSE-IRC is assumed for inter/intra spatial layer interference suppression. |
| UE noise figure | 9dB |
| UE speed | 3km/h |
| Unified handover margin | 3dB |
| Overhead | 3 symbols for DL CCHs |
| Network synchronization | Synchronized |
| Performance metrics | Average cell throughput, 5% cell edge throughput |
| Scheduling assumptions | No MU-MIMO is considered for 2X2 antenna configuration |
| Feedback assumptions | Non-ideal CRS channel/interference estimation Release 12 CSI feedback schemes Feedback delay 5ms Feedback periodicity 5ms |
| Receiver impairment modeling for demodulation | Non-ideal CRS channel estimation |
| EVM | TX EVM: 8%, UE RX EVM: 4% |