3GPP TSG RAN WG1 Meeting #83	R1- 156468

Anaheim, USA, November 15-22, 2015

Agenda Item:	6.2.6.1.1

Source:	Huawei, HiSilicon

Title:	Uplink Capacity Evaluation in Standalone Operation

Document for:	Discussion and decision

# Introduction

At RAN1 #82bis meeting, a number of contributions [1]-[3] showed system-level evaluation results based on various DL/UL numerologies with regard to the standalone operation mode. The methodologies defined in [4] were followed by these evaluations and one observation was drawn:

# Observation:

- All the results presented confirm that the NB-IoT solutions under consideration satisfy the capacity and latency targets

In this contribution, further evaluations for uplink capacity are provided taking into account of an additional UL FDMA sub-channel spacing (i.e. 2.5kHz) from our previous contribution [1]. An illustration on the 2.5kHz sub-channel spacing can be found in our companion contribution [5].

# BPL modeling

The BPL modeling results are shown in Figure 1, for both BPL scenarios (see Table D.2 and Table D.3 of [4]) and both agreed correlation coefficients (i.e. 0.5 and 0.75). These results show the CDF of the BPL for all the NB-IoT devices in the simulation, after each NB-IoT device has selected its preferred cell based on minimising the overall path plus penetration loss.

Figure 1. BPL modeling results

# Traffic profile and generation

The traffic profile defined in sub-clauses 5.2.2 [4] is followed. In this document, the information exchange due to the initiation of a MAR (mobile autonomous reporting) periodic or NC (network command) attempt is referred to as a “session”.

Due to the limitation on processing capability of the workstations running simulations, the actual simulation time (denoted by , in seconds) is normally in the order of hundreds or thousands of seconds (although the time needed to run the simulation is normally in the order of days).

The traffic profile is generated as follows,

- The number of MAR periodic sessions generated per sector per day is expressed as:

where  is the number of MSs configured per sector (see Annex E.1 of [4]).

- The total number of NC sessions generated per sector per day is expressed as:

- The total number of sessions generated per sector during the simulation is:

Note that the total number of sessions generated per cell site is .

- It is not clear in [4] how the MAR periodic and NC traffic is distributed over time. In this document it is assumed that they are uniformly distributed over time (i.e. similar to traffic profile 2).

# Other simulation settings

The NB-IoT system is assumed to reuse a single 200 kHz carrier in all cells and is deployed in a frequency reuse of 1/1.  The frequency reuse factor applied in this contribution is different from our previous contribution [1], where a frequency reuse of 1/3 is assumed. An additional 2.5kHz uplink sub-channel spacing as illustrated in our companion contribution [5] is employed.

Among all the available uplink sub-channels, fixed 18 sub-channels are allocated for RACH transmission to simplify the simulation. Note this does not imply any restriction on the flexibility of RACH sub-channel allocation.

The same L2S mapping methodology as in [1] is followed.

The power control mechanism described in subclause 7.3.3.2.2 [4] is adopted in the simulations.

The random access procedure described in subclause 7.3.4.5 [4] is followed in the simulations. The maximum allowed number of random access attempts for each MAR periodic or NC session is set to 5, and the parameter Twait_resend is set to increase linearly with the number of random access attempts.

The MCS is determined for each NB-IoT device such that the highest MCS index is selected, subject to the required SINR being lower than or equal to the NB-IoT device’s average SINR (derived by geometry).

An independent measurement error, according to N(0,3 dB), is applied to each base station. This implies that some users will not select the optimum serving cell, and also not the most appropriate MCS (the one that minimizes resource utilization).

Data transmission and retransmission described in subclause 7.3.4.6.3 [4] are followed.

No frequency hopping is applied in the simulation.

Other simulation assumptions follow Table D.1 in Annex D, which are recapped in Table 2.

# Simulation results

## Simulation cases

The simulation case applied is shown in Table 1.

To determine the maximum capacity of the system, the simulation case is run for a number of offered loads (denoted by “#devices per sector”).

Table 1. Definition of simulation case

| UL modulation | IP header compression | BPL scenario | BPL inter-site correlation coefficient |
| --- | --- | --- | --- |
| GMSK | No | Scenario 2 | 0.75 |

Note 1: Without IP header compression, the protocol overhead above (equivalent of) SNDCP layer is 65 bytes. See Table E.2-3 in [4] for more details. The header overhead of (equivalent of) SNDCP down to MAC (e.g. SNDCP, LLC, RLC/MAC in Gb mode) layer can be estimated to be 15 bytes (4 bytes for SNDCP + 6 bytes for LLC + 2 bytes for MAC + 3 bytes for CRC).

Note 2: BPL scenario 2 is defined in Table D.2 and Table D.3 of [4], respectively.

Suppose the total number of successful uplink reports collected from all cell sites is , the number of simulated cell sites is , and the number of 200 kHz carriers allocated to one cell site is . For the simulation case, the capacity result is given by:

## Results

The capacity is calculated based on counting uplink reports that are successfully delivered, as indicated in section 5.1. The black line represents the “ideal capacity” (i.e. assuming every uplink report is successfully delivered by the system), so is a straight line through the origin with gradient determined by the parameters of the traffic model. The capacity results for both of the two uplink terminologies (i.e. FDMA and SC-FDMA) are shown in Figure 2.

Figure 2. Uplink Capacity

The corresponding uplink report failure probability is shown in Figure 3.

Figure 3. UL report Failure Probability

The distributions for different MCSs are shown in Figure 4 and Figure 5 respectively for FDMA and SC-FDMA based uplinks.

Figure 4. Distribution for different MCSs in FDMA

Figure 5. Distribution for different MCSs in SC-FDMA

From the simulation results, it can be seen that with the introduction of an additional 2.5kHz sub-channel spacing, the FDMA based uplink solution has excellent capacity performance in the standalone operation mode. The capacity target (i.e. 52547 devices within a cell site sector) can be comfortably met and higher capacity is observed by the FDMA based uplink solution compared to the SC-FDMA based uplink solution due to the higher spectrum efficiency as shown in our companion contribution [6].

Observation 1: The FDMA based uplink solution with an introduction of a 2.5kHz sub-channel spacing achieves higher capacity and lower UL report failure probability than the SC-FDMA based uplink solution in standalone operation mode.

# Conclusions

In this document, system level simulation results for the standalone operation mode are provided with an introduction of an additional 2.5kHz uplink FDMA sub-channel spacing. The following observation is made:

Observation: The FDMA based uplink solution with an introduction of a 2.5kHz sub-channel spacing achieves higher capacity and lower UL report failure probability than the SC-FDMA based uplink solution in standalone operation mode.

# References

R1-155967, “System Level Simulation for Capacity and Latency Evaluation”, Huawei, HiSilicon, 3GPP RAN1 #82bis.

R1-156016, “Performance evaluation in stand-alone operation”, Ericsson, 3GPP RAN1 #82bis.

R1-155997, “System simulation of uplink capacity for NB-IoT”, ZTE, 3GPP RAN1 #82bis

3GPP TR 45.820 v2.1.0, “Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things”.

R1-156930, “Uplink FDMA physical layer design updates”, Neul Limited, Huawei Technologies Co., Ltd, HiSilicon Technologies Co. Ltd., 3GPP TSG RAN1 #83.

R1-156466, “Uplink coverage evaluation”, Huawei Technologies Co., Ltd, HiSilicon Technologies Co. Ltd., 3GPP TSG RAN1 #83.

# Appendix

Table 2. Assumptions for system level simulations in standalone operation mode

| No | Parameter |  | Assumption |
| --- | --- | --- | --- |
| 1 | Cellular Layout |  | Hexagonal grid, 19 cell sites, 3 sectors per site with wrap-around |
| 2 | Frequency band |  | 900 MHz |
| 3 | Inter site distance |  | 1732 m |
| 4 | MS speed |  | 0 km/h |
| 5 | User distribution |  | Users dropped uniformly in entire cell |
| 6 | Base station transmit power per 200 KHz (at the antenna connector) |  | 43 dBm |
| 7 | NB-IoT Tx power (at the antenna connector) |  | Max. 23 dBm per uplink physical channel with open loop power control |
| 8 | Path loss model |  | L=I + 37.6log10(R), R in kilometers I=120.9 for the 900 MHz band |
| 9 | Shadowing standard deviation |  | 8 dB |
| 10 | Correlation distance of Shadowing |  | 110 m |
| 11 | Shadowing correlation | Between cell sites | 0.5 |
|  |  | Between sectors of the same cell site | 1.0 |
| 12 | Antenna pattern (horizontal) (For 3-sector cell sites with fixed antenna patterns) |  | See table 5-7, 3GPP TR 45.914, 65° H-plane. |
| 13 | BS antenna gain |  | 18 dBi |
| 14 | NB-IoT device Antenna gain |  | -4 dBi |
| 15 | BS cable loss |  | 3 dB |
| 16 | Building Penetration Loss |  | Based on distributions derived from adapted COST 231 NLOS model. See Annex D.1 of [4]. |