3GPP TSG RAN WG1 Meeting #83	                                                                         R1-156513

Anaheim, USA, 15th – 22th November 2015

Agenda Item:	6.2.3.2

Source: 		Intel Corporation

Title:	On Transmission and Sequence Generation of PSS/SSS/CRS for LAA

Document for: 	Discussion/Decision

# Introduction

It was agreed that DRS consisting of PSS, SSS, CRS and optionally CSI-RS is sent by eNB in LAA systems, subject to LBT, to facilitate the cell detection, synchronization and RRM measurement. The following agreement regarding the transmission of DRS was achieved in RAN1 #82 meeting:

- Subject to LBT, allow the DRS without PDSCH to be transmitted in one of different time positions within the configured DMTC where the DMTC timing is relative to the PCell timing. In this case,

  - Duration of the signals in the DRS (which doesn’t include potential initial signal) is less than 1 ms (a single idle sensing interval is used for the LBT procedure prior to transmission of the DRS)

    - FFS: Duration of the DRS occasions when the duration is less than 1 ms

    - FFS: case where duration of the signal in DRS is 1 ms or longer

  - Allow DRS transmission in subframe other than #0 and #5

  - Position of DRS in a subframe is the same for all candidate subframes in the DMTC

  - Following alternatives can be considered for sequence generation for SSS/RS in subframes with DRS

    - Alt. 1: Generate SSS/RS sequences according to the subframe index of DRS

    - Alt. 2: Generate SSS/RS sequences irrespective of the subframe index of DRS

    - FFS: other alternatives

In addition, it has been agreed in RAN1 #82bis meeting that every subframe 0 and 5 that is transmitted by eNB should contain Rel-12 PSS/SSS/CRS with the aim of improving the cell detection and synchronization performance [1]. Detailed agreements achieved in RAN1 #82bis meeting are:

- Every subframe 0 and 5, when transmitted by the eNB, should contain Rel-12 PSS/SSS/CRS where PSS/SSS/CRS in the LAA DRS is a subset of these signals.

  - Number of CRS ports should be the same or higher than the number of CRS ports in the DRS.

  - FFS: Partial TTI in SF0 and SF5

  - FFS: Changes to scrambling for SSS/CRS

  - Note: A UE is only expected to detect and measure cells transmitting DRS during the configured DRS DMTC window (from RAN2 agreements)

- The PSS/SSS/CRS in LAA DRS is a subset of Rel-12 PSS/SSS/CRS.

  - FFS: Changes to scrambling for SSS/CRS

In this contribution, we discuss some of the above FFS points. In particular, the following issues are discussed in this contribution:

- The PSS/SSS/CRS transmission in partial subframes;

- The sequence generation of SSS/CRS transmitted in the DRS and subframes #0 and #5.

# PSS/SSS/CRS Transmission in Partial Subframes

In this section, we first give a brief introduction of the partial subframe in LAA systems, and then discuss the PSS/SSS/CRS transmission related issues raised from the introduction of partial subframes.

In RAN1 #82 meeting, it was agreed that partial TTI would be considered as an option for PDSCH transmission in a DL subframe on an LAA SCell. The rationale behind the adoption of partial subframe in LAA systems is as follows. An LBT can finish any time in between PCell subframe boundaries. If such PCell subframe boundary alignment restriction is enforced, the interval from the end of the successful LBT to the start of the following PCell subframe boundary would be wasted, as no data is transmitted by the eNB during this interval. To improve the resource utilization efficiency, the notion of partial subframes has been introduced in the LAA system, whose TTI is less than 1 ms. As discussed, the first subframe of a DL transmission burst can be partial as LBT can finish any time in between the subframe boundaries.  The last subframe of a DL transmission burst can also be partial if the burst needs to be shortened according to the maximum channel occupancy time (MCOT) limit.

Recall that it has been agreed in RAN1 #82bis that the transmission in every subframe #0 and #5 should contain Rel-12 PSS/SSS/CRS. As illustrated in Figure 1, SSS and PSS are transmitted at symbol #5 and symbol #6, respectively, in every subframe #0 and #5. CRS from antenna ports #0 and #1 is transmitted at symbols #0, #4, #7 and #11, while CRS from antenna ports #2 and #3 is transmitted at symbols #1 and #8. Note that CRS can be transmitted in subframes other than subframes #0 and #5. Figure 1 shows the CRS transmission with antenna ports #0 and/or #1 in subframes #0 and #5 for illustrative purpose.

Figure 1. Illustration of PSS/SSS/CRS location in subframes #0 and #5 in LAA systems

It has been agreed in RAN1 #82bis meeting that the ending partial subframe is configured based on the existing Downlink Pilot Time Slot (DwPTS) design, with possible ending positions limiting to symbols {2, 5, 8, 9, 10, 11}.

When the subframe #0 or #5 is partial, the PSS/SSS/CRS may not be transmitted based on the length of the partial subframe. For example, if the partial subframe consists of 3 symbols, the PSS, SSS and part of CRS cannot be transmitted. Thus, the PSS/SSS/CRS transmission may follow the options below.

- Option 1: PSS/SSS/CRS is transmitted only in regular subframes. If the subframe is partial, no PSS/SSS/CRS would be transmitted.

- Option 2: PSS/SSS/CRS is transmitted in partial subframes, as long as the length of partial subframe is long enough to carry these signals.

  - PSS/SSS option 2a: PSS and SSS can be enforced to be transmitted together. Thus, if the first subframe of a DL data burst is subframe #0 or #5, which starts at symbol #5 or earlier, the subframe will contain both PSS and SSS. On the other hand, if the first subframe is subframe #0 or #5, which starts at symbol #6 or later, then the subframe does not contain PSS or SSS. Similarly, when the last subframe of a DL data burst is subframe #0 or #5 and ends at symbol #6 or later, the subframe contains both PSS and SSS; otherwise, neither PSS nor SSS would be transmitted in this subframe.

  - PSS/SSS option 2b: PSS and SSS can be transmitted separately. Thus, if the first subframe is subframe #0 or #5 and starts at symbol #6, PSS can be transmitted without SSS. Similarly, if the last subframe is subframe #0 or #5 and ends at symbol #5, SSS can be transmitted without PSS.

  - CRS option 2a: All defined CRS symbols can be enforced to be transmitted together. Thus, if the first subframe is partial, the CRS cannot be transmitted.

  - CRS option 2b: A part of CRS can be transmitted as long as it fits into the partial subframe. For example, if the first subframe starts at symbol #3, the CRS from antenna ports #0 and #1 in symbols #4, #7 and #11, and the CRS from antenna ports #2 and #3 in symbols #8 can be transmitted.

It is preferred to support Option 2, particularly, PSS/SSS option 2b and CRS option 2b, which can increase the PSS/SSS/CRS transmission opportunities and thus help improve the related performance, e.g., the accuracy of synchronization and CSI measurement. Furthermore, PSS/SSS option 2b and CRS option 2b do not change rate matching pattern for PDSCH transmission from the legacy LTE except for shortened resources of partial subframes.

Proposal 1: PSS/SSS/CRS are transmitted in partial subframes, as long as the partial subframe covers the symbol locations to transmit PSS/SSS/CRS.

# SSS/CRS Sequence Generation

As agreed in RAN1 #82 meeting, the DRS without PDSCH is allowed to be transmitted in one of different time positions within the configured DMTC subject to LBT. Thus, the DRS consisting of PSS, SSS, CRS and optionally CSI-RS may be transmitted in subframes other than subframes #0 and #5. On the other hand, the Rel-12 SSS is only defined for subframes #0 and #5. Thus, SSS generation for subframes others than #0 and #5 needs to be defined. We also discuss about the corresponding CRS generation in this section.

SSS sequence generation

We provide the potential SSS generation methods as follows.

- SSS-option 1: An identical SSS sequence can be transmitted in all subframes, e.g., the Rel-12 SSS transmitted in subframe #0 can be used as the identical SSS in DRS in LAA systems independent of the subframe index.

- SSS-option 2: SSS sequence transmitted in subframe #0 in Rel-12 systems can be used as the SSS to be transmitted in subframes {0, 1, 2, 3, 4}, while SSS sequence transmitted in subframe #5 in Rel-12 systems can be used as the SSS to be transmitted in subframes {5, 6, 7, 8, 9} in LAA systems.

- SSS-option 3: New SSS sequences that depend on the subframe index can be designed.

SSS-option 1 is simpler compared to other options in the sense of UE complexity, but it needs changes in the existing SSS transmitted in either subframe #0 or #5. SSS-option 3 may lead to high complexity at UEs, as UEs may need to perform five times more hypotheses tests compared to the legacy LTE system, particularly for the detection of SCells that are not synchronized with UEs’ current PCell. Thus, SSS-option 2 is preferred for SSS generation.

CRS sequence generation

The possible CRS generation methods in LAA systems are given as follows.

- CRS-option 1: An identical CRS sequence can be transmitted in all subframes, e.g., the Rel-12 CRS transmitted in subframe #0 can be used as the identical CRS in DRS in LAA systems.

- CRS-option 2: Rel-12 CRS can be used in LAA systems, where the sequence generation depends on the subframe index.

- CRS-option 3: The CRS generation in subframes containing PSS/SSS follows the subframe index used for generating SSS. Specifically, subframes where Rel-12 SSS for subframe #0 is transmitted would include Rel-12 CRS generated for subframe #0, while subframes where Rel-12 SSS for subframe #5 is transmitted would include Rel-12 CRS generated for subframe #5.

CRS-option 1 looks simple in terms of UE complexity, as UEs do not need to perform hypothesis test. However, a verification by using CRS whether or not to successfully detect SSS sequences between subframe#0 and #5 is not possible. CRS-option 2 can be applied to the cases where the subframe index is known at UEs, e.g., by referencing the timing of PCell. On the other hand, in scenarios where UEs need to detect SCell which is not synchronized with UEs’ PCell, CRS-option 2 would lead to high complexity at UEs since UEs need to perform much more hypothesis tests to determine the transmitted CRS sequence. As for CRS-option 3, it is also simple and the UE complexity would be the same as CRS-option 1. In addition, it can provide a function of verification between SSS sequence detection (implementation choice though). So from the perspective of UE complexity, CRS-option 1 and CRS-option 3 are preferable, but CRS-option 3 provides more implementation benefit. Therefore, it is desirable to support CRS-option 3.

Proposal 2: SSS/CRS sequence generation in DRS follows

- SSS sequence transmitted in subframe #0 in Rel-12 is used as the SSS to be transmitted in subframes #0, #1, #2, #3, and #4, while the SSS sequence transmitted in subframe #5 in Rel-12 is used as the SSS to be transmitted in subframes #5, #6, #7, #8 and #9 in LAA systems.

- The CRS generation in subframes containing PSS/SSS follows the subframe index used for generating SSS. Specifically, CRS transmitted in subframes containing Rel-12 SSS for subframe #0 would be generated using subframe index 0, while CRS transmitted in subframes containing Rel-12 SSS for subframe #5 would be generated using subframe index 5.

# Conclusion

In this contribution, we have discussed the PSS/SSS/CRS transmission in partial subframes and sequence generation in DRS of LAA systems. Based on the discussion, we have made the following proposals.

Proposal 1: PSS/SSS/CRS are transmitted in partial subframes, as long as the partial subframe covers the symbol locations to transmit PSS/SSS/CRS.

Proposal 2: SSS/CRS sequence generation in DRS follows

- SSS sequence transmitted in subframe #0 in Rel-12 is used as the SSS to be transmitted in subframes #0, #1, #2, #3, and #4, while the SSS sequence transmitted in subframe #5 in Rel-12 is used as the SSS to be transmitted in subframes #5, #6, #7, #8 and #9 in LAA systems.

- The CRS generation in subframes containing PSS/SSS follows the subframe index used for generating SSS. Specifically, CRS transmitted in subframes containing Rel-12 SSS for subframe #0 would be generated using subframe index 0, while CRS transmitted in subframes containing Rel-12 SSS for subframe #5 would be generated using subframe index 5.

# References

- R1-156386, “RAN1 Agreements and Working Assumptions for Rel-13 LAA,” 3GPP TSG RAN WG1 Meeting #82bis, WI rapporteur, October, 2015.