3GPP TSG RAN WG1 Meeting #83	                                                                         R1-156514

Anaheim, USA, 15th – 22th November 2015

Agenda Item:	6.2.3.3

Source: 		Intel Corporation

Title:	Performance Evaluation of CRS Presence Detection

Document for: 	Discussion/Decision

# Introduction

It was agreed that DRS consisting of PSS, SSS, CRS and optionally CSI-RS is sent by eNB in LAA systems, subject to LBT, to facilitate the cell detection, synchronization, and RRM measurement [1]. As agreed in RAN1 #82 meeting, the DRS without PDSCH is allowed to be transmitted in one of different time positions within the configured DMTC subject to LBT. Thus, there are multiple potential time positions for DRS transmission within the DMTC and UE needs to blindly detect the presence of DRS. One method for the DRS presence detection is to leverage the CRS in DRS.

In addition, CRS based detection method can be used for the DL burst detection, including the detection of subframe on/off (i.e., the signal presence in the subframe) and the distinction of MBSFN/non-MBSFN subframes. In legacy LTE systems, MBSFN subframes are semi-statically configured via higher layer signalling. On the other hand, the resource utilization in LAA SCell would be more dynamic due to LBT. Thus, it may be preferable to have a more dynamic configuration for MBSFN subframes in LAA SCell. The CRS port 0/1 is transmitted only in symbol #0 within the MBSFN subframe (i.e., no CRS transmission in MBSFN region of the MBSFN subframe), while CRS port 0/1 is transmitted in symbols {0, 4, 7, 11} within the non-MBSFN subframes. Thus, the CRS based detection is one potential method for distinction of MBSFN/non-MBSFN subframes by using CRS presence detection between MBSFN region of the MBSFN subframe and non-MBSFN subframe (other than the first OFDM symbol).

In this contribution, we evaluate the link level performances of the following aspects:

(1) CRS based detection for DRS presence;

(2) CRS based subframe on/off detection;

(3) CRS based MBSFN/non-MBSFN differentiation.

# Performance Evaluation of CRS Based Detection

## Simulation Assumptions

- System bandwidth: 20MHz

- 1 Antenna port (CRS port 0)

- 2 receiver antennas

- Channel model: EPA-5Hz

- Target false-alarm probability (Pfa): 1%

## Simulation Results for DRS Presence Detection

In this section, we present the evaluation result for the DRS presence detection. The false-alarm probability (Pfa) refers to the probability that UE detects DRS presence when no signal is transmitted. The missing probability (Pmissing) is defined as the probability that UE determines no DRS presence when the DRS is transmitted, while the detection probability (Pdetection) is the probability that UE correctly detects the presence of DRS. As illustrated in Figure 1, the SNR required to achieve detection probability larger than 99% and 99.9% are -6.5dB and -5dB, respectively.

Figure 1. The performance of CRS based method for DRS presence detection

## Simulation Results for Subframe On/Off Detection and MBSFN/Non-MBSFN Differentiation

In this section, the simulation results for DL burst detection is presented. Figure 2 shows the result of MBSFN subframes, while Figure 3 illustrates the result of non-MBSFN subframes. The following performance metrics are plotted.

- Detection probability (Pdetection): the probability that UE correctly detects whether the subframe is on/off and the type of the subframe if it is on.

- Missing probability (Pmissing): the probability that there is a transmission in the subframe, but UE determines that no signal is present in the subframe.

- Probability of incorrect detection of subframe types (PMBSFNnon-MBSFN, or Pnon-MBSFNMBSFN): the probability that MBSFN subframe is detected as non-MBSFN subframe, or the non-MBSFN subframe is detected as MBSFN subframe. This is a conditional error probability, given that the subframe is detected to be on (i.e., with signal presence).

- Error probability (Perror): the probability which includes the missing probability and the probability of incorrect detection of subframe types.

- False-alarm probability (Pfa): the probability that no signal is transmitted in the subframe, but UE detects the presence of the signal.

Subframe on/off detection

The missing probability (Pmissing) characterizes the performance of subframe on/off detection. Specifically, the missing probability in Figure 2 refers to the probability that UEs determines no signal presence when a MBSFN is transmitted, and the missing probability in Figure 3 refers to the probability that UE determines no signal presence when a non-MBSFN subframe is transmitted. From Figures 2 and 3, we can observe that the SNR required to achieve subframe on/off detection probability larger than 99% in MBSFN and non-MBSFN subframes are -3.5dB and -6.5dB, respectively.

MBSFN/Non-MBSFN differentiation

The probability of incorrect detection of subframe types (PMBSFNnon-MBSFN, or Pnon-MBSFNMBSFN) characterizes the performance for MBSFN and non-MBSFN differentiation, given that the signal is detected to be present. As illustrated in Figures 2 and 3, given that the signal presence is detected, the SNR required to achieve correct differentiation between MBSFN and non-MBSFN are -5dB and -8dB for MBSFN and non-MBSFN subframes, respectively.

The performance including the subframe on/off detection and the MBSFN/non-MBSFN differentiation is characterized by detection probability (Pdetection). We can observe from Figures 2 and 3 that the SNR required to achieve the detection probability larger than 99% in MBSFN and non-MBSFN subframes are -2.8dB and -6dB, respectively. The performance of non-MBSFN subframes is better than the performance of MBSFN subframes. This is because the non-MBSFN subframe includes more CRS symbols, which can be exploited to improve the detection performance.

Figure 2. The performance of CRS based method for MBSFN subframe detection

Figure 3. The performance of CRS based method for non-MBSFN subframe detection

# Conclusion

In this contribution, we have evaluated the performance of CRS based detection method for DRS presence detection, subframe on/off detection and MBSFN/non-MBSFN differentiation. Based on the discussion, the following conclusion is drawn.

Observation: CRS based detection method can provide sufficient accuracy for the DRS presence detection, subframe on/off detection and MBSFN/non-MBSFN differentiation.

# References

- 3GPP TR 36.889, “Study on Licensed-Assisted Access to Unlicensed Spectrum”.