3GPP TSG RAN WG1 Meeting #83	                                                                         R1-156516

Anaheim, USA, 15th – 22th November 2015

Agenda Item:	6.2.3.3

Source: 		Intel Corporation

Title:	On the LAA DL Signalling

Document for: 	Discussion/Decision

# Introduction

In RAN1 #82bis meeting [1], the following working assumptions on the CRS and CSI-RS transmission power have been made:

- UE may assume that the CRS and CSI-RS transmission has a constant power in each subframe of DL transmission burst, and the UE should not assume that the CRS and CSI-RS transmission power is the same across transmission bursts

  - FFS: Whether there is any problem with AGC

  - FFS: Whether there is an issue with CSI measurements due to potential mismatch in UE’s measured subframe and eNB’s understanding of the measured subframe

  - FFS: How the UE identifies whether subframes belong to the same transmission burst

    - Note: The following options and other potential options for such identification are not precluded.

    - Such identification is left up to UE implementation.

    - Such identification is provided via assistance from the eNB.

Under the above working assumptions, it is desirable for UEs to be able to identify which subframes belong to a single DL burst, as CRS and CSI-RS transmission power is constant within a DL burst while it can change across different bursts. The DL signalling for the distinction of DL transmission bursts is discussed in this contribution.

Besides the issue related to DL burst distinction, another aspect which may need DL signalling is the indication of partial subframes. In RAN1 #82 meeting, it was agreed that partial subframe is considered as an option for PDSCH transmission on an LAA SCell. In addition, it was agreed in RAN1 #82bis meeting that [1]:

- DL transport block in the last subframe of a DL Tx burst can be transmitted using DwPTS structure, or a full subframe

  - FFS how to signal the structure of the last subframe

  - FFS whether to define a 13-symbol partial subframe

  - FFS whether DwPTS structure with 3 OFDM symbols can be used for the last subframe

  - FFS down selection

UEs associated with an LAA SCell are required to determine the presence of partial subframe for the purposes of PDSCH decoding and CSI reporting.

In this contribution, we discuss the LAA DL signalling that may help to distinguish the DL transmission bursts and indicate the partial subframes within the DL transmission burst.

# Signalling for DL Burst Distinction

At the RAN1 #82bis meeting, it was agreed that CRS and CSI-RS transmitted in the same DL burst can be assumed to have a constant power, but CRS and CSI-RS transmitted across DL bursts may have different transmission power. Therefore, it is desirable for UEs to be able to identify which subframes belong to a single DL burst. We provide possible indication methods as follows, where the discussed indication information needs to be signalled to all UEs associated with the LAA SCell.

- Option 1: Indicate both or either starting/ending subframe of the DL burst. Figure 1 shows an example to indicate the starting subframe of the DL burst via 1 bit information. No indication information in a subframe implies that the subframe is not the starting subframe of a DL burst.

Figure 1. Illustration of a method to indicate the starting subframe of each DL transmission burst, where the indication information is included in every starting subframe within the DL transmission burst. The red text in the figure is what included in the indication information.

- Option 2: Introduce a toggling information to indicate if the subframes belong to the same DL burst, where the same toggling information is carried in subframes within the same transmission burst, and different toggling information is carried in subframes belonging to the neighboring transmission bursts. Figure 2 illustrates an example with 2 bits toggling information transmitted in every subframe within the DL burst.

Figure 2. Illustration of the toggling method to distinguish DL bursts. The red text in the figure is what included in the indication information. 2 bits toggling information is used in this example.

- Option 3: Indicate the number of expected remaining subframes within the DL burst in every subframe. In this option, 4 bits can be sufficient for up to 16 ms TXOP.

- Option 4: Indicate the total number of symbols within the DL burst at the starting subframe of the DL burst. In this option, 8 bits can be sufficient for up to 18 ms TXOP.

- Option 5: Indicate the number of remaining symbols within the DL burst in every subframe, or in a subset of subframes within the DL burst (e.g., in every other subframe).

We can observe that Option 1 needs the least overhead among the above five options. The overhead needed for Options 2-5 depends on the length of the DL burst. For a DL burst with long duration, Option 4 may have less overhead than Options 2, 3 and 5. However, Options 1 and 4 may not be able to provide performance as reliable as the other options, since UEs cannot distinguish DL bursts if the starting subframe of a DL burst is missed. Thus, Options 2, 3 and 5 are preferable. Among these three options, it is preferred to support Option 2, as it requires less overhead than Options 3 and 5.

The signalling method for the indication information will be discussed in the following section.

Proposal 1: The distinction of DL bursts can rely on an indication based on the toggling information, where the same toggling information is transmitted in subframes within the same DL burst and different toggling information is transmitted in subframes across DL bursts.

# Indication of the Partial Subframes of the DL Burst

To improve the resource utilization efficiency, the notion of partial subframes has been introduced in the LAA system, whose TTI is less than 1 ms. The first subframe of a DL transmission burst can be partial as LBT can finish any time in between the subframe boundaries. The last subframe of a DL transmission burst can also be partial, e.g., if the burst needs to be shortened according to the maximum channel occupancy time (MCOT) limit. UEs associated with an LAA SCell are required to determine the presence of partial subframes for the purposes of PDSCH decoding and CSI reporting. We discuss the indication of the first and the last partial subframes of a DL burst as follows.

Indication of the first partial subframe of a DL burst

According to the discussion in [2], we focus on slot-based partial subframe for the first subframe of a DL burst in this contribution. Specifically, the slot-based partial subframe limits the first subframe of a DL burst to be either a normal subframe consisting of 2 slots or a partial subframe starting at the 2nd slot. We discuss about the indication of the first partial subframe with both self-carrier scheduling and cross-carrier scheduling.

- Self-carrier scheduling: No explicit indication is needed. The starting position can be reliably detected by CRS based detection, particularly with a typical 20 MHz bandwidth LAA operation as observed in [3].

- Cross-carrier scheduling: As starting partial subframe cannot be transmitted with cross-carrier scheduling, no indication is needed.

Indication of the last partial subframe of a DL burst

Recall that it has been agreed in RAN1 #82bis meeting that the last partial subframe of a DL burst can use DwPTS structure. In other words, the possible length of the last subframe can be {3, 6, 9, 10, 11, 12, 14} symbols, where the duration of 14 symbols corresponds to the normal subframe. The possible indication methods for the last partial subframe of a DL burst are given as follows, which can be applied to both self-carrier and cross-carrier scheduling.

- Option 1: Indicate the subframe length in every subframe of the DL burst. As the possible length of the last subframe is {3, 6, 9, 10, 11, 12, 14}, 3 bits are needed for the length indication in every subframe. Figure 3 shows an example of this indication method.

Figure 3. Illustration of a method to indicate the length information in every subframe. The red text in the figure is what included in the indication information.

- Option 2: Indicate the length information of the subframe only if the subframe is partial, as illustrated in Figure 4. In this method, the subframes without indication of the length information are normal subframes. This method requires 3 bits per partial subframe.

Figure 4. Illustration of a method to indicate the length of the last partial subframe within each DL transmission burst, where the subframes without indication information are normal subframes. The red text in the figure is what included in the indication information.

As all UEs need to know the presence of partial subframe assuming that CRS/CSI-RS transmission is dependent on the subframe length, the above information should be indicated to all UEs associated with the SCell. For the signalling overhead, Option 2 requires less overhead than Option 1.

Signalling method for the indication information

To signal the indication information for DL burst distinction and/or partial subframe indication, DCI based method can be used, where DCI within (e)PDCCH can carry the indication information and a common search space for SCell can be defined.

Proposal 2: The following methods can be considered for the indication of partial subframes.

- The first partial subframe of a DL burst does not need explicit indication.

- The last partial subframe of a DL burst may be indicated via the following methods.

- Option 1: Indicate the length information in each subframe within the DL burst.

- Option 2: Indicate the length information in partial subframes only, while no indication information is transmitted in normal subframes.

# Conclusion

In this contribution, we have discussed the methods for the distinction of DL bursts, and for the indication of start/end/length of the DL burst. Based on the discussion, we propose to introduce the following indication and signalling methods.

Proposal 1: The distinction of DL bursts can rely on an indication based on the toggling information, where the same toggling information is transmitted in subframes within the same DL burst and different toggling information is transmitted in subframes across DL bursts.

Proposal 2: The following methods can be considered for the indication of partial subframes.

- The first partial subframe of a DL burst does not need explicit indication.

- The last partial subframe of a DL burst may be indicated via the following methods.

- Option 1: Indicate the length information in each subframe within the DL burst.

- Option 2: Indicate the length information in partial subframes only, while no indication information is transmitted in normal subframes.

# References

- R1-156386, “RAN1 Agreements and Working Assumptions for Rel-13 LAA,” 3GPP TSG RAN WG1 Meeting #82bis, WI rapporteur, October, 2015.

- R1-156515, “On the starting and ending positions for DL LAA burst,” Intel Corporation.

- R1-156514, “Performance evaluation of CRS presence detection,” Intel Corporation.