3GPP TSG RAN WG1 Meeting #83		                                                       R1-156602

Anaheim, USA, 15th - 22th November 2015

Source:	CATT

Title:	On synchronization issues in PC5-based V2V

Agenda Item:	6.2.8.1.3

Document for:	Discussion and Decision

Introduction

In SA1 #70 meeting, 3GPP TR 22.885 [1] is updated to V0.2.0, there are a number of use cases of V2X. Base on the use cases, some of common requirements (mainly for V2V) are as follows:

- The RV V2V Service layer periodically broadcasts a message, indicating its current position, speed, acceleration and optional estimated trajectory.

- The E-UTRA(N) shall be able to support a maximum frequency of 10 V2V messages per second.

- The E-UTRA(N) shall be able to support high mobility performance(e.g. a maximum absolute velocity of 160 km/h).

In RAN1 #82meeting [4], it was agreed that the PC5's work carrier is up to 6G Hz. There are higher requirements to V2V compared to R12 D2D: higher capacity, higher speed and higher Carrier frequency.

Discussion

  - Timing and frequency offset in PC5-based V2V

In D2D there are four basic scenarios: within the same eNB coverage, inter-eNB, partial coverage and out of coverage.

| The same eNB coverage | Inter-eNB |
| --- | --- |
| Partial coverage | Out of coverage |

Figure. 1 Scenarios of D2D

The basic scenarios are the same in V2V. So the synchronization performances of D2D system in the scenarios are analyzed with high speed and high frequency as following. The large receiving timing and frequency offset of D2D system exist.

- The same eNB coverage：

Figure. 2 The same eNB coverage

In this scenario, transmission timing and receive timing at UE are the same as DL timing from eNB. In Figure 2, di indicates the distance from UEi to eNB and dij indicates the distance from UEi to UEj.

TTUE  is the maximum sending timing offset between two terminals UEi and UEj in D2D system.

TTUE=2*tss+di-djc		                            		                                                       (1)

The receiving timing offset range of UEj from the received signal of UEi is:

TRij=±2*tss+di-djc+dijc		                             		                           (2)

tss is the maximum estimation error for the receiving timing of synchronization source，setting tss=500ns  according to TA granularity. C is speed of light. Assuming that the maximum communication distance between the terminals is 300m, TRij∈-2,3μs.

For the convenience of description, we use vi indicating the velocity of UEi and vij indicating the relative velocity between UEi and UEj. Let fUE  to be the maximum sending frequency offset between two terminals UEi and UEj served by the same eNB.

FTUE=±2*fss+vi+vjc*f0				                                                        (3)

The receiving frequency offset range of the UEi signal received by the UEj is:

FRij=±2*fss+vi+vjc+vijc*f0                                                                                                     (4)

fss is the maximum frequency offset between the modulated carrier frequency and the carrier frequency received from the synchronization source and fss is assumed as 0.1ppm [2]. f0 is carrier frequency by setting 6G Hz. When maximum velocity of UE is 160km/h, FRij belongs to -4756,4756Hz.

- Inter-eNB

Figure. 3 Inter-eNB

In this scenario, transmission timing at UE is the same as DL timing at eNB while receive timing follows SLSS sent from UE at neighbouring cell.

The TTUE  will get larger when UEi and UEj belong to different eNBs.

TTUE=2*tss+TNB+di-djc		                                                           		(5)

TRijx=±2*tss+TtNB+di-djc+dix-djxc                                                                                 	(6)

TNB is sending timing offset of different eNBs and TNB equals to 3us[7] for TDD-LTE. TRijx is the receiving timing offset of UEi and UEj at UEx, and in Figure 3 UEx is UE3.

So, TRijx belongs to-6,6μs for synchronized network and TRijx may have larger range for asynchronized network.

Considering the frequency deviation between eNBs, the sending frequency offset of UEs from different eNBs is larger.

FTUE=±2*fss+2*FNB+vi+vjc*f0	               	                                                        (7)

FRijx is the receiving frequency offset of UEi and UEj at UEx

FRijx=±2*fss+2*FNB+vi1+vjc+vix++vjxc*f0                                                                        (8)

FNB is maximum frequency offset between sending frequency of eNB and f0, and FNB is equal to 0.05ppm[2]. FRijx belongs to -7133,7133Hz。

Observation 1: when there are network coverage, following D2D existing synchronization mechanism, receiving time and frequency offset from multiple Tx UEs would be large since UE always selects eNB including asynchronized eNBs as synchronization source. It may cause demodulation failure unless resource pools are configured different while resource pools division would limit capacity in V2V case, or eNB informs UE timing difference information.

Observation 2: No matter eNB synchronized to GNSS or not, above issue would happen if the existing D2D synchronization mechanism is directly reused.

- Partial coverage

Figure. 4 Partial coverage

The receiving timing offset range of two UEs which belong to the different timing clusters sending signals to the UEx gets larger.

TRmnx=±4*tss+TNB+di+din-dj-djmc+dmx-dnxc                                                                         (11)

So, TRmnx  belongs to -7,7μs for TDD-LTE and tmnx maybe have larger range for FDD-LTE.

FRmnx=±4*fss+2*TNB+v1i+vjc+vjm++vinc+vmx++vnxc*f0                                             (12)

So, FRmnx belongs to-8333,8333Hz

Observation 3: Since in-coverage resource pools are not forwarded to out of coverage, in partial coverage, demodulation impact will be server because of large time and frequency difference between multiple Tx UEs.

- Out of  coverage

In D2D system, there is no global synchronization source and no convergence mechanism of different timing clusters, and hence it is possible that there are different timing clusters out of coverage.

The receiving timing offset range of two UEs which belongs to the different clusters may be larger than -6,6μs. The receiving frequency offset range of two UEs which belong to the different clusters maybe lager than -7133,7133Hz .

Observation 4: Without global synchronization, time and frequency offset from multiple Tx UEs to a receiving UE would be even larger compared with the case having eNB as synchronization source.

  - Synchronization issues in PC5-based V2V

Based on the above analysis, in the same synchronization cluster, the receiving time deviation between the terminals of which sending timing is directly from the base station belongs to -2,3 μs , and the receiving time deviation between the terminals of which is once forwarded by other terminals belongs to-3,4 μs. In Fig 6, discovery signal in D2D is evaluated to show impact from timing offset.

Table 1：Simulation assumptions

| Simulation Parameters | Values |
| --- | --- |
| UE antenna configuration | 1 TX 2 RX |
| Channel model | ITU UMI NLOS |
| Bandwidth | 10 MHz |
| Discovery signal format | 1RB PUSCH with two slots |
| Multiple access type | SC-FDMA |
| Modulation type | QPSK |
| Coding Rate | 0.5 |
| UE mobile speed | 3km/h |

Figure. 5 Discovery probability with PUSCH (QPSK,1/2)

Base on simulation result in Figure 6, when receiveing timing offset is within [-1,6]us the demodulation performance is almost unaffected. In otherword, when the timing offset range is less than 7us, the the timing offset is acceptable. So the receiving timing offset of UEs which get sending timing at most once forwarded by other UEs in the same cluster is acceptable. But the receiving timing offset of UEs which are in different cluster is out of control and lead to some problems：

- Timing discontinuity

- Resources waste due to using different resource pool corresponding to different timing offsets

- Large timing offset cannot allow FDM in the same subframe or even adjacent subframes

- Resource overhead of sending synchronization signal and channel

- Multiple clocks and frequency tracking process at Rx UE

Also when different offsets from multiple transmitters arrive the same receive UE in the same subframe (e.g. FDM), it would lead to both ISI and ICI.

Observation 5: Synchronization needs to be enhanced with reducing the number of synchronous clusters by applying GNSS to eNB and UE and using global timing as much as possible as long as it does not affect cellular network.

Since it is V2V, UE is supposed to be equipped with GPS while it is possible GPS signal would be lost in some case, e.g. in a long distance tunnel. For those cases, UE-UE synchronization is still needed, that would be similar as that in D2D. Details can be seen in [6].

Also, even eNB is synchronized to GNSS and in the same eNB coverage, due to high carrier frequency and high mobile speed, the receiving frequency offset can also reach 4756 Hz. The same situation happens when Tx UEs without GPS signal received are synchronized to the same UE with GPS. In different clusters, there will be a higher receiving frequency offset. Those issues coming from high Doppler and high carrier frequency should be considered when enhancing DMRS, as discussed in [5].

When cellular traffic is co-channel with PC5-based V2V, even eNB is not synchronized to GNSS, eNB may need to instruct UE to use eNB or GNSS as higher synchronization source since it may impact the ongoing cellular traffic. While when it is not co-channel case, particularly when cellular traffic and PC5-based V2V happen in different frequency bands, it needs further consider to achieve global timing with higher priority on GNSS as synchronization source. The details are discussed in [6].

Observation 6. It is needed to further discuss the synchronization source priority at least when UE is served by eNB but cellular traffic and PC5 V2V traffic happen in the different frequency bands.

Conclusion

In this contribution, we discuss the synchronization issues t in PC5-based V2V. Observations are:

Observation 1: when there are network coverage, following D2D existing synchronization mechanism, receiving time and frequency offset from multiple Tx UEs would be large since UE always selects eNB including asynchronized eNBs as synchronization source. It may cause demodulation failure unless resource pools are configured different while resource pools division would limit capacity in V2V case, or eNB informs UE timing difference information.

Observation 2: No matter eNB synchronized to GNSS or not, above issue would happen if the existing D2D synchronization mechanism is directly reused.

Observation 3: Since in-coverage resource pools are not forwarded to out of coverage, in partial coverage, demodulation impact will be server because of large time and frequency difference between multiple Tx UEs.

Observation 4: Without global synchronization, time and frequency offset from multiple Tx UEs to a receiving UE would be even larger compared with the case having eNB as synchronization source.

Observation 5: Synchronization needs to be enhanced with reducing the number of synchronous clusters by applying GNSS to eNB and UE and using global timing as much as possible as long as it does not affect cellular network.

Observation 6: It is needed to further discuss the synchronization source priority at least when UE is served by eNB but cellular traffic and PC5 V2V traffic happen in the different frequency bands.

References

  - 3GPP TR 22.885 V0.2.0 (2015-04)  " Technical Specification Group Services and System Aspects;Study on LTE Support for V2X Services"

  - 3GPP TS 36.101: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception".

  - 3GPP TS 36.104: "Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception".

  - R1-154981 "Updated offline discussion summary on V2X evaluation assumptions"， LG Electronics

  - R1-156610   "DMRS enhancement in PC5-based V2V", CATT

  - R1-156603   " Synchronization enhancement in PC5-based V2V ", CATT

  - 3GPP TS 36.133 V13.0.0 (2015-07) " Requirements for support of radio resource management"