3GPP TSG RAN WG1 Meeting #83                                                                  R1-156630

Anaheim, USA, 15th – 22nd November 2015

Source:	ZTE

Title:	Simulation Results for DL MUST

Agenda Item:	6.2.7.1

Document for: 	Discussion and Decision

Introduction

At the RAN#82bis meeting, we provided evaluation results of non-orthogonal multiple access compared with orthogonal multiple access for FTP1 traffic model. Those results were based on ideal channel measurement and ideal estimation. Only wideband scheduling was simulated back then. In this contribution, system simulation results for MUST are presented with the assumption of non-ideal channel measurement and non-ideal channel estimation. EVM model is also taken into account. Both wideband and sub-band scheduling are considered.

CSI Feedback for MUST

In this section, we describe CSI and UE SINR computation methods at eNB that are relevant to downlink MUST.

In MUST, eNB needs to be informed about the CSIs of all UEs in order to select appropriate UE pairs. CSI includes RI, PMI, CQI or MCS. In the case of 2Tx and 2Rx, the feedback overheads of RI, PMI and CQI are 1bit, 2bit and 4bit, respectively. In the simulation, two paired UEs have the same precoding matrix, but very different CQIs. In closed-loop system, SINR of a UE cannot be fed back to eNB directly. UE SINR should be derived from CQI fed back by UE. The mapping from UE SINR to MCS is based on BLER curves.

Scheduling Methodology for System Level Simulation of MUST

- Basic processing flow in MUST scheduler

A key aspect of scheduler implementation for MUST is the selection algorithm for pairing users. Basic steps of the scheduler for sub-band operation are described below:

- For each sub-band, firstly, calculate the metric of candidate single UE using PF metric function. Select one UE having the highest metric as the optimal single UE.

- Secondly, select all possible combinations from current Sector, each pair satisfy the requirement of having the same PMI. These UE pair candidates are selected proper transmission power to maximize the throughput metric. The optimal UE pair is selected from the above UE pair candidates, given proper transmission powers.

- The optimal UE pair recommended by MUST scheduler and the optimal UE recommended by SU-MIMO scheduler are compared based on their metrics. The comparison gives the multiplexing type and the scheduled UEs.

- If the UE pair is scheduled in this sub-band, for example (UE1, UE2), UE1 is near UE, UE2 is far UE. In other sub-band, UE1 can be scheduled as single UE or near UE of MUST, UE2 can be scheduled as single UE or as far UE of MUST.

- Repeat steps 1-3 for all sub-bands.

- Rule for UE pairing

In order to reduce computation cost, those UE pairs that are obviously not suitable for MUST would be rejected at the beginning. Let us use UE1 and UE2 to represent two UEs in a pair. If the precoding matrices of UE1 and UE2 are significantly different, UE1 and UE2 would not be multiplexed. Furthermore, if the geometries (i.e., downlink wideband SINR) of UE1 and UE2 are very close, they would not be considered for pairing.

Precoding matrixes of UE1 and UE2 are denoted as W1 and W2. The rank of UE1 and UE2 are labeled as r1 and r2.

W1=v1,1v1,2⋯v1,r1                                               (1)

W2=v2,1v2,2⋯v2,r2                                                  (2)

vi,j=vi,jvi,j*vi,j                                                        (3)

Vector vi,j is the unitization of vector vi,j. i=1,2, j=1,2,⋯,ri.

The SINR of UE1 and UE2 are labeled as s1 and s2, in dBs. s2-s1 is the absolute value of the difference of  s1 and s2.

The multiplexing conditions of UE pair are listed below:

- At least one vector of set v1,1,v1,2,⋯,v1,r1 and one vector of set v2,1,v2,2,⋯,v2,r1 are same.

The number of the same vectors in the two sets is the multiplexing rank of the two UE, which is labeled as rM.

- s2-s1>sT. sT is the preconfigured threshold value, such as 10 dB, 15 dB, etc.

The above condition about SINR tries to ensure that a central UE and an edge UE, rather than two central UEs or two edge UEs, should be multiplexed in MUST. If s1>s2, UE1 is central UE and UE2 is edge UE. If not, UE1 is edge UE and UE2 is central UE.

- Selection of transmission power for UEs

For a UE pair candidate (UE1 and UE2) that meets the above condition, proper transmission power p1 and p2 should be set in order to maximize the throughput metric of the UE pair. The metric of a UE pair is the metric of the performance estimation of UE1 and UE2 multiplexing in power domain. Normally, the greater the metric, the greater the multiplexing performance of the two UEs is. p1 and p2 are subject to the constraint

p1+p2=1                                                              (4)

Suppose UE1 is central UE and UE2 is edge UE, The candidates of p1,p2 are 0.5-∆p,0.5+∆p,0.5-2∆p,0.5+2∆p,⋯,pmin,pmax. Here ∆p,pmin,pmax are subject to the constraints:

0<∆p<0.5                                                              (5)

pmin+pmax=1                                                            (6)

0<pmin<0.5                                                              (7)

0.5<pmax<1                                                           (8)

It is needed to conduct exhaustive search on the candidate set of p1,p2 and extract the best value pair p1,b,p2,b by eNB. Value p1,b,p2,b can maximize the metric of the current UE pair.

- Estimation of scheduling SINR of multiplexed UEs

When two UEs are assumed to multiplex in power domain, the scheduling SINR of the two UEs should be estimated. Suppose UE1 is near the cell center and UE2 is at the cell edge, the scheduling SINR s1 of UE1 and the scheduling SINR s2 of UE2 is calculated by the formulas below.

s1=s1+10log10p1,b                                                     (9)

s2=s2s1+110log10p2,b                                         (10)

Here, s1 and s2 are reported SINRs of UE1 and UE2, respectively. Value p1,b,p2,b is the proper transmission power of UE1 and UE2. And p1,b,p2,b is subject to the constraint below.

p1,b+p2,b=1                                                       (11)

- Calculation of multi-user PF metric

The metric of a UE-pair not only considers the performance benefit of paring these two users, but also takes into account of the history of scheduling of UE1 and UE2.  The metric tends to give UEs previously scheduled less times with more chances to be scheduled in the future. So the metric used by the scheduling algorithm is the metric with proportional fairness (PF). We used the scheduling metric with proportional fair described by [1].

Mk=RktTkt                                                        (12)

Here, Mk is the metric of k-th UE at time t. Rkt is the “requested rates” of k-th UE at the time t. Tkt is the average throughput of k-th UE until the time t.

Tkt=1-1∆tTkt-1+1∆tRkt                                   (13)

Here, ∆t is the length of the exponentially weighted window. Tkt-1 is the average throughputs of k-th UE until  time t-1.

Preliminary System Simulation Results

In this section, system level simulations on Full buffer model and FTP1 model are carried out. Wideband scheduling and sub-band scheduling are considered. Detailed simulation assumptions are described in Table A1.

Near UEs use SIC receiver to remove the interference from edge UEs. Interference to edge UEs can hardly be eliminated.

Table 1 Performance of SU-MIMO and MUST for full buffer model, wideband scheduling

| Wideband schdulling | Cell spectral efficiency (bps/Hz/cell) | Cell edge user spectral efficiency(bps/Hz/cell/user) |
| --- | --- | --- |
| SU-MIMO | 1.3483 | 0.0212 |
| MUST | 1.4072 | 0.0242 |
| Gain | 4.37% | 13.92% |

Table 2 Performance of SU-MIMO and MUST for full buffer model, sub-band scheduling

| Sub-band schdulling | Cell spectral efficiency (bps/Hz/cell) | Cell edge user spectral efficiency(bps/Hz/cell/user) |
| --- | --- | --- |
| SU-MIMO | 1.6481 | 0.0304 |
| MUST | 1.6703 | 0.0323 |
| Gain | 1.35% | 6.26% |

Performance SU-MIMO and MUST for full buffer traffic model is shown in Table 1 and Table 2. Table 1 is for wideband scheduling and Table 2 is for sub-band scheduling.  The results of Table 1 show cell edge performance gain is around 14% and cell average performance gain is around 4% for wideband scheduling. It is observed from Table 2 that the cell edge performance gain of MUST than SU-MIMO is around 6% for sub-band scheduling. The gain in the case of sub-band scheduling is lower than that in wideband scheduling.

Table 3 Performance of SU-MIMO and MUST for FTP1 model with packet size 0.1 Mbytes, wideband scheduling

| Wideband schdulling | SCT/OCT (%) | RU | Mean UPT (Mbps) | 5% UPT (Mbps) | 50% UPT (Mbps) |
| --- | --- | --- | --- | --- | --- |
| SU-MIMO | 0.9774 | 0.7325 | 7.0685 | 0.8827 | 4.5726 |
| MUST | 0.9868 | 0.7084 | 7.5386 | 0.9768 | 5.0842 |
| Gain |  |  | 6.65% | 10.65% | 11.19% |
| SU-MIMO | 0.9151 | 0.8715 | 4.4583 | 0.6529 | 2.5421 |
| MUST | 0.9382 | 0.8524 | 4.8930 | 0.6891 | 2.9106 |
| Gain |  |  | 9.75% | 5.54% | 14.50% |

Performance comparison of SU-MIMO and MUST on FTP1 is shown as Table 3 with wideband scheduling. When RU is about 0.7, the gain of the percent 5 UE throughput is about 10% and the gain of average spectral efficiency is about 6%. When RU is about 0.85, the gain of the percent 5 UE throughput is about 5% and the gain of average spectral efficiency is about 9%. The gain of the percent 5 UE throughput is lower compared with RU 0.7. The reason is that more files are dropped when the transmission time of those UEs is over Tdrop, which is also reflected in SCT/OCT.

Histogram of active users per cell in FTP1 is shown in Fig. 1 and Fig. 2. The maximal number of active users is 23 in Fig.1. The maximal number of active users is 24 in Fig.2. Histogram of simultaneously scheduled users in FTP1 is shown in Fig. 3 and Fig. 4. It is observed from Fig. 3 that 65% of times only one UE is scheduled in a cell. Only 7% times there are two users simultaneously scheduled. It is observed from Fig. 4 that 70% of times only one UE is scheduled in a cell. Only 13% times there are two users simultaneously scheduled, i.e., operating MUST.

Figure 1 Histogram of active users in FTP1 for ~70 %RU with packet size 0.1 Mbytes, wideband scheduling.

Figure 2 Histogram of active users in FTP1 for ~80 % RU with packet size 0.1 Mbytes, wideband scheduling.

.

Figure 3 Histogram of simultaneously wideband scheduled users in FTP1 for ~70% RU with packet size 0.1 Mbytes.

Figure 4 Histogram of simultaneously wideband scheduled users in FTP1 for ~80 %RU with packet size 0.1 Mbytes.

In the case of sub-band scheduling, performance comparison of SU-MIMO and MUST on FTP1 with packet size of 0.1Mbytes is shown as Table 4. It is observed that the gain of cell edge throughput is about 3% and the gain in average spectral efficiency is about 1% when RU is about 0.75. When RU is about 0.85, the gain of cell edge throughput is about 4.5% and the gain in average spectral efficiency is about 7%.

Table 4 Performance of SU-MIMO and MUST for FTP1 model with packet size of 0.1Mbytes, sub-band scheduling

| Sub-band schdulling | SCT/OCT (%) | RU | Mean UPT (Mbps) | 5% UPT (Mbps) | 50% UPT (Mbps) |
| --- | --- | --- | --- | --- | --- |
| SU-MIMO | 0.9890 | 0.7675 | 7.1938 | 1.0601 | 4.8967 |
| MUST | 0.9904 | 0.7613 | 7.2828 | 1.0935 | 5.0049 |
| Gain |  |  | 1.24% | 3.15% | 2.21% |
| SU-MIMO | 0.9582 | 0.8698 | 5.2627 | 0.7745 | 3.3139 |
| MUST | 0.9397 | 0.8492 | 5.6441 | 0.8095 | 3.6253 |
| Gain |  |  | 7.25% | 4.53% | 9.40% |

Histogram of active users in FTP traffic model 1 with packet size of 0.1Mbytes is shown in Fig. 3 and Fig. 4. The maximal number of active users is 23 in Fig. 3. The maximal number of active users is 32 in Fig.4. Histogram of simultaneously scheduled users in FTP1 is shown in Fig. 5 and Fig. 6. It is observed from Fig. 5 that 25% of times only one UE is scheduled in a cell. About 50% times there are two users simultaneously scheduled. It is observed from Fig. 6 that 20% of times only one UE is scheduled in a cell. About 65% times there are two users simultaneously scheduled.

Figure 3 Histogram of active users in FTP1 for 70 %RU with packet size of 0.1Mbytes, sub-band scheduling

Figure 4 Histogram of active users in FTP1 for 80 %RU with packet size of 0.1Mbytes, sub-band scheduling

Figure 5 Histogram of simultaneously sub-band scheduled users in FTP1 for ~70 %RU with packet size 0.1 Mbytes.

Figure 5 Histogram of simultaneously sub-band scheduled users in FTP1 for ~70 %RU with packet size 0.1 Mbytes.

The stability of FTP traffic model 1

In RAN1#82bis, some companies [4][5] provided statistics of number of active UEs and served to offered traffic ratio to investigate the stability of FTP1 model in the middle load and high load situations. In this section, an initial study on the stability of FTP 1 is presented, with the assumption of ideal channel measurement and ideal estimation.

Here the simulation time is 10s and wideband scheduling is considered. Packet size is 0.1Mbytes and lambda is 12. Tdrop in Fig. 5 is 1.6 s and Fig.6 is 8s. RU of Fig. 5 and Fig. 6 is 0.9091 and 0.9298 respectively.

Figure 7 Mean Resource Utilization Across different Sector of OMA

Figure 8 Mean Resource Utilization Across different Sector of OMA

It is observed from Fig. 5 and Fig. 6 that RU fluctuations between cells are not very significant.

Conclusions

In this contribution, we provide simulation results on full buffer traffic and FTP1 traffic based on non-ideal channel measurement and non-ideal channel estimation. Compared with SU-MIMO, MUST can significantly improve cell average and cell edge performance for full buffer traffic and FTP1 traffic.

- References

- 3GPP R1-153442, Way forward on further details of evaluation methodology for MUST, MediaTek, CHTTL, III, ITRI, NTT DoCoMo, ZTE.

- David Tse, Pramod Viswanath, Fundamentals of Wireless Communication, Posts and Telecom Press, Aug. 2009.

[3]    3GPP R1-156081, Further Simulation Results for DL MUST, ZTE

[4]    3GPP R1- 155686, Stability of FTP traffic models at high load, Ericsson.

[5]    3GPP R1-156110, System-level performance of MUST schemes, Huawei, HiSilicon.

- Annex

Table A1:  System-level simulation assumptions of DL MUST (realistic ones are highlighted in yellow).

| Cellular Layout | Hexagonal grid, 3 sectors per site, 7 macro sites |
| --- | --- |
| Inter-site distance | 500 m |
| Minimum distance between BS and UE | 25m |
| Distance-dependent path loss | ITU UMa, with 3D distance between an eNB and a UE applied |
| Penetration loss | For outdoor UEs: 0 dB For indoor UEs: (20+0.5din) dB (din: independent uniform random value between [0, 25] for each link) |
| Shadowing | ITU UMa |
| Fast fading channel between eNB and UE | ITU UMa |
| eNB antenna gain + connector loss | 17 dBi |
| UE antenna gain | 0 dBi |
| Channel Estimation | realistic |
| Channel Measurement | realistic |
| UE speed | 3 km/h |
| Carrier Frequency | 2.0 GHz |
| System bandwidth | 10 MHz |
| Antenna configuration | eNB: 2Tx, 0.5 lambda, cross-polarized UE: 2Rx, 0.5 lambda, cross-polarized |
| Number of UEs per cell | 10 |
| Maximum number of multiplexed UE | 2 |
| Delay time of scheduling | 6 ms |
| Traffic model | Wideband scheduling: Packet Size=0.1Mbytes, lambda=10 Packet Size=0.1Mbytes, lambda=11 Sub-band schdulling: Packet Size=0.1Mbytes, lambda=11 Packet Size=0.1Mbytes, lambda=12 |
| UE dropping | 20% UEs are outdoor; 80% UEs are indoor |
| Total BS TX power (total per carrier) | 46 dBm |
| BS noise figure | 5 dB |
| UE noise figure | 9 dB |
| Antenna Height | 25 m |
| UE antenna Height | 1.5 m |
| Codebook | LTE Rel. 8 |
| OLLA | Yes |
| Receiver | MMSE with IRC and SIC |
| EVM | EVM is modeled, 8% TX, 4% RX |
| Duration of the simulation | 40s for FTP1, 5s for full buffer |