3GPP TSG RAN WG1 Meeting #83	R1-156671

Anaheim, USA, 16 - 20 November 2015

Agenda Item:	6.2.7.3

Source: 	NEC

Title:	Downlink Control Signalling for MUST

Document for:	Discussion

# Introduction

3GPP study on MUST considers superposition transmission of PDSCHs for two or more UEs that could be differentiated in power domain.  For example, a UE which is at the cell center (near UE) can have a stronger link to the base station compared to a UE which is at the cell edge (far UE). In this case, the base station transmit power for the far UE can be much larger than that for the near UE. Large difference in transmit power levels could be exploited to increase multiple access gain by co-scheduling PDSCHs of the far and near UEs on the same time, frequency and spatial resources.

In principle, near UE’s signal being transmitted at low power may hardly reach the far UE. Therefore near UE’s signal may appear as noise at the far UE where it is sufficient to decode its own PDSCH.  However, far UE’s signal may appear as strong co-channel interference to the near UE which has to be decoded and cancelled from the received signal before being able to demodulate and decode its own.

To be able to cancel the far UE’s signal, the near UE needs to know information about the far UE’s transmit signal. Type of information required can depend on the MUST schemes (Sec 5.1[1]) and receiver types (Sec 5.2 [1]). Potential PDSCH assistance information for different MUST schemes and receiver types has been agreed in Sec 7.2.7.3 of [2]. Further, it was agreed in [2] that the following methods may be considered for obtaining assistance information:

  - blind detection

  - signalling

    - higher layer signalling or

    - dynamic signalling

  - tied to the UE’s scheduling information, or

  - tied to specific UE assumptions (e.g. UE assumes aligned resource allocation).

In this contribution, we discuss dynamic parameter signaling aspects of assistance information.

# Discussion

Dynamic parameters of the far UE relating to spatial and link adaptation, that may be required at the near UE for interference cancellation, have been identified in Sec 7.2.7.3 of [2]. Some of these parameters such as rank indication, precoding matrix indication and modulation may be estimated blindly by an enhanced NAICS like near UE [3].  However, the implementation complexity and power consumption of the near UE will increase with the number of dynamic parameters that the near UE is attempting to estimate. Moreover, it may not possible to blindly estimate some parameters such as transport block size and redundancy version that are needed for CWIC receivers which could potentially provide significant performance gain in MUST context[4].  Therefore, it is expected that at least the near UE will need to obtain some dynamically signalled information about the far UE. There are observed several ways in which such information can be obtained:

Option 1: Near UE is able to decode legacy DCI intended to the far UE partially or fully.

Option 2: Far UE information required for MUST is part of a new DCI intended to the near UE.

Option 3: Near and far UEs share common new DCI being transmitted on a single physical control channel.

Advantages and challenges of above options are summarized in Table 1.

Table 1: Advantages and Challenges of DCI options

| Option | Advantages | Challenges |
| --- | --- | --- |
| Option 1 | - Not much modification to current LTE implementation at the base station. - Be able to obtain any dynamically signalled parameter(s) of the far UE. - Eliminate the need for blind estimation of the legacy dynamically signalled parameters. | - Near UE’s physical control channel blind decoding complexity will increase especially if the paired UEs occupy different UE specific search spaces. - Additional number of blind detections may be required at the near UE to obtain MUST specific information. - Need to have knowledge of RNTI of far UE for detection purpose. |
| Option 2 | - No increase to physical control channel blind decoding complexity at the near UE. - MUST specific information could be part of new DCI. - Dynamically signalled parameters of the far UE that the near UE may not able to easily estimate could be part of the new DCI. | - Need new DCI(s) with potentially larger number of bits to carry information about far UE. As such, 1-CCE aggregation level may not be applicable to the new DCI. - Additional number of blind detections may be required if: - duplication of information between new DCI for the near UE and legacy DCI for the far UE is to be reduced, and - the size of new DCI to be reduced |
| Option 3 | - No increase to physical control channel blind decoding complexity at the near UE. - Saving of down link control resources as UEs share common new DCI being transmitted on a single physical control channel. - MUST specific information could be part of new DCI. | - Need new DCI(s) with potentially larger number of bits to carry information for both UEs. As such, 1-CCE aggregation level may not be applicable to the new DCI. - Need a design of DCI that may be partially decoded by legacy UE as far UE and fully decoded by advanced UE as near UE. - Potentially need new RNTI for being used in detection at near and far UEs. |

Observation:  Further discuss advantages and challenges of above three options for obtaining dynamically signaled information about the far UE.

# Conclusion

In this contribution, we discussed advantages and challenges of three potential options for obtaining dynamic information about the far UE for MUST.

Observation:  Further discuss advantages and challenges of above three options for obtaining dynamically signaled information about the far UE.

# References

- RP-155697, “Study on Downlink Multiuser Superposition Transmission”, 3GPP TR 36.859 V0.3.0 (2015-10), 3GPP TSG RAN WG1 Meeting #82bis Malmo, Sweden, 5th – 9th October 2015

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #82bis Malmo, Sweden, 5th – 9th October 2015

- Heunchul, L.; Jong-Han, L.; Sungyoon, C.; Sungsoo, K., “Interference cancellation based on blindly-detected interference parameters for LTE-advanced UE,” in Communications (ICC), 2015 IEEE International Conference on , vol., no., pp.3143-3148, 8-12 June 2015.

- Saito, K.; Benjebbour, A.; Kishiyama, Y.; Okumura, Y.; Nakamura, T., “Performance and design of SIC receiver for downlink NOMA with open-loop SU-MIMO,” in Communication Workshop (ICCW), 2015 IEEE International Conference on , vol., no., pp.1161-1165, 8-12 June 2015