3GPP TSG RAN WG1 Meeting #83	R1-156783

# Anaheim, CA, USA, 16th - 20th November 2015

Agenda item:	6.2.4.2.1

Source: 	Samsung

Title: 	CSI-RS RE mapping with CDM-4 for class A CSI reporting

Document for:	Discussion and Decision

# Introduction

In RAN1#82bis, the following agreements and working assumptions are drawn for 12 and 16 port CSI-RS mapping.

Working assumptions:

- For 12 or 16 ports, a CSI-RS resource for class A CSI reporting is composed as an aggregation of K CSI-RS configurations.

  - ∑k Nk ∈ {12,16}

  - Either CDM-2 or CDM-4 per CSI-RS resource can be configured for a UE

    - No need to send LS to RAN4 to change 6 dB power boosting assumption for CSI-RS transmission

  - RRC configuration parameters include:

    - A list of K CSI-RS resource configurations;

      - In case of CDM-2, the CSI-RS resource configurations indicate CSI-RS RE locations according to legacy resource configurations in 36.211

        - Nk ∈ {2, 4, 8}, and the same Nk = N used for all k

          - One or more values from {2, 4, 8} can be down-selected

      - FFS: In case of CDM-4, the CSI-RS resource configurations indicate CSI-RS RE locations:

    - A single set of subordinate parameters that will commonly apply to all resources within Rel-13 CSI-RS resource

      - number of antenna ports (N),

      - subframe config.,

      - scrambling ID,

      - QCL info.

Agreements:

- 12 and 16 Port CSI-RS construction for CDM-2

  - For 16 port construction:

    - Working assumption (N,K) = (8,2), (2,8)

  - For 12 port construction:

    - Working assumption (N,K) = (4,3), (2,6)

- CSI-RS RE mapping details for CDM-2

  - The ports of the aggregated resource correspond to the ports of component resources according to the following:

    - The aggregated port numbers are 15, 16, … 30 (for 16 CSI-RS ports)

    - The aggregated port numbers are 15, 16, … 26 (for 12 CSI-RS ports)

    - FFS: The aggregated port number is given by n=(k-1)*N+p, p = 15,…,14+N,

      - k (= 1,…,K) correspond to the k-th CSI-RS configuration.

- Working assumption CSI-RS RE mapping details for CDM-4

  - Full-port CSI-RS can be mapped in each OFDM symbol used for CSI-RS mapping.

  - CDM RE set construction

    - Alt 1: time domain only (4 OFDM symbols)

    - Alt 2: time and frequency domain (2 subcarriers x 2 OFDM symbols)

    - Down-selection or merging of the two alternatives FFS

This contribution presents link-level simulation results to show the advantage of CDM-4 and full-power utilization, and proposes for detail designs with CDM-4 based CSI-RS mapping

# Link-Level Simulation

CDM-4 based CSI-RS RE mapping allows full-power utilization, which can be translated into performance advantage for low-SINR UEs. To quantify such benefits, link-level throughput analysis has been performed. Details of the simulation assumptions can be found in Table 1.

Table 1 Link-level simulation parameters

| Parameters (common) | Value / Comment |
| --- | --- |
| Transmission bandwidth | 5MHz |
| Channel model, UE velocity, spatial correlation | 3GPP ETU channel without correlation, 3 km/h |
| Antenna configuration | 16Tx / 2 Rx antennas |
| Detector | MMSE receiver |
| PDCCH / PDSCH configuration | 2 / 12 OFDM symbols per sub-frame |
| Modulation, code rates | CQI based link adaptation |
| HARQ | Synchronous HARQ with Chase Combining (max 4 transmissions) |
| Number of allocated PRBs | 4 PRBs |
| CSI feedback | CQI/PMI/RI Mode 1-1 based on 16Tx codebook (rank1/2) |
| Feedback delay | 2 msec |
| Feedback periodicity | 5 msec |
| Channel estimation for demodulation | 2D MMSE |
| Channel estimation for CQI/PMI/RI computation | 1D MMSE |
| Rank adaptation | On |

Error! Reference source not found. illustrates the six different 16-port CSI-RS mapping patterns used for the simulation:

(a) CDM-2 pattern: two 8-port legacy CSI-RS REs are aggregated

- This is according to the agreed working assumption for CDM-2 based mapping.

(b) CDM-4 pattern 1: CDM-TF (in time-frequency domain) is applied across 2x2 time/frequency adjacent REs

- New 4-port CSI-RS resource is mapped onto 2x2 time/frequency adjacent REs and four such 4-port CSI-RS resources are aggregated. CDM-4 is applied across those four REs of each 4-port CSI-RS resource.

(c) CDM-4 pattern 2: CDM-TF is applied across 2x2 time-adjacent/frequency-separated REs.

- Four legacy 4-port CSI-RS resources are aggregated. CDM-4 is applied across those four REs of each 4-port CSI-RS resource.

(d) CDM-4 pattern 3: CDM-T (in time domain) is applied across 1x4 REs on a same subcarrier.

- Two legacy 8-port CSI-RS resources are aggregated, in such a way that the OFDM symbol spacing between the two resources are minimized. CDM-4 is applied across those four REs on a same subcarrier.

(e) CDM-4 pattern 4: CDM-T is applied across 1x4 REs on a same subcarrier.

- Two legacy 8-port CSI-RS resources are aggregated, in such a way that the OFDM symbol spacing between the two resources are maximized. CDM-4 is applied across those four REs on a same subcarrier.

(f) CDM-4 pattern 5: CDM-T is applied across 4 staggered REs.

- Two legacy 8-port CSI-RS resources are aggregated, in such a way that the OFDM symbol spacing between the two resources are maximized. CDM-4 is applied across those four REs, composed of the two REs on a n-th subcarrier of a first resource and two REs on a n-th subcarrier of a second resource, where n = 1, 2, 3, 4.

| (a) CDM-2 Pattern | (b) CDM-4 Pattern 1 (CDM-TF) | (c) CDM-4 Pattern 2 (CDM-TF) |
| --- | --- | --- |
| (d) CDM-4 Pattern 3 (CDM-T) | (e) CDM-4 Pattern 4 (CDM-T) | (f) CDM-4 Pattern 5 (CDM-Staggered) |

Figure 1. Simulated CSI-RS RE mapping patterns

Relative throughput gains of the CDM-4 patterns with 3km/h UE speed are shown in Table 2. The throughput gain of CDM-4 patterns is about 30% at 0dB, but no significant performance difference is found among different CDM-4 patterns. The 16-port CSI-RS used for CQI/PMI derivation are distributed across the 8~10 subcarriers and hence the estimated CQI/PMI is already far from perfect; and it seems that different CDM patterns do not produce much final performance difference.

Table 2 Relative average throughput gain(%) of CDM-4 patterns against the CDM-2 pattern with 3 km/h

| SNR CDM | -6 dB | -3 dB | 0 dB | 3 dB | 6 dB | 9 dB | 12 dB | 15 dB | 18 dB | 21 dB | 24 dB | 27 dB | 30 dB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (b) | 17 | 29 | 33 | 20 | 11 | 8 | 5 | 3 | 1 | 0 | 0 | 0 | 0 |
| (c) | 17 | 29 | 34 | 21 | 10 | 8 | 5 | 3 | 1 | 0 | 0 | 0 | 0 |
| (d) | 17 | 29 | 33 | 21 | 10 | 8 | 5 | 3 | 2 | 2 | 0 | 0 | 0 |
| (e) | 17 | 19 | 31 | 21 | 10 | 8 | 5 | 3 | 2 | 2 | 0 | 0 | 0 |
| (f) | 17 | 19 | 28 | 13 | 6 | 5 | 3 | 1 | 0 | 0 | 0 | 0 | 0 |

Observation 1: With CDM-4, about 30% throughput performance can be achieved at 0dB with 3km/h.

Observation 2: Relative performance difference of different CDM-4 patterns is marginal.

# Proposals on CDM-4 CSI-RS RE mapping

In order for the CSI-RS RE mapping pattern to ensure full-power utilization and up to 6-dB power boosting for 12-port or 16-port CSI-RS mapping, the following two conditions should be met:

- Condition 1: Full-port CSI-RS are mapped in each OFDM symbol used for CSI-RS mapping.

- Condition 2: CDM-4 has to be used either in time or in frequency domain.

These two conditions are satisfied with the illustrated examples of CDM-4 patterns 1-5 in Figure 1.

For specifying CDM-4 CSI-RS RE mapping, for full flexibility, it is proposed to aggregate legacy CSI-RS resources without imposing restriction on the pool of the legacy CSI-RS resources. This may imply that some CSI-RS patterns constructed with CDM-4 do not achieve full power utilization; however, some of these patterns may still achieve less power loss than CDM-2 counterparts [1]. In addition, un-restricted aggregation allows both CDM-T and CDM-TF, and this will maximize the reuse factor and the number of full-power achieving CSI-RS patterns.

Based upon these design principles, the following combination is proposed for constructing 12- and 16- port CSI-RS resources:

- 12-port CSI-RS: (N,K) = (4,3), (8,2)

- 16-port CSI-RS: (N,K) = (4,4), (8,2).

Figure 2. 12 or 16 port CSI-RS resource mapping with 2 x (8-port legacy resources)

When component legacy CSI-RS resources have 8 ports, CDM-4 should be applied across the 4 REs composed of two REs on the n-th subcarrier of the first resource and two REs on the n-th subcarrier of the second resource, where n = 1, 2, 3, 4 for 16-port case and n = 1, 2, 3 for 12-port case. In particular, in the 12-port cases, the REs belonging to n=4 can be used for PDSCH mapping for those UEs configured with the 12-port CSI-RS. When this mapping method is used, full-power utilization is achieved with both CDM-T and CDM-TF. This proposal is illustrated in Figure 2. In Figure 2, two legacy 8-port CSI-RS resources can be aggregated for 12- or 16-port CSI-RS mapping, and CDM-4 is applied across those REs labeled with the same alphabet, i.e., one of ‘A’, ‘B’, ‘C’ and ‘D’. For 12-port CSI-RS mapping, REs labeled with ‘A’,’B’ and ’C’ can be used for the RE mapping.

Figure 3. 12 or 16 port CSI-RS resource mapping with (3 or 4) x (4-port legacy resources)

When component legacy CSI-RS resources have four ports, CDM-4 should be applied across 4 REs corresponding to each component legacy CSI-RS resource. In this case, full-power utilization is achieved with CDM-TF only. This proposal is illustrated in Figure 3. In Figure 3, each alphabet, i.e., one of ‘A’, ‘B’, …, ‘J’, corresponds to a legacy 4-port CSI-RS resource. Three and four legacy 4-port CSI-RS resources can be aggregated respectively for 12- or 16-port CSI-RS mapping, and CDM-4 is applied across those REs labeled with the same alphabet.

# Conclusions

In this contribution, the following observations are made:

Observation 1: With CDM-4, about 30% throughput performance can be achieved at 0dB with 3km/h.

Observation 2: Relative performance difference of different CDM-4 patterns is marginal.

Finally, the proposal in Section 3 can be summarized as in the following:

Proposal: CDM-4 based CSI-RS mapping should be performed according to the following:

- 12-port CSI-RS: (N,K) = (4,3), (8,2)

- 16-port CSI-RS: (N,K) = (4,4), (8,2).

- For 8-port resource aggregation:

  - The time frequency resources for  are determined according to the legacy specification, for the two 8-port CSI-RS resources.

    - Note that in this case 4 REs will be associated with each of , ,  and .

  - Antenna ports  of the aggregated resource is related to  according to the following:

    - , , where .

      - Note that in this case four antenna ports will be associated with each of , ,  and .

  - A CDM group is composed of four antenna ports and four REs corresponding to each of , ,  and .

  - The 1st and the 2nd REs of the CDM group correspond to the two REs from the first resource; and the 3rd and the 4th REs of the CDM group correspond to the two REs from the second resource.

  - OCCs 0, 1, 2, 3 are respectively assigned to  (or  of the first and the second resources), for each .

- For 4-port resource aggregation:

  - Time frequency resources for the antenna port numbers  of the aggregated CSI-RS resource correspond to those four REs for the n-th CSI-RS resource with , where .

    - Note that antenna ports  of the aggregated CSI-RS resource will be mapped to four REs used for antenna ports  of the 1st, 2nd, 3rd and 4th CSI-RS resources, respectively for .

  - A CDM group is composed of four antenna ports  and the corresponding time frequency resources, for each .

  - The 1st and the 2nd REs of the CDM group correspond to the two REs for ; and the 3rd and the 4th REs of the CDM group correspond to the two REs for .

  - OCCs 0, 1, 2, 3 are respectively assigned to  for each .

- OCCs to apply on the four REs  are according to Table 3.

Table 3 OCC mapping

| OCC index | CDM Walsh cover to apply on the four REs |
| --- | --- |
| If corresponding OCC index for antenna port p is 0: | [+1 +1 +1 +1] |
| If corresponding OCC index for antenna port p is 1: | [+1 -1 +1-1] |
| If corresponding OCC index for antenna port p is 2: | [+1 +1 -1-1] |
| If corresponding OCC index for antenna port p is 3: | [+1 -1 -1 +1] |

# References

- [1] R1-155675	CSI-RS Design for Class A FD-MIMO	Ericsson