3GPP TSG RAN WG1 Meeting #83                                 R1-156832

# Anaheim, USA, 15th - 22nd November 2015

# Source:          ZTE

Title:            Higher Rank Codebook Design for Class A

Agenda Item:     6.2.4.3.1

Document for:    Discussion and Decision

- Introduction

Kronecker Product (KP) type codebook for non-precoded CSI-RS has been approved in [1]. In WF [2], the structure of W1 and class A rank 1 codebook is determined upon four configurations. In RAN1#82bis [3], the valid (O1,O2) combinations for each antenna array layout (N1,N2) was finalized. For rank 2 codebook design, companies provide various codebook schemes via e-mail discussion. For rank 3 and rank 4 codebook design, a new parameter k was introduced to construct different orthogonal beam groups [4].

In this contribution, we first discuss the aspects involved in rank 3 and 4 codebook design. Then we simulate the performance of the discussed aspects. Our proposal on rank 3 and 4 codebook is given based on both the discussion and simulation.

- Discussion on Rank 3-4 codebook Design

A proposal on rank 3 and 4 codebook is given in [4]. We also proposed our codebook during email discussion, which can be seen in Appendix (Note: we just change the codeword order compared with the submitted proposal). The differences between our codebook and the one proposed in [4] are listed as follows.

(1). The feedback mode of (δ1, δ2): in [4], (δ1, δ2), which is mapped to the value of k, is fed back in wideband, whereas in our codebook, (δ1, δ2) is fed back in sub-band. Performance gain can be observed with sub-band (δ1, δ2) feedback.

(2). The value of (δ1, δ2): our suggestion on (δ1, δ2) mapping is shown in Table 1.

# Table 1: Orthogonal beam type to (δ1, δ2) mapping

| (N1,N2) |  |  | k |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | δ | 0 | 1 | 2 | 3 | 4 | 5 | 6 |
| If N1>1 and N2>1 | N1≥N2 | δ1 | O1 | 0 | O1 | 2O1 | 2O1 | 3O1 | 3O1 |
|  |  | δ2 | 0 | O2 | O2 | 0 | O2 | 0 | O2 |
|  | N1<N2 | δ1 | O1 | 0 | O1 | 0 | O1 | 0 | O1 |
|  |  | δ2 | 0 | O2 | O2 | 2O2 | 2O2 | 3O2 | 3O2 |
| If N2=1 |  | δ1 | O1 | 2O1 |  |  |  |  |  |
|  |  | δ2 | 0 | 0 |  |  |  |  |  |

Note that available number of (δ1, δ2) is different for different type of antenna topology. Hence the value of (δ1, δ2) should be combined with (N1, N2). According to the description of the feed-back mode in (1), the combination of (δ1, δ2) and (N1, N2) can be indicated in sub-band PMI. Take N1≥N2 as an example. The rules for codeword selection from codebook config 1 is illustrated in Table 2.

Table 2: Codeword selected for different (N1,N2)

| (N1,N2) | Selected i2 index in Table 6-2 | Corresponding valid (δ1, δ2) combinations |
| --- | --- | --- |
| (2, 2) | 0-5 | (O1,0), (0, O2), (O1,O2) |
| (3, 2) | 0-9 | (O1,0), (0, O2), (O1,O2), (2O1,0), (2O1, O2) |
| (4, 2) | 0-13 | (O1,0), (0, O2), (O1,O2), (2O1,0), (2O1, O2), (3O1,0), (3O1, O2) |

(3). The design of W1: in our codebook, p1,p2=(1,1) for all the configs. However in [4],  p1,p2=(1,1) for config 1, p1,p2=(O12,O22) for config 2 and 3, and p1,p2=(O14,-) for config 4. If p1,p2=(1,1), master beam group is constructed by non-consecutive beams, which means each codeword in W1 is distributed in discrete beam grids. p1,p2≠(1,1) seems unreasonable due to the following two reasons. The first is that it does not conform to the design rules of W1 in rank 1 and 2 codebook, though for all configs it has either 1 column or 4 columns. In order to reserve the design rules as much as possible, p1,p2=(1,1)  is a more reasonable choice. The second is that non-consecutive structure brings performance loss. Since the purpose of UE feeding back wide-band W1 is to give a most probable master beam direction which changes little through the wide band. If the beams in W1 is distributed discretely, the original motivation of wide-band feed-back is lost. Therefore, selecting the beams near the master beam direction, instead of other discrete points, in helpful to increase the performance of wide-band feed-back.

(4). The choice of codeword type: Four rank 3 codeword can be constructed by two orthogonal beams x0 and x1, which can be classified into two types：

Type 1:   and

Type 2:   and

In [4], all the four codewords can be selected as a rank 3 codeword. However, in our codebook, only cw0 and cw3 can be selected, since simulation results show that the co-phasing between two pols does not have essential impacts on the system performance.

Note that the difference between two codewords within the same Type, i.e., cw0 and cw1, is only the polarization directions. For the codewords in different Types, i.e., cw0 and cw2,  cw0 takes x0 as the beam which has more contribution on the master beam, whereas cw2 takes x1 as the beam which has more contribution on the master beam. Based on the discussion above, the selection of appropriate mater beam direction has relatively obvious performance gain. Thus under the constraint on the bits of feed-back overhead, the selection of codeword Type has higher priority than the co-phasing between two pols.

- Performance Evaluation

We conduct system level simulations for all the aspects mentioned above. The scenario is 3D-Umi SU, and the service model is FTP1 with medium load. The antenna topology and oversampling factor are as follows: (N1, N2, O1, O2) = (4,2,8,4) for config 1 and config 2, (N1, N2, O1, O2) = (8,1,4,-) for config 4. We compare the performance of the codebook in [4] and our codebook. The simulation results are shown in Table 3. It can be seen from table 3 that our proposed codebook outperform the codebook proposed in [4], especially the cell edge performance.  Performance gain in the range of 12-17% can be obtained for all configs on cell edge performance.

# Table 3. Evaluation results of the proposed codebook

| 3D Umi scenario, SU-MIMO, FTP Traffic with medium load |  |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | Codebook in [4] |  | Proposed codebook |
|  |  | Alt 1 with k=0,1 | Alt 2 with k=0-7 |  |
| Config 1 | RU | 0.4930 | 0.4867 | 0.4841 |
|  | 5% | 11.45(+100%) | 12.03(+105.7%) | 12.85(+112.2%) |
|  | 50% | 38.72(+100%) | 38.96(+100.7%) | 38.84 (+100%) |
|  | Mean | 42.48(+100%) | 43.07(+101%) | 43.19(+101.4%) |
|  | Overhead (W1+W2) bits | 9+1 bits for both rank 3 and rank 4 | 13+1 bits for both rank 3 and rank 4 | 6+4 bits for both rank 3 and rank 4 |
| Config 2 | RU | 0.4857 | 0.4804 | 0.4731 |
|  | 5% | 12.38(+100%) | 13.49(+109%) | 14.18(+114.5%) |
|  | 50% | 38.98(+100%) | 40.01(+102.6%) | 39.07(+100%) |
|  | Mean | 42.91(+100%) | 43.37(+101%) | 42.89(+100%) |
|  | Overhead (W1+W2) bits | 8+4 bits for rank 3 8+3 bits for rank 4 | 9+4 bits for rank 3 9+3 bits for rank 4 | 6+4 bits for both rank 3 and rank 4 |
| Config 4 | RU | 0.49 | 0.48 | 0.45 |
|  | 5% | 12.98(+100%) | 13.94(+107.4%) | 15.26(+117.6%) |
|  | 50% | 37.74(+100%) | 39.45(+104.5%) | 42.05(+114.2%) |
|  | Mean | 41.39(+100%) | 42.41(+102.5%) | 43.79(+103.3%) |
|  | Overhead (W1+W2) bits | 5+4 bits for rank 3 5+3 bits for rank 4 | 6+4 bits for rank 3 6+3 bits for rank 4 | 4+4 bits for both rank 3 and rank 4 |

In Table 4 we investigate into the impact of removing the cophasing between two pols with the same type on the performance gain.

scheme 1：codebook config 2 of Alt 1 in [5], and all the four codeword are used

scheme 2：codebook config 2 of Alt 1 in [5], only cw0 and cw3 are reserved.

It is seen that removing the cophasing between two pols hardly impacts the system performance. Only the mean throughput is 1% down. It means that cophasing has very low priority.

# Table 4.  Evaluates the impact of removing co-phasing information

| Scheme X | RU | Mean | 5% | 50% | Overhead (W1+W2)bits |
| --- | --- | --- | --- | --- | --- |
| Scheme 1 | 0.4844 | 43.33 (+100%) | 12.49 (+100%) | 38.71 (+100%) | 8+4 bits for rank 3 and 8+3 bits for rank 4 |
| Scheme 2 | 0.4853 | 42.93 (+99.1%) | 12.33 (+98.72%) | 38.82 (+100.2%) | 7+3 bits for rank 3 and 8+3 bits for rank 4 |

By combining the results in Table 3 and Table 4, it can be shown that some of the performance gain comes from removing 1 bit cophasing and putting the bit in increasing the number of (δ1, δ2) selections, and (δ1, δ2) is fed back in sub-bands.

Base on the analysis and the evaluation, we have the following proposals:

Proposal 1: For different antenna configuration (N1,N2), use different number of (δ1, δ2) selections, and select (δ1, δ2) in sub-bands.

Proposal 2: Employ W1 with consecutive structure. i.e. p1,p2=(1,1)

Proposal 3: Under the constraint on the bits of feed-back overhead, the selection of codeword Type has higher priority than the co-phasing between two pols.

Proposal 4: Adopt the rank 3 and rank 4 codebook in Appendix for Config 1,2,4.

From our rank 2 evaluation in [6], Config 3 always have lower performance than other configs. Also, rank 3 and rank 4 codebook does not seem to need so many configurations as it targets at scenarios with less correlated channel with more scatters.  Therefore, we have the following proposal:

Proposal 5: Do not support Config 3 for higher rank codebook.

- Rank 5-8 codebook

From rank3-4 codebook evaluation, it can be observed that performance of higher rank codebook actually depends quite a lot on subband information. Proposal in [4] follows the principle of Rel-10 8Tx codebook with fixed W2 codeword.  In Rel-10, higher rank codebook is defined to meet the peak data rate requirement. It does not seem urgent for FD-MIMO to define rank5-8 codebook as most of the commercial UEs in the market does not even support rank4.  Therefore, more time should be spent to design a better rank5-8 codebook suitable for FD-MIMO.  Therefore, our proposal is:

Proposal 6: Rank5-8 is not supported in the new codebook in Rel-13. Further study rank5-8 codebook in the future release.

- Conclusion

Based on the discussion and simulation results, we have the following proposals：

Proposal 1: For different antenna configuration (N1,N2), use different number of (δ1, δ2) selections, and select (δ1, δ2) in sub-bands.

Proposal 2: Employ W1 with consecutive structure. i.e. p1,p2=(1,1)

Proposal 3: Under the constraint on the bits of feed-back overhead, the selection of codeword Type has higher priority than the co-phasing between two pols.

Proposal 4: Adopt the rank 3 and rank 4 codebook in Appendix.

Proposal 5: Do not support Config 3 for higher rank codebook.

Proposal 6: Rank5-8 is not supported in the new codebook in Rel-13. Further study rank5-8 codebook in the future release.

- References

- 3GPP TR36.897 v13.0.0, “Study on Elevation Beamforming/Full-Dimension (FD) MIMO for LTE”, June. 2015.

- R1-156217, WF on class A and class B CSI reporting for Rel.13 EB/FD-MIMO. AT&T, BeijingXinwei Telecom Tech., CATR, CATT, CHTTL, CMCC, Deutsche Telekom, Ericsson, ETRI, Huawei, HiSilicon, ITRI, Kathrein-Werke KG, KDDI, KT Corporation, Nokia Networks, NTT DOCOMO, Samsung, Sony Corporation.

- 3GPP TSG RAN1 #82bis, chairman’s Notes, Oct 2015.

- R1-156390, “Joint Proposal on rank 3-4 codebook for Class A CSI reporting”, Samsung, Ericsson, NTT DOCOMO, CATT.

- R1-156336, “Joint Proposal on rank 3-4 codebook for Class A CSI reporting”, Samsung, Ericsson.

- R1-156833, “Rank-2 Codebook Design for Class A”, ZTE

- Appendix

where .

The following tables are rank 3 and rank 4 codebooks constructed for N1≥N2. For the case N1<N2, exchange s1i11 and s1i11+O1 with s2i12 and  s2i12+O2, respectively.

# Table 6-1: Codebook for rank-3/4 CSI reporting

| Config X | Rank 3 codebook | Rank 4 codebook |  | Selected indices for |
| --- | --- | --- | --- | --- |
| Config 1 | Table 6-2 | Table 6-5 |  | 0-5 for (2, 2) 0-9 for (3, 2) 0-13 for (4, 2) |
| Config 2 | Table 6-3 | Table 6-6 |  | 0-15 for all |
| Config 4 | Table 6-4 | Table 6-7 |  | 0-7 for (2, 2) 0-15 for (3, 2) 0-15 for (4, 2) |

# Table 6-2  Config 1 codebook for 3 layer CSI reporting

|  | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
|  | 4 | 5 | 6 | 7 |
|  | 8 | 9 | 10 | 11 |
|  | 12 | 13 | 14 | 15 |

# Table 6-3  Config 2 codebook for 3 layer CSI reporting

|  | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
|  | 4 | 5 | 6 | 7 |
|  | 8 | 9 | 10 | 11 |
|  | 12 | 13 | 14 | 15 |

# Table 6-4  Config 4 codebook for 3 layer CSI reporting

|  | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
|  | 4 | 5 | 6 | 7 |
|  | 8 | 9 | 10 | 11 |
|  | 12 | 13 | 14 | 15 |

# Table 6-5  Config 1 codebook for 4 layer CSI reporting

|  | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
|  | 4 | 5 | 6 | 7 |
|  | 8 | 9 | 10 | 11 |
|  | 12 | 13 | 14 | 15 |

# Table 6-6  Config 2 codebook for 4 layer CSI reporting

|  | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
|  | 4 | 5 | 6 | 7 |
|  | 8 | 9 | 10 | 11 |
|  | 12 | 13 | 14 | 15 |

# Table 6-7  Config 4 codebook for 4 layer CSI reporting

| 0; 1; 2; 3. | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
|  | 4 | 5 | 6 | 7 |
|  | 8 | 9 | 10 | 11 |
|  | 12 | 13 | 14 | 15 |