3GPP TSG RAN WG1 Meeting #83                                                        R1-157149

Anaheim, USA, 15th - 22nd November 2015

Source: 	Ericsson

Title:	Physical layer aspects of short TTI for uplink transmissions

Agenda Item:	6.2.9.2

Document for:	Discussion and Decision

# Introduction

In this contribution, we provide our views on the physical layer design for TTI shortening [1][2] for uplink transmissions. We describe how to extend the design for PUSCH with a shortened TTI length. Further we discuss the scheduling mechanism for PUSCH with shortened TTI to be able to leverage the benefit of a short PUSCH.

# Discussion

The uplink shared channel PUSCH for LTE is based on single carrier transmissions (SC-FDMA, Single Carrier Frequency Division Duplex) by using DFTS-OFDM (Discrete Fourier Transform Spread OFDM). Here, two SC-FDMA symbols of each sub-frame are dedicated to reference signals and the remaining 12 SC-FDMA symbols (for normal cyclic prefix) can be allocated to user data. The last SC-FDMA symbol of each sub-frame might be used for Sounding Reference Signals (SRS) such that PUSCH is only transmitted during 11 SC-FDMA symbols.

## Reduced processing and feedback timing

The timing of HARQ feedback in relation to a received Transport Block (TB) on PUSCH depends on the required time for processing of PUSCH and preparation of the UL data and retransmission grants. Here the processing time depends on the number of SC-FDMA symbols used for PUSCH and the Transport Block Size (TBS). In contrast to this, the preparation of the UL grants do not depend on the length of the TTI of PUSCH but rather on the number of scheduled UEs.

UL HARQ operation is based on synchronous HARQ in current LTE. This implies that the retransmission time is fixed. Even if the HARQ RTT could be set to different values for different TTI lengths, this might be too restrictive for the eNB operation.

Even with synchronous operation, HARQ feedback in downlink for the UL TB may not necessarily be done in the same order as the TBs are received in the eNB, since the retransmission time may depend on the length of the TTI and TBs can have different lengths of the TTIs [1]. For some cases, two ACK/NACKs might need to be transmitted simultaneously in downlink. Thus synchronous operation can be challenging with shortened TTIs.

When the UE is operating with shortened TTI we therefore propose that asynchronous HARQ operation is applied. The HARQ principles identified in the LAA SI can be the basis for the design, with adjustment done for the needed number of HARQ processes. This number is determined from the estimated UE and eNB processing times. For simplicity, the same number of HARQ processes could be assumed for different TTI lengths, with the number chosen from the most stringent case.

- Proposal

- Recommend that asynchronous HARQ for PUSCH is used when operating with shortened TTI.

  - Asynchronous HARQ design follows the recommendations within the LAA TR 36.889.

## Uplink shared channel

An uplink shared channel PUSCH with shortened TTI should be based on one or a few data SC-FDMA symbols. These symbols for user data also require an additional symbol used for reference signal in the UL TTI. In principle, it is beneficial if the reference symbol can be sent early in the TTI allowing the eNB to start processing as soon as possible, however this may not always be practical.

If the RS overhead is a concern, there are possibilities to reduce the number of DMRS transmissions by sharing the same RS symbol between multiple UEs, while not sharing the data symbols. The eNB would then be able to separate the different UEs by assigning different cyclic shifts similar to UL MU-MIMO. Further reduction of the DMRS overhead can be achieved for UEs scheduled in multiple shortened TTIs within a limited time window by not transmitting RS in every TTI.

As an additional benefit the reference signal in a very short TTI (e.g. a 2-symbol TTI) can provide better channel estimation compared to the legacy 1ms TTI, especially when the Doppler spread is high, implying that very short TTIs have the potential to support high mobility scenarios.

Considering the above aspects we here highlight two design options with different TTI lengths. In Figure 1 an example is illustrated of a TTI length of 3 for data. Here two users share the same position for their UL DMRS. For multiplexing of more RS we expect the channel estimation to degrade due to interference between the different users.

Figure 1: PUSCH with a TTI of 3 user data SC-FDMA symbols and multiplexing of RS from 2 UEs in the same SC-FDMA symbol.

In the second example in Figure 2 the UL DMRS positions have been moved, to allow for a 1 symbol data TTI. Here the RS overhead is increased. Also in this example 2 UEs are sharing each UL DMRS.

Figure 2: PUSCH with a TTI of 1 SC-FDMA user data symbol and multiplexing of RS from 2 UEs in the same SC-FDMA symbol.

In a third example in Figure 3 each TTI has one RS sent before the data symbols, where only the scheduled UE transmit.

Figure 3: PUSCH with a TTI of 3 or 1 SC-FDMA user data symbols with separate RS in each TTI.

- Proposal

- Study the follow options

  - PUSCH data

    - Possible location of the data symbols

    - Amount of variability in the TTI length that is supported

  - PUSCH DMRS

    - Whether or not to fix the position independent from the PUSCH data location

    - Allow multiple UEs to share the same UL DMRS location

    - The number of UL DMRS

## Scheduling of reduced TTI

Following the discussions in [2] and [3] we observe that it is necessary to provide UL scheduling at the short TTI scale in order to fully utilize the latency benefits from shorter TTIs. In the companion DL paper [3], a design is outlined for a short PDCCH transmitted in every TTI.

As for the DL [3] the overall UL DCI overhead needs to be limited. One means for this is to split the scheduling information into two parts: not all control information needs to be transmitted for every shortened TTI, some can instead be transmitted once per subframe or configured by higher layers.

Observation:

- Due to the frequent transmission of control information in every shortened TTI, it is important to limit the amount of UL DCI overhead in DL.

The need for UL grants can be limited by using SPS-based scheduling and overhead efficient retransmission schemes. In addition the size of the fast DCI should be limited.

Observation:

- To limit control overhead in DL, the number and payload of the fast UL DCI could be limited.

# Conclusion

In this contribution we discussed our views on the physical layer design for short TTIs in uplink transmissions. The above discussion is summarized with the following observations and proposals:

Observations:

- Due to the frequent transmission of control information in every shortened TTI, it is important to limit the amount of UL DCI overhead in DL.

- To limit control overhead in DL, the number and payload of the fast UL DCI could be limited.

Proposals:

- Recommend that asynchronous HARQ for PUSCH is used when operating with shortened TTI.

  - Asynchronous HARQ design follows the recommendations within the LAA TR 36.889.

- Study the follow options

  - PUSCH data

    - Possible location of the data symbols

    - Amount of variability in the TTI length that is supported

  - PUSCH DMRS

    - Whether or not to fix the position independent from the PUSCH data location

    - Allow multiple UEs to share the same UL DMRS location

    - The number of UL DMRS

# References

R2-153489, Areas for latency reduction, Ericsson, RAN2#91, August 2015.

R1-157146, Overview of TTI shortening. Ericsson, November 2015.

R1-157148, Physical layer aspects of TTI shortening for downlink transmissions. Ericsson, November 2015.