# 3GPP TSG-RAN WG1 #83	R1-15xxxx

# 16th – 20th November 2015

# Anaheim, California, USA

Source:                    	Ericsson

Title:  		Uplink Shared Channel Evaluations in Standalone Deployment

Document for:        	Discussion

Agenda Item:         	6.2.6.1.1

# Introduction

At RAN#69, a new work item named NarrowBand IOT (NB-IOT) was approved, see [1]. The objective is to specify a radio access for cellular internet of things that addresses improved indoor coverage, support for massive number of low throughput devices, low delay sensitivity, ultra-low device cost, low device power consumption and (optimized) network architecture.

NB-IOT should support 3 different modes of operation:

1.	“Stand-alone operation” utilizing for example the spectrum currently being used by GERAN systems as a replacement of one or more GSM carriers

2.	“Guard band operation” utilizing the unused resource blocks within a LTE carrier’s guard-band

3.	“In-band operation” utilizing resource blocks within a normal LTE carrier

NB-IOT currently includes two options under consideration, SC-FDMA and FDMA with GMSK modulation. In   [2], an uplink design based on SC-FDMA with 2.5 kHz subcarrier spacing for M-PUSCH is described. In this contribution, we present performance evaluation results for this option in a standalone deployment.

# M-PUSCH formats for 2.5 kHz Subcarrier Spacing

Three M-PUSCH formats are provided, which are intended for users at different MCLs. The “basic” format is intended for users targeting 144 dB MCL, the “robust” format is used by those targeting 154 dB MCL and the “extreme” format is targeted at users with 164 dB MCL. The MCS for the three different formats is specified in Table 1.

Table 1: M-PUSCH formats for different MCLs with 2.5 kHz Subcarrier Spacing

|  | Extreme | Robust | Basic |
| --- | --- | --- | --- |
| Coding scheme | Turbo Code | Turbo Code | Turbo Code |
| Code rate (per repetition) | 0.21 | 0.26 | 0.23 |
| modulation | BPSK | QPSK | QPSK |
| # repetition | 1 | 1 | 1 |
| # subcarriers | 1 | 2 | 24 |
| Total transmission time | 1920 ms (320 M-subf.) | 384 ms (64 M-subf.) | 36 ms (6 M-subf.) |
| Effective data rate (measured above SNDCP) | 354 bps | 1.8 kbps | 18.9 kbps |

# Performance Evaluations

## Simulation Assumptions

The simulation assumptions are listed in Table 2.

Table 2: Simulation assumptions for link level evaluations

| Parameter | Value |
| --- | --- |
| Frequency band | 900 MHz |
| Propagation channel model | TU |
| Doppler spread | 1 Hz |
| Interference/noise | Sensitivity |
| Antenna configuration | BS: 1 Tx, 2 Rx MS: 1 Tx, 1 Rx |
| Timing error | Basic and Robust formats: Randomly distributed between [-3.125, 3.125] us. (see [4]) Extreme format: Randomly distributed between [-31.25, 31.25] us. (see [4]) |
| Frequency error | F_offset(t) = F_est_error + (F_drift_active * t). |
| NB LTE specific frequency error (F_est_error) | Randomly chosen from the set {-50, 50} Hz |
| Frequency drift rate (F_drift_active) | 22.5 Hz/second |
| MS transmit power (dBm) | 23 dBm |
| Thermal noise density (dBm/Hz) | -174 |
| BS Receiver noise figure (dB) | 3 |
| Receiver processing gain (dB) | 0 |

The NB-IOT users using the basic and robust formats use a root-raised cosine transmit filter with rolloff 0.1 and bandwidth 240 kHz. For the extreme format user, the gaussian transmit filter described in [5] is used to ensure low PAPR.

## Simulation Results

The results are shown in terms of Block Error Rates (BLER) with varying SNR. The results for 2.5 kHz subcarrier spacing are shown in Figure 1. The SNR required for achieving 10% block error rate (BLER) is used in the MCL calculation presented in Table 3. The MCL calculation methodology is according to table 5.1-1 of [3].

Figure 1: BLER vs SNR for different M-PUSCH formats with 2.5 kHz subcarrier spacing

Table 3: M-PUSCH MCL Evaluations

| Logical channel name | M-PUSCH (extreme) | M-PUSCH (robust) | M-PUSCH (basic) |
| --- | --- | --- | --- |
| Data rate(kbps) above SNDCP | 0.354 | 1.77 | 18.9 |
| Transmitter |  |  |  |
| (1) Tx power (dBm) | 23 | 23 | 23 |
| Receiver |  |  |  |
| (2) Thermal noise density (dBm/Hz) | -174 | -174 | -174 |
| (3) Receiver noise figure (dB) | 3 | 3 | 3 |
| (4) Interference margin (dB) | 0 | 0 | 0 |
| (5) Occupied channel bandwidth (Hz) | 2,500 | 5,000 | 80,000 |
| (6) Effective noise power = (2) + (3) + (4) + 10 log ((5)) (dBm) | -137.0 | -134.0 | -122.0 |
| (7) Required SINR (dB) | -5.8 | -0.6 | 0.8 |
| (8) Receiver sensitivity = (6) + (7) (dBm) | -142.8 | -134.6 | -122.4 |
| (9) Rx processing gain | 0 | 0 | 0 |
| (10) MCL = (1) (8) + (9) (dB) | 165.8 | 157.6 | 145.4 |

# Conclusions

In this contribution, we presented link level evaluation results for NB-IoT uplink using SC-FDMA in standalone deployment.

Observation 1: A properly chosen MCS can satisfy the coverage requirement of 164 dB MCL, while achieving a minimum data rate of 354 bps (which is greater than the requirement of 160 bps in [2]).

# References

RP-151621, “New Work Item: Narrowband IOT (NB-IOT),” Qualcomm, Sept. 2015, Phoenix, AZ, USA.

GP-150782, “Narrowband LTE – PDSCH and PUSCH Design and Performance”, Ericsson LM, Nokia Networks, GERAN #67, Aug. 2015, Yin Chuan, China.

3GPP TR 45.820, “Technical Specification Group GSM/EDGE Radio Access Network; Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things,” Aug. 2015, v.2.1.0.

R1-156011, “Narrowband LTE – Random Access Design”, Ericsson, RAN1 #82bis, Oct. 2015, Malmo, Sweden.

R1-156012, “NB-LTE – UL Modulation”, Ericsson, RAN1 #82bis, Oct. 2015, Malmo, Sweden.

The transmitted packet size is 85 bytes

Obtained from Figure 1