3GPP TSG RAN1 Meeting #83								R1-157411

Anaheim, CA, USA, 16th– 20th November, 2015

Source: 		Ericsson

Title:	NB-IoT - Battery lifetime evaluation for inband operation

Agenda Item:	6.2.6.1.3

Document for:	Discussion and decision

# Introduction

At RAN#69 a new work item on narrowband IoT and tasked RAN1 to evaluate two numerology options for both UL and DL:

- DL: 15 kHz sub-carrier spacing (with normal or extended CP) and 3.75 kHz sub-carrier spacing.

- UL: FDMA with GMSK modulation and SC-FDMA (including single-tone transmission as a special case of SC-FDMA)

One of the performance objectives is for the IoT device to meet the battery lifetime requirement of 10 years. Even if a possible down selection of DL and UL numerology could be seen as independent evaluations, in order to estimate the battery lifetime, a system with both DL and UL must be assumed for the evaluation. For this reason, in this contribution we have evaluated a NB-IoT system as described in [3] in the inband operation using the methodology given in TR45.820 [1]. The evaluation in [2] has been updated by aligning with the assumptions used in [1] for evaluations of NB-CIoT in and.

# NB-LTE energy consumption evaluation

## Updated Assumptions

This analysis follows the methodology as described in [2] with the following updates:

- The transmission time for the 200 byte IP report on PUSCH for 164 dB MCL at has been reduced to 2760 ms since the previous value was pessimistic resulting in an MCL of 166.1 dB. The new transmission time results in an MCL of 164.4 dB.

- Update to latest PBCH design [6]

- In order to align with the evaluation in section 7.3.6.4 in [1], the effect of BLER has only been included for PUSCH and for other channels no retransmissions are assumed.

- In order to align with the evaluation in section 7.3.6.4 in [1] and [5], the UE only wakes up three times during the 20 s ready timer. During the DRX, the UE is in standby mode.

## Results

The overall reception, transmission and idle times per report at different coverage based on the new assumptions are shown in Table 1.

Table 1 Transmission, Reception and Idle time per report (ms)

|  | 144 dB | 154 dB | 164 dB |
| --- | --- | --- | --- |
| Tx (50 bytes) | 48 | 334 | 2110 |
| Tx (200 bytes) | 80 | 1005 | 3720 |
| Rx | 160 | 482 | 2421 |
| Idle | 1271 | 1505 | 3123 |

The estimated lifetime in years are presented in Table 2for two different packet sizes, two reporting intervals and at different coverage.

Table 2 estimated battery lifetime in years

| Packet size, reporting interval | 144 dB | 154 dB | 164 dB |
| --- | --- | --- | --- |
| 50 bytes, 2 hours | 27.0 | 12.2 | 2.8 |
| 200 bytes, 2 hours | 24.2 | 5.8 | 1.7 |
| 50 bytes, 1 day | 36.8 | 32.3 | 18.5 |
| 200 bytes, 1 day | 36.3 | 26.1 | 13.9 |

We see that with the updated assumptions in section 2.1, the estimated battery life for NB-LTE is further improved fulfilling the 10 year target for battery lifetime and meets the 10 year requirement for the same scenarios as when operating standalone[7].

Observation 1:

- NB-LTE can meet the battery target of 10 years for the same scenarios when operating inband as for standalone operation.

# Conclusions

In this contribution we evaluated battery lifetime of the NB-LTE when operating inband and made the following observation.

Observation:

- A NB-LTE can meet the battery target of 10 years for the same scenarios when operating inband as for standalone operation.

# References

TR 45.820 v13.0.0, “Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things”

R1-156019, “NB LTE – Exception report latency evaluation in inband operation”, Ericsson

R1-156013, “NB-LTE – General L1 concept description”, Ericsson, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, Nokia, Intel, ZTE, Samsung

R1-156009, “Narrowband LTE – Synchronization Channel Design and Performance”, Ericsson

GP-150137, “NB M2M – Battery life analysis”, Neul

R1-157422, “NB-IoT - M-PBCH design update”, Ericsson

R1-157390, “NB-IoT - Battery lifetime comparison for standalone operation”, Ericsson