3GPP TSG RAN1 Meeting #83								R1-157412

Anaheim, CA, USA, 16th– 20th November, 2015

Source: 	Ericsson

Title:	NB-IoT - Exception report latency evaluation for inband operation

Agenda Item:	6.2.6.1.3

Document for:	Discussion and decision

# Introduction

At RAN#69 a new work item on narrowband IoT and tasked RAN1 to evaluate two numerology options for both UL and DL:

- DL: 15 kHz sub-carrier spacing (with normal or extended CP) and 3.75 kHz sub-carrier spacing.

- UL: FDMA with GMSK modulation and SC-FDMA (including single-tone transmission as a special case of SC-FDMA)

One of the performance objectives for the IoT device to meet is the latency target of 10 ms for an MAR exception report[1]. Even if a possible down selection of DL and UL numerology could be seen as independent evaluations, in order to estimate the latency, a system with both DL and UL must be assumed for the evaluation. For this reason, in this contribution we will show that a NB-IoT system as described in [2] with can meet the latency requirement when operating inband.

# Evaluation

The difference to the analysis in [3] is:

- Latency results have been added for all MCLs

- The latest PBCH design has been used [4]

Table 1 shows the latency results for delivery of a MAR exception report when operating inband. An additional case has been added for the 164 dB MCL case where the BLER target is set to the 1% in order to meet the 99% confidence level already in the first transmission reducing the latency for the maximum MCL case.

Table 1 Exception report delivery time with 90% confidence

| Coupling loss (dB) | 144 | 154 | 164 | 164 |
| --- | --- | --- | --- | --- |
| Initial BLER target | 10% | 10% | 10% | 1% |
| Tsync | 290 | 310 | 1110 | 1110 |
| TMIB | 151 | 351 | 1911 | 1911 |
| TPRACH | 324 | 688 | 1440 | 1440 |
| TCHANNEL FREE | 740 | 740 | 740 | 740 |
| TRAmsg2-4 | 44 | 200 | 840 | 840 |
| TULgrant | 47 | 29 | 129 | 129 |
| TULdata | 39 | 553 | 1923 | 2303 |
| Total time (90% conf) | 1635 | 2871 | 8093 |  |
| TAck | 41 | 47 | 157 |  |
| TULdata | 39 | 553 | 1923 |  |
| Total time (99% conf) | 1715 | 3471 | 10173 | 8473 |

One can see that with 10% initial BLER, the 10 s target is just missed, but 10 s with 99% confidence can be achieved using 1 % BLER target for the initial transmission. Based on these results we make the following observation:

Observation 1:

- When operating  inband, NB-LTE can me meet the latency target of 10 s for an exception report for all coverage classes

# Conclusions

This contribution presents the latency evaluation of delivering the exception report in NB-LTE. The following observations were made:

Observation 1:

- When operating  inband, NB-LTE can me meet the latency target of 10 s for an exception report for all coverage classes

# References

TR 45.820 v13.0.0, “Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things”

R1-156013, “NB-LTE – General L1 concept description”, Ericsson, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, Nokia, Intel, ZTE, Samsung

R1-156019, “NB LTE – Exception report latency evaluation in inband operation”,  Ericsson

R1-157422, “NB-IoT - M-PBCH design update”, Ericsson