3GPP TSG RAN1 Meeting #83								R1-157451

Anaheim, CA, USA, 16th– 20th November, 2015

Source: 	Ericsson

Title:	NB-IoT - Exception report latency evaluation for guardband operation

Agenda Item:	6.2.6.1.2

Document for:	Discussion and decision

# Introduction

At RAN#69 a new work item on narrowband IoT and tasked RAN1 to evaluate two numerology options for both UL and DL:

- DL: 15 kHz sub-carrier spacing (with normal or extended CP) and 3.75 kHz sub-carrier spacing.

- UL: FDMA with GMSK modulation and SC-FDMA (including single-tone transmission as a special case of SC-FDMA)

One of the performance objectives for the IoT device to meet is the latency target of 10 ms for an MAR exception report[1]. Even if a possible down selection of DL and UL numerology could be seen as independent evaluations, in order to estimate the latency, a system with both DL and UL must be assumed for the evaluation. For this reason, in this contribution we will show that a NB-IoT system as described in [2] with can meet the latency requirement when operating in the guard band. This analysis follows the same template as the inband evaluation in [3].

This contribution is an update of contribution [4] where the updated cell search results from [5] have been included.

# Evaluation

The difference to the inband analysis in [3] is that due to the lower DL overhead for guard band operation, the transmission times in the DL have been reduced according to the reduction in overhead. The same power boosting of 6 dB is assumed also for the guard band case. The cell search results for guard band operation from [5] have been used.

Table 1 shows the latency results for delivery of a MAR exception report when operating in the guard band. An additional case has been added for the 164 dB MCL case where the BLER target is set to the 1% in order to meet the 99% confidence level already in the first transmission reducing the latency for the maximum MCL case.

Table 1 Exception report delivery time with 90% confidence

| Coupling loss (dB) | 144 | 154 | 164 | 164 |
| --- | --- | --- | --- | --- |
| Initial BLER target | 10% | 10% | 10% | 1% |
| Tsync | 210 | 250 | 850 | 850 |
| TMIB | 151 | 351 | 1911 | 1911 |
| TPRACH | 324 | 688 | 1440 | 1440 |
| TCHANNEL FREE | 539 | 539 | 539 | 539 |
| TRAmsg2-4 | 44 | 194 | 696 | 696 |
| TULgrant | 47 | 32 | 121 | 121 |
| TULdata | 39 | 553 | 1923 | 2300 |
| Total time (90% conf) | 1354 | 2607 | 7420 |  |
| TAck | 41 | 47 | 117 |  |
| TULdata | 39 | 553 | 1923 |  |
| Total time (99% conf) | 1434 | 3207 | 9460 | 7797 |

Based on these results we make the following observation:

Observation 1:

- NB-LTE can me meet the latency target of 10 s for an exception report for all coverage classes when operating  in the guard band

# Conclusions

This contribution presents the latency evaluation of delivering the exception report in NB-LTE. The following observations were made:

Observation 1:

- NB-LTE can me meet the latency target of 10 s for an exception report for all coverage classes when operating  in the guard band

# References

TR 45.820 v13.0.0, “Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things”

R1-156013, “NB-LTE – General L1 concept description”, Ericsson, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, Nokia, Intel, ZTE, Samsung

R1-157412, “NB-IoT - Exception report latency evaluation for inband operation”,  Ericsson

R1-157403, “NB-IoT - Exception report latency evaluation for guardband operation”, Ericsson

R1-157455, “NB-IoT - Improved Synchronization Channel Design”,	Ericsson