3GPP TSG-RAN WG1 Meeting #83	                                                               R1-157541

Anaheim, USA, 15th - 22th November 2015

Agenda item:		6.2.7.1

Source:	Nokia Networks

Title:	System-level performance of MUST

Document for:		Discussion and Decision

# 1	Introduction

In the meeting #82, MUST schemes were categorized into 3 groups [1]. In this contribution we investigate the system level performance of Category 2 and Category 3 that is schemes based on Gray mapping and label-bit assignment. These two categories have similar characteristics. Category 3 is allowed to employ only LTE uniform constellations while Category 2 can be treated same as category 3, with additional definition of non-uniform constellations.

# 2	System performance results with full buffer traffic

This section presents the results obtained by LTE system-level simulator. We perform wideband scheduling and we restrict feedback to rank-1 only. The RML L2S has been described in detail in our previous contribution [2]. We simulate the full-buffer traffic with 10UEs per sector and Gray encoding is enforced to all super-constellation, no overlapping constellations have been simulated. Detailed simulation assumptions can be found in Appendix.

The goal of these simulations is to answer following questions:

- Are existing LTE uniform constellations sufficient for MUST operation?

- Can far-UE be restricted in QPSK only?

We have simulated MUST with the constellation combinations and power offsets cases summarized in Table 1. Cases 2,6,8,9 and 10 are employing uniform constellations, the rest employs non-uniform constellations.

Table 1 Simulated super constellations

| Case | Near UE power | Super-constellation | Far-MOD | Near-MOD |
| --- | --- | --- | --- | --- |
| 1 | 0.10 | Non-uniform 16QAM | 2 | 2 |
| 2 | 0.20 | LTE 16QAM | 2 | 2 |
| 3 | 0.10 | Non-uniform 64QAM | 2 | 4 |
| 4 | 0.15 | Non-uniform 64QAM | 2 | 4 |
| 5 | 0.20 | Non-uniform 64QAM | 2 | 4 |
| 6 | 0.238 | LTE 64QAM | 2 | 4 |
| 7 | 0.15 | Non-uniform 256QAM | 2 | 6 |
| 8 | 0.247 | LTE 256QAM | 2 | 6 |
| 9 | 0.059 | LTE 256QAM | 4 | 4 |
| 10 | 0.048 | LTE 64QAM | 4 | 2 |
| 11 | 0.012 | LTE 256QAM | 6 | 2 |

Three setups have been considered:

- Setup #1: uses uniform constellations cases 2, 6 and 8, allowing only cases where far-UE can be scheduled only in QPSK. This is the baseline MUST scheme, requiring the least signalling and network assistance.

- Setup #2: uses uniform constellation cases 2, 6, 8, 9, 10 and 11, allowing far-UE to enter higher modulation orders.

- Setup #3: uses constellation cases 1 to 8, allowing as well non-uniform constellation cases, however restricting far-UE to QPSK only.

The results are summarized in Table 2. It can be observed that difference between three different cases is small.  The baseline-MUST scheme can deliver majority of MUST gain. The small loss of baseline-MUST scheme can be partially recovered by employing non-uniform constellation cases, allowing flexible power ration between Near- and Far-UE.

Observation 1:  Forcing far-UE into QPSK brings most of MUST gains.

Table 2: MUST category 2/3 results for 2Tx with R-ML receiver and FB traffic

| Throughput (Mbps) | Baseline | MUST Category 2/3 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | MUST w Case 2,6, 8 | Gain | MUST w Case 2,6,8,9,10,11 | Gain | MUST w Case 1 to 8 | Gain |
| Cell average | 1.46 | 1.583 | 8.3 % | 1.608 | 10.0 % | 1.597 | 9.3% |
| Cell edge | 0.2658 | 0.2991 | 12.5 % | 0.2960 | 11.4 % | 0.3039 | 14.4% |
| Note: | Same-beam pairing only, legacy feedback, rank1 only |  |  |  |  |  |  |

# 3	System performance results with FTP traffic

This section presents the results obtained by LTE system-level simulator by considering FTP1 traffic with λ = 8 and 10 packets/s call arrival rate and 100 kB packet size. Gray encoding is enforced to the super-constellation in Table 3-Table 5. Detailed simulation assumptions can be found in the Appendix.

Table 3 shows the 2Tx performance of baseline SU-MIMO with wide-band scheduling and MUST Setup #2 with and without PMI forcing. The gains are modest due to very low MUST utilization.  PMI forcing assisted with 2nd best CQI and PMI feedback improves the performance of MUST in low and middle percentile.

Table 4 shows the same setup as Table 3, except RU is around 80%. eNBs starts to drop low performing packets after 1.6s. These packets would benefit from MUST, therefore MUST gains are not improved compared to Table 3.  PMI forcing assisted with 2nd best CQI and PMI feedback improves the performance of MUST in low and middle percentile.

Table 5 shows the same setup as Table 3, except eNB is equipped with 4Tx antennas. Gains are modest due to low MUST utilization. PMI forcing assisted with 2nd best CQI and PMI feedback improves the performance of MUST in coverage.

Table 6 and Table 7 shows Category 1 results with ideal CWIC receiver and RU ~65% and ~82%.  Gains are similar to Category 2/3  with RML.

Observation 2: System with FTP1 traffic and medium/high loads does not benefit from MUST operation.

Table 3: MUST category 2/3 results for 2Tx with R-ML receiver and FTP traffic

| Throughput (Mbps) | Low Load (~64% RU) with packet size of [100] KB |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Baseline WB | MUST Category 2/3 (rank 1 limited) WB |  |  |  |  |
|  |  | MUST with cases 1 to 8 | Gain | MUST with cases 1 to 8 PMI forcing | Gain |
| Mean UPT | 9.92 | 10.09 | +1.7% | 10.12 | +1.9% |
| 5%ile UPT | 1.26 | 1.34 | +6.3% | 1.42 | +12.7% |
| 50%ile UPT | 8.42 | 8.70 | +3.3% | 8.79 | +4.4% |
| 95%ile UPT | 23.5 | 23.53 | 0.1% | 28.86 | -2.7% |
| RU | 65.8% | 64.7% |  | 63.8% |  |
| Server/offered | 99.29% | 99.37% |  | 99.44% |  |
| λ | 8 packet/s |  |  |  |  |  |
| Note | Rank 1 limited results, 10s simulated in all cases. |  |  |  |  |  |

Table 4: MUST category 2/3 results for 2Tx with R-ML receiver and FTP traffic

| Throughput (Mbps) | Medium Load (~80% RU) with packet size of [100] KB |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Baseline WB | MUST Category 2/3 (rank 1 limited) WB |  |  |  |  |
|  |  | MUST with cases 1 to 8 | Gain | MUST with cases 1 to 8 PMI forcing | Gain |
| Mean UPT | 8.67 | 8.76 | +1.0% | 8.76 | +1.0% |
| 5%ile UPT | 0.78 | 0.81 | +4.4% | 0.83 | +7.3% |
| 50%ile UPT | 6.72 | 6.96 | +3.3% | 7.02 | +4.4% |
| 95%ile UPT | 22.22 | 22.22 | 0% | 22.22 | 0% |
| RU | 81.8% | 81.2% |  | 80.8% |  |
| Server/offered | 97.99% | 98.23% |  | 98.27% |  |
| λ | 10 packet/s |  |  |  |  |  |
| Note | Rank 1 limited results, 10s simulated in all cases. |  |  |  |  |  |

Table 5: MUST category 2/3 results for 4Tx with R-ML receiver and FTP traffic

| Throughput (Mbps) | Medium Load (~64% RU) with packet size of [100] KB |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Baseline WB | MUST Category 2/3 (rank 1 limited) WB |  |  |  |  |
|  |  | MUST with cases 1 to 8 | Gain | MUST with cases 1 to 8 PMI forcing | Gain |
| Mean UPT | 11.54 | 11.65 | +1.0% | 11.66 | +1.1% |
| 5%ile UPT | 1.68 | 1.75 | +4.4% | 1.79 | +6.5% |
| 50%ile UPT | 10.13 | 10.39 | +2.6% | 10.013 | +2.6% |
| 95%ile UPT | 25.81 | 25.81 | 0% | 25.81 | 0% |
| RU | 65.5% | 64.9% |  | 64.6% |  |
| Server/offered | 99.74% | 99.78% |  | 99.78% |  |
| λ | 10 packet/s |  |  |  |  |  |
| Note | Rank 1 limited results, 10s simulated in all cases. |  |  |  |  |  |

Table 6: MUST category 1 results for 2Tx with CWIC receiver and FTP traffic

| Throughput (Mbps) | Low Load (~65% RU) with packet size of [100] KB |  |  |
| --- | --- | --- | --- |
|  | Baseline WB | MUST Category 2/3 (rank 1 limited) WB |  |
|  |  | MUST with cases 1 to 8 | Gain |
| Mean UPT | 9.81 | 9.87 | +0.6% |
| 5%ile UPT | 1.25 | 1.32 | +5.6% |
| 50%ile UPT | 8.25 | 8.51 | +3.2% |
| 95%ile UPT | 23.53 | 22.86 | -2,85% |
| RU | 66.5% | 65.6% |  |
| Server/offered | 99.39% | 99.32% |  |
| λ | 8 packet/s |  |  |
| Note | Rank 1 limited results, 10s simulated in all cases. |  |  |

Table 7: MUST category 1 results for 2Tx with CWIC receiver and FTP traffic

| Throughput (Mbps) | Low Load (~82% RU) with packet size of [100] KB |  |  |
| --- | --- | --- | --- |
|  | Baseline WB | MUST Category 1 (rank 1 limited) WB |  |
|  |  | MUST with cases 1 to 8 | Gain |
| Mean UPT | 8.47 | 9.64 | +2.0% |
| 5%ile UPT | 0.71 | 0.77 | +8.5% |
| 50%ile UPT | 6.40 | 6.90 | +7.8% |
| 95%ile UPT | 23.53 | 21.62 | -2,7% |
| RU | 83.3% | 82.1% |  |
| Server/offered | 97.47% | 97.91% |  |
| λ | 10 packet/s |  |  |
| Note | Rank 1 limited results, 10s simulated in all cases. |  |  |

¨

# 4	Conclusions

In this contribution we have been presenting system-level results of MUST performance for schemes classified under categories 1,2 and 3. The following observations can be summarized:

Observation 1:  Forcing far-UE into QPSK brings most of MUST gains.

Observation 2: System with FTP1 traffic and medium/high loads does not benefit from MUST operation.

# References

- R1-154968, “Way forward on candidate MUST categories	“, MediaTek et al.

- R1-155891, “System-level evaluation of Category 2 and Category 3 MUST”, Nokia Networks.

# Appendix

Table 4 Simulation Assumptions FB

| Layout | Hexagonal grid, 3 sectors per site, 19 macro sites (ISD = 500 m) |
| --- | --- |
| System bandwidth per carrier | 10 MHz |
| Carrier frequency | 2.0 GHz |
| Total BS TX power (Ptotal per carrier) | 46 dBm |
| Distance-dependent path loss | ITU UMa |
| Penetration loss | For outdoor UEs:0dB For indoor UEs: 20dB+0.5din (din : independent uniform random value between [ 0, min(25,d) ] for each link) |
| Shadowing | ITU Uma |
| Antenna pattern | 3D (referring to TR36.819) |
| Antenna Height: | 25 m |
| UE antenna Height | 1.5 m |
| Antenna gain + connector loss | 17 dBi |
| Antenna gain of UE | 0 dBi |
| Fast fading channel between eNB and UE | ITU UMa |
| Antenna configuration | BS: 2Tx (0.5 lambda), cross-polarized UE: 2Rx (0.5 lambda), cross-polarized |
| Number of UEs per cell | 10 |
| UE dropping | 20% UEs are outdoor and 80% UEs are indoor. |
| Minimum distance from macro-cell to UEs | 35 m |
| Traffic model | Full buffer |
| UE receiver | IRC (CRS based) + RML per layer |
| Transmission mode | 2x2 TM4 (rank1 only) |
| UE noise figure | 9 dB |
| UE speed | 3 km/h |
| Cell selection criteria | RSRP |
| Handover margin | 3 dB |
| Scheduling algorithm | Proportional fairness maximization |
| HARQ | Redundancy Version |
| Feedback | WB rank1 only |
| CQI quantization | Yes |
| Codebook | 2Tx/4Tx LTE Rel. 8 |
| Power ratio sets | According to super-constellation |
| OLLA | Yes |
| Number of superposed signals in superposition transmission | 2 |
| Channel Estimation | Realistic |
| EVM | Tx/Rx 8/4% |

Table 5 Simulation Assumptions FTP

| Layout | Hexagonal grid, 3 sectors per site, 19 macro sites (ISD = 500 m) |
| --- | --- |
| System bandwidth per carrier | 10 MHz |
| Carrier frequency | 2.0 GHz |
| Total BS TX power (Ptotal per carrier) | 46 dBm |
| Distance-dependent path loss | ITU UMa |
| Penetration loss | For outdoor UEs:0dB For indoor UEs: 20dB+0.5din (din : independent uniform random value between [ 0, min(25,d) ] for each link) |
| Shadowing | ITU Uma |
| Antenna pattern | 3D (referring to TR36.819) |
| Antenna Height: | 25 m |
| UE antenna Height | 1.5 m |
| Antenna gain + connector loss | 17 dBi |
| Antenna gain of UE | 0 dBi |
| Fast fading channel between eNB and UE | ITU UMa |
| Antenna configuration | BS: 2(4)Tx, cross-polarized UE: 2Rx, cross-polarized |
| UE dropping | 20% UEs are outdoor and 80% UEs are indoor. |
| Minimum distance from macro-cell to UEs | 35 m |
| Traffic model | FTP1 |
| UE receiver | IRC (CRS based) for rank 1 + RR-ML/CWIC |
| Transmission mode | 2x2 TM4 (rank1 only) |
| UE noise figure | 9 dB |
| UE speed | 3 km/h |
| Cell selection criteria | RSRP |
| Handover margin | 3 dB |
| Scheduling algorithm | Proportional fairness maximization |
| HARQ | Redundancy Version |
| Feedback | WB rank1 only |
| CQI quantization | Yes |
| Codebook | 2Tx/4Tx LTE Rel. 8 |
| Power ratio sets | According to super-constellation |
| OLLA | Yes |
| Number of superposed signals in superposition transmission | 2 |
| Channel Estimation | Realistic |
| EVM | Tx/Rx 8/4% |