3GPP TSG RAN WG1 Meeting #83	R1-156447

Anaheim, USA, November 15-22, 2015

Agenda Item:	6.2.1.7

Source:	Huawei, HiSilicon

Title:	New DCI format to support efficient RAR transmission for MTC UEs

Document for:	Discussion and decision

# Introduction

In RAN1 #82bis meeting, the following agreement was concluded for RAR transmission [1]:

RAN1#82bis agreement:

- Confirm working assumption:

  - For RAR for Rel-13 low complexity UEs and UEs operating coverage enhancement, M-PDCCH-scheduled PDSCH carrying the message(s)

- For PDSCH carrying MAC RAR (Msg2)

  - The narrowband is indicated in the associated DCI

In this contribution, we give our considerations on new DCI formats to support efficient RAR transmission for MTC UEs.

# New DCI format for RAR transmission

In current specification, DCI format 1A and format 1C can be used for scheduling common messages (SIB, RAR and Paging). Therefore, DCI format 1A and format 1C can be regarded as the starting point for designing the new DCI format for RAR transmission.

## New DCI format based on DCI format 1A

### Fields needed based on DCI format 1A

The first column of table below lists the current field of DCI format 1A, and the second column of the table provides the usage remark about some fields when DCI format 1A is used for scheduling common message. The third column gives considerations on the necessary fields of new DCI format for scheduling RAR, and the detailed analysis can refer to our previous contribution [2].

Table 1: New DCI format based on DCI format 1A

|  | The remark in current specification | New DCI format for RAR |
| --- | --- | --- |
| Carrier indicator |  | Not needed |
| Flag for format0/format1A differentiation |  | FFS |
| Localized/Distributed VRB allocation flag |  | Not needed |
| Resource block allocation |  | New resource allocation |
| Modulation and coding scheme |  | Need or replaced by TBS indication field |
| HARQ process number | Reserved | Not needed |
| New data indicator | Reserved for | Not needed |
| Redundancy version |  | Not needed |
| TPC command for PUCCH | The most significant bit of the TPC command is reserved The least significant bit of the TPC command indicates column of the TBS table | Not needed |
| Downlink Assignment Index | Reserved | Not needed |
| SRS request | DCI formats scheduling PDSCH which are mapped onto the UE specific search space given by the C-RNTI | Not needed |
| HARQ-ACK resource offset | DCI format for EPDCCH | Not needed |

As seen from table 1, only the following three fields in current DCI format 1A may be needed when scheduling the RAR transmission for MTC UEs.

The flag for format0/format1A differentiation: This field may be needed if allowing UL grant transmitted within eCSS. Considering the limited eCSS capacity within one narrowband, and in order to reduce the M-PDCCH detection complexity, it can assume Rel-13 low complexity UEs and UEs operating coverage enhancement don’t decode UL grant when decoding the M-PDCCH with RA-RNTI.

Resource block allocation: For resource allocation for RAR transmission, the DCI can firstly indicate the narrowband location, and then indicates the specific PRBs within the narrowband.

For narrowband indication, the number of bits can be dependent on the number of available narrowbands  configured by eNB. Considering maximum 16 narrowbands (of 6PRB) within 20MHz system, the maximum bit number of narrowband indication should be 4.

For the resource block allocation within narrowband, as  supported for the current format 1C, =2 can be assumed to consider the paging transmission performance and reduce DCI overhead. Thus, 3 bits are needed for the resource block allocation.

Modulation and coding scheme: For MCS indication, if the current TBS table for DCI format 1C is reused with the restriction of maximum 1000 bits, there would have 24 possible TBSs. Thus, 5 bits would be needed to indicate the MCS.

### New Fields needed for RAR transmission

Considering the RAR transmission for Rel-13 low complexity UEs and coverage enhancement UEs, some new fields may be introduced in the new DCI format.

Repetition number of M-PDCCH transmission: For the indication of M-PDCCH repetition number, it was already agreed “the repetition R of the MPDCCH is indicated by 2-bit in the DCI” [1].

Repetition number of RAR transmission: To provide the flexibility of RAR transmission, the DCI can further adjust the repetition number of RAR transmission on top of the basic repetition number of RAR transmission, which is determined according to the repetition level of PRACH.

Preamble indication: According to the current mechanism, the RAPID is transmitted on PDSCH. As shown in the figure below, even if a UE successfully decodes control channel, it still needs to decode PDSCH to find whether the RAPID included in PDSCH matched with its transmitted preamble. If there is no matched RAPID, it means the UE wastes the detection on control channel and PDSCH.

Figure 1: Illustration of current RAR transmission

Considering coverage enhancement scenario, if RAR is scheduled by M-PDCCH, both M-PDCCH and PDSCH (carrying RAR) can require massive repetitions. For a coverage enhanced MTC UE, it successfully decodes M-PDCCH and PDSCH with great difficulty, but finds the RAPID is not matched. Thus, the power consumption of this UE is significantly wasted.

As shown in the figure below, a new method is to indicate RAPID information in DCI. Since maximum 1000 bits are allowed for the PDSCH carrying RAR, at most 17 MAC RARs can be included within one PDSCH (assuming 56 bits for one MAC RAR). Thus, it is reasonable to assume 16 available PRACH sequences within one PRACH resource set. Therefore, the DCI can include a 16 bits field of preamble indication (in bitmap manner) to indicate which preamble sequence is responded by an eNB. If there is no matched preamble indication in DCI, an MTC UE cannot decode PDSCH to reduce power consumption. Correspondingly, the MAC PDU does not include the RAPID of each MAC RAR, which will further reduce the size of MAC PDU and save UE’s power consumption on PDSCH detection.

Figure 2: Illustration of new RAR transmission

## Based on DCI format 1C

DCI format 1C is used for very compact scheduling of one PDSCH codeword. The following three fields are included in the current format 1C for common message transmission:

- Gap indication

- Resource block allocation

- Modulation and coding scheme

In current specification, there is no bit for gap indication for. Since Rel-13 low complexity UE and UE operating CE can only receive RAR within 6PRB, the gap indication is not needed.

The usage of resource block allocation and MCS indication can be the same as analyzed in above section.

Similarly, new fields, like repetition number of M-PDCCH transmission, repetition number of RAR transmission and preamble indication, should also be included in the new DCI format based on format 1C.

## Fields for RAR transmission

After analyzing above what needs to be retained, removed, or added to each DCI format, overall, the difference between new DCI format based on DCI format 1A and one based on DCI format 1C is that the new DCI format based on format 1A has an additional field of DCI format 0/new DCI format differentiation. To make a common design, it is preferred DCI format 0/new DCI format differentiation is not needed in the new DCI format.

Proposal 1: New DCI format for scheduling RAR should include fields of

- Resource allocation

- MCS indication

- Repetition number of M-PDCCH transmission

- Repetition number of RAR transmission

- Preamble indication

The following table summarized the fields of DCI format for RAR scheduling, and an example for the new DCI format is given in the right column.

Table 2: Payload size of new DCI format for RAR scheduling

| Field |  | Number of bits |
| --- | --- | --- |
| Resource allocation | Narrowband indication |  |
|  | Resource allocation within narrowband | 3 |
| MCS indication |  | 5 |
| Repetition number of M-PDCCH transmission |  | 2 |
| Repetition number of RAR transmission |  | 2 |
| Preamble indication |  | 16 |
| Total |  | 28~32 |

Proposal 2: New DCI format for scheduling RAR has the payload size of about 30 bits, depending on system bandwidth.

# Conclusions

In this contribution, the considerations on new DCI formats to support efficient RAR detection for MTC UEs are provided, and following proposals are given:

Proposal 1: New DCI format for scheduling RAR should include fields of

- Resource allocation

- MCS indication

- Repetition number of M-PDCCH transmission

- Repetition number of RAR transmission

- Preamble indication

Proposal 2: New DCI format for scheduling RAR has the payload size of about 30 bits, depending on system bandwidth.

# References

3GPP RAN1#82bis meeting, Chairman notes, Malmö, Sweden, 5th – 9th October, 2015.

R1-153755, “New DCI format to support efficient RAR detection for MTC UEs”, Huawei, HiSilicon, Beijing, China, August 24-28, 2015.