3GPP TSG RAN WG1 Meeting #83		R1-156528

Anaheim, USA, 16th – 20th November 2015

Source:	Intel Corporation

Title:	On synchronization signal design and performance for NB-IoT

Agenda item:	6.2.6.2.1

Document for:	Discussion/Decision

# Introduction

At the RAN #69 meeting, a new work item (WI) on specification support for Narrowband Internet of Things (NB-IoT) was approved [1] with the objective of supporting low complexity devices that support 180 kHz UE RF bandwidth. It was also agreed to support the following three modes of operation [1]:

- Mode 1: NB-IoT as a stand-alone deployment, e.g., by reusing one or more GSM carriers;

- Mode 2: NB-IoT deployed in the guard-band of an LTE carrier; and

- Mode 3: NB-IoT deployed in-band within a regular LTE deployment.

Further, it was agreed as one of the objectives of the WI, to support a single synchronization signal design for the three different modes of operation, including techniques to handle overlap with legacy LTE signals.

In this contribution, considering the NB-IoT solution described in [2], we provide performance evaluations for the cell-search procedure described in [2] and [4] for standalone and inband operation modes. We also propose a new NB-SSS design and provide performance results for it. Note that the complexity analysis is provided in [3].

# Cell Search

Cell search is used in LTE to acquire OFDM symbol time, frame and frequency synchronization at the user equipment, while also allowing the user to detect the Cell ID. In LTE, 504 unique cell identities are supported, and these are further divided into 168 cell identity groups, with three cell identities within each group. The primary synchronization signal (PSS) is used to acquire OFDM symbol time and frequency synchronization, while also providing one of the three cell identity groups. Secondary Synchronization Signal (SSS) is used to detect the frame boundary as well as 1 out of the 168 cell IDs, which in combination with the cell identity group from PSS gives the Cell ID of the base station.  Cell search in NB-IoT can follow the same design principles as in LTE, but with certain modifications to reduce device complexity and enhance cell search capabilities. The prefix “NB” is used to differentiate NB-IoT physical channels from LTE physical channels. Synchronization signal for NB-IoT are:

- NB-PSS: Only one NB-PSS is used to derive the Cell ID. The PSS can span over multiple symbols and is used to detect symbol timing and correcting the frequency offset.

- NB-SSS: The NB-SSS is used to achieve frame synchronization and detect one out of the N=504 cell Ids.

## Primary Synchronization Signal Design

In LTE, three Primary Synchronization Signal (PSS) are specified, which provide symbol timing acquisition as well as cell ID detection corresponding to three different cell identity groups. In addition, the PSS helps coarse estimation of the frequency offset. In NB-PSS, only one PSS is defined with the aim of reducing receiver processing complexity.  The NB-PSS sequence is generated in time and differentially encoded before passing through an FFT, subcarrier mapping and IFFT operation. The design of the NB-PSS is based on the description for NB-PSS design in [2]. More details can be found in [4].

## Secondary Synchronization Signal Design

The Secondary Synchronization Sequence (NB-SSS) we discuss here is slightly different from the one mentioned in [4]. NB-SSS is generated in the frequency domain, occupies 12 subcarriers and is spread across 6 OFDM symbols. The NB-SSS is composed of a length NZC= 71 Zadoff-Chu (ZC) sequence cyclically extended to occupy 72 resources. This 71 length ZC sequence can be generated using different roots and time domain cyclic shifts to provide support for 504 unique cell identities. We avoid zero-padding like in [4] to have better CM/PAPR properties. To support N=504  cell IDs, we propose the use of different root indices and scrambling codes. This NB-SSS sequence is placed at d different locations within a 80 ms NB-frame, where the density d is decided based on in-band/standalone/guard-band operation mode. For example, Fig. 1 (and Fig. 2) indicates the NB-PSS and NB-SSS placement for d = 4 (and d = 8 respectively) repetitions within a T = 80 ms NB-frame. To differentiate between these different NB-SSS locations, d different cyclic time shifts are applied to the frequency domain ZC sequence for frame synchronization information.

Figure 1. Time domain location of NB-PSS and NB-SSS transmission for NB-IoT (d = 4: Standalone mode)

Figure 2. Time domain location of NB-PSS and NB-SSS transmission for NB-IoT (d = 8: In-band/Guard-band mode)

We next provide the details of the sequence design for the NB-SSS sequence supporting 504 cell IDs, p=0,1,…,503. The following are the salient features of our design:

- We use a combination of the root of the ZC sequence and a scrambling code to represent the cell ID of a cell. Specifically, the root index, say mp and scrambling sequence index, say kp uniquely represent a cell ID p, where

mp=1+modp,71 and

kp= p71.

Note that mp takes the values from {1,…,71}, while kp takes on the values {0, 1, 2, …, 7}

- To represent the location at ‘d’ different radio frame locations, we use time-domain cyclic shifts lq, where q =0, 1, 2, …, dmax-1.

The transmitter is shown in Fig. 3.

Figure 3. Transmitter for NB-SSS generation

Thus, the NB-SSS sequence is given by sp,q(n)=apncqn, where the subscript p={0,1,…,N-1} denotes the cell ID and the subscript q={0,1,…,d-1} determines the location of the NB-SSS. The time domain cyclic shifts are defined as

cqn=e-j2πlqndmax,      lq={0, 1, 2, …, dmax-1}  ,

where dmax is the maximum number of SSS defined per NB-frame.

We have

apn=e-jπmpnn+1NZC bkp(n),                   n=0,1,…,NZC-1,

where  bkpn is the scrambling sequence. The scrambling sequence can be a M-sequence or a PN sequence with the kp different sequences chosen to have good autocorrelation and cross-correlation properties.

The major differences between the NB-SSS design in [4] and here are as follows:

- We use a longer ZC sequence instead of zero padding to have better CM/PAPR properties.

- We use a combination of root sequence index mp and scrambling sequence index kp to indicate the Cell ID and a time-domain cyclic shift lq to indicate the location within a NB-frame.

# Performance evaluation

## Standalone

The simulation assumptions are aligned with the assumptions agreed in [4] and tabulated in Table 1. The results are provided for standalone mode (as shown in Fig. 1) with 164 dB MCL (-4.6 dB). A transmit filter consisting of 19 taps is used in order that the NB-IoT signal fulfils the GSM PSD mask. This is essential to reduce the leakage outside the transmission bandwidth, thereby causing minimal interference to other systems, e.g., GSM.

Table 1: Simulation Parameters for Standalone Operation

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 900 MHz |
| Channel Model | TU 1 Hz |
| Subcarrier Spacing | 15 kHz |
| SNR | -4.6 dB |
| Sampling Frequency (Fs) | 1.92 MHz (A/D); 240 kHz (PSS/SSS detection) |
| Cyclic Prefix | 10 samples for the 1st and 7th OFDM symbol within a subframe, 9 samples for the rest |
| Timing offset | Uniformly distributed between in the interval of [0,80) ms in steps of size 1/Fs |
| Antenna Configuration | 1 Tx, 1 Rx |
| Maximum 80 ms blocks used for detection | 40 |
| Frequency Offset | Randomly generated as one of the values in the set of {-18 kHz, 18 kHz}, which corresponds to a frequency error of 20 ppm |
| Number of realizations | 1000 |

We first provide the result for PSS evaluation. The receiver starts with no timing reference and a frequency offset of |18| kHz is assumed. The incoming 20 ms window signal is differentially decoded before correlating with a stored sequence to give the timing information. Because of the low SNR evaluations, we accumulated the NB-PSS correlation results over multiple 20 ms windows. The result is shown in Fig. 4.

Figure 4: CDF of the time required (in ms) for NB-PSS detection

The observed residual timing estimation error is ~+/- 2.1 us.  After timing acquisition, frequency estimation is done using accumulation over multiple 20 ms window for noise averaging. The CFO is observed to be +-200 Hz.

Figure 5: CDF of the absolute value of CFO estimation error (in Hz)

Finally, after getting the timing estimate and frequency offset estimate, the cell ID estimation and location within the 40 ms NB-frame is performed. The results are shown in Fig. 6. The design in [4] is marked as Option 1 and the design presented here is marked as Option 2. As can be seen, ~70% of the UEs correctly detect the cell ID within the 1st NB-SSS received after timing and frequency offset correction has been done.

Figure 6: CDF of the NB-SSS estimation time (in ms)

## In-band

The simulation assumptions are aligned with the assumptions agreed in [4] and tabulated in Table 2. The results are provided only for inband mode (as shown in Fig. 2) with 164 dB MCL. Note that for In-band case, 164 dB MCL corresponds to an SNR of -12.6 dB.

Table 2: Simulation Parameters for In-band Operation

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 900 MHz |
| Channel Model | TU 1 Hz [4] |
| Subcarrier Spacing | 15 kHz |
| SNR | -12.6 dB |
| Sampling Frequency (Fs) | 1.92 MHz (A/D); 240 kHz (PSS/SSS detection) |
| Cyclic Prefix | 10 samples for the 1st and 7th OFDM symbol within a subframe, 9 samples for the rest |
| Timing offset | Uniformly distributed between in the interval of [0,80) ms in steps of size 1/Fs |
| Antenna Configuration | 1 Tx, 1 Rx [4] |
| Maximum 80 ms blocks used for detection | 100 |
| Frequency Offset | Randomly generated as one of the values in the set of {-18 kHz, 18 kHz}, which corresponds to a frequency error of 20 ppm |
| Number of realizations | 1000 |

We first provide the result for PSS evaluation. The receiver starts with no timing reference and a frequency offset of |18| kHz is assumed. The incoming 10 ms window signal is differentially decoded before correlating with a stored sequence to give the timing reference. As in the standalone mode, accumulations over multiple windows are needed to get correct timing estimate. The result is shown in Fig. 7.

Figure 7: CDF of the time (in ms) required for NB-PSS detection

The observed residual timing estimation error is ~+/- 2.1 us.  After timing acquisition, frequency estimation is done using accumulation over multiple 20 ms window for noise averaging. The CFO is observed to be +-200 Hz.

Figure 8: CDF of the absolute value of CFO estimation error (in Hz)

Finally, after getting the timing estimate and frequency offset estimate, the cell ID estimation and location within the 40 ms NB-frame is performed. The results are shown in Fig. 6. The design in [4] is marked as Option 1 and the design presented here is marked as Option 2.

Figure 9: CDF of the NB-SSS estimation time (in ms)

## Total Synchronization times

A comparison of the total synchronization time for initial cell search for a desired percentage of the mobile stations is provided in Table 3 for standalone and inband operation modes. Note that the total synchronization times includes time required for NB-PSS detection, frequency offset detection and NB-SSS (cell ID and frame timing) detection. Both Option 1 and Option 2 give approximately same numbers and thus we provide the synchronization time for only Option 2.

Table 2: Total synchronization time in ms

| % of Users | Standalone | In-band |
| --- | --- | --- |
| 50 | 320 | 2040 |
| 90 | 1240 | 7080 |

# 4. Conclusions

The preceding sections describe and show some performance evaluation results for a synchronization signal design for NB-IoT. We presented performance evaluations for standalone mode and inband modes of operation.

# References

- RP-151621, “New Work Item: NarrowBand IOT (NB-IOT),” Qualcomm, RAN #69, Phoenix, USA, September 2015.

- RP-151397, “NB LTE – Concept Description L1,” Ericsson, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, Nokia, Intel, ZTE, Samsung, LGE, RAN #69, Phoenix, USA, September 2015.

- R1- 156524, “On device complexity for NB-IoT,” Intel Corporation, 3GPP RAN1 #83, Anaheim, USA, Oct

- R1-156009, “Narrowband LTE – Synchronization Channel Design and Performance,” Ericsson, 3GPP RAN1 #82bis, Malmö, Sweden, October 2015.