3GPP TSG RAN WG1 Meeting #83                                                                   R1-157185

Anaheim, USA, 15th - 22th November 2015

Source:	NTT DOCOMO

Title:	Performance Evaluation of 2D Codebooks for Elevation Beamforming and FD-MIMO

Agenda Item:	6.2.4.3.1

Document for: 	Discussion and Decision

# Introduction

At the RAN1 #82bis meeting [1], a general framework for the 2D codebook and the rank-1 codebook design was discussed and agreed in [2]. Subsequent discussion on rank-2 to rank-8 codebook was carried out in the email discussion of [3]. Proposals were submitted in the email discussion and summarized by rapporteur in [4]. In this contribution, we provide performance evaluation and comparison of the proposed codebooks.

# 2D Codebook Proposals

After the RAN1 #82bis meeting, an email discussion was made to collect 2D codebook proposals for rank>=2 from companies. In [4], the rapporteur has briefly summarized all proposals, which are copied below in Table 1. It is noticed that in proposal 1, there is an alternative proposal for config 3, based on another beam subset selection pattern, which we denote by proposal 1-Alt.

Table 1: Summary of Codebook Proposals for High Ranks

| No. | Proposal | Codebook availability |  |  |
| --- | --- | --- | --- | --- |
|  |  | Rank 2 | Rank 3-4 | Rank 5-8 |
| 1 | Samsung/Ericsson/NTT DOCOMO/CATT | x | x | x |
| 1-Alt | Samsung/Ericsson/NTT DOCOMO/CATT alternative beam subset selection pattern for Config 3 | x | x |  |
| 2 | Alcatel-Lucent/ASB | x | x | x |
| 3 | Huawei/HiSilicon | x | x | x |
| 4 | CMCC | x | x | x |
| 5 | Intel | x | x |  |
| 6 | LGE | x | x | x |
| 7 | Qualcomm | x | x |  |
| 8 | ZTE | x | x |  |

In our accompanying contribution [5], we have analyzed pros/cons of each proposal, mainly from the implementation feasibility point of view. In this proposal, we provide performance evaluation results which can be used in combination with our analysis in [5] to compare proposals.

## Performance Comparison of Config 3 Proposals of Rank-2 Codebook

For rank-2 codebook design, the proposals are quite aligned for Config 1 and Config 4, whereas proposals are mainly different in Config 3 design. Then we mainly focus on the performance evaluation for Config 3.

Key evaluation parameter values are summarized in Table A. Since it is motivated to verify the high rank codebook design, low and middle traffic loads are considered. As the over-sampling factor of (O1, O2) = (8, 4) is mostly used, we mainly focus on the performance evaluation of these values. We simulate the CSI-RS ports of (N1, N2) = (4, 2), (2, 4), (3, 2) and (2, 3) and the results are summarized in Table 1, 2, 3 and 4. In those tables, we also show the beam group size and master codebook size information in order to briefly present our analysis made in [5].

Table 2: Rank-2 Config 3 codebook performance, (M,N,P,Q) = (8, 4, 2, 16), (N1, N2) = (4, 2)

Table 3: Rank-2 Config 3 codebook performance, (M,N,P,Q) = (8, 3, 2, 12), (N1, N2) = (3, 2)

Table 4: Rank-2 Config 3 codebook performance, (M,N,P,Q) = (8, 2, 2, 16), (N1, N2) = (2, 4)

Table 5: Rank-2 Config 3 codebook performance, (M,N,P,Q) = (9, 2, 2, 12), (N1, N2) = (2, 3)

From the system level evaluation results, and combined with the analysis made in [5]. We observe that proposal 1-Alt provides a best tradeoff between implementation complexity and system performance for the rank-2 config 3 codebook design.

Proposal: For rank-2 codebook, use the proposal in R1-156381 with the alternative design for Config 3, i.e., the rank-2 master codebook is given as follows.

Table 6: Master codebook table for rank-2.

|  | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| Precoder |  |  |  |  |
|  | 4 | 5 | 6 | 7 |
| i1,1,i1,2 |  |  |  |  |
|  | 8 | 9 | 10 | 11 |
| i1,1,i1,2 |  |  |  |  |
|  | 12 | 13 | 14 | 15 |
| i1,1,i1,2 |  |  |  |  |
|  | 16 | 17 | 18 | 19 |
| i1,1,i1,2 |  |  |  |  |
|  | 20 | 21 | 22 | 23 |
| i1,1,i1,2 |  |  |  |  |
|  | 24 | 25 | 26 | 27 |
| i1,1,i1,2 |  |  |  |  |
|  | 28 | 29 | 30 | 31 |
| i1,1,i1,2 |  |  |  |  |

Table 7: Codeword subset selection table for rank-2.

| Config | Selected indices | (s1,s2) |
| --- | --- | --- |
| 1 | 0-1 | (1,1) |
| 2 | 0-3, 8-9, 16-19, 22-23, 28-31 | (2,2) |
| 3 | 4-7, 16-27 | (2,2) |
| 4 | 0-15 | (2,2) |

# Summary

In this contribution, we made system level evaluation of the codebook proposals. Based on our investigation, the following proposal can be made.

Proposal: For rank-2 codebook, use the proposal in R1-156381 with the alternative design for Config 3.

# References

- 3GPP, “Draft Report of 3GPP TSG RAN WG1 #82bis v0.2.0.”

- 3GPP, R1-156217, Samsung, “WF on class A and class B CSI reporting for Rel.13 EB/FD-MIMO,” Oct. 2015.

- 3GPP, [82b-13], “Codebook proposals for EB/FD-MIMO - higher ranks,” Oct. 2015.

- 3GPP, Samsung, “Finalizing Rel.13 Rank>1 Codebook Design,” Nov. 2015.

- 3GPP, R1-157184, NTT DOCOMO, “2D Codebook Design for Elevation Beamforming and FD-MIMO,” Nov. 2015.

# Appendix

Table A: Evaluation assumptions

| Parameter | Values |
| --- | --- |
| Scenario / channel model | 3D-UMi (ISD: 200 m) |
| Carrier frequency | 2 GHz |
| System bandwidth | 10 MHz (50 RBs) |
| eNB antenna configurations | (dH, dV) = (0.5 , 0.8 ), etilt = 100 deg. |
| Total BS Tx power | 41 dBm |
| UE antenna configurations | 2 X-pol (0/90 deg.) |
| UE speed | 3 km/h |
| Indoor UE ratio | 80 % |
| MIMO scheme | SU -MIMO |
| UE receiver | Non-ideal channel estimation and interference modeling, detailed guidelines according to Rel. 12 [71-12] assumptions |
|  | LMMSE-IRC receiver, detailed guidelines according to Rel. 12 [71-12] assumptions |
| CSI feedback scheme | Subband PMI and CQI |
| CSI-RS transmission interval / CSI feedback interval | 5 ms for RI, PMI and CQI, 200 ms for beam selection |
| Traffic model | FTP Model 1 with packet size 0.5 Mbytes (low: ~20 % RU, medium: ~50 % RU, high: ~70 % RU) |
| Scheduler | Proportional fairness |
| Control delay | 6 ms |
| HARQ | Chase combining with 8 ms RTD |