3GPP TSG RAN1 Meeting #83							R1-157247

Anaheim, USA, 15th – 22nd November, 2015

Source: 	Nokia Networks

Title:	NB IoT – Battery lifetime evaluation in standalone operation

Agenda Item:	6.2.6.1.1

Document for:	Discussion

# Introduction

At RAN#69 a new work item on narrowband IoT was agreed. In this contribution an estimate of the battery lifetime for Narrowband IoT devices is presented for the NB IoT system as described in [2] following the methodology given in in TR45.820 [1] where the connection to the core network is assumed to be Gb based, since that is what has been agreed to be assumed in the GERAN study item. The evaluation is done in the same way as for the earlier standalone case [3] and in particular the power model remains the same.

# Battery lifetime evaluation

## Assumptions

The battery evaluation for NB-LTE follows the methodology described in [1] and used in [3].  The assumed transactions during an uplink reporting event are shown in Figure 1. Note that standalone deployment and a Gb based architecture is assumed for the battery evaluation, since that is what has been agreed to be assumed in this GERAN study item.

Figure 1: Message exchange during an uplink reporting event (no retransmission included in the figure)

As illustrated in Figure 1 there are four different operating states (Tx, Rx, Idle, Standby), each with different power requirement.  The details of these states are presented in Table 2. A battery of 5Wh is assumed per [1].

The power consumption per state used the assumption in Table 1.

Table 1 Current consumption assumptions.

|  | power [mW] |
| --- | --- |
| Battery power during Tx (assuming 44% PA efficiency) | 543 |
| Battery power for Rx | 90 |
| Battery power when Idle but not in PSS | 2,4 |
| Battery power in Power Save State (PSS) | 0,015 |

The physical layer design follows the LTE based NB IoT design as documents in [2]

Table 1 shows the different activities during an uplink reporting event and the corresponding terminal state. The following assumptions have been made:

- For network synchronization we assume initial synchronization with no interferers and PSS/SSS. The value in the Table 1 is the average synchronization time.

- The timing estimates are based on the results in [4].

- The extra wait time comprises the IP Ack waiting time (1000 ms), the ready timer (20000 ms), and the various scheduling delays (1000 ms). The total extra wait time is therefore 22000 ms.

## UE activity state and timing

Latency evaluation at 164 dB MCL for exception report (20 bytes application report, 65 bytes upper layer protocol header, and 15 bytes SNDCP/LLC/RLC/MAC/CRC overhead is assumed) transmission is shown in Table 2.

Table 2. Latency evaluation for standalone operation (164dB MCL).

| Activity | State | ms |
| --- | --- | --- |
| Synchronization | RX | 215 |
| MIB acquisition | Rx | 64 |
|  | Idle | 576 |
| PRACH | Tx | 160 |
|  | Idle | 640 |
| DCI + RAR | Rx | 72 |
| Msg3 | Tx | 340 |
| DCI + Msg4 | Rx | 72 |
| DCI (UL grant) | Rx | 36 |
| Report (50 bytes) | Tx | 1405 |
| Report (200 bytes) | Tx | 4648 |
| HARQ ACK | Rx | 36 |
| DCI | Rx | 36 |
| IP Ack | Rx | 200 |
| HARQ ACK | Tx | 288 |
| PDCCH monitoring | Rx | 1440 |
| Extra wait time | Idle | 22000 |

Table 2 shows the UE activity state and timing for 164dB MCL for the standalone operation.

## Battery lifetime estimation

The estimated lifetime in years are presented for two different packet sizes, two reporting intervals and at different coverage.

Table 3: Estimated battery lifetime in years

| Packet size, reporting interval | 164 dB |
| --- | --- |
| 50 bytes, 2 hours | 2,6 |
| 200 bytes, 2 hours | 1,2 |
| 50 bytes, 1 day | 18,0 |
| 200 bytes, 1 day | 11,0 |

# Conclusions

The energy consumption evaluation shows that the 10 year battery life objective can be met or exceeded for “once per day” traffic scenarios in standalone operation.

# References

TR 45.820 v13.0.0, “Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things”

R1-156010, “NB-LTE – General L1 concept description”, Ericsson

R1-156006, “NB LTE – Battery lifetime evaluation in stand-alone operation”, Ericsson, ZTE, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, Nokia Networks, Intel, Samsung, LGE

R1-157243, “Latency Evaluation for Standalone Operation”, Nokia Networks