3GPP TSG RAN WG1 Meeting #83                                                                                 R1-15xxxx

Anaheim, USA, 15th – 22nd November 2015

Source: 	Ericsson

Title:	Introduction of new UE category

Agenda Item:	6.2.2.2

Document for:	Discussion and Decision

# Introduction

Good progress has been achieved for the Rel-13 WI: LTE carrier aggregation enhancement beyond 5 carriers in the past few RAN working group meetings. However, up to now there is still no discussion on how to extend the UE capability to support up to 32 CCs. In this contribution, we discuss some aspects related to the introduction of new UE category to support up to 32 component carriers.

# Discussion

When CA was first introduced in Rel-10, a new UE category with extended capabilities was defined to support up to 5 CCs. Similarly in Rel-13 with the extended support of up to 32 carriers, there is a need to introduce a new UE category targeting the support of up to 32 CCs. It should be noted that although there seems still a long way for RAN4 to identify the possible bands and band combinations with 32 CCs, it is necessity for RAN1 to define a high-end category to reflect the highest potential of LTE Rel-13.

A UE Category defines a combined uplink and downlink capability. The parameters set by the UE Category are defined in subclause 4.2 in [1], where the uplink and downlink physical layer parameters are defined. When defining the new high-end UE category, one needs to consider the other advanced features such as MIMO and higher order modulation. Based on the assumption, this new UE category will support 25Gbps in downlink and 9.5Gbps in uplink.

For the DL UE capability, the main physical layer parameters are the “Maximum number of DL-SCH transport block bits received within a TTI” and the “Total number of soft channel bits”. With the support of up to 32 CCs, these capabilities will be affected. For a UE supporting maximum 8 spatial multiplexing layers and 256QAM, the maximum number of bits of a DL-SCH transport block received within a TTI can be expected to be 25065984.  With the same assumption, the total number of soft channel bits is expected to be 303562752.

For the UL UE capability, the main physical layer parameters are the “Maximum number of UL-SCH transport block bits transmitted within a TTI” and the “Maximum number of bits of an UL-SCH transport block transmitted within a TTI”. With the support of up to 32 CCs, these capabilities will be recalculated. For a UE supporting maximum 4 spatial multiplexing layers and 64QAM, the maximum number of bits of a UL-SCH transport block received within a TTI can be expected to be 149776. The Maximum number of bits of an UL-SCH transport block transmitted within a TTI is expected to be 9585664.

Based on above discussion, we propose to introduce a new UE category with the capabilities as shown in the following tables.

# Table 4.1A-1: Downlink physical layer parameter values set by the field ue-CategoryDL

| UE DL Category | Maximum number of DL-SCH transport block bits received within a TTI (Note 1) | Maximum number of bits of a DL-SCH transport block received within a TTI | Total number of soft channel bits | Maximum number of supported layers for spatial multiplexing in DL |
| --- | --- | --- | --- | --- |
| DL Category 0 (Note 2) | 1000 | 1000 | 25344 | 1 |
| DL Category 6 | 301504 | 149776 (4 layers, 64QAM) 75376 (2 layers, 64QAM) | 3654144 | 2 or 4 |
| DL Category 7 | 301504 | 149776 (4 layers, 64QAM) 75376 (2 layers, 64QAM) | 3654144 | 2 or 4 |
| DL Category 9 | 452256 | 149776 (4 layers, 64QAM) 75376 (2 layers, 64QAM) | 5481216 | 2 or 4 |
| DL Category 10 | 452256 | 149776 (4 layers, 64QAM) 75376 (2 layers, 64QAM) | 5481216 | 2 or 4 |
| DL Category 11 | 603008 | 149776 (4 layers, 64QAM) 195816 (4 layers, 256QAM) 75376 (2 layers, 64QAM) 97896 (2 layers, 256QAM) | 7308288 | 2 or 4 |
| DL Category 12 | 603008 | 149776 (4 layers, 64QAM) 195816 (4 layers, 256QAM) 75376 (2 layers, 64QAM) 97896 (2 layers, 256QAM) | 7308288 | 2 or 4 |
| DL Category 13 | 391632 | 195816 (4 layers, 256QAM) 97896 (2 layers, 256QAM) | 3654144 | 2 or 4 |
| DL Category 14 | 3916560 | 391656 (8 layers, 256QAM) | 47431680 | 8 |
| DL Category 15 | 749856-798800 (Note 3) | 149776 (4 layers, 64QAM) 195816 (4 layers, 256QAM) 75376 (2 layers, 64QAM) 97896 (2 layers, 256QAM) | 9744384 | 2 or 4 |
| DL Category 16 | 978960 -1051360 (Note 3) | 149776 (4 layers, 64QAM) 195816 (4 layers, 256QAM) 75376 (2 layers, 64QAM) 97896 (2 layers, 256QAM) | 12789504 | 2 or 4 |
| DL Category 17 | 25065984 | 391656 (8 layers, 256QAM) | 303562752 | 8 |
| NOTE 1: In carrier aggregation operation, the DL-SCH processing capability can be shared by the UE with that of MCH received from a serving cell. If the total eNB scheduling for DL-SCH and an MCH in one serving cell at a given TTI is larger than the defined processing capability, the prioritization between DL-SCH and MCH is left up to UE implementation. NOTE 2: Within one TTI, a UE indicating category 0 shall be able to receive up to 1000 bits for a transport block associated with C-RNTI/Semi-Persistent Scheduling C-RNTI/P-RNTI/SI-RNTI/RA-RNTI and up to 2216 bits for another transport block associated with P-RNTI/SI-RNTI/RA-RNTI. NOTE 3: The UE indicating category x shall reach the value within the defined range indicated by “Maximum number of DL-SCH transport block bits received within a TTI” of category x. The UE shall determine the required value within the defined range indicated by “Maximum number of DL-SCH transport block bits received within a TTI” of the corresponding category, based on its capabilities (i.e. CA band combination, MIMO, Modulation scheme). |  |  |  |  |

# Table 4.1A-2: Uplink physical layer parameter values set by the field ue-CategoryUL

| UE UL Category | Maximum number of UL-SCH transport block bits transmitted within a TTI | Maximum number of bits of an UL-SCH transport block transmitted within a TTI | Support for 64QAM in UL |
| --- | --- | --- | --- |
| UL Category 0 | 1000 | 1000 | No |
| UL Category 3 | 51024 | 51024 | No |
| UL Category 5 | 75376 | 75376 | Yes |
| UL Category 7 | 102048 | 51024 | No |
| UL Category 8 | 1497760 | 149776 | Yes |
| UL Category 13 | 150752 | 75376 | Yes |
| UL Category 14 | 9585664 | 149776 | Yes |

The combination for the new UE UL and DL Categories is given in the following table. The table also defines which UE Categories a UE shall indicate in addition to the combinations for new UE UL and DL Categories.

Table 4.1A-6: supported DL/UL Categories combinations set by the fields ue-CategoryDL and ue-CategoryUL and UE categories to be indicated

| UE DL Category | UE UL Category | UE categories |
| --- | --- | --- |
| DL Category 0 | UL Category 0 | N/A |
| DL Category 6 | UL Category 5 | Category 6, 4 |
| DL Category 7 | UL Category 13 | Category 7, 4 |
| DL Category 9 | UL Category 5 | Category 9, 6, 4 |
| DL Category 10 | UL Category 13 | Category 10, 7, 4 |
| DL Category 11 | UL Category 5 | Category 11, 9, 6, 4 |
| DL Category 12 | UL Category 13 | Category 12, 10, 7, 4 |
| DL Category 13 | UL Category 3 | Category 6, 4 |
| DL Category 13 | UL Category 5 | Category 6, 4 |
| DL Category 13 | UL Category 7 | Category 7, 4 |
| DL Category 13 | UL Category 13 | Category 7, 4 |
| DL Category 14 | UL Category 8 | Category 8, 5 |
| DL Category 15 | UL Category 3 | Category 11, 9, 6, 4 |
| DL Category 15 | UL Category 5 | Category 11, 9, 6, 4 DL Category 11 and UL Category 5 |
| DL Category 15 | UL Category 7 | Category 12, 10, 7, 4 |
| DL Category 15 | UL Category 13 | Category 12, 10, 7, 4 DL Category 12 and UL Category 13 |
| DL Category 16 | UL Category 3 | Category 11, 9, 6, 4 |
| DL Category 16 | UL Category 5 | Category 11, 9, 6, 4 DL Category 11 and UL Category 5 |
| DL Category 16 | UL Category 7 | Category 12, 10, 7, 4 |
| DL Category 16 | UL Category 13 | Category 12, 10, 7, 4 DL Category 12 and UL Category 13 |
| DL Category 17 | UL Category 14 | Category 8, 5 DL Category 14 and UL Category 8 |

# Conclusion

In this contribution we discussed the physical layer parameters for the new UE category with support of 32 carriers. The above discussion is summarized with the following proposal:

Proposal:

- Introduce a new UE category which supports 25Gbps in downlink and 9.5Gbps in uplink.

  - The physical layer parameters and UE category signaling follow the proposal in the contribution.

# References

3GPP TS 36.306 V12.6.0 “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA)”