3GPP TSG RAN1 Meeting #83								R1-157634

Anaheim, CA, USA, 16th– 20th November, 2015

Source: 		Ericsson

Title:	NB-IoT - Battery lifetime comparison for standalone operation

Agenda Item:	6.2.6.1.1

Document for:	Discussion and decision

# Introduction

At RAN#69 a new work item on narrowband IoT and tasked RAN1 to evaluate two numerology options for both UL and DL:

- DL: 15 kHz sub-carrier spacing (with normal or extended CP) and 3.75 kHz sub-carrier spacing.

- UL: FDMA with GMSK modulation and SC-FDMA (including single-tone transmission as a special case of SC-FDMA)

One of the performance objectives is for the IoT device to meet the battery lifetime requirement of 10 years. Even if a possible down selection of DL and UL numerology could be seen as unconnected evaluations, in order to estimate the battery lifetime, a system with both DL and UL must be assumed for the evaluation. For this reason, in this contribution we will compare the performance of a NB-IoT system as described in [3], in the following called NB-LTE, to the NB-CIoT system as described in [1].

First, we update the evaluation of NB-LTE in [2], using the methodology given in TR45.820 [1] and aligning with the assumptions used for evaluating NB-CIoT. Secondly, we compare the battery life time for the tow described systems.

This is an update of [7] updating the result with the new PUSCH results.

# NB-LTE energy consumption evaluation

## Updated Assumptions

This analysis follows the methodology as described in [2] and [7]with the following updates:

- The transmission time for PUSCH have been updated as shown in Table 1.

- Update of cell search times to latest design [6]

- In order to align with the evaluation in section 7.3.6.4 in[1], the effect of BLER has only been included for PUSCH and for other channels no retransmissions are assumed.

- In order to align with the evaluation in section 7.3.6.4 in[1] and [5], the UE only wakes up three times during the 20 s ready timer. During the DRX, the UE is in standby mode.

Table 1 Transmission times and actual MCL for PUSCH

|  | 144 dB |  | 154 dB |  | 164 dB |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | TX time | MCL | TX time | MCL | TX time | MCL |
| Tx (50 bytes) | 24 | 144.4 | 156 | 154.3 | 960 | 164.2 |
| Tx (200 bytes) | 54 | 144.9 | 444 | 154.2 | 2620 | 164.4 |

## Results

The overall reception, transmission and idle times per report at different coverage based on the new assumptions are shown in Table 2.

Table 2 Transmission, Reception and Idle time per report (ms)

|  | 144 dB | 154 dB | 164 dB |
| --- | --- | --- | --- |
| Tx (50 bytes) | 48 | 268 | 1720 |
| Tx (200 bytes) | 81 | 584 | 3546 |
| Rx | 163 | 223 | 804 |
| Idle | 1263 | 1423 | 2085 |

The estimated lifetime in years are presented in Table 3 for two different packet sizes, two reporting intervals and at different coverage.

Table 3 estimated battery lifetime in years

| Packet size, reporting interval | 144 dB | 154 dB | 164 dB |
| --- | --- | --- | --- |
| 50 bytes, 2 hours | 26.9 | 14.8 | 3.7 |
| 200 bytes, 2 hours | 24.1 | 9.1 | 1.9 |
| 50 bytes, 1 day | 36.8 | 33.6 | 21.3 |
| 200 bytes, 1 day | 36.3 | 30.1 | 14.9 |

We see that with the updated assumptions in section 2.1, the estimated battery life for NB-LTE is further improved fulfilling the 10 year target for battery lifetime.

Observations 1:

- For all MCL targets, a 10 year battery life is achievable with a reporting interval of one day for both 50 bytes and 200 bytes application payloads.

- For MCL target 144 dB, a 10 year battery life is achievable with a two hour reporting interval for both 50 bytes and 200 bytes application payloads.

- For a MCL target 154 dB with 200 byte application payload, or 164 dB with 50 or 200 byte application payload, a 10 year battery life is not achievable for a 2 hour reporting interval.

# Comparison of NB-LTE and NB-CIoT

The remainder of this contribution has not been updated from [7].

The estimated battery lifetime for NB-CIoT with integrated PA from [1] is repeated for convenience in Table 4.

Table 4 NB-CIoT estimated battery lifetime in years[1]

| Packet size, reporting interval | 144 dB | 154 dB | 164 dB |
| --- | --- | --- | --- |
| 50 bytes, 2 hours | 22.4 | 11.0 | 2.5 |
| 200 bytes, 2 hours | 18.2 | 5.9 | 1.5 |
| 50 bytes, 1 day | 36.0 | 31.6 | 17.5 |
| 200 bytes, 1 day | 34.9 | 26.2 | 12.8 |

Table 5 shows the increase in battery life time that can be achieved with NB-LTE [7] over NB-CIoT for the studied scenarios.

Table 5 Increase in estimated battery lifetime with NB-LTE

| Packet size, reporting interval | 144 dB | 154 dB | 164 dB |
| --- | --- | --- | --- |
| 50 bytes, 2 hours | 22% | 13% | 21% |
| 200 bytes, 2 hours | 34% | -1% | 19% |
| 50 bytes, 1 day | 2% | 3% | 11% |
| 200 bytes, 1 day | 4% | 0% | 13% |

As we cans see, for all cases except one, NB-LTE has longer estimated battery lifetime compared to NB-CIoT.

Observation 2:

- For all cases but one, where the difference is negligible, NB-LTE will have longer battery lifetime than NB-CIoT

# Conclusions

In this contribution we evaluated battery lifetime of the NB-LTE proposal further and compared it to the battery lifetime of NB-CIoT and make the following observations.

Observations 1:

- For all MCL targets, a 10 year battery life is achievable with a reporting interval of one day for both 50 bytes and 200 bytes application payloads.

- For MCL target 144 dB, a 10 year battery life is achievable with a two hour reporting interval for both 50 bytes and 200 bytes application payloads.

- For a MCL target 154 dB with 200 byte application payload, or 164 dB with 50 or 200 byte application payload, a 10 year battery life is not achievable for a 2 hour reporting interval.

Observation 2:

- For all cases but one, where the difference is negligible, NB-LTE will have longer battery lifetime than NB-CIoT

# References

TR 45.820 v13.0.0, “Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things”

R1-156006, “NB-IoT - Battery lifetime evaluation”, 	Ericsson, ZTE, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, Nokia, Intel, Samsung, LGE

R1-156013, “NB-LTE – General L1 concept description”, Ericsson, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, Nokia, Intel, ZTE, Samsung

R1-156009, “Narrowband LTE – Synchronization Channel Design and Performance”, Ericsson

GP-150137, “NB M2M – Battery life analysis”, Neul

R1-157455, “NB-IoT – Improved Synchronization Channel Design”, Ericsson

R1-157390, “NB-IoT - Battery lifetime comparison for standalone operation”, Ericsson