3GPP TSG RAN WG1 Meeting #84                                                                    R1-160345

St Julian’s, Malta, February 15th - 19th 2016

Source:	CATT

Title:	Discussion on some open issues

Agenda Item:	7.1.4

Document for:	Discussion / Decision

Introduction

In RAN1#83, FD-MIMO WI was successfully concluded and a list of RRC parameters was sent to RAN2 along with an LS and a set of Rel-13 RAN1 specifications have been published too. In this contribution we discuss some of the open issues which need clarifications.

Discussion

| Interference-Measurement-Restriction | new parameter for CSI process | UE specific | Indicate whether measurement restriction is on or off for interference measurement | 1 bit to indicate whether measurement restriction is on or off for interference measurement | N/A | Independently configured per CSI-IM per CSI process per subframe subset per cell. | 1=ON 0=OFF Note: RAN1 will further discuss whether all the CSI-IM in one CSI process should be configured with the same MR. Applicable to TM10 |
| --- | --- | --- | --- | --- | --- | --- | --- |

In the agreed RRC parameters list which was sent to RAN2, there is a following row requiring RAN1 decision.

In our view there is no use case for different CSI-IMs in one CSI process configured with different measurement restriction. We propose that all CSI-IM in one CSI process should be configured with the same MR and inform RAN2 about the decision.

Proposal: all CSI-IM in one CSI process should be configured with the same MR and inform RAN2 about the decision

There are two types of 4Tx codebooks which can be configured with the RRC parameter alternativeCodeBookEnabledFor4TX-r12. In case of class B K>1 and multiple CSI-RS resources are configured with 4 ports CSI-RS, this RRC parameter can be configured per CSI-RS resource. However, in our view since use of 4Tx codebook type highly depends on eNB antenna type, it is logical for this parameter to be configured same for all CSI-RS resources rather than per CSI-RS resource. We propose that the parameter alternativeCodeBookEnabledFor4TX-r12 to be configured per CSI process and inform RAN2 about the decision.

Proposal: alternativeCodeBookEnabledFor4TX-r12 is configured per CSI process and inform RAN2 about the decision.

It was agreed in the last meeting that when CRI and RI are present in the same subframe (PUCCH or PUSCH), CRI and RI are jointly encoded, where the RI bitwidth is determined by the maximum number of antenna ports of all configured CSI-RS resources. The channel coding for joint CRI and RI requires some clarification. Our understanding is that for each CSI-process, the uncoded bits of CRI and RI are concatenated, and the concatenated bits are sent to the channel encoder used for RI. On PUCCH, the same procedure applies to joint encoding of CRI + RI/W1, and joint encoding of CRI + RI/PTI. For PUSCH, if there are multiple CSI-processes or multiple DL cells, the concatenated bits of CRI/RI are concatenated according to CSI-process and cell IDs, as for legacy RI processing.

Proposal: Clarify that

- For joint coding of CRI and RI (or RI/W1 or RI/PTI) for a CSI-process on PUSCH/PUCCH, the uncoded bits of CRI and RI (or RI/W1 or RI/PTI) are concatenated prior to channel coding.

- For PUSCH, joint CRI/RI of different CSI-processes/DL cells are concatenated according to CSI-processes/cell IDs, as for legacy RI.

In Rel-11, a CSI process and its  'RI-reference CSI process' should be configured with the same number of CSI-RS ports to ensure the same RI is available for two processes. For a CSI-process configured with CSI reporting class B with K>1 CSI-RS resources in Rel-13, the number of CSI-RS ports used for RI report in one CSI process should also be the same as that for RI report its 'RI-reference CSI process'. Since a UE configured with CSI process with class B and K>1 would report RI based on a selected CSI-RS resource, the case that a CSI-RS resource for one CSI-process is different from a CSI-RS resource for its 'RI-reference CSI process' should be avoided.

Proposal: The UE is not expected to receive configurations for the CSI process and the 'RI-reference CSI process' that have a different:

- number of CSI-RS antenna ports for any two CSI-RS resources for the two CSI processes, if a UE is configured with higher layer parameter CSI-Reporting-Type, and CSI-Reporting-Type is set to ‘CLASS B’, and the number of configured CSI-RS resources is more than one for any one of the CSI processes

In [2], for class A and class B with K=1, a UE can be configured with up to two CSI-IM resources for a CSI process if the UE is configured with CSI subframe sets by the higher layer parameter for the CSI process. It is aligned with the legacy Rel-12 system. However, according to the agreed RRC parameters list [1], the CSI-IM resource is independently configured per CSI process per cell, which is not consistent with the description in [2]. Therefore, we have the following proposal:

Proposal:

- The CSI-IM-ID-List should be changed as

| CSI-IM-ID-List | new parameter for Rel.13 CSI process | UE specific | Indicate a set of csi-IM-ConfigId-r11 associated with this CSI process. Each CSI-IM-ConfigId is associated with one CSI-RS-ConfigNZPId | Size of the set is the same as NZP-CSI-RS-ID-List with one to one linkage between the IDs in NZP-CSI-RS-ID-List and CSI-IM-List. Each ID is an INTEGER of {1,…, 24} | N/A | Independently configured per CSI process per subframe set per cell. | Applies when CSIReportingType = Class B. and Class A (size=1) Applicable to TM10 |
| --- | --- | --- | --- | --- | --- | --- | --- |

- For Class B with K>1, subframe sets configuration should also be supported.

Conclusions

In this contribution, we discussed some issues which need RAN1 decision and require informing RAN2.

References

  - R1-157694, “LS on EB/FD-MIMO signalling”, RAN1, RAN1#83, Anaheim, USA, 15th - 22th November 2015

  - TS 36.213 13.0.1