3GPP TSG RAN WG1 Meeting #84		                                                      R1-160364

St Julian’s, Malta, 15th - 19th February 2016

Source:	CATT

Title:	                   Discussion on sub-channels in PC5-based V2V

Agenda Item:	7.3.2.2.2

Document for:	Discussion and Decision

Introduction

There are some conclusions related to resource allocation for PC5-based V2V from the last meeting [1] as follows:

  - Scheduling assignment

    - Each data transmission is scheduled by an SA. A UE knows at least time and frequency location of data transmission(s) after decoding the associated SA.

    - FFS the indication is implicit, explicit, or both

    - If SA and the associated data from a single transmitter is transmitted in different subframes:

    - FFS details

    - If SA and the associated data from a single transmitter are transmitted in the same subframe:

    - Q: Is it possible to support the case where data transmission in a subframe occurs without associated SA transmitted in the same subframe?

    - Alt 1: SA and Data are transmitted on separate physical channels (i.e., separated DFT precoding for SA and data):

      - RAN1 assumes that RAN4 will study the proper transmission characteristics (e.g, MPR) to support this.

      - FFS whether SA and data transmissions in the same subframe are always adjacent in the frequency domain.

      - In case of separate channels, study whether SA pool and data pool are orthogonal or can overlap.

    - Alt 2: A single DFT precoding applies to SA and data transmitted in the same subframe.

      - The whole bandwidth is divided into one or multiple sub-channels.

      - The transmission bandwidth of SA/data is fixed to the bandwidth of a single sub-channel.

    - Alt 3: SA and data are TDMed within one subframe.

      - The transmission bandwidth of SA is fixed.

    - Study the number of transmissions of a given TB

    - Study the number of transmissions of a given SA

    - FFS whether a single SA may schedule multiple TBs

    - FFS whether the time/frequency resources of a given SA is independent of the time/frequency resources of the associated data

No matter a single DFT or separated DFT applied, sub-channel solutions can be further discussed to reduce interference as much as possible. In this contribution, some further discussion on sub-channel division is provided.

Discussion and simulation results

Divide frequency domain resources to several sub-channels can effectively avoid interference and improve the system reliability. However, some waste of resources is inevitable if the packet size vary widely due to the fixed capacity. Thus sub-channel division should be further studied.

We compared three options of sub-channel as follows:

Option 1: A sub-channel is consists of 10 continuous PRBs. A small packet(190Byte) will occupies 1 sub-channel and a big packet(300Byte) will occupies 2 sub-channels.

Option 2: A sub-channel is consists of 5 continuous PRBs. A small packet(190Byte) will occupies 2 sub-channels and a big packet(300Byte) will occupies 4 sub-channels.

Option 3: There is no sub-channel. A small packet(190Byte) will occupies 10 continuous PRBs and a big packet(300Byte) will occupies 20 continuous PRBs.

System evaluation assumptions are provided in Appendix Table.

Figure 1 Performance comparison of three division options (freeway 70km/h)

Figure 2 Performance comparison of three division options (freeway 140km/h)

Figure 3 Performance comparison of three division options (Urban 60km/h)

From above simulation results we can demonstrate that the system performance of sub-channel division (option1) is always better than no sub-channel ones(option 3) in both freeway and urban scenes. Therefore, we suggest divide the Data bandwidth into multiple sub-channels.

Proposal: The Data bandwidth should be divided into multiple sub-channels. The number and configuration of sub-channel is FFS.

Conclusion

In this contribution, we have the following proposal:

Proposal: The Data bandwidth should be divided into multiple sub-channels. The number and configuration of sub-channel is FFS.

References

  - 3GPP TSG RAN WG1 Meeting #83bis, RAN1 Chairman’s Notes

  - 3GPP TR 22.885 V0.3.0 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on LTE Support for V2X Services(Release 14)

Appendix

Table 1: simulation assumptions

| Simulation Parameters | Value |
| --- | --- |
| Deployment scenario, Vehicle density/ velocity | Freeway and urban as defined in [2] |
| Carrier frequency | 5.9GHz |
| System bandwidth | 10MHz |
| Tx Power | 23dBm |
| Antenna gain | 3dBi |
| Traffic Model | Data transmission： - 100ms period, and periodic traffic as defined in [2] |
| Synchronization | Ideal |
| Carrier Frequency Offset | 0 |
| Pathloss model | As defined in [2] |
| Shadowing fading | As defined in [2] |
| Resource selection method | Random selection |
| Resource pool | Data pool period: 20ms Data occupy all 50 PRBs |
| SA reception assumption | Ideal |
| Data Tx number | Two times for either small packet and big packet |
| Modulation and coding scheme | Data transmission : - QPSK, Turbo coding, coding rate is 0.50/0.39(respectively for 190Byte and 300Byte, assuming 4 DMRS symbols) |