3GPP TSG RAN WG1 Meeting #84		R1-160719

St Julian’s, Malta, 15th - 19th February 2016

Source:	Panasonic

Title: 	Evaluation results on shortened TTIs

Agenda Item:		7.3.4.1

Document for:		Discussion and Decision

Introduction

Latency reduction study item has been approved in the RAN plenary #69 meeting [1]. The following areas should be studied in RAN1.

TTI shortening and reduced processing times

  - Assess specification impact and study feasibility and performance of TTI lengths between 0.5ms and one OFDM symbol, taking into account impact on reference signals and physical layer control signaling

  - backwards compatibility shall be preserved (thus allowing normal operation of pre-Rel 13 UEs on the same carrier);

In order to evaluate performance of shortened TTI, system level simulation is required. In RAN1 #83 meeting, simulation assumptions for system level simulation were discussed and agreed [2][3]. In this contribution, we show preliminary results of system level evaluation on shortened TTIs.

Evaluation assumptions

We assumed the following assumptions for shortened TTI. And the other evaluation assumptions are listed in Annex 1. It based on [2] and [3].

Table 1 Evaluation assumptions related to shortened TTI

| Parameter | Assumptions |
| --- | --- |
| TTI length | 1, 2, 7, and 14 OFDM symbols. |
| TCP ACK delay for SR, grant and UL transmission | 18 TTIs |
| Overhead of PDCCH/RS | Nothing |
| CRC | 24bit per transport block |
| CSI report period | 10 TTIs |
| CSI report delay | 6 TTIs |

In this evaluation, we applied different traffic load cases, low and high load cases. Resultant resource utilization(RU) was about 20% and 60% for low and high load case, respectively. In addition, we used small and large file sizes (100kbits and 500kBytes) to assess performance for different file sizes.

Evaluation results

We show the evaluation result of user throughput and packet delay on RU=20%, 60% and File size = 100kbits, File size = 500kbyte in Table 2 to Table 5. In the results, mean, 95%, 50% and 5% of CDF of user throughput and packet delay are shown, and gains(percentage) from 14symbol-TTI are also shown. Additionally, the graphs are shown in Annex 2. The gain of latency reduction would come from follows.

- Shorter TCP delay

- More accurate inner link adaptation (MCS selection) thanks to faster CSI report

- More accurate outer loop link adaptation (CQI adjustment) thanks to more/faster HARQ feedback

From the evaluation results, we can see improvement of user throughput and reduction of packet delay by using shortened TTI. It is because transmission time can be reduced by getting rapid increase of transmission data size in TCP protocol thanks to reduced TCP ACK delay.

The gain of File size = 100kbits is larger than it of 500kbyte. In small file size, the throughput is determined mainly TCP feedback loop latency. In large file size, the throughput is determined mainly how fast the data is transmitted over the air as sliding window is adjusted to the throughput of the air. Shortened TTI directly reduces TCP feedback loop latency and shows more gain. In addition, In case of 100kbits, interference is more fluctuating. Then faster CSI repot and quick outer loop link adaptation by shortened TTI would be effective.

The gain of 50% TP UE and 95% TP UE is larger than it of 5% TP UE. It is because that suitable higher MCS is selected for 50% TP UE and 95% TP UE by quick CSI feedback. For 5% TP UE, quick feedback has not so much gain since MCS is low.

In 5% TP UE, 1 symbol TTI is lower TP compared to it of 2 symbols TTI. We think the degradation is caused by CRC overhead. When very shortened TTI is used, data size in one transport block is smaller than it of normal TTI since time domain resources are limited. Therefore, the ratio of such overhead in the transport block is increased. We can say there are same issues for other overhead. Then for very shortened TTI, overhead of RS/PDCCH would impact on throughput. Further study is necessary for the impact of overhead.

The gain of RU20% is larger than it of RU60%. From the table 2 and 3 in RU 20%, we can see throughput gain for wide area from 95% TP UE to 5% TP UE because inter-cell interference is low in this situation. On the other hand, from the table 4 and 5 in RU 60%, gain at 5% TP UE is small compared to other UE. In RU 60%, the interference is dominant in the 5% TP UE. Therefore, 5% TP UEs spend long packet delay with lower MCS, then effect of TCP ACK delay reduction decreases.

RU=20 %

Table 2 File size = 100kbits, RU 20%

| TTI [Symbols] | User TP [Mbps] |  |  |  | Packet Delay [ms] |  |  |  | RU[%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Mean | 95% | 50% | 5% | Mean | 95% | 50% | 5% |  |
| 14 | 0.68 | 0.75 | 0.69 | 0.60 | 147.4 | 175.0 | 144.0 | 128.0 | 19.9 |
| 7 | 0.90 (31.5%) | 0.98 (31.0%) | 0.91 (31.9%) | 0.79 (30.9%) | 112.1 (-23.9%) | 131.5 (-24.9%) | 109.0 (-24.3%) | 99.5 (-22.3%) | 21.6 |
| 2 | 1.15 (68.8%) | 1.24 (66.7%) | 1.18 (70.9%) | 0.98 (62.8%) | 87.7 (-40.5%) | 102.1 (-41.6%) | 84.3 (-41.5%) | 79.6 (-37.8%) | 21.0 |
| 1 | 1.22 (78.9%) | 1.32 (76.5%) | 1.26 (82.8%) | 1.01 (68.4%) | 83.0 (-43.7%) | 96.9 (-44.6%) | 78.9 (-45.2%) | 75.4 (-41.1%) | 20.0 |

Table 3 File size = 500kbyte, RU 20%

| TTI [Symbols] | User TP [Mbps] |  |  |  | Packet Delay [ms] |  |  |  | RU[%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Mean | 95% | 50% | 5% | Mean | 95%) | 50% | 5% |  |
| 14 | 6.46 | 8.67 | 6.81 | 3.08 | 685.6 | 1214.9 | 579.6 | 449.5 | 20.8 |
| 7 | 8.20 (27.1%) | 11.16 (28.7%) | 8.68 (27.4%) | 3.63 (18.0%) | 560.3 (-18.3%) | 1067.0 (-12.2%) | 457.2 (-21.1%) | 347.9 (-22.6%) | 19.6 |
| 2 | 10.02 (55.2%) | 13.83 (59.5%) | 10.84 (59.2%) | 3.88 (26.2%) | 476.1 (-30.6%) | 988.4 (-18.6%) | 370.1 (-36.1%) | 282.2 (-37.2%) | 18.7 |
| 1 | 10.26 (58.8%) | 14.39 (65.9%) | 11.13 (63.5%) | 3.69 (19.9%) | 483.0 (-29.5%) | 1051.8 (-13.4%) | 357.2 (-38.4%) | 269.2 (-40.1%) | 19.9 |

RU=60 %

Table 4 File size = 100kbits, RU 60%

| TTI [Symbols] | User TP [Mbps] |  |  |  | Packet Delay [ms] |  |  |  |  | RU[%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Mean | 95% | 50% | 5% | Mean | 95% | 50% | 5% |  |  |
| 14 | 0.65 | 0.73 | 0.67 | 0.52 | 153.8 | 192.0 | 148.0 | 130.0 | 55.7 |  |
| 7 | 0.84 (28.1%) | 0.97 (31.7%) | 0.87 (29.8%) | 0.58 (11.7%) | 120.2 (-21.8%) | 159.5 (-16.9%) | 113.0 (-23.6%) | 101.5 (-21.9%) | 66.6 |  |
| 2 | 1.02 (56.1%) | 1.22 (66.0%) | 1.08 (61.1%) | 0.61 (17.3%) | 98.8 (-35.8%) | 140.6 (-26.8%) | 90.0 (-39.2%) | 81.1 (-37.6%) | 73.7 |  |
| 1 | 1.05 (60.9%) | 1.28 (74.9%) | 1.12 (67.5%) | 0.57 (9.2%) | 95.8 (-37.7%) | 141.1 (-26.5%) | 85.9 (-42.0%) | 77.1 (-40.7%) | 71.8 |  |

Table 5 File size = 500kbyte, RU 60%

| TTI [Symbols] | User TP [Mbps] |  |  |  | Packet Delay [ms] |  |  |  | RU[%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Mean | 95% | 50% | 5% | Mean | 95% | 50% | 5% |  |
| 14 | 4.90 | 7.63 | 5.05 | 1.82 | 960.0 | 2076.4 | 754.84 | 498.8 | 61.7 |
| 7 | 6.09 (24.2%) | 9.72 (27.3%) | 6.37 (26.3%) | 2.10 (15.7%) | 803.0 (-16.4%) | 1797.7 (-13.4%) | 610.41 (-19.1%) | 389.3 (-21.9%) | 59.7 |
| 2 | 7.13 (45.5%) | 11.75 (53.9%) | 7.37 (46.0%) | 2.19 (20.6%) | 720.0 (-25.0%) | 1675.6 (-19.3%) | 528.73 (-30.0%) | 317.3 (-36.4%) | 59.5 |
| 1 | 7.08 (44.4%) | 12.08 (58.2%) | 7.37 (46.0%) | 1.86 (2.1%) | 750.8 (-21.8%) | 1876.2 (-9.6%) | 526.16 (-30.3%) | 307.6 (-38.3%) | 60.1 |

Conclusion

In this contribution, we showed preliminary results of system level evaluation on shortened TTIs. We observed follows from evaluation results

- Improvement of user throughput and reduction of packet delay by using shortened TTI. It is mainly because of short TCP ACK delay.

- The gain of File size = 100kbits is larger than it of 500kbyte

- The gain of cell 95% TP UE and 50% TP UE is larger than it of cell 5% TP UE

- For very shortened TTI, overhead of RS/PDCCH would impact on throughput. Further study is necessary.

- The gain of RU20% is larger than it of RU60%

- Reference

- RP-150465, “New SI proposal Study on Latency reduction techniques for LTE”, Ericsson, Huawei

- RAN1#83 RAN1 Chairman’s Notes

- R1-154806, “WF on evaluation methodology for latency reduction”, Ericsson, Huawei, HiSilicon, ZTE, CATT, Sharp, Samsung, Intel, Qualcomm, Interdigital, LGE, CMCC, ETRI

- Annex 1

Table 6   Simulation Assumptions

| Parameter | Assumptions |
| --- | --- |
| Layout | 7 Macro eNBs, 3sectors per site |
| System bandwidth per carrier | 10MHz |
| Carrier frequency | 2GHz |
| Inter-site distance | 500m |
| Total BS TX power (Ptotal per carrier) | 46dBm |
| Fast UL Access schemes | Based on TTI length |
| RS and control signaling overhead | no overhead is assumed |
| TBS determination | Scalable with TTI length as baseline |
| HARQ RTT | 8 TTIs |
| Scheduler | Proportional fairness |
| Antenna configuration | 2Tx(eNB), 2Rx(UE), Cross-polarized |
| Number of UEs | 10 UEs per macro cell for FTP model 2 (latency reduction capable UEs only) |
| UE dropping | Randomly and uniformly dropped throughout the macro geographical area. 20% UEs are outdoor and 80% UEs are indoor. |
| Traffic model | FTP model 2, File size [100kbits, 500kB] RU[20%, 60%] |
| TCP models | TCP Reno model (RFC 2581) - SSThresh 65535 Bytes - Initial window size 1460 Bytes - Max segment size 1460 Bytes 40 Bytes TCP header are added to the initial window size and max segment size The three way handshake; not modeled TCP ACK feedback; error free |
| RLC model | None |
| UE receiver | MMSE-IRC |
| UE speed | 3km/h |
| Duplex mode | FDD |
| Network synchronization | Synchronized |
| Core, transport and internet network delay | 0ms, 6ms, 10ms |

- Annex 2

- User throughput                                                                      (b) packet delay

Figure 1 File size = 100kbits, RU 20%

- User throughput                                                                      (b) packet delay

Figure 2 File size = 500kbyte, RU 20%

- User throughput                                                                      (b) packet delay

Figure 3 File size = 100kbits, RU 60%

- User throughput                                                                      (b) packet delay

Figure 4 File size = 500kbyte, RU 60%