3GPP TSG-RAN WG1 Meeting #84	R1-160790

St. Julian’s, Malta, 15th – 19th February 2016

Agenda item:		7.3.4.1

Source:	Nokia Networks, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell

Title:	System-level Evaluation of TTI Shortening

Document for:		Discussion and Decision

# 1. Introduction

Based on the outcome of RAN#67 captured in the SI description in RP-150465, the following items have been identified specifically for RAN1 studies on TTI shortening and reduced processing times:

- Assess specification impact and study feasibility and performance of TTI lengths between 0.5ms and one OFDM symbol, taking into account impact on reference signals and physical layer control signaling

- Backwards compatibility shall be preserved (thus allowing normal operation of pre-Rel 13 UEs on the same carrier);

In RAN1#83, the simulation assumptions for system-level and link-level evaluations have been agreed [1]. In this contribution, we present the system-level evaluation results of TTI shortening based on the RAN1 agreed simulation assumptions.

# 2. System-level evaluation setup and results

Herein presented system simulations are performed for the 3GPP macro scenario, i.e. 3GPP Case 1 macro scenario, with 7 sites, 21 cells and wrap-around. There are 10 users dropped per macro cell area. A quasi-static model is used with static UE mobility and a spatial channel model with the speed of 3km/h. The utilized traffic model is FTP 3, where the cell load is varied by having different packet arrival rates of λ=0.25, 0.5 or 0.75 per second per user for a fixed file size of 0.5MB. The different cell loads are roughly translated to the resource utilization (RU) of 20% (low load), 40% (medium load) or 60% (medium-to-high load) per cell. The key simulation parameters, compliant to [1], are summarized in Appendix A.

We assume that the FTP traffic is transported using TCP. For a certain user, files are arriving according to the given arrival rate. This means that a single user is served with multiple files and even with multiple files at the same time. Please refer to [2] for more details. In addition, it is generally assumed that the TCP connection with slow start is only called at the first file transmission of the user. Also note that further in the text we denote the file as a packet.

With shorter TTI (sTTI), the overhead will be inevitably increased from both L1 and L2 perspective. As shown in Table B-1 in Appendix B for the case of legacy TTI length, the exact percentage of the L1 overhead depends on the number of REs occupied by the L1 control channel region and the reference signals that are utilized. Therefore, in the presented simulation results, the legacy 14-symbol TTI case is used as the baseline and we consider total overheads of 20%, 30% or 40% respectively. For the shorter TTI case, the exact overhead will depend on the final design of L1 control channel and reference signal patterns, being unknown at this point. Therefore, in the presented simulation results, the sTTI performance is evaluated as well with the total overhead assumption of 20%, 30% or 40% respectively.

As discussed in our companion contribution [3], practically the support for all possible 7 cases would complicate the sTTI design and seems to be unnecessary. Moreover, it is clearly an advantage to have the new sTTI length equal to an integer fraction of the legacy TTI length, i.e. an integer number of sTTI’s fitting within 1ms. This will simplify system operation and specification regarding the needed backward compatibility operation on a single LTE carrier. From this perspective, the considered sTTI lengths should be limited to 1, 2 and 7 OFDM symbols. Furthermore, a very large specification impact to both DL and UL is expected in order to support the 1-symbol sTTI option. Therefore, we are not in favor of considering the sTTI length of 1-symbol. As a consequence, we focused the performance evaluation on the cases of 2- and 7- OFDM symbol TTIs.

2.1 System-level Simulation Results:

We benchmark different schemes with respect to two performance metrics:

- Packet throughput, corresponding to the averaged throughput of one file.

- Packet transmission delay, corresponding to the time needed to download one file,

where the 5%-tile, 50%-tile, 95%-tile and mean value for each performance metric are shown.

Figure 1 shows the 5%-tile, 50%-tile, 95%-tile and mean packet throughputs at 3kmph, and at different packet arrival rate λ of 0.25, 0.5, and 0.75 per second in x-axis.

We will firstly focus on the comparisons and observations given the same percentage of overhead. As shown in Figure 1, the general curve trend is that for the 5%-tile coverage and 50%-tile median throughput, the shorter TTI gain is mostly observed at the low load (λ=0.25). With the increasing cell load and correspondingly increased RU, the gain of shorter TTI over the legacy TTI is reducing. At medium-to-high load (λ=0.75) the throughput performance of shorter TTI is actually quite close to that of the legacy TTI, especially with the overhead assumption of 40%. The highest gain of sTTI over the legacy TTI is observed for the 95%-tile peak throughput. This means that the slow start has more effect/impact to the peak packet throughputs compared to lower percentiles. In other words, cell center users seem to benefit the most from shortened TTI length.

With the increasing overhead, the packet throughput is jeopardized as expected. The impact is visible for both legacy and shorter TTI, however more impact can be observed for the shorter TTI cases. Especially for the 5%-tile coverage and 50%-tile median throughput, the shorter TTI gain at the low load is dropping rapidly with the increasing overhead.

When comparing the throughput performance gain of the 2-symbol and 7-symbol cases, for the 5%-tile coverage and 50%-tile median throughput, the 2-symbol sTTI has the largest gain over the 7-symbol sTTI at the low load, especially at low overhead assumption of 20%. However, the 2-symbol sTTI throughput gain over 7-symbol case drops rapidly with the increasing cell load, especially at higher overhead assumption of 30% or 40%. Starting from the medium load (λ=0.5) up to the medium-to-high load (λ=0.75), the 2-symbol sTTI has similar performance to 7-symbol sTTI for both 5%-tile and 50%-tile coverage throughput, where similar trend can be seen also in the curve of mean packet throughput.

| 5%-tile coverage throughput | 50%-tile median throughput |
| --- | --- |
| 95%-tile peak throughput | Mean packet throughput |

Figure 1 Packet throughput at 3kmph

The shorter TTI seems to provide reasonable gain in non-overloaded network. However, it has to be pointed out that the gain is only visible in the case of constant overhead comparison. As discussed in our companion contribution in [2], with shorter TTI the overhead is expected to increase from both L1 and L2 perspective.

Observation-1: Shorter TTI can provide throughput gain in low to moderate network load with the assumption of constant overhead.

Secondly, we will focus on the comparisons and observations for cases with unequal percentage of overhead.  We will compare following cases:

- 20% total overhead with legacy TTI,

- 30% total overhead with 7-symbol sTTI (i.e. additional 50% more overhead compared to legacy TTI),

- 40% total overhead with 2-symbol sTTI (i.e. additional 100% more overhead compared to legacy TTI).

For the 5%-tile throughput and unequal overhead assumption, the legacy TTI has actually better performance than sTTI in all the cell loads meaning that sTTI operation has a worse cell edge performance.

For the 50%-tile throughput and unequal overhead assumption, the sTTI still shows gains over legacy TTI in low load. But with increasing cell load the throughput of 2-symbol sTTI is dropping rapidly and performance loss is visible starting from the medium cell load. Similar performance is observed for 7-symbol sTTI and legacy TTI. Therefore, the 7-symbol sTTI seems to be the more robust candidate, considering varying cell loads, to be considered further in the shorter TTI study.

For the 95%-tile throughput and unequal overhead assumption, gains from sTTI over legacy TTI are still visible even with the added extra overhead consideration of 10% and 20% percent, respectively. But the throughput performance of 2-symbol sTTI and 7-symbol sTTI is rather similar for these example overhead assumptions.

In case of mean user throughput and unequal overhead assumption, the 7-symbol TTI with 30% overhead has similar performance to the 2-symbol TTI with 40% overhead. Therefore, to see the benefit from 2-symbol TTI, the control and RS absolute overhead increase, from 7 to 2 symbol shortening, needs to be smaller than 10%. This might be difficult to achieve, assuming that for 2-symbol TTI up to three times more DL control and UL grants would need to be sent compared to 7-symbol TTI, allowing for full scheduling flexibility.

Observation-2: Shorter TTI user throughput gains diminish and can become negative with the increased overhead due to TTI shortening. The advantages of 2-symbol TTI over one slot TTI is lost if the absolute overhead increase is around 10%.

| 5%-tile packet delay | 50%-tile packet delay |
| --- | --- |
| 95%-tile packet delay | Mean of packet delay |

Figure 2 Packet delay at 3kmph

Figure 2 shows the 5%-tile, 50%-tile, 95%-tile and mean packet transmission delay at 3kmph. The reduced packet transmission delay gain at 5%-tile, 50%-tile, 95%-tile with sTTI is translated to the corresponding packet throughput performance of 95%-tile, 50%-tile, 5%-tile. For example, the large packet throughput gain with sTTI, observed at 95%-tile in Figure 1, corresponds to significantly reduced packet transmission delay observed in Figure 2 at 5%-tile.

However, above reciprocity does not hold for the mean statistics. Compared to the mean throughput, the overhead has bigger impact on the mean packet delay. For example, when comparing the mean packet delay for 7-symbol sTTI (20/30% overhead) and 2-symbol sTTI (30/40% overhead), the 7-symbol sTTI has clearly superior performance in terms of delay performance irrespective of network load.

Observation-3: The slot-level 7-symbol TTI has a reasonable latency reduction over the legacy TTI case in all the simulated cell loads.

Observation-4: The slot-level 7-symbol TTI has smaller mean delay compared to 2-symbol TTI with 10% absolute overhead increase for all simulated network loads.

# 3. Summary

In this contribution we presented the system-level evaluation of the TTI shortening according to agreed RAN1 simulation assumptions. Compared to our previously contributed results in RAN2 [4], the observation with the RAN1 agreed simulation assumptions are similar:

Observation-1: Shorter TTI can provide throughput gain in low to moderate network load with the assumption of constant overhead.

Observation-2: Shorter TTI user throughput gains diminish and can become negative with the increased overhead due to TTI shortening. The advantages of 2-symbol TTI over one slot TTI is lost if the absolute overhead increase is around 10%.

Observation-3: The slot-level 7-symbol TTI has a reasonable latency reduction over the legacy TTI case in all the simulated cell loads.

Observation-4: The slot-level 7-symbol TTI has smaller mean delay compared to 2-symbol TTI with 10% absolute overhead increase for all simulated network loads.

# 4. References

- R1-157806, “Simulation assumptions for latency reduction”, RAN1#83, Anaheim, USA, 15-22 November 2015

- TR-36.889, “Feasibility Study on Licensed-Assisted Access to Unlicensed Spectrum”, RAN1 3GPP

- R1-160785, “On shorter TTI for latency reduction”, Nokia Networks, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, RAN1#84, St Julian’s, Malta, 15-19 February 2015

- R2-153223, “Performance evaluation of latency reduction enhancements”, Nokia Networks, RAN2 #91, Beijing, China, 24 - 28 August 2015

# Appendix A: Simulation parameters

| Parameter | Assumption |
| --- | --- |
| System bandwidth | 20 MHz |
| Duplex mode | FDD |
| Carrier frequency | 2GHz |
| Cell layout | Hexagonal grid, 7 sites, 21 cells per site, with wrap-around |
| Number of UEs per macro sector | 10 (80% indoor, 20% outdoor) |
| Inter-site distance | 500m |
| UE speed | 3 km/h, quasi-static model |
| Antenna configuration | 2x2, cross-polarized |
| Receiver DL | LMMSE-IRC |
| eNB TX power | 46 dBm |
| eNB antenna height | 25 m |
| Antenna pattern | 3D |
| UE Tx power | 23 dBm |
| UE antenna height | 1.5 m |
| Channel model | 3D-UMa |
| Pathloss model | UMa, with 3D distance between eNB and UE |
| Shadowing | UMa, with 3D distance between eNB and UE |
| Penetration loss | Outdoor UEs: 0 dB, Indoor UEs: 20 dB+0.5din |
| CSI feedback period | 5 ms |
| Feedback mode | 3-1 |
| CSI report delay | 6 ms |
| Channel and interference estimation | Ideal |
| SR to grant | 8 TTIs |
| HARQ RTT | 8 TTIs |
| SR Period | 5 ms |
| DRX | Disabled |
| Transport type | TCP |
| TCP ACKs | Error-free |
| Initial TCP Window | 3 x 1500 Bytes (MSS), RFC 5681, section 3.1 |
| Initial Ssthresh | 45 x 1500 Bytes (MSS) |
| Ssthresh | Dynamic according to RFC 5681, sections 3.1 and 3.2 |
| FTP file size | 0.5 MB |
| User Packet arrival rate λ | FTP model 3 with packet arrival according to Poisson process: 0.25, 0.5, 0.75 |
| Scheduler | TD: PF, FD: PF |
| Maximum number of scheduled users per TTI | 10 (max) |
| L1+L2 overhead | 20, 30, 40 % |
| Core network delay | 2ms |
| TTI Length | 2 symbols, 7 symbols, 14 symbols |
| MCS | QPSK, 16 QAM, 64 QAM |
| Network synchronization | Synchronous |

# Appendix B: L1 overhead considerations

Table B-1: Legacy 14-symbol TTI

| Legacy PSS/SSS/PBCH=264 REs per 5ms |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Legacy 14-symbol TTI |  |  |  |  |  |
| 42000 REs= 12subcarrier*7symbol*2slot*50PRB*5ms |  |  |  |  |  |
| CRS OH (outside PDCCH region) |  |  |  |  |  |
| CFI=1 |  | CFI=2 |  | CFI=3 |  |
| 2Tx | 4Tx | 2Tx | 4Tx | 2Tx | 4Tx |
| 3000 | 5000 | 3000 | 4000 | 3000 | 4000 |
| 7.1% | 11.9% | 7.1% | 9.5% | 7.1% | 9.5% |
| Control region OH |  |  |  |  |  |
| 3000 |  | 6000 |  | 9000 |  |
| 7.1% |  | 14.3% |  | 21.4% |  |
| Total OH |  |  |  |  |  |
| 14.9% | 19.7% | 22.1% | 24.4% | 29.2% | 31.6% |
| Adding DMRS OH on Top |  |  |  |  |  |
| Rank 1-2: 12*50*5 = 3000 REs |  |  |  |  |  |
| 22.1% | 26.8% | 29.2% | 31.6% | 36.3% | 38.7% |
| Rank > 2: 24*50*5 = 6000 REs |  |  |  |  |  |
| 29.2% | 34.0% | 36.3% | 38.7% | 43.5% | 45.9% |

Table B-2: Slot-level 7-symbol sTTI

| PSS/SSS OH=144 REs per 5ms |  |  |  |  |  | PBCH OH=132(2Tx) or 120(4Tx) REs per 5ms |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1st slot |  |  |  |  |  | 2nd slot |  |  |  |
| 12subcarrier*7symbol*50PRB*5ms = 21000 REs |  |  |  |  |  | 21000 REs |  |  |  |
| CRS OH (outside PDCCH region) |  |  |  |  |  | CRS OH |  |  |  |
| CFI=1 |  | CFI=2 |  | CFI=3 |  | CRS 2Tx: 8*50*5=2000 REs CRS 4Tx: 12*50*5=3000 REs |  |  |  |
| 2Tx | 4Tx | 2Tx | 4Tx | 2Tx | 4Tx |  |  |  |  |
| 10001) | 20002) | 10001) | 10001) | 10001) | 10001) |  |  |  |  |
| 4.8%3) | 9.5%4) | 4.8%3) | 4.8%3) | 4.8%3) | 4.8%3) | CRS 2Tx: (2000/21000)*100%=9.5% CRS 4Tx: (3000/21000)*100%=14.3% |  |  |  |
| Control region OH |  |  |  |  |  | Control OH (depends if there is sePDCCH) |  |  |  |
| 30005) |  | 60005) |  | 90005) |  | N/A |  |  |  |
| 14.3%6) |  | 28.6%6) |  | 42.9%6) |  |  |  |  |  |
| Total OH |  |  |  |  |  | Total OH (if no sePDCCH) |  |  |  |
| 19.7%7) | 24.5%7) | 34.0%8) | 34.0%8) | 48.3%9) | 48.3%9) | CRS 2Tx: (2000+132)/21000=10.2% CRS 4Tx: (3000+120)/21000=14.9% |  |  |  |
| Adding legacy DMRS OH on Top |  |  |  |  |  | Adding legacy DMRS OH on Top |  |  |  |
| Rank 1-2: 12*50*5 = 3000 Res |  |  |  |  |  | Rank 1-2: 12*50*5 = 3000 REs |  |  |  |
| 34.0% | 38.8% | 48.3% | 48.3% | 62.6% | 62.6% | CRS 2Tx: (2000+132+3000)/21000=24.4% CRS 4Tx: (3000+120+3000)/21000=29.1% |  |  |  |
| Rank > 2: 24*50*5 = 6000 Res |  |  |  |  |  | Rank > 2: 24*50*5 = 6000 REs |  |  |  |
| 48.3% | 53.1% | 62.6% | 62.6% | 76.9% | 76.9% | CRS 2Tx: (2000+132+6000)/21000=38.7% CRS 4Tx: (3000+120+6000)/21000=43.4% |  |  |  |
| Note: 1) 4*50*5=1000 REs 2) 8*50*5=2000 REs 3) (1000/21000)*100%= 4.8% 4) (2000/21000)*100%= 9.5% 5) 1or2or3*12*50*5=3000or6000or9000 REs 6) (3000or6000or9000/21000)*100%=14.3%or28.6%or42.9% 7) (1000or2000+3000+144)/21000=19.7%or24.5% 8) (1000+6000+144)/21000=34.0% 9) (1000+9000+144)/21000=48.3% |  |  |  |  |  | Assume 8 PRB frequency-pair OH for sePDCCH |  |  |  |
|  |  |  |  |  |  | CRS 2Tx |  | CRS 4Tx |  |
|  |  |  |  |  |  | 25.9% |  | 30.6% |  |
|  |  |  |  |  |  | Rank 1-2 | Rank>2 | Rank 1-2 | Rank>2 |
|  |  |  |  |  |  | 40.1% | 54.2% | 44.7% | 58.9% |