3GPP TSG RAN WG1 Meeting #84                             R1-160792

St Julian’s, Malta, 15th - 19th February 2016

Agenda item:		7.3.2.1.1

Source: 			Beijing Xinwei Telecom Techn.

Title:             Link performance of DMRS Enhancement for PC5-based V2V

Document for:		Discussion and Decision

- Introduction

In 3GPP RAN1#83, the feasibility study on LTE-based V2X Services [1] has completed the part of PC5 transport for V2V services. The RAN study concluded that it is feasible to support V2V services based on LTE PC5 interface with necessary enhancements. In RAN#70, a work item on LTE-based V2V Services was approved in [2]. The objectives of the work item are to specify LTE sidelink enhancements for V2V services defined in [3] and the first objective is:

- To specify enhancement to sidelink physical layer structure necessary for V2V services [RAN1]

The enhancement to sidelink physical layer structure mainly focus on the design of DMRS to handle high Doppler case. In this contribution, we provide link performance for different DMRS structures.

- Overview of DMRS enhancements

According to the V2V communication requirement [3], the maximum relative moving speed between two vehicles is 280km/h. With the large V2V carrier frequency, e.g., 6GHz, the Doppler shift can be up to 1556Hz, and the corresponding coherence time is about 0.27ms, which is smaller than the 0.5ms DMRS interval of the current D2D DMRS structure. On the other hand, the CFO estimation range also depends on the interval between two consecutive DMRS symbols, and the estimation range of current D2D DMRS structure is about 1 KHz, which is smaller than the possible maximum 1556Hz CFO caused by Doppler shift.

To solve the CFO and channel estimation problems, lots of DMRS enhancements were discussed during the past meetings and the most popular ones are summarized in Table 1 with their advantages and disadvantages.

Table 1   Comparison for DMRS enhancements

|  | advantages | disadvantages |
| --- | --- | --- |
| Increase DMRS density | easy to implement | increase the pilot overhead |
| DMRS with comb | increase CFO estimation range | worse performance of channel estimation |
| Horizontal DMRS pattern | robust to the channel variations | higher cubic metric |
| Increase subcarrier spacing | reduce latency | need more changes to the current LTE specification |

In RAN1#83, these DMRS enhancements were extensively discussed and the following agreements were reached [4].
Agreements:

- Confirm the baseline on SC-FDM is used for V2V transmission in each physical channel

- Working assumption: Increase DMRS density to 4 symbols per 1ms with reusing PUSCH DMRS sequence in each physical sidelink channel except for PSBCH

Options of DMRS location for evaluation (counting from #0)

- Other options are not precluded.

- For normal CP with 15 kHz subcarrier spacing

  - Option 1: #2, #5, #8, #11

    - Note: This is for regular spacing.

  - Option 2: #1, #5, #8, #12

    - Note: Reuse RS location of PUCCH format 2

  - Option 3: #2, #4, #9, #11

    - Note: Frequency offset estimation first using {#2, #4} and {#9, #11}

  - Option 4: #3, #6, #7, #10

    - Note: Frequency offset estimation first using {#6, #7}

  - Assumption: Transmissions in a single TTI (i.e., no HARQ retransmission). It is encouraged to evaluate both SA and data.

    - Baseline: QPSK with coding rate of 0.5

    - Optional: QPSK with coding rate of 0.7, 16QAM with coding rate 0.5 (only for data)

    - Frequency error: Baseline is to evaluate both {Case 1+Case B} and {Case 2+Case A}. Other cases can be considered, e.g., based on RAN4 feedback.

      - Case 1: The extreme case should be assumed, i.e., +0.1 PPM for TX and -0.1 PPM for RX w.r.t. UE’s sync reference.

        - Performance in Case 1 is to check whether the system can work in the extreme case.

      - Case 2: Frequency error in each UE is uniformly distributed [-0.1, 0.1] PPM w.r.t. UE’s sync reference.

    - Frequency error between sync references of TX and RX:

      - Case A: 0 error (i.e., the same reference)

      - Case B: The extreme case should be assumed, i.e., +0.05 PPM for TX’s reference and -0.05 PPM for RX’s reference w.r.t. the absolute frequency.

  - Companies should describe the receiver algorithm of the evaluated options.

- Link level performance

In this section, we compare the link level performance for different options of DMRS location as described in Section 2. The detailed simulation assumptions are listed in Appendix A.

The BLER performance for different simulation parameters e.g., vehicle moving speed, frequency error and packet size (MCS), are shown in figure 1. As we can see from the figures, the BLER performance of different DMRS options vary with the parameters. For example, when vehicle moving speed increases from 15km/h to 140km/h, the BLER performance become worse with the Doppler shift increasing; When frequency error becomes larger (i.e., from case2+caseA to case1+caseB) or MCS becomes higher (i.e., from 300byte to 800byte), the BLER performance become worse as well. Detailed BLER performance for other simulation parameters are provided in Appendix B.

Figure 1. BLER performances for different simulation parameters

To make the comparison simpler, maximum coupling loss (MCL) is used in the following part to evaluate the link level performance, which is calculated as :

MCL (dB) =maximum transmit power (dBm)+ transmit antenna gain + receive antenna gain –(thermal noise density (dBm/Hz) + receive noise figure (dB) +10log10(occupied channel bandwidth (Hz)) + required SINR (dB))

The parameters in the above equation can be found in [1]. We summarize the MCL values supporting BLER<10% in Table 3 to Table 5 for different packet size respectively.

Table 3 MCL supporting BLER<10% for 190 byte Packet

| Speed CFO DMRS | 15km/h |  | 60km/h |  | 140km/h |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | case2+caseA | case1+caseB | case2+caseA | case1+caseB | case2+caseA | case1+caseB |
| Option1 | 127.0 | 126.8 | 127.2 | 127.3 | 124.8 | NaN |
| Option2 | 127.1 | 127.1 | 127.2 | 127.3 | NaN | NaN |
| Option3 | 126.8 | 126.8 | 127.2 | 127.2 | 125.6 | 125.6 |
| Option4 | 126.5 | 126.5 | 126.9 | 127 | 123.1 | 122.7 |

Table 4 MCL supporting BLER<10% for 300 byte Packet

| Speed CFO DMRS | 15km/h |  | 60km/h |  | 140km/h |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | case2+caseA | case1+caseB | case2+caseA | case1+caseB | case2+caseA | case1+caseB |
| Option1 | 125.4 | 125.4 | 125.6 | 125.7 | 123.3 | NaN |
| Option2 | 125.5 | 125.5 | 125.9 | 125.7 | 117 | NaN |
| Option3 | 125.2 | 125.3 | 125.5 | 125.4 | 124 | 124 |
| Option4 | 125 | 125 | 125.4 | 125.2 | 121.9 | 121.9 |

Table 5 MCL supporting BLER<10% for 800 byte Packet

| Speed CFO DMRS | 15km/h |  | 60km/h |  | 140km/h |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | case2+caseA | case1+caseB | case2+caseA | case1+caseB | case2+caseA | case1+caseB |
| Option1 | 120.6 | 120.7 | 120.2 | 120.2 | 116.2 | NaN |
| Option2 | 120.8 | 120.8 | 120.2 | 120.2 | NaN | NaN |
| Option3 | 120.5 | 120.6 | 120.2 | 120.2 | 115.9 | 116 |
| Option4 | 120.2 | 120.3 | 119.8 | 119.7 | NaN | NaN |

The simulation results show that, for the low and medium Doppler cases, i.e., 15km/h and 60km/h, all the four DMRS options perform well and have similar link performance. Option 1 and option 2 perform slightly better than option 3 and option 4 since the DMRS symbol distributions of Option 1 and 2 are more uniform, leading to more accurate channel estimation.

For the high Doppler case, i.e., 140km/h, as the CFO estimation range of option 1 and option 2 can’t cover the large CFO, the performance deteriorate seriously and can not achieve 10% BLER in the extreme {Case 1+Case B}. With larger CFO estimation range, option 3 and option 4 still perform well and option 3 has better performance with good trade-off between CFO and channel estimation.

When using QPSK with coding rate of 0.5, i.e., packet size of 190byte and 300byte, the MCL performance of option 3 and option 4 can meet the PC5 V2V requirements according to the simulation results in [1, Annex B.2]. For higher level MCS, i.e., packet size of 800byte, option 3 will work with necessary enhancements, e.g., more transmission times or cooperative transmission scheme in [5].

Based on the simulation results above, we make the following observations and proposal.

Observations:

Observation 1: DMRS option 1 and option 2 have good channel estimation accuracy for the low and medium Doppler cases, but could not handle the high Doppler case.

Observation 2: DMRS option 3 and option 4 are more robust to the high CFO and can meet the PC5 V2V requirements when using QPSK with coding rate of 0.5.

Observation 3: With necessary enhancements, DMRS option 3 can meet the PC5 V2V requirements when using higher level MCS, e.g., QPSK with coding rate of 0.67.

Proposals:

Proposal 1: Use DMRS option 3 as baseline for PC5-based V2V transmission.

- Conclusions

In this contribution, we have extensively compared the performance of different DMRS location options under various speed, synchronization accuracy and MCS assumptions. Observations and proposals are summarized below.

Observations:

Observation 1: DMRS option 1 and option 2 have good channel estimation accuracy for the low and medium Doppler cases, but could not handle the high Doppler case.

Observation 2: DMRS option 3 and option 4 are more robust to the high CFO and can meet the PC5 V2V requirements when using QPSK with coding rate of 0.5.

Observation 3: With necessary enhancements, DMRS option 3 can meet the PC5 V2V requirements when using higher level MCS, e.g., QPSK with coding rate of 0.67.

Proposals:

Proposal 1: Use DMRS option 3 as baseline for PC5-based V2V transmission.

- References

[1] R1-157830, 3GPP TR 36.885 V0.3.0, Study on LTE-based V2X Services

[2] RP-152293, New WI proposal: Support for V2V services based on LTE sidelink, LGE, Huawei, HiSilicon, CATT, CATR

[3] 3GPP TR 22.885, Study on service aspects for LTE-based V2X, LGE

[4] Chairman's Notes RAN1_83 - v035

[5] R1-157534,	Discussion on enhancement of V2X resource allocation, Beijing Xinwei Telecom Techn.

- Appendix A: simulation assumptions

Table 2 simulation assumptions

| Carrier frequency | 6 GHz |
| --- | --- |
| Bandwidth | 10MHz |
| Packet size | 190byte, 300byte, 800byte |
| Packet modulation/coding | 190byte, 300byte: QPSK with coding rate of 0.5 800byte: QPSK with coding rate of 0.67 |
| Resource allocation | 190byte: 16 PRBs, 1 TTI 300byte: 25 PRBs, 1 TTI 800byte: 50 PRBs, 1 TTI |
| Antenna configuration | 1 Tx, 2 Rx |
| Channel model | ITU-R UMi CDL models with dual-mobility (NLOS) |
| Vehicle moving speed | 15km/h (relative speed of 30km/h) 60km/h (relative speed of 120km/h) 140km/h (relative speed of 280km/h) |
| CFO | Case1+CaseB, Case2+CaseA |
| CFO estimation method | based on the phase difference of different DMRS symbols |
| Channel estimation method | LS with quadratic smoothing |

Subframe structure for different DMRS location options:

- Appendix B: BLER performance

| 15km/h, case2+caseA, {190 300 800} byte | 15km/h, case1+caseB, {190 300 800} byte |
| --- | --- |

| 60km/h, case2+caseA, {190 300 800} byte | 60km/h, case1+caseB, {190 300 800} byte |
| --- | --- |

| 140km/h, case2+caseA, {190 300 800} byte | 140km/h, case1+caseB, {190 300 800} byte |
| --- | --- |