3GPP TSG RAN WG1 Meeting #84                                                               R1-160982

St Julian’s, Malta, 15th – 19th February 2016

Source:	ZTE

Title:	System-level performance evaluation for short TTI

Agenda Item:	7.3.4.1

Document for: 	Discussion and Decision

# Introduction

In the Release 13 study item on latency reduction techniques for LTE [1], RAN1 needs to study the TTI shortening and reduced processing times, including the following aspects:

- Assess specification impact and study feasibility and performance of TTI lengths between 0.5ms and one OFDM symbol, taking into account impact on reference signals and physical layer control signaling.

- Backwards compatibility shall be preserved (thus allowing normal operation of pre-Rel 13 UEs on the same carrier).

This contribution provides some system-level evaluation results for the TTI shortening.

# Evaluation assumptions

The evaluations in this contribution are performed in a ITU macro layout of 7 macro sites and 3 cells per site, with wrap-around feature turned on. The FTP traffic model2 is used with model parameters given in Table 1.

| Parameter | Assumption |
| --- | --- |
| FTP traffic model | FTP Traffic Model 2 [2] - Reading Time: Exponential Distribution, with parameter TBD for specific traffic loads |
| File size/ reading time D | 100Kbit / D =1/8s 100KB / D = 1s 500KB / D = 3s / D = 5s / D = 8s |

Table 1 FTP traffic model 2 parameters

To evaluate the short TTI gains in different load conditions, resource utilization varies from 26% to 57%. The HARQ RTT is assumed to be always 8 TTIs, i.e., scaled with TTI length shortening. Similarly, it is assumed that CSI report cycle is 5TTI and delay is 6TTI.

The overhead calculation is found to have important impacts to the evaluation conclusion, and therefore is described below with more details.

L1 overhead from reference signal and control signalling

The overall L1 overhead includes the REs occupied by 2-port CRS and downlink control signalling.

- For 1ms TTI with 14 OFDM symbols (baseline), PDCCH region is assumed to occupy 2 OFDM symbols, and the CRS contributes additional 12 RE overhead outside of PDCCH region per PRB pair. Therefore, the total L1 overhead percentage per TTI is (2*12+12)/(12*14) = 21%.

- For 0.5ms TTI with 7 OFDM symbols, new PDCCH region is assumed to occupy 1 OFDM symbol in the second TTI per subframe. Depending on whether the legacy PDCCH have one or two symbols, the L1 overhead can be either lower or higher than 21%. With the assumption that legacy PDCCH almost takes equal probabilities to occupy one and two symbols, the total L1 overhead percentage per TTI is assumed to be still 21%.

- For the TTI with 2 or 1 OFDM symbol, the total L1 overhead percentage is supposed to be larger than the one in legacy subframe structure. Without specific TTI structure design guidelines to be agreed in RAN1 at this stage, the evaluations in this contribution assume a total 30% L1 overhead per TTI [3].

Higher-layer overhead from headers and padding

For clarification purpose, the description below assumes the FTP file size equal to 0.5Mbyte and resource utilization equal to 26%.

- The header overhead comes from the total header bits (from the TCP layer to the MAC layer), which includes following but not the headers in the RLC/HARQ retransmission: TCP/IP header (8 bytes with ROHC compression and 40 bytes without ROHC compression), PDCP header (2 bytes), RLC header (2bytes) and MAC header (2 bytes). When the TTI length is shortened, the header overhead increases because the smaller TBS results in more data segments for the RLC/MAC layers, and therefore more header occurrences. This is shown in Figure 1(a).

- The padding overhead counts for the dummy bits used to fill up the transport block after the data from traffic source are put into transport block. When the TTI length is shortened, the padding overhead decreases because the system can have smaller granularity on TBS which further confines the upper-bound of padding for a given traffic packet. This is shown in Figure 1(b).

(a) Header overhead                         (b) Padding overhead                            (c) Overall overhead

Figure 1 Percentage changes of higher layer overheads comparing to 1ms TTI case

In the evaluation, the total higher-layer overhead percentage is counted by:

The total high-layer overhead % = (the total TBS size(s) used for data transmission – the total transmitted bits of FTP file) / the total TBS size(s) used for data transmission.

As shown in Figure 1(c), this total higher-layer overhead percentage decreases as the TTI length is shortened.

As TTI length is reduced, the header overhead increases but the padding overhead decreases; the combined total overhead decreases.

# Evaluation results

Performances upon different resource utilizations

The following evaluation results aim to show the impact of different resource utilizations varying from 26% to 57%, by meanwhile assuming the shortened TTI length equal to 0.5ms and FTP file size equal to 0.5Mbyte.

The results shown in Figure 2 indicate that, with the RU increasing, short TTI gain decreases. This is because that, with the increased loads, the more packet life time is spent in the TCP layer RTT buffer for purposes of queuing and processing, which reduces the effective user throughput. What’s more, it can seen from Figure 2 that, even though the 5% UPT gain (representing the cell edge performance) decreases as RU increases, the gain itself is always positive. Although the TBS of cell-edge UE is smaller than the TBS of central UE, which introduces larger RLC/MAC header overheads, it brings down the total overhead with the saving on padding, as observed in section 2.

Figure 2 UPT gain of 0.5ms TTI under different RUs

Figure 3 shows the gains of latency reduction under different loads. It can be seen that the mean, gains at 5%, 50% and 95% of latency decrease with the increased RU. Furthermore, TTI shortening always reduces the latencies at mean and all percentage measurements.

Figure 3 Latency reduction of 0.5ms TTI under different RUs

Observation 1: For the given file size, TTI shortening always brings up UPT and reduces latency, but the gains on both decrease as the RU increases.

Performances upon different file sizes

Figure 4 Gains of mean UPT with different file sizes

Figure 5 Gains of 5% UPT with different file sizes

Figure 6 Gains of 50% UPT with different file sizes

Figure 7 Gains of 95% UPT with different file sizes

As shown in Figure 4~Figure 7, the UPT gain increase with the reduction of the TTI duration and TTI shortening gives bigger gain in mean UPT performance for small-sized files. Meanwhile, we can see some gain even for larger file.

Observation 2: For the mean, 5%, 50%, and 95% UPT performance, TTI shortening can bring significant gains for different file sizes.

Figure 8 Reduction of mean latency with different file sizes

Figure 9  Reduction of 5% latency with different file sizes

Figure 10  Reduction of 50% latency with different file sizes

Figure 11 Reduction of 95% latency with different file sizes

As shown in Figure 8~Figure 11, the reduction gains of mean, 5%, 50% and 95% latency increase as the TTI duration reduces. Especially, similar to the UPT gain, some latency reduction gain for short TTI can be observed even for larger file.

Observation 3: TTI shortening can bring gains in latency performance for different file sizes.

# Conclusion

This contribution provides the system-level simulation results for TTI shortening, together with the following observations:

Observation 1: For the given file size, TTI shortening always brings up UPT and reduces latency, but the gains on both decrease as the RU increases.

Observation 2: For the mean, 5%, 50%, and 95% UPT performance, TTI shortening can bring significant gains for different file sizes.

Observation 3: TTI shortening can bring gains in latency performance for different file sizes.

# References

- RP-150465, “SI: Study on Latency reduction techniques for LTE”.

- 3GPP TR 36.912, Feasibility study for Further Advancements for E-UTRA (LTE-Advanced).

- R1-156820, “Preliminary system-level performance evaluation for TTI shortening” Samsung

- R1-157150, “Evaluations on latency reduction with TTI shortening” ZTE.

- R2-153578, “Performance evaluation on short TTI” ZTE.

- R1-157806, “WF on evaluation methodology for latency reduction”.

# Appendix1: Simulation results

Appendix1-1: Different resource utilizations

| ### RU ### (%) | ### TTI length | ### Mean UPT(Mbps) | ### 5% UPT | ### 50% UPT | ### 95% UPT | ### Mean latency(s) | ### 5% latency | ### 50% latency | ### 95% latency |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ### 26 | ### 14 OFDMs ### Gain | ### 3.3972 ### (0%) | ### 3.2336 ### (0%) | 3.4193 (0%) | 3.544 (0%) | 4.3529 (0%) | 2.1234 (0%) | 4.2451 (0%) | 6.2997 (0%) |
|  | ### 7 OFDMs ### Gain | ### 4.3484 ### (+28%) | ### 3.945 ### (+22%) | 4.2377 (+24%) | 4.4654 (+26%) | 3.4823 (-20%) | 1.6987 (-20%) | 3.5234 (-17%) | 4.9768 (-21%) |
| ### 43 | ### 14 OFDMs ### Gain | ### 3.3498 ### (0%) | ### 2.9582 ### (0%) | 3.3944 (0%) | 3.5746 (0%) | 5.9695 (0%) | 2.268 (0%) | 5.888 (0%) | 9.719 (0%) |
|  | ### 7 OFDMs ### Gain | ### 4.2207 ### (+26%) | ### 3.5794 ### (+21%) | 4.2091 (+24%) | 4.3968 (+23%) | 4.7876 (-19.8%) | 1.8371 (-19%) | 4.8282 (-18%) | 8.0668 (-17%) |
| ### 57 | ### 14 OFDMs ### Gain | ### 3.2179 ### (0%) | ### 2.554 ### (0%) | 3.3167 (0%) | 3.5201 (0%) | 8.3674 (0%) | 5.214 (0%) | 8.225 (0%) | 12.8 (0%) |
|  | ### 7 OFDMs ### Gain | ### 3.9581 ### (+23%) | ### 2.9626 ### (+16%) | 4.0132 (+21%) | 4.2945 (+22%) | 7.0286 (-16%) | 4.484 (-14%) | 7.1558 (-13%) | 11.2642 (-12%) |

Table 2 simulation result for different resource utilizations

Appendix1-2: Different file sizes

| ### File size | ### TTI length | ### Mean UPT(Mbps) | ### 5% UPT(Mbps) | ### 50% UPT(Mbps) | ### 95% UPT(Mbps) | ### Mean latency(s) | ### 5% latency(s) | ### 50% latency(s) | ### 95% latency(s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ### 100Kbit | ### 14OFDMs ### Gain | ### 1.6324 ### (0%) | ### 1.5171 ### (0%) | 1.6115 (0%) | 1.8409 (0%) | 3.2496 (0%) | 2.673 (0%) | 3.245 (0%) | 3.708 (0%) |
|  | ### 7OFDMs ### Gain | ### 2.0405 ### (+25%) | ### 1.8964 ### (+25%) | 2.0627 (+28%) | 2.1399 (+16%) | 2.8272 (-13%) | 2.3522 (-12%) | 2.8232 (-13%) | 3.115 (-16%) |
|  | ### 2OFDMs ### Gain | ### 2.3017 ### (+41%) | ### 1.9267 ### (+27%) | 2.1433 (+33%) | 2.3748 (+29%) | 2.7297 (-16%) | 2.3255 (-13%) | 2.7258 (-16%) | 3.078 (-17%) |
|  | ### 1OFDM ### Gain | ### 2.4486 ### (+50%) | ### 2.0435 ### (+34.7%) | 2.2078 (+37%) | 2.3932 (+30%) | 2.6322 (-19%) | 2.2988 (-14%) | 2.6784 (-17%) | 2.9664 (-20%) |
| ### 100Mbyte | ### 14OFDMs ### Gain | ### 3.2019 ### (0%) | ### 2.9566 ### (0%) | 3.2292 (0%) | 3.4139 (0%) | 4.0408 (0%) | 3.568 (0%) | 4.029 (0%) | 5.372 (0%) |
|  | ### 7OFDMs ### Gain | ### 3.87 ### (+21%) | ### 3.4297 ### (+16%) | 3.9024 (+21%) | 4.0551 (+19%) | 3.4309 (-15%) | 2.949 (-17%) | 3.3844 (-16%) | 4.5662 (-15%) |
|  | ### 2OFDMs ### Gain | ### 4.1944 ### (+31%) | ### 3.3410 ### (+13%) | 4.295 (+33%) | 4.4381 (+30%) | 3.273 (-19%) | 2.64 (-26%) | 3.1829 (-21%) | 4.6736 (-13%) |
|  | ### 1OFDM ### Gain | ### 4.3546 ### (+36%) | ### 3.6666 ### (+24%) | 4.4563 (+38%) | 4.6088 (+35%) | 3.2326 (-20%) | 2.569 (-28%) | 3.1426 (-22%) | 4.4588 (-17%) |
| ### 500Mbyte | ### 14OFDMs ### Gain | ### 3.2179 ### (0%) | ### 2.554 ### (0%) | 3.3167 (0%) | 3.5201 (0%) | 8.3674 (0%) | 5.214 (0%) | 8.225 (0%) | 12.8 (0%) |
|  | ### 7OFDMs ### Gain | ### 3.9581 ### (+23%) | ### 2.9626 ### (+16%) | 4.0132 (+21%) | 4.2945 (+22%) | 7.0286 (-16%) | 4.484 (-14%) | 7.1558 (-13%) | 11.2642 (-12%) |
|  | ### 2OFDMs ### Gain | ### 4.1832 ### (+30%) | ### 2.6562 ### (+4%) | 4.444 (+34%) | 4.8929 (+39%) | 6.9449 (-17%) | 3.2849 (-37%) | 6.087 (-26%) | 12.672 (-1%) |
|  | ### 1OFDM ### Gain | ### 4.312 ### (+34%) | ### 2.7328 ### (+7%) | 4.6102 (+39%) | 4.9633 (+41%) | 6.7776 (-19%) | 3.2327 (-38%) | 5.922 (-28%) | 12.544 (-2%) |

Table 3 simulation result for different file sizes

# Appendix2: Simulation parameters

Appendix2-1: General parameters

| Parameter | Assumptions |
| --- | --- |
| Layout | 7 macro eNB sites, 3 sectors per site |
| System bandwidth per carrier | 10MHz |
| Carrier frequency | 2GHz |
| Inter-site distance | 500m |
| Total BS TX power (Ptotal per carrier) | 46dBm |
| TTI length | 7 symbols |
| RS and control signaling overhead | 7OFDM symbols: 21% overhead 1 or 2OFDM symbols: 30% overhead |
| TBS determination | Scalable with TTI length as baseline |
| HARQ RTT | 8 TTIs |
| Scheduler | Proportional fairness |
| Distance-dependent path loss | ITU UMa[referring to Table B.1.2.1-1 in TR36.814], with 3D distance between an eNB and a UE |
| Penetration | For indoor UEs:20dB |
| Shadowing | ITU UMa |
| Antenna pattern | 3D |
| Antenna Height | 25m |
| UE antenna Height | 1.5m |
| Antenna gain + connector loss | 17 dBi |
| Antenna gain of UE | 0 dBi |
| Fast fading channel between eNB and UE | ITU UMa |
| Antenna configuration | 2Tx(eNB) Cross-polarized 2Rx(UE), Cross-polarized |
| Number of UEs | 10 UEs per macro cell |
| UE dropping | Randomly and uniformly dropped throughout the macro geographical area |
| CSI report period | 5 TTIs |
| CSI report delay | 6TTIs |
| UE receiver | MMSE-IRC |
| eNB noise figure | 5dB |
| UL antenna configuration | 2Rx(eNB), 1Tx(UE) |
| UE noise figure | 9dB |
| UE speed | 3Km/h |
| Duplex mode | FDD |
| Network synchronization | Synchronized |
| Performance metrics | Mean, 5%, 50% and 95% user perceived throughput Mean, 5%, 50% and 95% user packet delay |

Table 4 General parameters

Appendix2-2: TCP parameters

| Parameter | Assumptions |
| --- | --- |
| TCP models | - SSThresh 65535 Bytes - Initial window size 1460 Bytes - Max segment size 1460 Bytes - 40 Bytes TCP header + IP header are added to the initial window size and max segment size。 - The three way handshake is not modelled as baseline. |
| CN delay | 6ms |
| TCP ACK Delay | 0ms |

Table 5 TCP parameters