3GPP TSG RAN WG1 Meeting #84                                                           	                                 R1-161237

St Julian’s, Malta, 15th – 19th February 2016

Source:	Panasonic

Title: 	WF on encoding method of MCS/TBS values for NB-PDSCH

Agenda Item:		7.1.1.1.3

Document for:	Discussion

Background

Following was captured in chairman note. This document address the topic.

Proposal (further offline discussion – Suzuki): - The set of MCS/TBS values for NB-PDSCH are indicated by a 3-bit information field in DCI - FFS the set of MCS values

Discussion

MCS/TBS indication is closely related to how coding chain is realized and how resource assignment are realized. We assume following:

- Define a Rate Matched Block

  - Rate matching is realized to the size corresponding to a Rate Matched Block

  - Rate Matched Block is indicated (explicit or implicit) by NB-PDCCCH.

  - Rate Matched Block can be further repeated and the number of repetition is indicated in NB-PDCCH.

    - The number of repetition is a power of 2

We identified following options for the indication by NB-PDCCH.

|  | Option1 | Option 2 | Option 3 | Option4 |
| --- | --- | --- | --- | --- |
| Starting timing of PDSCH | separately indicated | jointly indicated | separately indicated | separately indicated |
| Rate Matched Block size | separately indicated |  | jointly indicated | separately indicated |
| MCS | separately indicated | separately indicated |  | jointly indicated |
| The number of repetition of Rate Matched Block | separately indicated | separately indicated | separately indicated |  |

Property of different options can be following.

- Option 1 is all information is separately encoded.

- Option 2 is similar to eMTC method of the resource allocation type 2 (starting timing and PRB positions) are converted to time domain.

- Option 3 allows to remove redundant combinations of rate matched size and MCS when the number of repetition is separately indicated. i.e. rate matched block size "2*n" without repetition and rate matched block size "n" with 2 times repetition.

- Option 4 allows to removes redundant combinations of MCS and repetitions. The higher coding rate with the repetition is not required.

- The combination of option 2 and option 4 are possible.

As an example, we calculate the overhead using following conditions.

- Starting timing of PDSCH can be 8 possible timings

- Rate matched block size can be up to 8 subframes

- MCS can indicate all current entries for QPSK i.e. ITBS 0 to 9

- Repetition can be 4 possible values.

Then calculation results is following.

|  | Option1 | Option1b | Option 2 | Option 3a | Option 3b | Option 4 |
| --- | --- | --- | --- | --- | --- | --- |
| Starting timing of PDSCH | 3 bits | 3 bits | 6 bits | 3 bits | 3 bits | 3 bits |
| Rate Matched Block size | 3 bits | 3 bits |  | 6 bits | 5 bits | 3 bits |
| MCS | 4 bits | 3 bits | 4 bits |  |  | 5 bits |
| The number of repetition of Rate Matched Block | 2 bits | 2 bits | 2 bits | 2 bits | 2 bits |  |
| sum | 12 bits | 11 bits | 12 bits | 11 bits | 10 bits | 11 bits |

Option 1 is simply to allow 10 entries. Option 1b is to reduce MCS size to 3 bits like proposed in the screen. Possible supported TBS entries could be like following as example.

|  | = Rate matched block size |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 0 | 16 | 32 | 56 | 88 | 120 | 152 | 176 | 208 |
| 1 | 24 | 56 | 88 | 144 | 176 | 208 | 224 | 256 |
| 2 | 32 | 72 | 144 | 176 | 208 | 256 | 296 | 328 |
| 3 | 40 | 104 | 176 | 208 | 256 | 328 | 392 | 440 |
| 4 | 56 | 120 | 208 | 256 | 328 | 408 | 488 | 552 |
| 5 | 72 | 144 | 224 | 328 | 424 | 504 | 600 | 680 |
| 6 | 88328 | 176 | 256 | 392 | 504 | 600 | 712 | 808 |
| 7 | 104 | 224 | 328 | 472 | 584 | 712 | 840 | 968 |
| 8 | 120 | 256 | 392 | 536 | 680 | 808 | 968 | 1096 |
| 9 | 136 | 296 | 456 | 616 | 776 | 936 | 1096 | 1256 |

On the option 3a, we removed the entries expressed by red colors.

|  | = Rate matched block size |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 0 | 16 | 32 | 56 | 88 | 120 | 152 | 176 | 208 |
| 1 | 24 | 56 | 88 | 144 | 176 | 208 | 224 | 256 |
| 2 | 32 | 72 | 144 | 176 | 208 | 256 | 296 | 328 |
| 3 | 40 | 104 | 176 | 208 | 256 | 328 | 392 | 440 |
| 4 | 56 | 120 | 208 | 256 | 328 | 408 | 488 | 552 |
| 5 | 72 | 144 | 224 | 328 | 424 | 504 | 600 | 680 |
| 6 | 88328 | 176 | 256 | 392 | 504 | 600 | 712 | 808 |
| 7 | 104 | 224 | 328 | 472 | 584 | 712 | 840 | 968 |
| 8 | 120 | 256 | 392 | 536 | 680 | 808 | 968 | 1096 |
| 9 | 136 | 296 | 456 | 616 | 776 | 936 | 1096 | 1256 |

On the option 3b, we removed the entries expressed by red colors.

|  | = Rate matched block size |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 0 | 16 | 32 | 56 | 88 | 120 | 152 | 176 | 208 |
| 1 | 24 | 56 | 88 | 144 | 176 | 208 | 224 | 256 |
| 2 | 32 | 72 | 144 | 176 | 208 | 256 | 296 | 328 |
| 3 | 40 | 104 | 176 | 208 | 256 | 328 | 392 | 440 |
| 4 | 56 | 120 | 208 | 256 | 328 | 408 | 488 | 552 |
| 5 | 72 | 144 | 224 | 328 | 424 | 504 | 600 | 680 |
| 6 | 88328 | 176 | 256 | 392 | 504 | 600 | 712 | 808 |
| 7 | 104 | 224 | 328 | 472 | 584 | 712 | 840 | 968 |
| 8 | 120 | 256 | 392 | 536 | 680 | 808 | 968 | 1096 |
| 9 | 136 | 296 | 456 | 616 | 776 | 936 | 1096 | 1256 |

On option 4, we calculated based on the following.

- The minimum number of the repetitions support all 10 MCS entries. For the case of more repetitions, only 5 MCS entries are supported. Then 25 entries are used.

Proposal

We evaluated different options of joint encoding.

- The amount of bit-width saving by the joint coding is the order of 1-2 bits. There may not be so worth to trying to optimize joint encoding from bit saving perspective.

- On the other hand, what TBS is supported in what conditions could be different depending on what coding is used. This aspect would be worth to further discuss it.