Source: 	MCC Support

Title:	Draft Report of 3GPP TSG RAN WG1 #84bis v0.2.0

(Busan, Korea, 11th – 15th April 2016)

Document for:	Comments

Table of contents

1	Opening of the meeting	7

1.1	Call for IPR	7

1.2	Competition law statement	7

1.3	Network usage conditions	7

2	Approval of Agenda	7

3	Approval of Minutes from previous meetings	8

4	Highlights from RAN Plenary	8

5	Incoming Liaison Statements	8

6	UTRA	11

6.1	Maintenance of UTRA Releases 4 – 13	11

6.1.1	FDD	11

6.1.2	TDD	11

6.2	Multi-Carrier enhancements for UMTS - WID in RP-152290.	11

6.2.1	Performance evaluation	11

6.2.2	Impact on current mechanisms	12

6.2.3	Other	12

6.3	DTX/DRX enhancements in CELL_FACH - WID in RP-160601.	12

6.3.1	DRX improvements	12

6.3.2	DTX improvements	12

7	E-UTRA	12

7.1	Maintenance of E-UTRA Releases 8 – 13	12

7.1.1	Maintenance of E-UTRA Release 8 – 12	13

7.1.2	Maintenance of Release 13 LTE carrier aggregation enhancement beyond 5 carriers	13

7.1.3	Maintenance of Release 13 licensed-assisted access to unlicensed spectrum	15

7.1.4	Maintenance of Release 13 elevation beamforming/full-dimension MIMO for LTE	19

7.1.5	Maintenance of Release 13 further LTE physical layer enhancements for MTC	21

7.1.6	Other	27

7.2	LTE Release 13	27

7.2.1	NB-IoT - WID in RP-152284.	27

7.2.1.1	Downlink physical channels and signals	29

7.2.1.1.1	NB-PDCCH	29

7.2.1.1.2	NB-PDSCH	30

7.2.1.1.3	System information and paging transmission	32

7.2.1.1.4	NB-PSS and NB-SSS	32

7.2.1.1.5	Other	34

7.2.1.2	Uplink physical channels and signals	34

7.2.1.2.1	NB-PUSCH	34

7.2.1.2.2	Uplink narrowband DM-RS	35

7.2.1.2.3	Uplink control information	37

7.2.1.2.4	Random access	38

7.2.1.2.5	Other	39

7.2.1.3	Other	40

7.3	LTE Release 14	41

7.3.1	Enhanced Licensed-Assisted Access to Unlicensed Spectrum - WID in RP-152272.	41

7.3.1.1	PUSCH design	41

7.3.1.2	SRS design	44

7.3.1.3	Uplink control information on an LAA SCell	45

7.3.1.4	PRACH design	47

7.3.1.5	Channel access framework	48

7.3.1.6	Other	49

7.3.2	Support for V2V services based on LTE sidelink - WID in RP-160649	49

7.3.2.1	Enhancement to sidelink physical layer structure	49

7.3.2.1.1	DM-RS for PSCCH and PSSCH	49

7.3.2.1.2	DM-RS for PSBCH	51

7.3.2.1.3	Other	51

7.3.2.2	Enhancement to sidelink resource allocation	51

7.3.2.2.1	Details of sensing with semi-persistent transmission	51

7.3.2.2.2	Signaling to/from eNB	53

7.3.2.2.3	Resource pool design	54

7.3.2.2.4	Scheduling assignment	55

7.3.2.2.5	Other	56

7.3.2.3	Coexistence of PC5-based V2V operation and legacy Uu operation	56

7.3.2.4	Enhancement to sidelink synchronization procedure	56

7.3.2.5	Other	58

7.3.3	Enhancements on Full-Dimension (FD) MIMO for LTE - WID in RP-160623	58

7.3.3.1	Reference Signals	58

7.3.3.1.1	Non-precoded CSI-RS	58

7.3.3.1.2	Beamformed CSI-RS	59

7.3.3.1.3	Uplink DM-RS	60

7.3.3.2	CSI Reporting	61

7.3.3.2.1	Non-precoded CSI-RS codebooks	61

7.3.3.2.2	CSI reporting for hybrid CSI-RS	61

7.3.3.2.3	Advanced CSI reporting and interference measurement	62

7.3.3.3	DMRS-based Open-Loop and Semi-Open-LoopTransmission	63

7.3.4	Downlink Multiuser Superposition Transmission for LTE - WID in RP-160680	64

7.3.4.1	Superposition transmissions	64

7.3.4.2	Mechanisms for efficient operation	65

7.3.4.3	Evaluation for MUST using up to 8Tx	66

7.3.4.4	Other	66

7.3.5	eMBMS enhancements for LTE - WID in RP-160675	67

7.3.5.1	Longer cyclic prefix for MBSFN subframe	67

7.3.5.2	Increased number of MBSFN subframes per radio frame	68

7.3.5.3	Study support for standalone eMBMS carrier	68

7.3.5.4	Study TV transport service of UEs not being authenticated	68

7.3.5.5	Other	68

7.3.6	SRS switching between LTE component carriers - WID in RP-160676	68

7.3.6.1	General Operation of SRS Carrier Based Switching	68

7.3.6.2	Specification Impacts to Support SRS Carrier based Switching	69

7.3.6.3	Other	70

7.3.7	Further Indoor Positioning enhancements for UTRA and LTE - WID in RP-160538	70

7.3.7.1	OTDOA Enhancements	70

7.3.7.2	PRS-based Beacons	70

7.3.7.3	E-CID Enhancements	70

7.3.7.4	Other	70

7.3.8	Uplink Capacity Enhancements for LTE - WID in RP-160664	70

7.3.9	Study on LTE-based V2X Services - SID in RP-160657	72

7.3.9.1	Evaluation results	72

7.3.9.1.1	Uu transport for V2V, V2P, and V2I/N	72

7.3.9.1.2	PC5 transport for V2P and V2I/N	73

7.3.9.2	Support of Uu transport for V2V, and PC5/Uu transport for V2I/N and V2P	73

7.3.9.2.1	Enhancement to multi-cell multicast/broadcast	73

7.3.9.2.2	Enhancements to unicast	74

7.3.9.2.3	Enhancements to sidelink	74

7.3.9.2.4	Other	74

7.3.9.3	Identification of high level coexistence approaches (long-term basis) between PC5 transport for V2V services and DSRC/IEEE 802.11p services in the same channel	75

7.3.10	Study on Latency reduction techniques - SID in RP-150465	75

7.3.10.1	Evaluation results	76

7.3.10.2	Channel design for shortened TTI	78

7.3.10.3	Processing time reduction and related procedures	81

7.3.10.4	Other	82

7.3.11	Other	83

8	NR	83

8.1	Study on New Radio Access Technology	83

8.1.1	Overview of new radio interface	83

8.1.2	Forward compatibility for new radio interface	90

8.1.3	Evaluation assumptions for new radio interface	92

8.1.4	Waveform and multiple access schemes for new radio interface	98

8.1.4.1	Waveform	98

8.1.4.2	Multiple access scheme	100

8.1.5	Numerology and frame structure for new radio interface	102

8.1.6	Channel coding and modulation for new radio interface	108

8.1.6.1	Channel coding	108

8.1.6.2	Modulation	111

8.1.7	Other	111

8.2	Study on channel model for frequency spectrum above 6 GHz - SID in RP-151606	112

8.2.1	Remaining details on large scale modelling for >6GHz	113

8.2.2	Remaining details on fast fading modelling for >6 GHz	118

8.2.3	Remaining details on modelling of additional features for >6GHz channel model	122

8.2.3.1	Remaining details on blockage modelling	122

8.2.3.2	Remaining details on spatial consistency modelling	124

8.2.3.3	Remaining details on large antenna array modelling	127

8.2.3.4	Remaining details on modelling of other additional features	128

8.2.4	Remaining details on link-level channel models	128

8.2.5	Channel modelling calibrations	130

8.2.5.1	Calibration scenarios and assumptions	130

8.2.5.2	Metrics for channel model calibrations	133

8.2.6	Consistent extension of the additional features to < 6GHz	134

8.2.7	Map-based methodology	134

8.2.8	Other	135

9	Closing of the meeting	135

Annex A:	List of Tdocs at RAN1 #84bis	137

Annex B:	List of CRs agreed at RAN1 #84bis	138

Annex C-1:	List of Outgoing LSs from RAN1 #84bis	142

Annex C-2:	List of Incoming LSs from RAN1 #84bis	143

Annex D:	List of Approved updated WIDs	144

Annex E:	List of draft TSs/TRs agreed at RAN1 #84bis	145

Annex F:	List of actions	146

Annex G:	List of participants at RAN1 #84bis	153

Annex H:	TSG RAN WG1 meetings in 2016 – 2017	154

Main facts summary

3GPP TSG WG RAN1 #84 bis meeting, hosted by Samsung, was held in Haeundae Grand Hotel in Busan, Korea.

The meeting started at 9:05 on Monday 11th April and finished at 17:15 on Friday 15th April 2016.

The number of attending delegates, having signed the participants’ paper list, was 379.

The schedule of the week was as follows:

- Monday: Common session on Agenda items 1, 2, 3, 4, 5, 7.1.2 (Maintenance of Release 13 LTE carrier aggregation enhancement beyond 5 carriers), 7.1.3 (Maintenance of Release 13 LAA) and 7.1.4 (Maintenance of Release 13 EB/FD-MIMO) followed by Study on Latency reduction techniques (AI 7.3.10.1 on Evaluation results, AI 7.3.10.2 on Channel design for shortened TTI)

- Monday afternoon (LTE)

  - Session on NB-IoT (AI 7.2.1) chaired by Matthew Baker.

- Tuesday (NR): chaired by Satoshi Nagata

- Tuesday (LTE session1): chaired by Matthew Baker

  - eMBMS enhancements for LTE (AI 7.3.5), SRS Carrier Based Switching for LTE (AI 7.3.6) and Enhancements on Full-Dimension (FD) MIMO for LTE (AI 7.3.3).

- Tuesday (LTE session2): chaired by Wanshi Chen

  - Rel-13 eMTC CRs (and possibly other CRs (e.g., LAA, MIMO, eCA), Further Indoor Positioning enhancements for UTRA and LTE (AI 7.3.7), Enhanced Licensed-Assisted Access Using LTE (AI 7.3.1) and Support for V2V services based on LTE sidelink (AI 7.3.2).

- Tuesday (LTE session3):

  - Study on channel model for frequency spectrum above 6 GHz (AI 8.2) chaired by Tuomo Säynäjäkangas from Nokia followed by session on NB-IoT (AI 7.2.1) chaired by Brian Classon from Huawei.

- Tuesday (HSPA): Multi-Carrier enhancements for UMTS (AI 6.2) chaired by Carmela Cozzo from Huawei.

- Wednesday (NR): chaired by Satoshi Nagata

- Wednesday (LTE session1): chaired by Matthew Baker

  - Rel-13 CRs Maintenance (AI 7.1), Uplink Capacity Enhancements for LTE (AI 7.3.8), Study on Latency reduction techniques (AI 7.3.10) and Enhancements on Full-Dimension (FD) MIMO for LTE (AI 7.3.3).

- Wednesday (LTE session2): chaired by Wanshi Chen

  - Support for V2V services based on LTE sidelink (AI 7.3.2), Study on LTE-based V2X Services (AI 7.3.9) and Enhanced Licensed-Assisted Access Using LTE (AI 7.3.1).

- Wednesday (LTE session3):

  - Study on channel model for frequency spectrum above 6 GHz (AI 8.2) chaired by Tuomo Säynäjäkangas from Nokia followed by session on NB-IoT (AI 7.2.1) chaired by Brian Classon from Huawei.

- Wednesday (HSPA): DTX/DRX enhancements in CELL_FACH (AI 6.3) chaired by Florent Munier from Ericsson followed by Maintenance.

- Thursday (NR): chaired by Satoshi Nagata

- Thursday (LTE session1): chaired by Matthew Baker

  - Rel-13 CRs Maintenance (AI 7.1), Enhanced Licensed-Assisted Access Using LTE (AI 7.3.1), Study on Latency reduction techniques (AI 7.3.10), Enhancements on Full-Dimension (FD) MIMO for LTE (AI 7.3.3) and Downlink Multiuser Superposition Transmission for LTE (AI 7.3.4).

- Thursday (LTE session2): chaired by Wanshi Chen

  - Rel-13 eMTC CRs (AI 7.1.5), Downlink Multiuser Superposition Transmission for LTE (AI 7.3.4), Study on LTE-based V2X Services (AI 7.3.9), Enhanced Licensed-Assisted Access Using LTE (AI 7.3.1) and Support for V2V services based on LTE sidelink (AI 7.3.2).

- Thursday (LTE session3):

  - Study on channel model for frequency spectrum above 6 GHz (AI 8.2) chaired by Tuomo Säynäjäkangas from Nokia and session on NB-IoT (AI 7.2.1) chaired by Brian Classon from Huawei.

- Friday: Revisions. Continued offline in the morning (eLAA,…).

The list of action points that required RAN1 close follow-up is listed in Annex F (end of document).

The number of contribution documents for this meeting was xxx, and those documents were categorized as followed.

| Agenda Item | Input Document | Discussed Document |
| --- | --- | --- |

Note: The amount of documents includes those discussed during the email discussion session post meeting.

# Opening of the meeting

Mr Satoshi Nagata (RAN1 Chairman) welcomed the participants of the RAN WG1 84 bis meeting and opened the meeting at 09:00.

Mr Younsun Kim welcomed the delegates on behalf of Samsung and detailed the domestic arrangements for the full week. The host kindly offered buffet lunch to all RAN1 delegates.

## Call for IPR

The attention of the members of this Technical Specification Group was drawn to the fact that 3GPP Individual Members have the obligation under the IPR Policies of their respective Organizational Partners to inform their respective Organizational Partners of Essential IPRs they become aware of.

The members take note that they are hereby invited:

-	to investigate whether their organization or any other organization owns IPRs which were, or were likely to become Essential in respect of the work of 3GPP.

-	to notify their respective Organizational Partners of all potential IPRs, e.g., for ETSI, by means of the IPR Information Statement and the Licensing declaration forms (e.g. see the ETSI IPR forms http://webapp.etsi.org/Ipr/).

## Competition law statement

The Chairman also drew Member’s attention to the fact that 3GPP activities are subject to antitrust and competition laws and that compliance with said laws is therefore required of any participant of this WG meeting including the Chairman and Vice Chairmen. In case of question, please contact your legal counsel.

The present meeting will be conducted with strict impartiality and in the interests of 3GPP.

Furthermore, the Chairman reminded Members that timely submission of work items/contributions in advance of WG meetings is important to allow for full and fair consideration of such matters.

## Network usage conditions

The PCG has laid down the following network usage conditions:

Users shall not use the network to engage in illegal activities. This includes activities such as copyright violation, hacking, espionage or any other activity that may be prohibited by local laws. Users shall not engage in non-work related activities that consume excessive bandwidth or cause significant degradation of the performance of the network. Since the network is a shared resource, users should exercise some basic etiquette when using the 3GPP network at a meeting. It is understood that high bandwidth applications such as downloading large files or video streaming might be required for business purposes, but delegates should be strongly discouraged in performing these activities for personal use. Downloading a movie or doing something in an interactive environment for personal use essentially wastes bandwidth that others need to make the meeting effective. The meeting chairman should remind end users that the network is a shared resource; the more one user grabs, the less there is for another. Email and its attachments already take up significant bandwidth (certain email programs are not very bandwidth efficient). In case of need the chair can ask the delegates to restrict IT usage to things that are essential for the meeting itself. 1. DON’T place your WiFi device in ad-hoc mode 2. DON’T set up a personal hotspot in the meeting room 3. DO try 802.11a if your WiFi device supports it 4. DON’T manually allocate an IP address 5. DON’T be a bandwidth hog by streaming video, playing online games, or downloading huge files 6. DON’T use packet probing software which clogs the local network (e.g., packet sniffers or port scanners)

# Approval of Agenda

R1-162141	Draft Agenda of RAN1#84bis meeting	RAN1 Chair

Satoshi Nagata (Chairman) proposed the agenda for the meeting, as well the schedule of the week.

Discussion: Fujitsu  for NR and NB-IoT, more details of the topics for discussion might be helpful

Panasonic who will be chairing the different sessions? NB-IoT (Monday) chaired by Matthew Baker, NB-IoT (Tuesday, Wed) chaired by Brian Classon – all NR sessions will be chaired by RAN1 chair – all others topics be split betwwen both RAN1 Vice Chairmen.

Mr Chair highlighted the difficulty to satisfy every one wishes while preparing the schedule of the week

ZTE sees an issue having certain topics discussed in // between channel model and NR sessions Chair: schecule can be revisited during the week based on the progress.

Decision: The agenda is approved.

# Approval of Minutes from previous meetings

R1-163406	Final Report of 3GPP TSG RAN WG1 #84 v2.0.0	ETSI	(R1-163378)

The document was presented by Patrick Mérias (ETSI Mobile Competence Center) and provides the outcomes of last meeting in Malta.

Discussion: MCC report is revised to correct a formating issue in the conclusion of the email thread [84-11].

MCC apologized for having forgotten two agreed CRs in the LTE_EBF_FDMIMO CR pack to RAN#71. Both CRs are resubmitted at this meeting (see below) and will go for approval by next plenarybefore inclusion into the specs.

Decision: The document is approved.

R1-162341	CR on CSI-RS transmission in DwPTS	Samsung	36.211 CR0216r2

Decision: The document is agreed.

R1-162342	Correction to the UE's assumption on DMRS ports	ZTE	36.213 CR0593r1

Decision: The document is agreed.

R1-163379	Final Report of 3GPP TSG RAN WG1 #2AH_NB-IoT v1.0.0	ETSI

The document was presented by Patrick Mérias (ETSI Mobile Competence Center) and provides the outcomes of last meeting in Anaheim.

Discussion: No comment.

Decision: The document is approved.

R1-162190	Draft Report of 3GPP TSG RAN WG1 #AH Channel Model	Nokia

The document was presented by Tuomo Säynäjäkangas from Nokia and provides the outcomes of the ad-hoc meeting in Ljubljana.

Discussion: No comment.

Decision: The document is approved.

# Highlights from RAN Plenary

R1-163377	Selected highlights from RAN#71 of relevance to RAN1	RAN1 Chair

The document was presented by RAN1 Chair and shows the main decisions from RAN#71 impacting RAN1 future work.

Decision: The document is noted.

# Incoming Liaison Statements

Dual connectivity related

R1-163370	Response LS on maximum UL Transmission timing difference in dual connectivity	RAN2, Huawei

The document was presented by RAN1 Chair on behalf of Huawei and suggests RAN4 to consider the following 2 scenarii simultaneously:

- Scenario 1 (addressed in TS 36.331): the pTAG in SCG is impacted due to exceeding the maximum uplink transmission timing difference between MCG and SCG when powerControlMode is configured to 1;

- Scenario 2 (addressed in TS36.321): only the SCell of sTAG is impacted due to exceeding the maximum uplink transmission timing difference between MCG and SCG when powerControlMode is configured to 1;

Discussion: No action to RAN1.

Decision: The document is noted.

R1-163372	Reply LS on Pcmax definition of asynchronous overlapping transmissions in DC	RAN4, InterDigital

The document was presented by Paul Marinier from InterDigital and provides the following answers to R1-153460.

- The statement ”power control changes are not allowed for one channel on one carrier in the middle of subframe” is correct and this is the RAN4 understanding as well, as the legacy Pcmax,c per carrier/per CG is applicable in RAN4 DC related specification.

- The Pcmax of a non-reference subframe does not change at the border of 2 reference subframes.

Decision: The document is noted.

LAA related

R1-163375	Liaison Statement to 3GPP from IEEE 802 LMSC	IEEE 802 LMSC

The document was presented by RAN1 Chair on behalf of IEEE 802 and is an LS containing comments related to the LAA specification prepared by the 802.19 Coexistence Working Group. The document contains comments by participants in IEEE 802, particularly focused on CR_R1_157922 [36.213] and CR_RP_152258 [36.211] in the interest of improving the performance of all systems, including LAA, in unlicensed spectrum.

IEEE 802 believes this process of collaboration will result in an LAA specification that better supports fair sharing of unlicensed spectrum by both 802.11 and LAA equipment. Further, it will also result in a better LAA specification that leverages the long experience of IEEE 802 in defining effective and fair sharing protocols in unlicensed spectrum.

Discussion: Qualcomm is preparing a draft reply in R1-163413 (not yet ready).

Decision: The document is noted. Continue offline and contact Qualcomm – revisit by Friday

Friday

R1-163413	Draft response LS to 802dot11 regarding LAA	Qualcomm Incorporated

The document was presented by Peter Gaal from Qualcomm.

Discussion: .More offline comments provided by Huawei and Broadcom… Qualcomm:  they have not been captured yet.

Just before closing the meeting, Broadcom raised the issue that some of the questions may be more appropriate to RAN4 and one particular to RAN2. Qualcomm does not agree and believe RAN1 can (should) cover all of them.

Decision: The document is noted. Reply LS will be discussed at the next meeting.

R1-163388	Liaison Statement regarding LAA	Wi-Fi Alliance Coexistence Task Group

The document was presented by RAN1 Chair on behalf of WFA and provides some comments on the LAA specification aimed at ensuring good coexistence between LAA and Wi-Fi. In addition, Wi-Fi Alliance highlights and provides complementary analysis on some of the comments contained in [R1-163375], which it believes are important to help ensure fair sharing and efficient use of unlicensed spectrum by LAA and Wi-Fi devices.

Discussion: Similar to previous LS, Qualcomm is preparing a draft reply in R1-163414 (not yet available).

Decision: The document is noted. Continue offline and contact Qualcomm – revisit by Friday

Friday

R1-163414	Draft response LS to Wi-Fi Alliance regarding LAA	Qualcomm Incorporated

The document was presented by Peter Gaal from Qualcomm.

Discussion: Qualcomm still need to consider some additional comments posted on the email refelctor.

Decision: The document is noted. Reply LS will be discussed at the next meeting.

eMTC related

R1-163369	Response LS on Paging for MTC	RAN2, Ericsson

The document was presented by Johan Bergman from Ericsson and reports the RAN2 agreements to introduce

- new UE category M1 (corresponding to the Rel-13 low complexity MTC capability), and

- new UE capabilities for CE Mode A and CE Mode B (corresponding to the Enhanced Coverage capability)

into existing Inter-node RRC message UERadioPagingInformation (TS36.331).

- a new Inter-node RRC message UEPagingCoverageInformation into TS36.331, for the purpose to upload enhanced coverage information from eNB to EPS (MME), and to be downloaded to eNB(s) at paging.

Discussion: No action to RAN1.

Decision: The document is noted.

V2X related

R1-163373	LS on V2V Requirements	RAN, Qualcomm, LG Electronics

The document was presented by Hanbyul Seo from LGE and asks RAN1 to take the potential requirements in [ETSI EN 302 571, “Intelligent Transport Systems (ITS)”], in particular the PSD limitation (assuming approximately 23dBm maximum Tx power UE, but not precluding the addition of higher Tx power UE in future), into consideration.

Discussion: Intel  what “future” means here? LGE: 23dBm maximum Tx power UE is already captured in the WID

Decision: The document is noted.

R1-163374	Reply LS on clarification of RSU types	SA1, LG Electronics

The document was presented by Hanbyul Seo from LGE and confirms that there are two types of Road Side Unit defined as follows in TS 22.185.

Road Side Unit: A stationary infrastructure entity supporting V2X applications that can exchange messages with other entities supporting V2X applications.

Note: RSU is a term frequently used in existing ITS specifications, and the reason for introducing the term in the 3GPP specifications is to make the documents easier to read for the ITS industry. RSU is a logical entity that combines V2X application logic with the functionality of an eNB (referred to as eNB-type RSU) or UE (referred to as UE-type RSU)

Decision: The document is noted.

PRACH enhancement in high speed scenario

R1-163371	LS on A New PRACH Cyclic Shift Restriction Set	RAN4, Nokia

The document was presented by Jari Lindholm from Nokia and asks RAN1to investigate whether the new restricted set of cyclic shifts for extra high speed cells proposed at Section 6.5.3.1 in [TS 36.878] and the solution proposed in [R4-152604] are suitable for supporting uplink Doppler offset, at least, up to +/-2.5KHz, and if the proposed new restricted set of cyclic shifts for extra high speed cells is a suitable solution, RAN4 would ask RAN1 to consider introducing the new restricted set into TS 36.211 in Rel-14.

Decision: The document is noted.

R1-162619	PRACH enhancement in high speed scenario	Huawei, HiSilicon

The document was presented by Guo Zhiheng from Huawei and is summarized as follows:

- Observation 1: Solution 1 (only use the root sequence without cyclic shifts) requires no UE enhancement and has no specification impact with the drawback that the number of available preamble sequence is very limited.

- Observation 2: highSpeedFlag should only be used for the unrestricted set or restricted set selection.

- Observation 3: Solution 2 (define new restricted set of cyclic shift) needs UE enhancement with the advantage that the root sequence with cyclic shift could be used as random access preamble.

- Observation 4: the valid random access sequence number of solutions 2 and 3 (improved new restricted set of cyclic shift) is larger than solution 1 and the number of available random access sequence of solution 3 is almost twice of solution 2.

- Observation 5: the number of available random access sequence of solution 2 is less than solution 1 while.

- Proposal: adopting solution 3 (improved new restricted set of cyclic shift) as PRACH enhancement solution.

Discussion: RAN1 Chair: what does this proposal aim to? Huawei to open the technical discussion.

Decision: The document is noted.

R1-162766	Considerations for PRACH options related to LS R4-161254	Ericsson

The document was presented by Daniel Larsson from Ericsson and shows an analysis of the different options in the LS [R1-163371] by comparison with options available using current specification. It shows that we can reach similar maximum Doppler, 3250 Hz, which corresponds to 650 km/h assuming a carrier frequency of 2.7 GHz in two ways. The first option is to add two additional hypotheses to get three in total. This has the advantage of being backwards compatible. However the PRACH capacity will suffer due to the risk of the BS sending the wrong responses, even if this can be mitigated by grouping explained in [R4-158133].

Decision: The document is noted.

R1-163199	DRAFT Reply LS on A New PRACH Cyclic Shift Restriction Set	Nokia

The document was presented by Jari Lindholm from Nokia and states that the current high speed mode of PRACH can be enhanced to support higher UE speeds with a new restricted set of cyclic shifts for extra high speed cells as described in TR 36.878.

Discussion: According to the RAN plenary decision the required RAN1 specification changes will be done during 2nd half of 2016.

Decision: The document is noted.

Rel-14 WI on eMBMS enhancements for LTE

R1-162623	[Draft] LS on RAN plenary eMBMS enhancements for LTE	Huawei, HiSilicon

The document was presented by Weiming Xiao from Huawei and is a follow-up to RAN#71 approval of Rel-14 WI on eMBMS enhancements for LTE [RP-160675], in which the following objective was included:

Specify means of using subframes 0, 4, 5, 9 (FS1) and 0, 1, 5, 6 (FS2) for MBSFN. (RAN2, RAN1)

The non-MBSFN subframes for unicast can only be used as Scell

RAN1 asks RAN to clarify the meaning of the above description” The non-MBSFN subframes for unicast can only be used as Scell”. RAN1 understands it may be used to address the concern of backward compatibility, but still does not understand how this sub-bullet resolves the backward compatibility issue.

Discussion: Ericsson (rapporteur) is fine to delete “The non-MBSFN subframes for unicast can only be used as Scell” but can be done directly in RAN – no need to send an LS

Qualcomm let discuss in RAN1 and agree what to do.

Decision: The document is noted. Will be treated in the eMBMB session.

NR related

R1-163390	Characteristics of terrestrial IMT systems for frequency sharing / interference analysis in the frequency range between 24.25 GHz and 86 GHz	RAN, NTT DOCOMO

The document was presented by RAN1 Chair and is a request for technical support and information relevant to the frequency range (24.25-86 GHz) from ITU-R WP5D group:

- provide the information on IMT-2020 system characteristics between 24.25 GHz and 86 GHz to be used in sharing and compatibility studies.

- WP 5D constraints are for

  - Initial system characteristics by October 2016 meeting of WP 5D

  - Final system characteristics by February 2017 meeting of WP 5D

Discussion: Coordination with RAN4 is needed.

Decision: The document is noted. Rapporteur shall take care of it.

R1-163395	LS on path loss accuracy and sensitivity in 5G channel modeling	New York University Wireless

The document was presented by RAN1 Chair on behalf of NYU WIRELESS - a multi-disciplinary academic research center at New York University in Brooklyn. Upon RAN chair's decision, the attached paper is made available in RAN1 and RAN as it is relevant for the RAN1 "Study on channel model for frequency spectrum above 6 GHz"

Discussion: Content shall be reviewed in channel model session (AI8.2.1).

Decision: The document is noted.

The following LS have been received during the course of the week:

R1-163685	LS to RAN1 on RAN2 agreements related to V2V	RAN2, Panasonic

Decision: The document is treated under AI 7.3.2.

R1-163686	LS on reviewing all LTE features up to Rel-13 for Category M1 UEs	RAN2, NTT DOCOMO

Decision: The document is treated under AI 7.1.5.

R1-163852	LS on Power Headroom report	RAN2, Huawei

Decision: The document is treated under AI 7.2.1.

R1-163936	LS on RRM measurement for NB-IOT	RAN4, Ericsson

Decision: The document (received post meeting) is treated under email thread [84b-19].

Postponed to next meeting.

R1-163896	LS on NB-IoT RRM requirements	RAN2, Huawei

# UTRA

R1-163610	Chairpersons' summary of the UMTS sessions	UMTS Session Chairpersons (Huawei, Ericsson)

The document was presented by Carmela Cozzo from Huawei and Florent Munier from Ericsson.

Decision: The document is endorsed and content is incorporated as below.

## Maintenance of UTRA Releases 4 – 13

### FDD

R1-163197	Clarification on measuring the transmitted code power for the downlink DPCH slot formats #17 and #18	Ericsson

Decision: The document is noted and CR0205 cat F to 25.215 (Rel-12) is agreed.

R1-163198	Clarification on measuring the transmitted code power for the downlink DPCH slot formats #17 and #18	Ericsson

Decision: The document is noted and CR0206 cat A to 25.215 (Rel-13) is agreed.

### TDD

None.

## Multi-Carrier enhancements for UMTS - WID in RP-152290.

### Performance evaluation

R1-162996	Evaluation of 2+10 ms TTI configuration in DB-DC-HSUPA	Huawei, HiSilicon

Decision: The document is noted.

R1-162997	Evaluation of 10+10 ms TTI configuration in DB-DC-HSUPA and DC-HSUPA	Huawei, HiSilicon

Decision: The document is noted.

R1-163192	Evaluation of Multicarrier Enhancements for DC-HSUPA	Ericsson

Decision: The document is noted.

R1-163193	Evaluation of Multicarrier Enhancements for DB-DC HSUPA	Ericsson

Decision: The document is noted.

Conclusions

- Continue evaluation using the link simulation setup used for the presented results

  - Expand the range of power values used in the two carriers:

    - Primary carrier: 0(no data) to 0.2W(no grant limit), with constant step size

  - Expand distance between UE and NodeB: 0 to 0.6Km (10m resolution)

  - Metrics:

    - DC

      - 10 vs. 2+2

      - 10+10 vs. 2+2

    - DB-DC (2+10 means 2ms in the lower band and 10ms in the higher band)

      - 2+10 vs. 2+2

      - 10+10 vs. 2+2

      - 10 vs. 2+2

    - Provide results showing with and without power limitation on the primary carrier

- Control channel overhead in DC and DB-DC cases is not considered.

### Impact on current mechanisms

No contributions touching that topic.

### Other

R1-162998	TR 25.707 v0.1.1 on Multi-Carrier Enhancements for UMTS	Huawei, HiSilicon

Include text proposals agreed in RAN1#84

Decision: The document is endorsed as v0.2.0 in R1-163609 for future updates.

R1-163194	Text proposal: Evaluation of Multicarrier Enhancements	Ericsson

Decision: The document is noted.

## DTX/DRX enhancements in CELL_FACH - WID in RP-160601.

### DRX improvements

R1-162239	Further Discussions on HS-SCCH DRX lite proposal	 Qualcomm Europe Inc.(France)

Decision: The document is noted.

R1-163195	On the DRX enhancements in CELL_FACH 	Ericsson

Decision: The document is noted.

Conclusion on DRX improvements

Qualcomm to double check:

  - reduction in scheduling capability due to power sharing across HS-SCCH codes / scheduling instances

  - redesign of the QC proposed solution in R1-162239 making sure both HS orders as well corresponding RX burst are within the same DRX cycle.

  - Get back on re-using existing IEs for the purpose of 640ms proposal.

### DTX improvements

R1-162240	A dicussion note on EUL Inter-Arrival times in Cell Fach	Qualcomm Europe Inc.(France)

Decision: The document is noted.

R1-162999	Evaluation of DTX in CELL_FACH	Huawei, HiSilicon

Decision: The document is noted.

R1-163000	DTX design in CELL_FACH	Huawei, HiSilicon

Decision: The document is noted.

R1-163196	On evaluating the implicit release of common E-DCH resources in CELL_FACH	Ericsson

Decision: The document is noted.

Conclusions on DTX improvements:

According to the CELL FACH traffic model studied during this work item, it was found that longer timers are impacting the cell capacity and the average blocking more significantly than shorter timers.

Evaluation will continue. Future simulations will include (but are not limited to) shorter timers (less than 80ms).

It is foreseen that DTX can provide gains in two scenarios:

  - DTX during inactivity timer (i.e. prior to timer expiration)

  - DTX during the time interval prior to retransmission

# E-UTRA

## Maintenance of E-UTRA Releases 8 – 13

R1-163837	Chairman's Notes of Agenda Item 7.1 on Maintenance of E-UTRA Releases 8 - 13	Session Chairman (Nokia)

The document was presented by Matthew Baker from Nokia and .provides the outcomes of the remaining maintenance issues (except eMTC)

Decision: The document is endorsed and the content incorporated below.

### Maintenance of E-UTRA Release 8 – 12

None.

### Maintenance of Release 13 LTE carrier aggregation enhancement beyond 5 carriers

R1-163200	Coding of higher layer parameter codebooksizeDetermination-r13	Nokia, Alcatel-Lucent iShanghai Bell

- codebooksizeDetermination-r13 = 0 is replaced with codebooksizeDetermination-r13 = dai and codebooksizeDetermination-r13 = 1 is replaced with codebooksizeDetermination-r13 = cc.

Decision: The document is noted and CR0197 to 36.212 is agreed in R1-163421.

R1-163201	Coding of higher layer parameter codebooksizeDetermination-r13	Nokia, Alcatel-Lucent Shanghai Bell

Decision: The document is noted and CR0622 to 36.213 is agreed in R1-163422.

R1-162104	Discussion on HARQ-ACK ordering in case of semi-static codebook configuration for eCA	Huawei, HiSilicon

The document was presented by Yongxia Lyu from Huawei.

- Observation 1: HARQ-ACK bit ordering with Alternative 2 follows the Rel-12 principle but a new bit ordering is used with Alternative 1.

- Observation 2: HARQ-ACK bit for DL SPS PDSCH without corresponding PDCCH/EPDCCH is captured with Alternative 2 but is omitted with Alternative 1.

- Observation 3: There is no substantial difference of HARQ-ACK feedback performance between Alternative 1 and Alternative 2.

- Proposal 1: For FDD, the description of semi-static HARQ-ACK codebook determination for PUCCH format 4/5 in TS 36.213 is removed and that in TS 36.212 is kept.

- Proposal 2: For TDD, the pseudo code for semi-static HARQ-ACK codebook determination in TS 36.213 is removed and the parameters and are defined for PUCCH format 4/5 in TS 36.213.

Discussion: CR is in R1-162142.

Decision: The document is noted.

R1-162142	Correction on HARQ-ACK ordering in case of semi-static codebook configuration of eCA in 36.213	Huawei, HiSilicon	Rel-13	36.213 - CR0608 F

Decision: The document is noted. It is agreed in principle to use the pseudo code in 36.212 and remove the corresponding part in 36.213. Continue offline discussion – (Huawei)

Monday evening

R1-163514	Correction on HARQ-ACK ordering in case of semi-static codebook configuration of eCA	Huawei, HiSilicon	Rel-13	36.213 - CR0608r1 F

Decision: The document is agreed.

R1-162105	Discussion on SPS HARQ-ACK bit handling in case of dynamic codebook configuration for eCA	Huawei, HiSilicon

The document was presented by Fredrik Berggren from Huawei.

Proposal. If the misalignment of HARQ-ACK codebook size between the eNodeB and the UE for the case the SPS PDSCH is not received should be avoided, consider to always append the SPS HARQ-ACK bit for all subframes that are activated for SPS transmission.

Discussion: CR is in R1-162143.

Decision: The document is noted.

R1-162143	Correction on SPS HARQ-ACK bit handling in case of dynamic codebook configuration of eCA in 36.213	Huawei, HiSilicon, Samsung, Lenovo	Rel-13	36.213 - CR0575r1 F

Decision: The document is noted. Continue offline discussion – (Huawei)

R1-163416	Corrections on HARQ-ACK generation for eCA in 36.213	CATT	(R1-162253)

The document was presented by Fang Chen Cheng from CATT.

Discussion: Huawei concern can be taken into account in a revision of R1-162143.

Decision: The document is noted.

Monday evening

R1-163500	Correction on SPS HARQ-ACK bit handling in case of dynamic codebook configuration of eCA in 36.213	Huawei, HiSilicon, Samsung, Nokia, Fujitsu, CATT	Rel-13	36.213 - CR0575r2 F

Decision: The document is agreed.

R1-162751	Correction on reporting mode 1-0 and 1-1 for aperiodic CSI of eCA in 36.212	Huawei, HiSilicon

The document (CR0193 to 36.212) was presented by Yongxia Lyu from Huawei.

- Modify the description of “Channel quality information formats for wideband CQI reports” in section 5.2.2.6.1 to be aligned with the section 7.2.1 of 36.213.

Discussion: DraftCR is in R1-162106. Ericsson’s alternative is in R1-163089.

Decision: The document is noted.

R1-162106	Alternative correction on reporting mode 1-0 and 1-1 for aperiodic CSI of eCA in 36.212	Huawei, HiSilicon

Decision: The document is noted.

R1-163089	Correction on aperiodic CSI reporting mode 1-0 and 1-1	Ericsson	Rel 13	36.212 - CR0194 F

Discussion: Also supported by Intel, CATT, ZTE

Huawei won’t object – is fine to follow the majority

Decision: The document is agreed.

R1-163090	Correction on aperiodic CSI reporting mode 1-0 and 1-1	Ericsson	Rel 13	36.213 - CR0615 F

Decision: The document is agreed.

R1-162254	Corrections on n_HARQ-ACK calculation for PUCCH format 4/5 in 36.213	CATT

Decision: The document is noted. Continue offline discussion – (CATT)

R1-162256	Corrections on Simultaneous HARQ-ACK and P-CSI in 36.213	CATT

- Describe UE behavior for HARQ-ACK and P-CSI multiplexing when falls back to Rel-12 behavor.

Decision: The document is noted and CR0623 to 36.213 is agreed in R1-163433.

Further revised and agreed in R1-163526 (CR623 Rev. 1).

R1-162258	Clarification on PUCCH format 4/5 resource derivation in 36.213	CATT

Discussion: Huawei point on UE behaviour is not needed

Decision: The document is noted. Continue offline discussion – (CATT)

R1-162422	Clarification on Simultaneous transmission of HARQ-ACK and SR in 36.213	CATT

Discussion: LGE don’t support the need for separate descriptions for PUCCH format 4/5 with dynamic HARQ-ACK codebook and fixed HARQ-ACK codebook

Decision: The document is noted. Continue offline discussion – (CATT)

R1-163507 	Clarification on Simultaneous transmission of HARQ-ACK and SR in 36.213	CATT, LGE

Decision: The document is noted.

R1-163530	Corrections on simultaneous transmission of HARQ-ACK and SR in 36.213	CATT, LGE, Nokia

Decision: The document is noted and agreed as CR0632 to 36.213 in R1-163533.

R1-163334	Discussion on the motivation to capture the timing for secondary cell activation / deactivation for eCA in 36.213	Huawei, HiSilicon

The document was presented by Yongxia Lyu from Huawei.

- Proposal 1: It is necessary to specify UE CSI report action explicitly if PUCCH SCell is not active in subframe n+8

- Proposal 2: It is necessary to specify the CSI timing requirement of subframe n+8 shall only be applied on the CSI reporting on an activated serving cell.

Discussion: What the UE should do does already exist in the spec.

Decision: The document is noted.

R1-162727	Correction on timing for secondary cell activation / deactivation for eCA in 36.213	Huawei, HiSilicon	Rel-13	36.213 - CR0610 F

- Add the related description to capture the timing agreements for PUCCH SCell activation/deactivation.

Decision: The document is noted. Continue offline discussion until Wednesday – (Huawei)

R1-163458	Correction on timing for secondary cell activation / deactivation for eCA in 36.213	Huawei, HiSilicon	Rel-13	36.213	CR0610r1	F

Decision: The document is agreed.

R1-162645	Remaining Issue on HARQ-ACK Codebook Determination	Samsung

The document was presented by Aris Papasakellariou from Samsung.

Proposal: For TDD, HARQ-ACK codebook size in the PUSCH is determined as for FDD.

Discussion: Remaining Issues on HARQ-ACK Codebook Determination are proposed in following 2 CRs.

Decision: The document is noted.

R1-162646	Draft CR on Correction for DAI Presence for eCA	Samsung

Decision: The document is noted. Continue offline discussion until Wednesday – (Samsung)

R1-163626	Correction on DAI Presence in DCI formats for eCA	Samsung, Nokia, Alcatel-Lucent Shanghai Bell, Panasonic	Rel-13	36.212	CR201	F

Decision: The document is agreed.

R1-162647	Draft CR on Correction for HARQ-ACK Codebook Determination in eCA	Samsung

Decision: The document is noted. Continue offline discussion until Wednesday – (Samsung)

R1-163527	CR on Correction for HARQ-ACK Codebook Determination in eCA	Samsung	Rel-13	36.213	CR0630	F

Decision: The document is agreed.

### Maintenance of Release 13 licensed-assisted access to unlicensed spectrum

Channel access

R1-162449	Discussion on contention window adjustment procedure	LG Electronics

The document suggests a TP to apply to the contention window adjustment procedure

Decision: The document is noted.

R1-162450	Correction on contention window adjustment procedure	LG Electronics

R1-163752	Draft CR on CWp adjustment	Qualcomm Incorporated

R1-162847	Draft CR for handling DTX,NACK/DTX,any state in HARQ feedback for LAA multi-carrier operation	BROADCOM LIMITED

Clarify how DTX, NACK/DTX, any states for TBs sent on the unlicensed channel are handled for the purpose of caluculating the percentage of ACK values in the reference subframe

Decision: The document is noted.

R1-162917	Draft CR for the contention window adjustment procedure in LAA downlink channel access	Nokia, Alcatel-Lucent Shanghai Bell

For the definition of “reference subframe”, the phrase “for which HARQ-ACK feedback is available” is changed to “for which at least some HARQ-ACK

Decision: The document is noted. Continue offline discussion until Wednesday – (Nokia)

Revised in R1-163462	Draft CR for the contention window adjustment procedure in LAA downlink channel access	Huawei, HiSilicon

Decision: The document is noted.

Revised in R1-163756	Draft CR for the contention window adjustment procedure in LAA downlink channel access	Huawei, HiSilicon, LGE, WILUS, Nokia, Alcatel-Lucent Shanghai Bell

Decision: The document is noted.

Revised in R1-163681	Draft CR for the contention window adjustment procedure in LAA downlink channel access	Huawei, HiSilicon, LGE, WILUS, Nokia, Alcatel-Lucent Shanghai Bell, Broadcom

Decision: The document is noted and agreed as CR0651 to 36.213 in R1-163831.

R1-162853	Discussion on slot misalignment between LAA and Wi-Fi	BROADCOM LIMITED

Observation 1: Misalignment of slot boundaries across different technologies can cause significant impact to performance.

Observation 2: The impact of slot misalignment decreases as the CCA duration within a CCA slot is increased from 4us.

Observation 3: Change in  and  from the specified values of 9us and 16us respectively will prevent any mechanism implemented in future to align slot boundaries across technologies.

Proposal 1: 3GPP shall consider enhancing LAA LBT in one of the following ways:

- LAA requires a mechanism by which an LAA eNB accurately time aligns its channel access slots with those of coexisting IEEE 802.11 devices. One means to do so is to detect and transmit the 802.11 PHY preamble.

- Alternatively, in the case of LAA LBT that is capable only of energy detection, LAA LBT procedure reduces the transmission burst ending position detection error by requiring a larger CCA period Xμs (X > 4, e.g. X=7) in each CCA slot, and requires detection results to be reported every 1μs during each CCA period

Proposal 2: LAA should conform to the CCA slot length of 9us exactly and it should exactly comply with the durations 9us and 16us for  and  respectively.

Discussion: Broadcom has concern changing the durations, also expressed when informing RAN4 via LS from last meeting.

Ericsson LS was agreed by RAN1, not by a few companies.

No consensus on the proposed technical enhancements  Mr Chair suggested to continue offline.

Broadcomm would like the slot misalignment issue being evaluated and considered by other companies. Can RAN1 agree that there is an issue?

Mr Chair technical solutions need to be reconsidered.

Decision: The document is noted. No consensus, continue offline discussion until Wednesday – (Broadcom)

Wednesday: Still no consensus, continue discussion until the next meeting

R1-162855	Further Discussion on LAA DL Multi-Channel Access Coexistence Issues	BROADCOM LIMITED

Observation 1: LAA lacks the similar channel frequency planning rules that are used by the 802.11ac devices for the channel bandwidth larger than 20 MHz in 5 GHz band. That means LAA eNB can use any multi-channel combination within 5 GHz band for the channel access, which is not necessarily aligned with 802.11ac multi-channel positions defined by IEEE.

Observation 2: In general, the Wi-Fi network performance is degraded when it coexists with the LAA network that uses the flexible channel grouping than when it coexists with the LAA network that uses the 802.11 channel grouping.

Observation 3: Using the self-deferral or not in the LAA multi-channel access can impact the Wi-Fi network coexistence performance.

Proposal 1: In the case of LAA operation using up to four 20 MHz channels in unlicensed band, those channels shall reside in a single specified 80 MHz channel group aligned with IEEE 802.11.

Discussion: Qualcomm all the points raised in this contribution have been already considered in recent past before agreeing coexistence decisions (RAN1#83). No need to reopen the discussion.

Ericsson better to wait for the outcomes of the current discussions in ETSI BRAN.

Mr Chair this was already discussed in previous RAN1 meetings, currently under discussion in ETSI BRAN – doesn’t really see the need for 3GPP to continue the discussion.

Decision: The document is noted. No consensus.

R1-162856	Draft CR for pausing of random backoff counter for LAA multi-carrier operation	BROADCOM LIMITED

Clarify that the random backoff counter is paused for all the channels which are not selected for transmission while the eNB is transmitting on a set of channels

Discussion: .

Decision: The document is noted. Continue offline discussion until Wednesday – (Broadcom)

Wednesday: Prepare a draft CR in R1-163459 – revisit on Thursday – (Broadcom).

Conclusion: Continue discussion until the next meeting

R1-162451	Correction to the usage of undefined terminology "channel"	LG Electronics

Decision: The document is noted and agreed as CR0625 to 36.213 in R1-163435.

R1-163296	Clarification on channel access procedure after an additional defer duration for DL LBT	WILUS Inc.

Discussion: .

Decision: The document is noted. TP is agreed in principle and CR shall be prepared accordingly in R1-163436 (CR0626 to 36.213). Later on Monday, R1-163436 is agreed.

R1-163386	CR on Freezing ECCA counter during LBT for DRS	Qualcomm Inc.

Discussion: LGE current spec is ok.

Decision: The document is noted. Continue offline discussion – (Qualcomm)

Conclusion: Proposed CR is not pursued.

Partial subframe

R1-162133	Discussion on the support of DwPTS in partial ending subframes	Huawei, HiSilicon, Samsung, CATT

The document was presented by Xia Yuan from Huawei.

- Proposal 1: Reuse the following existing FS2 DwPTS functionalities to the partial ending subframe on LAA Scell.

  - Duration of PDCCH up to 2 symbols in DwPTS

  - ECCE size determination in DwPTS

  - DMRS pattern in DwPTS

  - EPDCCH monitoring in DwPTS

  - TBS determination for PDSCH transmitted in DwPTS

- Proposal 2: there is no PSS in the third OFDM symbol for the partial ending subframe and the following data/control transmission can be supported by the central 6 PRBs.

| Condition |  | Type of data transmission supported by the central 6 PRBs |
| --- | --- | --- |
| The partial ending subframe is not subframe 0 or 5 |  | CRS and DMRS Based PDSCH transmission; EPDCCH transmission |
| The partial ending subframe is subframe 0 or 5 | The duration of the partial ending subframe is less than 12 OS |  |
|  | The duration of the partial ending subframe is 12 OS, (FS1 PSS/SSS is transmitted) | CRS based PDSCH transmission |

- Proposal 3: PSS/SSS is transmitted in the partial ending subframe following frame structure type 1 when the subframe duration is 12 OFDM symbols.

Decision: The document is noted.

R1-162914	CR Discussion for Ending Partial Subframe in LAA	Nokia, Alcatel-Lucent Shanghai Bell

Decision: The document is noted. Nokia is fine to follow R1-162133

R1-162915	Draft CR for 36.211 on the support of ending partial subframe in LAA	Nokia, Alcatel-Lucent Shanghai Bell

R1-162916	Draft CR for 36.213 on TBS determination for ending partial subframe in LAA	Nokia, Alcatel-Lucent Shanghai Bell

R1-163164	Clarification on subframe structure for ending partial subframe in 36.211	Samsung

R1-162638	Clarification on "special subframe" for frame structure type 3 in 36.213	Huawei, HiSilicon, Samsung, CATT

Decision: The document is noted.

R1-163525	Clarification on "special subframe" for frame structure type 3 in 36.213	Huawei, HiSilicon, Samsung, CATT, Nokia, Alcatel-Lucent Shanghai Bell, Ericsson, Qualcomm

Decision: The document is noted and agreed in R1-163531 as CR0631 to 36.213.

R1-163521	Draft CR for 36.211 on the support of ending partial subframe in LAA	Nokia, Alcatel-Lucent Shanghai Bell, CATT, Ericsson, Samsung, Huawei, HiSilicon

Decision: The document is noted and agreed in R1-163528 as CR0235 to 36.211.

R1-163205	Modulation Order Determination for Downlink Partial Subframes	Panasonic

The document was presented by Alex Golitschek from Panasonic.

- Observation 1: For a UE configured to use up to 64-QAM, when increasing the modulation level for MCS levels 7-9 & 13-16, 5 additional choices for MCS/TBS are available. For well-positioned UEs, the corresponding throughput gain for partial subframes has been observed in simulations to exceed 10%.

- Observation 2: For a UE configured to use up to 256-QAM, in a partial subframe 256-QAM cannot be currently assigned if the UE should attempt to decode the packet.

- Observation 3: For a UE configured to use up to 256-QAM, when increasing the modulation level, 256-QAM can be assigned and a peak throughput gain of 33.5% is observed.

- Proposal: Specify that for an initial partial subframe, the UE uses the proposed tables to determine the modulation order and the TBS (where existing tables are applied for )

Decision: The document is noted.

R1-163367	MCS Tables for Initial Partial TTI in LAA	Ericsson

The document was presented by Havish Koorapaty from Ericsson.

- Proposal 1 If new MCS tables are to be introduced for DL initial partial TTIs, the designs should be carefully optimized for high link and system performance.

- Proposal 2 New MCS tables for DL initial partial TTI are proposed.

Discussion: Panasonic believes that Ericsson’s proposal performs better while Panasonic’s proposal is simpler to implement matter of preference which one to choose but both are OK.

Nokia supports Ericsson’s MCS table.

Panasonic ok to follow the Ericsson proposal – would like to work out the CR wordings

Decision: The document is noted.

R1-163206	MCS/TBS Determination for initial partial DL subframes	Panasonic

R1-163368	MCS Tables for Initial Partial TTI in LAA	Ericsson

Decision: The document is noted. Further revised in:

R1-163508	MCS Table for Initial Partial TTI in LAA	Ericsson, Panasonic

Decision: The document is agreed as CR0628 to 36.213.

RRM measurement

R1-162134	Discussion on the support of 2CRS ports in DRS for LAA	Huawei, HiSilicon

Decision: The document is noted.

R1-162235	Discussion on the number of CRS ports in DRS	Qualcomm Inc.

Decision: The document is noted.

R1-162637	Clarification on CRS antenna port number in DRS in 36.211	Huawei, HiSilicon

Continue offline discussion until Wednesday – (Huawei, Qualcomm)

Wednesday

R1-162236	Draft CR on UE assumptions of number of CRS ports in DRS	Qualcomm Inc.

Decision: The document is noted. Continue offline discussion – (Qualcomm) – revisit after Thur am coffee break

Thursday

R1-163461	Clarification on UE assumptions on the number of CRS ports in the DRS	Qualcomm Inc.

Decision: The document is noted. Agreed as CR0648 to 36.213 in R1-163680.

Send LS to RAN4 to inform them of the agreement of this CR, also indicating that RAN1 does not intend this to impact RAN4 requirements.

Friday

MCC note: Due to a mix up in submission, TS number noted in R1-163680 is incorrect and the corresponding CR number as well as this is supposed to be a 211 CR. Therefore R1-163680 is withdrawn and CR is agreed as CR0244 to 36.211 in R1-163930.

R1-163682	[DRAFT] LS on PUSCH transmission for eLAA	Qualcomm

Decision: The document is noted and final LS is agreed in R1-163928.

Others

R1-163002	Draft CR on clarification for LAA CSI processing	Qualcomm Incorporated

The document was presented by Srinivas Yerramalli from Qualcomm.

- Add a condition on the subframe configuration being set to a valid value for the subframe to be used for CSI measurements.

Decision: The document is noted. Agreed as CR0627 to 36.213 in R1-163501.

Post meeting: it was noticed that the agreed CR0627 was written against the wrong version of TS36.213. Rev1 of CR0627 to 36.213 (v13.1.1) for email approval (email thread [84b-20]).

R1-162135	Draft CR on search space definition for LAA DL partial subframes in 36.213	Huawei, HiSilicon

The document was presented by Xia Yuan from Huawei.

- Clarify that when the UE is not configured with ‘s07’, the UE would not monitor the PDCCH common search space.

Decision: The document is noted. Continue offline discussion until Wednesday – (Huawei)

Wednesday: Consensus that CR is not needed.

R1-163384	CR on applicability of DL procedures to LAA	Qualcomm Inc.

The document was presented by Srinivas Yerramalli from Qualcomm.

Decision: The document is noted. Agreed as CR0200 to 36.212 in R1-163532.

R1-163385	CR on correction to EPDCCH procedures to FS3	Qualcomm Inc.

The document was presented by Srinivas Yerramalli from Qualcomm.

Decision: The document is noted. Agreed as CR0232 to 36.211 in R1-163502.

MCC post meeting: note that CR number in cover sheet should be read as CR0232 – not CR0200 – and correction will apply before submission to plenary.

R1-163420	Draft CR for 36.213 on multi-channel access procedure in LAA	Nokia, Alcatel-Lucent Shanghai Bell

The document was presented by Sigen Ye from Nokia.

Decision: The document is noted. Continue offline discussion until Wednesday – (Nokia)

Wednesday: Change to 15.1.5.1.1 is agreed in principle – updated in R1-163684 – agreed as CR0645 to 36.213 in R1-163672

Continue offline discussion – revisit on Thursday – (Nokia) – recommendation of Session Chair to RAN1 Chairman will be to proceed with a Working Agreement if the situation is unchanged.

Conclusion: The existence of multiple groups of channels is already agreed.

Ericsson noted that Broadcom participated in RAN1#83 and did not object to the agreement that mentions “a group carriers”, and participated in subsequent email discussion.

Broadcom states that they had a different interpretation of the agreement.

Broadcom stated that multiple groups without restrictions are not allowed.

Broadcom subsequently stated that their view is that multiple groups are not allowed.

Broadcom states that “the number of carriers” implies that there is only one group.

Broadcom is invited to submit a CR indicating their proposed restrictions for discussion - R1-163460.

Thursday

New (compromise) proposal in R1-163671	Draft CR for 36.213 on multi-channel access procedure in LAA	Nokia, Alcatel-Lucent Shanghai Bell

Decision: The document is noted – companies are invited to check and discuss this offline until Friday – revisit on Friday.

Friday: Continue discussion until the next meeting

R1-163431	Draft CR for 36.212 on subframe configuration for LAA	Nokia, Alcatel-Lucent Shanghai Bell

The document was presented by Sigen Ye from Nokia.

Decision: The document is noted. Agreed as CR0198 to 36.212 in R1-163503.

Friday

R1-163925	Initial CCA Behaviour in the Channel Access Procedure	Ericsson

Decision: The document is noted and CR0649 to 36.213 is agreed.

R1-163927	Draft CR on clarification for channel sensing	Qualcomm

Decision: The document is noted and CR0650 to 36.213 is agreed.

R1-163786	[Draft] LS on DRS Duty Cycle	Qualcomm

Decision: The document is noted and final LS is agreed in R1-163931

### Maintenance of Release 13 elevation beamforming/full-dimension MIMO for LTE

CSI-RS

R1-162435	Clarification of CSI-RS on extended CP	NEC

- Changed UE behavior to restrict CSI-RS transmission in DwPTS for extended CP.

- Changed the table 6.10.5.2-2 and the table title to support normal subframe only.

Decision: The document is noted, revised in R1-163522 and agreed as CR0236 to 36.211 in:

R1-163534	Clarification on CSI-RS for estended CP	NEC, Samsung, CATT

R1-162436	Clarification on PDSCH mapping to resource elements	NEC

Decision: The document is noted. Agreed as CR0233 to 36.211 in R1-163504.

R1-162652	Draft CR on CSI-RS description in TS 36.211	Samsung

Decision: The document is noted. Agreed as CR0234 to 36.211 in R1-163505 assuming the following:

-	one or more configurations for CSI reporting for which the UE shall assume non-zero transmission power for the CSI-RS, and

SRS

R1-163075	Correction on the description about UpPTS length for premable format 4 of PRACH	ZTE

- In section 5.7.1, the UpPTS length for preamble format 4 for PRACH is further cleared that the legacy UpPTS symbols still can be used for PRACH preamble format 4 as long as the length of legacy UpPTS is and

Decision: The document is noted. Agreed as CR0237 to 36.211 in R1-163535.

DM-RS

R1-162447	Correction on the description of DMRS table	ZTE

Decision: The document is noted. Agreed as CR0199 to 36.212 in R1-163506.

R1-163076	Correction on the description about DMRS	ZTE

- Clarify the condition when port={11,13} is used for transmission

- Fix the description of using DMRS ports for PDSCH transmission and DMRS sequence generation.

Decision: The document is noted, revised in R1-163819 and agreed in R1-163673 (CR0242 to 36.211) with original formula shown as strickthrough.

R1-163122	Correction on UE assumption on DMRS ports	ZTE

- Clarify UE assumption in case of the new DMRS table in DCI formats 2C and 2D is configured for single-antenna port transmission scheme.

Decision: The document is noted. Revised in R1-163688.

R1-163082	Clarification of OCC=2 and OCC=4 DMRS MU-MIMO Transmission	Ericsson

The document was presented by Mark Harrisson from Ericsson.

- A UE can use a simpler E-MMSE-IRC receiver while providing higher capacity via 3 MU-MIMO layers than if the UE only uses R-ML with two MU-MIMO layers.

  - Mixed OCC=2 and OCC=4 need not affect E-MMSE-IRC receivers when they use OCC=2 to estimate coscheduled interference.

  - The UE is free to use R-ML or E-MMSE-IRC according to channel conditions and implementation with mixed OCC=2 and OCC=4.

- The current specification does not identify UE behavior for 2 layer MU-MIMO transmission with OCC=2, and so requires correction.

- In order to not prevent eNB from multiplexing Rel-13 UEs with OCC=2 and OCC=4 as agreed, it is necessary to specify UE behaviour when port 7 is coscheduled with ports 8,13 and when port 8 is coscheduled with ports 7,11.

Discussion: CR is in R1-163085..

Decision: The document is noted.

R1-163085	MU-MIMO with Mixed Orthogonal Cover Code Length	Ericsson

Decision: The document (CR0613 to 36.213) is noted. CR0613 is postponed according to the conclusion below:

Conclusion: Views from UE vendors are invited until RAN1#85 – revisit at RAN1#85.

Class A feedback (including codebook)

R1-162649	Rationale for draft CR on FD-MIMO codebooks (36.213)	Samsung

Decision: The document is noted.

R1-162650	Draft CR on FD-MIMO codebooks (36.213)	Samsung

Discussion: Supported by Samsung, Ericsson, CATT, NTT DOCOMO, AT&T, Deutsch Telecom, KDDI, China Telecom, Nokia Networks, NEC, KT Corporation, ETRI, Mitsubish Electric, National Instruments, Sprint

Objected by Huawei, HiSilicon

Decision: The document is noted. Continue offline.

Conlusion: There is no consensus in RAN1.

R1-162655	Clarification on periodic CSI reporting for class A in TS 36.213	Samsung

- Observation: The parameter H’ in PUCCH mode 1-1 cannot be configured since current specification does not support simultaneous configuration of ‘widebandCQI’ and ‘subbandCQI’.

- Proposal: RAN1 should discuss and decide a way to configure H’.

Discussion: Samsung suggested the following 3 alternatives:

Alt 1: send LS to RAN2 to resolve the issue - supported by NEC

Alt 2: Fix H’ to 2 - No support

Alt 3: Define the relationship between MRI and H’ (Samsung preferred solution)

Qualcomm would like to use RRC reconfiguation without having new RRC configuration(s) – Alt 4 – also supported by LGE

Decision: The document is noted. Prepare a draft LS (R1-163512) to RAN2 asking them to solve the issue - (Samsung)

R1-163512	[Draft] LS on signalling of periodic CSI reporting for class A	Samsung

Decision: The document is noted and final LS is agreed in R1-163668.

R1-163091	Correction to S1 and S2 definition and i2 bit width	Ericsson

Decision: The document is noted and CR0195 to 36.212 is agreed.

Class B feedback

R1-162651	Draft CR on missing CRI-only table (36.212)	Samsung

- Add payload table for CRI-only reporting in section 5.2.2.6.1 and clarification in section 5.2.2.6.2

Discussion: .Huawei has detected a potential overlapping issue with already agreed CR from last meeting.

Decision: The document is noted. Continue offline.

Revised in R1-163660 and agreed as CR0203 to 36.212 in R1-163796, removing changes on cover page, changes-on-changes, and adding CR number.

R1-162653	Clarification on non-PMI based CSI reporting in TS 36.213	Samsung

Proposal 1: The conditions for non-PMI reporting mode for ‘CLASS B’ eMIMO-Type in PUCCH reporting should be correctly captured in the current specification.

Observation: Two options for UE assumption on precoding matrix for PUSCH mode 3-0/PUCCH mode 2-0 are available.

- Alternative 1: Remove all assumptions on precoding matrix in non-PMI based CSI reporting mode descriptions.

- Alternative 2: Release the assumption that ‘a single precoding matrix is selected from the CBSR’ and clarify that ‘for each subband a preferred precoding matrix is selected from the codebook subset assuming transmission only in the subband’ in non-PMI based CSI reporting mode descriptions.

Proposal 2: Support both options in observation 2 and revise the corresponding descriptions in the current specification.

Decision: The document is noted.

R1-162654	Draft CR on non-PMI based CSI reporting in TS 36.213	Samsung

Decision: The document is noted. Continue discussion until RAN1#85.

R1-162453	Correction on the linkage between CSI-RS and CSI-IM for Class B	LG Electronics

- Capture the agreed description on one to one linkage between the IDs in NZP-CSI-RS-ID-List and CSI-IM-List, and the other associated parameters on p-C-AndCBSR-PerResourceConfigList and alternativeCodebookEnabledFor4TXProc.

Decision: The document is noted. Agreed as CR0629 to 36.213 in R1-163513.

R1-162656	Clarification on CRI reporting	Samsung

The document was presented by Youngwoo Kwak from Samsung.

- Proposal 1: Usage of CSI-RS ID for class B in TM9 should be clarified.

- Proposal 2: Clarify linkages Linkages between CSI-RS, CSI-IM, alternative 4Tx codebook, PC and CBSR.

Decision: The document is noted. Proposal 1 agreed – prepare CR accordingly.

Wednesday

R1-163687	Draft CR on CSI-RS ID configuration for TM9 in 36.213	Samsung

Decision: The document is noted and agreed as CR0647 to 36.213 in R1-163670 with ME+RAN boxes checked on cover page.

Codebook (Rank 5 – 8)

R1-162343	Further evaluation on Rank 5-8 Codebook	ZTE

Proposal 1: Adopt the proposed codebook which reuses legacy 8Tx codebook as Rel-13 rank 5-8 codebook.

Decision: The document is noted.

R1-162452	Discussions on rank 5-8 codebook	LG Electronics

Proposal 1. Adopt rank 5-8 codebook in [R1-157461] (this option allows W2 feedback for rank 5-8) for Rel-13 FD-MIMO.

Decision: The document is noted.

R1-162967	Evaluation of Rel. 13 rank 5-8 codebook	NTT DOCOMO

Observation: Rank 5-8 codebooks in Scheme 1 and Scheme 2 have similar performance. Scheme 3 suffers from obvious performance degradation.

Proposal: Confirm the working assumption on rank 5~8 codebook.

Decision: The document is noted.

R1-163087	Rank 5-8 Class A FD-MIMO Codebook Alternatives and Performance	Ericsson

Observations:

- A 24% increase in transmissions involving ranks 5-8 is observed in 3D-UMi with the 16-port rank 5-8 codebooks when compared to the Rel-10 8-port rank 5-8 codebooks.

- A 32% increase in transmissions involving ranks 5-8 is observed in 3D-UMi with the 16-port rank 5-8 codebooks when compared to the Rel-10 8-port rank 5-8 codebooks.

- The 16-port ranks 5-8 codebooks yields 3.5% to 6.3% mean user throughput over the alternative 8-port ranks 5-8 codebooks.

Proposal: The current Rel-13 codebooks for CLASS A rank 5 to 8 should be kept.

Decision: The document is noted.

R1-163136	Simulation Results of RAN 5-8 Codebook	Beijing Xinwei Telecom Techn.

Observation 1: In the system level simulation, the percentage of UE with rank greater than or equal to 5 is less than 5%. Thus it is fairer to compare rank 5-8 codebook performance through link level simulation, in which rank 5-8 percentage would very high under high SNR.

Observation 2: Even for link level simulation where there are more UEs with rank greater than 5, R13 FD-MIMO codebook still has limited gains compared to R10 codebook.

Decision: The document is noted. Further revised in R1-163415.

Alt.1: rank 5-8 code book as captured in 157789 is supported (confirm the WA)

Alt 2: rank 5-8 code book as captured in 162343 is supported (ZTE)

Alt 3: rank 5-8 code book as captured 157461 is supported (LGE)

Possible conclusions

1: If RAN1 cannot reach a consensus until wed, then rank 5-8 class A codebooks are not supported in Rel-13

2: No new Rank 5-8 class A codebook (i.e above Alt 2 and 3) is introduced in Rel-13

3: RAN1 cannot find critical issues for existing WA about Rank 5-8 codebook

Still ZTE had concern about the above conclusion 3 – ZTE objected

Mr Chair, 3 companies having concern with conclusion 1, 2 companies having concern with conclusion 2 – let’s agree on conclusion 3

For the sake of making progress, ZTE agreed to the conclusion 3 but asked to note that their concern is on additional complexity of the rank 5-8 codebook without providing any performance gains

Conclusion:

- RAN1 can not find critical issues for existing WA about rank 5 – 8 codebooks

R1-163088	Discussion on Correction to Rank 5-8 Operation in Class A Config 1	Ericsson

R1-163086	Correction to rank 5-8 FD-MIMO CSI feedback	Ericsson

Decision: The document is noted and CR0614 to 36.213 is agreed.

Others

R1-162344	Remaining issues on CSI processing relaxation	ZTE

Decision: The document is noted. Continue discussion until RAN1#85.

### Maintenance of Release 13 further LTE physical layer enhancements for MTC

R1-163891	Chairman's Notes of Agenda Item 7.1.5 on eMTC CRs	Ad-hoc chair (Qualcomm)

The document was presented by Wanshi Chen from Qualcomm and provides the outcomes of the remaining maintenance issues related to eMTC.

Decision: The document is endorsed.

R1-163768	LS on PRACH preamble power for eMTC	Ericsson

Decision: The document is noted and final LS is agreed in R1-163791

R1-163686	LS on reviewing all LTE features up to Rel-13 for Category M1 UEs	RAN2

Decision: The document is noted.

R1-163749	Reply LS on RAN2 review of all LTE features up to Rel-13 for Category M1 UEs	Ericsson

Decision: The document is noted.

R1-163913	[Draft] Reply LS on RAN2 review of all LTE features up to Rel-13 for Category M1 UEs	Ericsson

Decision: The document is for email approval untill April 20th – (Ericsson)

Collision Handling

R1-162456	Correction on collision of dynamically scheduled data and semi-statically scheduled data for Rel-13 eMTC	LG Electronics

Decision: The document is noted.

Conclusion: The CR in R1-162456 is agreed in principle by adding “Half-duplex”. Also discuss further offline whether TA has impact on subframe definitions between DL and UL. Prepare final CR, to be visited later this week.

R1-163783	Correction on collision of dynamically scheduled data and semi-statically scheduled data for Rel-13 eMTC	LG Electronics

Decision: The document is noted and CR0637r1 to 36.213 is agreed in R1-163787, assuming the following update: “overlaps partially or fully  collides partially or fully”

R1-162659	Clarification on MPDCCH/PDSCH Collision with PSS/SSS/PBCH	Samsung

Decision: The document is noted.

R1-163353	Collision between PSS/SSS and MPDCCH/PDSCH for MTC	Ericsson

Decision: The document is noted.

R1-163352	Collision between PBCH and MPDCCH for MTC	Ericsson

Decision: The document is noted.

Proposed conclusion:

- For an M-PDCCH candidate, if colliding with PSS/SSS (even partially), it is dropped in the given subframe

- For a DM-RS based PDSCH transmission, for the REs colliding with PSS/SSS, the REs are punctured by PSS/SSS

- For CRS based PDSCH, if colliding with PSS/SSS, Alt 2 in R1-162659 is agreed

- Prepare a CR, to be visited later this week

Further discussion offline – including discussion on collision between M-PDCCH and PBCH

R1-163661	Collision between PSS/SSS/PBCH and MPDCCH/PDSCH for MTC	Ericsson, Samsung

Decision: The document is noted and CR0229r1 to 36.211 is agreed in principle. Correct a typo in the doc and agreed revision 2 of CR0229 in R1-163790.

R1-163817	Collision between PSS/SSS/PBCH and PDSCH for MTC	Ericsson

Decision: The document is for email approval until 21st April – (Ericsson)

R1-162594	On the collision between eMTC SIB and MPDCCH/PDSCH in TS 36.213	Huawei, HiSilicon

Decision: The document is noted. Further discussion offline, revisited later this week

The CR in 162594 is endorsed. Prepare final CR, revisited later this week

R1-163794	On the collision between eMTC SIB and MPDCCH/PDSCH in TS 36.213	Huawei, HiSilicon

Decision: The document is noted and CR0639 to 36.213 is agreed.

R1-163006	Remaining issues for eMTC	Qualcomm Incorporated

Decision: The document is noted.

R1-163093	Clarification on MBSFN subframe handling for BL/CE UEs	CATT

Decision: The document is noted.

Proposal:

- Clarify the behaviour of different unicast PDSCH TM with respect to valid MBSFN subframes as follows:

  - For TM1/2/6, the UE does not expect to receive PDSCH in the valid MBSFN subframes, and they are counted as a repetition.

  - For TM9, the UE receives PDSCH and is counted as a repetition.

Continue offline discussion – (Qualcomm)

R1-163780	Clarification of TM1/2/6 on MBSFN subframes	Qualcomm Inc

Decision: The document is noted and the draft CR in 163780 is endorsed. Prepare final CR, revisit later this week: R1-163812

R1-163812	Clarification of TM1/2/6 on MBSFN subframes	Qualcomm

Decision: The document is noted and CR0641 to 36.213 is agreed.

R1-163781	Correction of fallback behavior for TM9	Qualcomm Inc

Decision: The document is noted and the draft CR in 163781 is endorsed. Prepare final CR, revisit later this week: R1-163813

R1-163813 	Correction of fallback behavior for TM9	Qualcomm Inc

Decision: The document is noted and CR0642 to 36.213 is agreed.

Agreement:

- In case of collision between periodic CSI and PDSCH with repetitions, the periodic CSI is dropped for a half-duplex UE.

  - Note: may also be related to subframe definitions for DL/UL due to TA

- Prepare a CR, to be visited later this week – (Qualcomm)

- Covered by R1-163811

R1-163773	Collision between PUCCH format 2 and PDSCH with repetitions	Qualcomm Inc.

Decision: The document is noted and the draft CR in 163773 is endorsed. Further revised in:

R1-163811	Collision between PUCCH format 2 and PDSCH with repetitions	Qualcomm

Decision: The document is noted and CR0640 to 36.213 is agreed.

Proposal:

- For BL/CE UEs in CEModeB configured in transmission mode 9, resource elements occupied by cell-specific reference signals shall be counted in the PDSCH mapping but/and

  - not used for transmission of the PDSCH in non-MBSFN subframes

  - used for transmission of the PDSCH in MBSFN subframes

- For BL/CE UEs in CEModeA configured in transmission mode 1, 2 or 6, the UEs shall postpone receiving the transmission in the MBSFN subframes included in the set of BL/CE DL subframes.

- For BL/CE UEs in CEModeB configured in transmission mode 1, 2 or 6, the UEs shall drop receiving the transmission in the MBSFN subframes included in the set of BL/CE DL subframes.

Continue offline discussion – Yanping (CATT).

R1-163705	CR on correction on RE mapping in MBSFN subframe for BLCE UEs in CEModeB	CATT

Decision: The document is noted and CR0241 to 36211 is agreed in principle. Refine wording and revisit later this week. – revisit on Friday - R1-163679

R1-163679	Correction on RE mapping in MBSFN subframe for BL/CE UEs in CEModeB	CATT, Panasonic, Intel, Sony, Ericsson

Decision: The document is noted and rev1 of CR0241is agreed.

R1-163351	Collision between PBCH and PRS for MTC	Ericsson

Decision: The document is noted.

Conclusion:

- No standardized solution is necessary for handling colliding between PBCH and PRS

Re-tuning Handling

R1-162250	eMTC corrections for 36.211 (Proposal 4)	Sequans Communications

Decision: The document is noted.

R1-162251	eMTC corrections for 36.212	Sequans Communications

R1-162445	Issues with retuning for different UL transmissions in eMTC (TS 36.211)	NEC

Decision: The document is noted.

R1-163003	Discussion of issues with rate matching for UL narrow band change	Qualcomm Incorporated

Decision: The document is noted.

R1-163004	Draft CR on narrow band hopping	Qualcomm Incorporated, Ericsson, Nokia, Alcatel-Lucent Shanghai Bell, Intel

Decision: The document is noted.

Conclusion:

- The draft CR in R1-163004 is agreed in principle. Prepare final CR, to be visited later this week – (Qualcomm) The corresponding CR0239 to 36.211 in R1-163702 is agreed. MCC post meeting correct wrong revision on CR cover sheet

- Further discussion offline w.r.t. PUCCH to PUCCH retuning as identified in R1-162445 – (NEC)

R1-163709	WF on PUCCH to PUCCH retuning	NEC, Qualcomm, InterDigital, Sequans, Ericsson, LGE

Decision: The document is noted.

Proposal:

- Revoke the following agreement from RAN1#84:

  - Shortening of PUCCH 1/1a/1b in the first slot of a subframe is realized by using in the first slot with PUCCH 1/1a/1b resource mapping from symbols 1 through 6 in the first slot while maintaining the existing PUCCH 1/1a/1b DM-RS locations.

- Agree the following proposal:

  - For PUCCH 1/1a/1b, UL retuning gaps are generated by puncturing of the affected SC-FDMA symbols: last symbol and first symbol of the prior and the latter subframe respectively

    - Except if SRS subframe, shortened format of legacy behaviour is applied, i.e., shortened format is applied in the first subframe, and first symbol of the second subframe is punctured.

Continue discussion till RAN1#85.

MPDCCH Aggregation of up 2+4 PRBs

R1-162250	eMTC corrections for 36.211 (Proposal 3)	Sequans Communications

Decision: The document is noted.

R1-162657	Draft CR on 24 ECCE Aggregation Level for TS 36.211	Samsung

Decision: The document is noted.

R1-162658	Draft CR on 24 ECCE Aggregation Level for TS 36.213	Samsung

Decision: The document is noted.

R1-163156	draft CR for TS36.211 related to 2+4 PRB set	Panasonic Corporation

Decision: The document is noted.

R1-163159	draft CR for TS36.213 related to 2+4 PRB set	Panasonic Corporation

Decision: The document is noted.

R1-163133	MPDCCH remaining issues related to 2+4 PRB set	Panasonic Corporation

Decision: The document is noted.

Conclusion:

- The CRs in 2657, 2658, 3156, and 3159 are agreed in principle, except the following:

  - Text regarding “if the detected MPDCCH is located within 4 PRB set, is the sum between and the value obtained by above procedure.” (Note: it occurs in multiple places)

    - This text is for further discussion

- Provide updated CRs (for 36.211 and 36.213 respectively) including the above text – revisit on Friday – (Panasonic)

R1-163858	CR for TS36.211 related to 2+4 PRB set	Panasonic, Qualcomm, Samsung, Sequans, Sierra Wireless, Ericsson, Intel

Decision: The document is noted and CR0243 to 36.211 is agreed.

R1-163859	CR for TS36.213 related to 2+4 PRB set	Panasonic, Qualcomm, Samsung, Sequans, Sierra Wireless, Ericsson, Intel

Decision: The document is noted and CR0646 to 36.213 is agreed.

R1-162252	eMTC corrections for 36.213 (Proposal 1)	Sequans Communications

Decision: The document is noted. Further discussion offline regarding proposal 1 in R1-162252

R1-162613	Discussion on MPDCCH AL and search space for extended CP	Huawei, HiSilicon

Decision: The document is noted.

Proposal:

- For MPDCCH UE-specific search space configured with 2 or 4 PRB-pairs and mPDCCH-NumRepetition=1,

  - MPDCCH case A and B in Table 6.8B.1-2 of TS36.211 are defined similar as EPDCCH.

  - AL 1 should be included in Table 6.8B.1-2 of TS36.211 for MPDCCH format 0 in case B.

- For MPDCCH UE-specific search space configured with 2 + 4 PRB-pairs or mPDCCH-NumRepetition>1, and MPDCCH common search space,

  - MPDCCH case B in Table 6.8B.1-2 of TS36.211 applies for extended CP as well as special subframes with special subframe configuration 1,2,6,7,9 and normal CP, case A applies otherwise.

  - AL ϵ {1, 2, 4, 8, 12} and the agreed number of monitored candidates are used for MPDCCH in case B.

Further discussion offline, revisit later this week

R1-162961	Draft CR on MPDCCH format for Rmax=1 and 2/4 PRBs	InterDigital Communications

Decision: The document is noted.

Conclusion: The draft CR in R1-162961 is endorsed. Prepare final CR, to be visited later this week

Non-repetition case:

R1-163704	CR on MPDCCH format for Rmax=1 and 2/4 PRB	InterDigital Communications

Decision: The document is noted. Revised with ME+RAN boxes checked on cover sheet and finally agreed as rev1 of CR0240 to 36.211 in R1-163676.

R1-163755	Correction on M-PDCCH case definition	InterDigital, Qualcomm, Panasonic, LG Electronics, Huawei, HiSilicon

Decision: The document is noted. Revised with red text changed to black, (Note that Format 6-2 does not need to be included) and finally agreed as rev1 of CR0635 to 36.213 in R1-163844.

R1-163008	Miscellaneous corrections for eMTC (Proposals 4&5)	Qualcomm Incorporated

Decision: The document is noted.

Agreement:

- Clarify the MPDCCH case definition when using legacy EPDCCH candidate table.

- Use Case 1 for normal subframes and normal CP, and case 3 otherwise.

Prepare a CR, to be visited later this week.

Covered in R1-163677.

36.211

R1-162250	eMTC corrections for 36.211 (except Proposal 3)	Sequans Communications

Decision: The document is noted. Discuss further offline regarding proposal 2 in R1-162250.

R1-163561	 Correction of cyclic hopping term for PUSCH DMRS (TS 36.211)	NEC	(R1-162444)

Decision: The document is noted. Discuss further offline regarding R1-163561.

Continue discussion until RAN1#85.

R1-162610	On the granularity for SIB1-BR frequency hopping in TS 36.211	Huawei, HiSilicon

Decision: The document is noted. Discuss further offline.

CR is not needed.

R1-162611	Corrections to TS 36.211 for eMTC	Huawei, HiSilicon

Decision: The document is noted.

Conclusion: the draft CR in R1-162611 is endorsed. Prepare final CR, to be visited later this week.

Agreed in R1-163698 as CR0238 to 36211.

R1-163354	DMRS initialization of CSS for MTC	Ericsson

Decision: The document is noted and CR0230 to 36.211 is agreed.

R1-163355	Missing words in PRACH starting subframe paragraph for MTC	Ericsson

Decision: The document is noted and CR0231 to 36.211 is agreed.

36.212

R1-162593	On MPDCCH order identification in CE Mode B	Huawei, HiSilicon

Decision: The document is noted.

Agreement:

- When an M-PDCCH is used for M-PDCCH order, the two spare bits corresponding to the location of two MSB bits in the MCS field can be fixed to 11 (in addition to resource block assignment field set to all ‘1’s) based on option 2a or option 2b.

  - Option 2b is adoped

R1-163542	Identify MPDCCH order in CE mode B in TS 36.212	Huawei, HiSilicon, CATT, MediaTek, Sierra Wireless

Decision: The document is noted. Further discussion offline on 3542, prepare final CR, to be visited later week

- Including the editorial changes from 2614

Agreed in R1-163689 as CR0202 to 36.212.

R1-162592	Identify MPDCCH order in CE mode B in TS36.212	Huawei, HiSilicon

R1-163541	Identify MPDCCH order in CE mode B in TS 36.212	Huawei, HiSilicon, CATT, MediaTek,

R1-162614	Corrections to TS 36.212 for eMTC	Huawei

Decision: The document is noted.

R1-163567	 MCS field in DCI format 6-2 for paging for MTC	Ericsson, Huawei, HiSilicon	(R1-163357)

Decision: The document is noted and rev1 of CR0196 to 36.212 is agreed.

R1-163568	MCS field in DCI format 6-2 for paging for MTC	Ericsson, Huawei, HiSilicon	(R1-163362)

Decision: The document is noted and rev1 of CR0621 to 36.213 is agreed.

R1-163349	Channel interleaver correction for eMTC	Nokia, Alcatel-Lucent Shanghai Bell

Decision: The document is noted. Discuss further offline.

Revised in R1-163771

Continue discussions until the next meeting

36.213

R1-162252	eMTC corrections for 36.213 (except Proposal 1)	Sequans Communications

Decision: The document is noted.

R1-163417	Correction on CSS for MPDCCH configured by temporary C-RNTI and Type0 MPDCCH CSS resource	LG Electronics, Panasonic, Sequans	(R1-162457)

Decision: The document is noted. Discuss further offline, and prepare a final CR.

Agreed in R1-163782 as CR0636 to 36.213.

R1-163350	MDPCCH candidate overflow monitoring correction for eMTC	Nokia,  Alcatel-Lucent Shanghai Bell

Decision: The document is noted. Discussion further offline, and prepare a final CR.

R1-163772	MDPCCH candidate overflow monitoring correction for eMTC	Nokia, Alcatel-Lucent Shanghai Bell, Sequans Communications, NEC, Ericsson

Decision: The document is noted and agreed as CR0633r2 to 36.213 in R1-163677 with correction “any of its MPDCCH candidates”

R1-163361	MPDCCH repetition for paging and random access for MTC	Ericsson

Decision: The document is noted and CR0620 to 36.213 is agreed.

R1-162454	Correction on PDSCH transmission timing for Rel-13 eMTC	LG Electronics

Decision: The document is noted. The CR is endorsed, final CR to be visited later this week.

R1-163784	Correction on PDSCH transmission timing for Rel-13 eMTC	LG Electronics, NEC

Decision: The document is noted and agreed as CR0638r1 to 36.213 in R1-163678, with filename to be corrected with correct CR number, and check the RAN impact box on cover sheet.

R1-162612	Corrections to TS 36.213 for eMTC		Huawei, HiSilicon

Decision: The document is noted. The CR is endorsed, final CR to be visited later this week.

Agreed in R1-163697 as CR0634 to 36.213.

R1-162873	Correction of paging PDSCH transmission for MTC UE	Ericsson, Nokia, Alcatel-Lucent Shanghai Bell

Decision: The document is noted and CR0616 to 36.213 is agreed.

R1-163007	Clarifcation on CSI measurement with respect to R_CSI, max	Qualcomm Incorporated

Decision: The document is noted.

R1-163552 	WF on CSI measurement with respect to R_CSI, max	Panasonic, Qualcomm

Decision: The document is noted.

Proposal for the working assumption:

- For narrowband CSI

  - RCSIMax <= Ych

    - the reference subframes are defined as the last subframes of the period of nCQI_ref for each hopping narrowband.

  - RCSIMax > Ych

    - The time location of the reference subframes is not defined but UE should reflect the situation of the last subframes of Ych for each hopping narrowband.

    - The period of the reference subframes should take into account as the offset of CQI value i.e. if RCSIMax  = 2 ms, it means spectral efficiency is scaled by a factor of 2.

- For wideband CSI

  - RCSIMax < Ych x (number of hopping frequencies) and/or RCSIMax mod (Ych) is not zero

    - The time location of the reference subframes is not defined but UE should reflect the situation of the last subframes of Ych x (number of hopping frequencies).

    - The period of the reference subframes should take into account as the offset of CQI value i.e. if RCSIMax  = 2 ms, it means spectral efficiency is scaled by a factor of 2.

Discuss further offline

R1-163008	Miscellaneous corrections for eMTC	Qualcomm Incorporated

Decision: The document is noted.

Conclusion:

- The first 2 proposals in R1-163008 are agreed. Prepare a CR, revisit later

R1-163814	Correction on RV and MPDCCH starting position	Qualcomm

Decision: The document is noted and CR0643 to 36.213 is agreed.

R1-163359	Update RRC parameter names for MTC	Ericsson

Decision: The document is noted and CR0618 to 36.213 is agreed.

R1-163360	PUCCH repetition for Msg4 for MTC	Ericsson

Decision: The document is noted and CR0619 to 36.213 is agreed.

R1-162455	Correction on Type1 and Type2 MPDCCH CSS transmission for Rel-13 eMTC	LGE

Decision: The document is noted. Check offline whether 162455 is already included or not in other CRs

Others

R1-163005	Discussion on paging occasion overlap for eMTC	Qualcomm Incorporated

Decision: The document is noted. Continue discussion till RAN1#85

R1-163161	RRC parameters discussion on per CE or per cell	Panasonic Corporation

Decision: The document is noted.

Conclusion:

- Regarding the potential mix CQI and ACK/NACK in the same PRB in eMTC

  - This will not have no RRC impact.

  - Discuss till next meeting whether or not there is specification impact

R1-163100	Remaining issue on M determination for aperiodic CSI for eMTC before RRC configuration	LG Electronics

### Other

R1-162768 	Correction on RRC parameter for configuring new TBSs	Ericsson

Change the RRC parameter from tbsTableAlt to tbsIndexAlt in TS 36.213 to be aligned with RAN2 spec based on LS R1-160240 from RAN2

Decision: The document is noted and CR0612 to 36.213 Rel-12 is agreed.

R1-162767	Correction on RRC parameter for configuring new TBSs	Ericsson

Decision: The document is noted and shadow CR0611 to 36.213 Rel-13 is agreed.

R1-163001	On SRS dropping under different CPs or 6-symbol UpPTS in CA	Qualcomm Incorporated

The document was presented by Wanchi Chen from Qualcomm and discusses how to address the issue regarding SRS dropping in CA particularly when two or UL CCs have different CPs or when the 6-symbol UpPTS is configured.

- Proposal 1: In Rel-13 and beyond, when the UE is configured with CA of different CPs under a single TAG configuration, consider adopting the same rule regarding SRS dropping as in the case when the UE is configured with multiple TAGs in CA

- Proposal 2: In Rel-13 and beyond, when the UE is configured with a 6-symbol UpPTS under a single TAG, consider adopting the same rule reading SRS dropping as in the case when the UE is configured with multiple TAGs in CA

Discussion: Fujitsu ok to have a CR but limited to Rel-13. Fine to Qualcomm (CR not yet available).

Decision: The document is noted. Both proposals are agreed in principle . Prepare CR for Rel-13 only until Wednesday – Qualcomm

Friday

R1-163434	Corrections SRS dropping in CA in 36.213	Qualcomm Inc.	Rel-13	36.213	CR0624 	F

Decision: The document is noted and CR0624 to 36.213 Rel-13 is agreed.

## LTE Release 13

### NB-IoT - WID in RP-152284.

The objective is to specify a radio access for cellular internet of things, based to a great extent on a non-backward-compatible variant of E-UTRA, that addresses improved indoor coverage, support for massive number of low throughput devices, low delay sensitivity, ultra low device cost, low device power consumption and (optimised) network architecture.

R1-163608	Chairmen's Notes of Agenda Item 7.2.1 on NB-IoT	Ad-hoc chairs (Huawei, Nokia)

The document was presented by Matthew Baker from Nokia and provides the outcomes of the NB-IoT sessions

Decision: The document is endorsed and content is incorporated below.

R1-162634	Summary of RAN2 open items with possible RAN1 dependence	Huawei, HiSilicon

Decision: The document is noted.

Agreement: The DL NB-IoT UE category has total number of soft channel bits = 2112 bits.

R1-162635	Draft 36.212 CR for NB-IoT	Huawei

Decision: The document is noted and agreed in R1-163405 (CR0192 rev 1).

R1-163418	Introduction of NB-IoT (36.211 CR0224 rev 3)	Ericsson

Decision: The document is noted. Editor (Asbjorn) to provide update with comments by Wed.

R1-163593	Introduction of NB-IoT  (36.211 CR0224 rev 4)	Ericsson

Decision: The document is noted and CR0224r4 to 36.211 is agreed as the basis for the next update.

R1-163409	Draft 36.213 for NB-IoT	Motorola Mobility

Decision: The document is noted. Editor (Bob Love) to have an update by Friday.

Friday:

R1-163912	Draft 36.213 for NB-IoT	Motorola Mobility

Decision: The document is noted and agreed as the basis for the next update.

R1-162636	Introduction of NB-IoT	Huawei

Decision: The document (Draft 36.300 section 5 CR for NB-IoT) is noted and agreed as the basis for the next update. Next update will be provided by Friday - (Huawei)

R1-163899	Introduction of NB-IoT	Huawei

Decision: The document is noted.

R1-163918 	Introduction of NB-IoT	Huawei

Decision: The document is noted and the draft CR is endorsed

R1-163900	[DRAFT]Reply LS on updated TS 36.300 section 5 for NB-IoT	Huawei

Decision: The document is noted and final LS is agreed in R1-163932 by updating Tdoc number of attachment file.

MCC post meeting: It appears that the agreed LS to RAN2 should include R1-163918 as the attachment (latest endorsed draftCR), and not the previous version of the 36.300 draftCR (R1-163899). Final LS is agreed in R1-163937 by including the correct attachment file.

R1-163426	Text Proposal for TR 36.802 (NB-IoT; Technical Report for BS and UE radio transmission and reception)	Huawei

Decision: The document is noted.

R1-163446	[DRAFT]LS on NB-IoT 	Huawei

Decision: The document is noted and final LS to RAN4 is agreed in R1-163449 with bold/underlined addition in normal font.

Updated LS to account for 12 UL-DMRS sequences for the 3 tone case.

R1-163870	[Draft]LS on Correction of UL DMRS sequences	Ericsson

Decision: The document is noted and final LS to RAN4 is agreed in R1-163871, with the inclusion of the equation above the table to avoid confusion between m in the last LS and u in this one.

R1-163592	[DRAFT]LS on NB-IoT	Huawei

Decision: The document is noted and final LS to RAN2 is agreed in R1-163807, including the NB-PRACH subcarrier locations table and removing bold/underline.

R1-163578	L1 NB-IoT parameters spreadsheet	Huawei,HiSilicon

Decision: The document is noted and the spreadsheet of RRC parameters is agreed.

R1-163579	[DRAFT] LS on updated NB-IoT L1 parameters	Huawei

Decision: The document is noted and final LS to RAN2 is agreed in R1-163809, removing the sentence “RAN1 will continue to inform RAN2 about further updates to the list.”

R1-163738	[DRAFT]LS on NB-IoT	Huawei

Decision: The document is noted and final jumbo LS to RAN4 is agreed in R1-163739.

Incoming LS from RAN2

R1-163852	LS on Power Headroom report	RAN2, Huawei

So far RAN2 does not include PHR for other cases than MSG3 but RAN2 would like to understand whether there is any need to report PHR in other cases than in MSG3.

Conclusion on LS reply:

For the typical applications targeted by Rel-13 NB-IoT there may not be much benefit. In the future, there may be some benefit for longer sessions. RAN1 sees no additional impact at the physical layer for additional PHR reporting.

Draft LS reply to RAN2 in R1-163808 including the above text – (Huawei)

R1-163808	[DRAFT] Reply LS on Power Headroom report	Huawei

Decision: The document is noted and final LS is agreed in R1-163869.

Friday:

Editors of 36.211, 36.212 and 36.213 to provide updates including agreements and working assumptions from RAN1#84bis by April 29th. Email approval deadline is on May 5th.

#### Downlink physical channels and signals

R1-163439	WF on DL OFDM baseband signal generation	Intel

Decision: The document is noted.

Agreement:

- A common OFDM baseband signal generation for NB-IoT downlink for all operation modes is described with respect to the centre frequency of the NB-IoT carrier

  - Exact equation to be provided by the TS36.211 specification editor

##### NB-PDCCH

R1-162458	Remaining issues on NB-PDCCH	LG Electronics

Decision: The document is noted.

Agreement on starting subframes:

- The search space starting subframe reuses the principle of eMTC

  - The period reuse the principles from eMTC

    - T=R*G, G values to be determined

    - An additional offset on the value of T may be considered

- Postpone NPDCCH repetitions that overlap with transmission gap

- FFS whether to also allow an offset to the period (value FFS)

  - Additional offset = R*G1, G1=0,2,4,8

R1-163275	Remaining Details on DCI Contents	Samsung

Decision: The document is noted.

Agreement: In DCI format N1, ACK/NACK Resource Indication: 4 bits

R1-162773	NB-IoT - Remaining issues for NPDCCH	Ericsson

Decision: The document is noted.

Agreement: An NPDCCH block of subframes is scrambled in the same way as PDCCH and the scrambling is reset every 4th NPDCCH subframe.

R1-162757	Remaining issues on NB-PDCCH design for NB-IoT	ZTE

Decision: The document is noted.

Agreement: Always have 6 bits for subcarrier allocation in 36.212, 36.213 uses 5 of the 6 bits for the 15kHz case

R1-163587	WF on starting subframe	Huawei

Decision: The document is noted.

R1-163570	WF on starting subframe	ZTE

Decision: The document is noted.

R1-163260	Detail on scheduling delay for NPDSCH and NPUSCH	MediaTek Inc.

Decision: The document (section 2) is noted.

Agreements:

- G={1.5, 2, 4, 8, 16, 32, 48, 64}

- eNB only configures values that result in T>=4

- T=Rmax*G

R1-163801	Remaining details of starting subframes	Huawei, HiSilicon, Intel, MediaTek

Decision: The document is noted. Further revised in R1-163804.

R1-163804	WF on remaining details of NPDCCH starting subframes	Huawei, HiSilicon, Intel, MediaTek, LGE

Decision: The document is noted.

Agreement:

- For UE-SS, G is UE-specific

- For CSS for RAR/Msg3re-tx/Msg4, G is cell-specific and NPRACH resource-specific

R1-163167	Views on remaining issues of NB-PDCCH	NTT DOCOMO, INC.

Decision: The document is noted.

Not treated.

R1-162625	Remaining details of NB-PDCCH	Huawei, HiSilicon

R1-162626	DCI for NB-IoT	Huawei, HiSilicon

R1-162772	NB-IoT - DCI content	Ericsson

R1-162902	Remaining issues on NB-PDCCH	Nokia, Alcatel-Lucent Shanghai Bell

R1-162972	DCI design for NB-IoT	Intel Corporation

R1-162973	DL gaps and remaining details of timing relationships for NB-IoT	Intel Corporation

R1-163010	NB-PDCCH Design	Qualcomm Incorporated

R1-163271	NB-PDCCH Timing and Collision	Sony

R1-163494	WF on numbers of repetitions	Huawei, HiSilicon

##### NB-PDSCH

Numbers of repetitions

R1-163168	Views on remaining issues of NB-PDSCH	NTT DOCOMO, INC.

Decision: The document is noted.

Agreement: Repetitions can be applied to any supported MCS by NB-IoT

R1-162903	Remaining issues on NB-PDSCH	Nokia, Alcatel-Lucent Shanghai Bell

Decision: The document is noted.

Agreements:

- The predefined set of NB-PDSCH numbers of repetitions of the allocated subframes (i.e. NSF) is {1, 2, 4, 8, 16, 32, 64, 128, 192, 256, 384, 512, 768, 1024, 1536, 2048}.

- 4 bits in DCI (for both normal and paging NB-PDSCH) indicate one value in the above set

  - Note that this shall not imply anything about how the number of PUSCH repetitions is indicated!

R1-162974	Remaining details of NB-PDSCH design	Intel Corporation

Decision: The document is noted.

Proposal 2: The maximum number of repetitions for the NB-PDSCH is 2048 and the range for NB-PDSCH repetitions can be {1, 2, 4, 8, 16, 32, 64, 128, 256, 512, 1024, 2048}. The DCI indicates one of 8 values that are configured via RRC.

R1-163802	WF on scrambling sequence initialization	Huawei, HiSilicon, Nokia, Samsung, CATT, Ericsson, Sharp

Decision: The document is noted.

Agreements:

- For NB-PUSCH the scrambling sequence is according to Section 7.2 of TS 36.211

- For NB-PDSCH and NB-PUSCH

Gap configuration

R1-162774	NB-IoT - Remaining issues for NPDSCH design	Ericsson

Decision: The document is noted.

R1-162459	Remaining issues on NB-PDSCH	LG Electronics

Decision: The document (Section 2) is noted.

Agreements:

- Single gap configuration is defined for NB-IoT downlink transmission.

- There can be one gap configuration for the PRB with NBPSS/NBSSS/SIB1, and with multi-PRB one optional additional gap configuration for the other PRB(s). If not configured then the same gap is used.

R1-163447	WF on Application of DL Gap for NB-IoT	Ericsson, Intel, ZTE, Nokia, ASB, Sony

Decision: The document is noted.

Agreements:

- For each configuration, the cycle starts with transmission period followed by the subframes for the gaps

- The gap configuration is optional; if not configured it means there is no gap

- DL transmission gap does not apply to SIB transmission.

- Gaps configuration is applied for both NPDCCH CSS for paging and NPDSCH carrying the paging message

  - The same configuration is used for paging NPDCCH and NPDSCH as that of other DL transmission

  - For paging, R_NPDCCH_max refers to Rmax of paging CSS configured by eNB

R1-163569	WF on DL Gap Configuration for NB-IoT	ZTE, Huawei, HiSilion, Qualcomm, MediaTek

Decision: The document is noted.

Agreements:

- X1 can be configured by SIB.

- When NB-PDCCH/NB-PDSCH repetitions are overlapping with a DL gap, the NB-PDCCH/NB-PDSCH repetitions are postponed to the next valid DL subframe.

R1-163448 	WF on Further Details of DL Gap Configuration for NB-IoT	Ericsson, Intel

Decision: The document is noted.

Agreements:

- Starting point of gap period (SFN=0, sf=0)

- {Gap starting point periodicity, Gap size} are

  - signaled separately

    - 2 bits each

- X1 is 2 bits

  - {32, 64, 128, 256}

- Gap starting point periodicity and gap size are

  - On an absolute number of subframes

R1-163743	WF on Remaining Issues for DL Transmission Gap of NB-IoT	Ericsson

Decision: The document is noted.

Agreements:

- When DL transmission gap is configured, each cycle starts with subframe (nf, ns) where Tg is the Gap starting point periodicity, and

- 10240 is an integer multiple of Tg

R1-163742	WF on value set of Gap starting point periodicity and Gap size 	ZTE, Huawei, HiSilicon, Mediatek

Decision: The document is noted.

Agreement: Gap period :{{64, 128, 256, 512}

Agreement: Value set of Gap size is chosen from:

- Gap size: {1/8, 1/4, 3/8, 1/2} * Gap period

Other

R1-163441	WF on scheduling delay of NPDSCH and NPUSCH	MediaTek, Panasonic, Huawei, Intel, ZTE, Nokia, Ericsson, Samsung

Decision: The document is noted.

Agreement: Defined 4 fixed values as {8, 16, 32, 64}, the scheduling delay for NPUSCH is the value indicated by DCI

R1-163868	WF on scheduling delay of NPDSCH and NPUSCH	Mediatek

Decision: The document is noted.

Agreements:

- 3 bits indicate number of valid DL subframes. The gap between end of NPDCCH and the start of the associated NPDSCH equals kdelay valid DL subframes + 4ms

- Two fixed sets for different Rmax for the respective search space, above and below a threshold.

  - If Rmax <128

    - {0,4,8,12,16,32,64,128}

  - If Rmax>=128,

    - {0,16,32,64,128, 256,512,1024}

Agreement: The gap indicated by DCI used for NPUSCH scheduling delay indicates absolute number of subframes.

Not treated.

R1-162835	On non-anchor PRBs for NB-IoT multi-carrier operation	Sharp

R1-163011	NB-PDSCH Design	Qualcomm Incorporated

R1-163263	Remaining issues onDL tranmsisison gap	MediaTek Inc.

##### System information and paging transmission

Agreements:

- Confirm the working assumption that the NB-SIB1 modification period is 40.96sec

- Confirm the working assumption to use 2 bits in MIB to indicate the two LSB of hyper SFN

Paging

R1-163012	System information and paging	Qualcomm Incorporated

Decision: The document (Section 3) is noted.

R1-162758	Remaining issues on paging for NB-IoT	ZTE

Decision: The document is noted.

Agreement: The first valid DL subframe which is >= 4 ms later than the end of DL assignment for paging is the first subframe of NB-PDSCH transmission containing paging information.

R1-163272	Considerations on Paging	Sony

Decision: The document is noted.

R1-163343	Paging transmission	Huawei, HiSilicon

Decision: The document is noted.

Agreement: Confirm that the invalid subframes indicated in NB-SIB1 for the anchor carrier are also invalid for POs.

Not treated.

R1-162975	Remaining details of Paging for NB-IoT	Intel Corporation

R1-163213	TDM transmission using UL/DL gaps	Spreadtrum Communications

R1-163276	Remaining Details on Paging Transmission	Samsung

##### NB-PSS and NB-SSS

Agreements:

- Editor to remove statement in the specification on the relationship between PSS and SSS in 36.211

- Within a subframe, NB-PSS/NB-SSS use the same antenna port

- Similar to LTE there is no tx diversity scheme specified.

R1-163493	NB-SSS design for NB-IoT	LG Electronics	(R1-162460)

Decision: The document is noted.

R1-162627	NB-SSS design	Huawei, HiSilicon

Decision: The document is noted.

R1-163407	NB-SSS Design	Qualcomm Incorporated	(R1-163013)

Decision: The document is noted.

R1-162976	NB-IoT Secondary Synchronization Signal Design	Intel Corporation

Decision: The document is noted.

R1-163492	WF on NB-SSS	LG

Decision: The document is noted.

Agreements:

- NB-SSS periodicity is 20ms

- The following information is indicated by the NB-SSS:

  - One of 504 PCIDs

  - 80ms boundary (4 values)

- NB-SSS sequence is composed of a length-131 frequency-domain ZC sequence and a binary scrambling sequence

  - Root indices {3..128}

- PCID is indicated by combination of ZC root index and a binary scrambling sequence

- 80ms boundary is indicated by the one of 4 time-domain cyclic shifts, {0, 33, 66, 99}

- The binary scrambling sequence is a Hadamard sequence

R1-163596	NB-SSS Evaluations	Huawei, HiSilicon	(R1-163577)

Decision: The document is noted.

Agreements:

- The mapping function from NB-IoT cell ID to the root index and the index of the scrambling sequence can follow the rules as:

u=modPCI,126+3

q=PCI126

- Cyclic shift Ci (0≤i≤3) is applied to the NB-SSS signal in the (8*k + 2*i)-th frame, where 0≤k≤127.

R1-163728	WF on scrambling sequence for NB-SSS 	LGE

Decision: The document is noted.

R1-163735	WF on scrambling sequence design for NB-SSS	Huawei, HiSilicon, Intel

Decision: The document is noted.

Agreements:

- NB-SSS sequence is generated according to R1-162976 with following correction:

- The binary scrambling sequence for NB-SSS is a Hadamard sequence generated as

NB-PSS

R1-162977	Baseband signal generation for NB-IoT downlink	Intel Corporation

Decision: The document (Section 2) is noted.

Agreement: NB-PSS is mapped to sub-carriers #0-10 of the NB-IoT carrier.

Agreement: NB-PDSCH/PDCCH on a given NB-IoT carrier are not mapped to the subframes containing NB-PSS/NB-SSS/PBCH on that carrier

  - Any transmission in such a subframe is postponed

Not treated.

R1-162759	Remaining issues on NB-PSS and NB-SSS design for NB-IoT	ZTE

R1-162775	NB-IoT - NSSS Design	Ericsson

R1-163333	Remaining details on synchronization signal design for NB-IoT	Nokia, Alcatel-Lucent Shanghai Bell

R1-163442	WF on scrambling sequence for NB-SSS	Huawei, HiSilicon

R1-163495	WF on PCID indication	Huawei, HiSilicon

##### Other

Not treated.

R1-162632	Invalid subframes configuration and handling	Huawei, HiSilicon

R1-162760	Collision handling for NB-IoT	ZTE

R1-163009	NB-PBCH Design	Qualcomm Incorporated

#### Uplink physical channels and signals

##### NB-PUSCH

SRS Collision Avoidance

R1-163741	WF on SRS avoidance in NB-IoT	Huawei, HiSilicon, Intel , Sony

Decision: The document is noted.

Agreement:

- For a NB-IoT symbol that is assumed not to be used by NB-IoT UE due to avoidance of LTE SRS collision:

  - Puncturing is applied on this symbol for single tone NB-PUSCH transmission and multi-tone NB-PUSCH transmission without repetition (Z=1)

MCS Tables & RV

Agreements:

- The predefined set of NB-PUSCH numbers of repetitions of the allocated RUs is 
{1, 2, 4, 8, 16, 32, 64, 128}.

- 3 bits in DCI indicate one value in the above set

General

R1-163737	WF on resource mapping and channel interleaver for NB-PUSCH 	Huawei, HiSilicon, Intel, Panasonic, MediaTek, Ericsson, ZTE, Samsung, Nokia

Decision: The document is noted.

Agreements:

- Resource mapping for NB-PUSCH is implemented in the order of frequency first then time in TS36.211.

  - The “frequency first then in time” resource mapping  continues until the end of each RU

- The channel interleaver defined in Section 5.2.2.8 in TS 36.212 is used for NB-PUSCH regardless of single tone or multi-tone transmission

  - For resource mapping

Implementation is up to the editors

UL tx gaps

R1-163800	WF on remaining details of timing relationships	Huawei, HiSi, Intel, Sony

Decision: The document is noted.

Agreements:

- The start of a DL transmission is >=3ms later than the end of any NPUSCH transmission for the same UE

- For any NPUSCH transmission with duration greater than X ms, a final UL gap of duration Y is inserted at the end of the NPUSCH transmission

Not treated.

R1-162461	Remaining issues on NB-PUSCH	LG Electronics

R1-162629	Remaining details of SRS avoidance	Huawei, HiSilicon

R1-162761	Remaining issues on uplink data transmission for NB-IoT	ZTE

R1-162776	NB-IoT - Remaining issues for NPUSCH design	Ericsson

R1-162905	NB-PUSCH design	Nokia, Alcatel-Lucent Shanghai Bell

R1-162978	Remaining details of NB-PUSCH	Intel Corporation

R1-163015	UL Data Channel Design	Qualcomm Incorporated

R1-163273	Further considerations on uplink transmission gaps	Sony

R1-163341	NB-PUSCH design	Huawei, HiSilicon

##### Uplink narrowband DM-RS

R1-163342	NB-DMRS design	Huawei, HiSilicon

Decision: The document is noted.

R1-163423	WF on narrowband DM-RS design for single tone transmission	LG

Decision: The document is noted.

Agreement for single-tone:

- Use element-wise product of length-16 Hadamard code and Gold-sequence.

- Use the LTE Gold-sequence in section 7.2 of TS 36.211

  - Initialization value of second m-sequence is 35

R1-163437	WF on DMRS multi-tone evaluation methods	Ericsson, Nokia, Lenovo, LG, ZTE

Decision: The document is noted.

Agreements for multi-tone:

- DMRS symbol locations for multitone are the same as for LTE

- For 3-tone case, 12 base sequences are provided (new sequences, not truncated versions of the 6-tone sequences)

- For 6-tone case, 14 base sequences are provided

- The available base sequence in a cell for each number of tones can be indicated by a SIB; otherwise, for 3-tone and 6-tone cases it is given by CellID mod 12 for 3 tone and mod 14 for 6 tone.

  - 12-tone case follows legacy behaviour for ascertaining the base sequence.

- Define 3 cyclic shifts for the 3-tone case and 4 for the 6-tone case

  - No CSs for 12-tone case

- Cyclic shifts are indicated by SIB and not indicated in DCI

- No OCC

- Sequences are QPSK in frequency domain

FFS:

- Exact sequences for 3-tones and 6-tones are FFS based on cross-correlation evaluations – revisit on Wed

  - Cross-correlation method to be used for comparison:

    - xcorr_coeffs =  NFFT * IFFT(seq1 .* conj(seq2), NFFT) / length(seq1)

  - Starting point for comparison and selection is the sequences proposed in R1-162462, R1-162730, R1-162762, R1-162906, and R1-163016.

R1-163424	WF on narrowband DM-RS design for multi tone transmission		LGE

Decision: The document is noted.

R1-163591	WF on UL Narrowband DMRS	Ericsson

Decision: The document is noted.

Agreements:

- For each of the 3-tone base DMRS sequence, three cyclic shifts are defined as 𝑟 ̅(𝑛)=𝑒^(𝑗 2𝜋/3 𝛼𝑛) 𝑟(𝑛) where 𝑟(𝑛) is a base sequence, 0≤𝑛<3 and 𝛼 is the cyclic shift 0≤𝛼<3.

- For each of the 6-tone base DMRS sequence, four cyclic shifts are defined as 𝑟 ̅(𝑛)=𝑒^(𝑗 2𝜋/6 𝛼𝑛) 𝑟(𝑛),  where r(n) is a base sequence, 0≤𝑛<5 and 𝛼 is the cyclic shift 𝛼={0, 1, 2, 4}

Agreement: Take 14 of the sequences in slide 6 of R1-163591, as below except 1,11

| m | (0), (1), (2), (3), (4), (5), |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 1 | 1 | 1 | 3 | -3 |
| 1 | 1 | 1 | -1 | -3 | -3 | 3 |
| 2 | 1 | 1 | 3 | 1 | -3 | 3 |
| 3 | 1 | -1 | -1 | -1 | 1 | -3 |
| 4 | 1 | -1 | 3 | -3 | -1 | -1 |
| 5 | 1 | 3 | 1 | -1 | -1 | 3 |
| 6 | 1 | -3 | -3 | 1 | 3 | 1 |
| 7 | -1 | -1 | 1 | -3 | -3 | -1 |
| 8 | -1 | -1 | -1 | 3 | -3 | -1 |
| 9 | 3 | -1 | 1 | -3 | -3 | 3 |
| 10 | 3 | -1 | 3 | -3 | -1 | 1 |
| 11 | 3 | 3 | -3 | -3 | -3 | -1 |
| 12 | 3 | -3 | 3 | -1 | 3 | 3 |
| 13 | -3 | 1 | 3 | 1 | -3 | -1 |
| 14 | -3 | 1 | -3 | 3 | -3 | -1 |
| 15 | -3 | 3 | -3 | 1 | 1 | -3 |

R1-163734	WF on 3 tone DMRS	Qualcomm, Samsung, Ericsson, ZTE, Panasonic, Nokia, Sony, LG, Intel

Decision: The document is noted.

Superseded Agreement: 14 base sequences for 3-tone case are defined as

| m | (0),(1),(2) |  |  |
| --- | --- | --- | --- |
| 0 | 1 | -3 | -3 |
| 1 | 1 | -3 | -1 |
| 2 | 1 | -3 | 1 |
| 3 | 1 | -3 | 3 |
| 4 | 1 | -1 | -3 |
| 5 | 1 | -1 | -1 |
| 6 | 1 | -1 | 3 |
| 7 | 1 | 1 | -3 |
| 8 | 1 | 1 | -1 |
| 9 | 1 | 1 | 3 |
| 10 | 1 | 3 | -3 |
| 11 | 1 | 3 | -1 |
| 12 | 1 | 3 | 1 |
| 13 | 1 | 3 | 3 |

R1-163806	WF on 3 tone DMRS	Qualcomm

Decision: The document is noted.

Makes a correction to the table.

Agreement: 12 base sequences for 3-tone case are defined as

| u | (0),(1),(2) |  |  |
| --- | --- | --- | --- |
| 0 | 1 | -3 | -3 |
| 1 | 1 | -3 | -1 |
| 2 | 1 | -3 | 3 |
| 3 | 1 | -1 | -1 |
| 4 | 1 | -1 | 1 |
| 5 | 1 | -1 | 3 |
| 6 | 1 | 1 | -3 |
| 7 | 1 | 1 | -1 |
| 8 | 1 | 1 | 3 |
| 9 | 1 | 3 | -1 |
| 10 | 1 | 3 | 1 |
| 11 | 1 | 3 | 3 |

The following working assumption is confirmed.

- Working Assumption:

  - Support 3 DM-RS symbols per 7 symbol period

  - The DM-RS sequence is obtained as

    - Use  single tone DM-RS PUSCH spread by length 3 OCC sequence defined for PUCCH

      - The OCC ID is pseudo-randomly selected (from existing OCCs) according to

  - Working Assumption can be changed to 1 DMRS symbol at RAN1#84bis if significant gain is not observed compared to 1 DMRS symbol.

Agreements:

- DM RS symbols for 3.75kHz ACK/NACK are contained in the first three NB-IoT symbols

- DM RS symbols for 15kHz ACK/NACK are contained in the NB-IoT symbols 2,3, and 4 (starting from symbol 0)

Not treated.

R1-162462	Remaining issues on narrowband DMRS design	LG Electronics

R1-162730	UL DMRS sequence for NB-IoT	Lenovo (Beijing) Ltd

R1-163016	Remaining details of UL Reference Signal Design	Qualcomm Incorporated

R1-162777	NB-IoT - UL Reference signals	Ericsson

R1-163419	On UL DMRS design for NB-IoT	Nokia, Alcatel-Lucent Shanghai Bell	(R1-162906)

R1-163432	DM-RS sequence for NB-IoT	ZTE	(R1-162762)

R1-163712	Evaluations on UL DMRS sequences in NB-IoT	Huawei, HiSilicon

##### Uplink control information

R1-163736	WF on resource allocation for A/N transmission	Huawei

Decision: The document is noted.

R1-163440	WF on UCI	LGE

Decision: The document is noted.

Agreement for UCI:

- Fixed baseline subcarrier, and

  - 3.75kHz: 3 bits for frequency allocation, 1 bit for time allocation

    - Starts from subcarrier 45, and offset grows negatively

      - Offset values are  {0, -1, -2, -3, -4, -5, -6, -7}

    - Time offset is 0,1 x 8ms

  - 15kHz: 2 bits for frequency allocation, 2 bit for time allocation

    - Starts from subcarrier zero, and offset grows positively

      - Offset values are  {0, 1, 2, 3}

    - Time offset is 0,1,2,3 x 2ms

Proposed Agreement for timing reference:

- Alt. 1: A/N transmission is relative to a configured time domain reference point (at least 12 ms later than the end of NPDSCH). The time domain reference point is configured by higher layer signalling.

- Alt. 2: A/N transmission delay is relative to the end of the corresponding NPDSCH transmission plus 12ms.

Discuss Thursday

R1-163803	WF on time domain resource allocation for ACK/NACK transmission	Huawei, HiSilicon, Intel, Panasonic, Samsung, ZTE

Decision: The document is noted.

Proposed Agreement:

- Alt 1: Baseline time point for time domain allocation:

  - A/N transmission is relative to the first time reference point which is at least 12 ms later than the end of NPDSCH

  - The period of the time reference point is fixed to [16]ms.

    - The start of the period is from (SFN#0, subframe#0).

- Alt. 2: A/N transmission delay is relative to the end of the corresponding NPDSCH transmission plus 12ms.

Email approval until April 21st to decide between these two alternatives – (Huawei)

R1-163740 	WF on numbers of repetitions for ACK/NACK	Nokia, ASB

Decision: The document is noted.

Agreement: The predefined set of numbers of repetition for ACK/NACK resource unit is {1, 2, 4, 8, 16, 32, 64, 128}

Not treated.

R1-163278	Remaining Issues on UCI Transmission	Samsung

R1-163261	Remaining issue on UCI transmission	MediaTek Inc.

R1-162628	UCI for NB-IoT	Huawei, HiSilicon

R1-162778	NB-IoT - Remaining issues for UCI	Ericsson

R1-162463	Remaining issues on UCI	LG Electronics

R1-162763	Remaining issues on UCI transmission for NB-IoT	ZTE

R1-162836	A/N resource indication for NB-IoT	Sharp

R1-162907	Remaining issues on UCI	Nokia, Alcatel-Lucent Shanghai Bell

R1-162980	Remaining details of UCI and DL HARQ-ACK feedback	Intel Corporation

R1-163017	Remaining details of UL control design	Qualcomm Incorporated

##### Random access

R1-163805	WF on remaining issues about NB-PRACH power settings	ZTE, HW,Hisilicon

Decision: The document is noted.

Agreement: For the values of preambleInitialReceivedTargetPower, reuse the legacy LTE value

R1-163597	WF on remaining details of NB-PRACH	Huawei

Decision: The document is noted.

Agreement: The adjustment of the uplink transmission timing is applied from the start of the first NPUSCH transmission which starts at least 12ms after the end of the corresponding timing advance command transmission.

Agreement: The NB-PRACH subcarrier locations are according to the following table

|  |  | “Frequency location in subcarrier offset” |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 12 | 24 | 36 | 2 | 18 | 34 |
| “Number of subcarriers” | 12 | 0 – 11 | 12 – 23 | 24 – 35 | 36 – 47 | 2 – 13 | 18 – 29 | 34 – 45 |
|  | 24 | 0 – 23 | 12 – 35 | 24 – 47 | Invalid | 2 – 25 | 18 – 41 | Invalid |
|  | 36 | 0 – 35 | 12 – 47 | Invalid | Invalid | 2 – 37 | Invalid | Invalid |
|  | 48 | 0 – 47 | Invalid | Invalid | Invalid | Invalid | Invalid | Invalid |

Agreements on NB-PDCCH order:

- NB-PDCCH order is carried in format N1

- 1 bit field indicates NB-PDCCH order

- The starting number of NPRACH repetitions is included  (2 bits)

- The remaining bits in DCI format N1 are set to ‘1’.

Agreements:

- For a given NB-PRACH resource configuration, the NB-PRACH resource

  - starts at 0ms within each SFN periodicity, and the NB-PRACH opportunity occurs at “Starting time of period” within each NB-PRACH periodicity.

Agreement: The symbol value for NB-PRACH is 1

R1-163588	WF on the UL grant in RAR	Huawei

Decision: The document is noted.

Agreement on the UL grant in RAR:

- UL subcarrier spacing: 3.75 kHz or 15 kHz (1 bit)

- Msg3 subcarrier allocation: Same as in UL grant on NB-PDCCH

- MCS/TBS: Use 3 states to indicate the 3 options for TBS = 88 bits from agreed NB-PUSCH MCS/TBS tables. Reserve the other states for future.

  - 000 is pi/2 BPSK for ST and QPSK for MT, N_RU=4

  - 001 is pi/4 QPSK for ST and QPSK for MT, N_RU=3

  - 010 is pi/4 QPSK for ST and QPSK for MT, N_RU=1

  - Others are reserved

- Scheduling delay: Same as NB-PUSCH

- Msg3 repetition number: Same as NB-PUSCH

- Padding bits are up to RAN2 when specifying RAR

R1-163598	WF on remaining details of NB-PRACH configuration	Huawei

Decision: The document is noted.

Agreement:

- Up to 3 NPRACH resource configurations can be configured in a cell

- The fraction of the total number of subcarriers that are reserved for single-tone transmission of MSG3 cannot be zero in at least one of the resources with the number of NB-PRACH repetitions other than (32, 64, and 128)

- If a UE selects the resource reserved for single tone MSG3, MSG3 is allocated with a single tone

- 2 bits indicate the starting subcarrier index of the range reserved for indication of UE support for multi-tone Msg3 transmission by {0, 1/3, 2/3, 1}xN_sc^NPRACH

- The other subcarriers in the NPRACH resource configuration are the range for single-tone transmission of MSG3

- UE select randomly from within its range of subcarriers

- This implies that if all PRACH resources have no range reserved for the indication of UE support for multi-tone MSG3, the UE must use the NPRACH resources reserved for single tone transmission of MSG3

- NOTE: This is not an indication of UE capability which is indicated in MSG3

R1-162983	Discussion on NPRACH baseband signal generation	NEC Corporation

Discussion: NEC commented that draft CR to 36.211 is updated in line with the proposal, no formal presentation.

Decision: The document is noted.

R1-162779	NB-IoT - Remaining issues for NPRACH	Ericsson

Decision: The document is noted.

Agreement: The repetitions of NPRACH are transmitted back-to-back in the contiguous subframes within one period for NPRACH.

Not treated.

R1-162464	Remaining issues on RACH transmission for NB-IoT	LG Electronics

R1-162630	Remaining NB-IoT random access physical layer aspects	Huawei, HiSilicon

R1-162764	Remaing issues on PRACH for NB-IoT	ZTE

R1-162780	NB-IoT - Remaining issues for random access procedure	Ericsson

R1-162857	NB-PRACH design and resource partition	CMCC

R1-162908	Remaining issues on random access	Nokia, Alcatel-Lucent Shanghai Bell

R1-162962	PRACH for NB-IoT	InterDigital Communications

R1-162981	Remaining details of random access for NB-IoT	Intel Corporation

R1-162984	Discussion on description of NPRACH frequency hopping	NEC Corporation

R1-163018	Remaining details of Random Access Channel	Qualcomm Incorporated

R1-163262	UL grant in RAR	MediaTek Inc.

R1-163571	WF on Multi-tone Msg3 indication for NB-IoT	ZTE, CMCC

##### Other

Power control

R1-162631	Uplink power control	Huawei, HiSilicon

Decision: The document is noted.

R1-162921	NB-IoT - Remaining issues on UL power control	Ericsson

Decision: The document is noted.

Agreements:

- MNPUSCH,c(i): {1/4, 1, 3, 6,12}  (reflecting UL transmission resource bandwidth normalized by 15 kHz)

- No power adjustment parameter in DCI, i.e. fc(i)=0

- Power control for ACK/NACK: alpha = 1

- If number of repetitions (including the initial transmission) of the allocated RUs > 2, then UE uses max power

- The set of values for power offset between NB-RS and LTE CRS is {-6, -4.77, -3, -1.77, 0, 1, 1.23, 2, 3, 4, 4.23, 5, 6, 7, 8, 9} dB.

- Value set of powerRampingStep is the same as LTE: {0, 2, 4, 6} dB

R1-163445	WF on NB-PRACH power ramping	ZTE, Ericsson

Decision: The document is noted.

R1-163595	WF on NB-PRACH power ramping	ZTE, Huawei, HiSilicon

Decision: The document is noted.

Agreement:

- It is ran1 understanding that the following power ramping equation can be reused for NB-PRACH but one item (– 10*log10(numRepetitionPerPreambleAttempt)   to adjust  repetition’s effect need to be added

  - Set PREAMBLE_RECEIVED_TARGET_POWER to preambleInitialReceivedTargetPower + DELTA_PREAMBLE + (PREAMBLE_TRANSMISSION_COUNTER – 1) * powerRampingStep – 10*log10(numRepetitionPerPreambleAttempt)

DELTA_PREAMBLE is set to 0

PHR:

Agreements:

- NB-PHR is computed based on a 15kHz single-tone transmit power for NB-PUSCH data transmission regardless of the actual subcarrier spacing: 	
PH(i) = PCMAX,c(i)-{PO_NPUSCH,c(1)+αc(1)∙PLc}

- There will be 4 reportable values of PHR

  - Exact values to be determined by RAN4 and specified in 36.133.

Not treated.

R1-162904	UL power control for NB-IoT	Nokia, Alcatel-Lucent Shanghai Bell

R1-162765	Remaining issues on UL power control for NB-IoT	ZTE

R1-162837	Remaining issues on transmit power control for NB-IoT	Sharp

R1-163279	Remaining Details on Uplink Power Control	Samsung

R1-163443	WF on uplink power control for NB-PUSCH data transmission	ZTE, Ericsson, Nokia ,Sharp, Qualcomm

R1-163444	WF on uplink power control for ACK/NACK transmission	ZTE, Nokia

R1-163496	[Draft] WF on power control of uplink ACK/NAK	Huawei, HiSilicon, Samsung, Ericsson

R1-163497	[Draft] WF on Power Headroom Report	Huawei, HiSilicon

R1-163498	[Draft] WF on bandwidth component of uplink power control	Huawei, HiSilicon, Samsung, Sharp, Ericsson

R1-163499	[Draft] WF on fc(i) in uplink power control	Huawei, HiSilicon, Samsung, Intel

R1-163599	WF on UL Narrowband DMRS	Ericsson

#### Other

Valid Subframes

R1-162903	Remaing issues on NB-PDSCH	Nokia, Alcatel-Lucent Shanghai Bell

Decision: The document is noted.

Proposal 1: Valid subframes for downlink transmission parameter (downlinkBitmapNB) is either 10 bits or 40 bits. If the parameter is not present, then subframes that have not been configured or used for MBSFN, NB-PBCH, NB-PSS, and NB-SSS, are considered as valid subframes.

R1-162781	NB-IoT - Valid subframes	Ericsson

Decision: The document is noted.

R1-163590	WF on valid/invalid subframe configurations and MBSFN subframe configuration	Panasonic, ZTE, Mediatek

Decision: The document is noted.

Agreements:

- Valid subframe configuration

  - Valid subframe configuration are optionally bit-map configured over 10ms or 40ms for inband and 10ms for standalone/guardband by NB-SIB1.

    - NOTE: For standalone/guardband this optional signaling may be useful for future coexistence

  - In the subframes that contain PSS/SSS/PBCH/NB-SIB1 in the carrier that contain PSS/SSS/PBCH/NB-SIB1

    - these subframes are invalid subframes i.e. to postpone the transmission regardless of the signalling.

  - For the remaining subframes,

    - If no bit-map is provided, these subframes are valid subframes

    - Otherwise, to follow the signalling.

- For inband

  - NB-IoT UE do not know about MBSFN subframes; they are transparent to the NB-IoT UE.

  - The UE may assume the same assumptions on legacy CRS in all valid subframes for inband

  - It is up to the network to decide whether MBSFN subframes are configured as valid or invalid.

- For valid subframe configurations for multi-PRB

  - One bit-map for the PRB with NBPSS/NBSSS/SIB1 in SIB1

  - RAN2 can decide of one or both of:

    - An additional bit-map for other PRB may be provided with dedicated signaling

    - Additional bit-map(s) for other PRBs within SIB

      - Up to RAN2 if this is per operation mode or across operation modes

R1-163589	WF on Collision Handling for NB-IoT	Ericsson

Decision: The document is noted. Discuss offline

Tx collisions

R1-162633	Procedures for colliding transmissions	Huawei, HiSilicon

Decision: The document is noted.

Agreements:

- For standalone and guard band operations, NB-IoT channels and signals, except NB-PBCH, occupy the REs corresponding to “LTE CRS” calculated according to physical Cell_ID.

- There is no NB-IoT signaling of LTE CSI-RS or PRS.

- NB-PDCCH and NB-PDSCH are rate matched around LTE CRS for in-band operation.

R1-163553	WF on scheduling collision	Sony, Intel, MediaTek, Sequans

Decision: The document is noted. Continue offline

R1-163658	WF on scheduling collision	Sony, Intel, MediaTek, Sequans, Sierra Wireless, Virtuosys, CATT, Panasonic, Huawei, HiSilicon, Neul, ZTE, Samsung

Decision: The document is noted.

Agreement:

- An NB-IoT UE that has received a grant from an NB-PDCCH is not required to monitor NB-PDCCH for any further DL or UL grant during the time period between the end of the NB-PDCCH that schedules the grant and the start of the corresponding NB-PDSCH or NB-PUSCH transmission

R1-163766	WF on PRACH Collision Handling for NB-IoT	Ericsson, ZTE, Samsung

Decision: The document is noted.

Agreement:

- UEs in lower coverage treat NPRACH opportunities that collide with NPRACH opportunities of higher coverage level(s) as invalid.

  - UEs only use valid NPRACH opportunities for Msg1 transmission

Not treated.

R1-162782	NB-IoT - Collision handling	Ericsson

R1-162909	Collision handling for NB-IoT	Nokia, Alcatel-Lucent Shanghai Bell

R1-163214	On multi-carrier configuration signaling	Spreadtrum Communications

## LTE Release 14

### Enhanced Licensed-Assisted Access to Unlicensed Spectrum - WID in RP-152272.

R1-163892	Chairman's Notes of Agenda Item 7.3.1 on eLAA	Ad-hoc chair (Qualcomm)

The document was presented by Wanshi Chen from Qualcomm and .provides the outcomes of the eLAA session.

Decision: The document is endorsed. Contents incorporated below.

Not treated.

R1-162816	On LTE FS3 enhancements to support LAA UL	Nokia, Alcatel-Lucent Shanghai Bell

#### PUSCH design

R1-162728	Summary on [84-14] PUSCH frame structure in eLAA	Huawei, HiSilicon

Decision: The document is noted.

Agreement:

- One symbol in a subframe containing PUSCH can be blanked

  - FFS whether to have more than one blanked symbol

  - FFS whether it’s cell-specific or UE-specific

R1-163021	Email discusion summary on UL traffic multiplexing	Qualcomm Incorporated

Decision: The document is noted.

Scheduling and HARQ

R1-162322	Discussion on UL scheduling for LAA	ZTE Microelectronics, Nubia

Decision: The document is noted.

R1-162466	Multi-subframe scheduling in LAA	LG Electronics

Decision: The document is noted.

R1-162858	Further discussion on issues related to PUSCH transmission for LAA	CMCC

Decision: The document is noted.

R1-163565	WF on PUSCH multi-subframe scheduling in LAA	LG Electronics, Qualcomm, Sharp

Decision: The document is noted.

Agreements:

- DCI format(s) to schedule PUSCH transmission in k<= N subframes with single TB per subframe or two TBs per subframe

  - Value(s) of N is FFS

  - Value N is either semi-statically configured or hard-coded, to be further decided

- DCI format(s) will have the following scheduling information types:

  - Type A: common to all the scheduled subframes (appearing only once in a DCI)

    - carrier indicator, resource assignment, Cyclic shift for DM RS and OCC index

  - Type B: subframe specific information (appearing N times for N subframes scheduling)

    - NDI

  - FFS MCS is type A or type B

  - FFS HARQ process number and redundancy version are type A or type B

  - FFS details of scheduling timing indication, and whether it’s type A or type B

  - FFS: Type C: applied only to one of the scheduled subframes (appearing only once in a DCI)

    - CSI request, SRS request, TPC

  - Note: there are maybe other information fields in DCI, to be decided later

- Note: the DCI formats here may not be a complete list, e.g., depending on discussion on resource allocation for PUSCH

R1-163575	WF on eLAA PUSCH retransmission with synchronous HARQ 	Huawei, HiSilicon, LGE

Decision: The document is noted. Continue discussion till RAN1#85

R1-163572	Way forward on the PSD Issue with RB-level Multi-cluster Transmission	Intel, Broadcom, Cable Labs

Decision: The document is noted. Continue discussion offline

R1-162937	Control channel design for eLAA	Mediatek

Decision: The document is noted.

Resource Allocation

R1-162355	On the Intermodulation Distortion Issue with Multi-cluster Transmission	Intel Corporation

Decision: The document is noted.

R1-163020	PUSCH resoure allocation granularity	Qualcomm Incorporated

Decision: The document is noted.

R1-162660	UL Waveforms for PUSCH	Samsung

Decision: The document is noted.

R1-163573	Way forward on the IMD issue with Multi-cluster Transmission (>2)	Intel, Samsung, Broadcom

Decision: The document is noted.

R1-163563	WF on PUSCH resource allocation	LG Electronics, Samsung, ITL

Decision: The document is noted.

Agreements:

- For eLAA PUSCH transmission, one interlace is the basic unit of resource allocation, which is composed of 10RBs for 20MHz

  - Working assumption: the 10RBs are spaced equally in frequency domain for 20MHz

    - Ex for 20MHz eLAA SCell: interlace 0 is composed of RBs 0,10,20,...,90

    - Send an LS to RAN4 asking whether or not RAN4 sees issues with the working assumption. RAN1 also discussed the possibility of having unequal spacing in frequency domain for the 10-RB interlace based resource allocation – Jeongho (Intel) - R1-163703.

  - FFS the case of other system bandwidth(s)

- A UE can be assigned one or more interlaces

  - The total number of RBs used for transmission should be a multiple of 2,3 and 5

  - Decide one of the following alternatives:

    - Alt 1: UL resource allocation type 0 is used to allocate multiple interlaces to a UE

      - UL grant indicates start index and allocated number of interlaces with consecutive indices

    - Alt 2: bitmap based resource allocation

    - Alt 3: predefined resource allocation patterns

  - FFS: excluding some UL RBs from the resource allocation

R1-163703	[Draft] LS on PUSCH transmission for eLAA	Intel Corporation

Decision: The document is noted and final LS is agreed in R1-163683.

R1-163584	WF on PUSCH transmissions in eLAA	Huawei, HiSilicon

Decision: The document is noted.

Agreement:

- Confirm the working assumption

  - The minimum latency is 4ms between the subframe carrying the UL grant and subframe(s) of the corresponding PUSCH(s)

Agreement:

- For the UL DMRS for eLAA,

  - Reuse the legacy UL DMRS generation sequence

    - That is, the sequence is generated based on the transmitted bandwidth and mapped in a way similar to that of the legacy resource allocation type 1

  - The symbol position of the UL DMRS of eLAA is the same as the UL DMRS position in LTE PUSCH subframe

  - The frequency position of the UL DMRS is the same as the PUSCH REs

Agreements:

- Dynamic signaling indicates whether PUSCH in a UL subframe is transmitted from

  - Start of DFTS-OFDM symbol 0 or

  - Start of DFTS-OFDM symbol 1

  - FFS: Within DFTS-OFDM symbol 0

- Dynamic signaling indicates whether PUSCH in a UL subframe is transmitted up to OFDM symbol 13 or OFDM symbol 12

- Any combination of above options can be enabled by the dynamic signaling

R1-163758	WF on PUSCH structure 	LG Electronics, CATT, WILUS

Decision: The document is noted.

Friday

R1-163829	WF on further details of DCI contents for multi-TTI transmission	Qualcomm

R1-163830	WF on multi-TTI scheduling Mediatek

Above WFs are for email approval until May 5th  – Joon (LGE)

Not treated.

R1-162130	Scheduling design for eLAA	Huawei, HiSilicon

R1-162131	Discussion on CCA gap and symbol position for PUSCH and SRS for eLAA	Huawei, HiSilicon

R1-162259	PUSCH design for Rel-14 eLAA	CATT

R1-162260	UL scheduling for Rel-14 eLAA	CATT

R1-162320	HARQ for UL LAA	ZTE Microelectronics, Nubia

R1-162321	On UL Resources Allocation for PUSCH	ZTE Microelectronics, Nubia

R1-162323	Consideration of LBT failure in LAA UL HARQ operation	ZTE Microelectronics, Nubia

R1-162351	Uplink Waveform for eLAA	Intel Corporation

R1-162352	Signaling for Flexible Timing UL Scheduling	Intel Corporation

R1-162353	On the Support of Cross-burst Scheduling	Intel Corporation

R1-162354	On the Coexistence Issue with Multi-Cluster Transmission	Intel Corporation

R1-162439	PUSCH resource allocation for LAA	NEC

R1-162442	Discussion on multiple-subframe scheduling for LAA	NEC

R1-162443	Discussion on LAA UL partial subframe transmission	NEC

R1-162465	PUSCH resource allocation in LAA	LG Electronics

R1-162554	eLAA Measurements and Carrier Selection Procedure	Sony

R1-162583	Resource allocation for RB interleaved PUSCH for eLAA	Huawei, HiSilicon

R1-162604	Design for frame structure 3 with DL and UL subframes for eLAA	Huawei, HiSilicon

R1-162661	Discussion on DTX handling methods for UL HARQ in eLAA	Samsung

R1-162662	Discussion on UL power control for UL LAA	Samsung

R1-162663	Discussion on scheduling options for eLAA	Samsung

R1-162664	Discussion on multi-subframe scheduling for eLAA	Samsung

R1-162755	Considerations for eLAA UL Carrier Allocation	Sequans Communications

R1-162756	RB allocation for PUSCH	Sequans Communications

R1-162783	On LAA UL/DL Scheduling Combinations	Nokia, Alcatel-Lucent Shanghai Bell

R1-162784	On LAA UL scheduling and UL grant enhancements	Nokia, Alcatel-Lucent Shanghai Bell

R1-162785	On LAA UL HARQ operation	Nokia, Alcatel-Lucent Shanghai Bell

R1-162799	Discussion on PUSCH design for eLAA UL	NTT DOCOMO, INC.

R1-162800	Discussion on UL scheduling design for eLAA	NTT DOCOMO, INC.

R1-162817	UL waveform for LAA PUSCH	Nokia, Alcatel-Lucent Shanghai Bell

R1-162838	Uplink subframe structure in LAA carrier	Sharp

R1-162910	On UL data transmission for eLAA	INTERDIGITAL COMMUNICATIONS

R1-162936	eLAA PUSCH design	Mediatek

R1-162985	Resource allocation and channel access for PUSCH	ITRI, Acer Inc.

R1-163019	PUSCH design details	Qualcomm Incorporated

R1-163022	Control details for UL	Qualcomm Incorporated

R1-163023	PUSCH power control	Qualcomm Incorporated

R1-163131	Discussions on the design of PUSCH for LAA	CATR

R1-163138	Uplink Resource Allocation Design for Enhanced LAA	Ericsson

R1-163139	On DMRS design for UL in Enhanced LAA	Ericsson

R1-163140	Multi-subframe scheduling design for Enhanced LAA	Ericsson

R1-163141	On UL Subframe Structure for Enhanced LAA	Ericsson

R1-163165	Discussion on Partial Subframe Utilization for PUSCH	Beijing Xinwei Telecom Techn.

R1-163207	Uplink Subframe Structure	Panasonic

R1-163208	Multi-Cluster Scheduling Methodology	Panasonic

R1-163564	WF on PUSCH/SRS structure in LAA	LGE

R1-163576	WF on blocked CCA at UE	Samsung, Panasonic, ZTE, Nubia

R1-163583	WF on PUSCH frame structure in eLAA	Huawei, HiSilicon, CATR, ETRI

R1-163898	WF on DCI for PUSCH multi-subframe scheduling in LAA	LGE

#### SRS design

R1-162729	Summary on [84-15] SRS without PUSCH in eLAA	Huawei, HiSilicon

Decision: The document is noted.

R1-163585	WF on SRS without PUSCH in eLAA	Huawei, HiSilicon, Qualcomm

Decision: The document is noted.

Agreement:

- Working assumption: Periodic SRS without PUSCH is NOT supported in Rel-14 eLAA WI

- A-periodic SRS without PUSCH is supported at least in DL ending partial subframes in Rel-14 eLAA WI

  - FFS the case of UL subframes

- LBT for aperiodic SRS without PUSCH

  - 25us one-shot LBT within eNB MCOT;

  - Cat 4 LBT outside of eNB MCOT

    - FFS contention window size parameters, including possibility of non-adapting contention window size

  - FFS no LBT if SRS is transmitted within 16us of the DL transmission within eNB MCOT

  - FFS whether or not the eNB can indicate a LBT type

Agreement:

- Aperiodic SRS without PUSCH can be triggered by dynamic signaling at least via DL grants

  - UE is configured by RRC one or more sets of SRS parameters

    - The configuration details are FFS

  - FFS other possible triggering mechanisms,  e.g.:

    - Group DCI

    - UE-specific DCI

    - One or several of the above options may apply

R1-163586	WF on general SRS aspects in eLAA	Huawei, HiSilicon, CATT

Decision: The document is noted.

Agreement:

- Only wideband SRS transmission in supported in eLAA

  - Existing max # SRS RBs for a given system bandwidth is the baseline

    - FFS whether or not to extend/shift to # of RBs > max # RBs in the legacy case

- Working assumption: SRS is based on legacy comb structure

  - As a baseline, comb = 2 and 4

  - FFS whether or not support different comb value(s)

Proposal – Continue discussion till RAN1#85

- Alt 1: A-periodic SRS without PUSCH is supported also in regular UL subframes, where the SRS is located in the 2nd symbol of the subframe

  - Supported by: Nokia, ASB, Qualcomm, LGE (2nd choice), Huawei, HiSilicon, NTT DOCOMO, Kyocera, Intel, WILUS, ITRI, ZTE, Ericsson (2nd choice)

  - Objected by: Samsung, Panasonic, MediaTek, Broadcom

- Alt 2: A-periodic SRS without PUSCH is supported also in regular UL subframes, not to be multiplexed with other UEs’ PUSCH transmission in the same subframe and the SRS is located in the last symbol of the subframe

  - Supported by: Ericsson, CATT, LGE, MediaTek (2nd choice)

  - Objected by: Panasonic, Broadcom

- Alt 3: A-periodic SRS without PUSCH is NOT supported also in regular UL subframes

  - Supported by: Samsung, MediaTek, Panasonic, Sharp, Broadcom, Lenovo

  - Objected by: Huawei, HiSilicon, NTT DOCOMO, Qualcomm, Nokia, ASB, CATT

Agreement: SRS, if present in a UL subframe, is transmitted at the end of the subframe

Not treated.

R1-162101	Further details on SRS transmission for eLAA	Huawei, HiSilicon

R1-162261	SRS design for Rel-14 eLAA	CATT

R1-162324	SRS design for eLAA UL	ZTE Microelectronics, Nubia

R1-162356	SRS Transmission for eLAA	Intel Corporation

R1-162467	SRS transmission in LAA	LG Electronics

R1-162665	Discussion on SRS for UL LAA	Samsung

R1-162752	SRS design for eLAA	Kyocera Corporation

R1-162801	Discussion on SRS design for eLAA UL	NTT DOCOMO, INC.

R1-162911	On SRS for eLAA	INTERDIGITAL COMMUNICATIONS

R1-162918	On LAA SRS Design	Nokia, Alcatel-Lucent Shanghai Bell

R1-162938	Considerations on SRS for eLAA	Mediatek

R1-162986	SRS position in eLAA	ITRI

R1-163024	SRS design details	Qualcomm Incorporated

R1-163124	Aperiodic SRS transmission without PUSCH for eLAA	ETRI

R1-163142	On Aperiodic SRS Transmission with PUSCH for Enhanced LAA	Ericsson

R1-163143	On Aperiodic SRS Transmission without PUSCH for Enhanced LAA	Ericsson

R1-163297	SRS transmission in eLAA	WILUS Inc.

R1-163304	SRS design in Rel-14 eLAA	Innovative Technology Lab Co.

#### Uplink control information on an LAA SCell

R1-162262	UCI transmission for Rel-14 eLAA	CATT

Decision: The document is noted.

R1-162939	PUCCH design in eLAA	Mediatek

Decision: The document is noted.

R1-163145	Transmission of uplink control information on an LAA SCell in DL partial subframes	Ericsson

Decision: The document is noted.

Agreement:

- PUCCH on LAA SCell is not introduced in eLAA within the current scope of the work item

  - The introduction of PUCCH at a later stage in Rel-14 is not precluded

Agreement:

- Simultaneous L-cell PUCCH and LAA SCell PUSCH transmission is supported by the UE

  - Note: Not configuring this would impact PUSCH transmission opportunities on the LAA SCell

  - Note: whether this feature is optional or mandatory is a separate discussion

  - Note: this does not necessarily imply that a UE needs support simultaneous PUCCH and PUSCH within one band or adjacent bands

Working assumption:

- eLAA supports transmission of UCI including at least HARQ-ACK on PUSCH within a “cell group” consisting of only LAA SCells at least for self-scheduling

  - No PUCCH on any SCell in the CG

  - This cell group is not referring to a PUCCH cell group

  - FFS: Timing relationship between DL transmissions and HARQ-ACK

Agreement:

- Transmission of aperiodic CSI on an LAA SCell is supported at least for aperiodic CSI for an unlicensed carrier

  - Both aperiodic CSI only (without UL-SCH) and aperiodic CSI with UL-SCH are supported

    - FFS the conditions for aperiodic CSI only on an LAA SCell

- Transmission of periodic CSI on an LAA SCell is not supported within the current scope of the eLAA work item

  - The introduction of periodic CSI on an LAA Scell at a later stage in Rel-14 is not precluded

Friday: Above Working assumption is replaced by:

Working assumptions:

- eLAA supports transmission of UCI including at least HARQ-ACK on PUSCH within a “UCI cell group” consisting of only LAA SCells at least for self-scheduling

  - No PUCCH on any SCell in the UCG

  - This cell group is not referring to a PUCCH cell group

  - FFS: Timing relationship between DL transmissions and HARQ-ACK

  - FFS: Whether the UCI cell group can also include an SCell in the licensed band

- All HARQ-ACKs for SCells within the UCI cell group are always carried on PUSCH on one or more SCells within the UCI cell group when the UCI cell group is configured

Email approval about above FFS parts and confirm WA until May 5th – (Ericsson)

Not treated.

R1-162318	SR/P-CSI transmission on eLAA carrier	Guangdong OPPO Mobile Telecom.

R1-162319	HARQ-ACK transmission on eLAA carrier	Guangdong OPPO Mobile Telecom.

R1-162325	Discussion on UCI transmission on LAA SCell	ZTE Microelectronics, Nubia

R1-162357	Transmission of Uplink Control Information on an LAA Scell	Intel Corporation

R1-162441	PUCCH enhancement for LAA	NEC

R1-162468	UCI transmission in LAA	LG Electronics

R1-162469	PUCCH design in LAA	LG Electronics

R1-162584	PUCCH format for eLAA	Huawei, HiSilicon

R1-162605	UCI transmission for eLAA	Huawei, HiSilicon

R1-162666	Discussion on PUCCH for UL LAA	Samsung

R1-162667	Discussion on UCI transmission for eLAA	Samsung

R1-162731	UCI transmission on LAA Scell	Lenovo (Beijing) Ltd

R1-162802	Discussion on UCI transmission and PUCCH design for eLAA UL	NTT DOCOMO, INC.

R1-162818	DL HARQ-ACK for eLAA	Nokia, Alcatel-Lucent Shanghai Bell

R1-162819	UL waveform for control information in eLAA	Nokia, Alcatel-Lucent Shanghai Bell

R1-162859	Discussion on Uplink control information transmission on LAA Scell	CMCC

R1-162912	UCI on eLAA SCell	INTERDIGITAL COMMUNICATIONS

R1-163025	PUCCH design details	Qualcomm Incorporated

R1-163144	Transmission of uplink control information on an LAA SCell in UL subframes	Ericsson

R1-163566	WF on HARQ-ACK transmission for LAA SCell	LG Electronics, Samsung, Sharp

R1-163729	WF on UCI transmission for LAA sCell	NTT DOCOMO

R1-163730	WF on channel access of UCI transmission for LAA sCell	NTT DOCOMO, OPPO

R1-163731	WF on PUCCH on LAA sCell	NTT DOCOMO

R1-163759	WF on HARQ-ACK transmission for LAA Scell	LG Electronics, Samsung, Sharp, Intel

#### PRACH design

R1-162471	Random access procedure in LAA	LG Electronics

Decision: The document is noted.

R1-163785	WF on PRACH procedure in LAA 	LG Electronics, Intel, Samsung

Decision: The document is noted.

Working Assumption:

- PRACH resource configuration in time and frequency in LAA SCell:

  - RRC configuration of frequency resource(s), sequence set, and time domain resources

    - Omitting time domain resources from the RRC configuration can be studied, considering e.g. interactions with LBT.

  - FFS whether any of the following can be dynamically adapted by PDCCH, or deterministically derived by the UE:

      - time domain resources

      - frequency resources for actual transmission

      - sequences for actual transmission

    - If using PDCCH, FFS whether common or UE-specific

R1-163818	WF on PRACH waveform		Nokia, Alcatel-Lucent Shanghai Bell, CATT, Panasonic, Samsung

Decision: The document is noted.

Proposals:

- LAA-PRACH is based on LTE PRACH format 0 and/or format 4 repeated N times in frequency

  - The value of N is configurable and includes at least {1, 2, ...}

  - The placement of the PRACH clusters in frequency shall be able to fulfil the ETSI regulation on occupied channel bandwidth

    - PRBs for LAA-PRACH for each value of N are defined in the specification, exact positions are FFS

  - FFS: CP of the PRACH format 0 is shortened by [1 symbol] to provide room for LBT

  - FFS: The value of Timing Advance for LAA-PRACH transmission is configurable

R1-163853	WF on PRACH waveform		Ericsson

Decision: The document is noted.

Proposal:

- Preamble durations of up to 1 ms are supported in eLAA.

- PRACH is transmitted in one or more of RB interlaces

  - The interlaces to use is configured by RRC

- PRACH preamble is based upon repetition of one DFTS-OFDM symbol of the same length as a PUSCH DFTS-OFDM symbol.

- A region for LBT is provided by not transmitting one or more OFDM symbols

Continue evaluation both options as listed below till RAN1#85

R1-163821	On Performance of PRACH for Enhanced LAA	Ericsson	(R1-163147)

Decision: The document is noted.

Not treated.

R1-162132	PRACH for eLAA	Huawei, HiSilicon

R1-162263	PRACH design for Rel-14 eLAA	CATT

R1-162326	PRACH Design for LAA	ZTE Microelectronics, Nubia

R1-162358	PRACH Design for eLAA	Intel Corporation

R1-162470	Random access preamble in LAA	LG Electronics

R1-162527	Discussion on PRACH design for LAA	HTC

R1-162668	PRACH Transmission Aspects	Samsung

R1-162669	Discussion on enhanced RA procedure for UL LAA	Samsung

R1-162803	Discussion on PRACH for eLAA UL	NTT DOCOMO, INC.

R1-162919	Discussion on PRACH design for eLAA	Nokia, Alcatel-Lucent Shanghai Bell

R1-162940	PRACH desgin in eLAA	Mediatek

R1-162995	Discussion on PRACH for eLAA	Shenzhen Coolpad Technologies

R1-163026	PRACH design details	Qualcomm Incorporated

R1-163146	PRACH Design for Enhanced LAA	Ericsson

R1-163209	PRACH Bandwidth on Unlicensed Carriers	Panasonic

#### Channel access framework

Extending the agreement reached during the Rel-13 WI on LAA and for corresponding agreed channels and signals

Agreement:

- If the sum total duration of DL and UL transmissions [and UL LBT] is less than the obtained channel occupancy duration, it is sufficient for the UE(s) to perform a single 25us LBT to access the channel and perform UL transmission

  - FFS the conditions, if any, on the usage of 25us LBT especially w.r.t. traffic class

  - FFS the […] part

R1-163632	WF on UL Channel Access Schemes	Ericsson

Revised in R1-163810

Decision: The document is noted.

R1-162102	Further details on UL LBT to enable UE multiplexing of uplink transmissions	Huawei, HiSilicon

Decision: The document is noted.

R1-162851	Discussion on LAA UL LBT Design	BROADCOM LIMITED

Decision: The document is noted.

R1-162852	Discussion on LAA SRS and interlaced transmission for PUSCH	BROADCOM LIMITED

Decision: The document is noted.

R1-162982	Energy Detection Threshold for UL LAA Channel Access Mechanism	CableLabs

Decision: The document is noted.

R1-163150	On Channel Access Procedures for Enhanced LAA	Ericsson

Decision: The document is noted.

R1-163148	On Performance of UL Channel Access Procedures for PUSCH with Self-Carrier Scheduling	Ericsson

Decision: The document is noted.

R1-163707	Way forward on UL LBT	Intel, Samsung

Decision: The document is noted.

Proposal:

- The scheduled UL subframe is not within an obtained channel occupancy duration, Cat-4 LBT is used for UL LBT.

  - FFS: Whether the UL maximum contention window size can be smaller than that for DL category 4 LBT

- Energy detection threshold used for UL LBT for both self-carrier and cross-carrier scheduling is based on the energy detection threshold rule for DL LBT.

- LBT for UL grant only transmission by eNB follows Cat-4 LBT with high priority.

  - FFS: the priority class to be used for UL grant only transmission.

R1-163810	WF on UL Channel Access Schemes	Ericsson

Decision: The document is noted. Email approval about channel access until May 5th - (Ericsson)

Not treated.

R1-162128	UL category 4 LBT with eNB assistance	Huawei, HiSilicon

R1-162129	Contention window size adjustment for UL category 4 LBT for eLAA	Huawei, HiSilicon

R1-162264	UL channel access schemes for Rel-14 eLAA	CATT

R1-162327	Frame design and signalling for LAA UL based on FS3	ZTE Microelectronics, Nubia

R1-162328	Discussion on UL LBT for LAA	ZTE Microelectronics, Nubia

R1-162329	LBT for Frequency reuse and Multiplexing in Uplink	ZTE Microelectronics, Nubia

R1-162330	TPC and PHR for UL LAA	ZTE Microelectronics, Nubia

R1-162331	On UL Channel Access Framework for Enahnced LAA	Fujitsu

R1-162359	UL LBT Details	Intel Corporation

R1-162360	Introduction of non-scheduled UL access for eLAA	Intel Corporation

R1-162440	Channel access for LAA UL	NEC

R1-162472	Evaluation of LBT operation in LAA UL	LG Electronics

R1-162473	LBT schemes in LAA UL	LG Electronics

R1-162474	Impact of UE-to-UE interference on LAA performance	LG Electronics

R1-162528	Dynamic carrier selection for LAA	HTC

R1-162555	UL Channel access mechanism design in eLAA	Sony

R1-162556	eLAA contention window size adjustments	Sony

R1-162557	UE measurements and reporting in eLAA	Sony

R1-162575	LBT for UL grant transmission	Intel Corporation

R1-162602	Discussion on the applications of LBT options in eLAA	Huawei, HiSilicon

R1-162603	Other issues related to LBT for eLAA	Huawei, HiSilicon

R1-162670	LBT for cross-carrier scheduling	Samsung

R1-162671	Discussion on LBT for self-carrier scheduling	Samsung

R1-162672	Discussion on LAA-LAA co-existence	Samsung

R1-162732	LBT mechanism for LAA uplink	Lenovo (Beijing) Ltd

R1-162753	Channel Access Scheme for UE multiplexing	Kyocera Corporation

R1-162804	Discussion on channel access framework for eLAA UL	NTT DOCOMO, INC.

R1-162839	Methods of UL LAA channel access	Sharp

R1-162913	UL channel access for eLAA	INTERDIGITAL COMMUNICATIONS

R1-162920	Channel Access for LAA UL	Nokia, Alcatel-Lucent Shanghai Bell

R1-162941	eLAA Uplink channel access	Mediatek

R1-162942	On priority classes for uplink LBT	Mediatek

R1-163027	UL channnel access	Qualcomm Incorporated

R1-163028	Cross-carrier scheduling for UL	Qualcomm Incorporated

R1-163123	Discussion on UL LBT for eLAA	ETRI

R1-163149	On Performance of UL Channel Access Procedures for PUSCH with Cross-Carrier Scheduling	Ericsson

R1-163151	Channel Access Procedures for UL Grant for Enhanced LAA	Ericsson

R1-163152	On Signaling of UL channel access parameters	Ericsson

R1-163153	On Channel Access Procedures for PRACH and SRS Transmissions	Ericsson

R1-163191	Discussion on LAA Uplink Channel Access	III

R1-163204	Error Cases due to Failed Clear Channel Assessment for UL Transmissions	Panasonic

R1-163298	Discussion on UL channel access in eLAA	WILUS Inc.

R1-163305	UL LBT design in Rel-14 eLAA	Innovative Technology Lab Co.

R1-163631	WF on Signaling of UL Channel Access Parameters	Ericsson, Huawei, HiSilicon

R1-163760	WF on UE-to-UE interference handling for LAA	LGE

R1-163822	WF on Triggered Modification of UL LBT Parameters	Ericsson

R1-163828	Way Forward on supporting trigger based transmission for UL in eLAA	Qualcomm, Nokia, Alcatel-Lucent Shanghai Bell, Huawei, Hisilicon, Intel, Fujitsu

#### Other

Not treated.

R1-162475	Initial signal in LAA uplink	LG Electronics

R1-162476	Power control in LAA uplink	LG Electronics

R1-163762	WF on UL power control	LG

R1-162543	HARQ feedback for UL LAA transmission	Spreadtrum Communications

R1-162644	Synchronous and asynchronous HARQ method for eLAA	Huawei, HiSilicon

### Support for V2V services based on LTE sidelink - WID in RP-160649

R1-163893	Chairman's Notes of Agenda Item 7.3.2 on V2V	Ad-hoc chair (Qualcomm)

The document was presented by Wanshi Chen from Qualcomm and .provides the outcomes of the V2V session.

Decision: The document is endorsed. Contents incorporated below.

R1-163685	LS to RAN1 on RAN2 agreements related to V2V	RAN WG2

Decision: The document is noted.

R1-163857	V2V offline discussion summary	LGE

Decision: The document is noted.

Friday: Editors will submit 36.211, 36.212, 36.213 skeleton CRs until May 20th for information

#### Enhancement to sidelink physical layer structure

##### DM-RS for PSCCH and PSSCH

R1-162118	Further evaluations of DMRS for PSSCH/PSCCH	Huawei, HiSilicon

Decision: The document is noted.

R1-162362	On DMRS physical structure enhancements for PSCCH and PSSCH	Intel Corporation

Decision: The document is noted.

R1-162538	Evaluation of DMRS enhancement for V2V with high Doppler	Nokia, Alcatel-Lucent Shanghai Bell

Decision: The document is noted.

R1-162673	DMRS enhancement for PSSCH/PSCCH	Samsung

Decision: The document is noted.

R1-162826	Link Level Simulations for Sidelink	Ericsson LM

Decision: The document is noted.

R1-163030	Techniques to address PSD limitation	Qualcomm Incorporated

Decision: The document is noted.

R1-163155	Link performance of DMRS Enhancement for V2V PSCCH/PSSCH	Beijing Xinwei Telecom Techn.

Decision: The document is noted.

R1-153611	WF on supporting 500km/h relative speed case	LG Electronics, Huawei, HiSilicon, Samsung, CATT

Decision: The document is noted.

R1-163524	WF on DMRS Mapping for PC5 based V2X	Ericsson, Qualcomm, Vodafone

Decision: The document is noted.

R1-163624	WF on support of the increased subcarrier spacing for V2V communication	Intel, ITRI

Decision: The document is noted.

Conclusion:

- Send an LS to RAN4 asking for urgent feedback especially regarding feasibility, RAN4 specification impact, and implementation impact of the following 4 design alternatives, and kindly request a response as soon as possible. Note that RAN1 is intended to conclude this topic during this week  - Stefano (Ericsson)

  - The company names will NOT be included in the LS

  - Add as the background that RAN#71 agreed:

    - The outcome of this work item should be able to support a relative speed of up to 500 km/h with enhancements/changes (if necessary) to the physical layer structure designed for the relative speed up to 280 km/h.

The four alternatives:

- Alt 1:

  - “4V structure” for PSSCH/PSCCH is kept (which is already an agreement in RAN1)

  - In order to support 500 km/h relative speed case,  lowering the coding rate can be used

    - FFS how to adapt MCS, RB size, and/or number of transmission subframes depending on the situation

      - This may or may not have any specification impact

  - Confirm the working assumption:

    - 15 kHz subcarrier spacing with 1 msec TTI length

  - Supported by:

    - LG Electronics, Huawei, HiSilicon, Samsung, CATT, ZTE, Nokia, ASB, OPPO (9)

  - Objected by:

    - Ericsson, Qualcomm

- Alt2

  - For PC5-based V2V:

    - A DMRS RE is transmitted every 6th subcarrier in all symbols of the scheduled transmission

      - DFT precoding is NOT applied to data REs

      - Offset of the DMRS RE within a RB is FFS

      - FFS between rate matching and puncturing

      - FFS details of the DMRS sequence

  - Confirm the working assumption:

    - 15 kHz subcarrier spacing with 1 msec TTI length

  - Supported by: Ericsson, Qualcomm, Vodafone (3)

  - Objected by: Samsung, Huawei, HiSilicon, CATT, Nokia, ASB

- Alt 3:

  - Increased subcarrier spacing from 15kHz to 60kHz is supported for LTE PC5 V2V communication

  - Supported by: Intel, ITRI (2)

- Alt 4: Alt 2 + Alt 3 (with 30kzh tone spacing)

  - Supported by: Intel (1)

Friday

- Alt 4: Alt 2 + Alt 3 (with 30kzh tone spacing)

  - Supported by: Intel, Ericsson, Qualcomm, ITRI (4)

- Alt 5: Alt 1 + adapt MCS, the number of RBs, and number of transmission subframes depending on the UE absolute speed and UE synchronization source (e.g, GNSS or eNB)

  - FFS: One or more PSCCH format(s) need to be supported

Supported by Ericsson, Huawei, HiSilicon, LGE, ZTE, CATT, Nokia, Vodafone, CATR, Samsung, Sharp, Sony, ASB, Lenovo, OPPO, Xinwei

RAN1 Chair can alt.5 be agreed

Objection from Qualcomm – sees control channel issue and would like to get RAN4 feedback – ok to take it as working assumption

CATT afraid that situation will be identical at next meeting

T-Mobile 16 companies supporting, one against – why not to declare a working agreement and move forward

Intel prefer to keep the WA approach

Working assumption: Alt. 5

Conclusion:

- All proponents are requested to analyze/evaluate Alt. 5 until the next meeting, and RAN1 will decide (confirm or not the working assumption) in RAN1 #85 meeting

R1-163692	LS on PC5 DMRS and L1 format for V2V	Ericsson

Decision: The document is noted and final LS is agreed in R1-163693, by removing “demodulation” from “demodulation feasibility”.

Not treated.

R1-162265	DMRS enhancement in PC5-based V2V	CATT

R1-162409	Link level performance of DMRS for V2V	ZTE

R1-163382	Discussion on DMRS for PSCCH and PSSCH	LG Electronics	(R1-162477)

R1-162539	Considerations on V2V Enhancements for 500kmph	Nokia, Alcatel-Lucent Shanghai Bell

R1-162540	On DMRS enhancements for V2V - Summary	Nokia, Alcatel-Lucent Shanghai Bell

R1-162559	Discussion on DMRS enhancement and operation for V2V	Sony

R1-162860	Discussion on DMRS Enhancement for PC5-based V2V	CMCC

R1-162990	DM-RS Enhancement to sidelink physical layer structure in the High Doppler Case for V2V Services based on LTE sidelink	ITRI

R1-163029	DM-RS Enhancements for V2V PSCCH and PSSCH	Qualcomm Incorporated

##### DM-RS for PSBCH

R1-162266	DMRS enhancement for PSBCH	CATT

Decision: The document is noted.

R1-162412	DMRS design of PSBCH	ZTE

Decision: The document is noted.

Not treated.

R1-162674	Email discussion on PSBCH evaluation assumptions	Samsung

R1-162478	Discussion on DMRS for PSBCH	LG Electronics

R1-162675	DMRS design and periodicity for PSBCH	Samsung

R1-162119	PSBCH DMRS enhancements	Huawei, HiSilicon

R1-162391	Discussion on DM-RS for PSBCH for sidelink-based V2V communication	Fujitsu

R1-162932	On DMRS physical structure enhancements for PSBCH	Intel Corporation

R1-163031	DM-RS Enhancements for V2V PSBCH	Qualcomm Incorporated

R1-163158	Link Performance of DMRS Enhancement for V2V PSBCH	Beijing Xinwei Telecom Techn.

##### Other

Not treated.

R1-162361	Enhancements of sidelink physical layer structure for PC5 V2V communication	Intel Corporation

R1-162825	L1 Format for V2V Transmissions using Sidelink	Ericsson LM

#### Enhancement to sidelink resource allocation

##### Details of sensing with semi-persistent transmission

R1-162268	Evaluation results of PC5-based V2V	CATT

Decision: The document is noted.

R1-162363	On sensing design details for sidelink V2V communication	Intel Corporation

Decision: The document is noted.

R1-162480	System level evaluation results of PC5 based V2V resource allocation options	LG Electronics

Decision: The document is noted.

R1-162830	System Level Simulation Results of Potential PC5 Design Options for V2X	Ericsson LM

Decision: The document is noted.

R1-163032	Details of sensing for V2V		Qualcomm Incorporated

Decision: The document is noted.

R1-163176	Discussion on details of sensing with semi-persistent transmission	NTT DOCOMO, INC.

Decision: The document is noted.

R1-163425 	Evaluation Results on Sensing Based and Geo Based Resource Scheduling for V2V	GM - OnStar Europe	(R1-163309)

Decision: The document is noted.

R1-163612	WF on resource sensing	LG Electronics, Huawei, HiSilicon, CATT, Nokia, OPPO, Samsung, Sony, ITL, Beijing Xinwei, Sequans, Ericsson, DOCOMO, Panasonic, Potevio, ITRI, Sharp, NEC, ZTE, Fujitsu

Decision: The document is noted.

Agreement:

- Both SA decoding and energy measurement are supported for sensing in UE autonomous resource selection.

  - FFS how each of SA decoding and energy measurement is used.

  - Note: strive for a design with managable complexity especially in terms of SA blind decodes, UE buffer, etc.

Agreement:

- If at TTI n resource selection/reselection is triggered in UE autonomous resource selection mode,

  - The UE at least senses between TTI n-a and TTI n-b (FFS a and b with a>b>0), where a and b are integers

    - Working assumption: The values a and b are common for V2V UEs.

  - UE selects time-frequency resource(s) for PSSCH

  - UE transmits SA in TTI n+c where c is an integer

    - FFS whether c is a fixed value (>= 0) or variable.

Agreement:

- In UE autonomous resource selection mode, SA can be transmitted for every TB.

  - FFS whether to support transmitting and/or receiving TB without SA

  - FFS whether every data (re)transmission for the same TB has the associated SA transmission.

Agreement:

- In UE autonomous resource selection mode,

  - UE transmits SA at TTI n+c indicating the associated data which is transmitted at TTI n+d (FFS d with d>=c), where c and d are integers

  - UE indicates whether it intends to reuse the frequency resource signaled for transmission at TTI n+d for potential transmission at TTI n+e for another TB (FFS e with d<e), where e is an integer

    - FFS whether this indication is implicit or explicit.

    - FFS if and how to signal the value for e

    - FFS how the UE determines the value for e

    - FFS whether e is a single value or can be multiple values

    - FFS whether, and if so how, a UE can notify later that it no longer intends to use the resource at TTI n+e.

    - FFS how the UE decides to indicate this

    - Other details FFS

R1-163623	WF on Priority Support for V2V Communication	Intel, Ericsson, III

Decision: The document is noted.

R1-163879	Harmonized WF on Priority Support for V2V Communication	Intel, Ericsson, III, Huawei, HiSilicon, Nokia, Alcatel-Lucent Shanghai Bell, CATT, ITRI, LGE, Samsung

Decision: The document is noted.

Agreement:

- Proposal 1:

  - Priority handling is supported over PC5 for eNB-scheduled and UE- autonomous V2V communication

- Proposal 2:

  - The priority information is taken into account in the resource (re)selection for UE-autonomous mode

    - Further details are FFS

- Proposal 3:

  - Down-select between two alternatives at RAN1#85 meeting

    - Alt.1 Priority information is signaled in SCI

    - Alt.2 Priority information is not signaled in SCI

Conclusion for energy measurement for sensing:

- Continue discussion until next meeting for the following options for the purpose of resource selection for PSSCH

  - Measurement of reception power of the decoded SA

  - Measurement of energy in SA resource

  - Measurement of energy in data resource

Agreements:

- Reselection is triggered if

  - A counter meets an expiration condition, or

    - The counter is reset to a value when reselection is triggered.

      - FFS whether this value is random, fixed, and/or the same across UEs

  - UE identifies that the current resource allocation cannot fulfil the requirement, e.g., latency, reliability, priority, fairness, or QoS requirement (details FFS), or

    - FFS how to impose limitation in the resource size selected by a UE

  - Working assumption: UE detects resource allocation (including the indication of the intention of reusing the frequency resource as per the existing agreement) of another UE which potentially overlaps with its current resource allocation (including the indication of the intention of reusing the frequency resource as per the existing agreement)  and the detected situation meets a condition, or

    - The condition is FFS, not precluding applying different conditions for different priorities (if any relevant case happens in priority handling from RAN1 point of view).

  - eNB triggers reselection, or

    - FFS whether some information needs to reported from UE to eNB

  - FFS additional condition(s), e.g.,

    - If it is supported that a UE can notify later that it no longer intends to use the resource at a future TTI, reselection triggering condition related to this notification.

    - Reselection triggered by notification on resource collision from another UE

    - Reselection triggered by sensing of resources utilization above a threshold

    - Reselection triggered by higher layer

    - Other conditions are not precluded

Send an LS to RAN2 to capture the agreements on resource allocation – Hanbyul (LGE)

Friday:

R1-163904	[Draft] LS on resource allocation for V2V	LG Electronics [RAN1]

Decision: The document is noted and final LS is agreed in R1-163906.

Not treated.

R1-162267	Discussion on UE autonomous resource allocation in PC5-based V2V	CATT

R1-162340	Discussion on the SPS configuration	Guangdong OPPO Mobile Telecom.

R1-162413	Discussion on V2V SPS resource scheme	ZTE

R1-162479	Discussion on sensing with semi-persistent transmission for PC5 based V2V	LG Electronics

R1-162560	Discussion on sensing for autonomous resource selection	Sony

R1-162641	Details of sensing based collision avoidance	Huawei, HiSilicon

R1-162676	Sensing and resource selection for collision avoidance	Samsung

R1-162823	Distributed Resource Allocation for V2X over PC5	Ericsson LM

R1-162933	On combination of sensing and geo-based transmission techniques for sidelink V2V communication	Intel Corporation

R1-163128	Resource allocation and selection enhancement for V2V communications	CATR

R1-163202	On resource control/selection with sensing	Nokia, Alcatel-Lucent Shanghai Bell

R1-163366	On sensing and semi-persistent transmission for V2V sidelink	INTERDIGITAL COMMUNICATIONS

R1-163657	System Level Simulation Results of Potential PC5 Design Options for V2X	Ericsson

R1-163815	WF on sub-channelization	LG Electronics, CATT

##### Signaling to/from eNB

Including details of SPS signaling and UE reporting geographical information to eNB

R1-162533	Signalling design to support SPS activation/release in V2V	Panasonic

Decision: The document is noted.

R1-162677	Semi-persistent transmission support for SL	Samsung

Decision: The document is noted.

R1-163537	WF on SPS for V2V mode 1	Panasonic, Huawei, HiSilicon, OPPO, LG Electronics

Decision: The document is noted.

Agreement:

- For SPS of V2V traffic for mode-1 SPS on PC5:

  - The eNB may configure multiple SPS configurations for a given UE

    - At least SPS-configuration-specific MCS (if MCS is part of the SPS-configuration) and SPS-configuration-specific periodicity can be configured

      - FFS if/which other SPS parameters can differ across the SPS-configurations

    - The eNB can dynamically trigger/release the different SPS-configurations by use of (E)PDCCH

      - Details of the trigger/release are FFS

  - Working assumption: The UE can indicate to the eNB that it does not intend to transmit data before a transmission associated to an SPS configuration

    - FFS any details of the signaling protocol

    - FFS whether eNB acknowledgment of the UE indication is needed

R1-163883	WF on radio measurement reporting	LG Electronics, CATT, Docomo, ITRI, OPPO, III

Decision: The document is noted. Continue discussion till RAN1#85

Not treated.

R1-162122	SPS enhancement for V2V	Huawei, HiSilicon

R1-162269	Discussion on eNB assistant resource allocation in PC5-based V2V	CATT

R1-162270	Discussion on UE reporting geo-info	CATT

R1-162271	Discussion on physical layer design supporting congestion control	CATT

R1-162336	Discussion on the mode-2 resource allocation for V2V 	Guangdong OPPO Mobile Telecom.

R1-162364	Signaling details for eNB controlled sidelink V2V communication	Intel Corporation

R1-162392	Discussion on the resource allocation enhancement for sidelink-based V2V communication	Fujitsu

R1-162429	Reporting of sidelink measurement to eNB	NEC

R1-162481	Discussion on eNB scheduling enhancement for sidelink resource allocation	LG Electronics

R1-162482	Discussion on geolocation information reporting for V2V resource allocation	LG Electronics

R1-162544	An Enhanced Scheduling Request Method for V2X	Spreadtrum Communications

R1-162591	Geo reporting for V2V	Huawei, HiSilicon

R1-162678	Reporting geo information to eNB	Samsung

R1-162733	UE reporting of geo-information	Lenovo (Beijing) Ltd

R1-162820	Reporting of UE Geo-Info for V2X to the eNB	Ericsson LM

R1-162828	Contents of Downlink Control Information for V2V over PC5	Ericsson LM

R1-163033	eNodeB signaling for V2V resource allocation	Qualcomm Incorporated

R1-163154	Discussion on V2V Semi-Persistent Scheduling Procedure	Beijing Xinwei Telecom Techn.

R1-163177	Necessity signaling between eNB and UE for PC5 based V2V	NTT DOCOMO, INC.

R1-163308	Geo Based Resource Scheduling for V2V with Accurate Geographical Information	GM - OnStar Europe

R1-163630	WF on geographical-information reporting	LG Electronics, Samsung, Huawei, HiSilicon, Intel, CATT, ITL, KRRI

##### Resource pool design

R1-162120	Resource pool design for V2V	Huawei, HiSilicon

Decision: The document is noted.

R1-162272	Discussion on SA and data resource pool allocation	CATT

Decision: The document is noted.

R1-162365	Enhancements to sidelink resource pool design for V2V communication	Intel Corporation

Decision: The document is noted.

R1-162541	On resource pool design	Nokia, Alcatel-Lucent Shanghai Bell

Decision: The document is noted.

R1-162754	Discussion on number of SA transmissions for V2V	Sequans Communications

Decision: The document is noted.

R1-162991	Discussion on explicit and implicit autonomous resource allocation enhancements	ITRI

Decision: The document is noted.

R1-163613	WF on multiplexing SA and data	LG Electronics, Nokia

Decision: The document is noted.

R1-163627	WF on TDM  of SA and data	Samsung, Intel, Sequans

Decision: The document is noted.

R1-163625	WF on MPR for transmission of PSCCH and PSSCH in the same subframe	Intel

Decision: The document is noted.

Agreement: When SA and the associated data are transmitted in the same TTI, they can be transmitted in non-adjacent RBs.

Working assumption:

- In V2V, SA resource and data resource are always FDMed from system perspective

  - If significant issues are found, can consider further supporting TDM

Agreement:

- A data pool is always associated with an SA pool.

  - An RB of an SA pool in a TTI cannot be included in the associated data pool.

  - An RB of an SA pool in a TTI cannot be included in another SA pool (if exists).

  - Working assumption: At least an RB of a data pool in a TTI can be included in another data pool (if exists).

  - An RB of an SA pool in a TTI cannot be included an un-associated data pool (if exists)

Agreements:

- The following two cases are supported:

  - SA and the associated data are transmitted in the same TTI,

  - SA and the associated data are transmitted in different TTIs

    - The scheduling timing between SA and associated data is variable

      - In UE-autonomous resource selection mode, the timing is chosen by the transmitting UE from a configurable range

      - In eNB-scheduling mode, the timing is determined by eNB

      - SA includes information about the scheduling timing

  - Note: the association does not necessarily include the case of intention of using the resources for a different TB, if any (which is FFS)

Not treated.

R1-162121	Variable number of transmissions for V2V	Huawei, HiSilicon

R1-162273	Considerations on V2V traffic priority and relative resource allocation	CATT

R1-162274	Resource pool  enhancement in PC5-based V2V	CATT

R1-162394	Discussion on Resource Pool for PC5 based V2V	Fujitsu

R1-162410	Discussion on resource pools and resource pattern	ZTE

R1-162430	Timing relation between SA and data for PC5-based V2V	NEC

R1-162483	Discussion on resource pool design for PC5 based V2V	LG Electronics

R1-162679	Resource pool for V2V	Samsung

R1-162734	Resource pool for V2V	Lenovo (Beijing) Ltd

R1-162992	Dynamic zone management in V2V service	ITRI

R1-163034	Resource pool design for V2V	Qualcomm Incorporated

R1-163073	Discussion on V2V resource pool design	Potevio

R1-163137	Discussion on V2V Resource Pool Enhancement	Beijing Xinwei Telecom Techn.

R1-163178	Discussion on Resource Pool for PC5 based V2V Communications	NTT DOCOMO, INC.

R1-163306	Considerations on resource allocation mechanism for V2V	Innovative Technology Lab Co.

R1-163337	Congestion control for V2V	Huawei, HiSilicon

##### Scheduling assignment

R1-162275	Discussion on contents of scheduling assignment	CATT

Decision: The document is noted.

R1-162827	Contents of PSCCH for V2V over PC5	Ericsson LM

Decision: The document is noted.

R1-163035	Scheduling Assignments for V2V	Qualcomm Incorporated

Decision: The document is noted.

R1-163074	Contents of PSCCH for V2V communication	Potevio

Decision: The document is noted.

Not treated.

R1-162366	Control signaling design details for V2V communication	Intel Corporation

R1-162414	SA and data transmission scheme for V2V	ZTE

R1-162431	Resource collision detection and handling	NEC

R1-162484	Discussion on scheduling assignment for PC5 based V2V	LG Electronics

R1-162532	Discussion on SA to support same subframe scheduling in V2V	Panasonic

R1-162642	Scheduling assignment design for V2V	Huawei, HiSilicon

R1-162680	SA design for V2V	Samsung

R1-162735	SA contents for V2V	Lenovo (Beijing) Ltd

R1-163126	Considerations on SA content for V2V	CATR

R1-163179	Discussion on enahancement of scheduling assignement in V2V communications	NTT DOCOMO, INC.

R1-163750	WF on SCI for V2X	Huawei, HiSilicon, Ericsson, LGE

##### Other

Not treated.

R1-162393	Prioritization-based resource allocation for sidelink-based V2V communication	Fujitsu

R1-162815	Observations on CAM Message Periodicity and Payload	Ericsson LM

R1-162833	Discussion on V2X PC5 Scheduling, Resource Pools and Resource Patterns	Ericsson LM

#### Coexistence of PC5-based V2V operation and legacy Uu operation

Including inter-UE coexistence in the same carrier frequency, intra-UE coexistence in the same carrier frequency, and intra-UE coexistence in different carrier frequency

R1-163036	Coexistence of PC5-based V2V operation and legacy Uu operation	Qualcomm Incorporated

Decision: The document is noted.

R1-163180	Discussion on co-existence between PC5-based V2V and legacy Uu	NTT DOCOMO, INC.

Decision: The document is noted.

R1-162824	[DRAFT] LS on V2X multicarrier configuration	Ericsson LM

Decision: The document is noted.

Conclusion: The draft LS in R1-162824 is endorsed in principle. Further discussion offline to refine the wording. Revisit later this week – (Ericsson)

R1-163745	[DRAFT] LS on V2X multicarrier configuration	Ericsson

Decision: The document is noted and final LS is agreed in R1-163746, by the following updates:

- Accepting the changes in change marks

- multichannel operation for PC5 V2V

- Change reference [2] to TS 102 687

R1-163633	WF on coexistence of PC5-based V2V and WAN	LG Electronics, NTT DOCOMO, ZTE

Decision: The document is noted.

Agreement:

- The followings are supported for the purpose of coexistence between PC5-based V2V and WAN:

  - Sidelink open loop power control is re-used for SL TX for V2V

    - FFS RSRP based resource selection

  - SL TX for V2V can be prioritized over WAN TX

    - FFS the details (e.g., applicability to Mode 1 and/or Mode 2, etc), especially whether existing D2D mechanism can be reused,

    - The prioritization is managable by eNB. Details FFS.

  - The same receiver capability of D2D communication UEs is assumed for V2V UEs. That is, a Rx chain is available at all time to receive V2V signals without affecting WAN reception (from RAN1 perspective) when the UE is configured to receive V2V.

Not treated.

R1-162415	Coexistence of PC5-based V2V operation and legacy Uu operation	ZTE

R1-162485	Discussion on coexistence of PC5 based V2V operation and legacy Uu operation	LG Electronics

R1-162681	Discussions on coexistence of PC5-based V2V and legacy Uu	Samsung

R1-163338	Coexistence of PC5 V2V operation and legacy Uu	Huawei, HiSilicon

#### Enhancement to sidelink synchronization procedure

Low priority is given to enhancements to Rel-12/13 SLSS-based synchronization.

R1-162337	Discussion on the synchronization source priority  	Guangdong OPPO Mobile Telecom.

Decision: The document is noted.

R1-162416	Enhancement to sidelink synchronization	ZTE

Decision: The document is noted.

R1-162682	Synchronization  for PC5 based V2V	Samsung

Decision: The document is noted.

R1-163037	Synchronization Procedure for V2V	Qualcomm Incorporated

Decision: The document is noted.

R1-163203	Sidelink synchronization procedure supporting V2V	Nokia, Alcatel-Lucent Shanghai Bell

Decision: The document is noted.

R1-163725	WF on SLSS ID for enabling LTE-based ITS coexistence	Ericsson

Decision: The document is noted.

Proposal:

- SL SS transmitted by a UE participating in ITS safety services in a given carrier is distinguishable from SL SS transmitted by a UE participating ITS non-safety services in the same carrier.

  - If a UE participates in both ITS safety and ITS non-safety services in a given carrier, then it transmits only SL SS associated with ITS safety

  - Other details FFS

  - Note: this is not intended to impact the working assumption on the sequence design of SL SS and the agreement on the location of SL SS

  - Note: how to capture it in the specification is up to RAN2

  - Send an LS to RAN2 – Ricardo (Ericsson)

Continue discussion till RAN1#85

R1-163628	Way Forward on SLSS for V2V PC	5	Huawei, HiSilicon, Samsung, Sharp, ITRI, CATR, LG Electronics, NTT DOCOMO, Potevio

Decision: The document is noted.

Proposal:

- For the V2V PC5 SLSS:

  - SLSS ID from GNSS timing can be defined by some reserved value(s) of the SLSS IDs:

    - FFS: the detailed reserved value(s) of the SLSS IDs

    - Notes: the reserved value(s) of the SLSS IDs can be (pre-)configured or pre-defined

Continue discussion till RAN1#85

R1-163654	WF on V2V and D2D co-existence	Samsung, Huawei, HiSilicon, Sharp, ITRI

Decision: The document is noted.

Proposal:

- SLSS should be differentiated between Rel-14 V2V and Rel-13 D2D

  - Details FFS

Continue discussion till RAN1#85

R1-163710	Way Forward on V2V PC5 timing	Huawei, HiSilicon, ITRI, OPPO, CATR, Potevio, NEC, Nokia, ASB, Samsung

Decision: The document is noted.

Proposal:

- PC5 TTI index/subframe number and frame number are derived from the UTC timing at least when GNSS is the direct synchronization reference for PC5 based V2V transmission

  - FFS: how to derive the TTI index/subframe number and frame number

Continue discussion till RAN1#85

R1-163882	Way forward on synchronization source priority	NTT DOCOMO, Oppo, LGE

Decision: The document is noted.

Agreement:

- Whether eNB timing or GNSS timing is prioritized can be per carrier cell-specifically configured by the cell on which an UE camps

  - The carrier that UE camps on can be different carrier other than the carrier that the priority is configured

  - If the UE does not receive such a configuration, UE selects synchronization source with the same priority order as out of coverage case

    - FFS: The priority order for out of coverage case

- Send an LS to RAN2 and RAN4 – (NTT DOCOMO)

Friday:

R1-163902	[DRAFT] LS on V2X synchronization procedure	NTT DOCOMO, [RAN WG1]

Decision: The document is noted and final LS is agreed in R1-163907.

R1-163711	Way Forward on V2V PC5 sync priority for In-Coverage	Huawei, HiSilicon, Potevio, NEC, Nokia, ASB, Samsung

Decision: The document is noted. Also supported by Sony

Not treated.

R1-162103	Network protection of out of coverage V2V UEs	Huawei, HiSilicon

R1-162123	Timing alignment of different synchronization sources for V2V	Huawei, HiSilicon

R1-162276	Synchronization enhancements in PC5-based V2V	CATT

R1-162277	Discussion on physical layer design for synchronization in PC5-based V2V	CATT

R1-162278	Discussion on PSBCH	CATT

R1-162432	Synchronisation in V2X	NEC

R1-162486	Discussion on sidelink synchronization enhancement for PC5-based V2V	LG Electronics

R1-162639	Synchronization priority and timing consideration for V2V	Huawei, HiSilicon

R1-162640	SLSS and PSBCH consideration for V2V PC5	Huawei, HiSilicon

R1-162643	Discussion on open issues for synchronization	Huawei, HiSilicon

R1-162805	Discussion on V2V synchronization remaining issues	PANASONIC

R1-162829	Distributed Synchronization Procedure for V2X over PC5	Ericsson LM

R1-162840	Sidelink Enhancement for V2X Synchronization	Sharp

R1-162934	Sidelink Synchronization Procedure Enhancements for V2V Communication	Intel Corporation

R1-162993	Discussion on sidelink synchronization procedure  for PC5-basde V2V communication	ITRI

R1-163181	SLSS Enhancements for GNSS based Synchronization	NTT DOCOMO, INC.

R1-163307	Considerations on enhancement to sidelink synchronization for V2V	Innovative Technology Lab Co.

R1-163629	WF on V2V PCS timing	Huawei, HiSilicon, ITRI, OPPO, CATR, Potevio, NEC

#### Other

Not treated.

R1-162683	Transmission power control for V2V	Samsung

### Enhancements on Full-Dimension (FD) MIMO for LTE - WID in RP-160623

R1-163838	Chairman's Notes of Agenda Item 7.3.3 on Enhancements on Full-Dimension (FD) MIMO for LTE	Session Chairman (Nokia)

The document was presented by Matthew Baker from Nokia and .provides the outcomes of the FDMIMO session.

Discussion: .

Decision: The document is endorsed.

The document was presented by … from … and .

Discussion: .

Decision: The document is noted.

R1-162390	Rel.14 eFD-MIMO WID Progress Plan	Samsung, Nokia Networks, CATT

#### Reference Signals

##### Non-precoded CSI-RS

For {20, 24, 28, 32} ports with mechanism for overhead reduction

R1-162861	General views on non-precoded CSI-RS enhancement	CMCC

R1-162684	Discussions on CSI-RS enhancements for class A CSI reporting in eFD-MIMO	Samsung

R1-162345	Discussion on non-precoded CSI-RS design for eFD-MIMO	ZTE

R1-162806	CSI-RS Design Supporting up to 32 Antenna Ports	Nokia, Alcatel-Lucent Shanghai Bell

R1-162367	Discussion on CSI-RS for Class A FD-MIMO	Intel Corporation

R1-163079	CSI-RS Design for Class A eFD-MIMO	Ericsson

R1-162437	Design for Non-precoded CSI-RS for overhead reduction	NEC

R1-162596	CSI-RS pattern design for up to 32 ports	Huawei, HiSilicon

R1-162279	CSI-RS enhancement for {20,24,28,32}  ports	CATT

R1-162968	Discussion on CSI-RS port extension	NTT DOCOMO

R1-163038	Enhancements on Non-precoded CSI-RS for eFD-MIMO	Qualcomm Incorporated

R1-162685	Port-based CSI-RS overhead reduction	Samsung

R1-162280	Discussion on CSI-RS overhead reduction	CATT

R1-162395	Discussion on non-precoded CSI-RS	Fujitsu

R1-162448	Aperiodic CSI-RS for non-precoded CSI-RS	ZTE

R1-162487	New CSI-RS designs for eFD-MIMO	LG Electronics

R1-163162	Non-precoded CSI-RS overhead reduction for FD-MIMO	Intel Corporation

R1-163580	WF on enhancements for non-precoded CSI-RS LG Electronics, Ericsson, ETRI, NEC, Samsung

Agreements:

- For {20, 24, 28, 32} ports, a CSI-RS resource for class A CSI reporting is composed as an aggregation of K CSI-RS configurations [i.e. RE patterns].

  - The number of REs in the kth configuration Nk ∈ {4, 8}

    - The same Nk = N can be used for all k

      - FFS whether the same Nk = N for all k is the only permitted configuration

      - FFS whether the set of values of Nk might be further restricted for some numbers of CSI-RS ports

    - FFS whether a different set of Nk might apply in case of CDM4

    - FFS on including Nk=2.

  - Aim to enable the support of CSI-RS port sharing with Rel-13 and Rel-12 UEs

  - The per-port CSI-RS density is FFS based on one or more of the following alternatives:

    - FDM

    - TDM

    - Partial port

    - Partial overlapping, e.g. for 32 ports, ports 15-38 in PRB#1, ports 23-46 in PRB#2

    - Aperiodic CSI-RS with partial bandwidth

    - Measurement restriction in frequency domain

    - CDM, e.g. 2 x Nk ports transmitted in a single Nk resource

    - Other schemes

    - Note that the following are not precluded:

      - per-port CSI-RS density per PRB = 1

      - different per-port CSI-RS densities for different CSI-RS ports is not precluded

R1-163617	WF on Design Principles for {20, 24, 28, 32} ports NZP CSI-RS resource in Rel-14	Qualcomm, Intel, Xinwei, ZTE

Proposal:

- The aggregation of CSI-RS resource configuration is used to support {20, 24, 28, 32} NZP CSI-RS ports

- Backward compatibility with Rel-13 and Rel-12 UEs for CSI-RS port sharing shall be supported

- The use of CDM-length 2 and 4 is maintained for {20, 24, 28, 32} CSI-RS ports

  - The CDM-length 8 is not precluded. Details FFS.

- FFS on lower density and overhead reduction techniques

R1-163457	WF on CSI-RS RE density for non-precoded CSI-RS	Samsung, Nokia, Alcatel-Lucent Shanghai Bell, NEC, LGE, ZTE, ETRI, NTT DOCOMO, Huawei

Revision of R1-163616

R1-163549	WF on additional REs for Class A NZP CSI-RS resource	Intel

##### Beamformed CSI-RS

More efficient resource utilization for UE-specific BF CSI-RS, including aperiodic CSI-RS

R1-162686	Specification impact of aperiodic CSI-RS	Samsung

R1-162599	Efficient utilization of BF CSI-RS	Huawei, HiSilicon

R1-163039	Enhancements on Beamformed CSI-RS for eFD-MIMO	Qualcomm Incorporated

R1-162368	On possible approaches to CSI-RS overhead reduction for Class B FD-MIMO	Intel Corporation

R1-162281	On aperiodic CSI-RS transmission	CATT

R1-162807	Proposals for Improving RS Utilization for beamformed CSI-RS Transmission Schemes	Nokia, Alcatel-Lucent Shanghai Bell

R1-162346	Enhancements on Beamformed CSI-RS	ZTE

R1-162488	Enhancements on beamformed CSI-RS including aperiodic CSI-RS	LG Electronics

R1-162969	Discussion on Aperiodic CSI-RS	NTT DOCOMO

R1-162987	Discussion on resource utilization for UE-specific BF CSI-RS	ITRI

R1-163134	Considerations on Aperiodic CSI-RS	Beijing Xinwei Telecom Techn.

R1-163582	WF on enhancements for aperiodic CSI-RS	LG Electronics, NTT DOCOMO, Samsung, ZTE

Proposal:

- Aperiodic NZP CSI-RS resource is supported in Rel-14, where

  - It can only be configured for PUSCH based A-CSI reporting.

  - Two types of CSI-RS resource configurations are supported:

    - Type 1: Reuse of legacy (semi-static) CSI-RS resource allocation/configuration

    - Type 2: A new aperiodic CSI-RS resource allocation/configuration

      - Without Subframe_config

      - Whether aperiodic CSI-RS resource allocation/configuration (e.g. # of ports, CSI-RS pattern, port subset) is semi-static or dynamic or a combination between semi-static and dynamic is FFS

      - Note: If resource allocation/configuration is fully semi-static, no extra bit in the UL-related DCI is needed to support aperiodic CSI-RS

  - Aperiodic CSI-RS transmission instance is indicated by UL-related DCI:

    - For Type 2, aperiodic CSI-RS transmission is in the same DL subframe as the associated UL-related DCI

    - For Type 2, aperiodic CSI-RS can be transmitted in any DL subframe that the UL-related DCI can be present.

  - Other alternatives, e.g., indicated by DL-related DCI, are FFS.

- Details on rate matching needed due to aperiodic NZP/ZP CSI-RS transmission(s) are FFS.

- FFS: Aperiodic CSI-IM resource is supported in Rel-14.

R1-163618	Way Forward on overhead reduction for Class B Beamformed CSI-RS	Qualcomm, Intel, Ericsson, Xinwei

Agreements:

- Until RAN1#85, down-selection/merge the following alternatives for reducing the overhead for beamformed CSI-RS, considering overhead reduction, performance, backward compatibility, UE implementation complexity:

  - Alt.1: Aperiodic CSI-RS

    - E.g., One-shot CSI-RS transmission with transmission on the same subframe as triggering subframe

  - Alt.2: Dynamic CSI-RS activation and/or deactivation

    - E.g., One-shot or multi-shot CSI-RS transmission after the triggering subframe

  - Alt.3: Reduced frequency density via decimation over part or all of the bandwidth

  - Alt.4: MR with dynamic sharing of the beams on CSI-RS resources among the UEs

  - Alt.5: Triggered measurement of one of multiple CSI-RS configurations

  - Alt.6: Spatially multiplexed beamformed CSI-RS

  - Other approaches are not precluded

R1-163546	WF on overhead reduction for Class B FD-MIMO with K = 1 	Intel, Nokia, ASB

##### Uplink DM-RS

Evaluate and, if needed, specify more orthogonal ports with partially overlapping BW allocations

R1-162282	Discussion on UL DMRS enhancement	CATT

R1-162347	Discussion on UL DMRS enhancement for eFD-MIMO	ZTE

R1-162369	Uplink DM-RS enhancements for FD-MIMO	Intel Corporation

R1-162438	Discussion on uplink DM-RS enhancement	NEC

R1-162489	Discussion on uplink DMRS enhancements for eFD-MIMO	LG Electronics

R1-162601	Uplink DMRS enhancement to support more orthogonal partial overlapped ports	Huawei, HiSilicon

R1-162687	Discussion on uplink DMRS for partially overlapping bandwidth allocations	Samsung

R1-162808	Proposals of Evaluation Assumptions of UL MU-MIMO for FD-MIMO	Nokia, Alcatel-Lucent Shanghai Bell

R1-162809	Motivation and Initial Evaluation Results of UL DMRS Enhancement	Nokia, Alcatel-Lucent Shanghai Bell

R1-163040	Improving Uplink DM-RS Orthogonality for MU-MIMO	Qualcomm Incorporated

R1-163135	Discussion on uplink DM-RS enhancement	Beijing Xinwei Telecom Techn.

R1-163602	WF on UL DMRS	Intel Corporation, Nokia, Alcatel-Shanghai Bell, Samsung, CATT

Also supported by NEC

R1-163834	WF on Uplink DMRS	Intel, Samsung

Also supported by Nokia & ASB

Agreements:

- Evaluation assumptions on Slides 2 and 3.

Conclusions:

- Consider the following possible DMRS enhancements in the evaluation:

  - IFDMA DMRS with a comb number of 2 and/or 4

  - More DMRS symbols (with or without larger OCC), e.g.:

    - IFDMA DMRS subcarriers with PUSCH REs, or

    - across 2-subframe pairing

  - Enhancement of DMRS sequence generation, including DMRS transmitted in a wider BW than the associated PUSCH

  - Frequency domain OCC

- Other solutions are not precluded

- Companies are to state their Rel-13 baseline scheme.

#### CSI Reporting

R1-162688	Discussions on periodic CSI reporting for class A in eFD-MIMO	Samsung

##### Non-precoded CSI-RS codebooks

For newly defined number of ports for NP CSI-RS for a subset of possible port layouts,both 1D and 2D

R1-162283	Antenna port layouts for Rel-14 FD-MIMO	CATT

R1-162284	Codebook design for {20,24,28,32} ports	CATT

R1-162348	Discussion on rank 1-2 codebook design for eFD-MIMO	ZTE

R1-162490	Discussions on codebook design for eFD-MIMO	LG Electronics

R1-162597	Discussion on codebook design for up to 32 CSI-RS ports	Huawei, HiSilicon

R1-162689	View on class A codebook extension	Samsung

R1-162810	Codebook Design Supporting Antenna Configurations Up to 32 Ports	Nokia, Alcatel-Lucent Shanghai Bell

R1-162970	Codebook enhancement for non-precoded CSI-RS	NTT DOCOMO

R1-162988	Discussions of NP CSI-RS codebook for newly defined number of ports	ITRI

R1-163041	Non-precoded CSI-RS Codebooks for eFD-MIMO	Qualcomm Incorporated

R1-163084	Parameterization and CSI reporting for up to 32 port FD-MIMO	Ericsson

R1-163186	Performance evaluation of Class A codebooks with 32 ports	Intel Corporation

##### CSI reporting for hybrid CSI-RS

Hybrid between NP and BF CSI-RS or different types of BF CSI-RS

R1-162285	CSI reporting for hybrid CSI-RS	CATT

R1-162690	CSI reporting for hybrid CSI-RS	Samsung

R1-163042	Discussion on Hybrid CSI-RS Operation for eFD-MIMO	Qualcomm Incorporated

R1-163081	Hybrid CSI Reporting Design Targets	Ericsson

R1-163187	CSI enhancements for hybrid FD-MIMO	Intel Corporation

R1-162349	Discussion on CSI reporting for hybrid CSI-RS	ZTE

R1-162491	Discussion on CSI reporting enhancements for hybrid CSI-RS	LG Electronics

R1-162598	CSI reporting mechanism for joint utilization of different CSI-RS types	Huawei, HiSilicon

R1-162691	Hybrid PMI codebook and CSI reporting	Samsung

R1-162692	CSI reporting for hybrid CSI-RS	Samsung

R1-162811	Hybrid CSI reporting using BF and NBF CSI-RS	Nokia, Alcatel-Lucent Shanghai Bell

R1-162862	Considerations on hybrid non-precoded CSI-RS and beamformed CSI-RS	CMCC

R1-162971	CSI reporting for hybrid CSI-RS	NTT DOCOMO

R1-163120	Discussion on Joint Utilization of Different CSI-RS Types	Beijing Xinwei Telecom Techn.

R1-163581	WF on Hybrid CSI Reporting	Samsung, Alcatel-Lucent Shanghai Bell, CATT, Nokia, Qualcomm, LG Electronics, Beijing Xinwei, Interdigital, AT&T, China Telecom, Huawei, HiSilicon, CMCC, CATR

Revised in R1-163665	Samsung, Alcatel-Lucent Shanghai Bell, CATT, Nokia, Qualcomm, LG Electronics, Beijing Xinwei, Interdigital, AT&T, China Telecom, Huawei, HiSilicon, CMCC, CATR, Ericsson

(Note that slide 3 is illustrative only)

Agreements:

- Specify at least one of the following enhancements on Rel.14 CSI reporting:

  - One CSI process configured with 2 independent eMIMO-Types associated with different sets of parameters

    - Each eMIMO-Type is associated with a set of K≥1 NZP CSI-RS resources

    - Note that the 2 independent eMIMO-Types may be of the same Class

    - Note that if it decided that the two independent eMIMO-Types are always of the same class, then only 1 eMIMO-Type with 2 different sets of parameters would be sufficient.

  - An additional mechanism for CSI calculation applied to a configuration with a pair of CSI processes each of which is configured with 1 eMIMO-Type

- Notes:

  - This does not preclude the possibility of specifying a new eMIMO type in Rel-14 and the use of this new type for Hybrid CSI reporting

  - Down-selection (if any) between the two enhancements will be decided in RAN1#85

R1-163548	WF on hybrid Class A and Class B operation 	Intel

Revised in R1-163763	WF on hybrid Class A and Class B operation   Intel, Samsung, LG

Observations:

The following alternatives have been discussed and may be studied:

- Study the following alternatives for the CSI reporting for hybrid Class A and Class B K=1 FD-MIMO and compare with the baseline of Alt 1 including i2 and CQI

  - Alt 1: Class A CSI with single i1 + RI

  - Alt 2: Class A CSI with multiple i1 + RI

  - Alt 3: Class A CSI with single i1

  - Alt 4: Class A CSI with multiple i1

  - Note for the Alt 3-4 RI can be restricted to 1

  - Note that any of the above alternatives may be reported with or without CQI and/or i2

  - Note that the CSI measurement may be based on partial or whole set of CSI-RS ports

  - Other alternatives and merging of alternatives are not precluded

- Study the following alternatives for the CSI reporting for hybrid Class B and Class B FD-MIMO and compare with the baseline of both Class Bs with full CSI

  - Alt 1: The first Class B has K>1 with CRI, and optionally i1, second Class B has K =1

  - Alt 2: The first Class B has K>1 with K independent CSI reports, second Class B has K =1

  - Alt 3: The first Class B has K=1 with PMI, second Class B has K =1

  - Alt 4: The first Class B has K>1 with CRI, and optionally i1, second Class B has K >1

  - Alt 5: The first Class B has K>1 with K independent CSI reports, second Class B has K >1

  - Alt 6: The first Class B has K=1 with PMI, second Class B has K >1

  - Other alternatives and merging of alternatives are not precluded

##### Advanced CSI reporting and interference measurement

(Second priority) Evaluate and, if needed, specify enhancements on CSI reporting to improve eNB precoding and interference measurement for MU transmissions

R1-162595	Discussion on analog CSI feedback	Huawei, HiSilicon

R1-162693	Linear combination codebook and CSI reporting	Samsung

R1-162812	Initial Evaluation Results with Advanced CSI Reporting	Nokia, Alcatel-Lucent Shanghai Bell

R1-163454	WF on Rel.14 Advanced CSI	Nokia, Alcatel-Lucent Shanghai Bell, Samsung, CATT, Huawei, HiSilicon, Qualcomm

Conclusions:

- Evaluate proposed advanced CSI schemes

- Baseline for comparison:

  - Implicit CSI feedback based on Rel.13 codebook (as well as its extension to >16 ports) or other legacy codebooks if applicable

- Performance and feedback overhead should be provided

- Simulation scenarios:

  - FTP model 1 with 70% and 50% traffic loading

  - Evaluations should include dynamic switching between MU- and SU-MIMO

  - Other simulation details can refer to 36.897 EB/FD MIMO SI

Candidates for PMI Enhancement:

- Codebook based (implicit feedback)

  - Linear combination codebook (enhanced W2) for Non-precoded CSI-RS and beamformed CSI-RS

  - MU CSI (e.g., additional i1, i1,1, or i1,2)

  - Other new or modified codebooks

- Explicit feedback

  - Channel quantization

  - Eigenvector quantization

  - Covariance matrix quantization

- Analog feedback

- Note: other schemes are not precluded

Candidates for CQI Enhancement:

- CQI Derivation

  - Interference estimation based on NZP CSI-RS

  - MU-CQI conditioned on MU hypotheses and CSI-IM

- Reduced CQI feedback delay

- Note: other schemes are not precluded

R1-162127	Discussion on enhanced CQI measurement and feedback	Huawei, HiSilicon

R1-162286	On Interference enhancement for multi-user MIMO operation	CATT

R1-162561	CSI Reporting for MU transmission	Sony

R1-162694	Advanced CSI reporting paradigm	Samsung

R1-162695	Discussions on MU CSI enhancement in eFD-MIMO	Samsung

R1-162696	Discussions on interference measurement enhancement in eFD-MIMO	Samsung

R1-162863	Enhancing interference measurement for MU transmissions	CMCC

R1-163043	High level View on CSI enhancements for MU-MIMO	Qualcomm Incorporated

R1-163080	Potential for Advanced CSI Reporting Enhancements	Ericsson

R1-163185	Discussion on SINR calculation accuracy using NZP CSI-RS	Intel Corporation

#### DMRS-based Open-Loop and Semi-Open-LoopTransmission

Evaluate and, if needed, specify necessary CSI reporting scheme and open-loop (without PMI feedback) as well as semi-open-loop (with reduced PMI feedback) transmission scheme

R1-162287	Scenarios and evaluation assumptions for DMRS-based open-loop and semi-open-loop operation	CATT

R1-163456	WF on Evaluation Assumptions for DMRS based Semi-OL Samsung

Proposed modifications:

- FFS UE distribution including outdoor/indoor partitioning

- FFS UE speed

- FFS penetration loss

R1-163765	WF on Evaluation Assumptions for DMRS based OL and semi-OL	Qualcomm, NEC, Beijing Xinwei, Intel

Discuss the above two proposals for scenarios and assumptions offline, and aim for a converged proposal for approval on Thur – Youngwoo - R1-163666.

Agreed in R1-163832

R1-163833	Outcome of offline session	Samsung

R1-162288	Discussion on DMRS based open-loop MIMO	CATT

R1-162289	Discussion on DMRS based semi-open-loop MIMO	CATT

R1-162350	Discussion on DMRS-based Open-Loop and Semi-Open-Loop Transmission	ZTE

R1-162370	Discussion on the enhancements for open-loop transmission schemes	Intel Corporation

R1-162371	On CSI enhancements for open loop MIMO	Intel Corporation

R1-162492	Discussion on OL and Semi-OL MIMO scheme	LG Electronics

R1-162600	Discussion on DMRS based open loop transmission	Huawei, HiSilicon

R1-162697	Discussions on open and semi-open loop transmission in eFD-MIMO	Samsung

R1-162813	Initial Evaluation Results of DMRS-based open-loop transmission schemes	Nokia, Alcatel-Lucent Shanghai Bell

R1-163044	UERS-based Open-Loop and Semi-Open-Loop Transmission	Qualcomm Incorporated

R1-163083	Use Cases for (Semi-) Open Loop DMRS Transmission	Ericsson

R1-163121	Discussion on DMRS-based Semi-openloop Transmissions	Beijing Xinwei Telecom Techn.

R1-163127	Consideration on DM RS based open loop transmission	CATR

R1-163212	Need for discussions on DMRS based Open/Semi-open loop MIMO schemes for eFD-MIMO	CEWiT

R1-163455	WF on DMRS-based Open-Loop and  Semi-Open-Loop MIMO	CATT, Alcatel-Lucent Shanghai Bell, CATR,Intel, Huawei, HiSilicon, Nokia, Samsung

Agreement:

- Evaluate open-loop and semi-open-loop MIMO schemes until RAN1#85

  - Example schemes proposed in RAN1#84bis are provided below

  - NOTE: other schemes are not precluded

- Companies are encouraged to provide performance, CSI feedback and transmission details

Example: a CSI-process with K = 1 CSI-RS

- Alt-1a:  Transparent DMRS (i.e. DMRS and PDSCH precoded identically)

  - PMI feedback points to one of N precoding hypotheses, where each precoding hypothesis corresponds to a set of M>1 precoding matrices on a subband

    - PMI overhead log2(N) bits, where N can be 1

  - Example:

    - PRB-level or subcarrier(s)-level cycling of M precoders

    - Cycling for a subset of codebook by Codebook-subset-restriction

    - Cycling for i2, not for i1 (W1 reporting only for dual-stage codebook, e.g. Rel.10 8Tx, Rel.12 4Tx, Rel.13 Class A and its possible Rel.14 enhancements)

- Alt-1b:  Non-transparent DMRS (i.e. DMRS and PDSCH precoded differently)

  - Received PDSCH signal as y = HW1W2s + n, where, S is unprecoded PDSCH, W2 refers to OL-precoding from L layer PDSCH to D DMRS ports, W1 refers to CL-precoding from DMRS/PDSCH to CSI-RS ports (or non-precoded channel)

  - RI/PMI feedback: quantization of W1,  based on H and knowledge of W2

  - CQI feedback: quantization of HW1W2

  - Example of W2 precoding (open-loop)

    - Rank-1: SFBC (RE-level)

    - Rank-2: LD-CDD or layer permutation

Example: a Class B CSI-process with K>1 CSI-RS

- Alt-2a:

  - A single wideband CRI report and/or long-term PMI

  - Open-loop/semi-open-loop with K=1 is performed in the CSI-RS resource indicated by CRI

- Alt-2b:

  - CRI cycling, no CRI report

  - open-loop/semi-open-loop with K=1 is performed in the corresponding CSI-RS resource indicated by CRI

### Downlink Multiuser Superposition Transmission for LTE - WID in RP-160680

R1-163839	Chairman's Notes of Agenda Item 7.3.4 on Downlink Multiuser Superposition Transmission for LTE	Session Chairman (Nokia)

The document was presented by Matthew Baker from Nokia and .provides the outcomes of the MUST session.

Discussion: .

Decision: The document is endorsed.

No issues to revisit on Friday

The document was presented by … from … and .

Discussion: .

Decision: The document is noted.

#### Superposition transmissions

Down select between single and multiple power ratios, potential power ratios, modulation, etc.

R1-162218	Performance evaluation of MUST Category 2 with 2Tx antennas	ZTE

R1-162745	Selection of Power Ratios and Modulation for MUST PDSCH	MediaTek Inc.

R1-162841	Multiple transmission power ratios for MUST	Sharp

R1-163169	Discussion on candidate multiuser superposition transmission scheme	NTT DOCOMO, INC.

R1-163299	On the operation and network assistance for MUST Case 1 and 2	Nokia, Alcatel-Lucent Shanghai Bell

R1-163340	Consideration on power allocations for MUST	Huawei, HiSilicon

R1-163788	WF on modulation order and spec enhancement	LG Electronics, Intel, MediaTek

Note:

- MUST-Near/MUST-Far UEs will be used in the relevant discussions.

Agreement:

- For Case 1 and 2 described in MUST WID,

  - MUST-Far UE’s modulation order is limited at least to QPSK when it is co-scheduled with MUST-Near UE in a given subframe

    - FFS whether or not to support 16QAM for MUST-Far UEs

      - More study is necessary

R1-163776	Way forward on superposition transmission for MUST	MediaTek, Huawei, HiSilicon

Agreement:

- For Case 1 & 2, up to two co-scheduled UEs per spatial layer are supported

- For Case 1 & 2, MUST category 2 with one or more transmission power ratios for co-scheduled MUST UEs in each constellation combination is supported

  - One or more transmission power ratios for each constellation combination are supported

    - FFS: The number of multiple power ratios is down-selected from 1 to 8

  - The superposed constellation corresponding to one of transmission power ratios in each constellation combination is a legacy constellation

    - For (MOD_N, MOD_F) = (QPSK, QPSK), 16QAM legacy constellation

    - For (MOD_N, MOD_F) = (16QAM, QPSK), 64QAM legacy constellation

    - For (MOD_N, MOD_F) = (64QAM, QPSK), 256QAM legacy constellation

  - If 2 or more power ratios are supported, the other multiple transmission power ratios for a MUST-far UE in each constellation combination can be selected from the following value ranges:

    - For (MOD_N, MOD_F) = (QPSK, QPSK), the power ratio range as a starting point is [0.6, 0.95]

    - For (MOD_N, MOD_F) = (16QAM, QPSK), the power ratio range as a starting point is [0.6, 0.95]

    - For (MOD_N, MOD_F) = (64QAM, QPSK), the power ratio range as a starting point is [0.6, 0.95]

  - FFS the impact if 16QAM for MUST-Far UE is supported

  - For further down-selection on the set of transmission power ratios, companies are encouraged to provide the scheduling PDF of power ratios and the corresponding performance for different sets of power ratios

Agreements:

For all MUST evaluations (cases 1, 2 & 3):

•For 2Tx & Case 1 and 2, up to two co-scheduled UEs per spatial layer are considered

•For 2Tx & Case 3, up to two co-scheduled UEs within a cell are considered

•For 4/8Tx & Case 3, up to four co-scheduled UEs within a cell are considered

Note that the above scheduling options may be considered per PRB, per group of PRBs, or per scheduled UE’s bandwidth

R1-162219	Transmitter side details of MUST Category 2	ZTE

R1-162290	Selection of superposition transmission scheme and parameterization	CATT

R1-162493	Discussion on power ratio and modulation order	LG Electronics

R1-162848	On modulation order and power ratio for MUST	China Telecommunications

R1-162879	down-selection for superposition scheme	Sequans Communications

R1-163045	Superposition transmissions for MUST	Qualcomm Incorporated

R1-163188	Discussion on superposition transmission	Intel Corporation

R1-163301	On the specification approach of MUST	Nokia, Alcatel-Lucent Shanghai Bell

R1-163335	MUST with legacy composite constellation	Huawei, HiSilicon

#### Mechanisms for efficient operation

Related to RRC signaling, DCI, UE blind detection, etc.

R1-162291	Support of downlink superposition transmission	CATT

R1-162433	MUST operational and signalling aspects	NEC

R1-162494	Signaling schemes for MUST assistance information	LG Electronics

R1-162849	On downlink MUST operation mechanism	China Telecommunications

R1-163046	Mechanisms for efficient operation for MUST	Qualcomm Incorporated

R1-163092	Efficient MUST Operation	Ericsson

R1-163777	Way forward on UE assumptions for CFI in MUST	MediaTek, Nokia, ASB

Also supported by Huawei, HiSi

Continue discussion offline

R1-163824	Way forward on control signaling for MUST		ZTE, LG Electronics

LGE withdrew the co-sourcing

Continue discussion offline

R1-163774	Way forward on MUST mechanisms for efficient operation	NTT DOCOMO, INC., MediaTek

Agreement:

- For MUST case 1 and case 2, the candidate assistance information for signalling or blind detection by the MUST-near UE include:

  - Existence of MUST interference per spatial layer

  - Transmission power allocation per spatial layer of its PDSCH and of the MUST-far UE’s PDSCH

  - Modulation order of each codeword of MUST paired UE’s PDSCH

    - This information is only needed if modulation order of MUST-far UEs is not limited to QPSK

- For MUST case 3, in addition to the above:

  - PMI or DMRS port/sequence of the MUST-paired UE

- Each of the above may be either:

  - per PRB, or

  - per group of PRBs, or

  - single value across the UE’s scheduled bandwidth

R1-163775	Draft LS to RAN4	Mediatek

Revised and agreed in R1-163836.

R1-163797	WF on assistance information for MUST-near UE	Huawei, HiSilicon, DOCOMO

Also supported by Sharp

Continue discussion offline

R1-163835	WF on DCI signalling	ZTE

R1-162220	Control signaling aspects of MUST Cateogy 2	ZTE

R1-162562	Considerations of Efficient Control Signaling for MUST	Sony

R1-162615	Consideration on parameter blind detection for MUST UE	Huawei, HiSilicon

R1-162746	Parameters for Blind Detection in MUST	MediaTek Inc.

R1-162747	Draft LS on Potential Parameters for Blind Detection in MUST	MediaTek Inc.

R1-162748	Mechanisms for Operation of MUST	MediaTek Inc.

R1-162881	configuration of superposition scheme	Sequans Communications

R1-163170	Views on network assistance signaling for DL MUST	NTT DOCOMO, INC.

R1-163189	On downlink control signalling for MUST	Intel Corporation

R1-163282	Discussion on Downlink Control Signaling for MUST 	ASUSTEK COMPUTER (SHANGHAI)

R1-163302	On the network assistance and blind detection for MUST Case 3	Nokia, Alcatel-Lucent Shanghai Bell

R1-163303	Further details on the operation and network assistance for MUST Case 3	Nokia, Alcatel-Lucent Shanghai Bell

#### Evaluation for MUST using up to 8Tx

Additional evaluation methodology/assumptions, evaluation results (if needed)

R1-162221	Evaluation for MUST using up to 8Tx antennas	ZTE

R1-162616	Additional evaluation methodology and assumptions for 4Tx and 8Tx	Huawei, HiSilicon

R1-162749	Issues on Evaluation for MUST When Different Precoders Are Used for Near and Far Users	MediaTek Inc.

R1-162864	Evaluation results for MUST using 8Tx with different precoders	CMCC

R1-163300	System-level Evaluation of Case 1 and 3 MUST	Nokia, Alcatel-Lucent Shanghai Bell

R1-163778	WF on additional evaluation methodology for Rel-14 MUST	Mediatek, Huawei, HiSilicon

Also supported by CMCC

Agreements for System Level Evaluations:

- Follow the UE receiver assumptions described in TR36.859

- Each company is encouraged to provide the following information together with system-level results:

  - Detailed method of link-to-system mapping applied in the system-level evaluation

  - Assumptions on CSI feedback, network assistance signalling, and blind detection

    - Note that realistic feedback should be assumed

  - Assumptions on DMRS configurations

#### Other

R1-162396	Discussion on enhancement for MUST	Fujitsu

R1-162495	Discussion on Gray mapping for superposed constellation	LG Electronics

### eMBMS enhancements for LTE - WID in RP-160675

R1-163840	Chairman's Notes of Agenda Item 7.3.5 on eMBMS enhancements for LTE	Session Chairman (Nokia)

The document was presented by Matthew Baker from Nokia and .provides the outcomes of the eMBMS session.

Discussion: .

Decision: The document is endorsed.

To revisit on Friday:

R1-163452: Scenarios and simulation assumptions for longer CP (David Bhatoolaul, Nokia):

Try to agree it quickly; if it needs more time, email approval until Friday 22nd April.

R1-163285	Work plan for  eMBMS enhancements in LTE	Ericsson

Decision: The document is noted.

Clarifications regarding the WID:

- Objective a may be supported on carrier on which legacy UEs can be scheduled

  - It is assumed that subframes contain a new numerology are signalled as MBSFN subframes

  - Signalling of services to legacy and new UEs is handled by higher layers, so RAN1 assumes that legacy UEs would not be configured to receive MBMS services in an MBSFN subframe that had a new numerology

  - If legacy UEs are to be supported on the carrier, the legacy control region shall not be impacted by the new numerology in a subframe

  - FFS whether MBMS MDT measurements would be impacted by any new numerology

- Objectives b and c mean that legacy UEs cannot be scheduled on the carrier

- Carriers supporting objective b may also support unicast traffic for Rel-14 UEs

- FFS what SI has to be supported on a carrier supporting objective b

- FFS whether carriers supporting objective c may also support unicast traffic for Rel-14 UEs

- FFS whether the reduced density of CRS with objectives b and c impacts time/frequency tracking, and if so, how this is handled

- Note that if there is additional RAN1 impact from objective d, it is in the area of how to support the self-contained SI as mentioned in WID

R1-163286	Skeleton TR for eMBMS enhancements for LTE	Ericsson

Decision: The document is noted.

Correct the typos in TR and include the conclusion regarding objective f - R1-163451 – approved as v0.1.0 – TR number to be allocated by MCC.

Send endorsed skeleton TR to RAN2 in an LS and delegate responsibility for populating the TR to RAN2 - R1-163470 – approved in R1-163450 with the following change: “With the above conclusion, RAN1 expects that most of the work in the objectives that are related to the study part of the work item is in the remit of RAN2.”

Not treated.

R1-162736	General discussion on eMBMS enhancements	Lenovo (Beijing) Ltd

#### Longer cyclic prefix for MBSFN subframe

Priority at RAN1#84bis is to define the required simulation assumptions.

Simulation assumptions

R1-163363	Longer cyclic prefix evaluation considerations	Nokia, Alcatel-Lucent Shanghai Bell

Decision: The document is noted.

Note typo in table 3: 20-tap Rayleigh fading channel, plus caption relates to Scenario 2.

FFS what UE speeds need to be / can be supported.

Intel – should consider a high mobility scenario

R1-163288	Simulation assumptions for evaluation of longer CP for MBSFN subframes	Ericsson

Decision: The document is noted.

R1-163163	Discussion on longer CP for MBSFN subframe	Intel Corporation

Decision: The document is noted.

R1-162138	Simulation assumption and evaluation method for longer cyclic prefix	Huawei, HiSilicon

Decision: The document is noted.

R1-163047	Evaluation assumptions for longer cyclic prefix	Qualcomm Incorporated

Decision: The document is noted.

Offline discussion during this week to summarise the scenarios and simulation assumptions on which consensus can be reached – David Bhatoolaul (Nokia) - R1-163452 (Fri, or email approval)

Initial simulation results

R1-162137	Consideration on longer cyclic prefix	Huawei, HiSilicon

Decision: The document is noted.

R1-163048	Evaluation results for longer cyclic prefix	Qualcomm Incorporated

Decision: The document is noted.

Discussion

R1-163049	Design options for longer cyclic prefix for MBSFN subframes	Qualcomm Incorporated

Decision: The document is noted.

R1-163287	Implications of longer CP for MBSFN subframes	Ericsson

Decision: The document is noted.

#### Increased number of MBSFN subframes per radio frame

R1-163289	Radio frame design for extended MBSFN subframe allocation	Ericsson

Decision: The document is noted.

R1-163364	Additional subframes for MBSFN usage considerations	Nokia, Alcatel-Lucent Shanghai Bell

Decision: The document is noted.

R1-163050	Increased number of MBSFN subframes per radio frame	Qualcomm Incorporated

Decision: The document is noted.

R1-162139	Consideration on increasing amount of MBSFN subframes	Huawei, HiSilicon

Decision: The document is noted.

#### Study support for standalone eMBMS carrier

R1-163365	Standalone eMBMS carrier considerations	Nokia, Alcatel-Lucent Shanghai Bell

R1-163290	Standalone eMBMS carrier	Ericsson

R1-163051	Standalone eMBMS carrier	Qualcomm Incorporated

R1-162620	Consideration on support for standalone MBSFN carrier	Huawei, HiSilicon

#### Study TV transport service of UEs not being authenticated

Study whether there is any impact to the RAN1 specification by UEs receiving a TV transport service in a cell without being authenticated

R1-162621	Consideration on TV reception without authentication	Huawei, HiSilicon

R1-163052	Specification impact of anonymous TV service	Qualcomm Incorporated

R1-163291	RAN1 impact of supporting TV services reception for unauthenticated UEs	Ericsson

Conclusion: There is no RAN1 impact from objective f

Capture this in the update of the TR

#### Other

### SRS switching between LTE component carriers - WID in RP-160676

R1-163841	Chairman's Notes of Agenda Item 7.3.6 on SRS Carrier Based Switching for LTE	Session Chairman (Nokia)

The document was presented by Matthew Baker from Nokia and .provides the outcomes of the SRS session.

Discussion: .

Decision: The document is endorsed.

No issues to revisit on Friday

R1-162587	Work plan for SRS switching WI	Huawei, HiSilicon

Decision: The document is noted.

#### General Operation of SRS Carrier Based Switching

Develop general mechanism.

R1-163211	Scenarios and requirements on SRS switching	SoftBank Corp.

Decision: The document is noted.

R1-163166	Discussion on SRS carrier based switching	Nokia, Alcatel-Lucent Shanghai Bell

Decision: The document is noted.

R1-162585	General Operation of SRS Carrier Based Switching	Huawei, HiSilicon

Decision: The document is noted.

R1-162737	Discussion on SRS carrier switching	Lenovo (Beijing) Ltd

R1-162769	On General Operation of SRS Carrier Based Switching	Ericsson

Decision: The document is noted.

R1-162770	[DRAFT] LS on SRS carrier switching interruption time	Ericsson

Decision: The document is noted. Approved in R1-163453 with the following changes:

RAN1 expects that there will be changes to the interruption time requirements due to the introduction of SRS carrier based switching and would like RAN4 to:

- Provide the applicable RF retuning time and interruption time (in terms of µs rather than subframes) due to SRS carrier based switching considering

  - Inter-band CA, Intra-band CA and combination of intra- and inter-band CA

- Inform RAN1 whether it is possible to apply SRS based carrier based switching between any carriers

- Inform RAN1 whether there is any impact to DL reception in the UE due to the application of SRS carrier based switching

Note that since the feature is intended to support reciprocity based channel estimation, the antenna or antennas used for SRS switching should be the same as used for DL reception on the given CC.

R1-162292	Discussion on carrier based SRS switching	CATT

R1-163053	General operation of SRS switching	Qualcomm Incorporated

R1-163054	Draft LS on switching time for SRS	Qualcomm Incorporated

R1-163614	Way Forward on SRS carrier based switching	Huawei, HiSilicon, Qualcomm, Softbank

Decision: The document is noted.

Agreement:

Design Principles for SRS carrier based switching:

- SRS carrier based switching support should follow these general guidelines:

  - Applicable for deployment scenarios that have a TDD CC without the capability to have the corresponding UL CC configured

    - FFS: For eIMTA

  - Strive to reduce negative impacts on other transmissions and reception, especially UCI

  - Take into consideration switching times

Agreements:

- SRS switching solution(s) should be applicable to any TDD UL-DL configurations and special subframe configurations.

- SRS switching solution(s) should be applicable to any combination of UL-DL and special subframe configurations among CCs that are not prevented for other reasons

- The solution should not require to introduce any new TDD UL-DL configuration or special subframe configuration

#### Specification Impacts to Support SRS Carrier based Switching

Focus is on identifying more detailed specification changes / enhancements in order to support SRS carrier based switching

R1-163055	Specification impact of supporting SRS switching	Qualcomm Incorporated

Decision: The document is noted.

R1-162586	Specification Impacts to Support SRS Carrier based Switching	Huawei, HiSilicon

Decision: The document is noted.

R1-162698	SRS Carrier Switching Aspects	Samsung

R1-162771	Specification Impacts to Support SRS Carrier based Switching	Ericsson

Decision: The document is noted.

Conclusions:

Potential standards impacts to consider include:

- Configurations for SRS on TDD CCs without PUSCH

- SRS switching indication signaling

  - FFS: RRC or L1 or both

- Collision handling

- FFS: Possible new symbol positions for SRS

- Power control for SRS on TDD CCs without PUSCH

- Timing advance for SRS on TDD CCs without PUSCH

- FFS: Which of periodic and/or aperiodic SRS is supported, and corresponding details/enhancements

#### Other

### Further Indoor Positioning enhancements for UTRA and LTE - WID in RP-160538

R1-163894	Chairman's Notes of Agenda Item 7.3.7 on IndPos_enh	Ad-hoc chair (Qualcomm)

The document was presented by Wanshi Chen from Qualcomm and .provides the outcomes of the IndPos_enh session.

Discussion: .

Decision: The document is endorsed.

R1-163283	Considerations on Distributed Antenna System (DAS) Scenario for Indoor Positioning	China Unicom

Decision: The document is noted.

#### OTDOA Enhancements

R1-162606	Discussion on the same PCI case for OTDOA enhancements	Huawei, HiSilicon

Decision: The document is noted.

R1-162935	On OTDOA enhancements	Intel Corporation

R1-163056	Introduction of OTDOA enhancements for the shared Cell-ID scenario	Qualcomm Incorporated

R1-163057	Draft CR Introduction of OTDOA enhancements for the shared Cell-ID scenario	Qualcomm Incorporated

R1-163551	WF on sorting out the options for OTDOA enhancements	Huawei, HiSilicon

Decision: The document is noted.

Agreement:

- As a starting point for Rel-14 WI, RAN1 decides to prioritize the following:

  - OTDOA enhancements for the same PCI:

    - Support positioning related reporting by a UE corresponding to the respective PRS muting pattern per TP

    - Support virtual cell ID per TP

      - FFS: How to generate PRS sequence /v_shift with the virtual cell ID per TP until RAN1#85

R1-162874	Further details on PRS pattern enhancement	Ericsson

Decision: The document is noted.

Discuss further till next meeting

R1-162875	Richer RSTD reporting for OTDOA enhancements	Ericsson

Decision: The document is noted.

R1-162876	Adaptive RSTD measurement reporting for OTDOA enhancements	Ericsson

Decision: The document is noted.

R1-163058	Introduction of UE inter-frequency RSTD calibration accuracy reporting	Qualcomm Incorporated

Decision: The document is noted.

Further discussion offline

#### PRS-based Beacons

R1-163059	Introduction of PRS based Terrestrial Beacon Systems	Qualcomm Incorporated

Decision: The document is noted.

Further discussion offline

#### E-CID Enhancements

R1-162607	Discussion on the same PCI case for E-CID enhancements	Huawei, HiSilicon

Decision: The document is noted.

R1-162608	Discussion on the UE Rx-Tx measurement over multiple serving cells	Huawei, HiSilicon

Decision: The document is noted.

Further discussion offline

R1-162877	E-CID Enhancement for Indoor Positioning	Ericsson

Decision: The document is noted.

Further discussion offline

#### Other

R1-162878	Indoor/Outdoor Classification for Indoor Positioning Enhancement	Ericsson

### Uplink Capacity Enhancements for LTE - WID in RP-160664

Specification of 256QAM in uplink transmission, with focus on the design of MCS table, 256QAM related signalling, and UE category. In addition, specify support of UL PUSCH in special subframe (the work starts from Q3 2016).

R1-163842	Chairman's Notes of Agenda Item 7.3.8 on Uplink Capacity Enhancements for LTE	Session Chairman (Nokia)

The document was presented by Matthew Baker from Nokia and .provides the outcomes of the UpCap session.

Discussion: .

Decision: The document is endorsed.

To revisit on Friday:

R1-163463 LS to RAN4 on 256QAM support per band & band combination - (Klaus, Nokia)

- should be quick to approve.

R1-163344	Work Plan for the UL Capacity Enhancement for LTE WI	Ericsson, CMCC

Note – according to the WID agreed by RAN plenary, work on PUSCH in UpPTS will not start until Q3 – hence companies are welcome to submit contributions earlier but there will not be an agenda item or online discussion before Q3.

Constellation

R1-163269	Non-uniform 256QAM for UL Capacity Enhancement	Sony

R1-163650	WF on UL 256QAM modulation mapping	Huawei, HiSilicon, CATR, Nokia, Alcatel-Lucent Shanghai Bell, Ericsson

Agreement:

Re-use the modulation mapping in section 7.1.5 of 36.211 for UL 256QAM

MCS & TBS tables

R1-163347	Physical Layer Design Aspects of UL 256QAM	Ericsson

Agreements:

- Assume the same number of bits for the MCS field in the DCI message to support UL 256QAM as it is currently, i.e. 5 bits

- MCS table supporting UL 256QAM can only be used in the UE specific search space for DCI format 0 and DCI format 4, and any new UL DCI format introduced for LAA, when the CRC is scrambled with C-RNTI

  - FFS whether the configuration of the UL 256QAM MCS table can be done separately for DCI formats 0 and 4

  - Configure the use of the MCS table for UL 256QAM separately per uplink power control subframe set

- Introduce Qm corresponding to 256QAM for UL at applicable places in the specification

- UL 256QAM and DL 256QAM are configured independently.

R1-163060	Discussion of UL 256QAM	Qualcomm Incorporated

R1-162865	Discussion on UL 256QAM design	CMCC

R1-162624	TBS table design for UL 256QAM	Huawei, HiSilicon

R1-163346	On the Performance of UL 256QAM	Ericsson

R1-163345	MCS Table Design for UL 256QAM	Ericsson

R1-162140	MCS table for UL 256QAM	Huawei, HiSilicon

R1-163696	Discussion on standardization impacts of UL 256QAM	Intel Corporation

Sections 2.1

Revision of R1-162372

R1-162787	On 256QAM UL MCS Table design	Nokia, Alcatel-Lucent Shanghai Bell

R1-163690	WF on UL 256QAM	Ericsson, Nokia, Alcatel-Lucent Shanghai Bell

R1-163652	WF on UL 256QAM MCS table	Huawei, HiSilicon, CATR, ZTE

Agreements:

- The minimum TBS index for 16QAM is 11

- The minimum TBS index for 64QAM is 20

- The minimum TBS index for 256QAM is 30

- 3 entries are reserved for retransmissions

- The number of PUSCH MCS entries to support 256QAM is down-selected between

  - Alt 1: 6

  - Alt 2: 5

  - Alt 3: 4

R1-163651	WF on UL 256QAM TBS table	Huawei, HiSilicon, CATR, CMCC, China Unicom, China Telecom, ZTE, Deutsche Telekom, Sprint

Also supported by AT&T.

Agreements:

- Reference number of REs available in PUSCH for UL 256QAM TBS design is 144 REs/PRB

- The maximum coding rate for PUSCH with 256QAM should not exceed 0.931

- FFS whether or not to introduce new TBS index(es) for PUSCH with 256QAM

  - Companies are encouraged to submit evaluations to RAN1#85

EVM

R1-163061	Initial link level evaluation results for UL 256QAM	Qualcomm Incorporated

RAN4 is assumed to be already doing this work.

UE categories

R1-162622	Discussion on UE categories with introduction of UL 256QAM	Huawei, HiSilicon

R1-162786	On 256QAM UL UE categories and UE capability signalling	Nokia, Alcatel-Lucent Shanghai Bell

R1-163348	UE Category and UE Capability Support for UL 256QAM	Ericsson

R1-163696	Discussion on standardization impacts of UL 256QAM	Intel Corporation

Section 2.3

Revision of R1-162372

R1-163708	WF on UL 256QAM UE capabilities	Nokia, Alcatel-Lucent Shanghai Bell, Ericsson

Agreements:

- Introduce at least 3 new UL UE categories achieving peak data rate for 256QAM PUSCH

  - Based on UL UE category 5

  - Based on UL UE category 13

  - Based on UL UE category 14

  - FFS: Based on UL UE category 8

Note: the 256QAM UL peak data rate is around 30-40% higher than the base categories listed above.

- Extend the UL/DL UE category combinations in TS 36.306 to include for each current DL UE category paired with a 64QAM UL UE category a new combination with the respective enhanced 256QAM UL UE category

  - A UE signalling a 256QAM category shall also signal its base category

- UE 256QAM UL support/capability can only be indicated by a UE belonging to the new UL UE Categories achieving 256QAM UL peak data rate (i.e. ue-CategoryUL >14).

- Send an LS to RAN4 to ask for their input on whether the 256QAM support should be per UE or per band & band combination - R1-163463 (Klaus, Nokia)

Note that RAN1 assumes that UE capability signalling details are up to RAN2.

### Study on LTE-based V2X Services - SID in RP-160657

R1-163895	Chairman's Notes of Agenda Item 7.3.9 on V2X	Ad-hoc chair (Qualcomm)

The document was presented by Wanshi Chen from Qualcomm and .provides the outcomes of the V2X session.

Decision: The document is endorsed.

Email approval for a TP capturing the agreements/conclusions until 22nd April – (LGE).

#### Evaluation results

To collect the evaluation results to be included in TR36.885

##### Uu transport for V2V, V2P, and V2I/N

Including evaluation of the feasibility in terms of meeting latency requirements, network coordination required, resource efficiency, and energy efficiency of UE

R1-162373	Performance analysis of Uu transport for V2V communication	Intel Corporation

Decision: The document is noted.

Not treated.

R1-162496	Evaluation results on the enhancement of Uu transport for V2V	LG Electronics

R1-162497	Evaluation results on the feasibility of Uu transport for V2P and V2I/N	LG Electronics

R1-162834	Simulation Results for Uu-Based V2V	Ericsson LM

##### PC5 transport for V2P and V2I/N

Including potential V2V performance degradation

Not treated.

R1-162293	Evaluation results on PC5-based V2I	CATT

R1-162294	Evaluation results on PC5-based V2P	CATT

R1-162699	Evaluation results for PC5 based V2P and V2I/N	Samsung

R1-163062	PC5 for V2V and V2P	Qualcomm Incorporated

R1-163183	Evaluation of PC5 transport for V2P/V2I/I2V communications	NTT DOCOMO, INC.

R1-163389	Evaluation results for LTE-based V2X	ZTE

#### Support of Uu transport for V2V, and PC5/Uu transport for V2I/N and V2P

##### Enhancement to multi-cell multicast/broadcast

Including optimization on set of cells performing the same multicast/broadcast and multicast/broadcast transmission based on PDSCH/PMCH

R1-163182	Evaluation of Uu transport for V2x communications	NTT DOCOMO, INC.

Decision: The document is noted.

R1-162374	Discussion on multi-cell multicast/broadcast enhancements for V2V communication	Intel Corporation

Decision: The document is noted.

R1-162499	Discussion on potential enhancements for multi-cell multicast/broadcast	LG Electronics

Decision: The document is noted.

R1-162124	Uu DL enhancements for V2X	Huawei, HiSilicon

Decision: The document is noted.

R1-163063	System level performance using SC-PTM and eMBMS	Qualcomm Incorporated

Decision: The document is noted.

R1-163604	WF on enhancements for multi-cell multicast/broadcast	LG Electronics, CATR, CMCC, Deutsche Telekom, General Motors, ETRI, Fujitsu, Huawei, HiSilicon, III, ITL, KRRI, KT Corporation, LG Uplus, OPPO, Samsung, SK Telecom, Xinwei, ZTE

Decision: The document is noted.

Conclusion:

- RAN1 has observed performance benefit with the following enhancements to DL multicast/broadcast :

  - Dynamic scheduling for multicast/broadcast transmissions

    - I.e., PDCCH-based scheduling of TBs associated to a TMGI

  - Semi-static scheduling for multicast/broadcast transmissions

  - Use of DM-RS based transmission scheme from multiple TPs including reception for idle UEs

    - This does not imply introduction of any new TM

  - DMRS-based single-cell multicast

  - PDSCH transmitted from multiple TPs

  - CRS based PDSCH/PDCCH transmitted from TP(s) that differ from the TP(s) transmitting the system information

  - DMRS based PDSCH/EPDCCH transmitted from different TP(s)

  - Use of normal CP

  - HARQ feedback

  - CSI feedback

  - Note 1: Gain, complexity, and specification impact of each enhancement are expected to be different.

  - Note 2: some enhancements may be possible without spec impact.

  - Note 3: Network/UE complexity has not been studied in RAN1.

  - Note 4: some enhancement(s) listed above may assume some level of inter-TP coordination, which does not necessarily result in spec changes

  - Note 5: some enhancement(s) listed above may be based on contribution(s) from a limited number of company or companies

- RAN1 is still continuing study at least some of the above enhancements

- The above is to be captured in the TR

- Send LS to RAN2 to inform the following feature is beneficial from RAN1 viewpoint:

  - UE identifies which broadcast transmissions (e.g., TMGI) are relevant to it, e.g., depending on the position of the UE.

R1-163860	LS on DL broadcast/multicast for V2X	LGE

Decision: The document is noted and final LS is agreed in R1-163862.

R1-163701	Way Forward on DL enhancement for Uu-based V2X		Huawei, HiSilicon, Ericsson

Decision: The document is noted.

Not treated.

R1-162418	Multi-cell multicast/broadcast for V2V	ZTE

R1-162700	PDSCH-based enhancements for V2X multicasting	Samsung

R1-163850	WF on enhancements for V2X	LG Electronics, Ericsson, Huawei, HiSilicon, Samsung

##### Enhancements to unicast

Including UL enhancement (e.g., SPS enhancement, SR enhancement)

R1-162419	Uu transport enhancement for V2X	ZTE

Decision: The document is noted.

R1-162500	Discussions on UL enhancements for V2X	LG Electronics

Decision: The document is noted.

R1-162563	SR enhancements on uplink unicast for V2X services	Sony

Decision: The document is noted.

R1-163603	WF on UL SPS for Uu	Ericsson, Huawei

Decision: The document is noted.

R1-163827	Harmonized WF on UL SPS for Uu and PC5		Ericsson, Huawei, LGE, Panasonic

Decision: The document is noted.

##### Enhancements to sidelink

Including identification of the case requiring further enhancements over PC5-based V2V and complexity of supporting transmission of pedestrian UEs

R1-162831	Uu Enhancements for V2X	Ericsson LM

Decision: The document is noted.

R1-163634	WF on PC5-based V2P/P2V LG Electronics, NTT DOCOMO, Ericsson

Decision: The document is noted.

Conclusion:

- The following observations are made in PC5-based V2P/P2V.

  - The scenario of P-UE TX to V-UE RX is more battery efficient than the scenario of V-UE TX to P-UE RX

    - Note: this observation is made based on evaluations from a limited number of companies

    - Analysis result of P-UE power consumption based on figure 1 in R1-163062 (details to be discussed during TP preparation) is captured in TR 36.885.

  - For the purpose of for P-UE TX to reduce the power consumption and UE complexity, at least the followings are beneficial:

    - Random resource selection.

    - FFS Sensing operation during a limited time

Not treated.

R1-162295	Discussion on energy efficiency in PC5-based V2P	CATT

R1-162420	Discussion on V2P design aspects	ZTE

R1-162434	Discussion on potential issues and enhancements for V2P	NEC

R1-162501	Discussions on SL enhancements for V2X	LG Electronics

R1-162832	Discussion on P2x-x2P	Ericsson LM

R1-163184	Discussion on PC5-based V2P operation	NTT DOCOMO, INC.

##### Other

Not treated.

R1-162421	RSU enhancement	ZTE

#### Identification of high level coexistence approaches (long-term basis) between PC5 transport for V2V services and DSRC/IEEE 802.11p services in the same channel

R1-163854	WF on DSRC/LTE-based ITS Coexistence	Ericsson

Decision: The document is noted.

Conclusion:

- The LS reply from RAN1 to RAN for LTE/DSRC coexistence includes in principle the points:

  - RAN1 has discussed co-existence between DSRC and LTE-based ITS (sidelink) for safety ITS. RAN1 believes that ITS systems are important and should protect each other and in particular safety ITS.

  - Technology neutrality is essential in order to enable choice of most suitable radio technology for each ITS service as well as to enable a technology phasing in the future (e.g., towards 5G)

  - RAN1 believes that technology neutrality is enabled by describing common coexistence rules that are followed by all potential ITS technologies. Such rules may be specific for a certain region. RAN1 believes that the details of the common coexistence rules are out of RAN1 scope and should be discussed in relevant SDOs in each region.

  - For deployment of PC5-based LTE-V2V and 802.11p in the same geographical area, the ideal option is when they use different frequency channels. Note that co-deployment of both technologies is not likely to happen in all regions.

    - FFS in terms of providing example(s)

  - FFS other aspects

Not treated.

R1-162502	Discussions on co-channel coeixsistence of PC5-based V2V and IEEE802.11p	LG Electronics

R1-162609	Co-channel of PC5 V2V with other techniques	Huawei, HiSilicon

R1-162702	DSRC co-existence with V2V	Samsung

R1-162814	Coexistence approaches between different technologies for ITS services	Ericsson LM

R1-163064	Co-channel coexistence between V2V and DSRC	Qualcomm Incorporated

### Study on Latency reduction techniques - SID in RP-150465

Focus on this SI in 2Q 2016 will follow approved WF (RP-160650)

R1-162116	Discussion on enhanced frame structure  for latency reduction in TDD	Huawei, HiSilicon

R1-163523	Way Forward on enhanced frame structure type 2 for latency reduction	Huawei, HiSilicon

The document was presented by Yongxia Lyu from Huawei.

- Enhanced frame structure type 2 is recommended to be supported for latency reduction

  - Additional subframe type(s) with symbol(s) for downlink transmission, GP and symbol(s) for uplink transmission should be supported

    - Downlink transmission includes DL control and DL data

    - Uplink transmission includes UL control and UL data

      - FFS SRS

    - FFS GP position and GP duration

  - More than 2 Downlink-to-Uplink Switch-points in a radio frame should be supported

    - FFS the number of the new Downlink-to-Uplink Switch-points in a radio frame

    - FFS the position(s) of the new Downlink-to-Uplink Switch-points in a radio frame

  - Backward compatibility should be preserved

    - Additional subframe type(s) can be used to replace a subset of the following subframe(s)

      - Alt. 1:  MBSFN subframe(s) and UL subframe(s) configured to legacy UEs

      - Alt. 2:  UL subframe(s) according to the UL/DL configuration indicated by SIB1

      - Alt. 3:  MBSFN subframe(s) configured to legacy UEs

      - Alt. 4:  DL subframes and UL subframe(s) according to the UL/DL configuration indicated by SIB1

      - FFS which alternative to be selected.

    - Special subframe(s) according to the UL/DL configuration indicated by SIB1 can be used as additional subframe type(s) for new UEs

Discussion: Also supported by Panasonic.

Decision: The document is noted. Continue offline discussions until Wednesday – (Huawei)

R1-163843	Chairman's Notes of Agenda Item 7.3.10 on Study on Latency Reduction Techniques	Session Chairman (Nokia)

The document was presented by Matthew Baker from Nokia and .provides the outcomes of the Latency session.

Discussion: .

Decision: The document is endorsed.

To revisit on Friday:

- R1-163466: Link level evaluation results (Daniel, Ericsson) – should be quick to approve.

- R1-163664: Findings from link level evaluation results (Daniel, Ericsson) – should be quick to approve.

- R1-163464: Link level evaluation results (Laetitia, Ericsson) – should be quick to approve.

- R1-163718: Findings from link level evaluation results (Laetitia, Ericsson) – should be quick to approve.

- R1-163545: WF on sPDCCH (Zhizong, ZTE) – not urgent, and probably still no consensus – postpone to RAN1#85.

- R1-163726: WF on sPUCCH design (Yunjung, LG) – not urgent, and probably still no consensus – postpone to RAN1#85.

- R1-163748: WF on enhanced FS2 (Yongxia, HW) – not urgent, and probably still no consensus – postpone to RAN1#85.

(All other WFs not urgent and can wait till RAN1#85.)

#### Evaluation results

System and link evaluation results

Link level evaluations

R1-163517	Text proposal on link evaluation results	Ericsson

Continue offline discussions until Wednesday – Laetitia (Ericsson)

Revised in R1-163719;

Revised in R1-163732

To be updated to include results from R1-163465, plus throughput results for PUSCH and PDSCH from other companies which are to be provided to Daniel (Ericsson) by 8:30am Thur at the latest (BLER curves without associated throughput results will be deleted).

Revision in R1-163466 – Friday.

R1-163720	Findings from link level evaluation results	Ericsson

Revised in R1-163733 – to be updated including any comments from companies (on RAN1 reflector and to Daniel) and latest results - R1-163664 – Friday.

R1-162399	Link level evaluation of PUCCH for shortened TTI	ZTE

R1-162400	Link-level evaluation of TTI shortening for PDSCH	ZTE

R1-162401	Link level evaluation of SPDCCH for shortened TTI	ZTE

R1-162402	Link level evaluation of sPUSCH for shortened TTI	ZTE

R1-162842	Link level evaluation results of shortened TTI for CRS based PDSCH 	Sharp

R1-162843	Link level evaluation results of shortened TTI for DM-RS based PDSCH 	Sharp

R1-162844	Link level evaluation results of shortened TTI for PUSCH	Sharp

R1-162845	Link level evaluation results of shortened TTI for PUCCH 	Sharp

R1-163066	Link-level evaluation of TTI shortening in DL	Qualcomm Incorporated

R1-163067	Link-level evaluation of TTI shortening in UL	Qualcomm Incorporated

R1-163264	Link-level Evaluation of shorter PDSCH	Nokia, Alcatel-Lucent Shanghai Bell

R1-163313	Link evaluation for PDSCH for short TTI with CRS based transmission	Ericsson

R1-163314	Link evaluation for PDSCH for short TTI with DMRS based transmission	Ericsson

R1-163315	Link evaluation for PDCCH for short TTI	Ericsson

R1-163316	Link evaluation for PUSCH for short TTI	Ericsson

R1-163317	Link evaluation for PUCCH for short TTI	Ericsson

System level evaluations

R1-162107	Performance evaluation on TTI shortening	Huawei, HiSilicon

The document was presented by Yongxia Lyu from Huawei.

- Observation 1: UPT performance of 1ms TTI decreases a lot once FTP file size becomes larger than SSThresh, when TCP connection is kept for a long time.

- Observation 2: From 0ms to 10ms CN delay, the UPT gain decreases in 20MHz system.

- Observation 3: RU does not significantly affect the UPT for 500KB and 100KB, but for 12.5KB high RU brings much more UPT gain than lower RU.

- Observation 4: 2 symbols and 4 or 3 symbols TTI have significant mean UPT gain in both 10MHz and 20MHz system. 1 symbol TTI has marginal mean UPT gain for 500KB FTP file in 10MHz system. 7 symbols TTI has smaller mean UPT gain than other TTI lengths in both 10MHz and 20MHz system.

- Observation 5: Short TTI has more mean UPT gain in 120km/h than 3km/h.

  - The mean UPT gain of 2 symbols TTI in case of 3km/h and 120km/h is similar to 4 or 3 symbols TTI in 10MHz and 20MHz, but larger than other TTI length cases.

- Proposal 1: 2 symbols and 4 or 3 symbols TTI length could be further considered.

Decision: The document is noted.

R1-162536	Evaluation results on shortened TTI with dynamic overhead of control channel	Panasonic

The document was presented by … from Panasonic.

Observations

- Improvement of user throughput and reduction of packet delay by using shortened TTI. It is mainly because of short TCP ACK delay.

- The gain of File size = 100kbits is larger than it of 500kbyte

- The gain of cell 95% UPT UE and 50% UPT UE is larger than it of cell 5% UPT UE

- For very shortened TTI, overhead of RS/PDCCH would impact throughput and latency. Especially for 1-symbol TTI, the performance of UPT and latency would be quite challenging for cell-edge UEs.

- The gain of low load is larger than it of high load

Proposal 1: Different UEs may use different TTI length and it could be configured by eNB.

Proposal 2: RAN1 should discuss how to optimize overhead of RS and DCI for shortened TTI. In order to reduce overhead, shared RS among sTTIs and compact DCI could be considered.

Decision: The document is noted.

R1-162943	System-level performance evaluation for TCP throughput enhancement with dynamic TTI tuning	Mediatek

The document was presented by Weidong Yang from MediaTek.

- Proposal 1: RAN 1 decides sPDCCH design, and evaluates reasonable L1/L2 control overhead, so that latency reduction gain of shortened TTI for different scenarios can be correctly evaluated.

- Observation 2: Whether dynamic TTI scheme provides significant latency reduction gain over shortened TTI scheme depends heavily on the design of shortened TTI. If shortened TTI is not so efficient, e.g., involved with a larger proportion of control overhead compared to legacy TTI, dynamic TTI schemes have a larger room to enhance latency performance.

- Observation 3: Given a scenario of longer file transmission time, e.g., low Uu throughput scenario or a larger file size, dynamic TTI schemes provide more throughput gain over shortened TTI scheme, but less throughput gain over legacy TTI scheme.

- Observation 4: With file size is small, dynamic TTI scheme has the same delay gain as that of shortened TTI scheme.

- Observation 5: As file size increases, delay gain of shortened TTI scheme and dynamic TTI scheme are reduced.

- Observation 6: The delay gain of dynamic TTI over either shortened TTI or legacy TTI is non-negative for different file size.

- Observation 7: As file size increases, dynamic TTI scheme has better delay performance than either the legacy TTI scheme or the shortened TTI scheme.

- Proposal 2: RAN 1 evaluates the feasibility of dynamic TTI schemes so as to guarantee positive latency reduction gain for shortened-TTI enabled UE.

- Proposal 3. RAN 1 includes the evaluation of dynamic TTI to the technical report of RAN 1 LATED, i.e. TR 36.881.

Decision: The document is noted.

R1-163518	Text proposal on system level results for latency reduction	Ericsson

The document was presented by Laetitia Falconetti from Ericsson and suggests capturing in Annex B of the TR the system evaluation results of sTTI.

Discussion: Ericsson does not include all availble results to this meeting (for formatting reason).

Decision: The document is noted. Continue offline until Wednesday (Ericsson)

Wednesday

Revised in R1-163717

In case of fixed control overhead, add the following information in the note field (companies to provide this info to Laetitia):

- “Total control overhead per subframe compared to all available REs in the sTTIs [%]:”

- “Maximum number of UEs that can be scheduled per sTTI:”

Companies are asked to check 3717 and provide any comments (on RAN1 reflector and to Laetitia (Ericsson)) by 8:30am Thur.

Updated version to be agreed on Friday - R1-163464.

R1-163519	Text proposal on findings from the system level evaluation of latency reduction	Ericsson

The document was presented by Laetitia Falconetti from Ericsson and .

Discussion: Ericsson does not include all availble results to this meeting (for formatting reason).

Decision: The document is noted. Continue offline until Wednesday (Ericsson)

Wednesday

Revised in R1-163718  - be updated including any comments from companies (on RAN1 reflector and to Laetitia) revisit on Fri.

R1-163716	Notes from offline session on system level evaluations	Ericsson

The document was presented by … from … and .

Discussion: .

Decision: The document is noted.

R1-162117	Performance Evaluation for enhanced TDD frame structure	Huawei, HiSilicon

R1-162296	Evaluation results of shortend TTI with control channel constraint	CATT

R1-162375	System level evaluation of shortened TTI	Intel Corporation

R1-162398	System-level performance evaluation for TTI shortening	ZTE

R1-162399	Link level evaluation of PUCCH for shortend TTI	ZTE

R1-163387	Link-level evaluation of TTI shortening for PDSCH	ZTE

Revision of R1-162400

R1-162401	Link level evaluation of SPDCCH for shortened TTI	ZTE

R1-162402	Link level evaluation of sPUSCH for shortened TTI	ZTE

R1-162503	System-level simulation results for reduced TTI length	LG Electronics

R1-162618	Evaluation on fast link adaptation	Huawei, HiSilicon

R1-162703	SLS results of TTI shortening: Effect of FTP file sizes	Samsung

R1-162704	SLS results of TTI shortening: Effect of HARQ RTT	Samsung

R1-162705	SLS results of TTI shortening: Effect of UL access delay	Samsung

R1-162842	Link level evaluation results of shortened TTI for CRS based PDSCH 	Sharp

R1-162843	Link level evaluation results of shortened TTI for DM-RS based PDSCH 	Sharp

R1-162844	Link level evaluation results of shortened TTI for PUSCH	Sharp

R1-162845	Link level evaluation results of shortened TTI for PUCCH 	Sharp

R1-163065	System-level evaluation of TTI shortening	Qualcomm Incorporated

R1-163066	Link-level evaluation of TTI shortening in DL	Qualcomm Incorporated

R1-163067	Link-level evaluation of TTI shortening in UL	Qualcomm Incorporated

R1-163072	System level evaulation with shortened TTI for joint HARQ and CQI feedback	Qualcomm Incorporated

R1-163098	DL system-level performance evaluation for shortened TTI	Motorola Mobility Germany GmbH

R1-163259	System-level evaluations for latency reduction in DL	Fraunhofer HHI

R1-163264	Link-level Evaluation of shorter PDSCH	Nokia, Alcatel-Lucent Shanghai Bell

R1-163265	System-level Evaluation of TTI Shortening	Nokia, Alcatel-Lucent Shanghai Bell

R1-163312	System level evaluation of short TTI	Ericsson

R1-163313	Link evaluation for PDSCH for short TTI with CRS based transmission	Ericsson

R1-163314	Link evaluation for PDSCH for short TTI with DMRS based transmission	Ericsson

R1-163315	Link evaluation for PDCCH for short TTI	Ericsson

R1-163316	Link evaluation for PUSCH for short TTI	Ericsson

R1-163317	Link evaluation for PUCCH for short TTI	Ericsson

R1-163517	TP on link evaluations results for Latency	Ericsson

#### Channel design for shortened TTI

Provide channel design including reference signals for s(E)PDCCH, sPDSCH, sPUSCH, sPUCCH, and details on multiplexing between existing non-sTTI and sTTI UEs.

R1-163515	Way forward on sPDCCH	Huawei, HiSilicon, Ericsson

The document was presented by Yongxia Lyu from Huawei.

- sPDCCH (PDCCH for short TTI) needs to be introduced for short TTI.

  - Each short TTI on DL contains an sPDCCH region

    - An sPDCCH assigns a sPDSCH allocated within the same short TTI

      - The number of possible  sPDSCH within an short TTI is for further study

    - An sPDCCH grants a sPUSCH

      - The number of possible  sPUSCH to grant from an sPDCCH(s) within an sTTI is for further study

- Both CRS-based sPDCCH and DMRS-based sPDCCH are recommended to be supported

  - In the first sTTI of a subframe, legacy PDCCH region can be shared by CRS-based sPDCCH and legacy PDCCH;

  - In time domain, sPDCCH should be placed

    - For CRS-based sPDCCH, in first one or two symbols within a sTTI;

    - For DMRS-based sPDCCH,

      - Alt.1: in first one symbol or two symbols within a sTTI;

      - Alt.2: in all symbols within a sTTI.

  - In frequency domain, sPDCCH can be placed

    - Alt.1: in a distributed manner;

    - Alt.2: in a localized manner.

  - sPDSCH assigned by a sPDCCH can be allocated to unused REs within the sPDCCH region if sPDCCH region is not shared with PDCCH region

    - Details are for further study

  - sPDCCH is aggregated by a certain number of CCEs, where each CCE consists of 36 REs

  - For DMRS-based sPDCCH, FFS on whether sharing the same DMRS for sPDCCH and its scheduled sPDSCH or not.

- On sPDCCH monitoring region,

  - Frequency resource for sPDCCH monitoring region can be informed by eNB;

    - FFS: semi-static vs dynamic, cell-specific vs UE-specific, explicit vs implicit

- The number of maximum PDCCH BDs for UE SSs within a subframe is no more than

  - 32 for UEs not supporting UL transmission mode 2;

  - 48 for UEs supporting UL transmission mode 2.

- Two-level DCI can be considered for sTTI scheduling

  - Slow DCI is carried in legacy PDCCH region and transmitted at most once per subframe

  - Fast DCI is carried by sPDCCH and transmitted within a sTTI.

Decision: The document is noted.

Agreement:

- sPDCCH (PDCCH for short TTI) needs to be introduced for short TTI.

  - Each short TTI on DL may contain sPDCCH decoding candidates

The remaining parts of R1-163515 are for offline discussion until Wednesday – (Huawei, Ericsson)

R1-163516	Way forward on TMs for sTTI	Huawei, HiSilicon, Ericsson, Mitsubishi Electric

The document was presented by Yongxia Lyu from Huawei.

- Both CRS based TMs and DMRS based TMs are  recommended to be supported for sTTI transmission.

  - CRS can be used without modification to demodulate sPDSCH.

    - FFS: 4-layer MIMO using 4 CRS antenna ports.

  - It would be necessary to introduce new DMRS design(s) for sPDSCH demodulation

    - For a certain TTI length, PRB bundling may be necessary to achieve sufficient channel estimation accuracy.

    - FFS: the number of DMRS antenna ports that can be supported for a given short-TTI length.

    - The following DMRS patterns are further studied

      - CDM in time domain to design the DMRS pattern for sTTI

      - CDM in frequency domain to design the DMRS pattern for sTTI

Decision: The document is noted, modified as follows:

Agreements

- Both CRS based TMs and DMRS based TMs are recommended to be supported for DL sTTI transmission

  - No change for CRS definition

    - FFS: Supporting more than 2 layers for sPDSCHs

  - Further study is needed about DMRS design(s) for sPDSCH demodulation

    - For a certain TTI length, increased PRB bundling sizes may be necessary to achieve sufficient channel estimation accuracy.

    - FFS: the number of DMRS antenna ports that can be supported for a given short-TTI length.

    - For a certain TTI length, new DMRS design(s) may be needed

R1-163539	WF on sPDSCH	LGE

The document was presented by Yunjung Yi from LGE.

- A UE is not expected to receive legacy TTI unicast PDSCH and short TTI unicast PDSCH simultaneously on one carrier

- The configured short TTI length can be different per UE

- TTI length of PDSCH can be dynamically changed

Decision: The document is noted.

Possible agreements:

- A UE is not expected to receive legacy TTI unicast PDSCH and short TTI unicast PDSCH simultaneously on one carrier

- Single short-TTI length can be configured for a UE

- The configured short TTI length can be different per UE

- TTI length of PDSCH can be dynamically changed

Continue offline discussion until Wednesday – Yunjung (LGE)

R1-163538	WF on sPUCCH design	LG Electronics, Ericsson

The document was presented by Yunjung Yi from LGE.

- At least two sPUCCH formats are recommended to be supported

  - One sPUCCH format for HARQ-ACK and/or SR feedback for a serving cell

    - The maximum supportable payload: Two HARQ-ACK bits + One SR

    - Intra-TTI frequency hopping is recommended to be supported

  - One sPUCCH format for CSI feedback and/or multiple HARQ-ACK bits, e.g. as in CA and frame structure type 2

    - FFS: Supported payload size

    - FFS: Intra-TTI frequency hopping

Discussion: .

Decision: The document is noted. Continue offline discussion until Wednesday – Yunjung (LGE)

R1-163545	Way Forward on sPDCCH	ZTE, Mediatek

The document was presented by Zhisong Zuo from ZTE.

- SPDCCH  should be introduced in sTTI

  - SPDCCH  is located in the first or first few OFDM symbol(s) of a sTTI

  - Frequency resource for SPDCCH should be configurable

  - Demodulation based on CRS

- Legacy PDCCH or SPDCCH can be used to schedule sPDSCH in one sTTI

- Number of blind decodes for one UE in a sTTI should be limited

- Efficient multiplexing of Data and Control, e.g. REs  on the first few symbols not used for control can be used for Data

Discussion: .

Decision: The document is noted. Continue offline discussion until Wednesday – Zhisong (ZTE)

The document was presented by … from … and .

Discussion: .

Decision: The document is noted.

R1-162108	Views on TTI length	Huawei, HiSilicon

R1-162109	PDCCH design for short TTI	Huawei, HiSilicon

R1-162110	PUCCH design for short TTI	Huawei, HiSilicon

R1-162111	Discussion on DL HARQ timing  for short TTI	Huawei, HiSilicon

R1-162112	Discussion on DL CRS  for short TTI	Huawei, HiSilicon

R1-162113	Discussion on DL DMRS  for short TTI	Huawei, HiSilicon

R1-162114	Discussion on UL RS  for short TTI	Huawei, HiSilicon

R1-162115	Short TTI for UL transmissions	Huawei, HiSilicon

R1-162297	Design of sPDCCH and sPDSCH for shortened TTI	CATT

R1-162298	Design of sPUCCH for shortened TTI	CATT

R1-162299	Design of sPUSCH for shortened TTI	CATT

R1-162376	DL design aspects of TTI shortening	Intel Corporation

R1-162377	UL design aspects of TTI shortening	Intel Corporation

R1-162403	PUCCH design for shortened TTI	ZTE

R1-162404	Downlink RS in shortened TTI	ZTE

R1-162405	Downlink control channels for shortened TTI	ZTE

R1-162406	Study on PUSCH transmission in shortened TTI	ZTE

R1-162504	Reference signal for sPDCCH demodulation	LG Electronics

R1-162505	Reference signal for sPDSCH demodulation	LG Electronics

R1-162506	Time/Frequency resource of sPDCCH and sPDSCH	LG Electronics

R1-162507	Discussion on PUCCH design for HARQ-ACK in shortened TTI	LG Electronics

R1-162508	Discussion on PUCCH design for CSI in shortened TTI	LG Electronics

R1-162509	Discussion on PUSCH design with TTI shortening	LG Electronics

R1-162529	Discussion on control signaling design for reduced TTI	HTC

R1-162531	Discussion on reference signal design for shortened TTI	Panasonic

R1-162535	Multiplexing between non-sTTI and sTTI UEs	Panasonic

R1-162542	Design of UL reference signal for sPUSCH	Mitsubishi Electric

R1-162545	Discussion on DL control channel design for short TTI	Spreadtrum Communications

R1-162546	HARQ feedback for sTTI scheduling	Spreadtrum Communications

R1-162558	Discussion on channel design for shortened TTI	Sony

R1-162588	DCI design for short TTI	Huawei, HiSilicon

R1-162589	Discussion on CSI feeback for short TTI	Huawei, HiSilicon

R1-162590	Consideration on multiplexing of non-sTTI and sTTI in the same carrier	Huawei, HiSilicon

R1-162706	Downlink physical channel design for TTI shortening	Samsung

R1-162707	Uplink physical channel design for TTI shortening	Samsung

R1-162738	Design of DL channels for shortened TTI	Lenovo (Beijing) Ltd

R1-162739	Design of UL channels for shortened TTI	Lenovo (Beijing) Ltd

R1-162788	Details of Shorter TTI operation for Latency Reduction	Nokia, Alcatel-Lucent Shanghai Bell

R1-162821	PUCCH design for sTTI	Nokia, Alcatel-Lucent Shanghai Bell

R1-162846	Shortened TTI for DL transmission	Sharp

R1-162850	Discussion on design of shortened TTI	China Telecommunications

R1-162944	Discussion on TTI shortening for TDD	Mediatek

R1-162945	Considertions on latency reduction for FDD	Mediatek

R1-162946	Control channel design for latency reduction	Mediatek

R1-162947	PUSCH Performance with short TTIs	Mediatek

R1-162963	Short-TTI PDCCH Design	InterDigital Communications

R1-162964	Short-TTI PDSCH Design	InterDigital Communications

R1-162965	Short TTI PUCCH Design	InterDigital Communications

R1-162966	Short TTI PUSCH Design	InterDigital Communications

R1-163068	DL channel design for shortened TTI	Qualcomm Incorporated

R1-163069	UL channel design for shortened TTI	Qualcomm Incorporated

R1-163099	Shortened DL TTI design	Motorola Mobility Germany GmbH

R1-163101	Support different TTI lengths for data channels and PUCCH	LG Electronics

R1-163171	DL channel designs for shortened TTI	NTT DOCOMO, INC.

R1-163172	PUCCH design for shortened TTI	NTT DOCOMO, INC.

R1-163173	PUSCH design for shortened TTI	NTT DOCOMO, INC.

R1-163210	On multiplexing of PDSCH with different TTIs and related enhancements	Intel Corporation

R1-163266	On multiplexing of sTTI with TTI in DL	Nokia, Alcatel-Lucent Shanghai Bell

R1-163267	On design of DL control channel for shorter TTI operation	Nokia, Alcatel-Lucent Shanghai Bell

R1-163268	Impact of shortened TTI on uplink transmission	ASUSTEK COMPUTER (SHANGHAI)

R1-163318	Physical layer aspects for PDSCH for short TTI	Ericsson

R1-163319	Physical layer aspects for PDCCH for short TTI	Ericsson

R1-163320	Physical layer aspects for PUSCH for short TTI	Ericsson

R1-163321	Physical layer aspects for PUCCH for short TTI	Ericsson

R1-163322	Downlink control signaling design for short TTI	Ericsson

R1-163323	Definition of DCI bit fields for short TTI	Ericsson

R1-163324	Short TTI operation with Carrier Aggregation	Ericsson

R1-163325	Evaluation and overview of enhanced frame structure 2	Ericsson

R1-163332	Discussion on PUSCH transmission in a short TTI	KT Corp.

#### Processing time reduction and related procedures

Discussion about processing time reduction for both DL and UL channels

Details on applicable procedures to support processing time reduction, e.g., HARQ operation

R1-162300	TDD system design for shortened TTI and related procedures for latency reduction	CATT

R1-162339	Discussion on TTI shortening	Guangdong OPPO Mobile Telecom.

R1-162407	Latency reduction for TDD	ZTE

R1-162408	Processing time reduction and related procedures	ZTE

R1-162446	Discussion on TTI shortening for DL/UL transmissions	NEC

R1-162510	Discussion on TTI shortening for TDD	LG Electronics

R1-162511	Physical layer aspect of processing time for shortened TTI	LG Electronics

R1-162534	Discussion on processing time reduction in physical layer	Panasonic

R1-162617	Processing time reduction for short TTI	Huawei, HiSilicon

R1-162708	Study on eNB and UE processing time for TTI shortening	Samsung

R1-162740	HARQ procedure for DL and UL data channels with shortened TTI	Lenovo (Beijing) Ltd

R1-162789	Processing time considerations for supporting latency reduction	Nokia, Alcatel-Lucent Shanghai Bell

R1-162790	On UL HARQ for Supporting Latency Reduction	Nokia, Alcatel-Lucent Shanghai Bell

R1-162866	Discussion on HARQ timing design for reduced TTI	CMCC

R1-163070	Processing time reduction and related procedures for DL	Qualcomm Incorporated

R1-163071	Processing time reduction and related procedures for UL	Qualcomm Incorporated

R1-163284	Discussion on HARQ Process for Slot-based TTI	ASUSTEK COMPUTER (SHANGHAI)

R1-163326	On processing time reduction and related procedures	Ericsson

#### Other

TDD

R1-163523	Way Forward on enhanced frame structure type 2 for latency reduction	Huawei, HiSilicon

Also supported by Panasonic

Possible agreements:

- Enhanced frame structure type 2 is recommended to be supported for latency reduction

  - Additional subframe type(s) with symbol(s) for downlink transmission, GP and symbol(s) for uplink transmission should be supported

    - Downlink transmission includes DL control and DL data

    - Uplink transmission includes UL control and UL data

      - FFS SRS

    - FFS GP position and GP duration

  - More than 2 Downlink-to-Uplink Switch-points in a radio frame should be supported

    - FFS the number of the new Downlink-to-Uplink Switch-points in a radio frame

    - FFS the position(s) of the new Downlink-to-Uplink Switch-points in a radio frame

  - Backward compatibility should be preserved

    - Additional subframe type(s) can be used to replace a subset of the following subframe(s)

      - Alt. 1:  MBSFN subframe(s) and UL subframe(s) configured to legacy UEs

      - Alt. 2:  UL subframe(s) according to the UL/DL configuration indicated by SIB1

      - Alt. 3:  MBSFN subframe(s) configured to legacy UEs

      - Alt. 4:  DL subframes and UL subframe(s) according to the UL/DL configuration indicated by SIB1

      - FFS which alternative to be selected.

    - Special subframe(s) according to the UL/DL configuration indicated by SIB1 can be used as additional subframe type(s) for new UEs

Continue offline discussions until Wednesday – Yongxia (Huawei)

R1-163748	WF on enhanced FS2 for latency reduction	Huawei

Revised in R1-163856

R1-163849	Way Forward on reducing latency for TDD & FDD CMCC, Spreadtrum, CATT, Ericsson, OPPO, Mediatek, Nokia, Alcatel-Lucent Shanghai Bell, Intel, CATR, NTT DOCOMO

R1-163174	Discussion about TDD Frame Structure for Latency Reduction	NTT DOCOMO, INC.

R1-162822	Considerations on shorter TTI for TDD duplex mode	Nokia, Alcatel-Lucent Shanghai Bell

R1-162116	Discussion on enhanced frame structure  for latency reduction in TDD	Huawei, HiSilicon

R1-162117	Performance Evaluation for enhanced TDD frame structure	Huawei, HiSilicon

R1-162709	TDD-specific issues on TTI shortening	Samsung

R1-163325	Evaluation and overview of enhanced frame structure 2	Ericsson

R1-162944	Discussion on TTI shortening for TDD	Mediatek

R1-162300	TDD system design for shortened TTI and related procedures for latency reduction	CATT

R1-162407	Latency reduction for TDD	ZTE

R1-162510	Discussion on TTI shortening for TDD	LG Electronics

Friday

R1-163851	TDD evaluation assumptions 	LG

The document was presented by Yunjung Yi from LGE.

- For SLS/analysis of latency reduction for TDD

  - Legacy TDD DL/UL configuration #0,#1, and #2 can be evaluated.

  - The sets of “fixed” DL and UL subframes are assumed

    - Subframe #0 and #5 are assumed as normal fixed downlink subframe

    - Subframe #2 are assumed as uplink subframe

  - Evaluation sets include at least the followings

    - Reference set: Legacy TDD DL/UL configuration with legacy TTI

    - Set 1: full flexibility on other subframes

      - All downlink subframes are MBSFN subframes and can be replaced with additional subframe type

      - All uplink subframes can be replaced with additional subframe type

      - Special subframe can be replaced with additional subframe type

    - Set 2: full flexibility only on UL subframes

      - All downlink subframes are fixed as downlink subframes

      - Special subframes are fixed as special subframes

      - All uplink subframes can be replaced with additional subframe types

    - Set 3: keep legacy TDD DL/UL configuration

      - All downlink subframes are fixed as downlink subframes

      - All uplink subframes are fixed as uplink subframes

      - Special subframes are fixed as special subframes

  - Simulation results is recommended to include at least

    - Performance comparison of Set 3 compared to reference set

    - Performance comparison of other sets compared to Set 3

    - Comparison among different TTI  lengths within the same set to evaluate the gain from TTI shortening.

  - Coexistence issue needs to be address in set 1 and set 2

Discussion: Ericsson not clear how the coexistence issue will be evaluated.

Decision: The document is noted.

Miscellaneous

R1-162530	Uplink latency reduction with prescheduling	HTC

R1-162581	SRS enhancements for latency reduction	Mitsubishi Electric Co.

### Other

R1-162125	Introduction of larger max TBS for TDD UL for eMTC in TS36.212	Huawei, HiSilicon, CATR, CATT, CMCC, Potevio, Spreadtrum, Xinwei

R1-162126	Introduction of larger max TBS for TDD UL for eMTC in TS36.213	Huawei, HiSilicon, CATR, CATT, CMCC, Potevio, Spreadtrum, Xinwei

R1-162136	Introduction of larger max TBS for TDD UL for eMTC in TS36.306	Huawei, HiSilicon, CATR, CATT, CMCC, Potevio, Spreadtrum, Xinwei

R1-162378	Work plan for FeCoMP study item	Intel Corporation

The document was presented by … from … and .

Discussion: .

Decision: The document is noted.

# NR

## Study on New Radio Access Technology

SID in RP-160671

R1-163103	Workplan for Study on Study on New Radio (NR) Access Technology	NTT DOCOMO, INC.

The document was presented by Kazuaki Takeda from NTT dOCOMO. For information and provides initial RAN1 work plan for the NR study. Work plan shall be further updated according to the progress of the study.

Decision: The document is noted for information.

Thursday: Rapporteur will submit an updated work plan involving all WGs in the next meeting.

R1-163104	3GPP & ITU-R WP 5D Coordination on "5G"	NTT DOCOMO, INC.

The document summarizes items related to 3GPP & ITU-R Working Party 5D (WP 5D) coordination on “5G”. There are two on-going activities in WP 5D which would need coordination with 3GPP:

- IMT-2020 candidate technology submission.

- IMT-2020 parameters required for sharing studies in preparation for WRC-19 agenda item 1.13.

Discussion: Mr Chair need for a close coordination with RAN4 to reply to ITU-R. Refer to RP-160598.

Decision: The document is noted.

Not treated.

R1-163376	Draft TR 38.802 Study on New Radio (NR) Access Technology - Physical Layer Aspects	NTT DOCOMO, INC.

### Overview of new radio interface

High level design proposals addressing all usage scenarios, requirements and deployment scenarios, e.g., eMBB, MTC and URLLC. Companies are encouraged to limit the number of papers to two per company and to three pages per contribution.

R1-162144	Flexible air interface for 5G	Huawei, HiSilicon

The document was presented by David Mazzarese from Huawei.

- Proposal 1: To ensure forward compatibility for the new radio, physical layer functionalities (such as signals, channels and signaling mechanisms) specific to a service/usage scenario should be confined within its flexibly-configured time/frequency partition. Sharing of some common physical layer functionalities among some services should be supported. UEs should be able to operate properly once a new service is added and occupies subcarriers (e.g. PRBs) within the UE’s system bandwidth. Therefore the new radio should not require the transmission of wideband signals and channels.

- Proposal 2:  The new radio interface should be designed to support being configurable to meet the diverse requirements. This includes flexibility to configure waveforms and multiple access schemes, OFDM numerologies and frame structure, transmission protocols and channel coding/modulation schemes.

- Proposal 3: The new radio interface should be designed to support the coexistence of different numerologies on a single continuous block of spectrum (with minimized or no guard band). Frequency localized OFDM-based waveform is one candidate technique to be studied.

Decision: The document is noted.

R1-162145	Overview of radio access mechanism for 5G	Huawei, HiSilicon

- Proposal 1: In order to provide a boundary-less radio access with UE cell-centre-like experience for the new radio, RAN1 should study the following in a dense urban deployment scenario:

  - Radio access design without rigid association with a physical cell. Physical layer design that is based, as much as possible, on an identifier that can identify a UE across coverage of a logical access entity.

  - Network-oriented measurement and discovery with uplink signals to assist in UE-TRP association. Tracking mechanisms based on uplink transmissions that can minimize mobility interruption.

- Proposal 2: In order to provide low latency low overhead radio access, RAN1 should study the design of physical channels, reference signals and sequences that enable:

  - A lighter or no random access procedure for UE fast re-entry to the system

  - Uplink transmission mechanisms without dynamic request and grant.

Decision: The document is noted.

R1-162171	General design principles for 5G new radio interface: System operations	Samsung

The document was presented by Younsun Kim from Samsung.

- Design principle 1: Provide specification support for both sub-6GHz and over-6GHz spectrum in 5G phase 1.

- Design principle 2: Support system bandwidth larger than 20MHz for both sub-6GHz and over-6GHz spectrums.

- Design principle 3: Support both FDD and TDD modes of operation.

- Design principle 4: Design 5G so that it can coexist with LTE when deployed on LTE spectrum.

- Design principle 5: Support forward compatibility resources on time and frequency domain resources which are not used for essential system operations.

- Design principle 6: Strive for a common structure for both sub-6GHz and over-6GHz considering different channel characteristics and target verticals.

- Design principle 7: Minimize transmissions which must occupy entire system bandwidth or always turned ON.

- Design principle 8: Strive for a common specification design to support similar set of features.

Discussion: LGE asked clarifying the figure on spectrum allocation – especially TDD allocation.

Not a Samsung’s proposal – reflects current understanding, shared with RAN4

Decision: The document is noted.

R1-162172	General design principles for 5G new radio interface: Key functionalities	Samsung

- Proposal 1: OFDMA is the baseline for 5G new radio DL/UL with additional consideration of SC-FDMA for UL.

- Proposal 2: Strive for minimal number of numerology sets while taking into account the different requirements on wide range of frequency spectrum, various use case, and complexities.

- Proposal 3: Consider adoption of a new channel coding scheme for 5G new radio and support LDPC as a candidate

- Proposal 4: FD-MIMO and its enhancements is the baseline for 5G new radio MIMO

- Proposal 5: Adopt joint time-varying analog and digitial beamforming (hybrid beamforming) for over-6GHz band system design.

- Proposal 6: Support reduced latency for TDD mode of operation of 5G new radio by reducing the DL-UL periodicity or by introducing dynamic TDD.

- Proposal 7: 5G new radio should support both standalone operation and non-standalone operation.

Decision: The document is noted.

R1-162192	Frequency scalable NR design from < 1GHz to mmW	Qualcomm Inc.

The document was presented by Tingfang Ji from Qualcomm.

- Proposal 1: NR should be optimized for licensed/shared/unlicensed spectrum access, e.g., self-contained waveform/transactions.

- Proposal 2: NR should support different carrier frequency and/or deployment scenarios with scalable numerologies, e.g., following 2N scaling.

- Proposal 3: mmW-specific design should be introduced to the air interface in addition to designs optimized for sub6 GHz access, e.g., support of both RF beamformed and non-beamformed broadcast/control channels.

- Proposal 4: Sub6 GHz and mmW specification should share common features that define the radio hardware envelope, e.g., channel coding, receiver MIMO processing, HARQ timeline, etc.

Discussion: Linear scaling was debated.

Decision: The document is noted.

R1-162222	Design principles of NR in RAN1	ZTE

The document was presented by Yifei Yuan from ZTE.

- Proposal 1: Abstract (separate) some common parts from Layer1to form L0 to support all kinds of different scenarios

- Proposal 2: List some important issues that may affect compatibility especially forward compatibility

- Proposal 3: Non-orthogonality access e.g. MUSA (multi users shared access).should be considered in NR especially for mMTC/IoT

Decision: The document is noted.

R1-162238	A Dynamic TDD Radio Interface 	National Instruments Corp.

The document was presented by Nikhil Kundargi from National Instruments and introduces guidelines for designing a new frame structure suitable for Massive MIMO. This frame structure has been prototyped in multiple real world test beds to validate its practicability in ITU 2020 deployment scenarios. We also presented a flexible dynamic RAT to accommodate extensions of the numerology to 100 MHz channels based on this frame structure.

Discussion: on the concept of long CP (overhead issue)

Introduction of variable CP for flexibility – complexity issue to be considered

Decision: The document is noted.

R1-162301	5G New Radio Interface Design	CATT

The document was presented by Fang Chen Cheng from CATT and lists the following proposals as the potential guideline of 5G interface design

- Wide range of system bandwidth

- Further enhancement of link adaptation

- Dynamic multi-antenna technologies

- Efficient support of large packets and small packets with extended coverage and power saving

- Minimized RS overhead

- Opportunistic radio access

- Variable length of TTI with self content control information

- Support central and distributed network control

- Universal frame structure for downlink, uplink and sidelink air interface design

- New waveform for asynchronous multi-subcarrier reception

- Efficient Broadcast and multicast

- Non-NR carrier embedded in the 5G carrier

Decision: The document is noted.

R1-162332	Overview of New Radio Interface	Fujitsu

The document was presented by Tim Moulsley from Fujitsu.

- Proposal 1: Identify features expected to be supported in Phase 1 and Phase 2 and study designs for Phase 1 that can be forward compatible with Phase 2 (and beyond).

- Proposal 2: Investigate efficient TDD frame structure options which can achieve efficient utilization of radio resources and low latency.

- Proposal 3: Supporting enhanced inter-cell operation control for UDN should be studied (including interference control and joint transmission).

- Proposal 4: The multiple connectivity can be studied in both inter-frequency and intra-frequency cases.

- Proposal 5: Dynamic TDD can be studied for the NR.

- Proposal 6: The combination of multiple connectivity (for FDD and TDD) and dynamic TDD can be studied in NR.

- Proposal 7: Aim for a unified uplink transmission framework which is beneficial for both mMTC and URLLC scenarios.

Decision: The document is noted.

R1-162379	Overview of new radio access technology requirements and designs	Intel Corporation

The document was presented by Ralf Bendlin from Intel and shows high level views on some of the important design aspects for a next generation new radio access technology including non-orthogonal multiple access, frame structure, waveform, low overhead system information acquisition, coexistence of different numerologies and services as well as channel coding.

Decision: The document is noted.

R1-162426	Consideration on the design of NR	NEC Corporation

The document was presented by … from NEC.

- Proposal 1: New RAT should be highly flexible and scalable to meet diverse requirements for three usage scenarios.

- Proposal 2: New RAT should support tight interworking; DC/CA can be performed among 5G new RATs, and DC can be performed between 5G new RAT and LTE.

- Proposal 3: FDD and TDD should be considered for the carrier frequency below 6GHz, and TDD can be considered for carrier frequency above 6GHz.

Decision: The document is noted.

R1-162512	Overview of new radio interface design	LG Electronics

The document was presented by … from LGE.

- Frequency-band specific design should be considered

  - Multiple set of numerologies to cover very wide range of frequencies should be supported

  - Band-specific transmission scheme should be considered.

- The new RAT system should be very flexible and dynamic

  - Always-on or periodic signaling/channel should be minimized

  - Flexible HARQ timing should be supported

  - Flexible duplex should be enabled

Decision: The document is noted.

R1-162513	Discussion on new radio design and requirements in consideration of various use cases	LGE

Decision: The document is noted.

R1-162547	Some Considerations on Enabling Radio Technologies for 5G NR	Spreadtrum Communications, CATR

The document was presented by Jinhui Chen from Spreadtrum.

Decision: The document is noted.

R1-162564	Consideration on multiple access for new usage scenarios	Sony

The document was presented by … from Sony.

- Observation 1: There may be shortages in the resources to transmit in new usage scenarios because these scenarios are characterized by a simultaneously massive number of device connections, low latency, and high reliability capabilities.

- Proposal 1: Non-Orthogonal Multiple Access (NOMA) technology should be considered to improve the spectral efficiency for New RAT.

- Proposal 2: Not only downlink NOMA but also Uplink NOMA should be a target candidate technology.

- Proposal 3: RAN1 should consider reducing the use of user specific information in each transmit functional block to be able to easily apply NOMA technologies.

Decision: The document is noted.

R1-162567	Considerations for eMBB in the new RAT	ETRI

The document was presented by Gosan Noh from ETRI.

- Tight interworking between the macro cell and the small cell should be considered.

- Mobility-related structures and functions should be considered in a high speed deployment scenario.

- The unified design of the new RAT frame structure should take both below and above 6GHz frequency bands into account.

Decision: The document is noted.

R1-162577	Flexible framework for New Radio	INTERDIGITAL COMMUNICATIONS

The document was presented by Paul Marinier from InterDigital.

- Framing and numerology

  - Support for multiple TTI’s and symbol durations

  - Low latency decoding

  - Low HARQ-ACK transmission delay

  - Control signaling split

- Multiplexing and scheduling

  - Orthogonal multi-carrier baseline

  - Advanced UE transmission waveform

  - Physical layer processing robust to short bursts of interference

  - UE-based scheduling within allocated pool of resources

- Deployment

  - Minimization of always-on signals

  - Blank resources

  - Flexible provision of reference signals and resources

  - Use of LTE signals in tightly integrated LTE-NR scenarios

Decision: The document is noted.

R1-162580	Overview of NR techniques	Mitsubishi Electric Co.

The document was presented by Fumihiro Hasegawa from Mitsubishi.

- Proposals related to Massive MIMO

  - User interference cancellation at transmitter: precoding/postcoding design

  - CSI reporting with higher resolution

  - Investigation on impact of realistic assumptions on channel reciprocity ( i.e., mismatches

  - in RF circuit characteristics between TX and RX)

  - RS design, including SRS, to reduce pilot contamination

Decision: The document is noted.

R1-162741	Discussion on new radio interface	Lenovo (Beijing) Ltd

The document was presented by Xinghua Song from Lenovo.

- Proposal 1: Consider common/symmetric DL/UL waveform design at least for above 6GHz.

- Proposal 2: Support multiple/scalable numerologies in NR to support frequency bands up to 100GHz.

- Proposal 3: Frame structure should flexibly support eMBB/mMTC/URLLC in the carrier.

- Proposal 4: NR should keep the always-on signals as minimum as possible to achieve higher efficiency and better forward compatibility.

- Proposal 5: Ultra-dense deployments should be emphasized in the NR design and TRP coordination should be studied including inter-TRP interference management, resource coordination, joint scheduling, mobility management, etc.

- Proposal 6: Study RAN functional split and interface in different deployments and carrier frequencies considering RAN virtualization and its impact on PHY.

- Proposal 7: Provide RAN1 feedback on the RAN functional split to RAN2/3.

Discussion: RAN1 chair is proposal 6 also be proposed to RAN2/3? Same approach

Decision: The document is noted.

R1-162791	Overview of next generation new radio access technology	MediaTek Inc.

The document was presented by … from MediaTek.

- Proposal 1: For eMBB including medium and high end MTC service, urban macro and dense urban scenarios with below ~40GHz spectrum and the wide system bandwidth are focused for optimal design.

- Proposal 2: For URLLC, small cell layer in dense urban scenario or indoor scenario with below ~40GHz spectrum is focused for optimal design.

- Proposal 3: For mMTC targeting the low end MTC service, rural scenario with below 1GHz spectrum (e.g., 700Mhz) is focused for optimal design.

- Proposal 4: Multiplexing of the different usage scenarios should be transparent for a device in system design.

- Proposal 5: The set of waveform numerologies should be restricted with the justified scenarios to reduce the UE implementation complexity.

- Proposal 6: The mechanism to support the flexible narrow band and wide band operation should be studied for UE energy efficiency.

- Proposal 7: The more advanced receiver should be considered to improve the spectrum efficiency and data throughput.

- Proposal 8: Reduction on PHY control and pilots can be considered to achieve the high performance.

- Proposal 9: A codec, achieving the high throughput with low power consumption, should be studied to meet 5G requirements with feasible implementation.

- Proposal 10: Support of flexible frame structure should be considered in the system design.

Decision: The document is noted.

R1-162867	Design principles for new radio	CMCC

The document was presented by Tong Hui from CMCC.

- Proposal 1: The deployment scenarios and requirements in RAN SI "Scenarios and Requirements for Next Generation Access Technologies" should be the basic principle for new radio interface design.

- Proposal 2: Forward compatibility should be supported for Phase 1 (R15) design of new radio.

- Proposal 3: A software defined air interface including multiple air interface profiles is required to meet diverse requirements of all three usage scenarios. The AI profile should be tailored for specific service requirements and network/UE capabilities.

Decision: The document is noted.

R1-162882	Requirements for the 5G New Radio physical layer	Nokia, Alcatel-Lucent Shanghai Bell

The document was presented by Karri Ranta-aho from Nokia.

- Both FDD and TDD should be supported.

- Multiple different OFDM numerologies should be supported for different deployment environments and frequency bands.

- Multiple different TTI lengths or should be supported for different latency and cell range targets.

- Support for flexible/dynamic TDD should be considered for configurations optimized for bands above 3 GHz.

- mMTC point of optimization should be around or below 1 GHz.

- Single carrier waveforms should be considered for configurations optimized for very high bands, e.g. > 40 GHz, as well as for mMTC.

- LBT mechanism should be supported for Licenced-Assisted Access.

Decision: The document is noted.

R1-162929	Overview of OTFS Waveform for Next Generation RAT	Cohere Technologies, AT&T, CMCC, Deutsche Telekom, Telefonica, Telstra

The document was presented by … from Cohere and requests that the Orthogonal Time Frequency & Space (OTFS) modulation and reference signal multiplexing scheme is thoroughly evaluated on its merits for the air interface of 5G systems and included in the Technical Report TR38.802 “TR for Study on New Radio Access Technology Physical Layer Aspects”.

Decision: The document is noted.

R1-163105	Overview of eMBB opration for NR access techonology	NTT DOCOMO, INC.

The document was presented by Fred Takeda from NTT DOCOMO.

- Proposal 1: Study numerology parameters for OFDM-based waveform for NR.

  - Scalable numerology parameters can be a starting point.

  - Feasibility and benefit of modified OFDM-based waveforms can also be studied.

- Proposal 2: Consider lean radio-frame with flexible dynamic TDD as a candidate radio frame design for NR.

  - Study design/procedure of dynamic subframe such that highly flexible TDD operation is enabled.

  - Study design/procedure of fixed DL subframe such that various scenarios can efficiently be supported.

- Proposal 3: Study native support for massive MIMO and/or virtual cell operations for NR.

  - Various massive MIMO operations/configurations should be investigated.

Decision: The document is noted.

R1-163129	Overviewon new air interface	CATR

The document was presented by … from CATR.

- Proposal 1: eMBB, mMTC and URLLC should have the  same priority  to be study and specified. Fundamental physical layer signal structure design should take all three usage scenario into account

- Proposal 2: 5G NR framework should be capable to provide diverse usages in one carrier without guard band.

- Proposal 3: NR supports set of numerologies to meet all deployment scenarios.

- Observation 1: The main expected spectrum for NR will be TDD and dynamic TDD design should be supported at the very beginning.

Decision: The document is noted.

R1-163157	Requirements for New Radio Interface 	IITH

The document was presented by Kiran Kuchi from IITH.

Decision: The document is noted.

R1-163190	New RAT Using imparity CCFD for mMTC	III

The document was presented by … from III and proposes the CCFD architecture in new RAT.

- Using the imparity CCFD with TDD, allows UL and DL be allocated in the same time and in the same bandwidth. Moreover, decide resource schedule according with the position or direction of UE can reduce the interference between UE and UE to achieve the same service coverage as non-CCFD TDD, so that we can get better performance.

Decision: The document is noted.

R1-163215	Overview of NR	Ericsson

The document was presented by Stefan Parkvall from Ericsson.

Decision: The document is noted.

R1-163280	On new radio interface design targets and principles	China Unicom

The document was presented by … from China Unicom and .

- Proposal 1: for design the unified air interface, reducing CAPEX and OPEX should be considered; the enabled technologies should be identified for different usage scenarios and coexistence and coordination between these technologies should be further studied.

- Proposal 2: For E2E network slicing, the key issues should be considered including how to mapping the core network slicing to radio interface slicing and how to calculate the actual bearing physical resources.

Decision: The document is noted.

R1-163292	Overviews of physical layer signal structure for New RAT	Sharp

The document was presented by … from Sharp.

Observation 1:

- For downlink, OFDM-based waveform is the baseline for NR.

  - Suitable for flexible UE multiplexing in spatial and/or power domains, such as MU-MIMO and MUST.

- For uplink, both single carrier signal and OFDM-based waveform can be considered for flexible UE multiplexing.

Observation 2: Full flexible UL/DL subframe allocation is one of the key requirements for NR to support.

  - The ratio of UL/DL traffic becomes more flexible based on:

    - Location, time, etc

Observation 3: UE complexity for supporting multiple 3GPP-RATs should be taken into account when coding scheme is studied

  - mMTC and/or URLLC specific terminal may support only NR. In this case, mMTC and/or URLLC UE may not need to support turbo decoder.

Decision: The document is noted.

R1-163293	Unlicensed bands support in New RAT	Sharp

Observation 1: The increasing number of supported band is serious issue in terms of NR global terminal since frequency range will be dramatically increased in NR.

Observation 2:

- Unlicensed spectrum becomes more important and can be used as an “international band”

- Native support of unlicensed spectrum is one of the key requirements for NR.

  - FFS: standalone on unlicensed spectrum

Decision: The document is noted.

R1-163615	Way Forward on Ovverview of NR	NTT DOCOMO, Alcatel-Lucent Shanghai Bell, Huawei, HiSilicon, Nokia, Samsung

The document was presented by Kazuaki Takeda from NTT DOCOMO.

- Largest component carrier bandwidth not smaller than 80 MHz

- Waveform is based on OFDM

  - Multiple numerologies are considered

  - A form of DFT-S-OFDM and/or OFDM with filtering/windowing is further considered

  - FFS for case where PA efficiency is an issue

- Frame structure(s) supporting

  - FDD and TDD duplex arrangement

  - Downlink, uplink, and sidelink transmission

  - Access and backhaul

  - Operation in licensed and at least licensed-assisted operation in unlicensed band

- Study flexible/dynamic TDD, including both downlink and uplink transmissions in the same subframe interval

- Study enhanced massive MIMO analog/digital/hybrid beam-forming

- Study multiple access mechanisms including UL-grant less transmission, contention-based transmission, non-orthogonal multiple access

Decision: The document is noted, modified as follows:

Agreements

- Largest component carrier bandwidth not smaller than 80 MHz for at least one numerology is supported

- Waveform is based on OFDM

  - Multiple numerologies are supported

  - Additional functionality on top of OFDM such as DFT-S-OFDM, and/or variants of DFT-S-OFDM, and/or filtering/windowing, and/or OTFS is further considered

    - Complementary non-OFDM based waveform is not precluded for some specific usecases (e.g., mMTC use case)

- Study frame structure(s) supporting at least

  - FDD duplex arrangement

  - TDD duplex arrangement

  - Downlink transmission

  - Uplink transmission

  - Sidelink transmission

  - Access link

  - Backhaul/relay link

  - Stand alone operation in licensed band

  - Non stand alone operation in licensed band

  - Licensed-assisted operation in unlicensed band

- Study flexible/dynamic TDD, including both downlink and uplink transmissions in the same subframe interval

- Study enhanced massive MIMO analog/digital/hybrid beam-forming

- Study multiple access mechanisms including UL-grant less transmission, contention-based transmission, non-orthogonal multiple access

- Study flexible duplex

Not treated.

R1-162302	Consideratoin of mMTC in 5G system design	CATT

R1-162380	Overview of antenna technology for new radio interface	Intel Corporation

R1-162578	Reference signal principles for New Radio	INTERDIGITAL COMMUNICATIONS

R1-163394	Basic principles for the 5G New Radio access technology	Nokia, Alcatel-Lucent Shanghai Bell	(R1-162883)

R1-163106	Overview of mMTC and URLLC for NR access technology	NTT DOCOMO, INC.

R1-163216	Some design principles for NR	Ericsson

### Forward compatibility for new radio interface

Solutions how to achieve forward compatibility with Phase II specification and beyond.

R1-162193	Forward compatible NR design	Qualcomm Inc.

The document was presented by Tingfang Ji from Qualcomm.

- Proposal 1: Waveforms for initial access and paging should have limited time/frequency footprint.

- Proposal 2: Self-contained data and mobility transactions are highly desirable, such that UE activity during a transaction is limited to confined time/frequency footprint.

- Proposal 3: Persistent signals such as CRS should be configurable based on different use cases.

- Proposal 4: Identify the characteristics for each feature/services for forward compatibility analysis

- Proposal 5: Forward compatibility signalling of blank time/frequency resources should be defined for the introduction of unknown features

Decision: The document is noted.

R1-163392	Consideration on forward compatibility for new radio interface	ZTE	(R1-162223)

The document was presented by Yifei Yuan from ZTE.

- Regarding frame structure, self contained frame structure, flexible data subframes and flexible numerology should be considered.

- Regarding user centric design, the design principle should be to minimize mandatory signals in the network for lower interference and better network efficiency.  Reference signals should be designed in on-demand and UE specific manner.  Broadcast channel for system information should include only the common and essential system information.

- Regarding MIMO framework, CSI and transmission scheme should be decoupled.  We should strive for universal MIMO framework for different scenarios and use cases.  Multi-level CSI feedback is considered to flexibly adapt to different CSI requirements.  In additional to data channel, design of MIMO framework should also consider support of beam based control channel, broadcast channel and synchronization signal.

Decision: The document is noted.

R1-162514	Consideration on forward compatible new radio interface design	LG Electronics

The document was presented by Eunsun Kim from LGE.

- Always-on or periodic signalling/channel should be minimized

- Flexible and dynamic DL/UL configuration should be supported

- Flexible HARQ timing should be supported

- New waveform of NR should provide multiplexing of different numerologies/usecases

Decision: The document is noted.

R1-162553	Forward compatibility aspect of NR	Panasonic Corporation

The document was presented by Hidetoshi Suzuki from Panasonic.

- Proposal 1: Different usage scenarios are realized by sub-RATs, where certain amount of independency is achieved among sub-RATs.

- Proposal 2: The system is designed as to allow different sub-RATs to use different time/frequency resource. Overlapping among sub-RATs is subject to case-by-case discussion.

- Proposal 3: The system is designed to allow the introduction of new waveform in different time/frequency resource usage.

- Proposal 4: The system is designed to reduce the inter-sub-RAT interference (emission).

- Proposal 5: The need of sub-RAT specific synchronization signals are subject to case-by-case discussion.

- Proposal 6: The use of CRS or DMRS depends on use cases and could be different among sub-RATs.

- Proposal 7: PBCH/MIB like very initial common channels should be minimized but cell detection latency should also be taken into account.

- Proposal 8: The need of sub-RAT specific PBCH/MIB like very initial common channels is subject to case- by-case discussion.

- Proposal 9: On demand SIBs transmission or constant periodic SIBs transmission depends on the frequency of incoming UEs. Depending on the use cases, both may be required and it is sub-RAT specific.

- Proposal 10: Cell common L1 control signalling needs careful design for forward compatibility.

- Proposal 11: Signal dynamic range should be taken into account for the possibility of future compatibility among multiple sub-RATs.

- Proposal 12: Signal dynamic range should be taken into account for the flexibility on DL, UL and sidelink duplex.

- Proposal 13: Signal dynamic range should be taken into account for sharing frequencies among operators.

Decision: The document is noted.

R1-163294	Forward compatibility for New RAT	Sharp

The document was presented by John Kowalski from Sharp.

- Observation from Co-cell/site/carrier operation perspective:

  - Phase II NR should co-exist with Phase I NR on the same carrier component.

    - FFS: FDM, TDM or any other multiplexing scheme within a carrier component

    - FFS: how dynamically reallocate Phase I NR resources and Phase II NR resources

- Observation from eNB perspective:

  - Phase II NR eNB should support the connectivity to Phase I NR UE.

    - FFS: Phase II eNB support Phase I UE and Phase II UE at the same time and frequency

  - Phase I eNB should support the connectivity to Phase II UE.

    - FFS between following two alternatives

      - Alt.1: Phase II UE is mandated Phase I functionality.

      - Alt.2: Phase II NR has the backward compatibility with Phase I NR.

        - E.g. common signaling and/or air-interface

- Observation from UE perspective:

  - At least Phase I NR UE should be allowed to connect to Phase II NR eNB.

  - Phase II NR UE does not need to support all Phase I functions.

    - FFS:how to divide the feature sets for forward compatibility.

- Observation from Specification perspective:

  - From the specification perspective, UE specific physical layer parameters may provide flexible extensions of physical layer structures in Phase II even for cell specific use cases.

Decision: The document is noted.

R1-163555	WF on Forward Compatibility	Huawei, HiSilicon

The document was presented by David Mazzarese from Huawei.

Observations for forward compatibility and for compatibility of different features within each release:

- Support of Coexistence over a single continuous block of spectrum

  - NR and other 3GPP RAT (e.g. LTE, NB-IoT)

  - different NR numerologies

  - different services

- Support of multiple services (in any phase/release)

  - services using future designs (e.g. waveform, frame structure, multiple access, etc)

  - services using the same OFDM-based waveform with different numerology

- Support of accommodating new technical features

  - seamlessly integrate new device categories

  - enable UEs that do not support the full system BW (e.g. low-cost MTC)

Proposal: Phase 1 and later phases of NR should be designed with the following principles to ensure forward compatibility and compatibility of different features:

- Support of blank resources in time/frequency

- waveform with lower emissions outside the useful subbands compared to CP-OFDM without windowing and filtering

- Strive for

  - Minimizing the amount of signals/resources that needs to be transmitted irrespective of incoming traffic

  - Maximizing the amount of resources that can be flexibly utilized without causing backward compatibility issues in the future

  - Minimizing transmission of (always-on) wideband signals and channels

- Flexible configuration of signals, channels and resources

  - Strive to avoid short periodicity of always-on periodic signals and channels

  - Rely more on dynamically configured quantities

- Physical layer functionalities (signals, channels, signaling) should be confined within its flexibly-configured time/frequency resource (for control, data, reference signals)

- Rely more on UE-specific quantities

Decision: The document is noted.

Agreements:

- Phase 1 and later phases of NR should be designed with the following principles to ensure forward compatibility and compatibility of different features:

  - Strive for

    - Maximizing the amount of time and freq. resources that can be flexibly utilized or that can be left blanked without causing backward compatibility issues in the future

      - Blank resources can be used for future use

    - Minimizing transmission of always-on signals

    - Confining signals and channels for physical layer functionalities (signals, channels, signaling) within a configurable/allocable time/freq. resource

Possible observations

- For forward compatibility and for compatibility of different features within each release, the following observations are made:

  - Support of Coexistence over a single continuous block of spectrum

    - NR and other 3GPP RAT (e.g. LTE, NB-IoT)

    - different NR numerologies

    - different services using the same OFDM-based waveform with different numerology

  - Support of multiple services (in any phase/release)

    - services using future designs (e.g. waveform, frame structure, multiple access, etc)

  - Support of accommodating new technical features

    - seamlessly integrate new device categories

    - enable UEs that do not support the full system BW (e.g. low-cost MTC)

R1-163823	WF on Observations on Forward Compatibility	Huawei, HiSilicon

- For forward compatibility and for compatibility of different features within each release, the following motivations were provided in contributions:

  - Support of coexistence over a single continuous block of spectrum

    - NR and other 3GPP RAT (e.g. LTE, NB-IoT)

    - different (including future) NR numerologies of the same waveform

    - different waveforms, each well localized in frequency domain

  - Support introduction of new services and features (in any release)

    - seamlessly integrate new device categories

    - enable UEs that do not support the full system BW (e.g. low-cost MTC)

Decision: The document is noted.

Not treated.

R1-162146	Consideration on forward compatibility	Huawei, HiSilicon

R1-162173	Discussion on forward compatibility for 5G new radio interface	Samsung

R1-162381	Forward compatibility for new radio interface	Intel Corporation

R1-162884	On the forward compatibility of the 5G New Radio	Nokia, Alcatel-Lucent Shanghai Bell

R1-163107	Forward compatibility for NR access technology	NTT DOCOMO, INC.

R1-163217	On forward compatibility	Ericsson

R1-162565	Consideration on numerologies in LTE/LTE-A and NR	Sony

R1-162566	Potential options of numerology setting for support of forward compatibility	Sony

R1-162868	Discussions on forward compatibility for new radio	CMCC

R1-163218	On positioning requirements for NR	Ericsson

R1-163219	On sidelink support in NR	Ericsson

R1-163220	Access-integrated backhaul	Ericsson

R1-163281	Considerations on forward compatibility for new radio interface	China Unicom

### Evaluation assumptions for new radio interface

Discussions on models, scenarios, parameters, and methodology that can be used for evaluating technology proposals.

R1-163659	Evaluations assumptions for NR	NTT DOCOMO

The document was presented by Takeda Kazuaki from NTT DOCOMO.

Discussion: Very first attempt (drawn by Tuesday’s session) – aiming for review and try to agree some assumptions within the next couple of days (common consensus needed).

Chair: any companies can propose their views to the rapporteur.

Companies wondered whether the content can be very useful (if all can suggest whatever they like and nobody is forced performing evaluations and providing feedback)

Concern with the complexity of evalutions that assumptions may triggered

Decision: The document is noted. Continue offline discussion until Thursday – (NTT DOCOMO)

Tuesday conclusion:

Continue offline discussion about waveform and multiple access assumptions – Wuyong (Huawei), (also LGE, Qualcomm, ZTE, DOCOMO (KPI, metrics))

Continue offline discussion about channel coding and modulation related assumptions – Keeth (Nokia) (also LGE)

Continue offline discussion about antenna modeling assumptions (R1-163547) until Wednesday- (Intel)

Wednesday

R1-163557	Way forward on assumptions for multiple access evaluation	Huawei, HiSilicon

Decision: The document is noted.

R1-163556	Way forward on assumptions for waveform evaluation	Huawei, HiSilicon

Decision: The document is noted.

Possible agreements:

- Link level simulation is used for waveform evaluation.

  - Whether and how to do system level simulation for waveform is FFS.

- Four evaluation cases should be used in link level simulation, which are

  - Case 1: DL single numerology case

  - Case 2: DL mixed numerology case

  - Case 3: UL single numerology case (asynchronous transmission)

  - Case 4: UL mixed numerology case (synchronous transmission)

(refer to their illustrations from slides 6 – 8 in R1-163556)

R1-163713	Way Forward on requirements for the evaluation of channel coding scheme	LG Electronics, Ericsson, CATT, AT&T, Fujitsu

Decision: The document is noted.

R1-163700	WF on Channel coding for 5G NR	Nokia, Alcatel-Lucent Shanghai Bell

The document was presented by Keeth Jayasinghe from Nokia.

Decision: The document is noted. Further revised in R1-163757.

R1-163757	WF on Channel Coding Evaluation for 5G New Radio	Nokia, Alcatel-Lucent Shanghai Bell, Samsung, Huawei, Sony, ZTE, Intel, Qualcomm, InterDigital, NTT DOCOMO

Discussion: Ericsson  preference to consider 256QAM modulation.

Decision: The document is noted, modified and agreed as follows:

Agreements: Coding Candidates

- Identified channel coding schemes for each usage scenario

| eMBB | mMTC | URLLC |
| --- | --- | --- |
|  | Convolutional codes | Convolutional codes |
| LDPC | LDPC | LDPC |
| Polar | Polar | Polar |
| Turbo | Turbo | Turbo |

- Common simulation assumptions are required to evaluate theoretical performance of proposed coding schemes

- Selection of the coding scheme should also consider various other aspects

Initial Simulation Assumptions

- Focus mainly on the BLER performance of candidate coding schemes.

- Evaluate performance of coding schemes with similar code rates and block sizes.

- Exact code constructions should be provided.

  - Example: Parity check matrices, polar code construction, ..

- Encoding/decoding complexity of the adopted algorithms should be described.

Agreements: Simulation assumptions : eMBB

- Evaluate the block error rate (BLER) performance versus SNR

| Channel* | AWGN |  |  |
| --- | --- | --- | --- |
| Modulation | QPSK, 64 QAM |  |  |
| Coding Scheme | Turbo | LDPC | Polar |
| Code rate | 1/5, 1/3, 2/5, 1/2, 2/3, 3/4, 5/6, 8/9 |  |  |
| Decoding algorithm** | Max-log-MAP | min-sum | List-X |
| Info. block length*** (bits w/o CRC) | 100, 400, 1000, 2000, 4000, 6000, 8000 Optional(12K, 16K, 32K, 64K) |  |  |
| * Fading channels will be simulated in the next stage ** These algorithms are starting points for further study. Other variants of agreed algorithms can be used for encoding and decoding (Complexity details should be illustrated) *** At least these info. block length and code rate shall be evaluated. Other info. block lengths and code rates are not precluded. Similar info. and encoded block lengths should be used for the evaluation. Total coded bits = info. Block length/code rate. Note: these info. block length and code rate are only for initial performance evaluations. They are not interpreted as design targets or assumptions for complexity analysis. |  |  |  |

- General guidelines

  - Existing code constructions can be used for evaluation

  - Whenever feasible, performance comparison should adopt coding constructions with matching computational complexities

Simulation assumptions : URLLC and mMTC

- Evaluate BLER performance versus SNR

| Channel* | AWGN |  |  |  |
| --- | --- | --- | --- | --- |
| Modulation | QPSK, 16 QAM |  |  |  |
| Coding Scheme | Convolutional codes | LDPC | Polar | Turbo |
| Code rate | 1/12, 1/6, 1/3 |  |  |  |
| Decoding algorithm** | List-X Viterbi | min-sum | List-Y | Max-log-MAP |
| Info. block length*** (bits w/o CRC) | 20, 40, 200, 600, 1000 |  |  |  |
| * Fading channels will be simulated in the next stage ** These algorithms are starting points for further study. Other variants of agreed algorithms can be used for encoding and decoding (Complexity details should be illustrated) *** At least these info. block length and code rate shall be evaluated. Other info. block lengths and code rates are not precluded. Similar info and encoded block lengths should be used for the evaluation. Total coded bits = info. Block length/code rate. Note: these info. block length and code rate are only for initial performance evaluations. They are not interpreted as design targets or assumptions for complexity analysis. |  |  |  |  |

- General guidelines

  - Existing code constructions can be used for evaluation

  - Whenever feasible, performance comparison should adopt coding constructions with matching computational complexities

  - BLER simulations down to 10-4 is recommended (to observe the error floor) for URLLC

R1-163547	WF on antenna modelling and related parameters for NR evaluations	Intel, Samsung

Decision: The document is noted. Continue offline discussion about antenna modeling assumptions (revision in R1-163706) until Thursday- (Intel)

Thursday

R1-163706	WF on antenna modelling and related parameters for NR evaluations	Intel, Samsung

Discussion: Telecom Italia Concern that realistic antennas configurations be assumed for low bands (700 MHZ, 4GHZ) – repeated comment already formulated during the last plenary

Decision: The document is noted. Continue offline discussion about antenna modeling assumptions (revision in R1-163863) - (Intel)

R1-163863	WF on antenna modelling and related parameters for NR evaluations	Intel, Samsung, CATT

Decision: The document is noted. Agreed in R1-163885, assuming the following updates

- In page 5,

  - 6-sector TRP antenna model is not precluded

  - TRP antenna model for high speed train is not precluded

- In page 4,

  - For number of TRP antenna elements, over-6GHz (30GHz, 70GHz)

    - 30GHz: Up to 256 Tx /Rx antenna elements

    - 70GHz: Up to 256 Tx /Rx antenna elements

    - Note: RAN1 continues to discuss exact number of Tx/Rx antenna elements

  - For number of UE antenna elements, over-6GHz (30GHz, 70GHz)

    - 30GHz: Up to 32 Tx /Rx antenna elements

    - 70GHz: Up to 32 Tx /Rx antenna elements

    - Note: RAN1 continues to discuss exact number of Tx/Rx antenna elements

R1-163861	Summary of evaluation assumptions for NR	NTT DOCOMO

The document was presented by Kazuaki Takeda from NTT DOCOMO.

Discussion: Telecom Italia same comment “realistic antennas configurations be assumed for 700 MHZ, 4GHZ” applies.

Fujitsu: various companies interested in evaluating the case of a carrier frequency around 4GHz for the micro layer in the dense urban scenario - should be captured.

Decision: The document is noted.

RAN1 agrees all proposals of R1-163861 including the following changes for carrier frequency for dense urban

Proposal: Macro layer: Around 4 GHz

Proposal: Micro layer: Around 30GHz, 4 GHz

Note that RAN1 will continue simulation assumptions

Note that antenna modeling and parameters will be updated

Revised and agreed in R1-163884 with the following changes:

- For carrier frequency for dense urban,

  - Proposal: Macro layer: Around 4 GHz

  - Proposal: Micro layer: Around 30GHz, 4 GHz

Note that RAN1 will continue simulation assumptions

Note that antenna modeling and parameters will be updated

- Delete BS antenna tilting value line

- Delete brackets BS antenna element gain + connector loss for below 6 GHz

- For traffic model, “Consider full buffer and FTP model 1/2/3 with packet size 0.5 Mbytes (other value is not precluded). Other traffic models are not precluded.”

- For traffic load (Resource utilization), “50% (other value is not precluded)”

- 9dB UE noise figure below 6 GHz

- Delete [TBD for TDD] in Tx power

Agreements:

- Each company can decide BS antenna tilting value and each company is requested to provide the used BS antenna tilting value (if applied) in their contribution

- Each company can decide aggregated system BW, until aggregated system BW will be decided, and each company is requested to provide the aggregated system BW in their contribution

R1-163560	Way forward on assumptions for multiple access evaluation	Huawei, HiSilicon

Decision: The document is noted.

Agreements:

- Link-level simulation (LLS) and system-level simulation (SLS) are used for multiple access evaluation.

  - LLS* is used for feasibility investigation of new MA proposals, comparison of different proposals in typical scenarios

  - SLS is used for comparison of proposals, and verification with traffic/scheduling/multi-cell interference dynamics

* LLS includes LLS with optional analytical model.

Agreement: Slides 4, 5, 6, 7 in R1-163560 are agreed.

Conclusion:

- Preliminary LLS evaluation results are encouraged to be provided for RAN1#85

R1-163881	Evaluation parameters for multiple access		Huawei, HiSilicon

Decision: The document is noted.

Agreement: Evaluation parameters – LLS for UL

| Parameters | Values or assumptions |
| --- | --- |
| Carrier Frequency | 2 GHz |
| Waveform | OFDM /SC-FDMA Other waveform is not precluded |
| Numerology | Same as Release 13 |
| System Bandwidth | 10 MHz |
| Target spectral efficiency | Proponents report per UE spectral efficiency and the number of UEs multiplexed if multi-UEs LLS is assumed |
| BS antenna configuration | 2/4 Rx as baseline 8Rx optional |
| UE antenna configuration | 1Tx |
| Transmission mode | TM1 (refer to TS36.213) |
| SNR distribution of Multiple UEs | Proponents report if single-user or multi-user LLS is used, and what SNR distribution is assumed. |
| Propagation channel & UE velocity | TDL for in TR38.900 as mandatory EPA, EVA, ETU as optional 3km/h, 30km/h, 120km/h |
| Max number of HARQ transmission | 1, 4 |

NOTE: Non-ideal effects (e.g., channel estimation, frequency offset) evaluation FFS.

Agreement: Evaluation parameters – LLS for DL

| Parameters | Values or assumptions |
| --- | --- |
| Carrier Frequency | 2 GHz |
| Waveform | OFDM Other waveform is not precluded |
| Numerology | Same as Release 13 |
| System Bandwidth | 10 MHz |
| Target spectral efficiency | Proponents report per UE spectral efficiency and the number of UEs multiplexed if multi-UEs LLS is assumed |
| BS antenna configuration | 2/4 Tx as baseline 8Tx optional |
| UE antenna configuration | 2 Rx |
| Transmission mode | TM2 as starting point (refer to TS36.213) |
| SNR distribution of Multiple UEs | Fixed gap {0, 5, 10, 15, 20} dB between UEs |
| Power allocation between UEs | Dynamic |
| Propagation channel & UE velocity | CDL in TR38.900 as mandatory EPA, EVA, ETU as optional 3km/h, 30km/h, 120km/h |
| Max number of HARQ transmission | 1, 4 |

NOTE: Non-ideal effects (e.g., channel estimation, frequency offset) evaluation FFS.

Email approval about LLS assumptions for multiple access until April 28th – (Huawei)

Email approval about SLS assumptions for multiple access until May 9th – (Huawei)

R1-163923	WF on the parameters for UE antenna pattern model 1	Intel, Samsung, QC, NTT DOCOMO, Sharp

Decision: The document is noted.

Friday:

Email approval about all bracket and essential FFS parts in R1-163863, transmission power assumptions above 6 GHz, UE noise figure above 6 GHz, necessity to revise the number of TRP antenna elements, over-6GHz (70GHz) to “Up to 1024 Tx /Rx antenna elements”, necessity to revise the number of UE antenna elements, over-6GHz (70GHz) to “Up to 64 Tx /Rx antenna elements”, and UE antenna pattern (R1-163923)  until 28th April - (Intel)

R1-163867	Updated summary of evaluation assumptions for NR	CMCC

Decision: The document is noted. High speed train assumptions in R1-163887 are agreed.

Continue discussions until RAN1 #85 meeting about highway and urban grid scenarios.

R1-163558	Way forward on assumptions for waveform evaluation	Huawei, HiSilicon

The document was presented by Long Yi from Huawei.

Decision: The document is noted and the following is agreed:

Agreements:

- Link level simulation is used for waveform evaluation.

  - Whether and how to do system level simulation for waveform is FFS.

- Four evaluation cases can be used in link level simulation depending on evaluation purposes of each usage scenario, which are

  - Case 1a, 1b: single numerology case

    - 1a: Downlink

    - 1b: Uplink, only one UE with narrow bandwidth is located at the edge of wide frequency band. It is assumed that no wide-band filter upon the whole frequency band.

  - Case 2: DL mixed numerology case

  - Case 3: UL single numerology case (asynchronous reception between UEs)

  - Case 4: UL mixed numerology case (synchronous reception between UEs)

(refer to their illustrations in pages 5 – 9 in R1-163558)

R1-163889	Way forward on assumptions for waveform evaluation	Huawei, HiSilicon, InterDigital,

Decision: The document is noted.

Agreements:

- Consider the RF nonlinearity in the evaluation cases of R1-163558

- RAN1 can consider the following models for PA modeling, i.e. Rapp model [1] (AM/AM, AM/PM) and/or Clipping model with different thresholds

  - Companies should provide the model parameters (operating point, back-off value etc.) and justification (e.g., EVM, OOBE/PSD)

- Huawei to draft a LS to RAN4 until Friday in R1-163890 to ask on the applicability/fidelity of the models above for both UE and BS, different carrier frequency and signal bandwidth, and recommended parameters (operating point, back-off value etc.) to be used in the models or recommended realistic other PA models.

[1]“Comparison of Power Amplifier Non-linearity Impact on 60 GHz Single Carrier and OFDM Systems”, Maltsev at al.,  IEEE CCNC 2010.

R1-163897	Way forward on evaluation assumptions for 5G waveform	Huawei, HiSilicon, InterDigital

Decision: The document is agreed.

R1-163890	Draft LS on realistic power amplifier model for 5G waveform evaluation	Huawei

Decision: The document is noted. Continue offline (revise in R1-163914)

R1-163914	LS on realistic power amplifier model for 5G waveform evaluation	Huawei

Decision: The document is noted. Replace 5G by NR.

R1-163926 	Draft LS on realistic power amplifier model for NR waveform evaluation	Huawei

Decision: The document is noted and final LS is agreed in R1-163934

R1-163924	Way forward on evaluation assumptions for 5G waveform	Huawei, HiSilicon, InterDigital

Decision: The document is noted.

R1-163933 	Way forward on evaluation assumptions for 5G waveform	Huawei, HiSilicon, InterDigital

Decision: The document is noted.

R1-163935	Way forward on evaluation assumptions for 5G waveform	Huawei, HiSilicon, InterDigital

Discussion: ZTE UE moving direction not included

Decision: The document is noted and agreed, except for MIMO mode and channel model.

Email approval about MIMO mode and channel model parameters until April 21st  – (Huawei)

Not treated.

R1-162147	General consideration on evaluation work for 5G new radio study	Huawei, HiSilicon

R1-162148	Evaluation methodology discussion for 5G waveform	Huawei, HiSilicon

R1-162149	Considerations on evaluation of new channel coding schemes	Huawei, HiSilicon

R1-162154	Evaluation methodology for 5G non-orthogonal mulltiple access	Huawei, HiSilicon

R1-162175	Evaluation assumptions for 5G new radio interface: details and KPIs for SLS	Samsung

R1-162176	Evaluation assumptions for 5G new radio interface: LLS aspects	Samsung

R1-162194	eMBB KPI evaluation assumptions	Qualcomm Inc.

R1-162195	mMTC KPI evaluation assumptions	Qualcomm Inc.

R1-162196	URLLC KPI evaluation assumptions	Qualcomm Inc.

R1-162197	Other KPI evaluation assumptions	Qualcomm Inc.

R1-162224	Evaluation assumptions for NR in RAN1	ZTE

R1-162303	Evaluation scenarios and configuration parameters for new radio interface	CATT

R1-162304	Evaluation methodology and models for evaluating technology proposals	CATT

R1-162382	General assumptions for evaluation of the new radio interface technology	Intel Corporation

R1-162383	Discussion on antenna modelling for the new radio interface technology evaluations	Intel Corporation

R1-162427	Discussion on evaluation issues for new RAT	NEC Corporation

R1-162515	Discussion on the evaluation assumptions for NR	LG Electronics

R1-162792	Discussion on evaluation assumptions for new radio interface	MediaTek Inc.

R1-162869	Evaluation method for new radio	CMCC

R1-162885	On the phase noise model for 5G New Radio evaluation	Nokia, Alcatel-Lucent Shanghai Bell

R1-162886	On the power amplifier model for the 5G New Radio evaluation	Nokia, Alcatel-Lucent Shanghai Bell

R1-162887	On the network energy efficiency requirement and evaluation for the 5G New Radio	Nokia, Alcatel-Lucent Shanghai Bell

R1-162888	On the link and system evaluation assumptions for the 5G New Radio	Nokia, Alcatel-Lucent Shanghai Bell

R1-162948	Considerations on numerology evaluation	Huawei, HiSilicon

R1-163108	Deployment scenarios and evaluation methodology for NR	NTT DOCOMO, INC.

R1-163109	Evaluation methodologies for waveform study	NTT DOCOMO, INC.

R1-163221	Scenarios and Evaluation Methodology for NR	Ericsson

R1-163562	Evaluation assumptions for 5G new radio interface: Scenarios for SLS	Samsung

### Waveform and multiple access schemes for new radio interface

Not treated.

R1-162150	Room for possible improvement of LTE spectral efficiency in new radio of 5G	Huawei, HiSilicon

R1-163125	Considerations on multiple access sheme for new radio interface	CATR

R1-163310	Need for study on In-Band Full Duplex communication	CEWiT

#### Waveform

R1-162248	Waveform for the Next Generation Radio Interface	Idaho National Laboratory

The document was presented by Ramon Khalona from INL and .

- Observation: To overcome all of the above inefficiencies of OFDM and to meet the new requirements a new waveform should be considered.

- Observation: FBMC systems provide much better spectral shaping and relaxed output filter requirements than OFDM.

- Proposal: Filtered OFDM/UFMC should be considered as one of the waveform candidates; however, a proper analysis must be performed to understand the trade-offs against the conventional OFDM.

- Proposal: FBMC should be considered as one of the waveform candidates, including a detailed analysis of its benefits vs. complexity trade-offs.

Decision: The document is noted.

R1-163311	Discussion on new waveforms	CEWiT

The document was presented by … from CEWiT and proposes to consider and study both OFDM and non-OFDM based waveforms as candidate waveforms for the NR systems.

Decision: The document is noted.

R1-162152	OFDM based flexible waveform for 5G	Huawei, HiSilicon

The document was presented by Long Yi from Huawei and .

Observation: OFDM-based orthogonal waveforms have the following advantages:

  - Supporting a flexible and forward compatible air interface;

  - Enhanced spectrum localization performance;

  - Reuse of legacy OFDM-based transmission thanks to the complex-domain orthogonality retained within sub-bands;

  - Amicable to MIMO technologies;

  - Simpler insertion of reference signals.

Proposal: OFDM based orthogonal waveforms including filtered-OFDM, windowing-OFDM, UF-OFDM should be considered for further evaluation.

Decision: The document is noted.

R1-163110	Initial link level evaluation of waveforms	NTT DOCOMO, INC.

The document was presented by Petra Weitkemper from NTT DOCOMO.

- Observation 1: W-OFDM, F-OFDM and FBMC/OQAM could much improve the OOB emission compared to traditional CP-OFDM.

- Observation 2: F-OFDM could reach overall better OOB performances compared to W-OFDM.

- Observation 3: W-OFDM and F-OFDM are more suited for short burst transmission compared to FBMC/OQAM.

- Observation 4: Without DFT spread, all the waveforms have similar PAPR performance.

- Observation 5: Introducing DFT spread to F-OFDM and W-OFDM has similar PAPR impact compared to CP-OFDM.

- Observation 6: Introducing DFT spread to FBMC/OQAM lead to much lower PAPR reduction compared to the other waveforms.

- Observation 7: The BLER performances of W-OFDM, UF-OFDM and FBMC/OQAM is close to traditional CP-OFDM, while f-OFDM suffers loss for high coding rates.

- Observation 8: Introducing tone offset subcarriers improves the f-OFDM BLER performance with high coding rates. The loss compared to other waveforms becomes minor.

- Observation 9: Under extreme high mobility, FBMC/OQAM outperforms other waveforms.

- Observation 10: All considered waveforms perform similar with 2x2 MIMO spatial multiplexing and linear receivers. Observations 7, 8 and 9 are also valid here.

- Observation 11: Non-linear MIMO schemes like Alamouti or sphere detection cannot be easily combined with OQAM modulation. This is drawback for FBMC/OQAM.

- Observation 12: W-OFDM, UF-OFDM, f-OFDM and FBMC/OQAM outperform CP-OFDM under asynchronous access transmission due to their reduced OOB emissions.

- Observation 13: FBMC/OQAM and f-OFDM needs much smaller of guard subcarriers to achieve the same performance as the synchronous CP-OFDM compared UF-OFDM and W-OFDM.

- Observation 14: Introducing tone offset subcarriers degrades the f-OFDM BLER performance under asynchronous access transmission. The wider filter bandwidth increases the interference to the neighboring users.

- Proposal 1: CP-OFDM should be considered as the baseline waveform.

- Proposal 2: W-OFDM and F-OFDM should be further considered as new waveform candidates.

Decision: The document is noted.

R1-162750	Link-level performance evaluation on waveforms for new RAT	Spreadtrum Communications

The document was presented by Jinhui Chen from Spreadtrum.

- Observation 1: When uplink transmission are synchronized in time but not in frequency, F-OFDM achieves small gain over OFDM against CFO, while FBMC achieves significant gain over OFDM against CFO except for the closest subcarrier that suffers from loss of real-orthogonality.

- Observation 2: When uplink transmission are synchronized in frequency but not in time, F-OFDM achieves moderate gain over OFDM against time offset while  FBMC achieves significant gain over OFDM except for the closest subcarrier that suffers from loss of real-orthogonality.

- Observation 3: For OFDM, interference from the numerology with large subcarrier spacing to that with small subcarrier spacing is higher than the other way around.

- Observation 4: When numerologies with different subcarrier spacing coexist, F-OFDM achieves marginal gain over OFDM, while FBMC achieves significant gain at a cost of one subcarrier as the guard band.

Decision: The document is noted.

R1-162925	Design considerations on Waveform in UL for New Radio systems	INTERDIGITAL COMMUNICATIONS

The document was presented by … from InterDigital.

- Proposal: Waveforms which use DFT-S-OFDM as a basis should be studied for the UL of the new radio.

Decision: The document is noted.

R1-163619	OTFS Waveform for New RAT	Cohere Technologies, AT&T, CMCC, Deutsche Telekom, Telefonica, Telstra	(R1-162930)

The document was presented by … from Cohere.

- Proposal: It is requested that the OTFS modulation and reference signal multiplexing scheme is thoroughly evaluated on its merits for the air interface of 5G systems and included in the Technical Report TR38.8XX “TR for Study on New Radio Access Technology Physical Layer Aspects”.

Decision: The document is noted.

Not treated.

R1-163327	Waveform for New Radio	Motorola Mobility Germany GmbH

R1-162151	Considerations on 5G Waveform	Huawei, HiSilicon

R1-162177	Waveform and multiple access for 5G new radio interface	Samsung

R1-162198	Waveform requirements	Qualcomm Inc.

R1-162199	Waveform candidates	Qualcomm Inc.

R1-162200	Waveform evaluation proposals	Qualcomm Inc.

R1-162225	Discussion on new waveform for new radio interface	ZTE

R1-162333	On new waveforms for New RAT	Fujitsu

R1-162384	Considerations on waveform selection for new radio interface	Intel Corporation

R1-162516	Flexible CP-OFDM with varible ZP	LG Electronics

R1-162537	Views on NR waveforms	Mitsubishi Electric

R1-162548	Considerations on Waveform Design for New RAT	Spreadtrum Communications

R1-162551	Considerations on waveform design for new radio interface	Panasonic Corporation

R1-162793	Waveform Candidates for Next Generation New Radio Access	MediaTek Inc.

R1-162889	OFDM based Waveform for 5G new radio interface	Nokia, Alcatel-Lucent Shanghai Bell

R1-162890	5G Waveforms for the Multi-Service Air Interface below 6 GHz	Nokia, Alcatel-Lucent Shanghai Bell

R1-163222	Waveform for NR	Ericsson

R1-163223	Using OFDM in NR	Ericsson

R1-163224	Feasibility of Mixing Numerology in an OFDM System	Ericsson

#### Multiple access scheme

R1-163160	Generalized Precoded OFDMA (GPO)	IITH

The document was presented by Kiran Kuchi from IITH.

Decision: The document is noted.

R1-162153	Overview of non-orthogonal mutliple access for 5G	Huawei, HiSilicon

The document was presented by Wang Yi from Huawei.

- Observation 1: The expected functionalities for 5G MA are different in each usage scenario and are summarized (see Table 1).

- Observation 2: Non-orthogonal MA can have advantage over OMA in many aspects (see Table 2), which deserves careful investigation in 5G NR.

- Proposal 1: Code domain non-orthogonal design is needed in addition to the power domain superposition to deal with channel and interference uncertainties.

- Proposal 2:  Sparse spreading is an important feature in the code domain design to enable high level of overloading with close to ML detection performance and reasonable detection complexity.

Decision: The document is noted.

R1-162155	Sparse Code Multiple Access (SCMA) for 5G Radio Transmission	Huawei, HiSilicon

- Multi-dimensional modulation and sparse spreading should be further studied as a candidate scheme for non-orthogonal multiple access for 5G new radio.

Decision: The document is noted.

R1-162870	On Unified Framework for Multi-Access Schemes	CMCC

The document was presented by Jiqing Ni from CMCC.

- Proposal 1: Considering diversified 5G scenarios and enhanced performance requirements, new multiple access schemes should be studied.

- Proposal 2: It is preferred to compare the structure of various non-orthogonal multiple access schemes for a better understanding of their mechanisms which may facilitate future down selection and harmonization in RAN1. A unified framework of multiple access techniques is proposed for such comparison.

- Proposal 3: Current proposed non-orthogonal multiple-access schemes may be merged to a single non-orthogonal multiple-access scheme based on the unified framework, while the orthogonal multiple-access scheme should be a special of the merged non-orthogonal multiple-access scheme.

Decision: The document is noted.

R1-162203	NR multiple access evaluation proposals	Qualcomm Inc.

The document was presented by … from Qualcomm.

- Proposal 1: different multi-access candidates can be evaluated/optimized for different use cases, such as eMBB, eMTC and uRLLC. However, it is recommended to start with analytic comparisons before directly going to full blown system level simulations. This can include, but not limited to:

  - Capacity analysis for NOMA vs OMA for eMBB

  - Link budget vs capacity trade-off for mMTC

  - Signaling overhead for NOMA vs OMA for mMTC

- Proposal 2: If there is a clear need to do system level simulations for evaluation/decision, agreement has to be made regarding the evaluation assumptions, then different scheme can be compared focusing on the key performance matric list (see section 3).

Decision: The document is noted.

R1-163383	Candidate Solution for New Multiple Access	CATT	(R1-162306)

The document was presented by … from CATR and deals with a code based NOMA scheme with following characters to alleviate error propagation problem of SIC:

- It takes joint optimization of transmitter and receiver, with unequal transmission diversity order but identical diversity order after detection.

- It uses sparse coding to map user data onto sharing resources.

System level evaluation results show that the proposed NOMA scheme is beneficial in both spectrum efficiency and connection density.

Decision: The document is noted.

R1-162517	Considerations on DL/UL multiple access for NR	LG Electronics

The document was presented by … from LGE.

- Suggestion 1: OMA (e.g. OFDMA for DL, SC-OFDMA for UL) should be considered as a baseline MA scheme for NR study.

- Suggestion 2: OFDMA can also be considered as MA scheme for UL in NR if PAPR of the UE operating in NR system is not important issue.

- Suggestion 3: To consider NoMA schemes as a candidate of the MA schemes in NR, it is required to justify the advantages and/or to complement the defects.

- Suggestion 4: It can be considered to use NoMA scheme such as NCMA in order for supporting massive connectivity or contention based MA in special scenarios.

- Suggestion 5: Combination of multiple MA schemes, including OMA and NoMA schemes can be considered for NR design in special scenarios.

Decision: The document is noted.

R1-163695	WF on multiple access for NR	ZTE, Qualcomm Incorporated

- Non-orthogonal multiple access should be evaluated for all three scenarios: mMTC, eMBB, ULLRC.

- For uplink transmission for mMTC,

  - Non-orthogonal transmission is grant-free based

  - The candidate non-orthogonal schemes include (but not limited to):

    - Multi-user shared access (MUSA)

    - Resource spread multiple access (RSMA )

    - Sparse code multiple access (SCMA)

    - Pattern defined multiple access (PDMA)

    - Non-orthogonal coded multiple access (NCMA)

    - Frequency domain spreading

Decision: The document is noted.

Observations:

- Examples non-orthogonal schemes include (but not limited to):

  - For UL, Multi-user shared access (MUSA) (e.g., R1-162226)

  - Resource spread multiple access (RSMA) (e.g., R1-163510)

  - Sparse code multiple access (SCMA) (e.g., R1-162153)

  - Pattern defined multiple access (PDMA) (e.g., R1-163383)

  - Non-orthogonal coded multiple access (NCMA) (e.g., R1-162517)

  - Low code rate spreading (e.g., R1-162385)

  - Frequency domain spreading (e.g., R1-162385)

  - Non-orthogonal multiple access (NOMA) (e.g., R1-163111)

Not formally presented, just noted:

R1-163111	Initial views and evaluation results on non-orthogonal multiple access for mMTC uplink	NTT DOCOMO, INC.

R1-162226	Discussion on multiple access for new radio interface	ZTE

R1-162385	Multiple access schemes for new radio interface	Intel Corporation

R1-163510	Candidate NR multiple access schemes	Qualcomm Inc.

R1-163656	WF on multiple access for NR	CMCC, Huawei, HiSilicon, Fujitsu, CATT, China Telecom

The document was presented by Jiqing Ni from CMCC.

Observation: Non-orthogonal multiple access methods proposed for the  NR within this meeting show potentials to have benefits over orthogonal MA for eMBB, mMTC and URLLC respectively

- Non-orthogonal MA (No-MA) should be investigated for diversified NR usage scenarios and use cases, including at least eMBB (UL and DL), mMTC (UL), and URLLC (UL)

- Code and/or power domain No-MA schemes should be studied

- At least for UL mMTC, autonomous/grant-free/contention based No-MA should be studied

- Flexible MA framework should be studied which can select and/or adjust  between OMA and No-MA

Discussion: Better not use 5G in 3GPP.

Decision: The document is noted.

Agreements:

- Non-orthogonal multiple access should be investigated for diversified NR usage scenarios and use cases

- At least for UL mMTC, autonomous/grant-free/contention based non-orthogonal multiple access should be studied

Not treated.

R1-162178	Multiple access for different vertical services in 5G new radio interface	Samsung

R1-162305	Multiple Access for 5G New Radio Interface	CATT

R1-162334	On non-orthogonal multiple access technologies	Fujitsu

R1-162552	Considerations on multiple access schemes for new radio interface	Panasonic Corporation

R1-162568	Multiplexing and multiple access for mmWave networks	ETRI

R1-162794	Contention-based uplink access	MediaTek Inc.

R1-162891	Scheduling framework and related requirements	Nokia, Alcatel-Lucent Shanghai Bell

R1-162892	Uplink contention-based access in 5G New Radio	Nokia, Alcatel-Lucent Shanghai Bell

R1-162922	Discussion on Multiple Access for New Radio systems	INTERDIGITAL COMMUNICATIONS

R1-163383	Candidate Solution for New Multiple Access	CATT

R1-163509	NR multiple access requirements	Qualcomm Inc.

### Numerology and frame structure for new radio interface

R1-163536	Numerology and frame structure for 5G new radio interface: over-6GHz	Samsung	(R1-162180)

The document was presented by … from Samsung.

- Proposal 1: The numerology design of over-6 GHz (e.g., around 30 GHz and 70 GHz) and sub-6 GHz (e.g., around 4 GHz) needs to be separately optimized.

- Proposal 2: For the numerology design over 6 GHz, study of bands below 40 GHz is prioritized over that of bands above 40 GHz.

- Proposal 3: The numerology above 6 GHz should be designed to support at least the coverage of ISD = 200m.

- Proposal 4: CP length should be long enough to compensate the channel delays and other effects while keeping a reasonable CP overhead ratio (e.g., longer than 300 – 400 ns).

- Proposal 5: Based on observations 8, 11 and 12, it is preferred to use 75 kHz as subcarrier spacing for 5G new radio interface over 6 GHz, especially for the carrier frequency range between 28 GHz and 40 GHz.

- Proposal 6: It is preferred to use 150 kHz as subcarrier spacing for 5G new radio interface over 40 GHz if subcarrier spacing larger than 75 kHz is needed.

- Proposal 7: Taking into account implementation complexity, maximum FFT size should be limited and we prefer to use 2048 FFT size and 100 MHz bandwidth per CC for 75 kHz subcarrier spacing as a baseline.

- Proposal 8: For 5G new air interface over 6 GHz, TDD is preferred rather than FDD and Self-contained frame structure is supported for forward compatibility.

Decision: The document is noted.

R1-162206	Frame structure requirements	Qualcomm Inc.

The document was presented by … from Qualcomm.

- Proposal 1: Frame structure should be composed of two fundamental units, i.e., TTIs and SFs. A Transmission Time Interval (TTI) is the smallest time unit over which transmission can occur, in multiples of symbols. A Subframe (SF) is the smallest unit consisting of one or multiple TTIs, such that both a transaction from the source to destination and a transaction from the destination to the source are completed. There may be one or multiple TTIs per SF.

- Proposal 2: Frame structure should allow for scalability to address different services. This should include supporting of services with a target HARQ-RTT latency of 1ms, or services with an end-to-end application layer latency of 1ms.

- Proposal 3: Frame structure should allow for multiplexing of different services, such as EMBB and URLLC.

- Proposal 4: Frame structure should accommodate user multiplexing according to the topologies of basestation-to-UE uplink and downlink, and UE-to-UE sidelink.

- Proposal 5: Frame structure should support very wideband as well as narrowband deployment scenarios, including TDD and FDD allocations, and address both sub-6GHz and mmW spectrum.

- Proposal 6: Frame structure should be able to exploit channel reciprocity efficiently in TDD through availability of frequent sounding opportunities.

- Proposal 7: Frame structure should enable dynamic TDD assignment with support for managing interference between source and destination pairs (e.g., among uplink, downlink, or sidelink).

- Proposal 8: Frame structure should enable self-contained subframes, whereby the source-to-destination transmission as well as the response to that transmission, going from the destination-to-source, are both contained in the same subframe.

- Proposal 9: Frame structure should enable self-contained subframes with a single interlace structure.

- Proposal 10: Frame structure may include multiple interlaces subject to forward compatibility constraints.

- Proposal 11: Frame structure may include multiple interlaces subject to overhead constraints for given target latency.

- Proposal 12: Frame structure should allow for modes where a short intermediate round of communication between the source and destination can occur within a subframe.

- Proposal 13: Frame structure should allow for tight coupling across aggregated carriers.

Decision: The document is noted.

R1-163397	Numerology requirements	Qualcomm Inc.	(R1-162204)

The document was presented by Tingfang Ji from Qualcomm.

- Proposal 1: Numerology design should be determined based on the general requirements in Section 2 for the specific deployment scenario.

- Proposal 2: It is proposed to have scaled subcarrier spacing = Δf0*2k in order to achieve high multiplexing efficiency between different numerologies.

- Proposal 3: It is proposed to have 2^M symbols per TTI to ensure flexible TTI downscaling for URLLC from 2^M symbols to 1.

- Proposal 4: It is proposed to have 2^M symbols per TTI and scaled subcarrier scaling = Δf0*2k at the same time to ensure flexible numerology scaling up and down in the same TTI. Scalable short symbols are to support low latency and scalable long symbols are to ensure robustness in MBSFN/COMP type of features.

- Proposal 5: it is proposed to have NR numerology family as:

  - subcarrier spacing Δfk = Δf0*2k,

  - CPk = CP0/2k,

  - 16*2k symbols per ms,

where one example is: Δf0 = 17.5 kHz, CP0 = 5.36us, 16 symbols per ms and k is an integer.

Decision: The document is noted.

R1-162386	Numerology for new radio interface	Intel Corporation

The document was presented by Joonyoung Cho from Intel.

- Proposal 1: For operating frequencies in low-to-mid frequency bands (for e.g., > 6 GHz and < 40 GHz), 75 kHz subcarrier spacing is preferred because of frequency drift and phase noise considerations.

- Proposal 2: For high frequency bands (e.g., >=40 GHz), a much higher subcarrier spacing (e.g., 375 kHz) can be considered.

- Proposal 3: The radio frame length is recommended to be kept as 10 ms, regardless of subcarrier spacing for efficient inter operations between LTE and NR-Phase 1 and NR-Phase 2.

- Proposal 4: For high frequency bands (e.g., >=40 GHz), designing a system targeting delay spread of omnidirectional channel can result in high overheads, thus re-configurable CP lengths can be considered as an option to reduce overheads and improve spectral efficiency.

- Proposal 5: A TTI of 0.2 ms consisting of 14 symbols when 75 kHz subcarrier spacing is used should be considered for NR for low-to-mid frequency bands (for e.g., > 6 GHz and < 40 GHz).

Decision: The document is noted.

R1-162742	Frame structure design for next radio interface	Lenovo (Beijing) Ltd

The document was presented by Xinghua Song from Lenovo.

- Proposal 1: Multiple TTI lengths with scalability should be supported in NR.

- Proposal 2: For NR, the following subframe types are supported

  - Special subframe containing DL(data and/or control), GP, and UL (data and/or control)

  - DL only subframe

  - UL only subframe

- Proposal 3: Dynamic TDD should be supported and how to signal the flexible TDD frame structure should be studied.

Discussion: Fujitsu what subframe duration being considered. Lenovo may depend on the scenarios to be supported

Decision: The document is noted.

R1-162795	Numerology Considerations for Next Generation New Radio Access	MediaTek Inc.

The document was presented by Pekai Liao from MediaTek.

- Proposal 1: A set of OFDM-based numerologies supporting all the deployment scenarios should be studied, including one that is backward compatible with LTE and one tailored to dense urban with sub-carrier spacing of 60 or 75 kHz.

- Proposal 2: The design should consider the system aspects such as cyclic-prefix overhead, resource block granularity as well as the device implementation aspects such as FFT size, clock rate, etc.

Decision: The document is noted.

R1-163112	Initial views on frame structure for NR access technology	NTT DOCOMO, INC.

The document was presented by Fred Takeda from NTT DOCOMO.

- Observation 1: Lean radio-frame with flexible dynamic TDD would be a good candidate radio frame design for the new RAT.

- Observation 2:

  - Fixed DL subframe is transmitted by TP periodically to deliver necessary channels/signals/RSs.

    - Necessary functions and their periodicities could be different depending on use-case and scenario.

  - Further investigation is necessary on how to design the fixed DL subframe.

    - Forward compatibility should be taken into account.

- Observation 3: Further study is necessary on how to design dynamic subframe.

  - It is highly important to ensure high flexibility in dynamic subframe operations.

    - It is desirable to indicate the usage of each subframe by the DL control signalling in the subframe.

    - It is desirable to support flexible (explicit) operations of HARQ and CSI feedback.

  - Self-contained operation would bring different benefits and can also be another solution.

    - Very quick HARQ-ACK feedback can realize extra low latency operation.

    - Overhead saving by linking time/frequency resource information between the channels/signals.

Decision: The document is noted.

R1-163227	Numerology for NR	Ericsson

The document was presented by Erik Dahlman from Ericsson and proposes to adopt 3 different OFDM numerologies for 5G new RAT; 15, 30, and 60 kHz. All numerologies are derived from the baseline 15 kHz numerology via scaling. The 15 kHz numerology is identical to LTE.

- Proposal 1: 5G new RAT will define a set of OFDM numerologies

- Proposal 2: Other OFDM numerologies of the set are derived from the baseline OFDM numerology of the set via scaling

- Proposal 3: A subcarrier spacing is integer dividable by all smaller subcarrier spacings of the set

- Proposal 4: The baseline OFDM numerology for 5G new RAT is the LTE numerology including both normal and extended cyclic prefix (Δf1, Tofdm,1, Tcp,1, Tcp,1')

- Proposal 5: For 5G new RAT Phase I, in addition to the baseline OFDM numerology the following OFDM numerologies are defined: (Δf2=2∙15 kHz=30 kHz,  Tofdm,2=66.672μs=33.33 μs,  Tcp,2=4.692 μs=2.35 μs, Tcp,2'=16.672 μs=8.33 μs ) and (Δf3=4∙15 kHz=60 kHz,  Tofdm,3=66.674μs=16.67 μs,  Tcp,3=4.694 μs=1.17 μs, Tcp,3'=16.674 μs=4.17 μs ).

Discussion: Compatibility with IoT is key aspect when refarming LTE to 5G new RAT.

Decision: The document is noted.

R1-163605	WF on NR Numerology	NTT DOCOMO, INC.

The document was presented by Fred Takeda from NTT DOCOMO.

- For NR, it is necessary to support more than one values of subcarrier-spacing

  - A value of subcarrier-spacing is derived from a particular value of subcarrier-spacing multiplied by N where N is an integer

    - Alt.1: At least one of LTE subcarrier-spacing (15 kHz) is one of the supported values

      - FFS: Whether other LTE subcarrier-spacing are also the supported values

    - Alt.2:  Any LTE subcarrier-spacing is not one of the supported values

    - FFS: exact value of the particular value and possible values of N

  - The values of possible subcarrier-spacing will be further narrowed-down in RAN1#85

Decision: The document is noted.

Possible agreements

For NR, it is necessary to support more than one values of subcarrier-spacing

- Values of subcarrier-spacing are derived from a particular value of subcarrier-spacing multiplied by N where N is an integer

  - Alt.1: Subcarrier-spacing values include 15 kHz subcarrier spacing (i.e LTE based numeroly)

  - Alt.2: Subcarrier-spacing values are based on multiple or division of 15 kHz subcarrier spacing without 15 kHz subcarrier spacing (LGE’s proposal)

  - Alt.3: Subcarrier-spacing values include 17,5 kHz subcarrier spacing with uniform symbol duration including CP length

    - Note: other values are not precluded

  - FFS: exact value of the particular value and possible values of N

- The values of possible subcarrier-spacing will be further narrowed-down in RAN1#85

Agreements

- For NR, it is necessary to support more than one values of subcarrier-spacing

  - Values of subcarrier-spacing are derived from a particular value of subcarrier-spacing multiplied by N where N is an integer

    - Alt.1: Subcarrier-spacing values include 15 kHz subcarrier-spacing (i.e., LTE based numerology)

    - Alt.2 Subcarrier-spacing values include 17.5 kHz subcarrier-spacing with uniform symbol duration including CP length

    - Alt.3: Subcarrier-spacing values include 17.06 kHz subcarrier-spacing with uniform symbol duration including CP length

    - Alt.4: Subcarrier-spacing values 21.33 kHz

    - Note: other alternatives are not precluded

    - FFS: exact value of a particular value and possible values of N

  - The values of possible subcarrier-spacing will be further narrowed-down in RAN1#85

- Companies are encouraged to provide detailed analysis and input the views in the following table

|  | Proponent name |  |  |  |
| --- | --- | --- | --- | --- |
|  | Set 1 | Set 2 | Set 3 | …Set X |
| Subcarrier-spacing (kHz) |  |  |  |  |
| Component carrier bandwidth (MHz) |  |  |  |  |
| OFDM symbol length (µs) |  |  |  |  |
| CP length (µs) |  |  |  |  |
| Nb of symbols / subframe |  |  |  |  |
| Subframe length (ms) |  |  |  |  |
| Frame length (ms) |  |  |  |  |
| Note |  |  |  |  |

R1-162931	Frame Structure and Numerology for New RAT	Cohere Technologies

- Proposal: Consider a configurable frame and numerology to address the diverse next generation deployment scenarios

  - In particular, consider a configurable TTI length to meet the performance targets for all use cases

Decision: The document is noted.

R1-162227	Numerology design of new RAT	ZTE

Decision: The document is noted.

Wednesday

R1-163620	15 KHz and 17.5 KHz based numerology comparison	Qualcomm Incorporated

The document was presented by Tingfang Ji from Qualcomm.

Decision: The document is noted.

R1-163715	WF on Numerology Comparison	Qualcomm Incorporated, ZTE

The document was presented by Jing Jiang from Qualcomm.

Scalable numerology: 15kHz vs. 2N family comparison

- Link-level numerology comparison (simulation)

  - Typical 3GPP channel models: scaled EPA/EVA/ETU depending on deployment scenarios

  - Numerology evaluation over mmWave band

- TTI scaling down (analysis)

  - Comparison between 15kHz (non-integer symbol) vs. 17.5kHz (scalable number of symbols)

- Link-level evaluation of long CP (simulation)

  - Long DS channel model performance evaluation at the link-level

  - 15kHz family based on LTE ECP vs. 17.5kHz scaled SCS (8.75kHz or 4.375kHz)

- NB-IOT coexistence evaluation (simulation/analysis)

  - Guardband overhead comparison

- LTE TDD Coexistence (analysis):

  - Additional GP overhead quantification

- Sampling Rate Complexity (analysis)

Decision: The document is noted. Continue offline.

R1-163559	WF on Band Agnostic Numerology Specification	Huawei, HiSilicon, Ericsson

The document was presented by David Mazzarese from Huawei.

- RAN1 specifications and PHY features of the New Radio should continue to be specified in a band-agnostic manner

  - OFDM numerologies supported by NR will not be limited to any specific band or frequency range in RAN1 specifications

  - In particular, the higher-order numerologies should not only be limited to higher frequencies

  - strive for a common structure regardless of frequency-band of operation considering different channel characteristics and target verticals

Discussion: Huawei suggested having at least that the band agnostic aspect be a common understanding.

Samsung too early to agree such statement  want to let RAN4 decide

Decision: The document is noted.

Thursday

R1-163770	WF on Scaling of subcarrier spacing	Ericsson, Qualcomm, Nokia, Alcatel Lucent Shanghai Bell, NTT DOCOMO

The document was presented by Stefan Parkvall from Ericsson.

- The subcarrier spacing for the NR scalable numerology should scale as

  - fsc = f0 * 2m where

    - f0 is FFS

    - m is an integer chosen from a set of possible values

Decision: The document is noted.

Possible agreement

- The subcarrier spacing for the NR scalable numerology should scale as

  - fsc = f0 * m where

    - f0 is FFS

    - m is an integer chosen from a set of possible positive values

Possible agreement

- The subcarrier spacing for the NR scalable numerology should scale as

  - fsc = f0 * mn where

    - f0 is FFS

    - m is one value and is an integer chosen from a set of possible positive values

    - n is an integer chosen from a set of possible values

Possible agreements:

- For future evaluations, RAN1 assumes

  - The subcarrier spacing for the NR scalable numerology should scale as

  - fsc = f0 * 2m where

    - f0 is FFS

    - m is an integer chosen from a set of possible values

- Other values are not precluded

  - Possible other values include followings

    - The subcarrier spacing for the NR scalable numerology should scale as

    - fsc = f0 * m where

      - f0 is FFS

      - m is an integer chosen from a set of possible positive values

- RAN1 continues further study until #85 meeting

Agreements:

- RAN1 will continue further study and conclude between following alternatives in the next meeting

  - Alt. 1:

    - The subcarrier spacing for the NR scalable numerology should scale as

    - fsc = f0 * 2m where

      - f0 is FFS

      - m is an integer chosen from a set of possible values

  - Alt. 2:

    - The subcarrier spacing for the NR scalable numerology should scale as

    - fsc = f0 * M where

      - f0 is FFS

      - M is an integer chosen from a set of possible positive values

- All companies are requested to analyze/evaluate following aspects

  - Realistic phase noise

  - How each alternative allows mixing different numerologies

- All companies are requested to propose exact values of

  - f0, m, and M

R1-163554	WF on Band Agnostic Numerology Specification	Huawei, HiSilicon

The document was presented by David Mazzarese from Huawei.

- For the study of NR, RAN1 assumes that OFDM numerologies with different subcarrier spacings are allowed to be used in the same band and frequency range

  - For example 15 kHz subcarrier spacing and 60 kHz subcarrier spacing could both be used below 6 GHz

Decision: The document is noted and is modified as follows:

Agreements:

- For the study of NR, RAN1 assumes that multiple (but not necessarily all) OFDM numerologies can apply to the same frequency range

  - Note: RAN1 does not assume to apply very low value of subcarrier spacing to very high carrier frequency

Not treated.

R1-162156	Scenario & design criteria on flexible numerologies	Huawei, HiSilicon

R1-162158	Support for wide bandwidth for 5G	Huawei, HiSilicon

R1-162159	Deployment of 5G spectrum below 6 GHz	Huawei, HiSilicon

R1-162179	Numerology and frame structure for 5G new radio interface: sub-6GHz	Samsung

R1-162205	Numerology evaluation criteria	Qualcomm Inc.

R1-162207	Candidate frame structures	Qualcomm Inc.

R1-162228	Frame structure design of new RAT	ZTE

R1-162307	Numerology for the 5G new radio interface	CATT

R1-162308	Frame structure design for 5G new radio interface	CATT

R1-162335	On new frame structure for New RAT	Fujitsu

R1-162338	Numerology and Frame Structure Design Considerations for NR	Guangdong OPPO Mobile Telecom.

R1-162428	Discussion on numerology and frame structure for NR	NEC Corporation

R1-162518	Discussion on numerology design for New RAT	LG Electronics

R1-162519	Discussion on frame structure for NR	LG Electronics

R1-162549	Discussion on numerology and frame structure	Spreadtrum Communications

R1-162550	Initial considerations on the framework for 5G new RAT	Spreadtrum Communications

R1-162569	Scalable TTI for new radio frame structure	ETRI

R1-162579	Numerology and frame structure for New Radio	INTERDIGITAL COMMUNICATIONS

R1-162726	Frame structure for new radio interface	Intel Corporation

R1-162796	Views on frame structure for new radio access technology	MediaTek Inc.

R1-162871	Discussion on the numerology and frame structure design for new radio	CMCC

R1-162893	Basic frame structure principles for 5G New Radio	Nokia, Alcatel-Lucent Shanghai Bell

R1-162894	Flexible numerology for 5G New Radio	Nokia, Alcatel-Lucent Shanghai Bell

R1-162895	Support for Beam Based Common Control Plane in 5G New Radio	Nokia, Alcatel-Lucent Shanghai Bell

R1-163113	Initial views on numerology for NR access techonology	NTT DOCOMO, INC.

R1-163118	Considerations on 5G new RAT numerology	Beijing Xinwei Telecom Techn.

R1-163132	Discussions on the frame structure design for NR	CATR

R1-163225	Duplex scheme for NR	Ericsson

R1-163226	Frame structure for NR	Ericsson

R1-163228	Flexible bandwidth in NR	Ericsson

R1-163229	Hybrid-ARQ acknowledgements for NR	Ericsson

R1-163230	Latency in NR	Ericsson

R1-163295	Scalable numerology for New RAT	Sharp

R1-163328	Numerology for New Radio	Motorola Mobility Germany GmbH

R1-163816	WF on frame structure for NR	LG Electronics, Qualcomm Incorporated, ITL, Lenovo, ZTE

R1-163880	Way Forward on Subframe for NR	Qualcomm Incorporated, NTT DOCOMO

Friday

R1-163864	Alternative options to a achieve 2^n symbols per 1 ms numerology	ZTE, Qualcomm Incorporated

- Modular design for 2^n symbols per 1 ms is desired

- Concerns have been raised regarding

  - CP length only scales with symbol duration

  - Sampling rate does not line up with LTE sampling rate

- How to line up with LTE sampling rate

  - Keep fs = 30,72 MHz, change FFT size

- How to enable longer CP without changing symbol (CP+useful symbol) duration

  - Increase the CP part, reduce the useful symbol part in the symbol

- Define common test cases for evaluation of different numerologies

- Interested companies are encouraged to select and evaluate numerologies

Decision: The document is noted.

R1-163903	Way Forward on Frame Structure for NR	Qualcomm Incorporated, NTT DOCOMO, Panasonic

- At least the following should be considered in the NR frame structure study:

  - A time interval with demodulation RS, the assignment, the acknowledgement, and data

  - The RS used for the demodulation of the data is only within this time interval

  - The design should not preclude the possibility for the following

    - The assignment for the data in this time interval and corresponding RS used for the demodulation are only within this time interval.

    - The acknowledgment for the data in this time interval and corresponding RS used for the demodulation are only within this time interval.

Discussion: LG different proposal in R1-163886

Huawei Time interval characteristics far to be clear- too early to agree.

Ericsson good starting point

Decision: The document is noted.

R1-163886	WF on frame structure for NR	LG Electronics, Qualcomm Incorporated, Lenovo

- At least followings are considered in design of frame structure for NR

  - Three types of schedulable time unit can be defined, each consisting of:

    - 1) DL transmission only, 2) UL transmission only, 3) both DL and UL transmissions

    - DL transmission can include DL control and/or DL data

    - UL transmission can include UL control and/or UL data

    - Further study is necessary on:

      - Combination of control/data part within each type of time unit

      - Location of control/data resource within each type of time unit

Decision: The document is noted. No consensus for neither of the proposals.

Ericsson  would be worth trying to list a number of topics to look at (considered as key aspects for further progress)

RAN1 chair: can set up an email discussion to address the frame structure proposals

Samsung  do not understand why the proposals should be discussed

NTT DOCOMO: email discussion may help to capitalize on the offline discussions made during the week and further get rid of the confusions

Focus could be on understanding time interval regarding demodulation RS, assignment, acknowledgement and data.

Conclusion:

Email discussion until May 9th on the frame structure with focus on understanding DL/UL/Sidelink time domain structure containing RS, the assignment, the acknowledgement, and data –(Qualcomm)

(Note that this email discussion will not try to have approval)

### Channel coding and modulation for new radio interface

Not treated.

R1-162229	Discussion on channe coding and modulation for new radio interface	ZTE

R1-163270	Considerations for NU-QAM and LDPC codes for the NR	Sony

#### Channel coding

R1-162181	Channel coding for 5G new radio interface	Samsung

The document was presented by … from Samsung.

- Proposal 1: Companies are encouraged to provide qualitative and/or quantitative info on hardware complexity in order to support 20Gbps decoding throughput (for both new channel coding scheme and LTE turbo code)

- Proposal 2: HW complexity should be one of the key criterions for down selection of new channel coding schemes in RAN1. Details on how to do down selection should be discussed.

- Proposal 3: In addition to HW complexity, other factors such as encoding/decoding latency and performance at extremely low error rate that relate to 5G KPIs should be considered.

- Proposal 4: Flexibility related to code block size and code rate are important aspects to be considered when determining the channel coding scheme for 5G new air interface. RAN1 should discuss how to capture this aspect in the study item phase.

- Proposal 5: Determine parameters for evaluation of new channel coding schemes for 5G new interface considering different requirements for eMBB, URLLC, and mMTC. Initial evaluations can be done using QPSK modulation in AWGN channel considering

- Proposal 6: Approve the set of parameters for evaluation in Table 2 as a starting point

- Proposal 7: Approve the evaluation and selection procedures:

  - Evaluate the feasibility of implementation and down-select first

  - Decide clear definitions for flexibility and evaluation parameters

  - Evaluate the coding performance for down-selected proposals and select the most adequate

Decision: The document is noted.

R1-162896	Channel coding requirements for 5G New Radio	Nokia, Alcatel-Lucent Shanghai Bell

The document was presented by Keeth Jayasinghe from Nokia.

Proposal 1: Different requirement shall be considered to select the channel coding scheme or schemes for next generation radio technologies. Key requirements of each usage scenario of next radio access can be identified as in the following table:

| eMBB | mMTC | URLLC |
| --- | --- | --- |
| Good error performance with high throughput | Good error performance with low throughput | Very good error performance with low/medium throughput |
| High energy efficiency | Simple implementation | Low latency encoding/decoding |
| High chip area efficiency | High energy efficiency | Very low error floor |
| Low latency encoding/decoding |  |  |

Proposal 2: Single channel coding scheme from Turbo, LDPC, and Polar shall be selected for eMBB.

Proposal 3: mMTC and URLLC usage scenarios should consider the channel coding scheme to be adopted for eMBB as the baseline, and adopting a different channel coding scheme than for eMBB should only be introduced if compelling benefits are identified.

Proposal 4: mMTC and URLLC usage scenarios shall adopt simple channel coding scheme if it is different from the eMBB channel coding scheme.

Decision: The document is noted.

R1-163274	Evaluation methodology for 5G New Radio channel coding	Nokia, Alcatel-Lucent Shanghai Bell

Decision: The document is noted.

R1-162162	High level comparison of candidate FEC schemes for 5G	Huawei, HiSilicon

The document was presented by Carmela Cozzo from Huawei and shows a preliminary comparison of some FEC candidates for the new radio, specifically on Polar Code compared to the LTE Turbo Code and Tail-biting Convolutional Code (TBCC).

Decision: The document is noted.

R1-162230	Discussion on channel coding for new radio interface	ZTE

The document was presented by Huaming Wu from ZTE.

- Proposal 1:  TBCC and its performance improvement should be considered for short packet transmission.

- Proposal 2: Binary and Non-binary LDPC codes should be considered for eMBB in the new RAT.

- Proposal 3: Polar code and its application in the new RAT is FFS.

- Proposal 4: Packet coding should be considered for the new RAT to improve error correction and HARQ performance while reducing decoding complexity and latency.

- Observation 1: Turbo code may not be suitable for the case of large bandwidth, high reliability and short packet transmission.

- Observation 2: Packet coding has better performance and steeper slope performance curve. It is very suitable for high throughput scenarios with high reliability.

- Observation 3: Packet coding can reduce the complexity and latency of encoders & decoders, while keeps the same performance of transmission packet as a long codeword. The packet coding can be used in low complexity and low latency Scenarios.

Decision: The document is noted.

R1-162397	Outer erasure code	Qualcomm Incorporated

The document was presented by Tingfang Ji from Qualcomm.

- Proposal 1: Outer erasure codes should be considered as part of 5G SI

- Proposal 2: To use outer erasure code to combat bursty puncturing/interference to achieve high efficiency, reliability with low latency and low uplink feedback and downlink signaling overhead

- Proposal 3: To use outer erasure code to reduced latency and buffering requirements

- Proposal 4: To use outer erasure code over broadcast channel to combat bursty interference, time selective fading and shadowing

- Proposal 5: Outer erasure codes should be designed with rate compatibility, support outer HARQ on top of PHY inner HARQ. Erasure coding and the corresponding outer HARQ process can be enabled/disabled efficiently with low signaling overhead

- Proposal 6: Outer erasure code performance should be evaluated based on key metrics such as spectrum efficiency, reliability, latency, overhead and encoding/decoding complexity

Decision: The document is noted.

R1-162797	HARQ enhancement for improved data channel efficiency	MediaTek Inc.

The document was presented by … from MediaTek.

- Observation 1: Single HARQ partition per TB in LTE will suffer large retransmission penalty for future large TB sizes.

- Proposal 1: Finer ACK/NACK partition in a TB for the chance of eliminating retransmissions of correct data in a TB.

- Proposal 2: Allow retransmission data of a HARQ process to be multiplexed with new data of another HARQ process in the same transmission.

- Proposal 3: Impose proper correlation between the encoder outputs of retransmission data and new transmission data multiplexed in the same transmission so that new transmission data volume can be increased by partly leveraging retransmission combing gain.

Decision: The document is noted.

R1-163232	Performance study of existing turbo codes and LDPC codes	Ericsson

The document was presented by Yufei Blankenship from Ericsson.

- Finer granularity in LTE turbo code rate provides system-wide performance advantage, especially for the low code rate regime.

- Incremental-redundancy based HARQ available to LTE turbo codes has great performance advantage, compared to 802.11n LDPC codes with Chae combining only.

- With the same set of (code rate, info block length), 3GPP LTE turbo code and 802.11ad LDPC codes achieve comparable performance.

- With the same set of (code rate, info block length), 3GPP LTE turbo code requires much fewer iterations compared to the LDPC code defined for IEEE 802.11ad.

Decision: The document is noted.

R1-163130	Considerations on channel coding for NR	CATR

The document was presented by … from CATR.

- Observation 1: Coding schemes for NR should support variety TBS, flexible coding rates selection, low decoding error rate and fast decode.

- Proposal 1: Coding schemes for control channel, broadcast channel and data channel should be designed independently.

- Proposal 2: In NR, error floor should be avoided for high reliability transmission.

- Proposal 3: Decoding latency, data storage and power consumption should be carefully evaluated to make coding schemes selection.

- Proposal 4: RM, TBCC, Polar code could be considered for broadcast and control channel.

- Proposal 5: Turbo, LDPC and Polar code could be considered for data channel.

Decision: The document is noted.

R1-163662	Way Forward on Channel Coding Scheme for 5G New Radio	Samsung, Nokia, Qualcomm, ZTE, Intel, Huawei

- Candidates for 5G new RAT data transmission are identified as the following

  - LDPC code

  - Polar code

  - Convolutional code

- Selection of 5G new RAT channel coding scheme(s) will consider,

  - Performance

  - Implementation complexity

  - Latency (Decoding/Encoding)

  - Flexibility

Decision: The document is noted and modified as follows:

Agreements:

- Candidates for 5G new RAT data transmission are identified as the following

  - LDPC code

  - Polar code

  - Convolutional code (LTE and/or enhanced convolutional coding)

  - Turbo code (LTE and/or enhanced turbo coding)

  - Note: It is RAN1 common understanding that combination of above codes is not precluded

  - Note: Outer erasure code is not precluded

- Selection of 5G new RAT channel coding scheme(s) will consider,

  - Performance

  - Implementation complexity

  - Latency (Decoding/Encoding)

  - Flexibility (e.g., variable code length, code rate, HARQ (as applicable for particular scenario(s)))

R1-163793	Outer erasure code WF	Qualcomm Inc, ZTE, InterDigital, Sony

- Phase 1 and later phases of NR should investigate the benefits/tradeoff of incorporating outer coding for the following:

  - Forward compatibility to URLLC/eMBB coexistence

  - Bursty interference handling

- Proposed aspects for investigation:

  - Identify specific use cases,

    - e.g. burst interference modeling, data rate, etc.

  - Identify outer code candidates

  - Performance, latency and complexity

  - Possible HARQ enhancement with outer coding

    - Associated signaling/feedback overhead in UL and DL

Decision: The document is noted.

Not treated.

R1-162160	Motivation of 5G New FEC	Huawei, HiSilicon

R1-162161	Overview of Polar Code	Huawei, HiSilicon

R1-162208	Requirements and evaluation	Qualcomm Inc.

R1-162209	LDPC - HARQ, rate	Qualcomm Inc.

R1-162210	LDPC - throughput latency block size	Qualcomm Inc.

R1-162211	Polar - HARQ, rate	Qualcomm Inc.

R1-162212	Polar - throughput latency block size	Qualcomm Inc.

R1-162213	TBCC - HARQ, rate	Qualcomm Inc.

R1-162214	TBCC - throughput latency block size	Qualcomm Inc.

R1-162309	Channel coding for the 5G new radio	CATT

R1-162387	Channel coding for new radio interface	Intel Corporation

R1-162520	Discussion on channel coding for NR	LG Electronics

R1-162798	5G High Throughput and Low Power Consumption Codec	MediaTek Inc.

R1-162897	Performance and complexity of Turbo, LDPC and Polar codes	Nokia, Alcatel-Lucent Shanghai Bell

R1-162898	Channel coding for URLLC	Nokia, Alcatel-Lucent Shanghai Bell

R1-162927	Design considerations on channel coding with very large blocklength	INTERDIGITAL COMMUNICATIONS

R1-162928	Design considerations on channel coding with short blocklength	INTERDIGITAL COMMUNICATIONS

R1-163114	Channel Coding for NR	NTT DOCOMO, INC.

R1-163231	On channel coding for NR	Ericsson

#### Modulation

R1-162163	Applying Multi-Dimensional Modulation to Non-Orthogonal Multiple Access	Huawei, HiSilicon

The document was presented by Wang YI from Huawei.

- Observation 1: Multi-dimensional modulation provides significant spectral efficiency improvement compared to linear spreading, especially when higher-order modulation is used.

- Observation 2: Multi-dimensional modulation provides lower detection complexity by allowing lower number of projection points compared to linear spreading.

- Proposal: Further consider multi-dimensional modulation for non-orthogonal multiple access scenarios in 5G.

Decision: The document is noted.

R1-162521	Discussion on modulation schemes for NR	LG Electronics

- Suggestion 1: M-QAM (i.e. QPSK/16QAM/64QAM/256QAM) defined in LTE/LTE-A should be considered as a baseline modulation scheme for NR study when NR system is designed under OFDM waveform.

- Suggestion 2: Study on proper modulation order according to frequency bands (i.e. below 6GHz and above 6GHz) and implementation conditions (e.g. EVM, geometry, etc.) would be necessary for NR.

Decision: The document is noted.

R1-162923	Discussion on Modulation for New Radio systems	INTERDIGITAL COMMUNICATIONS

- Proposal 1: Advanced modulation schemes need to be evaluated for eMBB.

- Proposal 2: Consider non-coherent modulation for the ultra-reliable mode of operation.

- Proposal 3: The impact of interference on modulation need to be taken into account in case the non-orthogonal multiple access was agreed for mMTC.

- Proposal 4: The design of modulation scheme for mMTC should strive for high energy efficiency.

Decision: The document is noted.

Not treated.

R1-162215	NR modulation requirements	Qualcomm Inc.

R1-162216	Candidate NR modulation schemes	Qualcomm Inc.

R1-162899	Digital Modulation for 5G New Radio	Nokia, Alcatel-Lucent Shanghai Bell

R1-163233	On modulation schemes for NR	Ericsson

### Other

Not treated.

R1-162164	Considerations on New Radio Operation for Ultra Dense Networks	Huawei, HiSilicon

R1-162165	Motivations and Considerations on Massive MIMO for New RAT	Huawei, HiSilicon

R1-162166	Beam based Access for 5G M-MIMO	Huawei, HiSilicon

R1-162167	Considerations on vehicular communications for 5G	Huawei, HiSilicon

R1-162168	Support for flexible backhauling for 5G	Huawei, HiSilicon

R1-162169	Support for energy efficient 5G	Huawei, HiSilicon

R1-162170	Discussions on the frequency bands above 6GHz	Huawei, HiSilicon

R1-162182	MIMO for 5G new radio interface: sub-6GHz	Samsung

R1-162183	MIMO/beamforming for 5G new radio interface for >6GHz: system architecture and design aspects	Samsung

R1-162184	Overall system operation and network topology for 5G new radio interface	Samsung

R1-162185	Interference coordination for 5G new radio interface	Samsung

R1-162186	Co-existence of LTE and 5G new radio interface	Samsung

R1-162187	Wireless relay at mmWave frequency for 5G new radio interface	Samsung

R1-162188	Shared spectrum support for 5G new radio interface	Samsung

R1-162189	Ultra-reliability with low-latency support in 5G new radio interface	Samsung

R1-162217	Sidelink design for NR	Qualcomm Inc.

R1-162249	Underlay Common Control Channel for Unlicensed Spectrum Access and Underlay Low-Rate Data Channel for Licensed Spectrum	Idaho National Laboratory

R1-162582	Proposal for relaying capability	KDDI Corporation

R1-162900	On Hybrid ARQ functionality for 5G New Radio	Nokia, Alcatel-Lucent Shanghai Bell

R1-162901	MIMO and Beamforming for 5G New Radio	Nokia, Alcatel-Lucent Shanghai Bell

R1-162924	Discussion on Massive MIMO for New Radio systems	INTERDIGITAL COMMUNICATIONS

R1-162926	Design considerations on URLLC for New Radio systems	INTERDIGITAL COMMUNICATIONS

R1-163234	Downlink control signaling in NR	Ericsson

R1-163235	Instant uplink access	Ericsson

R1-163236	Transmission principles for data channels	Ericsson

R1-163237	Initial considerations on system access in NR	Ericsson

R1-163238	Unified CSI framework in NR	Ericsson

R1-163239	On the use of channel reciprocity in NR	Ericsson

R1-163240	On UE side beamforming in NR	Ericsson

R1-163241	Simulation results for a large number of antennas	Ericsson

R1-163242	Flexibility and performance of self contained transmissions	Ericsson

R1-163243	Supporting unlicensed spectrum in NR	Ericsson

R1-163244	Characteristics of NR impacting operation in unlicensed spectrum	Ericsson

R1-163621	On TTI	Ericsson

R1-163622	On subframes	Ericsson

## Study on channel model for frequency spectrum above 6 GHz - SID in RP-151606

R1-162191	Chairman’s Notes of Agenda Item 8.2 on Channel model for >6 GHz	Nokia

The document was presented by Tuomo Säynäjäkangas from Nokia and provides the outcomes of AI8.2.

Decision: The document isendorsed. Contents incorporated below.

Offline session notes

R1-163482	Minutes of offline session on Tuesday		Nokia

Decision: The document is noted

TR update

R1-162710	TP for TR38.900 Section 7.1 - coordinate systems	Samsung, Nokia

Decision: The document is agreed

R1-162711	TP for TR38.900 Section 7.2 and 7.3 - scenarios and antenna modeling	Samsung, Nokia

Discussion:

ZTE: For indoor office and shopping mall, do you assume multi floor case?

Samsung: No

ZTE: We need many floors for the calibration. Are we going to increase the number of floors in the future?

Samsung: It can be discussed in the future. This TP is for scenarios and antenna modelling.

Decision: The document is agreed

R1-162712	TP for TR38.900 Section 7.4 - pathloss, LOS, O-I modeling		Samsung, Nokia

Discussion:

Ericsson: One issue on LOS PL discontinuities need to be captured.

Samsung: This is based on working assumption from last meeting. Further changes can be made during this meeting if needed.

ZTE: 7.4.1.1 note says the central frequency. PL f is not going to be fc if BW is large.

Samsung: That is a typo.

Decision: The document is revised in 3468.

R1-163468	TP for TR38.900 Section 7.4 - pathloss, LOS, O-I modeling		Samsung, Nokia

Decision: The document is agreed

R1-162713	TP for TR38.900 Section 7.5 - fast fading		Samsung, Nokia

Discussion:

Ericsson: LSP has strong frequency dependencies. We are not comfortable with that.

Samsung: This is based on working assumption from last meeting. Changes can be reflected in the next version.

Ericsson: We prefer brackets instead.

Nokia: We agree with Samsung. This is based on working assumption. Of course that need to be confirmed.

Decision: The document is agreed

R1-162714	TP for TR38.900 Section 7.6 - additional modeling components	Samsung, Nokia

Discussion:

ZTE: Oxygen loss offset need to be clarified. Is it LOS distance? Is it deterministic or random?

Samsung: TP is based on agreements from last meeting.

Nokia: Calculation in Keysight document is not entirely correct.

CATT: Examples are not clear.

Huawei: We also found some errors in Keysight calculations. We could take this offline to find the correct equation.

Nokia: We should agree this now capturing agreements from last meeting.

Samsung: We could fix the oxygen error

Decision: The document is revised in 3469.

R1-163469	TP for TR38.900 Section 7.6 - additional modeling components	Samsung, Nokia

Decision: The document is agreed

R1-162715	TP for TR38.900 Section 7.7 and 7.8 - Link-level channel model and calibration	Samsung, Nokia

Decision: The document is revised in 3470.

R1-163470	TP for TR38.900 Section 7.7 and 7.8 - Link-level channel model and calibration	Samsung, Nokia

Decision: The document is agreed

R1-163396	Correction in “Large Bandwidth and Large Antenna Array” section of R1-162714	Keysight

Mathematical error was found in the new equation to cope “Large Bandwidth and Large Antenna Array” in [1]. The correction to the problem is proposed in section 3.

Please, note that the “additional model component” for large bandwidth and large antenna arrays defined in [1] does not solve issues for spherical wave modelling and non–WSS propagation across antenna elements.

Chair: Tdoc was requested late

Decision: The document is noted

R1-163488	TP for TR38.900		Samsung, Nokia

Discussion:

Huawei: We are OK but comment on fast fading model where distance is km. We could unify the units.

Samsung: WE can correct for the next version

Decision: The document is agreed.

### Remaining details on large scale modelling for >6GHz

Remaining details on LOS probability, pathloss (including penetration loss) and shadow fading

LOS probability

R1-162522	Remaining details on LOS probability in indoor open office scenario	LG Electronics

Proposal 1: For indoor open office scenario, adopt table 3 for parameterizations.

Discussion:

Decision: The document is not treated

R1-162716	LoS probability model for indoor open-office scenario	Samsung

Proposal 1: Adopt the proposed LoS probability model for indoor open-office scenario:

PLoS=1,                                                   d≤3e-d-314.6,                                    3<d≤21e-d-2169.2*0.15,                           d>21

Discussion:

Decision: The document is not treated

R1-162241	LOS probability for indoor open office	Huawei, HiSilicon, AT&T, CMCC, Ericsson, Intel, KT Corporation, Nokia, NTT DOCOMO, Qualcomm, Samsung

Observation 1: considering the continuity of the model, the LOS probability model for open office agreed in R1-161727 is of the same form as the LOS probability for mixed office agreed in R1-161640.

Proposal 1: the LOS probability model for indoor open office should be:

Discussion:

Decision: The document is agreed

Penetration loss

R1-163097	Additional Considerations on Building Penetration Loss Modeling for 5G System Performance Evaluation	Straight Path Communications

Proposal: For building penetration loss modelling, the low-loss model shall be used with 80% probability, the high-loss model shall be used with 10% probability, and a medial loss model (the average of the high-loss and the low-loss model) shall be used for 10% of the probability, with the low-loss model and the high-loss model as defined in R1-161687.

Discussion:

Decision: The document is revised in 3408.

R1-163408	Additional Considerations on Building Penetration Loss Modeling for 5G System Performance Evaluation	Straight Path Communications

Proposal 1: The low loss model should be used with 80% probability, the high loss model should be used with 10% probability, and a medial loss model (the average of the high-loss and the low-loss model) should be used for 10% of the probability.

Proposal 2: The low loss model should be used with 80% probability and the high loss model should be used with 20% probability for Urban Micro; the low loss model should be used with 90% probability and the high loss model should be used with 10% probability for Urban Macro.

We request RAN1 to select one of these two proposals for New Radio (NR) system performance evaluation purposes.

Discussion:

Decision: The document is not treated

R1-163255	Removing square brackets in the O2I penetration loss modeling	Ericsson, Nokia, NTT DOCOMO, Samsung

Proposal 1: Use a value of 5 dB for compensating the path loss through the external wall for non-perpendicular incidence angles

Discussion:

Decision: The document is agreed

R1-163256	Removing square brackets in the O2I penetration loss modeling, part 2	Ericsson, NTT DOCOMO, Samsung

Proposal: Use a standard deviation of 7 dB to represent the variation of the building penetration loss

Discussion:

Decision: The document is agreed

R1-163485	WF on O2I LSPs	Ericsson, Huawei

Discussion:

Decision: The document is agreed

Path loss modeling

Chair: See also late LS from NYU Wireless in R1-163395 with following findings:

Results show the CI model with a 1-m reference distance is suitable for outdoor environments, while the CIF model is more appropriate for indoor channel modeling. The CI and CIF models are easily implemented in existing 3GPP models by making a very subtle modification—by replacing a floating non-physically based constant with a frequency-dependent constant that represents free-space path loss in the first meter of propagation. This paper shows this subtle change offers better model parameter stability, better accuracy in sensitivity tests over a vast range of microwave and mmWave frequencies, scenarios, and distances, is  easier to use in analysis, and has intuitive appeal while using a simpler model with fewer parameters.

R1-163258	Shortcomings of the proposed LOS path loss models	Ericsson

Observation 1: The new LOS path loss models suffer from strong discontinuities at the break point.

Observation 2: The 3D SCM LOS path loss models can represent also the new measurements well due to the similarity of the parameters

Proposal: Adopt the 3D SCM 36.873 LOS path loss models for use in the UMa and UMi scenarios.

Discussion:

NTT: Does this propose using 3D CM for 6-100 GHz?

Ericsson: Yes.

NTT: We agree with discontinuity problem but higher bands shall be verified also.

Ericsson: Both models have the same frequency dependence.

NTT DOCOMO: We should indicate actual estimation accuracy.

ZTE: If we change the PL model how about SF then?

Ericsson: We have inconsistency also with SF.

Huawei: We have a late contribution on how to solve this.

Decision: The document is noted

R1-162231	Comparison of CIF vs. ABG model for pathloss modeling	Qualcomm Inc.

Proposal: While the CIF and ABG models lead to comparable fits in modeling the path loss, the ABG model is poorer than the CIF model on two counts: i) more parameters are used in the ABG model than the CIF model, ii) the ABG model is more unstable than the CIF model in predictive power. Thus, the CIF model should be used for capturing path loss in >6 GHz channel modeling.

Discussion:

Decision: The document is not treated

R1-162310	Discussion on NLOS path loss modeling	CATT

Proposal: For NLOS path loss channel modelling, the close-in model may be preferable for its physics basis.

Discussion:

Decision: The document is not treated

R1-162570	Comparing ABG and CI pathloss model	Nokia, Qualcomm, Alcatel-Lucent Shanghai Bell

Observation-1: The close-in (CI) reference distance path loss model does a significantly better job at predicting path loss performance in frequency, distance, and across environments.

Observation-2: The CI path loss model has a much more stable parameterization than the alpha-beta-gamma (ABG) path loss model when determining the parameters on different sets of distances, frequencies, and environments.

Observation-3: A linear modeling of the increase or decrease with the log of frequency does not accurately model real world conditions.

Proposal: Adopt the CI path loss model for both LOS and NLOS conditions for at least the outdoor channels.

Discussion:

AT&T: Conclusion says outdoor only?

Nokia: We mean at least for outdoor. It should be sufficient also for indoor.

NTT DOCOMO: We should use ABG model.

Qualcomm: We have analysed also indoor case in our document.

Ericsson: You say linear modeling is invalid but also CI model in linear. It is actually worse than ABG model. It does not model the reality accurately.

Nokia: Our observation is based on the measurement data. Prediction capability is important aspect.

AT&T: We could go with CI for LOS and ABG for NLOS cases.

Nokia: That is a good point. Both models have their merits. We need to make progress in this meeting.

Huawei: We also have similar thinking on usage cases.

ZTE: For LOS either one is fine but for NLOS case the ABG is better.

Nokia: We support CI as we don’t have enough measurement data. Gamma value is not stable.

Decision: The document is noted

R1-162571	System level simulation using ABG and CI modelling	Nokia, Alcatel-Lucent Shanghai Bell

Decision: The document is withdrawn

R1-162425	Path loss model for indoor environment	NTT DOCOMO, NTT

Proposal: If ABG model is used for indoor NLOS case, single slope model is used.

Discussion:

Decision: The document is not treated

R1-162717	Discussion on pathloss modeling for UMi NLoS environment	Samsung

Proposal: Keep the ABG formula used in ITU and 3GPP pathloss model for the consistent modeling, and there is no significant reason to change the formula from ABG model to CI model

Discussion:

Decision: The document is not treated

R1-162989	38GHz Pathloss Model for UMi Scenario Street Canyon Scenario	ITRI, MediaTek Inc.

Proposal 1: For LOS case, the working assumption model [1] is quite close to our 38GHz data fitted model.

Proposal 2: For NLOS case, the ABG model can provide better modeling accuracy than CI model.

Proposal 3: The measurement results and fitted parameters can be captured in the TR.

Discussion:

Decision: The document is not treated

R1-162247	Path loss modeling for NLOS propagation		Huawei, HiSilicon, Ericsson, NTT DOCOMO, Samsung

Proposal 1: for NLOS propagation case, the pathloss model in Table 6 should be adopted as working assumption.

Proposal 2: The inclusion of path loss dependency on the UE and eNB height similar to that in the 3D SCM model should be further considered

Proposal 3: The consistency with the 3D SCM model for below 6 GHz should be further considered

Discussion:

Nokia: We found ABG model with small distance better than free space PL. What is the explanation on that?

Ericsson: Behaviour is the same also for CI model.

Huawei: We propose the dual slope for indoor.

NTT DOCOMO: Does table 6 propose a frequency and distance range? Do you capture the range in a TR?

Huawei: These are based on measurements. We plan to capture in a TR.

Ericsson: Proposal 1 may be controversial. Can we agree on proposals 2 and 3?

Nokia: Consistency is important but could be studied in future work items.

Decision: The document is noted

R1-163428	Amendment for LOS pathloss working assumption	Huawei, HiSilicon

Chair: Late contribution

Proposal: the LOS pathloss in table can be agreed.

Discussion:

Ericsson: Original 3D model distance dependency is consistent with physics. This is not a physical behaviour.

Decision: The document is noted

R1-163471	WF on NLOS pathloss for UMa, Umi-Street Canyon and Indoor-office	Nokia

Discussion:

Chair: In offline discussion the status was as follows

Alt.1 is objected by Huawei

Alt.2 is objected by Ericsson, Qualcomm

Alt.3 is objected by Nokia, Qualcomm

Huawei: We also prepared a WF. We could ask supporting companies.

Nokia: We need to find a compromise solution so we prefer to ask objecting companies.

Huawei: We agree we need to find a compromise. That’s why we proposed another way.

NTT DOCOMO: We may be able to agree step by step with different scenarios.

Qualcomm: We are OK to work with alternative 2 as well.

Ericsson: Alt 2 UMa model is CI. This is incomplete proposal regarding UE height. We are happier with Alt 1.

Nokia: ABG model do not have UE height dependency either. We could make Alt 2 as working assumption. We could confirm that on May meeting.

Ericsson was not OK at this point of time.

Samsung: We need to start the calibration in this meeting. PL issues shall be agreed in this meeting. We encourage companies to compromise.

Nokia: Alt 2 as working assumption. UE height to be clarified in the next meeting.

Ericsson: We could say the same also for Alt 1.

Nokia can live also with Alt 1.

Qualcomm: We should down select and make the progress.

Huawei: We have 9 supporting companies for Alt 3. That is majority view from our point.

Nokia: Huawei’s WF is already captured but this is consensus based group.

Huawei: We should check the least restrictive alternative.

Ericsson: We could start with alternative with majority support which is Alt 3.

Nokia: That is not a compromise.

CATT: We could take 873 3D CM approach as a working assumption.

Nokia: How should we do work based on that?

Ericsson: FFS would be adapting parameters with new measurements.

Nokia: We can further compromise. Alt 1 with indoor scenario with both ABG and CI models.

Huawei: That is confusing. We should treat all scenarios in a similar way together.

Samsung: We are not treating scenarios in a different way.

Decision: The document is revised in 3486.

R1-163486	WF on NLOS pathloss for UMa, Umi-Street Canyon and Indoor-office	Nokia

Discussion:

Decision: The document is revised in 3637.

R1-163637	WF on NLOS pathloss for UMa, Umi-Street Canyon and Indoor-office	Nokia, Samsung, CATT, Ericsson, NTT Docomo, Intel, Qualcomm, ZTE

Discussion:

ZTE: 1st bullet has FFS. Models in 2nd bullet can be changed.

Nokia: Yes

Decision: The document is agreed

R1-163483	WF on LOS pathloss model	Huawei, HiSilicon

Discussion:

Nokia: You propose 3D UMa for LOS. What is your consideration on >6GHz

Huawei: We believe the information based on measurements is useful

Nokia: Can we agree this together with our WF?

Huawei: This tries to solve the working assumption. These are separate issues.

Ericsson: We propose to re-use 3D CM for Uma and UMi LOS. This is different approach.

Huawei: We have introduce lot of new measurements >6GHz.

Nokia: We did not compare the LOS case for 3D CM applicability for >6GHz.

Ericsson: We did compare the LOS case with 3D CM with CI model with new measurements and the match is pretty good.

Decision: The document is noted

R1-163484	WF on NLOS pathloss model	Huawei, HiSilicon, Ericsson, ETRI, Fraunhofer HHI, Keysight, LGE, Samsung, Spirent

For NLOS pathloss

  - For UMi-Street Canyon, adopt ABG model

  - For Indoor-office, adopt ABG single slope model

  - For UMa, adopt ABG model

Discussion:

Nokia: We cannot accept this which is actually alt 3 in our WF.

Huawei: It is and preferred way for sourcing companies.

NTT DOCOMO: WE can support this WF.

Decision: The document is noted

Rural Macro model

R1-163640	WF on RMa model	Ericsson

Adopt the ITU-R M.2135 Rural Macro model for rural macrocell simulations

Enhance the RMa model to 3D using elevation parameters taken from the 5G UMa model

Tuning of the elevation parameters is FFS

For example, the elevation angle spread is expected to be lower in rural scenarios

Capture the model in the TR by reference to M.2135 + a table with the additional parameters

Put square brackets around the ”UMa” parameters pending further tuning

Discussion:

Samsung: This is good starting point. This seems to be out of scope as CM SI scope is for >6GHz.

Nokia: Do we really have to extend it to 3D? If not we could use ITU CM.

Samsung: ZoD distribution is going to be challenging but there are some merits on this. We can utilize elevation beamforming. It would be good to assume 3D CM for RMa.

Nokia: Antenna heights are higher for rural deployment. We cannot simply re-use 3D CM parameters for RMa.

Ericsson: WE agree. We need to tune them. Antenna height is 35 m. It is small effort compared to UMa 25 m.

Samsung: We can discuss this in the main NR session as this is out of the scope of CM SI.

Ericsson: 5G evaluation need CM also for RMa. Who does CM for <6GHz if we don’t?

Nokia: We are neutral. It would be good to include but we have already agreed 5 scenario for the SI.

Sharp: We agree with Nokia but also sympathize the Ericsson proposal. CM is needed ASAP.

Ericsson: Doing this in NR session instead then we do we document this?

Huawei: Would it be possible to postpone to decision for the next meeting. We need to understand operatos positions better.

Nokia: If we need to extend the scope that need to be decided in plenary.

Samsung: SI is for >6GHz. Rural is for <6GHz.

Decision: The document is noted.

Friday

R1-163901	WF on LOS pathloss model	Huawei, HiSilicon, Ericsson, Nokia, NTT DOCOMO, NTT, Samsung, Qualcomm

Decision: The document is agreed.

R1-163908	WF on NLOS path loss model and UE dropping for calibration	Ericsson, Huawei, HiSilicon, Nokia, Samsung, NTT DOCOMO, NTT, ZTE, Qualcomm, LGE

Decision: The document is agreed.

R1-163909	WF on the Rural macro scenario	Ericsson, Vodafone, Samsung, Nokia,

Decision: The document is noted. Also supported by AT&T, Deutsche Telekom, Sprint.

Agreements:

For the Rural macrocell (RMa) scenario as described in TR 38.913 [6], agree on the RMa channel model parameters in Appendix A in R1-163909 as a working assumption

- This model is valid up to 7 GHz

- TR38.900 to capture this channel model

  - RMa scenario description is also captured in the TR.

### Remaining details on fast fading modelling for >6 GHz

Remaining details on large scale, small scale parameters, channel generation steps, etc.

Measurements

R1-163252	New InH measurements of the frequency dependence of delay spread	Ericsson, Telstra

Proposal: Delay spread modeling in the InH scenario should be frequency independent

Discussion:

Decision: The document is not treated

R1-163257	Requirements for Comparability of Radio Channel Measurements	Ericsson

Proposal: When comparing different measurement data at different frequencies and/or from different campaigns the following requirements should be fulfilled:

- The same measurement bandwidth should be used

- Equal antenna pattern should be used (may also be synthesized)

- Equal dynamic range should be used, both in delay and angle domains

- When the channel is sampled in space the same volume in terms of number of wavelengths should be used

- The environment should be equal i.e. same location and same variability in terms of moving people and vehicles etc.

- Locations of antennas should be the same

- The oxygen absorption at 60 GHz should be compensated for using propagation path lengths as agreed in [1]. This is important for securing smooth frequency properties when extrapolating/interpolation using 60 GHz.

Discussion:

Decision: The document is not treated

Channel generation

R1-162576	Joint proposal on fast fading channel generations	Samsung, AT&T, CMCC, ETRI, Huawei, HiSilicon, Intel, KT Corporation, Nokia, NTT DOCOMO, Qualcomm

Proposal 1: Adopt the full detailed method of channel generation steps in Section 7.3 of TR36.873 at least in the following cases:

  - Case 1: For low complexity evaluations

  - Case 2: To compare with earlier simulation results,

  - Case 3: When none of the additional modeling components are turned on.

  - Other cases are not precluded

Proposal 2: Modification of the steps in the other cases can be further studied.

Discussion:

Decision: The document is agreed

R1-162954	Text proposal to TR 38.900 to add channel coefficient generation procedures	Huawei, HiSilicon

This contribution proposes some content to the 12 steps in section-2 in TR36.873.

Discussion:

CATT: Changing BS and UE to TX and RX that will impact on many other places as well. Maybe better to leave as is.

Intel agree with CATT.

ZTE: We support this change. CATT comment is editorial issue which can be considered later if needed.

Ericsson: Intention is good but there is a problem. BS could be TX and RX. We have different parameters in BS and UE side.

Huawei: We can clarify if requirement is for UL or DL but we need to consider also UE to UE aspects.

Nokia: In general we support the change but we also agree with Ericsson. We can agree this in later phase.

Decision: The document is noted

R1-162957	Text proposal to TR 38.900 to add dual mobility Doppler	Huawei, HiSilicon

This contribution proposes some content to the 12 steps in section-2 in TR36.873.

Discussion:

Samsung: It is not clear when should we apply this. More considerations are needed. We can clarify this later.

Decision: The document is noted

Large scale parameters

R1-162311	Analysis on measurement data and proposed modeling	CATT

Proposal 1: The azimuth and zenith angular spread of arrival may have the same tendency corresponding to increasing frequency.

Proposal 2: More data should be used to get the proper values of α and β for the fitting equation α101+f+β.

Proposal 3: Since 30GHz and 70GHz could be target frequency for system simulation, at least the parameters for these two frequencies should match the measured data.

Discussion:

Decision: The document is not treated

R1-162572	Refinement on UMa LSP		Nokia, Samsung, AT&T, CMCC, Ericsson, Huawei, HiSilicon, Intel, KT Corporation, NTT DOCOMO, Qualcomm

Proposal: For UMa, adopt the large-scale parameters from [2] with Tables 5 and 12 replaced with the ones from this contribution.

Discussion:

Decision: The document is agreed

R1-162719	Discussion on large-scale parameters in indoor office environment	Samsung

Proposal: For indoor office scenario,

- Adopt the LS parameters in Table 2.

- Adopt the LS cross-correlation parameters from IMT-Advanced channel model in ITU-R M.2135.

Discussion:

Ericsson: Table 2 summary summaries only some frequency bands. We have considered constant values in other scenarios. Have you compensated oxygen loss in 60 GHz?

Samsung: This table is for information. We are not considering frequency dependency. No compensation for oxygen loss.

Huawei: We could capture this data in the future.

Decision: The document is noted

R1-162956	Discussion on the sum-of-sinusoids based LSPs	Huawei, HiSilicon

Observation: Sum-of-sinusoid method provides spatially consistent LSPs such as shadowing with reasonable complexity.

Discussion:

Decision: The document is not treated

R1-162232	Indoor and outdoor measurements on frequency-dependent delay spreads	Qualcomm Inc.

Proposal: Frequency dependent models for large scale parameter(s) appear to be intuitive and measurement results support this intuition.  In particular, frequency dependent delay spread models should be considered for >6 GHz channel modeling.

Discussion:

Decision: The document is not treated

R1-163253	On the frequency dependence of LSPs in reported UMi and InH measurements	Ericsson

Proposal: Use a functional form for frequency dependence of LSPs and SSPs containing a constant term and a frequency-dependent term of the following form: LSP=a+b⋅function(f), where the strength of the frequency-dependent term is FFS

Discussion:

Decision: The document is not treated

R1-163254	On the frequency dependence of LSPs	Ericsson

Proposal: Use a functional form for frequency dependence of LSPs and SSPs containing a constant term and a frequency-dependent term of the following form: LSP=a+b⋅function(f), where the strength of the frequency-dependent term is FFS

Discussion:

Qualcomm: We have done measurement and we have seen frequency dependency. More analysis is still needed.

ZTE: Do LSP refer to one parameter or two parameters?

Ericsson: We should be careful while specifying frequency dependency. We have separate contribution on measurements in R1-163257.

CATT: Our measurement campaign in indoor scenarios observe frequency dependency

Decision: The document is noted

Small scale parameters

R1-162243	Discussion on frequency dependent SSP	Huawei, HiSilicon

Proposal 1: for indoor office, frequency dependent cluster ASA and cluster ZAS should be modelled.

Discussion:

Ericsson: We cannot find the info on used antenna.

Huawei: Wide beam for RX and rotation 10 degree horn antenna for TX.

Decision: The document is noted

Large and small scale parameters

R1-162242	Large-scale and small-scale channel modeling for outdoor to indoor scenario	Huawei, HiSilicon

Proposal1: the O-to-I large-scale and small-scale parameters should be modified due to new measurement above 6GHz.

Proposal2: the O-to-I model parameters could be adopted as defined from Table 2 to Table 4 for UMi and UMa scenarios.

Discussion:

Decision: The document is not treated

R1-162246	Large-scale and small-scale channel modeling for indoor hotspot	Huawei, HiSilicon, AT&T, CMCC, Ericsson, Intel, KT Corporation, Nokia, NTT DOCOMO, Qualcomm, Samsung, ETRI

Proposal 1:.for indoor office, the recommended LSP from Table 1 to Table 4 should be adopted for large-scale channel modeling of this SI.

Proposal 2:.for indoor office, the recommended SSP in Table 5 and Table 6 should be adopted for cluster-level small–scale channel modeling of this SI.

Discussion:

Decision: The document is revised in 3427.

R1-163427	Large-scale and small-scale channel modeling for indoor hotspot	Huawei, HiSilicon, AT&T, CMCC, Ericsson, Intel, KT Corporation, Nokia, NTT DOCOMO, Qualcomm, Samsung, ETRI

Proposal 1:.for indoor office, the correlation distance and cross correlation of LSP in IMT-A can be reused for azimuth domain channel characteristics. The recommended LSP from Table 1 to Table 4 should be adopted for large-scale channel modeling of this SI.

Proposal 2:.for indoor office, the number of rays per cluster and per cluster shadowing in IMT-A can be reused. The recommended SSP in Table 5 and Table 6 should be adopted for cluster-level small–scale channel modeling of this SI.

Discussion:

ZTE: Table 5 and 6 mention number of clusters. We think 4 is small number. Table 3 => 3rd number from the bottom gives a negative value.

CATT: Number of clusters are small. Listed results are not complete. Table 1 frequency dependency requires further studies.

Huawei: We can update frequency dependency in the future. Values as in brackets.

Ericsson: Number of clusters could also be in brackets.

Decision: The document is revised in 3472.

R1-163472	Large-scale and small-scale channel modeling for indoor hotspot	Huawei, HiSilicon, AT&T, CMCC, Ericsson, Intel, KT Corporation, Nokia, NTT DOCOMO, Qualcomm, Samsung, ETRI, CATT

Proposal 1:.for indoor office, the correlation distance and cross correlation of LSP in IMT-A can be reused for azimuth domain channel characteristics. The recommended LSP from Table 1 to Table 4 should be adopted for large-scale channel modeling of this SI.

Proposal 2:.for indoor office, the number of rays per cluster and per cluster shadowing in IMT-A can be reused. The recommended SSP in Table 5 and Table 6 should be adopted for cluster-level small–scale channel modeling of this SI.

Discussion:

ZTE: You changed to fixed number which is not frequency dependent at all.

Decision: The document is agreed.

R1-162718	Updated joint proposal on large-scale and small-scale parameters for UMi scenario	Samsung, AT&T, CMCC, Ericsson, Huawei, HiSilicon, Intel, KT Corporation, Nokia Networks, NTT DOCOMO, Qualcomm, ETRI

Proposal 1: For UMi Street canyon, adopt the updated delay spread model in Table 1 and the updated ZoD model in Table 2 to replace the proposal in [1]

Proposal 2: The current K-factor parameters in 3D-SCM UMi can be reused for UMi Street canyon scenario.

Proposal 3: For UMi Street canyon, adopt the correlation distance parameters in Table 3

Proposal 4: For UMi Street canyon, adopt the cross-correlation parameters in Table 4, which parameters are taken from 3GPP 3D-SCM UMi scenario

Discussion:

ZTE: Cross correlation table, is it assumption or observation?

Samsung: Correlations are analysed in Annex. There is no strong frequency dependency.

Decision: The document is agreed.

R1-163251	Modelling of LSPs and SSPs for the outdoor to indoor propagation	Ericsson, Nokia Networks, AT&T, CMCC, ETRI, Huawei, HiSilicon, Intel, KT Corporation, NTT DOCOMO, Qualcomm, Samsung

Proposal 1: Adopt the first or the second approach for the LSP and SSP parameters for the outdoor to indoor links in the UMa and UMi scenarios

Proposal 2: Consider further modifications of the LSPs if additional O2I measurement data is made available

Discussion:

Decision: The document is agreed

Clustering

R1-162233	Clustering methodology and results for indoor and outdoor scenarios	Qualcomm Inc.

Proposal 1: The design of good beamforming protocols differ significantly depending on whether a small/large number of clusters capture the multi-antenna channel. Reusing 36.873 parameters for number of clusters should be done with precaution.

Proposal 2: Mixing ray tracing data with measurements in determining cluster statistics is not a good idea.

Proposal 3: Given the extensive measurements on azimuthal and spherical scans performed at Qualcomm at millimeter wave frequencies, Qualcomm’s data on indoor office and shopping mall should be included in further studies that lead to development of cluster statistics for > 6 GHz channels.

Discussion:

Decision: The document is not treated

R1-162994	Implementation of cluster delay spread	NTT DOCOMO, NTT

Proposals 1: The Alternative 1 and 2 can be used for the case of channel bandwidth is narrow with low complexity of implementation

Proposals 2: The Alternative 3 can be beneficial for various channel bandwidths with low complexity of implementation.

Proposals 3: The Alternative 4 should be used for the case of requirement for high accuracy of channel with various bandwidths.

Discussion:

Decision: The document is not treated

R1-163115	A hierarchical agglomerative clustering algorithm for channel modelling	Intel Corporation, Ericsson, Nokia Networks, AT&T, CMCC, ETRI, Huawei, HiSilicon, KT Corporation, NTT DOCOMO, Qualcomm, Samsung, University of Southern California

This contribution illustrates the merits of the HAC algorithm:

- It jointly processes delay, angle and space information for clustering MPCs.

- It is able to find compact, yet not necessarily convex, clusters in delay-angular domain and it does so without excluding the presence of outliers.

- Low sensitivity with respect to the resolution of the input data, thus ensuring consistent results across measurements taken in the same environment but with different resolutions.

- It produces spatially consistent clusters.

- It allows to cluster MPCs disregarding on whether these are fully specified both in angle and delay domain.

Discussion:

Decision: The document is agreed

Time variation

R1-163246	Channel modeling with time-varying delays and angles	Ericsson

Proposal: Modify the equivalent of equation (7.21) in [2] in the new model according to the suggestion in section 3.

Discussion:

Decision: The document is not treated

R1-163636	WF on modeling of time-varying angles	Ericsson

Proposal 1: Replace the factor expj2πυn,m⋅tin the channel coefficient equation

Proposal 2: Proposal 1 needs only to be considered in case the Doppler frequencies are time-varying. This could be due to:

Non-linear UE movement

Time-varying angles of arrival due to spatial consistency modeling

Discussion:

Sharp: Does this work for wideband scenarios?

Ericsson: We are not sure actually. Doppler could change over the BW.

Intel: What mobility model is used? Anchor point could be related to phase discontinuity.

Ericsson: Simulations were done with 873 implementation. We should not get phase discontinuity.

Samsung: Which part in the current TR you want to change?

Ericsson: That is up to rapporteur.

Intel: UE moving around the circle the values change.

Nokia: Updating anchor point for non-linear movement. How often to update anchor point?

Ericsson: There is a criteria for that.

ZTE: When to use this?

Ericsson: Whenever the Doppler frequencies are not constant.

Huawei: Title should be time varying Doppler. Which frequency was assumed? We have concerns on non-linear movement.

Ericsson: Title can be changed. Non-linear movement is only one example.

LGE: Are you suggesting this to drop based model?

Huawei: More time is needed to check results.

Decision: The document is noted.

### Remaining details on modelling of additional features for >6GHz channel model

#### Remaining details on blockage modelling

R1-162234	TP on Dynamic Blockage Modeling	Qualcomm Inc.

Propose (TP) the parameters for modeling dynamic blockage according to a stochastic model.

Discussion:

Intel: Basically this is similar view than ours except some parts.

CATT: LOS and NLOS states to be considered. Blocking should not be lower than NLOS.

Interdigital. We agree with this except some parameters.

Samsung: We need further discussion in this area.

Decision: The document is noted.

R1-162312	Clarification on blocking model	CATT

Decision: The document is withdrawn

R1-162573	Remaining details on blockage modelling	Nokia, Alcatel-Lucent Shanghai Bell

Discussion:

Decision: The document is revised in 3381.

R1-163381	Remaining details on blockage modelling		Nokia, Alcatel-Lucent Shanghai Bell

Proposal-1: Consider K+1 blockers for each user where K blockers to mimic the blockage effect due to moving objectives around the UE and one blocker to mimic the blockage effect due to very close range of obstacles to the UE (self-blockage)

Proposal-2: The one blocker for self-blockage doesn’t need to be consistent in spatial domain. The K blockers for surrounding objectives should be spatially consistent. The centers of K blockers are generated by correlation grids with corresponding blockage correlation distance and interpolation.

Propsoal-3: A single blocking radius (fixed or statistical) is applied per cluster.

Proposal-4: The centers of blockage shall be temporally consistent by using a 3D (spatial/temporal) correlation grid.

Proposal-5: Enabling blockage modeling shall not dramatically change large scale parameters specified for each scenario.

Proposal-6: Focusing on remaining modeling parameters listed in open issues

Discussion:

Decision: The document is not treated

R1-163078	Power Attenuation in Dynamic Blockage	INTERDIGITAL COMMUNICATIONS

Proposal 1: Consider the second approach in Proposal A5 in [5] and use the log-normal distribution for shadowing loss.

Discussion:

Decision: The document is not treated

R1-162523	Remaining details on blockage modeling for channel modeling >6GHz	LG Electronics

Proposal 1: The temporal consistency should be modeled for blockage modeling only in case of assuming severe transition of test environment such as radio link failure test. Other condition is not precluded and further study might be needed.

Proposal 2: Considering implementation complexity, it is preferable to adopt simple interpolation-based approach in option 2.

Proposal 3: The decorrelation time for center of blocking region would be different between far-field and near-field blockage.

Discussion:

Decision: The document is not treated

R1-162388	Remaining details on blockage modelling	Intel Corporation

Proposal 1: The additional shadowing of one blocker should consider the link between the transmitter and the blocker for indoor scenario as in equation (2-1).

Proposal 2: When calculating the additional shadowing on the LoS channel cluster, the blocker model including center and span, can still be built in the receiver’s polar coordinate system. Then the blocker model in the transmitter’s coordinate system can be derived by center (φRx-π, π-θRx, rTx), azimuth angular span ARxrTx/rRx, zenith angular span ZRxrTx/rRx.

Proposal 3: For NLoS channel cluster, a virtual transmitter can be assumed at the extension line of AoA/ZoA.

Proposal 4: The additional shadowing on the NLoS channel cluster can be calculated based on the link between virtual transmitter and receiver in the same way as LoS channel cluster.

Proposal 5: The distance between the virtual transmitter and the blocker can be randomly generated in the range of rRx ⁡dTx→Rx-rRx.

Proposal 6: The definition of LoS/Shadow region can be clarified, e.g. using figure 3, for knife edge model.

Proposal 7:  The candidate of number of angularly blocked regions can be {1, 2, 3, 4}.

Proposal 8:  The additional shadowing of hand-held blockage can be modelled as a fixed attenuation and frequency dependency can be considered.

Proposal 9:  The existence/absence of hand-held blockage can be modelled as a discrete event.

Discussion:

Decision: The document is not treated

R1-162720	Discussion on blockage modeling for above 6GHz channel model	Samsung

Proposal 1: The framework proposed in A8 should be used in the blockage modeling for above 6 GHz channels.

Proposal 2: The distance between the receiver and the blocking screen should be randomly generated.

Proposal 3: The average blocking period should be used in the simulation. The average blocking period is determined by Eq. (4).

Proposal 4: The blockage attenuation is constant during the blockage period. Linear transition periods of 0.01s are introduced at both sides of the blockage period.

Proposal 5: The number of blockage regions is modeled by a Poisson distributed random variable as Eq. (5).

Proposal 6: Scenario-dependent parameters listed in Table 2 need to be determined via measurements.

Proposal 7:  The channel generation procedure with blockage should apply the flowchart in Figure 4.

Discussion:

Decision: The document is revised in 3393.

R1-163393	Discussion on blockage modeling for above 6GHz channel model	Samsung

Proposal 1: The framework proposed in A8 should be used in the blockage modeling for above 6 GHz channels.

Proposal 2: The distance between the receiver and the blocking screen should be randomly generated.

Proposal 3: The average blocking period should be used in the simulation. The average blocking period is determined by Eq. (4).

Proposal 4: The blockage attenuation is constant during the blockage period. Linear transition periods of 0.01s are introduced at both sides of the blockage period.

Proposal 5: The number of blockage regions is modeled by a Poisson distributed random variable as Eq. (5).

Proposal 6: Parameters of the angular blocking region in (6) can be presented by (7) and (11).

Proposal 7: Scenario-dependent parameters listed in Table 2 need to be determined via measurements.

Proposal 8:  The channel generation procedure with blockage should apply the flowchart in Figure 5.

Discussion:

Decision: The document is not treated

R1-163247	Further considerations on blockage modeling	Ericsson

The following additions to the agreed WF on blocking are proposed:

Proposal A3-1: The blocking objects are modeled as screens of a fixed size randomly placed in physical locations

Proposal A4-1: Each UE should only consider the nearest K blockers (excluding self-blocking)

Proposal A5-1: The attenuation of each ray or cluster due to a certain blocker is calculated using the formula given in R1-161696

Proposal A6-1: UEs may share blockers if these are simultaneously among the K closest of both UEs

Proposal A7-1: The blocking objects may be moving

Proposal A3/7-2: Typical blocker sizes and speeds are FFS. The values given in Table 1 of R1-161623 can be considered as starting points for further discussion

Discussion:

Decision: The document is not treated

R1-163473	WF on blockage modelling    Qualcomm Inc., Intel, Samsung, Nokia, Shanghai Bell, Ericsson, Interdigital

Down-select between either A or B options for proposal 3,5,6 and 7.

Discussion:

Intel: With B6 we need to consider also blocking between UEs. More considerations are needed with B6.

Ericsson: It is possible we do not disregard that with this model.

CATT: Figure below proposal 7 is not clear. Distance is related also with size of the blocker.

Ericsson: There is a difference between A and B options regarding blocking and the size of the object.

Decision: The document is agreed.

#### Remaining details on spatial consistency modelling

Time evolution

R1-162949	Discussion on time evolution of angles	Huawei, HiSilicon

Proposal 1: Adopt the model of time-variant angles given by equations (1)~(3) under the framework of spatial consistency model in [2].

Discussion:

Decision: The document is not treated

Angular consistency

R1-162244	Discussion on angular consistency model	Huawei, HiSilicon

Proposal 1: The composite PASs in azimuth and zenith dimension of all clusters are generated by

,  ,

where kc is the calibration coefficient to slow down the varying rate of angles in position.

Proposal 2: kc = 16 is a suitable value for the new model in UMi scenario.

Discussion:

Decision: The document is not treated

Spatial consistency

R1-162313	Clarification on spatial consistency and blockage model	CATT

Proposal 1: The propagations should be LOS or NLOS only within a grid, and the correlation distance could have relation with ASA and AOA.

Proposal 2: The probability, position, and size of the blockers should be defined clearly.

Proposal 3: The additional shadowing of the user in LOS condition should be compared with the path loss difference between LOS and NLOS to decide the appropriate state of the user.

Discussion:

Decision: The document is not treated

R1-162389	Remaining details on spatial consistency modelling	Intel Corporation, InterDigital

Proposal 1: For drop based MU-MIMO simulation, use baseline model for spatial consistency.

Proposal 2: For mobility based simulation such as beam tracking, use baseline model for initial UE dropping, followed by updating cluster delay using equation (4-1) and updating cluster angles with the limit of equation (4-2).

Proposal 3: If moving between different channel states is modelled, define soft LOS/NLOS and indoor/outdoor states and the path loss and fast fading channel are defined as a function of soft LOS/NLOS and indoor/outdoor states as in equation (5-1) and (5-2).

Proposal 4: Spatial consistency can also be calibrated as:

- Drop UE pairs with fixed distance between the two UEs of one pair, e.g. 3 meters.

- Define system wide statistics between the links of two UEs to the same TRP, e.g. the serving TRP of the first dropped user in each pair.

- For phase-1 calibration, the difference of the LOS/NLOS state and indoor/outdoor state of each UE pair can be used as system wide statistics metrics.

- For phase-2 calibration, the Eigen value/or Eigen value ratio of the channel cross covariance matrix of each UE pair can be used as system wide statistics metrics.

Discussion:

Huawei: What is the baseline model for Proposal 1? Do you assume shared clusters among users?

Intel: Working assumption and shared clusters are assumed. We have not done MU-MIMO simulations.

Huawei: That is important before agreeing proposal 1. Figure 2 leads to negative delays.

Intel: We have rounded to the minimum delay.

Huawei: What is the max moving distance? How the covariance matrix is defined?

Intel: 500 m. Matrix can be defined similarly as before, between the UEs.

ZTE: Equation 4-2. How to implement the limitation on change of angle? It is not necessary that all clusters are continuous.

Intel: Change is defined in equation. Existing stochastic model do not model this.

Ericsson: Proposal 1 assume baseline model but proposal 2 is not in line with that. It is difficult to say now how the model will be used in the future.

LGE: How can you simulate indoor / outdoor cases?

Intel: We force UE to be always outdoor.

Decision: The document is noted.

R1-162524	Remaining details on spatial consistency modeling for channel modeling >6GHz	LG Electronics

Proposal 1: The working assumption should be confirmed and further discussion might be needed on how to support mobility or large array.

Proposal 2: It would be more preferable to fix the grid size to one value for large/small scale parameters and the value can be potentially the maximum correlation distance among parameters.

Discussion:

Decision: The document is not treated

R1-162721	Discussion on spatial consistency for above 6GHz channel model	Samsung

Proposal 1: Alternative 1 with the proposed Step 1 to Step 5 is used in spatial consistency modeling for above 6GHz channels. Delays/AoAs/shadow fading of users in the same group are initialized according to Eq. (2) to Eq. (8).

Proposal 2: If UEs are moving in straght lines in the simulation, exponential function should be used as the filter function to generated spatially consistent random variables.

Discussion:

Decision: The document is not treated

R1-162743	Discussion of spatial consistency modeling		Lenovo (Beijing) Ltd

Proposal: Generate spatially correlated random variables with the 2D filtering method.

Discussion:

Decision: The document is not treated

R1-162880	Discussion on spatial consistency for above 6GHz	CMCC

Proposal: Scatters’ position could be assumed based on information of the cluster power, delay and angles. The assumption of scatterers’ position could faciliate the modeling of spatial consistency, for example, for UEs at difference locations (or when UE moves), the scatterer position can be used to modify angles of scatterers for different UEs (or for one UE at different positions).

Discussion:

Decision: The document is not treated

R1-162950	Text proposal to TR 38.900 to add spatial consistency model	Huawei, HiSilicon

This contribution proposes some content to the 12 steps in section-2 in TR36.873.

Discussion:

Ericsson: This goes beyond of agreements so far.

Huawei: We intend to progress with common parts.

Ericsson: We prefer proposal instead of TP.

CATT: Do we need to consider handover in simulating multiple cells?

Huawei: That is not excluded.

Decision: The document is noted.

R1-162958	Text proposal to TR 38.900 to add spatially consistent drops	Huawei, HiSilicon

This contribution proposes some content to the section 7 of TR38.900.

Discussion:

Ericsson: This do not necessary simplify. Better to consider consistency for all the users.

CATT: Is this in line with agreement from last meeting?

Nokia: It would be better to have spatial consistency always.

Qualcomm: We should consider also complexity.

Samsung: We need to study details further as shown in our contribution.

Sharp: How do you determine the correlation distance?

Decision: The document is noted.

R1-163077	An Alternative Interpolation Formula for Spatial Consistency	INTERDIGITAL COMMUNICATIONS, Intel Corporation

Proposal 1: If the interpolation of i.i.d. random variables is to be used for spatial consistency, then consider the interpolation formula in (3).

Discussion:

Decision: The document is not treated

R1-163248	Further considerations on spatial consistency	Ericsson

Proposal: The revised steps 5-7 proposed in section 3 should be adopted for channel generation

Discussion:

Intel: Mathematically this looks nice but the change is significant. Few people in this session participated in the work with original steps. It is better not to change the steps. Are there measurements supporting this? Also correlation distance is impacted.

ZTE: Our view is in the middle. This provides good solution but we are confused by using uniform delay.

Ericsson: There is no measurement support but the current model has artificial behaviour. Larger the delay more paths you see.

Intel: Nobody is sure whether this is convenient approach.

Huawei: It is difficult to say the relation of angle and delay from UE side. If there is no reason to keep the order then we could also change it. We should analyse further with measurement results.

Ericsson: Other models have this kind of behaviour.

Decision: The document is noted.

R1-163250	Further details on LOS and indoor state spatial consistency modelling	Ericsson

Proposal 1: Introduce soft LOS states in the spatial consistency modeling

Proposal 2: Do not introduce soft indoor/outdoor states. FFS how user mobility dropping in simulations should avoid users transiting between the states

Proposal 3: Use the knife edge-based method described in section 4 for modeling the soft LOS states

Discussion:

Decision: The document is not treated

R1-163467	WF on spatial consistency	Intel Corporation, InterDigital, Huawei, HiSilicon

Discussion:

Decision: The document is revised in 3475.

R1-163475	WF on spatial consistency	Intel Corporation, InterDigital, Huawei, HiSilicon, Qualcomm, Ericsson, LGE, Nokia, Alcatel Shanghai Bell, NTT DoCoMo, Samsung, CATT, ETRI, Lenovo, Motorola Mobility, NTT, ZTE, AT&T

Discussion:

Decision: The document is agreed.

R1-163480	WF on channel temporal evolution	Intel Corporation, InterDigital, Huawei, HiSilicon, Qualcomm

Discussion:

Ericsson: We support proposal for soft LOS on indoor states but we have concerns on temporal channel evolution in R1-163248.

Chair: Discuss both options further for the next meeting.

Decision: The document is noted.

R1-163481	WF on generating spatial consistent random variables	Lenovo, Motorola Mobility, InterDigital, Ericsson, CATT, Fujitsu

Proposal 1: Adopt the 2D filtering method to generate spatial consistent random variables.

Proposal 2: To obtain random variables within a grid area, FFS between interpolating using the generated random variables at the corners of the square grid or approximating with the random variable at the closest grid point.

Discussion:

Intel: We agree with the observation but the implementation was not captured in a TR. It was left for implementation.

Lenovo: Companies may then make errors impacting the calibration.

Huawei: This is important topic but we have also contribution R1-162956 on this. Did you consider the complexity? What is the max size of the map?

Lenovo: You can generate the area around the UE.

Intel: We could put a note in autocorrelation section referring to this.

Ericsson: TR could have informative section or annex. It would be good to have metrics for the calibration campaign.

OK for Lenovo.

Nokia: This is more like an implementation details. We don’t need to define every single details.

Huawei: This is not only implementation issue. We need 2D or even higher D spatial consistent paramaters.

Decision: The document is noted.

#### Remaining details on large antenna array modelling

R1-162314	Further discussion on large antenna array	CATT

Proposal 1: The Large BW and large antenna modeling applies only when either of the following conditions holds:

(1) Antenna aperture is greater than meter in either azimuth or elevation.

(2) BW is greater than  Hz.

Discussion:

Decision: The document is not treated

R1-162722	Discussion on large antenna modeling	Samsung

Proposal 1: For >6GHz, X = 16 and for <6GHz, X = 8.

Proposal 2: Large bandwidth support applies when the bandwidth is larger than 0.1 GHz.

Proposal 3: TOAs are modeled for the strongest 3 clusters.

Proposal 4: Intra-cluster are only considered in the first 3 strongest clusters

Discussion:

Huawei: How to test phase change? What is too much? Do you assume different requirements for different frequencies?

Samsung: Few degree seems to be good criteria.

NTT DOCOMO: Larger BW will have more clusters.

Ericsson: Table 1 phase errors to be considered. Large arrays will be more common in BS side.

Qualcomm: Phase error depends on implementation.

Huawei: We should not mix antenna to the channel which is changing all the time.

LGE: Wed are preparing WF

Decision: The document is noted.

R1-163245	Channel modeling considerations for supporting large antenna arrays	Ericsson

Proposal 1: The amplitude distribution within a cluster is modeled with a linear decay in dB units from the cluster center in azimuth and elevation.

Proposal 2: The distribution of sub-paths is uniform in azimuth and elevation.

Discussion:

Decision: The document is not treated

R1-163474	WF on large antenna modeling	LG Electronics, ETRI, InterDigital, Keysight, Samsung

Proposal 1: One of the following alternatives is agreed as condition to apply large BW and large antenna array modeling: Alt1, Alt 2, Alt 3

Proposal 2: The channel response of section 7.6.2 in R1-162714 needs to be updated as follows

Discussion:

Samsung: Alternatives 2 and 3 are not following the agreements from previous meeting. We support Alt 1.

Ericsson: We see some issues with previous agreements. Other alternatives correct those.

Nokia: Alt 1 value Y could be related to carrier frequency.

Ericsson: No. It is a relation with BW and array size. Confusion with connection between proposals 1 and 2.

CATT: Alt 2 and 3 are basically the same. Alt 1 is more comprehensive.

Huawei: There are 3 different aspects. Proposal 2 solves the one of them. We support Alt 2 to solve one problem.

Nokia: Alternatives 2 and 3 are against agreements from last meeting.

Samsung: We support Alt 1 and proposal 2.

Ericsson: We need to analyse further. We could discuss further over email.

Decision: The document is revised in 3487.

R1-163487	WF on propagation delay modeling over large antenna array	LG Electronics, ETRI, InterDigital, Keysight, Samsung, Ericsson, Huawei

Discussion:

Decision: The document is agreed.

#### Remaining details on modelling of other additional features

R1-162315	Further discussion on atmospheric attenuation	CATT

Proposal 1: Subtract the impact of oxygen absorption from the measurement data before parameterizing the model

Proposal 2: Apply the frequency-dependent oxygen loss in frequency-domain channel response for the large channel bandwidth. Possible simplification can be considered to reduce the channel modeling complexity.

Proposal 3: Apply a single oxygen loss to the channel impulse response of a cluster for narrow bandwidth, e.g.,  <1GHz, which is a special case of Proposal 2

Discussion:

Decision: The document is agreed.

R1-162955	Discussion on stochastic reflection modeling	Huawei, HiSilicon

Proposal: Specular reflection term is taken into account in the channel model as an optional term.

Discussion:

Ericsson: If you add into LOS what would be the K-factor? Do we need to split the K-factor?

Huawei: Yes

Ericsson: That would be very complicated. We propose to apply only to NLOS.

Huawei: We could keep LOS as FFS.

CATT: Why the reflection is the 2nd part? Any impacts on other parameters like blocking?

LGE: Why do you use this optional term?

Huawei: It depends on the case.

Decision: The document is noted.

R1-163479	WF on UE rotational motion	INTERDIGITAL COMMUNICATIONS, Qualcomm, Ericsson, Intel, Lenovo

Discussion:

Fraunhofer: Antenna pattern should include time dependent angles. Then you need to change both pattern and angles. We have concerns with changing pattern.

InterDigital: Pattern itself is not changing.

ZTE: Is this always on?

InterDigital: Can be turned off.

LGE: What is the criteria to turn off?

InterDigital: Not decide yet.

Nokia: This says it is for mobility. You should consider movement together with rotation.

Qualcomm: We need to consider mobility at the same time

Samsung: Current model is for global coordinate system. We don’t need to revise the equation. We don’t need to discuss when to turn on/off.

Intel: Rotation can be turned on individually without the mobility.

InterDigital: We agree can be turned on individually.

Decision: The document is noted.

### Remaining details on link-level channel models

Remaining details on beamforming modelling, scaling, etc.

Summary of email discussion

R1-162960	Summary of email discussion on link level channel model	Huawei, HiSilicon

Proposal 1: The summary of email discussion described in section “2. Summary of email discussion” above is agreed as way forward.

Discussion:

Ericsson: Is this intended as WF for this meeting?

Huawei: Yes

Decision: The document is agreed.

Linke level modeling

R1-162723	Discussion on LLS channel modeling	Samsung

Proposal: For simplified LLS channel modelling, the normalized CDL model can be converted to TDL channel coefficient by applying Eq. (1) with brick-wall spatial filter defined by Eq. (2).

Discussion:

Decision: The document is not treated

R1-162951	Discussion on channel model for link level simulation	Huawei, HiSilicon, Spirent, Keysight

Proposal: The link level channel models described in this document should be included in the channel model TR.

Discussion:

Decision: The document is not treated

R1-162959	Discussion on channel model for link level simulation in LoS scenario	Huawei, HiSilicon, Spirent, Keysight, NTT DOCOMO

Proposal: The LOS link level channel models described in this document should be included in the channel model TR.

Discussion:

Decision: The document is not treated

R1-163249	Further details on link level channel models	Ericsson

Proposal 1: Use angular scaling of CDLs such that a range of angular spreads can be represented by a single CDL

Proposal 2: Use angular translation of CDLs such that a range of mean angles can be represented by a single CDL

Proposal 3: If LOS link level models are introduced, these should include scalable K-factors.

Discussion:

Decision: The document is not treated

R1-163429	Text proposal for TR 38.900 to add link level channel models including LOS	Huawei, HiSilicon, CATT

Chair: Late contribution

Discussion:

Ericsson: This includes some parts of the email discussion but LOS case is not discussed and agreed.

Samsung: We have concerns on LOS table.

ZTE: Number of clusters was not agreed either for system level model. We should avoid the inconsistency between link and system level models.

Decision: The document is revised in 3476.

R1-163476	Text proposal for TR 38.900 to add link level channel models	Huawei, HiSilicon, CATT

Discussion:

Samsung: How to capture editorial errors in TR is up to rapporteur.

NTT DOCOMO: Comment on delay.

Decision: The document is agreed.

R1-163489	WF on link level channel models for LOS		Huawei, HiSilicon, Spirent, NTT DOCOMO, Vodafone, CMCC, CATT, CATR

Proposal 1: Link level channel models for the LOS case are included in the TR.

Proposal 2: The LOS channel models are generated by the same method as the NLOS channel models were already generated.

Proposal 3: The LOS channel model tables in following pages are adopted as way forward.

Discussion:

Ericsson: What is the gain between LOS and NLOS cases? Our document discuss LOS channels. We have concern adding detailed model. K-factor scaling should be included.

Huawei: It is a K-factor. We try to minimize the number of variables. Existing models are based on NLOS.

Samsung: Ericsson proposal sounds good but we support this WF.

Ericsson: We can support if we include K-factor scaling.

Nokia: How to capture that in the model? We can also support if solve also K-factor issue.

Decision: The document is revised in 3491.

R1-163491	WF on link level channel models for LOS		Huawei, HiSilicon, Spirent, NTT DOCOMO, Vodafone, CMCC. CATT, CATR, AT&T, Ericsson

Proposal 1: Link level channel models for the LOS case are included in the TR.

Proposal 2: The LOS channel models are generated by the same method as the NLOS channel models were already generated.

Proposal 3: The LOS channel model tables in following pages are adopted as way forward.

Proposal 4: The K factor will be scalable. Example values of K factor and guidelines on how to scale the K factor will be added to the TR.

Discussion:

Decision: The document is agreed.

R1-163490	WF on spatial filtering for TDL channel models	Huawei, HiSilicon, Qualcomm

Discussion:

Ericsson: There are 2 different alternatives in step. We propose to approve the 2nd one. With that we can co-source. We need to know what alpha is. What should we write to the TR.

LGE: Would it be possible to use either options or do we need to down select?

Huawei: We can have both in TR or we can also decide here.

Qualcomm: WE can leave the detail later.

Samsung: We support Ericsson suggestion step 4 scaling parameter is introduced.

Decision: The document is revised in 3643.

R1-163643	WF on spatial filtering for TDL channel models	Huawei, HiSilicon, Qualcomm, Samsung, Ericsson

Discussion:

Decision: The document is agreed.

### Channel modelling calibrations

#### Calibration scenarios and assumptions

Including UE antenna modelling

UE antenna modelling

R1-162237	UE antenna modeling for link level calibration purposes	Qualcomm Inc.

Proposal 1: It is important to have a model that captures the basic aspects of subarrays and directional coverage while keeping the modeling complexity low.

Proposal 2: The antenna gain and array size should match expectations in a typical device.

Discussion:

Decision: The document is not treated

R1-162744	Discussion of antenna assumption for channel model calibration	Lenovo (Beijing) Ltd, Motorola Mobility

Proposal 1: Define different antenna configurations for different frequency range.

Proposal 2: Consider defining multiple antenna configurations corresponding to different types of MS.

Proposal 3: Frequency dependent antenna radiation pattern is FFS.

Proposal 4:  The full connectivity model is the baseline. Hybrid beamforming is FFS.

Discussion:

Qualcomm: Why do we need to talk about the connectivity model?

Lenovo: How antenna elements are connected will have an impact on performance.

Huawei: We don’t need to model many models. It is better to assume typical case for calibration purposes.

Nokia: We don’t see a need to have multiple models.

Ericsson: Scope of this SI is channel model. Calibration will validate that implemented 12 steps are comparable. We can assume very generic antenna model.

Lenovo: We agree no detailed model is needed for the CM.

Decision: The document is noted

Scenarios

R1-162316	Further discussion on calibration scenarios	CATT

Proposal 1:  The 3-sector deployment can be used for indoor-office calibration scenario.

Proposal 2: the antenna pattern defined in table 7.1-1[5] and antenna modeling [4] is also applicable for indoor-office.

Proposal 3:  Equation 4 can be used for calculating downtilt angle for indoor office.

Discussion:

Decision: The document is not treated

R1-162952	Discussion on channel modeling calibration scenarios	Huawei, HiSilicon

Proposal 1: layouts of indoor hotspot, urban micro and urban macro scenario mentioned above should be adopted in calibration of channel model.

Proposal 2: parameters in table 1 should be adopted in calibration of channel model.

Discussion:

Decision: The document is not treated

Scenarios and assumptions

R1-162423	Further details on calibration scenarios and assumptions	NTT DOCOMO, NTT

Proposal 1: Multiple carrier frequency bands can be considered for each scenario, e.g., indoor-office, UMi and UMa.

Proposal 2: Multiple eNB antenna configurations can be considered, depending on the calibration requirement. The necessity of considering multiple UE antenna configurations is FFS.

Proposal 3: If 3-sector antenna array model is used for the indoor scenario for calibration purpose, consider the antenna array placement model in Figure 1.

Proposal 4: Consider the placement of the antenna panel on the roof with the bore-sight facing the ground, and consider omni-directional antenna pattern in the azimuth and directive antenna pattern in the elevation in this case.

Proposal 5: RAN-1 shall study the beamwidth and tilting angle for the indoor antenna array placement.

Proposal 6: If random eNB antenna dropping shall be modeled, consider to reuse the model described in Section A.2 of TR 36.897.

Proposal 7: Take the simulation assumptions in Table 1 and Table 2 for channel model calibration.

Discussion:

Decision: The document is not treated

R1-162525	Discussion on calibration scenarios and assumptions	LG Electronics

Proposal 1: For calibration purpose, we think that BS antenna structure with a single panel is sufficient, i.e. MgNg = 1, and adopt Table 1 for BS antenna configurations.

Proposal 2: Adopt Table 2 for UE antenna modeling.

Discussion:

Decision: The document is not treated

R1-162724	Calibration scenarios and assumptions	Samsung

Proposal 1: In phase 1, calibrate above 6GHz channel model according to Table 1

Proposal 2: In phase 2, calibrate above 6GHz channel model according to Table 2, and if needed, account for feedback from phase 1 calibration.

Proposal 3: Phase 3 calibration is FSS.

Discussion:

Decision: The document is not treated

R1-162574	Scenarios and simulation assumption for calibration	Nokia, Ericsson, AT&T, CMCC, ETRI, Huawei, HiSilicon, Intel, KT Corporation, NTT DOCOMO, Qualcomm, Samsung

Proposal 1: Use the scenarios, assumption and metrics from phase-1 and phase-2 calibrations in TR36.873 as starting point.

Proposal 2: Include 3D-UMa, 3D-UMi-Street Canyon, Indoor-office as the calibration scenarios

Proposal 3: Calibration assumption should consider multiple carrier frequencies, e.g. 6, 30, 70 Ghz. (further down-selection is possible)

Proposal 4: Adopt the detail calibration assumption in Section 3.

Proposal 5: Target to include the additional features in the general test metrics as much as possible.

Discussion:

Samsung: This is good starting point but we realized now that 1 and 2 GHz BWs may not need to be calibrated. We could consider 100 MHz and 200 MHz.

CATT: Is the intention to omit phase 3?

Nokia: We did not touch that. We don’t need to have baseline calibration for this SI.

ZTE: If we simulate only lower BWs do you then assume only frequencies <6GHz?

Samsung: No

LGE: Do we really need to connect system BW with carrier frequency?

Nokia: Reason is to reduce the calibration cases. We can put system BWs in brackets.

Interdigital: Do you assume wrap around also in the office?

Nokia: Only for Uma and UMi.

Decision: The document is revised in 3477.

R1-163477	Scenarios and simulation assumption for calibration	Nokia, Ericsson, AT&T, CMCC, ETRI, Huawei, HiSilicon, Intel, KT Corporation, NTT DOCOMO, Qualcomm, Samsung

Discussion:

Decision: The document is agreed

R1-163635	WF on simulation assumption for large-scale calibration 	Nokia, Samsung

Discussion:

Ericsson: For O2I it would make sense to have 50/50 for calibration purposes.

Samsung: We are not sure that’s better.

Huawei: We agree with Ericsson. We have 2 different penetration losses. 50/50 is more reasonable.

Nokia: How do we resolve the amount of indoor users? When we drop the users we don’t need to worry about O2I.

Huawei: Penetration loss is not dependent on distribution.

NTT DOCOMO: 80% indoors was agreed last time.

Huawei: We should assume 50% high loss and 50% low loss.

ZTE: Why do we discuss this now? We could take just whatever number.

Nokia: We have also other non-realistic assumptions.

Ericson: This is one of the important issue. We should be changed to 50/50.

Samsung: We should consider the numbers again in the next meeting.

Fraunhofer: MS ant configuration says only 1 element polarized 90 degrees. What does it means.

Samsung: Ant pattern is omnidirectional.

NTT DOCOMO: We need to define the orientation of antenna panel.

Decision: The document is revised in 3641.

R1-163641	WF on simulation assumption for large-scale calibration 	Nokia, Samsung

Discussion:

Huawei: With minimum distance of 1 m would be safer.

Agreed with clarification: minimum distance is only in 2D.

Decision: The document is agreed

R1-163638	WF on simulation assumption for full calibration 	Nokia, Samsung

Discussion:

Ericsson: We should add 60GHz. Which angle spreads these are?

Samsung: DoA.

LGE: UE array orientation parameters. Why don’t we use the same than for 3D CM?

Samsung: WE didn’t check 3D CM but OK with that.

Fraunhofer: CDF definition in 3D CM is based on eigen value of covariance matrix. Could we change it to average value instead?

Samsung: We had similar discussion in 873.

Intel: 2 angles having 60 degree difference? We could consider 0 and 0.

Samsung: No strong view on that.

Ericsson: 1st and 2nd could be better than largest and smallest.

NTT DOCOMO: 2 different ant antenna ray configurations and CSI port mapping. Does it mean 4 different configurations?

Samsung: We can do editing.

ZTE: We could ask companies to prove table. Is this full calibration for the next phase?

Samsung: This is for calibrating the channel model. We can check in the next meeting.

Nokia: Ant configuration 1 is good enough for most of the calibrations except eigen value. Is configuration 2 only for eigen value.

Decision: The document is revised in 3642.

R1-163642	WF on simulation assumption for full calibration 	Nokia, Samsung

Discussion:

Samsung: WE propose the email discussion for the calibration campaign:

- 1 week for LS fading

- By next meeting for the full calibration

Ericsson: How do we proceed with FFS for PL model?

NTT DOCOMO: We are not sure if this kind of strict deadline for email discussion is necessary.

Samsung: Motivation is the target for finalizing things step by step.

NTT DOCOMO: Are there change to have 1st calibration round until June plenary?

Samsung: It may be possible until August.

Huawei: Deadline should be explicit.

Samsung: 25 Apr is the deadline.

Qualcomm: What exactly are we agreeing now?

Samsung: LS fading we use the values in brackets.

Finalize details by email discussions for the next meeting. Samsung will lead the email discussion.

Decision: The document is working assumption

Friday

Email discussion until April 25th for large scale parameter calibration and detailed parameters for full calibration – (Samsung)

Email discussion until the next meeting for full calibration – (Samsung)

R1-163639	WF on calibration of additional features		Huawei, HiSilicon

At least the following additional features should be included into the calibration procedure of this SI.

Spatial consistency.

Blockage

Discussion:

Samsung: Previous 2 WFs can be used also for additional features.

Ericsson: We should re-use the same table and add mobility of the users.

Huawei: There should be some new metrics to be used.

Nokia: How do we generate the change of angles? We have to model UE position changing.

Huawei: We have different approaches. Not to assume mobility or to assume mobility.

Nokia: Angle of cluster will be changing for moving user.

Huawei: Same user dropping approach can be used.

Qualcomm: It is good idea to consider few other metrics. These metrics are not clear at this point.

Intel: We agree with Qualcomm. We could have very simple extension to current approach.

Nokia: Channel autocorrelation based on user distance would be good parameter.

Ericsson: We haven’t finalized these metrics yet. We could discuss over the email, for the next meeting.

Huawei: 1st bullet may be easy to agree in high level. Exact metrics depends on the final solutions.

Nokia: Do we assume angle between users or within one user?

Huawei: We can discuss details over the mail.

Decision: The document is noted. The 1st bullet is agreed.

#### Metrics for channel model calibrations

R1-162245	Initial results of calibration phase 1	Huawei, HiSilicon

Initial calibration results of phase1 for indoor hotspot, urban micro and urban macro scenarios

Proposal: take the coupling loss and geometry results into account in phase 1 of channel modelling calibration for multiple company comparison.

Discussion:

Decision: The document is not treated

R1-162317	Discussion on calibration phases and metrics	CATT

Proposal 1: The channel calibration simulation for above 6GHz can be divided into three phases similar to the simulation phases in TR 36.873.

Proposal 2: The metrics in each phase in TR 36.873 can be chosen as the baseline metrics in corresponding phases in simulation for above 6GHz, the corresponding additional metric/s may be defined and added to respective phases with implementation of additional features in channel coefficient generation procedure.

Proposal 3: Simulation assumptions shown in table 1 can be used for phase 1 calibration for above 6GHz channel modeling.

Discussion:

Decision: The document is not treated

R1-162424	Metrics on channel model calibrations	NTT DOCOMO, NTT

Proposal 1: Phase-1 calibration of >6GHz channel model is based on the phase-1 calibration of the 3D channel model. Coupling loss and geometry are used for phase-1 calibration.

Proposal 2: Phase-2 calibration of >6GHz channel model is based on the phase-2 calibration of the 3D channel model. Wideband SINR and singular values are used for phase-2 calibration.

Proposal 3: Capture the metrics defined in Table 1 and Table 2 in TR for phase-1 and phase-2 calibration respectively.

Observation: Some of the additional features developed for >6GHz channel model are required by specific 5G technologies and can be calibrated during the technology study item.

Proposal 4: RAN 1 shall decide for each additional feature, whether include that feature in phase-1 and phase-2 calibration or establish a separate calibration for it.

Discussion:

Huawei: On proposal 4, it would be good to complete the calibration on all additional features.

NTT DOCOMO: Can we complete all additional features for the next meeting? What is phase 3?

Huawei: It depends on the progress. Phase 3 means additional features.

Decision: The document is noted

R1-162526	Discussion on metrics for channel modeling calibration	LG Electronics

Proposal 1: The metrics in 3D channel model calibration can be reused for the calibration of the above 6GHz channel modeling.

Proposal 2: To align with a practical system, one antenna element is assumed for the coupling loss and geometry in the calibration. Also, to compare with the previous channel modeling, vertically fixed beam with N=1 can be assumed for the coupling loss and geometry in the calibration.

Discussion:

Decision: The document is not treated

R1-162725	Metrics for channel model calibrations	Samsung

Decision: The document is withdrawn

R1-162953	Discussion on channel modeling calibration procedures	Huawei, HiSilicon

Proposal 1: channel modeling calibration should verify the support for channel model requirements.

Proposal 2: channel modeling calibration should align the implementation of channel model among companies.

Proposal 3: the 4-step procedures should be considered in channel modeling calibration.

Proposal 4: the parameters and metrics in table1-4 should be considered in channel modeling calibration.

Discussion:

Decision: The document is not treated

R1-163430	WF on channel model calibration procedures	Huawei, HiSilicon

Chair: Late contribution

Discussion:

CATT: We support this WF.

Nokia: Step 4 is unnecessary. Step 3 is not needed necessary for each feature.

Samsung: Nokia proposal already propose full details.

Lenovo: Steps 4 is unnecessary at this point.

Decision: The document is noted.

### Consistent extension of the additional features to < 6GHz

No contributions.

### Map-based methodology

Summary of email discussion

R1-163117	Email discussion on map-based hybrid model 	Keysight

This contribution was able to report the email discussion as such without any edition work because of the relatively small number of messages sent.

Discussion:

Decision: The document is noted

Mab-based hybrid model

R1-163095	Simulations of map-based hybrid channel model	ZTE

Observation 1: The proposed map-based hybrid model provides at least the following in terms of spatial consistency:

- The realistic transition between LoS condition and NLoS condition.

- The smooth and close-to-real evolutions of power-delay profile, power-azimuth profile and power-zenith profile over UE's moving trajectory for the significant clusters or rays.

- The realistic birth-death processes for significant clusters or rays.

Observation 2: The time consumption of the proposed map-based hybrid model is more than acceptable for the system-level simulations.

Discussion:

Decision: The document is not treated

R1-163096	Map-based hybrid model to meet modeling requirements	ZTE

Observation: The map-based hybrid model fully meets all agreed modelling requirements.

Discussion:

Decision: The document is not treated

R1-163116	Generality of map-based hybrid model	Keysight

This contribution has presented few simulation results to demonstrate versatility of output parameters of deterministic channel models. The simulation results show substantial statistical variation of parameters. We conclude that with a sufficient size and realistic, i.e. not too homogenous, map and a rich deployment a satisfactory generality can be reached. When adding more stochastic components, as is the case with the map–based hybrid model, even more diverse radio channels can be generated.

Discussion:

Decision: The document is not treated

R1-163119	Distance Dependent Cluster–Wise Angular Spread	Keysight

This contribution has presented a distance dependent cluster–wise rms angular spread for the map–based hybrid model. The proposal utilizes the information of both the interaction type and the distance to/from first/last interaction points that are inherently available for deterministic clusters of the map-based hybrid model. The proposed enhancement increases the model accuracy and realism with a minimal increase of computational complexity.

Discussion:

Decision: The document is not treated

Text Proposals

R1-163094	Detailed implementation and text proposal for map-based hybrid model	ZTE

Proposal: RAN1 agrees the text proposal in appendix, as an alternative methodology for high-frequency channel modeling.

Discussion:

Decision: The document is revised in 3404.

R1-163102	TP for map-based hybrid model	Keysight

The attached TP is to provide an update to TR 38.900. It adds the section 8 to the TR.

Discussion:

Decision: The document is not treated

R1-163404	Joint text proposal for map-based hybrid channel model	ZTE, Keysight

Proposal: RAN1 agrees, in principle, the text proposal in appendix, as an alternative methodology for high-frequency channel modeling. How to finalize the sharing of certain modeling details with stochastic-only channel model is FFS.

Discussion:

Sharp: We are confused on which clusters to be removed. Are you minimizing the information loss?

ZTE: For removal of random clusters, we believe info from digital map is enough.

Ericsson: Important part is in the beginning on when to use the hybrid model. We need better guidance on which model to use and when. Consistency between results is important.

ZTE: We can revise the wording but the intention is that hybrid model is good compliment for stochastic model. If people are interested they can then use the model. E.g.  agreed UMa model is based on both methods. Stochastic model is based on measurements and ray tracing.

Sharp: If results are different then how to draw the conclusion?

ZTE: That applies also to stochastic only model.

Samsung: People not participated channel model work may not know which model to use in future evaluations. We could say that bybrid model is not calibrated. It would be OK to have text as informative. This is based on inputs from 2 companies only.

Decision: The document is revised in 3478.

R1-163478	Joint text proposal for map-based hybrid channel model	ZTE, Keysight, CATR

Discussion:

Nokia: We are fine in principle but hoe to capture should be left for rapporteur.

Ericsson: Concern having map based model. If it is validated with measurements then we are fine.

ZTE: WE have discussed this over email after AH. What parameters you like to see?

Ericsson: It says now the map based model is not calibrated. We don’t know if it match with reality.

ZTE: Deterministic will be closer to realistic than stochastic model. Now we combine these models.

Ericsson: Deterministic typically worse regarding PL than stochastic.

ZTE: We gave lot of details in Malta meeting already. What else do you want to see?

Ericsson: Typical metrics for calibration.

ZTE: We can agree this as working assumption and provide results for the next meeting.

Ericsson fine but we may need to revise this based on results.

Decision: The document is working assumption.

### Other

mmMAGIC results

R1-162872	Initial 60 GHz channel sounding results from the mmMAGIC project for corner diffraction and surface scattering	Keysight

This paper has presented results from an isolated corner diffraction measurement and surface scattering in indoor and outdoor environments of a modern building. A wideband (2 GHz) channel sounder was used at a carrier frequency of 60 GHz.

An “add-on module” needs to be developed to extend the current stochastic channel models to support spatial consistency in expected use cases. This will allow the model to accuracy represent received signal strength and multi-path variations along a route of consecutive points. The model should address the transition from LoS to NLoS (i.e. the role of corner diffraction) as well as the effects of diffuse scattering from rough and smooth building and terrain surfaces.

More measurement data is required to determine the isolated diffuse scatter process for a single interaction at mmWave frequencies, and then to determine the model for the process.

Discussion:

Decision: The document is not treated.

# Closing of the meeting

Special thanks to Samsung for hosting. Rel-14 and the new radio topic have brought up a very high attendance.

RAN1 chair raised the concern that there is a risk of even higher number of participants in the near future. As next meeting is in China, RAN1 chair and MCC will closely monitor the registration aspects aiming at minimising the risk of meeting rooms in different hotels.

Meeting was closed at 17:15.

See you all in Nanjing.

# Annex A:	List of Tdocs at RAN1 #84bis

Please see excel file attached to this report

# Annex B:	List of CRs agreed at RAN1 #84bis

| TDoc # | Title | Source |  | Release | Spec # | Version |  | Related WIs |  | CR # |  | CR revision # |  | CR category |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1-162341 | CR on CSI-RS transmission in DwPTS | Samsung |  | Rel-13 | 36.211 | 13.1.0 |  | LTE_EBF_FDMIMO-Core |  | 0216 |  | 2 |  | F |  |
| R1-162342 | Correction to the UE's assumption on DMRS ports | ZTE |  | Rel-13 | 36.213 | 13.1.0 |  | LTE_EBF_FDMIMO-Core |  | 0593 |  | 1 |  | F |  |
| R1-162767 | Correction on RRC parameter for configuring new TBSs | Ericsson |  | Rel-13 | 36.213 | 13.1.0 |  | TEI12 |  | 0611 |  |  |  | A |  |
| R1-162768 | Correction on RRC parameter for configuring new TBSs | Ericsson |  | Rel-12 | 36.213 | 12.9.0 |  | TEI12 |  | 0612 |  |  |  | F |  |
| R1-162873 | Correction of paging PDSCH transmission for MTC UE | Ericsson, Nokia, Alcatel-Lucent Shanghai Bell |  | Rel-13 | 36.213 | 13.1.1 |  | LTE_MTCe2_L1-Core |  | 0616 |  |  |  | F |  |
| R1-163086 | Correction to rank 5-8 FD-MIMO CSI feedback | Ericsson |  | Rel-13 | 36.213 | 13.1.1 |  | LTE_EBF_FDMIMO-Core |  | 0614 |  |  |  | F |  |
| R1-163089 | Correction on aperiodic CSI reporting mode 1-0 and 1-1 | Ericsson |  | Rel-13 | 36.212 | 13.1.0 |  | LTE_CA_enh_b5C-Core |  | 0194 |  |  |  | F |  |
| R1-163090 | Correction on aperiodic CSI reporting mode 1-0 and 1-1 | Ericsson |  | Rel-13 | 36.213 | 13.1.1 |  | LTE_CA_enh_b5C-Core |  | 0615 |  |  |  | F |  |
| R1-163091 | Correction to S1 and S2 definition and i2 bit width | Ericsson |  | Rel-13 | 36.212 | 13.1.0 |  | LTE_EBF_FDMIMO-Core |  | 0195 |  |  |  | F |  |
| R1-163197 | Clarification on measuring the transmitted code power for the downlink DPCH slot formats #17 and #18 | Ericsson |  | Rel-12 | 25.215 | 12.1.0 |  | UTRA_DCHenh |  | 0205 |  |  |  | F |  |
| R1-163198 | Clarification on measuring the transmitted code power for the downlink DPCH slot formats #17 and #18 | Ericsson |  | Rel-13 | 25.215 | 13.0.0 |  | UTRA_DCHenh |  | 0206 |  |  |  | A |  |
| R1-163354 | DMRS initialization of CSS for MTC | Ericsson |  | Rel-13 | 36.211 | 13.1.0 |  | LTE_MTCe2_L1-Core |  | 0230 |  |  |  | F |  |
| R1-163355 | Missing words in PRACH starting subframe paragraph for MTC | Ericsson |  | Rel-13 | 36.211 | 13.1.0 |  | LTE_MTCe2_L1-Core |  | 0231 |  |  |  | F |  |
| R1-163359 | Update RRC parameter names for MTC | Ericsson |  | Rel-13 | 36.213 | 13.1.1 |  | LTE_MTCe2_L1-Core |  | 0618 |  |  |  | F |  |
| R1-163360 | PUCCH repetition for Msg4 for MTC | Ericsson |  | Rel-13 | 36.213 | 13.1.1 |  | LTE_MTCe2_L1-Core |  | 0619 |  |  |  | F |  |
| R1-163361 | MPDCCH repetition for paging and random access for MTC | Ericsson |  | Rel-13 | 36.213 | 13.1.1 |  | LTE_MTCe2_L1-Core |  | 0620 |  |  |  | F |  |
| R1-163405 | Introduction of Rel-13 feature of NB-IoT in 36.212 | Huawei, HiSilicon |  | Rel-13 | 36.212 | 13.1.0 |  | NB_IOT-Core |  | 0192 |  | 1 |  | B |  |
| R1-163421 | Coding of higher layer parameter codebooksizeDetermination-r13 | Nokia, Alcatel-Lucent Shanghai Bell |  | Rel-13 | 36.212 | 13.1.0 |  | LTE_CA_enh_b5C-Core |  | 0197 |  |  |  | F |  |
| R1-163422 | Coding of higher layer parameter codebooksizeDetermination-r13 | Nokia, Alcatel-Lucent Shanghai Bell |  | Rel-13 | 36.213 | 13.1.1 |  | LTE_CA_enh_b5C-Core |  | 0622 |  |  |  | F |  |
| R1-163433 | Corrections on Simultaneous HARQ-ACK and P-CSI in 36.213 | CATT |  | Rel-13 | 36.213 | 13.1.1 |  | LTE_CA_enh_b5C-Core |  | 0623 |  |  |  | F |  |
| R1-163434 | Corrections SRS dropping in CA in 36.213 | Qualcomm Inc. |  | Rel-13 | 36.213 | 13.1.1 |  | TEI13, LTE-Phys |  | 0624 |  |  |  | F |  |
| R1-163435 | Correction to the usage of undefined terminology "channel" | LGE |  | Rel-13 | 36.213 | 13.1.1 |  | LTE_LAA-Core |  | 0625 |  |  |  | D |  |
| R1-163436 | Correction on channel access procedure after an additional defer duration for DL LBT | WILUS Inc. |  | Rel-13 | 36.213 | 13.1.1 |  | LTE_LAA-Core |  | 0626 |  |  |  | F |  |
| R1-163458 | Correction on timing for secondary cell activation / deactivation for eCA in 36.213 | Huawei, HiSilicon |  | Rel-13 | 36.213 | 13.1.1 |  | LTE_CA_enh_b5C-Core |  | 0610 |  | 1 |  | F |  |
| R1-163500 | Correction on SPS HARQ-ACK bit handling in case of dynamic codebook configuration of eCA in 36.213 | Huawei, HiSilicon, Samsung, Nokia, Fujitsu, CATT |  | Rel-13 | 36.213 | 13.1.1 |  | LTE_CA_enh_b5C-Core |  | 0575 |  | 2 |  | F |  |
| R1-163501 | Clarification for LAA CSI processing | Qualcomm Incorporated |  | Rel-13 | 36.213 | 13.1.1 |  | LTE_LAA-Core |  | 0627 |  |  |  | F |  |
| R1-163502 | Correction to EPDCCH procedures for LAA FS 3 | Qualcomm Inc. |  | Rel-13 | 36.211 | 13.1.0 |  | LTE_LAA-Core |  | 0232 |  |  |  | F |  |
| R1-163503 | Draft CR for 36.212 on subframe configuration for LAA | Nokia, Alcatel-Lucent Shanghai Bell |  | Rel-13 | 36.212 | 13.1.0 |  | LTE_LAA-Core |  | 0198 |  |  |  | F |  |
| R1-163504 | Clarification on PDSCH mapping to resource elements | NEC |  | Rel-13 | 36.211 | 13.1.0 |  | LTE_EBF_FDMIMO-Core |  | 0233 |  |  |  | F |  |
| R1-163505 | CR on CSI-RS description in TS 36.211 | Samsung |  | Rel-13 | 36.211 | 13.1.0 |  | LTE_EBF_FDMIMO-Core |  | 0234 |  |  |  | F |  |
| R1-163506 | Correction on the description of DMRS table | ZTE |  | Rel-13 | 36.212 | 13.1.0 |  | LTE_EBF_FDMIMO-Core |  | 0199 |  |  |  | F |  |
| R1-163508 | MCS Table for Initial Partial TTI in LAA |  | Ericsson, Panasonic | Rel-13 | 36.213 | 13.1.1 | LTE_LAA-Core |  | 0628 |  |  |  | F |  |
| R1-163513 | Correction on the linkage between CSI-RS and CSI-IM for Class B |  | LG Electronics | Rel-13 | 36.213 | 13.1.1 | LTE_EBF_FDMIMO-Core |  | 0629 |  |  |  | F |  |
| R1-163514 | Correction on HARQ-ACK ordering in case of semi-static codebook configuration of eCA |  | Huawei, HiSilicon | Rel-13 | 36.213 | 13.1.1 | LTE_CA_enh_b5C-Core |  | 0608 |  | 1 |  | F |  |
| R1-163526 | Corrections on Simultaneous HARQ-ACK and P-CSI in 36.213 |  | CATT | Rel-13 | 36.213 | 13.1.1 | LTE_CA_enh_b5C-Core |  | 0623 |  | 1 |  | F |  |
| R1-163527 | CR on Correction for HARQ-ACK Codebook Determination in eCA |  | Samsung | Rel-13 | 36.213 | 13.1.1 | LTE_CA_enh_b5C-Core |  | 0630 |  |  |  | F |  |
| R1-163528 | Corrections on the support of ending partial subframe in LAA |  | Nokia, Alcatel-Lucent Shanghai Bell, CATT, Ericsson, Samsung, Huawei, HiSilicon | Rel-13 | 36.211 | 13.1.0 | LTE_LAA-Core |  | 0235 |  |  |  | F |  |
| R1-163531 | Clarification on "special subframe" for frame structure type 3 in 36.213 |  | Huawei, HiSilicon, Samsung, CATT, Nokia, Alcatel-Lucent Shanghai Bell, Ericsson, Qualcomm | Rel-13 | 36.213 | 13.1.1 | LTE_LAA-Core |  | 0631 |  |  |  | F |  |
| R1-163532 | Clarification on the applicability of DL procedures for LAA Scell |  | Qualcomm Inc. | Rel-13 | 36.212 | 13.1.0 | LTE_LAA-Core |  | 0200 |  |  |  | F |  |
| R1-163533 | Corrections on simultaneous transmission of HARQ-ACK and SR in 36.213 |  | CATT, LGE, Nokia | Rel-13 | 36.213 | 13.1.1 | LTE_CA_enh_b5C-Core |  | 0632 |  |  |  | F |  |
| R1-163534 | Clarification of CSI-RS on extended CP |  | NEC, Samsung, CATT | Rel-13 | 36.211 | 13.1.0 | LTE_EBF_FDMIMO-Core |  | 0236 |  |  |  | F |  |
| R1-163535 | Correction on description about UpPTS length for preamble format 4 for PRACH |  | ZTE | Rel-13 | 36.211 | 13.1.0 | LTE_EBF_FDMIMO-Core |  | 0237 |  |  |  | F |  |
| R1-163567 | MCS field in DCI format 6-2 for paging for MTC |  | Ericsson, Huawei, HiSilicon | Rel-13 | 36.212 | 13.1.0 | LTE_MTCe2_L1-Core |  | 0196 |  | 1 |  | F |  |
| R1-163568 | MCS field in DCI format 6-2 for paging for MTC |  | Ericsson, Huawei, HiSilicon | Rel-13 | 36.213 | 13.1.1 | LTE_MTCe2_L1-Core |  | 0621 |  | 1 |  | F |  |
| R1-163593 | Introduction of NB-IoT |  | Ericsson LM | Rel-13 | 36.211 | 13.1.0 | NB_IOT-Core |  | 0224 |  | 4 |  | B |  |
| R1-163626 | Correction on DAI Presence in DCI formats for eCA |  | Samsung, Nokia, Alcatel-Lucent Shanghai Bell, Panasonic | Rel-13 | 36.212 | 13.1.0 | LTE_CA_enh_b5C-Core |  | 0201 |  |  |  | F |  |
| R1-163670 | CR on CSI-RS ID configuration for TM9 in TS 36.213 |  | Samsung | Rel-13 | 36.213 | 13.1.1 | LTE_EBF_FDMIMO-Core |  | 0647 |  |  |  | F |  |
| R1-163672 | CR for 36.213 on multi-channel access procedure Type A2 in LAA |  | Nokia, Alcatel-Lucent Shanghai Bell | Rel-13 | 36.213 | 13.1.1 | LTE_LAA-Core |  | 0645 |  |  |  | F |  |
| R1-163673 | Correction on the description about DMRS |  | ZTE | Rel-13 | 36.211 | 13.1.0 | LTE_EBF_FDMIMO-Core |  | 0242 |  |  |  | F |  |
| R1-163676 | CR on MPDCCH format for Rmax=1 and 2/4 PRBs |  | InterDigital | Rel-13 | 36.211 | 13.1.0 | LTE_MTCe2_L1-Core |  | 0240 |  | 1 |  | F |  |
| R1-163677 | MDPCCH candidate overflow monitoring correction for eMTC |  | Nokia, Alcatel-Lucent Shanghai Bell, Sequans Communications, NEC, Ericsson | Rel-13 | 36.213 | 13.1.1 | LTE_MTCe2_L1-Core |  | 0633 |  | 2 |  | F |  |
| R1-163678 | Correction on PDSCH transmission timing for Rel-13 eMTC |  | LG Electronics, NEC | Rel-13 | 36.213 | 13.1.1 | LTE_MTCe2_L1-Core |  | 0638 |  | 1 |  | F |  |
| R1-163679 | Correction on RE mapping in MBSFN subframe for BL/CE UEs in CEModeB |  | CATT, Panasonic, Intel, Sony, Ericsson | Rel-13 | 36.211 | 13.1.0 | LTE_MTCe2_L1-Core |  | 0241 |  | 1 |  | F |  |
| R1-163689 | Identify MPDCCH order in CE mode B in TS 36.212 |  | Huawei, HiSilicon, CATT, MediaTek, Sierra Wireless | Rel-13 | 36.212 | 13.1.0 | LTE_MTCe2_L1-Core |  | 0202 |  |  |  | F |  |
| R1-163697 | Corrections to TS 36.213 for eMTC |  | Huawei, HiSilicon | Rel-13 | 36.213 | 13.1.1 | LTE_MTCe2_L1-Core |  | 0634 |  |  |  | F |  |
| R1-163698 | Corrections to TS 36.211 for eMTC |  | Huawei, HiSilicon | Rel-13 | 36.211 | 13.1.0 | LTE_MTCe2_L1-Core |  | 0238 |  |  |  | F |  |
| R1-163702 | Narrow band hopping |  | Qualcomm Incorporated, Ericsson, Nokia, Alcatel-Lucent Shanghai Bell, Intel | Rel-13 | 36.211 | 13.1.0 | LTE_MTCe2_L1-Core |  | 0239 |  |  |  | F |  |
| R1-163782 | Correction on CSS for MPDCCH configured by temporary C-RNTI and Type0 MPDCCH CSS resource |  | LG Electronics, Panasonic, Sequans | Rel-13 | 36.213 | 13.1.1 | LTE_MTCe2_L1-Core |  | 0636 |  |  |  | F |  |
| R1-163787 | Correction on collision of dynamically scheduled data and semi-statically scheduled data for Rel-13 eMTC |  | LG Electronics | Rel-13 | 36.213 | 13.1.1 | LTE_MTCe2_L1-Core |  | 0637 |  | 1 |  | F |  |
| R1-163790 | Collision between PSS/SSS/PBCH and MPDCCH/PDSCH for MTC |  | Ericsson, Samsung | Rel-13 | 36.211 | 13.1.0 | LTE_MTCe2_L1-Core |  | 0229 |  | 2 |  | F |  |
| R1-163794 | On the collision between eMTC SIB and MPDCCH/PDSCH in TS 36.213 |  | Huawei | Rel-13 | 36.213 | 13.1.1 | LTE_MTCe2_L1-Core |  | 0639 |  |  |  | F |  |
| R1-163796 | CR on missing CRI-only table (36.212) |  | Samsung | Rel-13 | 36.212 | 13.1.0 | LTE_EBF_FDMIMO-Core |  | 0203 |  |  |  | F |  |
| R1-163811 | Collision between PUCCH format 2 and PDSCH with repetitions |  | Qualcomm | Rel-13 | 36.213 | 13.1.1 | LTE_MTCe2_L1-Core |  | 0640 |  |  |  | F |  |
| R1-163812 | Clarification of TM1/2/6 on MBSFN subframes |  | Qualcomm | Rel-13 | 36.213 | 13.1.1 | LTE_MTCe2_L1-Core |  | 0641 |  |  |  | F |  |
| R1-163813 | Correction of fallback behavior for TM9 |  | Qualcomm | Rel-13 | 36.213 | 13.1.1 | LTE_MTCe2_L1-Core |  | 0642 |  |  |  | F |  |
| R1-163814 | Correction on RV and MPDCCH starting position |  | Qualcomm | Rel-13 | 36.213 | 13.1.1 | LTE_MTCe2_L1-Core |  | 0643 |  |  |  | F |  |
| R1-163831 | CR for the contention window adjustment procedure in LAA downlink channel access |  | Huawei, HiSilicon, LGE, WILUS, Nokia, Alcatel-Lucent Shanghai Bell, Broadcom | Rel-13 | 36.213 | 13.1.1 | LTE_LAA-Core |  | 0651 |  |  |  | F |  |
| R1-163844 | Correction on M-PDCCH case definition |  | InterDigital, Qualcomm, Panasonic, LG Electronics, Huawei, HiSilicon | Rel-13 | 36.213 | 13.1.1 | LTE_MTCe2_L1-Core |  | 0635 |  | 1 |  | F |  |
| R1-163858 | CR for TS36.211 related to 2+4 PRB set |  | Panasonic Corporation | Rel-13 | 36.211 | 13.1.0 | LTE_MTCe2_L1-Core |  | 0243 |  |  |  | F |  |
| R1-163859 | CR for TS36.213 related to 2+4 PRB set |  | Panasonic Corporation | Rel-13 | 36.213 | 13.1.1 | LTE_MTCe2_L1-Core |  | 0646 |  |  |  | F |  |
| R1-163925 | Initial CCA Behaviour in the Channel Access Procedure |  | Ericsson | Rel-13 | 36.213 | 13.1.1 | LTE_LAA-Core |  | 0649 |  |  |  | F |  |
| R1-163927 | CR on clarification for channel sensing |  | Qualcomm Incorporated | Rel-13 | 36.213 | 13.1.1 | LTE_LAA-Core |  | 0650 |  |  |  | F |  |
| R1-163930 | CR on UE assumptions on number of CRS ports in DRS |  | Qualcomm | Rel-13 | 36.211 | 13.1.0 | LTE_LAA-Core |  | 0244 |  |  |  | F |  |
| R1-163941 | Collision between PSS/SSS/PBCH and PDSCH for MTC |  | Ericsson | Rel-13 | 36.213 | 13.1.1 | LTE_MTCe2_L1-Core |  | 0644 |  | 1 |  | F |  |

# Annex C-1:	List of Outgoing LSs from RAN1 #84bis

| TDoc # | Title | Source | Release | Related WIs | Reply to | To | Cc | Original LS | Reply in |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1-163449 | LS on NB-IoT | RAN1, Huawei | Rel-13 | NB_IOT-Core |  | RAN4 |  |  |  |
| R1-163450 | TR for eMBMS enhancements for LTE | RAN1, Ericsson | Rel-14 | MBMS_LTE_enh2-Core |  | RAN2 | RAN3, RAN4 |  |  |
| R1-163453 | LS on SRS carrier switching interruption time | RAN1, Ericsson | Rel-14 | LTE_SRS_switch-Core |  | RAN4 |  |  |  |
| R1-163668 | LS on signalling of periodic CSI reporting for class A | RAN1, Samsung | Rel-13 | LTE_EBF_FDMIMO-Core |  | RAN2 |  |  |  |
| R1-163683 | LS on PUSCH transmission for eLAA | RAN1, Intel | Rel-14 | LTE_eLAA-Core |  | RAN4 |  |  |  |
| R1-163693 | LS on PC5 DMRS and L1 format for V2V | RAN1, Ericsson | Rel-14 | LTE_SL_V2V-Core |  | RAN4 |  |  |  |
| R1-163739 | LS on NB-IoT | RAN1, Huawei | Rel-13 | NB_IOT-Core |  | RAN4 |  |  |  |
| R1-163791 | LS on PRACH preamble power for eMTC | RAN1, Ericsson | Rel-13 | LTE_MTCe2_L1-Core |  | RAN2 | RAN4 |  |  |
| R1-163807 | LS on NB-IoT | RAN1, Huawei | Rel-13 | NB_IoT-Core |  | RAN2 |  |  |  |
| R1-163809 | LS on RRC parameter list for NB-IoT | RAN1, Huawei | Rel-13 | NB_IoT-Core |  | RAN2 |  |  |  |
| R1-163836 | LS on Potential Parameters for Blind Detection in MUST | RAN1, MediaTek | Rel-14 | LTE_MUST-Core |  | RAN4 |  |  |  |
| R1-163862 | LS on DL broadcast/multicast for V2X | RAN1, LGE | Rel-14 | FS_LTE_V2X |  | RAN2 |  |  |  |
| R1-163869 | Reply LS on Power Headroom report | RAN1, Huawei | Rel-13 | NB_IoT-Core | R1-163852 | RAN2 |  |  |  |
| R1-163871 | LS on Correction of UL DMRS sequences | RAN1, Ericsson | Rel-13 | NB_IoT-Core |  | RAN4 |  |  |  |
| R1-163906 | LS on resource allocation for V2V | RAN1, LGE | Rel-14 | LTE_SL_V2V-Core |  | RAN2 |  |  |  |
| R1-163907 | LS on V2X synchronization procedure | RAN1, NTT DOCOMO | Rel-14 | LTE_SL_V2V-Core |  | RAN2, RAN4 |  |  |  |
| R1-163915 | LS on UE/band specific support of UL 256QAM | RAN1, Nokia | Rel-14 | LTE_UL_CAP_enh-Core |  | RAN4 | RAN2 |  |  |
| R1-163922 | LS on TR update for latency reduction | RAN1, Ericsson | Rel-14 | FS_LTE_LATRED |  | RAN2 |  |  |  |
| R1-163928 | LS on PUSCH transmission for eLAA | RAN1, Qualcomm | Rel-13 | LTE_LAA-Core |  | RAN4 |  |  |  |
| R1-163931 | LS on DRS duty cycle | RAN1, Qualcomm | Rel-13 | LTE_LAA-Core |  | RAN2 | RAN4 |  |  |
| R1-163934 | LS on realistic power amplifier model for 5G waveform evaluation | RAN1, Huawei | Rel-15 | FS_NR_newRAT |  | RAN4 |  |  |  |
| R1-163937 | Reply LS on updated TS 36.300 section 5 for NB-IoT | RAN1, Huawei | Rel-13 | NB_IoT-Core | R1-161818 | RAN2 |  |  |  |
| R1-163746 | LS on V2X multicarrier configuration | RAN1, Ericsson | Rel-14 | LTE_SL_V2V-Core |  | RAN2, RAN4 |  |  |  |
| R1-163938 | Reply LS on RAN2 review of all LTE features up to Rel-13 for Category M1 UEs | RAN1, Ericsson | Rel-13 | LTE_MTCe2_L1-Core | R1-163686 | RAN2 |  |  |  |
| R1-163944 | Reply LS on RRM measurement for NB-IOT | RAN1, Huawei | Rel-13 | NB_IoT-Core | R1-163936 | RAN4 | RAN2 |  |  |

# Annex C-2:	List of Incoming LSs from RAN1 #84bis

| TDoc # | Title | Source | Release | Related WIs | Reply to | To | Cc | Original LS | Reply in |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1-163369 | Response LS on Paging for MTC | RAN2, Ericsson | Rel-13 | LTE_MTCe2_L1-Core | R2-153015 | RAN3 | CT1, SA2, RAN1 | R2-161788 |  |
| R1-163370 | Response LS on maximum UL Transmission timing difference in dual connectivity | RAN2, Huawei | Rel-12 | LTE_SC_enh_dualC-Core | R2-161020 | RAN4 | RAN1 | R2-161998 |  |
| R1-163371 | LS on A New PRACH Cyclic Shift Restriction Set | RAN4, Nokia | Rel-14 | LTE_high_speed |  | RAN1 |  | R4-161254 |  |
| R1-163372 | Reply LS on Pcmax definition of asynchronous overlapping transmissions in DC | RAN4, InterDigital | Rel-12 | LTE_SC_enh_dualC-Core | R1-153460 | RAN1 |  | R4-161325 |  |
| R1-163373 | LS on V2V Requirements | RAN, Qualcomm, LG Electronics | Rel-14 | LTE_SL_V2V-Core |  | RAN1, RAN4 |  | RP-160684 |  |
| R1-163374 | Reply LS on clarification of RSU types | SA1, LG Electronics | Rel-14 | FS_LTE_V2X | R1-157821 | RAN1 | RAN2, SA2 | S1-160521 |  |
| R1-163375 | Liaison Statement to 3GPP from IEEE 802 LMSC | IEEE 802 LMSC | Rel-13 | LTE_LAA-Core |  | RAN, RAN1 |  | IEEE 802.19-16/0037r9 |  |
| R1-163388 | Liaison Statement regarding LAA | Wi-Fi Alliance Coexistence Task Group | Rel-13 | LTE_LAA-Core | RP-152280 | RAN, RAN1 |  | 20160404 Wi-Fi Alliance LS on LAA |  |
| R1-163390 | Characteristics of terrestrial IMT systems for frequency sharing / interference analysis in the frequency range between 24.25 GHz and 86 GHz | RAN, NTT DOCOMO | Rel-15 | FS_NR_newRAT |  | RAN1, RAN4 |  | RP-160508 |  |
| R1-163395 | LS on path loss accuracy and sensitivity in 5G channel modeling | New York University Wireless | Rel-15 | FS_6GHz_CH_model |  | RAN1 | RAN |  |  |
| R1-163685 | LS to RAN1 on RAN2 agreements related to V2V | RAN2, Panasonic | Rel-14 | LTE_SL_V2V-Core |  | RAN1 |  | R2-163008 |  |
| R1-163686 | LS on reviewing all LTE features up to Rel-13 for Category M1 UEs | RAN2, NTT DOCOMO | Rel-13 | LTE_MTCe2_L1-Core |  | RAN1 |  | R2-163108 |  |
| R1-163852 | LS on Power Headroom report | RAN2, Huawei | Rel-13 | NB_IoT-Core | R1-162065 | RAN1 | RAN4 |  |  |
| R1-163896 | LS on NB-IoT RRM requirements | RAN2, Huawei | Rel-13 | NB_IoT-Core |  | RAN4 | RAN1 |  |  |
| R1-163936 | LS on RRM measurement for NB-IOT | RAN4, Ericsson | Rel-13 | NB_IoT-Core |  | RAN1 | RAN2 | R4-162881 |  |

# Annex D:	List of Approved updated WIDs

None

# Annex E:	List of draft TSs/TRs agreed at RAN1 #84bis

| Tdoc Number | Title | Source | Conclusion/Decision |
| --- | --- | --- | --- |

# Annex F:	List of actions

- Outgoing LS.

[84b-01] Johan (Ericsson)

R1-163913	[Draft] Reply LS on RAN2 review of all LTE features up to Rel-13 for Category M1 UEs   Ericsson

Email approval till 20th April

Done: Final LS is agreed in R1-163938 as per email decision posted by Mr Chair on April 22nd.

[84b-19] Matthew (Huawei)

R1-163936	LS on RRM measurement for NB-IOT	RAN4, Ericsson

Email approval until 21st April.

Done: Final LS is agreed in R1-163944 as per email decision posted by Mr Chair on April 26th.

- CR approval

[84b-02] Editors

NB-IoT CRs for email approval till 5th May

Editors (211,212,213, 214) to provide updates including agreements and working assumptions from RAN1#84bis by April 29.

Rapporteur (Asbjorn) will provide working assumptions and agreement lists.

Done: The list of NB-IoT agreements is recorded as R1-163943.

R1-163951	Introduction of Rel-13 feature of NB-IoT in 36.212	Huawei, HiSilicon	36.212	13.1.0	CR0192r2	B

R1-163952	Introduction of NB-IoT	Ericsson LM	36.211	13.1.0	CR0224r6	B

[84b-07] Editors

V2V CRs

Editors will submit 211, 212, 213 skeleton CRs until 20th May for information

Rapporteur (Hanbyul) will provide working assumptions and agreement lists.

Done: The list of V2V agreements is recorded as R1-163953.

[84b-18] Johan (Ericsson)

R1-163817	Collision between PSS/SSS/PBCH and PDSCH for MTC	Ericsson

Email approval till 21st April

Done: CR0644r1 to 36.213 is agreed in R1-163941 as per email decision posted by Mr Chair on April 22nd.

[84b-20] Srinivas (Qualcomm)

CR0627 on Clarification for LAA CSI processing, agreed in R1-163501 was written against the wrong version of the spec.

Prepare rev1 of the CR in R1-163945 to fix the specification issue - Email approval until 29th April.

Done: CR0627r1 to 36.213 is agreed in R1-163945 as per email decision posted by Mr Chair on May 11th.

- Text proposal for TS and TR

[84b-10] Hanbyul (LGE)

TP for V2X for email approval until 21st April, capturing the agreements/conclusions.

Done: Final version of the TP is agreed in R1-163940 as per email decision posted by Mr Chair on April 22nd.

- Miscellaneous

[84b-03] Xiaolei (HiSilicon)

Time domain resource allocation for ACK/NACK transmission for NB-IoT

Email approval until 21st April to decide between two alternatives

Done: Conclusion drawn on April 26th states that there is no consensus in RAN1 regarding the following alternatives:

- Alt 1 (supported by Huawei, HiSilicon, Panasonic, Intel, MediaTek, ZTE, Samsung) : Baseline time point for time domain allocation:

  - A/N transmission is relative to the first time reference point which is at least 12 ms later than the end of NPDSCH

  - The period of the time reference point is fixed to [X]ms.

    - X={2, FFS other values}

    - The start of the period is from (SFN#0, subframe#0).

- Alt. 2 (supported by Sony, LG, Qualcomm, Sierra Wireless, Nokia, ASB, Ericsson) : A/N transmission delay is relative to the end of the corresponding NPDSCH transmission plus 12ms.

[84b-04] Joon (LGE)

R1-163829	WF on further details of DCI contents for multi-TTI transmission	Qualcomm

R1-163830	WF on multi-TTI scheduling Mediatek

Above WFs are for email approval until May 5th

Done: According to Mr Chair’s email on May 10th , below proposals are agreed:

Agreements:

- MSF(multi-subframe scheduling) DCI at least includes the following information (in addition to previous agreements)

  - Scheduled subframes per MSF

    - FFS: details of how to decide/indicate scheduled subframes

  - Common MCS value for all the scheduled subframes

  - UL TPC per MSF

    - UL TPC applies for all subframes non-cumulatively in both accumulated and absolute modes

  - SRS triggering per MSF

  - Starting PUSCH DFT-S-OFDM symbol

    - FFS for details

  - Ending PUSCH DFT-S-OFDM symbol

    - FFS for details

  - Subframe timing for PUSCH transmission

    - FFS for details

  - FFS: LBT type (e.g., 25us LBT or Cat4 LBT, CWS, etc.)

  - FFS: details of signaling (e.g. joint coding to reduce the number of bits) for the above information

Working assumption:

- For LAA uplink, and for a given PUSCH TM, UE doesn’t need to detect MSF DCIs with different payload sizes for each TM.

  - FFS: configurability of maximum number of subframes where a MSF DCI can schedule

- For LAA uplink with single codeword, maximum number of HARQ processes = 16 are supported

- For LAA uplink with two codewords, supportable number of HARQ processes is doubled without introducing explicit HARQ process IDs. The same principles of handling two codewords in DL scheduling are applied for UL scheduling

[84b-05] Havish (Ericsson)

UCI transmission for eLAA

Email approval about FFS parts and confirm WA until 5th May

[84b-06] Sorour (Ericsson)

R1-163810	WF on UL Channel Access Schemes	Ericsson

Email approval about channel access until 5th May

[84b-08] Mark (Ericsson)

RMa channel model

Email approval until 28th April to revise RMa channel model

Done: According to Mr Chair’s email on April 29th , there is no consensus to replace the 2D RMa model with the 3D RMa model for fsemi-)open loop evaluations in eFD-MIMO.

[84b-09] David Bhatoolaul (Nokia)

Scenarios and simulation assumptions for eMBMS for email approval until 21st April

Done: According to Mr Chair’s email on April 22nd , use cases and corresponding simulation settings for eMBMS performance evaluation are agreed in R1-163939.

[84b-11] Wuyong (Huawei)

LLS assumptions for multiple access for NR

Email approval about LLS assumptions for multiple access until 28th April

Done: According to Mr Chair’s email on May 10th, Table 1 and Table 2 below are agreed:

Table 1 Evaluation parameters – LLS for UL

| Parameters | Values or assumptions |
| --- | --- |
| Carrier Frequency | 2 GHz |
| Waveform | OFDM /SC-FDMA Other waveform is not precluded |
| Channel coding | LTE Turbo as start point, other coding schemes are not precluded. |
| Numerology | Same as Release 13 |
| System Bandwidth | 10 MHz |
| Total allocated bandwidth for transmission | Companies need to report this value. |
| Overhead | 2 DMRS symbols, no SRS, i.e., 144 available RE per RB for data transmission, or equivalent overhead |
| Target spectral efficiency | Proponents report per UE spectral efficiency and the number of UEs multiplexed if multi-UEs LLS is assumed |
| BS antenna configuration | 2/4 Rx as baseline 8Rx optional |
| UE antenna configuration | 1Tx |
| Transmission mode | TM1 (refer to TS36.213) |
| SNR distribution of Multiple UEs | Proponents report if single-user or multi-user LLS is used, and what SNR distribution is assumed. |
| Suggested SNR distribution of multiple UEs | Equal average SNR (short-term variation remains) Unequal average SNR (the SNR distribution is FFS, e.g., uniformly distributed within a range of 3dB, and proponents should report their assumption) |
| Propagation channel & UE velocity NOTE2 | TDL for in TR38.900 as mandatory EPA, EVA, ETU as optional 3km/h, 30km/h, 120km/h |
| Max number of HARQ transmission | 1, 4 |
| Given BLER level (to calculate sum throughput) | 0.1 for 1 transmission as starting point, other numbers not precluded, e.g., 0.01 for 1 transmission |
| Overloading factor (Optional, definition refers to R1-163881) | Some example values: 100%, 150%, 200%, 300% |
| NOTE: Non-ideal effects (e.g., channel estimation, frequency offset) evaluation FFS. NOTE2: Companies could choose the propagation model for bringing evaluations at RAN1#85 Nanjing meeting, but companies are expected to provide evaluations at least for the channels listed in the table by August 2016. |  |

Table 2 Evaluation parameters – LLS for DL

| Parameters | Values or assumptions |
| --- | --- |
| Carrier Frequency | 2 GHz |
| Waveform | OFDM Other waveform is not precluded |
| Channel coding | LTE Turbo as starting point, other coding schemes are not precluded. |
| Numerology | Same as Release 13 |
| System Bandwidth | 10 MHz |
| Total allocated bandwidth for transmission | Companies need to report this value. |
| Overhead | 2 PDCCH symbols, 2 CRS ports for TM2 , i.e., 132 REs per RB for data transmission, or equivalent overhead |
| Target spectral efficiency | Proponents report per UE spectral efficiency and the number of UEs multiplexed if multi-UEs LLS is assumed |
| BS antenna configuration | 2/4 Tx as baseline 8Tx optional |
| UE antenna configuration | 2 Rx |
| Transmission mode | TM2 as starting point (refer to TS36.213) |
| SNR distribution of Multiple UEs | Fixed gap {0, 5, 10, 15, 20} dB between UEs |
| Number of UEs | 2 UEs as start point |
| SNR of the reference UE | 0dB (The SNR of the other UE would be 0, 5, 10, 15, or 20dB) |
| Power allocation between UEs | Dynamic |
| Propagation channel & UE velocity NOTE2 | CDL in TR38.900 as mandatory EPA, EVA, ETU as optional 3km/h, 30km/h, 120km/h |
| Max number of HARQ transmission | 1, 4 |
| Given BLER level (to calculate sum rate region) | 0.1 for 1 transmission as starting point |
| NOTE: Non-ideal effects (e.g., channel estimation, frequency offset) evaluation FFS. NOTE2: Companies could choose the propagation model for bringing evaluations at RAN1#85 Nanjing meeting, but companies are expected to provide evaluations at least for the channels listed in the table by August 2016. |  |

The following issues are not agreed, but companies are encouraged to consider these parameters.

1. Total allocated bandwidth for transmission for UL and DL

| Parameters | Values or assumptions |
| --- | --- |
| Total allocated bandwidth for transmission (UL) | Companies need to report this value. Some example values: 4RB (0.72MHz), 12RB (2.16MHz), 1RB (0.18MHz). Other values are not precluded. |
| Total allocated bandwidth for transmission (DL) | Companies need to report this value. Some example values: 4RB (0.72MHz), 12RB (2.16MHz) Other values are not precluded. |

2. Suggested target SE for UL

| Parameters | Values or assumptions |
| --- | --- |
| Suggested target spectral efficiency Definition: required transmission bits per user / total number of resource elements shared for data transmission | Companies need to report the values of “required transmission bits per user”, some example values are: For 4RB case (without CRC): 120 bits, 192 bits, 264 bits For 12RB case (without CRC): 408 bits, 624 bits, 840 bits Other values are not precluded. MCS can be selected by company to meet the target spectral efficiency. |

[84b-12] Wuyong (Huawei)

SLS assumptions for multiple access for NR

Email approval about SLS assumptions for multiple access until 9th May

[84b-13] Alexei (Intel)

Evaluation assumptions for NR

Email approval about all bracket and essential FFS parts in R1-163863, transmission power assumptions above 6 GHz, UE noise figure above 6 GHz, necessity to revise the number of TRP antenna elements, over-6GHz (70GHz) to “Up to 1024 Tx /Rx antenna elements”, necessity to revise the number of UE antenna elements, over-6GHz (70GHz) to “Up to 64 Tx /Rx antenna elements”, and UE antenna pattern (R1-163923)  until 28th April

[84b-14] Frank (Huawei)

MIMO mode and channel model parameters for NR for email approval until 21st April

Done: According to Mr Chair’s email on May 6th , below proposals are agreed:

Agreements:

- Proposal 1 on MIMO mode:

  - Companies should bring results for SISO at RAN1#85.

  - If companies bring results for MIMO, it is recommended to use at least one constant modulus precoding scheme in cases 1a, 1b and 2. Other precoding scheme is not precluded. Companies need to provide their CSI and precoding assumptions for MIMO evaluations. MIMO correlation matrices should be low correlation (i.e. uncorrelated) for RAN1#85 in case of MIMO simulations.

  - The detailed assumptions for MIMO mode will be decided at RAN1#85.

- Proposal 2 on Channel Model:

  - All values of DS {10, 30, 100, 300, 1000} ns are evaluated with the selected TDL-DS combinations, i.e. TDL-A for DS {10,30}ns, TDL-B for DS {100 }ns, TDL-C for DS {300,1000}ns. Companies are allowed to choose additional combination(s) of other DS values and TDL–A and/or TDL-C in TR38.900.

  - ETU/EPA/EVA are optional.

  - Mobility: 3km/h or 30 km/h or 120 km/h, higher speed is not precluded.

- Note: For RAN1#85: TDLs in TR38.900 or ETU/EPA/EVA can be used. Power-delay profiles of TDL-{A,B,C} are scenario agnostic.

[84b-15] Joseph (Qualcomm)

Frame structure for NR for email discussion until May 9th on the frame structure by focusing on understanding of DL/UL/Sidelink time domain structure containing RS, the assignment, the acknowledgement, and data. (Note that this email discussion will not try to have approval)

Done: Status to be checked at next meeting.

[84b-16] Younghan (Samsung)

R1-163642	WF on simulation assumption for full calibration 	Nokia, Samsung

Email discussion until  April 25th for large scale parameter calibration and detailed parameters for full calibration

Done: According to Mr Chair’s email on April 26th , there is no additional parameters necessary for full calibration. Proposed WF for full calibration is agreed.

[84b-17] Younghan (Samsung)

Full calibration for channel model

Email discussion until the next meeting for full calibration

# Annex G:	List of participants at RAN1 #84bis

Please see excel file attached to this report

# Annex H:	TSG RAN WG1 meetings in 2016 – 2017

| TITLE | TYPE | DATES | LOCATION | CTRY |
| --- | --- | --- | --- | --- |
| 3GPPRAN1#85 | WG | 23 – 27 May 2016 | Nanjing | China |
| 3GPPRAN1#86 | WG | 22 – 26 Aug 2016 | Goteborg | Sweden |
| 3GPPRAN1#86bis | WG | 10 – 14 Oct 2016 | Lisbon | Portugal |
| 3GPPRAN1#87 | WG | 14 – 18 Nov 2016 | TBD | US |
| 3GPPRAN1#88 | WG | 13 – 17 Feb 2017 | Athens | Greece |
| 3GPPRAN1#88bis | WG | 03 – 07 Apr 2017 | TBD | US |
| 3GPPRAN1#89 | WG | 15 – 19 May 2017 | TBD | China |
| 3GPPRAN1#90 | WG | 21 – 25 Aug 2017 | Berlin (TBC) | Germany |
| 3GPPRAN1#90bis | WG | 09 – 13 Oct 2017 | Prague (TBC) | Rep Czech |
| 3GPPRAN1#91 | WG | 27 Nov – 01 Dec 2017 | TBD | US |

| MEETING TYPES |  |
| --- | --- |
| AH = Ad Hoc | CM = Chairmen's meeting |
| JM = Joint | OR = Ordinary |
| PM = Preparatory Meeting | RG = Rapporteurs Group |
| RM = Resolution Meeting | SG = Steering Group |
| ST = Startup Meeting | TG = Task Group |
| WG = Working Group | XO = Extraordinary |

End of document