3GPP TSG-RAN WG1 #84bis  													                           R1-164151

# Nanjing, China, 23 – 27 May, 2016

Source: 	Intel Corporation

Title:	Evaluation of open loop schemes

Agenda item:	    6.2.3.3

Document for:	Discussion and Decision

# 1 Introduction

The agreed  eFD-MIMO WI [1] includes DMRS based open loop in its scope: “Evaluate and, if needed, specify enhancement to support DMRS-based open-loop transmission with the existing numbers of CSI-RS ports as well as the newly supported number of CSI-RS ports, in the following areas”. Open loop techniques have been widely used in legacy LTE networks such as TM 2/3. The main benefits of open loop MIMO compared to closed loop MIMO is its robustness in high Doppler scenario and scenarios in which reliable CSI feedback cannot be obtained. However, those legacy open loop TMs are CRS based and lack forward compatibility when network evolves towards DMRS based TMs. It maybe arguable whether we can simply use DMRS-based closed loop TMs as standard transparent open loop transmission, we still believe standardized DMRS-based has additional benefits in terms of performance and feedback overhead. In fact, we have contributed such techniques to 3GPP in a much earlier release [2]. This contribution provides some further evaluations of open loop and semi-open loop techniques based on the agreed simulation assumption in RAN1 #84b meeting [3].

# 2. Review of TM3

In TM3, the precoder for PDSCH with large delay CDD can be changed every v subcarriers when rank v is greater than one. The large delay CDD operation concatenates three matrices. The first matrix is a random beamformer selected from the 2Tx/4Tx codebook and the other two are CDD matrix and a rotation matrix, respectively. The precoding for large delay CDD spatial multiplexing is defined as (1).

where W(i) is changed per v subcarriers and D(i) is changed per subcarrier.

For every v contiguous subcarriers, the same random beamformer forms v beams. Within the v contiguous subcarriers, the CDD matrix D(i) cyclically change the mapping of v intermixed data symbols to the v beams formed by the random beamformer.  The per-subcarrier rotation of the effective precoder makes the channel qualities of the transport blocks close to each other. Thus UE only needs to feed back one CQI even when the rank is greater than one. For rank one open loop transmission in TM3, SFBC is used.

# 3. DMRS-based open loop SU-MIMO for rank > 1

When UE-specific RS is used for channel estimation, large delay CDD is not directly applicable. However, with some modifications as below, large delay CDD can be applicable to UE-RS.

For the UE-RS, only per RB based random beamforming is applied. The CDD matrix D(i) and the rotation matrix U will not be used for UE-RS precoding. That is:

where W(iRB) is the precoder matrix staying constant for each RB or RB bundling, and stands for the reference signal for different layer. W(iRB) changes every PRB or RB bundling. For data symbols in PDSCH, precoder  W(iRB), the D(i) matrix, and the U matrix are used for precoding. The matrix D(i) changes from subcarrier to subcarrier just as TM3 in Rel.8:

For 8 DMRS ports, W(iRB), D(i) and U can be defined similarly as 2 CRS and 4 CRS ports cases in TM3. Thus the eNodeB could cyclically assign different precoders W(iRB) to different RBs. In particular, the precoder W(iRB) can be selected from 8 ports codebook in the corresponding layers.  The matrix D(i) and U for different layers can be with the same as the current TM3 design for 2/4 antenna ports and can be extended to 8 antenna ports with similar structure.

# 4. DMRS-based open loop SU-MIMO for rank one

For rank one data transmission, we can define open loop SU-MIMO using either transmit diversity or per-RE based precoder cycling. In both cases, DMRS may be transmitted using 2 or 4 ports. For example, if the cycle of per-RE based precoder cycling is 2, 2 DMRS antenna ports can be used and each precoder is applied to one port. For transmit diversity, SFBC can be applied to a pair of 2 or 4 REs consecutive in the frequency domain. For per-RE precoder cycling, data REs are alternatively associated to multiple DMRS ports for channel estimation. Although it is well known that SFBC has better diversity effect than per RE based precoder cycling, per RE precoder cycling does not create any orphan REs which may not be suitable for SFBC transmission. Thus we slightly prefer per RE precoder cycling to SFBC for rank one transmission.

Proposal 1: If DMRS-based open loop transmission mode is introduced, pre RE precoder cycling is slightly preferred to transmit diversity such as SFBC for rank one transmission.

# 5. Performance evaluations

In this section, link level simulations are performed for open loop and semi-open loop transmissions. Detailed simulation assumptions can be found in [3] and also listed in the Appendix for easy reference. The simulation results are obtained by multiple trials with different fading channels generated based on 3D UMa aligned with 36.873 model. We chose TM9 closed loop MIMO as the baseline scheme in the comparison. For four antenna ports, we compared two additional schemes:

- Open loop with CDD: this scheme depicted in section 3, which applies per RB based random beamforming for both UE-RS and PDSCH and applies per subcarrier large delay CDD (i.e. D(i)U) for PDSCH.

- Open loop without CDD: this scheme applies only per RB based random beamforming for both UE-RS and PDSCH.

For 8 ports and 16 ports, we additionally evaluated semi-open loop scheme with a 2D antenna array setup. In the evaluated semi-open loop scheme, UE only feeds back i1,1 index and the precoder will change for each PRB with different i1,2 and i2 index.

Figure 1 to Figure 3 provide the link level performance comparison of the open loop and semi-open loop schemes for 4/8/16 Tx with 30km/h, 60km/h and 120km/h UE speed. For open loop with CDD, only single CQI is fed back. For open loop without CDD, TM9 like feedback is used. And eNB performs open loop transparently by scaling the CQI. For semi-open loop, UE feedback i1,1 index together with one/two CQI for rank one and above respectively. More simulation assumptions are listed in the appendix.

Figure 1: 4Tx, 2Rx, 30km/h (left), 60km/h (middle) and 120km/h (right)

Figure 2: 8Tx, 2Rx, 30km/h (left), 60km/h (middle) and 120km/h (right)

Figure 3: 16Tx, 2Rx, 30km/h (left), 60km/h (middle) and 120km/h (right)

From the link level simulation results, it can be observed that in all the three UE speed and antenna configurations, open loop with LD-CDD has the best link level performance. Thus, we propose:

Proposal 2: If DMRS-based open loop transmission mode is introduced, UERS based LD-CDD should be defined for rank 2-8.

# 6. Conclusion

In this contribution we propose a standardized solution for DMRS-based open loop SU-MIMO, which is an extension of TM3 in Rel.8. The proposed solution has better performance and less overhead than per RB based random beamforming. For operators who have already deployed TM3, it will provide a smooth migration path when the network is evolving from CRS based transmission modes to CSI-RS/UE-RS based transmission modes. We recap our proposals as:

Proposal 1: If DMRS-based open loop transmission mode is introduced, pre RE precoder cycling is slightly preferred to transmit diversity such as SFBC for rank one transmission.

Proposal 2: If DMRS-based open loop transmission mode is introduced, UERS based LD-CDD should be defined for rank 2-8.

# References

- RP-160623, New WID Proposal: Enhancements on Full-Dimension (FD) MIMO for LTE, Samsung

- R1-110976, Standardized UE-RS based open loop SU-MIMO, Intel Corporation

- R1-163832, WF on Evaluation Assumptions for DMRS based Semi-OL (Offline Agreement), Samsung

# Appendix

Table 1 Simulation Assumptions

| Parameters | Value |
| --- | --- |
| System bandwidth | 10MHz (50RBs) |
| Antenna configuration | 4/8/16 ports (other values of the number of ports are not precluded) 2 Rx antennas (cross-polarization) |
| Channel model | 3D UMa 500m |
| UE velocity | 30 km/h, 60 km/h, and/or 120km/h |
| Carrier Frequency | 2GHz |
| Overhead | 2 PDCCH symbols per sub-frame |
| Modulation, code rates | Based link adaptation |
| HARQ | HARQ with max 4 transmissions |
| Number of allocated PRBs | 4 PRBs |
| CSI feedback | Open loop with CDD: TM3 like feedback Open loop without CDD: TM9 like feedback Close loop: TM9 Semi-open loop: TM9 like feedback; Only feedback i1,1 |
| Feedback delay | 5 msec |
| Feedback periodicity | 5 and/or 10 msec |
| Channel estimation for demodulation and CSI estimation | Non-ideal |
| Receiver | MMSE-IRC |
| Rank adaptation | ON |
| Baseline | TM 9 based closed loop transmission |