3GPP TSG RAN WG1 Meeting #85                                                                            R1-164239

Nanjing, China 23rd - 27th May 2016

Source:	CATT

Title:	Discussion on scenarios and evaluation assumptions for CoMP enhancement

Agenda Item:	6.2.11.1

Document for:	Discussion and Decision

Introduction

The study item on further enhancement on CoMP was approved in the 3GPP RAN#71 meeting [1]. Objectives of the study item are to identify and evaluate performance benefits of enhancements related to CoMP schemes:

- Support of non-coherent joint transmission (JT) (e.g. support of MIMO layers transmission by the different transmission points in the single-user MIMO)

- Extension of beamforming and scheduling coordination (CS/CB) for Rel-13 FD-MIMO on the transmission points

In this contribution, deployment scenarios and assumptions for performance evaluation will be discussed.

Evaluation scenarios

- Generally two types of deployment scenarios can be considered: homogeneous deployment and heterogeneous deployment.

- For homogeneous deployment, 3D-UMa and 3D-UMi scenarios used in FD-MIMO study can be starting point. To evaluate dense deployment, 3D-UMa scenario with inter-site-distance (ISD) of 200m shall be considered. Two homogeneous scenarios are shown in Figure 1.

- Figure 1: Homogeneous deployment scenario (a) 3D-UMi (b) 3D-UMa with ISD = 200m

- In a heterogeneous deployment, there are two layers: macro layer and small cell layer. The reason for evaluating heterogeneous deployment is to evaluate the benefit of carrying out CoMP transmission between small cells as well as between macro cells and small cells. For co-channel heterogeneous deployment (Figure 2(a)), small cells and macro cells are deployed on the same center frequency. For non-co-channel (Figure 2(b)) heterogeneous deployment, small cells and macro cells operate on different center frequency. We consider 3.5GHz for small cell and 2GHz for macro cell in the evaluation. ISD between macro sites is 500m and antenna height of macro site is 25m. Antenna height of small cell is 10m. Considering antenna height and radius of macro cell and small cell, it is reasonable to use 3D-UMa for macro-UE link and 3D-UMi for small cell-UE link. Omni-directional small cell deployment is considered in the evaluation, and the dropping methodology of small cells can be same as what we used in small cell enhancement SI [2].

- Figure 2: Heterogeneous deployment scenarios (a) Co-channel scenario (b) Non-co-channel scenario

- Considering that CoMP enhancement studies should focus on joint transmission and CS/CB with FD-MIMO, the homogeneous deployment scenarios are slightly preferred against the heterogeneous scenarios. Therefore, heterogeneous scenarios could be considered as optional evaluation scenarios for enhanced CoMP studies.

Evaluation assumptions

Baseline for comparison

Scheme supported in Rel-13 that achieves the best performance shall be the baseline, including CoMP, FD-MIMO transmission and their combinations.

Antenna configuration

For non-coherent joint transmission study, 2Tx and 4Tx with 1D array shall be given higher priority since gain of non-coherent joint transmission is expected to diminish with the increasing of antenna number. As a second priority, more than 4Tx and 2D array could be evaluated.

For CS/CB study, 2D antenna array configuration in FD-MIMO study shall be used. CSI-RS port 8, 12, 16, and CSI-RS port defined in Rel-14 shall be considered.

Traffic model

Both full buffer and non-fuller-buffer traffic shall be considered in the evaluation. For full buffer simulation, 10 UEs are dropped per cell. Considering the pratical network operating point, the more realistic non-full-buffer model can be weighted more than the full buffer model.

The detailed evaluation assumptions for homogeneous and heterogeneous deployments scenarios are given in Table I and Table II, respectively.

Table I: Evaluation assumptions of homogeneous deployment scenarios

| Parameters | Values |  |
| --- | --- | --- |
| Scenarios | Scenario 1: 3D-UMa | Scenario 2: 3D-UMi |
| Tx power | 41 dBm | 41 dBm |
| Channel model | 3D-UMa | 3D-UMi |
| ISD | 200m | 200m |
| BS antenna height | 25m | 10m |
| Minimum BS-UE distance | 10m | 10m |
| Polarized antenna modeling | Model -2 from 36.873 [3] |  |
| BS Antenna configuration | For non-coherent joint transmission study: 2TX , 4Tx cross-polarized antenna with 0.5 wavelength spacing For CS/CB study: 2D antenna array configuration with (M, N, P, Q) = (8, 4, 2, 16), or (8, 4, 2, 32), or (8, 4, 2, 64) |  |
| Traffic model | Mandatory: FTP Model 1 with packet size 0.5 Mbytes (low ~20% RU*, medium ~50% RU, high ~70%RU), the number of UEs is variable and according to desired load for bursty Optional: Full buffer model *RU clarification: multiple SU or MU layers are not counted multiple times towards RU, max RU=100% |  |
| Wrapping method | Geographical distance based |  |
| Handover margin | 3dB |  |
| Metrics | Mean, 5%, 50% UPT for FTP traffic Cell average SE and cell edge user SE for full buffer traffic |  |
| System bandwidth | 10MHz (50 PRBs) |  |
| UE attachment | Based on RSRP (formula) from CRS port 0 |  |
| Carrier Frequency | 2GHz |  |
| Network synchronization | Synchronized |  |
| UE Speed | 3km/h |  |
| UE distribution | According to 36.873 |  |
| UE array orientation | ΩUT,a uniformly distributed on [0,360] degree, ΩUT,b = 90 degree, ΩUT,g = 0 degree |  |
| UE antenna pattern | Isotropic antenna gain pattern A’(θ’,ф’) = 1 |  |
| Receiver | Non-ideal channel estimation and interference modeling, detailed guidelines according to Rel-12 [71-12] assumptions |  |
|  | LMMSE-IRC receiver, detailed guidelines according to Rel-12 [71-12] assumptions |  |
| UE Rx antenna configuration | 2 Rx cross-polarized (0/+90); 4Rx cross-polarized(0/+90) with 0.5 wavelength spacing |  |

Table II: Evaluation assumptions of heterogeneous deployment scenarios

| Parameters | Values |  |  |  |
| --- | --- | --- | --- | --- |
| Scenarios | Scenario 3: Co-channel scenario |  | Scenario 4: Non-co-channel scenario |  |
|  | Macro cell | Small cell | Macro cell | Small cell |
| Carrier frequency | 2GHz | 2GHz | 2GHz | 3.5 GHz |
| Tx power | 46 dBm | 30 dBm | 46 dBm | 30 dBm |
| Channel model | 3D-UMa | 3D-UMi | 3D-UMa | 3D-UMi |
| ISD between macro site | 500m |  | 500m |  |
| BS antenna height | 25m | 10m | 25m | 10m |
| Minimum BS-UE distance | 35m | 10m | 35m | 10m |
| BS antenna configuration | For non-coherent JT study: 2TX , 4Tx cross-polarized antenna with 0.5 wavelength spacing For CS/CB study: 2D antenna array configuration with (M, N, P, Q) = (8, 4, 2, 16), or (8, 4, 2, 32), or (8, 4, 2, 64) | 2TX , 4Tx cross-polarized antenna with 0.5 wavelength spacing | For non-coherent JT study: 2TX , 4Tx cross-polarized antenna with 0.5 wavelength spacing For CS/CB study: 2D antenna array configuration with (M, N, P, Q) = (8, 4, 2, 16), or (8, 4, 2, 32), or (8, 4, 2, 64) | 2TX , 4Tx cross-polarized antenna with 0.5 wavelength spacing |
| Number of small cells per macro cell | 4, 10 |  | 4, 10 |  |
| Polarized antenna modeling | Model -2 from 36.873 [3] |  |  |  |
| Traffic model | Mandatory: FTP Model 1 with packet size 0.5 Mbytes (low ~20% RU*, medium ~50% RU, high ~70%RU), the number of UEs is variable and according to desired load for bursty Optional: Full buffer model *RU clarification: multiple SU or MU layers are not counted multiple times towards RU, max RU=100% |  |  |  |
| Wrapping method | Geographical distance based |  |  |  |
| Handover margin | 3dB |  |  |  |
| Metrics | Mean, 5%, 50% UPT for FTP traffic Cell average SE and cell edge user SE for full buffer traffic |  |  |  |
| System bandwidth | 10MHz (50 PRBs) |  |  |  |
| UE attachment | Based on RSRP (formula) from CRS port 0 |  |  |  |
| Network synchronization | Synchronized |  |  |  |
| UE Speed | 3km/h |  |  |  |
| UE distribution | According to 36.873 |  |  |  |
| UE array orientation | ΩUT,a uniformly distributed on [0,360] degree, ΩUT,b = 90 degree, ΩUT,g = 0 degree |  |  |  |
| UE antenna pattern | Isotropic antenna gain pattern A’(θ’,ф’) = 1 |  |  |  |
| Receiver | Non-ideal channel estimation and interference modeling, detailed guidelines according to Rel-12 [71-12] assumptions |  |  |  |
|  | LMMSE-IRC receiver, detailed guidelines according to Rel-12 [71-12] assumptions |  |  |  |
| UE Rx antenna configuration | 2 Rx cross-polarized (0/+90); 4Rx cross-polarized(0/+90) with 0.5 wavelength spacing |  |  |  |

Conclusions

In this contribution, we proposed deployment scenarios and evaluation assumptions for enhanced CoMP study. For deployment scenarios, the following four scenarios are suggested for the CoMP study:

•	Scenario 1: (Mandatory) 3D-UMa homogeneous scenario with ISD=200m

•	Scenario 2: (Mandatory) 3D-UMi homogeneous scenario with ISD=200m

•	Scenario 3: (Optional) Heterogeneous network with co-channel deployment

•	Scenario 4: (Optional) Heterogeneous network with non-co-channel deployment

The proposed simulation assumptions are summarized in Table I and Table II.

- References

- RP-160665, “New SID: Further enhancements to Coordinated Multi-Point Operation,” Göteborg, Sweden, March 7 - 10, 2016.

- 3GPP TR 36.872, “Small cell enhancements for E-UTRA and E-UTRAN - Physical layer aspects”.

    - y