3GPP TSG RAN WG1 Meeting #85	                                                         R1-164270

Nanjing, China, 23rd – 27th May 2016

# Source: 	ZTE

# Title:	Receiver implementation for MUSA

# Agenda Item:	7.1.3.2

# Document for:	Discussion

# Introduction

Grant-free access can significantly reduce the overhead and power consumption for massive low cost terminals with small packet transmission thus is preferred in massive machine type communication [1-5]. A grant-free accessing scheme naming Multi user shared access (MUSA) for mMTC was proposed on last meeting [6]. An m-ary complex value spreading sequence with small length is applied in MUSA, which makes the high overload grant-free access come true.

As it is shown in [6,7] excellent performance can be achieved by MUSA by utilizing small length m-ary complex spreading codes and ideal MMSE-SIC receiver. Actually, in [6,7] ideal receiver are used to exclude the effects of receiver and shown the performance bound of various spreading codes. With the help of ideal receiver, the grant-free accessing users’ data can be successfully decoded by the base station (BS) even in highly overloaded situation.

In this contribution, realistic receiver for grant-free MUSA is discussed. In grant-free access, the important assumptions for ideal receiver, such as the active users’ spreading codes and their fading channel may not be easily known by the BS before the decoding, since some procedures are omitted in the access to save the signalling overhead and power consumption. Fortunately, taking full advantage of the characteristic of grant-free accessing, e.g. inherent random near-far phenomenon and thanking for MUSA’s special features, e.g. short spreading code, blind multi-user detection (MUD) with very high performance would come true. Here blind MUD means that, BS can separate all the accessing users’ data without the a-prior knowledge of the number of active users, users’ spreading code and users’ fading channel. Some initial evaluation results are provided here for discussion.

# Non-orthogonal multiple access receivers for mMTC

The key KPI for mMTC is the huge number of connections, e.g. the target for connection density should be a million devices per square kilometer in urban environment [8, 9].

The idea of spreading is refined in non-orthogonal access for massive connectivity. The non-orthogonal resource allocation facilitates grant-free transmission so that the system would not be strictly limited by the amount of available resources and their scheduling granularity. Several non-orthogonal multiple access receivers for uplink are described in the following: MUSA, RSMA, SCMA and PDMA.

- MUSA

The received signal of MUSA can be written as

y=i=1Nhisixi+ni

here hi is the channel of the i-th user,  si is the spreading code of the i-th user and it a column vector, xi is the modulated symbol of the i-th user,  ni is the additional white Gaussian noise. N is the number of the total users.

According to the user channels and the spreading codes, the equivalent channel is obtained as H=h1s1,h2s2,…hNsN. The MMSE weight can be analytically calculated as

W=H*(HH*+σ2I)-1

Here ()* represents the hermitian operation. The post-SINR of each user can also be analytically calculated after we know H. We assume the i-th user has the highest post-SINR, then the i-th user signal can be estimated by

xi=wi y

After channel decoding and CRC check success, the signal is reconstructed and its effect is cancelled from the received signal. Then the next user signal is detected, and so on until all users’ signals are detected.

- RSMA

The user data is spread on time, frequency, code and spatial domain. Multiple users share the same resource block. To recovery each user’s data, for single carrier RSMA, matched filter can be applied in low SNR regions. While in high SNR regions, advanced receiver, such as SIC based receiver may be used. For RSMA + FDMA, advanced receiver is needed too.

- SCMA

It is a type of code division scheme using spreading at the transmitter and MAP processing at the receiver. SCMA uses low density or sparse non-zero element sequence to reduce the complexity of MPA processing [10-12].

The optimal MUD can be done by using the maximum joint a posteriori probability (MAP) detection with excessive search. With the low density spreading structure employed in SCMA, near ML performance MUD can be derived by using message passing algorithm (MPA). The SCMA receiver is shown in Fig. 1.

Figure 1 SCMA receiver

- PDMA

The code matrix design for PDMA is to make the user to have unbalance post-SINR at receiver side to facilitate its signal detection [13, 14]. PDMA can be detected by SIC receiver or MAP detector. The former has low complexity while the latter has potentially better performance. Thus it is flexible in terms of receiver complexity. When SIC type receiver is applied, the signal with the largest post-SINR is detected first and then its interference is removed from the received signal in order to detect the other user’s signal.

Observation 1: SIC type receiver is preferred by most non-orthogonal multiple access scheme due to its moderate implementation complexity at receiver.

# Performance evaluation for MUSA

Here we provide performance evaluation for grant-free MUSA uplink by using blind MUD.

## Receiver design for MUSA

Multi-user link level simulations are conducted to evaluate the performance of the proposed MUSA scheme [6,7].

Because MUSA is a type of grant-free access schemes, the user terminal should be able to autonomously access the system without BS scheduling. Each user terminal chooses or generates its spreading code randomly, so in the MUD for MUSA, the number of active user, user spreading code and user channel would be unknown to the BS at the beginning, making it difficult to determine the optimal MMSE weights. This key channel related information can be obtained by other ways, such as by using long preamble sequences. However, the overhead for long preamble reduces system efficiency.

We prefer to use blind MUD in MUSA. Long preambles are not employed in MUSA to obtain the number of active user number, user spreading code and user channel.  Blind MUD is characterized as:

- Number of active user is not known at BS.

- Spreading code of each user is not known at BS.

- Channel of each user is not known at BS.

Short complex spreading code used by MUSA [6] allows efficient operation of blind MUD and can achieve high performance. Because the spreading code length is short and its element has limited values, BS can generate large number of local spreading codes with low correlation. With these local spreading codes and the received signal, the performance can very closely approach the optimal MMSE estimator. Such procedure is called blind estimation.

The blind MUD in MUSA including followings three key components

- SIC type receiver

SIC type receiver can take full advantage of the near-far phenomenon usually observed in grant-free systems. And compared with MAP type receiver, SIC type receiver has moderate complexity with acceptable performance.

- Blind estimation

MMSE has the optimal estimating performance but requires the much information which cannot be a prior known before the estimation in grant-free access. Fortunately, by making full use of the characteristics of the spreading codes and the received signal, some estimators can be constructed blindly whose performance is very close to optimal MMSE estimator.

With blind estimation for the user with the highest post-SINR in current SIC stage, the decoding performance of the detected user can be guaranteed. With the help of the following advanced channel estimation and SIC, the post-SINR of the next detected user signal is enhanced and can be successfully decoded.

- Blind and advanced channel estimation

Long preambles are not needed in the channel estimation to save the overhead. Furthermore, even the pilot /reference signal can be saved in some scenarios. The successfully decoded data can be employed as known signal/data pilot for channel estimation. As more users’ signals are successfully decoded, more signals become known and can be used for channel estimation, and therefore more accurate channel estimation results can be achieved. It is worth mentioning that the more accurate channel estimation are very beneficial and necessary for the detection of the weaker users in grant-free access with inherent random near-far problem [5].

The simulation parameters are shown in Table 1. For MUSA, eight physical resource blocks (PRB) are used by multiple users in a grant-free manner.  Bi-level random complex spreading code is used with length of 4. Here, the user load is defined as the ratio of the number of users to the spreading factor. For MUSA, each modulated symbol is spread by a random complex spreading code of length L to be transmitted on L time and/or frequency resources. If the number of users is K, user load can be defined as K/L. And user overloading occurs when K is more than L.

Fig.2 shows the BLER performance of LTE and MUSA with different user load. From the simulation results, it can be observed that the BLER of MUSA, with blind SIC receiver, is smaller than 0.1 even when the user load gets as high as 300%.

Figure 2 BLER performance of MUSA with different user load by using blind MUD

Table 1 Simulation parameters for MUSA blind detection

| Parameters | Values or assumptions |
| --- | --- |
| Carrier Frequency | 2 GHz |
| Waveform | OFDM /SC-FDMA Other waveform is not precluded |
| Numerology | Same as Release 13 |
| System Bandwidth | 10 MHz |
| Total BW for transmission | 180KHz |
| Waveform | OFDM |
| Target spectral efficiency | Proponents report per UE spectral efficiency and the number of UEs multiplexed if multi-UEs LLS is assumed |
| BS antenna configuration | 1 Rx |
| UE antenna configuration | 1Tx |
| Transmission mode | TM1 (refer to TS36.213) |
| SNR distribution of Multiple UEs | 4 to 20 dB |
| Suggested SNR distribution of multiple UEs | Uniform |
| Propagation channel & UE velocity | Flat Rayleigh fading |
| Max number of HARQ transmission | No HARQ |
| Given BLER level (to calculate sum throughput) | 0.1 to 0.001 |
| Overloading factor | 100%, 150%, 200%, 250%, 300% |
| Modulation | BPSK |
| Length of spreading factor | 4 |
| Spread code | Random |
| Information bits for each user | 144 info bits+24CRC = 168bits |
| Channel estimation | Real |
| Channel coding | Convolutional code with 1/2 rate |
| Receiver | Blind Codeword-level SIC |

# Conclusion

In this contribution, MUSA scheme is put in realistic grant-free setting where eNB receiver has no knowledge of the user’s channel and the spreading code. Blind multiuser detection (MUD) is described and the performance of MUSA with realistic blind MUD is presented. We have the following observations:

Observation 1: BS does not need to know the spreading code applied by the active UT. Blind estimation can be applied to closely approximate the optimal MMSE.

Observation 2: Dedicated reference signal is not necessary for channel estimation. The successfully decoded data can be applied as data pilot for channel estimation in SIC.

Observation 3：Large user overload can be supported by using blind MUD for MUSA.

Proposal 1: MUSA with blind MUD employed can be applied for mMTC scenario to support massive user connection and reduce the signalling overhead and UT power consumption.

# References

- 3GPP RP-160351, New SID Proposal: Study on New Radio Access Technology, NTT DOCOMO, RAN #71.

- 3GPP, RP-160279, Motivation of Uplink Non-orthogonal Transmission for Massive MTC, ZTE, RAN #71.

- 3GPP, R1-163111, Initial views and evaluation results on non-orthogonal multiple access for NR uplink, NTTDOCOMO.

- 3GPP, R1-162202  Candidate NR Multiple Access Schemes, Qualcomm.

- 3GPP, R1-164268, Grant-based and grant-free multiple access for mMTC, ZTE

- 3GPP, R1-162226, Discussion on multiple access for new radio interface, ZTE.

- Zhifeng Yuan, Guanghui Yu, Weimin Li, Yifei Yuan, Xinhui Wang, Multi-User Shared Access for Internet of Things,  accepted by IEEE VTC2016-Spring Conference.

- 3GPP TR 38.913 V0.3.0, “Study on Scenarios and Requirements for Next Generation Access Technologies (Release 14)”, 2016-03.

- 3GPP, R1-162224, Evaluation Assumptions for NR in RAN1, ZTE.

- M. Taherzadeh, H. Nikopour, A. Bayesteh, SCMA codebook design, in Proc. IEEE VTC Fall 2014, Sep. 2014, pp. 1–5.

- 5G White Paper V2.0, Part D - Alternative Multiple access v1, FuTURE MOBILE COMMUNICATION FORUM, http://www.future-forum.org/2009cn/.

- H. Nikopour and H. Baligh, Sparse code multiple access, in Proc. IEEE PIMRC 2013, Sep. 2013, pp. 332–336.

- 3GPP, R1-162306  Candidate Solution for New Multiple Access, CATT.

- X. Dai et al., “Successive Interference Cancelation Amenable Multiple Access (SAMA) for Future Wireless Communications,” Proc. IEEE ICCS 2014, pp. 222–226, Nov. 2014.