3GPP TSG RAN WG1 Meeting #85		R1-164361

Nanjing, China 23rd - 27th May 2016

Source:	Ericsson

Title:	Turbo Code Enhancements

Agenda Item:	7.1.5.1

Document for:	Discussion and Decision

# Introduction

In RAN#71, a new study item, “Study on New Radio Access Technology,” has been approved. The initial work of the study item is expected to focus on fundamental physical layer signal structure for new RAT, of which channel coding schemes is listed as an area to investigate.

In RAN1#84bis meeting, the following was agreed with regard to channel coding candidates for NR.

Agreements: - Candidates for 5G new RAT data transmission are identified as the following - LDPC code - Polar code - Convolutional code (LTE and/or enhanced convolutional coding) - Turbo code (LTE and/or enhanced turbo coding) - Note: It is RAN1 common understanding that combination of above codes is not precluded - Note: Outer erasure code is not precluded - Selection of 5G new RAT channel coding scheme(s) will consider, - Performance - Implementation complexity - Latency (Decoding/Encoding) - Flexibility (e.g., variable code length, code rate, HARQ (as applicable for particular scenario(s)))

In this paper, we discuss the enhancements to the 3GPP LTE turbo code for NR.

# Lower Code Rate to R=1/5

In LTE, the turbo code has mother code rate 1/3 with transfer function of

G(D) =

To lower the mother code rate to 1/5, a second parity bit is added for each of the two constituent codes, so that 5 bits are generated for each info bit (here the tail effect is ignored).  The transfer function of the 8-state constituent code for the PCCC is modified to:

G(D) =

where

g0(D) = 1 + D2 + D3,

g1(D) = 1 + D + D3,

g2(D) = 1 + D+ D2 + D3.

Note that this is the same transfer function adopted by 3GPP2. The rate 1/5 turbo code structure is illustrated in Figure 1, where yk and y k correspond to g2(D).

A simple and backward compatible extension to the circular buffer to support the rate 1/5 code can be adopted correspondingly. That is, simply append the existing circular buffer with the multiplexed bits of g2(D) from the two component codes. More specifically, the composition of the extended circular buffer is as follows:

- The first part contains 32 columns of systematic bits xk;

- The second part contains 64 columns of bit-multiplexed coded bits from g1(D) of the two component codes, zk and z k.

- The third part contains 64 columns of bit-multiplexed coded bits from g2(D) of the two component codes, yk and y k.

- Lower mother code rate of LTE turbo code to R=1/5 by introducing an additional parity bit to the constituent code.

Figure 1. Structure of rate 1/5 turbo encoder (dotted lines apply for trellis termination only).

# Increased Info Block Size K

In LTE, the info block size K ranges from 40 bits to 6144 bits, where K is used for turbo encoding of one code block. Each K value has its corresponding QPP interleaver, which is defined by the parameter pair {f1, f2}.

For NR, if the info block sizes K is to be increased, then new QPP interleavers need to be defined for the new K values. In Table 1 below, new K values ranging from 6208 to 8192 are proposed with K granularity of 64 bits. In Table 1, the K=6144 entry is copied from existing QPP interleaver in LTE to show the extension of K. For each new K value, the QPP interleaver parameters {f1, f2} is provided.

- Increase info block sizes of turbo code and define QPP interleaver for each of the new sizes.

Table 1. QPP interleavers for K=6208 to K=8192.

| K |  |  |
| --- | --- | --- |
| 6144 | 263 | 480 |
| 6208 | 49 | 194 |
| 6272 | 97 | 224 |
| 6336 | 65 | 132 |
| 6400 | 49 | 100 |
| 6464 | 51 | 202 |
| 6528 | 25 | 102 |
| 6592 | 51 | 206 |
| 6656 | 51 | 104 |
| 6720 | 53 | 210 |
| 6784 | 27 | 106 |
| 6848 | 53 | 214 |
| 6912 | 53 | 108 |
| 6976 | 55 | 218 |
| 7040 | 27 | 110 |
| 7104 | 55 | 222 |
| 7168 | 55 | 112 |
| 7232 | 57 | 226 |
| 7296 | 29 | 114 |
| 7360 | 57 | 230 |
| 7424 | 57 | 116 |
| 7488 | 77 | 156 |
| 7552 | 29 | 118 |
| 7616 | 59 | 238 |
| 7680 | 59 | 120 |
| 7744 | 31 | 352 |
| 7808 | 31 | 122 |
| 7872 | 61 | 246 |
| 7936 | 61 | 124 |
| 8000 | 31 | 160 |
| 8064 | 83 | 168 |
| 8128 | 63 | 254 |
| 8192 | 33 | 64 |

# Other ways to Enhance Turbo Code

In addition to lower mother code rate and large info block sizes, other enhancements can be considered for turbo code. These include:

- Decrease the code rate further by introducing a third constituent code, and correspondingly a second turbo interleaver. For example, adding a third constituent code in Figure 1 lowers the code rate to 1/7. This is beneficial for further lowering the error floor of turbo code.

- Increase the number of states of the constituent code. Instead of 8-state convolutional code, 16-state convolutional code can be considered as a constituent code. This is also beneficial for further lowering the error floor of turbo code.

- Duo-binary instead of binary input. This is beneficial for improving the data throughput of the turbo decoder.

While enhancements proposed in Section 2 and 3 substantially reuse the LTE turbo code, the potential enhancements listed above involve much bigger change to the LTE turbo code. It should be carefully investigated if benefits of these enhancements outweigh the cost of redesign the turbo code.

# Conclusion

In this contribution, we discuss potential enhancements to the LTE turbo code. Based on the discussion, we have the following proposals:

- Lower mother code rate of LTE turbo code to R=1/5 by introducing an additional parity bit to the constituent code.

- Increase info block sizes of turbo code and define QPP interleaver for each of the new sizes.

# References

RP-160671, “New SID Proposal: Study on New Radio Access Technology,” NTT DOCOMO, 3GPP TSG RAN Meeting #71, Göteborg, Sweden, 7.-10. March, 2016.

3GPP TS 36.212 v13.0.0.

R1-163232, “Performance study of existing turbo codes and LDPC codes,” Ericsson.

R1-164358, “Performance Evaluation of Turbo Codes and LDPC Codes at Lower Code Rates,” Ericsson.

R1-164359, “Performance Evaluation of Turbo Codes and LDPC Codes at Higher Code Rates,” Ericsson.

R1-165036, “Impact of Number of Iterations in Decoding Turbo and LDPC codes,” Ericsson.