3GPP TSG RAN WG1 Meeting #85	  				  	R1-164553

Nanjing, China, 23rd - 27th May 2016

______________________________________________________________________Agenda item: 7.1.2

Source: LG Electronics, Deutsche Telekom

Title: Evaluation assumptions for eV2X

Document for: Discussion and Decision

- Introduction

In 3GPP RAN plenary #70 agreed NR study item in TR38.913 which contains eV2X study. In the previous RAN1 meeting, the following agreements are made for the evaluation assumptions for the high speed train:

Agreed high speed train assumptions in [1]

Continue discussions until RAN1 #85 meeting about highway and urban grid scenarios

In this contribution, the evaluation assumptions for eV2X in NR are proposed, which is shown in Appendix.

- Performance metric and evaluation methodology

For the performance metric, we can consider a new metric which is communication availability and resilience (CAR) as shown in the TR38.913. It will be very complicated evaluation work if the CAR is simulated in a single system level simulator where multiple UEs are dropped and conduct the mobility management procedure and the packet transmission/reception procedure at the same time. So it is proposed to derive CAR from the two components of the packet loss, each of which can be evaluated separately: A packet loss will happen

- if interruption happens in the eV2X message transmission/reception, e.g., during the mobility management procedure and the interruption lasts for the latency requirement, or

- if the eV2X message transmission/reception takes place without any interruption but the actual packet delivery within the latency requirement failed due to, interference, noise, etc.

Considering these two packet loss components, CAR can be approximated as follows:

CAR = (1-X)*(1-Y)

where X is the portion of time duration during which the communication for eV2X message transmission is not available, and Y is the error probability of delivering eV2X message to the recipient when the communication is available. We can call 1-X the communication availability, and 1-Y can be the packet reception ratio.

For the evaluation of sidelink, X, which determines the communication availability, is assumed to be zero, since it is possible to avoid interruption in SL transmission and reception even when some link problem happens in UL/DL. An example can be the exceptional case operation introduced in Rel-12 D2D. X can be set to zero for the evaluation of DL multicast/broadcast, because even a UE having no network connection is able to receive multicast/broadcast transmissions. For the UL/DL unicast case, X needs to be via a separate simulation similarly to the study on HetNet mobility (TR36.839), but in this case, a single UE can be dropped in the entire cell layout as far as the inter-cell interference level is set to a reasonable value (e.g., 50% resource usage in all the cells).

The parameter Y can be evaluated similarly to the LTE V2X cases. For the multicast/broadcast case, the definition of PRR in TR 36.885 can be reused. For the uncast case, PRR needs to be evaluated only for the target receiver, so one-to-one paring is needed between transmitter and the receiver. In the sidelink unicast case, for example, the receiver UE can be chosen within a range from the transmitter UE.

- Details of evaluation assumptions

- Layout

We can think about two evaluation scenarios which have been used also in LTE V2X: Urban grid and highway scenarios where hexagonal grid or straight line model with road configuration can be used for macro BS as shown in TR38.913, TR36.885. At the beginning, we first simulate the system without RSU.

Afterwards, On top of macro BS, road side unit (RSU) also can be considered for the deployment scenario to support communications among vehicles. RSU is expressed in TR38.913 and defined as the logic entity as written in LS [2] from SA1: ("A stationary infrastructure entity supporting V2X applications that can exchange messages with other entities supporting V2X applications."). For RSU, two types of equipment can be defined separately: BS with a lower power than macro BS (BS-type-RSU) and a stationary UE (UE-type-RSU).

RSU is dropped at each intersection in hexagonal grid, and is also uniformly allocated with 100m spacing in the middle of the highway in straight line model. For the straight line model, considering cell size, it seems to be proper for RSU to be installed where macro BS is deployed with 1732m.

The evaluation firstly can be started without RSU, afterwards, we can see in the simulation with RSU how much additive gains are obtained adding RSU.

- Channel model

Channel model for the eV2X evaluation can be simplified as reusing 2D channel model used in LTE V2X evaluation assumptions. For the case of channel model between vehicle/pedestrian UE and RSU to/from vehicle/pedestrian UE, V2X channel model in TR36.885 can be reused. However, if considering vertical beamforming by MIMO application, it is better that the channel for macro to/from vehicle/pedestrian UE is assumed to be 3D channel model.

In the case of 1732m of inter macro, 2D channel model can simply be adopted. However, if vertical antenna elements are considered, it may not be natural to use 2D channel model. In this case, it is FFS whether 3D channel model is used or not.

- Conclusion

In this contribution, we proposed evaluation assumptions for eV2X as shown in Appendix.

______________________________________________________________________

Reference

[1] R1-163887, “Updated evaluation assumptions for NR High speed scenario”, LG Electronics.

[2] R1-163374, “Reply LS on clarification of RSU types”, LG Electronics.

- Appendix. Evaluation assumptions for eV2X

| Parameters | Urban grid for eV2X | Highway for eV2X |
| --- | --- | --- |
| Layout | Option 1: Macro only (with the road configuration in Figure 6.1.10-1 in TR38.913) Option 2: Macro + RSUs (with the road configuration in Figure 6.1.10-1 in TR38.913) Note: An RSU can be a BS with a lower power than macro BS or a stationary UE. It can be considered to replace RSU with LPN (low power node) for the consistency with SA definition on RSU ("A stationary infrastructure entity supporting V2X applications that can exchange messages with other entities supporting V2X applications.") | Option 1: Macro only ([straigntline eNB placement with Road configuration in TR36.885]) Option 2: Macro only ([Hex. Grid with Road configuration in TR36.885]) Option 3: Macro + RSUs ([straightline eNB with Road configuration in TR36.885]) Option 4: Macro + RSUs ([Hex. Grid with Road configuration in TR36.885]) Note: An RSU can be a BS with a lower power than macro BS or a stationary UE. It can be considered to replace RSU with LPN (low power node) for the consistency with SA definition on RSU ("A stationary infrastructure entity supporting V2X applications that can exchange messages with other entities supporting V2X applications.") |
| Inter-BS distance | Inter Macro: 500m Inter RSU: RSU is dropped at each intersection | Inter Macro: 1732m for option 1, 3 in layout 500m for option 2, 4 in layout Inter RSU: for options 3 and 4 In layout, uniform allocation with 100m spacing in the middle of the highway |
| Carrier frequency | Macro to/from vehicle/pedestrian UE : 4 GHz Between vehicle/pedestrian UE: 4 GHz or 6 GHz BS-type-RSU to/from vehicle/pedestrian UE : 4 GHz UE-type-RSU to/from vehicle/pedestrian UE: 6 GHz | Macro to/from vehicle/pedestrian UE : 4 GHz Between vehicle/pedestrian UE: 4 GHz or 6 GHz BS-type-RSU to/from vehicle/pedestrian UE : 4 GHz UE-type-RSU to/from vehicle/pedestrian UE: 6 GHz |
| Aggregated system bandwidth | Up to 200 MHz (DL+UL) Up to 100 MHz (SL) | Up to 200 MHz (DL+UL) Up to 100 MHz (SL) |
| Simulation bandwidth | 20 or 40 MHz (DL+UL) 10 or 20 MHz (SL) | 20 or 40 MHz (DL+UL) 10 or 20 MHz (SL) |
| Channel model | Macro to/from vehicle/pedestrian UE : 3D UMa Between vehicle/pedestrian UE: V2X Channel model in TR36.885 RSU to/from vehicle/pedestrian UE : V2X Channel model in TR36.885 | Macro to/from vehicle/pedestrian UE: 2D RMa for option 1, 3 in layout 3D UMa for option 2, 4 in layout Between vehicle/pedestrian UE: V2X Channel model in TR36.885 RSU to/from vehicle/pedestrian UE : V2X Channel model in TR36.885 Note: For option 1, 3 in layout, it is FFS whether 3D channel model is considered. |
| Tx power | Macro BS: 49dBm PA scaled with simulation BW when system BW is higher than simulation BW. Otherwise, 49dBm [TBD for TDD] BS-type-RSU: 24dBm PA scaled with simulation BW when system BW is higher than simulation BW. Otherwise, 24dBm Vehicle/pedestrian UE or UE type RSU: 23dBm Note: 33dBM for RSU or UE is not precluded | Macro BS: 49dBm PA scaled with simulation BW when system BW is higher than simulation BW. Otherwise, 49dBm [TBD for TDD] BS-type-RSU: 24dBm PA scaled with simulation BW when system BW is higher than simulation BW. Otherwise, 24dBm Vehicle/pedestrian UE or UE type RSU: 23dBm Note: 33dBM for RSU or UE is not precluded |
| BS antenna configuration | Macro BS: Up to 256 TX/RX antenna elements BS-type-RSU: Up to 8 TX/RX antenna elements | Macro BS: Up to 256 TX/RX antenna elements BS-type-RSU: Up to 8 TX/RX antenna elements |
| BS antenna pattern | Macro BS: Follow the modeling of TR 36.873 BS-type RSU: Follow the modeling of TR 36.873 | Macro BS: Follow the modeling of TR 36.873 BS-type RSU: Follow the modeling of TR 36.873 |
| BS antenna height | Macro BS: 25m BS-type-RSU: 5m | Macro BS: 35m for option 1, 3 in layout 25m for option 2, 4 in layout BS-type-RSU: 5m |
| BS antenna element gain + connector loss | Macro BS: 8 dBi BS-type-RSU: 8dBi | Macro BS: 8 dBi BS-type-RSU: 8dBi |
| BS receiver noise figure | Macro BS:5dB BS-type-RSU: 5dB | Macro:5dB BS-type-RSU: 5dB |
| UE antenna elements | Vehicle/pedestrian UE: Up to 8 TX/RX antenna elements UE-type RSU: Up to 8 TX/RX antenna elements | Vehicle/pedestrian UE: Up to 8 TX/RX antenna elements UE-type RSU: Up to 8 TX/RX antenna elements |
| UE antenna pattern | Vehicle/pedestrian UE: Half spherically uniform distribution with upper direction UE-type-RSU: Half spherically uniform distribution with bottom direction Note: directional antenna pattern is not precluded | Vehicle/pedestrian UE: Half spherically uniform distribution with upper direction UE-type-RSU: Half spherically uniform distribution with bottom direction Note: directional antenna pattern is not precluded |
| UE antenna height | Vehicle/pedestrian UE: 1.5m UE-type-RSU: 5 m | Vehicle/pedestrian UE: 1.5m UE-type-RSU: 5 m |
| UE antenna gain | Vehicle UE: 3dBi Pedestrian UE: 0dBi UE-type RSU: 3dBi | Vehicle UE: 3dBi Pedestrian UE: 0dBi UE-type RSU: 3dBi |
| UE receiver noise figure | Vehicle UE: 9dB UE-type RSU: 9dB | Vehicle UE: 9dB UE-type RSU: 9dB |
| Traffic model | [50 messages] per 1 second with absolute average speed of [100-250 km/h] (relative speed: 200 – 500km/h) in TR38.913 Note: This value is tentative. After SA1 input, it can be modified | [50 messages] per 1 second with absolute average speed of [100-250 km/h] (relative speed: 200 – 500km/h) in TR38.913 Note: This value is tentative. After SA1 input, it can be modified |
| Traffic load (Resource utilization) | FFS | FFS |
| UE distribution | Follow the modeling of TR 36.885 with UE speed in TR38.913 Vehile UE location update should be used for the evaluation of PRR in sidelink or communication interruption in uplink/downlink. Vehicle UE location update may not be assumed for the evaulation of PRR in uplink/downlink | Follow the modeling of TR 36.885 with UE speed in TR38.913 Vehile UE location update should be used for the evaluation of PRR in sidelink or communication interruption in uplink/downlink. Vehicle UE location update may not be assumed for the evaulation of PRR in uplink/downlink |
| UE receiver | MMSE-IRC as the baseline receiver Note: Advanced receiver is not precluded. | MMSE-IRC as the baseline receiver Note: Advanced receiver is not precluded. |
| Feedback assumption | FFS | FFS |
| Channel estimation | Realistic | Realistic |
| Performance metric | Communication availability and resilience | Communication availability and resilience |