3GPP TSG RAN WG1 #85		R1-164682

May 23rd - 27th, 2016; Nanjing, China

Agenda item:	7.1.2

Source: 	Qualcomm Incorporated

Title: 	Other KPI evaluation assumptions

Document for:	Discussion and Decision

# Introduction

In the context of the ongoing Rel-14 RAN study, and latest Technical Report, on Scenarios and Requirements for Next Generation Access Technologies [1], this paper provides some evaluation considerations and proposals for the Long range scenarios.

# Background

This section includes scenarios attributes, as currently defined in the RAN TR [1], mainly for background and reference. Note that a RAN email discussion is ongoing to resolve open points and FFSs ([2]).

Two scenarios are defined in the RAN TR: “extreme rural” and “extreme long range”, as shown in the following tables.

Table 1. Attributes for extreme rural

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Below 3GHz With a priority on bands below 1GHz Around 700MHz |
| System Bandwidth | 40MHz (DL+UL) |
| Layout | Single layer: Isolated Macro cells |
| Cell range | 100 km range (Isolated cell) to be evaluated through system level simulations. 100 km is a starting point, and may be varied to meet traffic density and user experienced data rate targets Feasibility of Higher Range shall be evaluated through Link level evaluation (for example in some scenarios ranges up to 150-300km may be required). |
| User density and UE speed | [TBD] users/km² Speed up to 160 km/h |
| Traffic model | [Average data throughput at busy hours/user: 30kbps Traffic density: [TBD] kbps/km² User Experienced Data Rate: up to 2Mbps while stationary and 384kbps while moving] |

Table 2. Attributes for extreme long range

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Below 3GHz |
| System Bandwidth | [40] MHz (DL+UL) |
| Layout | Single layer: Isolated Macro cells ad hoc / isolated |
| Cell Range | 150 km] for bands above 1GHz [250 km] for between 700 MHz and 1GHz [400 km] or more for bands below 700MHz |
| User density and UE speed | TBD] users/km² Speed up to [160km/h] |
| Traffic model | [Average data throughput at busy hours/user: [30kbps] Traffic density: [380-500kbps/km²] User Experienced Data Rate: up to [2]Mbps while stationary and [384kbps] while moving] |

# Discussion

As part of the ongoing RAN email discussion ([2]), a way forward has been proposed about the above scenarios, and their evaluation. Such way forward has received some support so far, and is considered a good baseline, thus provided here for RAN1 discussion and potential endorsement.

General proposals are:

- Combine the description of the two scenarios into a single one.

- Agree on a single cell radius target, and simulate that environment with the requirement that the range edge users are serviced with a target data rate, and then observe how many users such a cell site can serve.

### Details of the proposed way forward are below (text proposal for the RAN TR), with main changes (compared to [1]) highlighted in yellow. Further specific (RAN1) simulation parameters are left for further investigation.

## 3.1 Way Forward on Long Range

========================= TR text proposal =====================================

## 6.1.X Extreme rural with extreme Long Range

The extreme Long Range deployment scenario is defined to allow for the Provision of services for very large areas such as wilderness or areas where only highways are located primarily for humans and for the Provision of minimal services over long distances for Low ARPU and Low density areas including both humans and machines. The key characteristics of this scenario are Macro cells with very large area coverage supporting basic data speeds and voice services, with low to moderate user throughput and low user density.

Table 6.1.x-1: Attributes for extreme long range

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Below 3GHz With a priority on bands below 1GHz Around 700MHz |
| System Bandwidth | 40MHz (DL+UL) |
| Layout | Single layer: Isolated Macro cells |
| Cell range | 100 km range (Isolated cell) to be evaluated through system level simulations. 100 km is a starting point, and may be varied to meet traffic density and user experienced data rate targets Feasibility of Higher Range shall be evaluated through Link level evaluation (for example in some scenarios ranges up to 150-300km may be required). |
| User density and UE speed | User density: NOTE1 UE speed: Up to 160 km/h |
| Traffic model | Average data throughput at busy hours/user: 30kbps User Experienced Data Rate: up to 2Mbps while stationary and 384kbps while moving |

NOTE1: Evaluate how many users can be served per cell site when the range edge users are serviced with the target user experience data rate.

========================== End of TR text proposal ===============================

# Conclusion and Proposal

It is proposed to review and discuss the above way forward for the characterization and evaluation of NR Long Range scenarios. In particular:

Proposal: Agree on a single cell radius target (e.g. 100km), and simulate that environment with the requirement that the range edge users are serviced with a target data rate, and then observe how many users such a cell site can serve.

# 5. References

[1] 3GPP TR 38.913 V0.3.0: SI TR on Scenarios and Requirements for Next Generation Access Technologies

[2] [RAN#71-03] Open issues on scenarios & KPIs <Part 3 (Deployment scenarios: 6.1.6, 6.1.7 Extreme long range)>